| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5100315 (5.10.3.15: Transition initiated by driver) Total: 6 Test case(s) |  |
| REF: | Subset-076-5-2-5100315 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | Creation / New in BS3 | INECO |
| Version 2.9.1 16.05.2013 | All | First version / TCs Template updated; TCs not affected by UNISIG remarks from 30/04/2013 - “Followed By” and “Proceeded By” links kept. | INECO |
| Version 2.9.2 01.08.2013 | All | Second version/Internal modification; TC_Editorv27; Correction of Requirements blank space; Requirements related to table 4.7.2- Additional Data removed from TC1. | INECO |
| Version 2.9.3 03.01.2014 | TC1 | Third version/ Requirement related with table 4.5.2 inserted. | INECO |
| Version 2.9.4 03.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | Train is at standstill and a radio communication session is opened. The driver changes the level from 2 or 3 to any other stored in the table of priority. The system switches to selected level. On-board equipment reports the new level to the RBC. When receiving the level change report, the RBC orders the communication session to be terminated. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS/ LS/ SR/ OS/ NL L3: FS/ LS/ SR/ OS/ NL |  |  |
| Target of Test | The target of this test is to check that: The on-board switches to selected level after driver selection amongst those stored in the table of priority, The on-board equipment reports the new level to the RBC. RBC orders the communication session to be terminated when the report has been received. The on-board switches to the level selected by the driver. The function “Report train position when change of level due to driver request“is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-3.18.4.2.5 |  |
|  |  | Subset-026-5.10.3.15.3 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to driver request |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The level initially displayed on the DMI changes to the one selected by the driver amongst those stored in the table of priority. Check on the JRU that : The new current level is RECORDED Position report (message 136) is RECORDED. Termination of the communication session is RECORDED. (only if the selected level is different from L2 or 3) |
| Constraints | Train at standstill.(V_TRAIN=0) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS/ OS/ SR/ NL/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/L3 |
| Table of priority of trackside supported levels | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" or “Level 3” is displayed | O | DMI | Symbols LE04/LE05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS / SR / OS / NL | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L2 / L3 | FS / LS / SR / OS / NL | - |
| 2 | L2 / L3 | FS / LS / SR / OS / NL | Driver selects "level X". | I | DMI | Level X different from the applicable one and among those stored in the table of priority.(L0/LNTC/L1) | L2 / L3 | FS / LS / SR / OS / NL | - |
| 3 | L2 / L3 | FS / LS / SR / OS / NL | The driver confirms "Level" | I | DMI | - | L2 / L3 | FS / LS / SR / OS / NL | - |
| 4 | L2 / L3 | FS / LS / SR / OS / NL | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2 / L3 | FS / LS / SR / OS / NL | - |
| 5 | L2 / L3 | FS / LS / SR / OS / NL | The Level symbol "Level 0/NTC/1" is displayed | O | DMI | One of the Symbols LE01/LE02/LE03 | LX | FS / LS / SR / OS / NL | - |
| 6 | LX | FS / LS / SR / OS / NL | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit01=1/ Bit02=1/Bit03=1) is recorded | O | JRU | - | LX | FS / LS / SR / OS / NL | - |
| 7 | LX | FS / LS / SR / OS / NL | SA- DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | Level change report | LX | FS / LS / SR / OS / NL | - |
| 8 | LX | FS / LS / SR / OS / NL | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | LX | FS / LS / SR / OS / NL | - |
| 9 | LX | FS / LS / SR / OS / NL | RBC orders the communication session to be terminated | - | - | USE_FT3050500.5 | LX | FS / LS / SR / OS / NL | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | - |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 11 / 12 | FS/ OS/ SR/ NL/ LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | FINITE VALUE | CHANGED to LX (X = different from the initial one) |
| Table of priority of trackside supported levels | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level X” is displayed | O | DMI | new selected level (symbol LE0X) |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | After SoM, the Driver manually changes level to L2/3 and one Mobile Terminal is registered to a Radio Network and a valid RBC ID/ phone number is available. Immediately after the ERTMS/ETCS on-board equipment establishes a communication session with the RBC. |  | 2 |
| Applicable Mode/Level Combinations | L0 : UN/ SB/ NL LNTC : SB/ NL/ SN L1: FS/ LS/ OS/ SR/ SB/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that (after a Start of Mission): The driver manually changes to level 2 or 3 On-board Equipment immediately establishes a communication session with the available RBC |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 d) |  |
|  |  | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 a) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The level transition to L2/L3 initiated by the driver is DISPLAYED New selected level (2 or 3) is DISPLAYED Check on the JRU that: DRIVER actions are RECORDED. New current level is RECORDED. The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. Driver ID is valid Mobile TERMINAL registered to a RADIO NETWORK. RBC IDENTITY + PHONE NUMBER STATUS are VALID. Radio link is not Connected. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12/ 13 / 14 | FS/ OS/ SR/ UN/ SB/ TR/ NL/ LS/ SN/ RV |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 0/ 1 / 2 | L0/ LNTC / L1 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level X" is displayed | O | DMI | Level X = LE01/ 02 /03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 2 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The driver selects "Level 2 or “Level 3" and "Contact last RBC" | I | DMI | Level different from the applicable one and among those stored in the table of priority. | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 3 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The driver confirms "Level" | I | DMI | - | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 4 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 5 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | The Level symbol "Level 2"/ "Level 3" is displayed | O | DMI | One of the Symbols LE04 or LE05 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 6 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit04=1/ Bit05=1) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 7 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-CONNECT. Request is transmitted | O | RTM | Immediately, the on-board establishes a communication session with the RBC | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 8 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-CONNECT. Confirm is received | I | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 9 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 10 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 11 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 12 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 13 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted. | O | RTM | - | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |
| 14 | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside | L2/L3 | FS/ LS/ OS/ SR/ UN/ SB/ TR/ NL/ SN/ RV | - |

| Step 9: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 11: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 13: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 11 / 12/ 13 / 14 | FS/ OS/ SR/ UN/ SB/ TR/ NL/ LS/ SN/ RV |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3 / 4 | CHANGED to L2/L3 VALID |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | - |
| RBC ID/Phone Number | STORED | - |
| RBC/RIU System Version | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2/ Level 3" is displayed | O | DMI | new selected level (symbol LE04 or LE05) |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | The driver enters/validates the Radio data (RBC Identity + Phone Number Status).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. After SoM and after the driver has changed manually the level to L2/3. (covered by FT3180400.1) |  | 3 |
| Applicable Mode/Level Combinations | L2 : FS/ LS/ OS/ SR / PT /NL L3: FS/ LS /OS /SR / PT /NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission after the driver has changed manually to level to L2/3 : The driver selects “Radio data” from Radio data window list, because no valid RBC ID/Phone Number stored on-board The driver enters and confirms the RBC ID/Phone Number The on-board equipment establishes a radio communication session with the registered RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 h) |  |
|  |  | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 b) |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |
|  |  | Subset-026-8.7.12 Message 32: RBC/RIU System Version |  |
|  |  | Subset-026-8.6.17 Message 159: Session established |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: “Radio data” is available for driver selection Check on the JRU that: The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. If the level transition leads to TR mode, the request for Radio data is only displayed once the ERTMS/ETCS on-board equipment is in PT mode. Driver ID is valid. LEVEL status is Valid. Mobile TERMINAL is registered to RADIO NETWORK and RBC IDENTITY + PHONE NUMBER STATUS are INVALID. Radio communication session is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The data from "Radio data" window are displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | N/A | N/A | - |
| 2 | N/A | N/A | The driver presses "Radio data". | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters "Radio data". | I | DMI | RBC ID/ Phone number | N/A | N/A | - |
| 4 | N/A | N/A | The driver confirms "Radio data". | I | DMI | RBC ID/ Phone number | N/A | N/A | - |
| 5 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24) is recorded | O | JRU | The variable Q_RBCENTRY=2 (Enter Radio data) and the variables NID_C, NID_RBC and NID_RADIO identical to the data entered by the driver. | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | the on-board initiates a radio communication session with the RBC | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Communication session established for on-board. | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 2 is transmitted. | O | RTM | - | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Communication session established for trackside | N/A | N/A | - |

| Step 8: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a communication session |
| L_MESSAGE | 10 | 74 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |

| Step 10: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | RBC/RIU System Version |
| L_MESSAGE | 10 | 82 | - |
| T_TRAIN | 32 | FINITE VALUE | Time of the train approximated by RBC |
| M_ACK | 1 | 0 | No ack |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 12: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | Session established |
| L_MESSAGE | 10 | 74 | In case of including optional packets, length would be greater |
| T_TRAIN | 32 | FINITE VALUE | Time of the train |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID of the Train |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3/4 | L2/L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | The driver enters/validates the Radio data (Radio Network ID).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. After SoM and after the driver has changed manually to level to L2/3. (covered by FT3180400.2) |  | 4 |
| Applicable Mode/Level Combinations | L2 : FS/ LS/ OS/ SR/ PT /NL L3: FS/ LS /OS /SR / PT /NL |  |  |
| Target of Test | The target of this test is to check that, after a Start of Mission after the driver has changed manually to level to L2/3 : The driver selects “Radio network ID” from Radio data window, because no Mobile Terminal is registered to a Radio Network The driver enters the Radio network ID information The on-board equipment establishes a radio communication session with the registered RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.10.3.15.2 |  |
|  |  | Subset-026-5.10.3.15.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: “RBC network ID” is available for driver selection Check on the JRU that: The on-board establishing a Radio communication session is RECORDED. |
| Constraints | Train is at standstill. If the level transition leads to TR mode, the request for Radio data is only displayed once the ERTMS/ETCS on-board equipment is in PT mode. Driver ID is valid. LEVEL status is Valid. No Mobile TERMINAL is registered to the RADIO NETWORK. Radio communication session is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 11 / 12 | FS / OS / SR / PT / NL / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | Entered after the desk is opened VALID |
| Radio Network ID | NOT STORED | - |
| RBC ID/Phone Number | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / PT / NL / LS | Train is at standstill. | I | INT | (V_TRAIN=0; TIME= 60) | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 2 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver presses "Radio network ID". | I | DMI | - | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 3 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver selects a new “Radio network ID ". | I | DMI | - | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 4 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver confirms “Radio network ID ". | I | DMI | Validates the Radio network ID | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 5 | L2/L3 | FS / OS / SR / PT / NL / LS | RADIO NETWORK ID ENTERED BY THE DRIVER (NID_MESSAGE_JRU=48) is recorded. | O | JRU | The variable NID_MN identical to the data selected by the driver. | L2/L3 | FS / OS / SR / PT / NL / LS | - |
| 6 | L2/L3 | FS / OS / SR / PT / NL / LS | The driver enters the Radio data (RBC Identity + Phone Number Status).The ERTMS/ETCS on-board equipment establishes a communication session with the RBC. | - | - | USE_FT5100315.3 | L2/L3 | FS / OS / SR / PT / NL / LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| Radio communication session | ESTABLISHED | CHANGED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | STORED | - |
| Radio Network ID | STORED | VALID |
| RBC ID/Phone Number | STORED | VALID |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | Train is at standstill and a radio communication session is opened. The driver changes the level from 2 or 3 to any other. After switching to selected level, the on-board equipment reports the new level to the RBC and no order to terminate the session is received within a fixed waiting time (see Appendix A.3.1) from the time the position report was sent. On-board equipment repeatedly sends a position report with the fixed waiting time after each repetition, until the defined number of repetitions (see Appendix A.3.1) has been reached. As no reply is received within the fixed waiting time after the last repetition, the on-board equipment initiates the termination of the communication session. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS / OS / SR / SB / NL / LS L3: FS / OS / SR / SB / NL / LS |  |  |
| Target of Test | The target of this test is to check that: The on-board switches to the level selected by the driver, The on-board equipment sends a Position Report indicating the level transition to level 0, NTC, 1. No order to terminate the session is received from the RBC. The on-board repeats a defined number of times (3 times each 15s) the sending of a Position Report. After waiting within the fixed waiting time (60s), for RBC to respond, the termination of the communication session is initiated by the on-board equipment. No acknowledgement is received from the RBC. The on-board repeats a defined number of times (3 times each 15s) the sending of the radio message 156 “Termination of communication session”. After waiting within the fixed waiting time (60s), the on-board equipment considers the Radio communication session terminated and no further message is sent by the on-board once the radio message 156 has been sent. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 g) |  |
|  |  | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-5.10.3.15.3 |  |
|  |  | Subset-026-5.10.3.15.4 |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to driver request |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The level initially displayed on the DMI changes to the one selected by the driver. Check on the JRU that : The new current level is RECORDED The four radio messages 136 (Train Position Report) sent by the on-board are recorded with the correct timing. The four radio message 156 (Termination of a communication session) sent by the on-board are recorded with the correct timing. No more message transmitted by the on-board equipment after the fourth message 156 is recorded and after waiting within the fixed waiting time. |
| Constraints | Train at standstill.(V_TRAIN=0) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" or “Level 3” is displayed | O | DMI | Symbols LE04 or LE05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / SB / NL / LS | Train is at standstill (V_TRAIN=0d) | I | INT | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 2 | L2 / L3 | FS / OS / SR / SB / NL / LS | The driver selects "Level <L0/LNTC/L1>" | I | DMI | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 3 | L2 / L3 | FS / OS / SR / SB / NL / LS | The driver confirms "Level" | I | DMI | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 4 | L2 / L3 | FS / OS / SR / SB / NL / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 5 | L2 / L3 | FS / OS / SR / SB / NL / LS | The Level symbol "Level <L0/LNTC/L1>" is displayed | O | DMI | One of the Symbols LE01/LE02/LE03 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 6 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit01=1¦ Bit02=1¦Bit03=1) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 7 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 8 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 9 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No order to terminate the session is received from RBC and a time out occurs (15 s). The first repetition of position report message is sent. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 10 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 11 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No order to terminate the session is received from RBC and a time out occurs (15 s). The second repetition of position report message is sent. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 12 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 13 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No order to terminate the session is received from RBC and a time out occurs (15 s). The third repetition of position report message is sent. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 14 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 15 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | No order to terminate the session is received from RBC and a time out occurs (60 s). | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 16 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 17 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No Message 39 (Acknowledgement of termination of a communication session) is received from RBC and a time out occurs (15 s). | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 18 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | First repetition | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 19 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 20 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No Message 39 (Acknowledgement of termination of a communication session) is received from RBC and a time out occurs (15 s). | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 21 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Second repetition | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 22 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 23 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No Message 39 (Acknowledgement of termination of a communication session) is received from RBC and a time out occurs (15 s). | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 24 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Third repetition | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 25 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 26 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | NOT SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | No Message 39 (Acknowledgement of termination of a communication session) is received from RBC and a time out occurs (15 s). | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 27 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DISCONNECT. Request is transmitted | O | RTM | No answer is received from the RBC and the time out occurs (60 s). The on-board releases the safe connection. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 15: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 18: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 21: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| Step 24: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / OS / SR / UN / SB / NL / LS / SN |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | FINITE VALUE | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level 0” or “Level NTC” or “Level 1” is displayed | O | DMI | New selected level symbol LE01 or LE02 or LE03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.10.3.15: Transition initiated by driver |  | 5100315 |
| Test Case of Feature | Train is at standstill and a radio communication session is opened. The driver changes the level from 2 or 3 to any other. After switching to selected level, the on-board equipment reports the new level to the RBC and no order to terminate the session is received within a fixed waiting time (see Appendix A.3.1) from the time the position report was sent. On-board equipment repeatedly send a position report with the fixed waiting time after each repetition, until the order to terminate the session is received from RBC. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS / OS / SR / SB / NL / LS L3: FS / OS / SR / SB / NL / LS |  |  |
| Target of Test | The target of this test is to check that: The on-board switches to the level selected by the Driver, The on-board equipment sends a Position Report indicating the level transition to level 0, NTC, 1. No order to terminate the session is received from the RBC. The on-board repeats the sending of Position Report, until the order to terminate the session is received from the RBC and then the on-board equipment terminates the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 g) |  |
|  |  | Subset-026-3.18.4.2.4 |  |
|  |  | Subset-026-5.10.3.15.3 |  |
|  |  | Subset-026-5.10.3.15.4 |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when change of level due to driver request |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The level initially displayed on the DMI changes to the one selected by the driver. Check on the JRU that : The new current level is RECORDED The radio messages 136 (Train Position Report) sent by the on-board are recorded with the correct timing. Termination of the communication session is RECORDED. |
| Constraints | Train at standstill.(V_TRAIN=0) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level 2" or “Level 3” is displayed | O | DMI | Symbols LE04 or LE05 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / SB / NL / LS | Train is at standstill (V_TRAIN=0d) | I | INT | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 2 | L2 / L3 | FS / OS / SR / SB / NL / LS | The driver selects "Level <L0/LNTC/L1>" | I | DMI | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 3 | L2 / L3 | FS / OS / SR / SB / NL / LS | The driver confirms "Level" | I | DMI | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 4 | L2 / L3 | FS / OS / SR / SB / NL / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | L2 / L3 | FS / OS / SR / SB / NL / LS | - |
| 5 | L2 / L3 | FS / OS / SR / SB / NL / LS | The Level symbol "Level <L0/LNTC/L1>" is displayed | O | DMI | One of the Symbols LE01/LE02/LE03 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 6 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; Bit01=1¦ Bit02=1¦Bit03=1) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 7 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 8 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 9 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No order to terminate the session is received from RBC and a time out occurs (15 s). The first repetition of position report message is sent. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 10 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 11 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. | O | RTM | No order to terminate the session is received from RBC and a time out occurs (15 s). The second repetition of position report message is sent. | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 12 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |
| 13 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | The on-board equipment terminates the communication session as ordered by RBC | - | - | Use_FT3050500.5 | L0 / LNTC / L1 | FS / OS / SR / UN / SB / NL / LS / SN | - |

| Step 7: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 9: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | L0 / LNTC / L1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 11 / 12 / 13 | FS / OS / SR / UN / SB / NL / LS / SN |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | FINITE VALUE | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol “Level 0” or “Level NTC” or “Level 1” is displayed | O | DMI | New selected level symbol LE01 or LE02 or LE03 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill. | I | INT | - |