| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080422 (4.8.4: Accepted Information depending on the modes - Location Identity (NID_C + NID_BG transmitted in the balise telegram)) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-4080422 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 20.08.2013 | All | General revision | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Location Identity (NID_C + NID_BG transmitted in the balise telegram) |  | 4080422 |
| Test Case of Feature | The Location Identity (NID_C + NID_BG) of the BG message is accepted |  | 1 |
| Applicable Mode/Level Combinations | L0: UN, SH, PS, NL, SL, SB, TR LNTC: SN, NL, SL, SB, SH, PS, TR L1: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, TR, PT, SB, NL, RV |  |  |
| Target of Test | The target of this test is to check that: When no linking information is used or when linking information is used on-board, BGs marked as linked are accepted |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Location Identity (NID_C + NID_BG transmitted in the balise telegram) |  |
|  |  | Subset-026-4.8.4.2 Location Identity (NID_C + NID_BG) |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-7.4.2.39 Packet Number 255: end of information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The balise group message is RECORDED. The variable TRAIN_POSITION is recorded in the header of the JRU message with the correct value of NID_LRBG |
| Constraints | A cab needs to be active (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11/ 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 255 is received | I | BTM | The train overpasses all the balises in the BG. The BG is marked as linked. No message consistency problem: All the telegrams are correctly received. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; NID_LRBG=<equal to the identity of the previous BG>) is recorded | O | JRU | The variable TRAIN_POSITION is recorded in the header of the JRU message with the correct value of NID_LRBG (i.e., the value of the Location Identity -NID_C + NID_BG-, given in the BG) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11/ 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |