| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3150400 (3.15.4: Reversing of movement direction) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-3150400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.09.2013 | All | review | DLR |
| Version 2.9.2 25.11.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.4: Reversing of movement direction |  | 3150400 |
| Test Case of Feature | Update of the reversing area information. |  | 1 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT, RV L3: FS, LS, OS, SR, SB, PT, RV |  |  |
| Target of Test | The target of test is to check that: • The on-board has reversing area information stored, and it receives new reversing area information that replaces it. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.15.4.1.1 |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-3.7.3.1 |  |
|  |  | Subset-026-3.7.3.1 k) |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-3.15.4.3 |  |
|  |  | Subset-026-3.15.4.2.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU that : • A message sent from RBC containing reversing information has been RECORDED. |
| Constraints | Reversing Area Information is stored on-board Acknowledgement of train data must have been received. In PT mode the information “Recognition of exit from TR mode” has been received |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 / 14 | FS / OS / SR / SB / PT / LS / RV |
| Radio communication session | ESTABLISHED | - |
| Mode Profile | STORED | Mode profile shall be stored in all applicable modes except PT. In PT mode, the message “Recognition of exit from TR mode” shall be stored instead. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 138 and packet 139 is received | I | RTM | The message is accepted. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | Train enters in reversing area | N/A | N/A | - |
| 4 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 138 | Reversing area information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | - |
| L_REVERSEAREA | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 139 | Reversing supervision information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | - |
| V_REVERSE | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 / 14 | FS / OS / SR / SB / PT / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Reversing Area Information | STORED | The new Reversing information must be different from the previous stored one |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.4: Reversing of movement direction |  | 3150400 |
| Test Case of Feature | Update of the reversing area information. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB, RV |  |  |
| Target of Test | The target of test is to check that: - The on-board has reversing area information stored, and it receives new reversing area information that replaces it. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.4 |  |
|  |  | Subset-026-3.15.4.1.1 |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Supervision Information |  |
|  |  | Subset-026-4.8.3.1.1 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Area Information |  |
|  |  | Subset-026-4.8.4.2 Reversing Supervision Information |  |
|  |  | Subset-026-7.4.2.34 Packet Number 138: Reversing area information |  |
|  |  | Subset-026-7.4.2.35 Packet Number 139: Reversing supervision information |  |
|  |  | Subset-026-3.15.4.3 |  |
|  |  | Subset-026-3.15.4.2.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the BTM that : • The on-board system receives a general telegram including Packets 138 (Reversing area information) and 139 ( Reversing supervision information) Check on the JRU that : • The general telegram including Packets 138 (Reversing area information) and 139 ( Reversing supervision information) is RECORDED |
| Constraints | Reversing Area Information is stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 / 14 | FS / OS / SR / SB / LS / RV |
| ERTMS/ETCS level | 2 | L1 |
| Reversing Area Information | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 138, packet 139 and packet 255 is received | I | BTM | The message is accepted. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | Train enters in reversing area | N/A | N/A | - |
| 4 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | The Status symbol "Reversing permitted" is displayed | O | DMI | Symbol ST06 in C6 area is displayed | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit43=1>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group. Value must be the same for the whole group of balises To make sure the balise group is not considered as missed. (see SRS 2.2.2: 3.16.2.4.6). |
| NID_C | 10 | 00 | Country or region No duplicate |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 138 | Reversing area information |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STARTREVERSE | 15 | FINITE VALUE | Distance to start of reversing permitted area |
| L_REVERSEAREA | 15 | FINITE VALUE | Length of the reversing permitted area |
| NID_PACKET | 8 | 139 | Reversing supervision information |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REVERSE | 15 | FINITE VALUE | Distance in RV mode |
| V_REVERSE | 7 | FINITE VALUE | Maximum speed in RV mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 / 14 | FS / OS / SR / SB / LS / RV |
| ERTMS/ETCS level | 2 | L1 |
| Reversing Area Information | STORED | The new Reversing information must be different from the previous stored one |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |