| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3050500 (3.5.5: Terminating a communication session) Total: 11 Test case(s) |  |
| REF: | Subset-076-5-2-3050500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 31.03.2013 | All | review | DLR |
| Version 2.9.2 02.12.2013 | 7 | Added packet 72 for step 5 | DLR |
| Version 2.9.3 19.12.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment shall terminate the communication session according to the orders received from the trackside (balise group or Radio Infill units) or when the level 1 is left |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that : • The Onboard terminates the communication session to the RIU when the train has left the Level 1 area. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-3.9.3.16 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The Onboard has a connection to the RIU • The train receives a Level transition order with an order to switch to Level y (L0, LNTC, L2, L3) • The train overpasses the position where the new level is required. • The Onboard terminates the communication session to RIU |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS, OS, SR, LS |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | N/A | One Balise Group message containing packet 41 and packet 255 is received | I | BTM | M_LEVELTR = 0 or 1 or 3 or 4 | L1 | N/A | - |
| 2 | L1 | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | N/A | - |
| 3 | L1 | N/A | Use_FT5100400.4,8: “The driver acknowledges before entering the L0 / LNTC area.” | - | - | If D_LEVELTR > 0 and new level is 0 or NTC. | L1 | N/A | - |
| 4 | L1 | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0 / 1 / 3 / 4>) is recorded | O | JRU | The train switches to Level y (0, NTC, 2, 3) | L0 / LNTC / L2 / L3 | N/A | - |
| 5 | L0 / LNTC / L2 / L3 | N/A | The Level symbol "Level 0" OR the Level symbol "Level NTC" OR the Level symbol "Level 2" OR the Level symbol "Level 3" is displayed | O | DMI | - | L0 / LNTC / L2 / L3 | N/A | - |
| 6 | L0 / LNTC / L2 / L3 | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit01=1¦Bit02=1¦Bit04=1¦Bit05=1>) is recorded | O | JRU | Level 0 MASK=0000000000000000000001; Level NTC MASK=0000000000000000000002; Level 2 MASK=0000000000000000000008; Level 3 MASK=0000000000000000000010 | L0 / LNTC / L2 / L3 | N/A | - |
| 7 | L0 / LNTC / L2 / L3 | N/A | Use_FT5100400.6,9: “The driver acknowledges […] after the Level transition.” | - | - | If D_LEVELTR = 0 and new level is 0 or NTC. | L0 / LNTC / L2 / L3 | N/A | - |
| 8 | L0 / LNTC / L2 / L3 | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | L0 / LNTC / L2 / L3 | N/A | - |
| 9 | L0 / LNTC / L2 / L3 | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | N/A | - |
| 10 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | The session is terminated | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The onboard tries to release the safe connection | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | 1 | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | 0 or 1 or 3 or 4 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 10: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | Acknowledgement of termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS, OS, SR, LS |
| Radio communication session | NOT ESTABLISHED | RELEASED |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is NOT CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The Onboard terminates the communication session to the RBC until an End of Mission is executed, but no acknowledgement from RBC (Message 39) is received. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that : • The Onboard terminates the communication session to the RIU/RBC until an End of Mission is executed, but no acknowledgement from RBC (Message 39) is received. Please make sure, that this TC will be tested at the end of a test sequence |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.3.1 |  |
|  |  | Subset-026-3.5.5.3.2 |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-A.3.1 - Waiting time before radio message repetition |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The Onboard has a connection to the RUI/RBC • The train stops • The driver executes an EoM (Closing of Desk) • The Onboard tries to terminate the communication session to RIU/RBC • No Acknowledgement from RIU/RBC is received |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | FS, OS, SR, LS | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | N/A | FS, OS, SR, LS | - |
| 2 | N/A | FS, OS, SR, LS | Cab not active | I | TIU | The driver closes the desk The Onboard switches to SB mode. | N/A | FS, OS, SR, LS | - |
| 3 | N/A | FS, OS, SR, LS | CAB STATUS (NID_MESSAGE_JRU=38) is recorded | O | JRU | The driver’s selection is recorded on the JRU; M_CAB_A_STATUS =0 | N/A | FS, OS, SR, LS | - |
| 4 | N/A | FS, OS, SR, LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | Mode change to SB is RECORDED | N/A | SB | - |
| 5 | N/A | SB | SA-DATA.Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) | O | RTM | - | N/A | SB | - |
| 6 | N/A | SB | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | SB | - |
| 7 | N/A | SB | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received | I | RTM | - | N/A | SB | - |
| 8 | N/A | SB | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | SB | - |
| 9 | N/A | SB | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | N/A | SB | - |
| 10 | N/A | SB | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded OR MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Depending on current level: for L1, RIU. For levels 2 / 3, RBC | N/A | SB | - |
| 11 | N/A | SB | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | First repetition after 15 s No Message 39 (Acknowledgement of termination of a communication session) has been received. | N/A | SB | - |
| 12 | N/A | SB | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded OR MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Depending on current level: for L1, RIU. For levels 2 / 3, RBC | N/A | SB | - |
| 13 | N/A | SB | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Second repetition 15 s after first repetition. | N/A | SB | - |
| 14 | N/A | SB | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded OR MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Depending on current level: for L1, RIU. For levels 2 / 3, RBC | N/A | SB | - |
| 15 | N/A | SB | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | Third repetition 15 s after second repetition. | N/A | SB | - |
| 16 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded OR MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Depending on current level: for L1, RIU. For levels 2 / 3, RBC | N/A | N/A | - |
| 17 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | 15s after the third and last repetition the on-board considers the communication session terminated. | N/A | N/A | - |

| Step 5: Radio Message 150 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 150 | End of Mission |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | 2 / 3 / 4 | L1 / L2 / L3 |

| Step 7: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value “Contact last known RBC” |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | not relevant if NID_RBC has value “Contact last known RBC” |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 9: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 11: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 13: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 15: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | Termination of a communication session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | RELEASED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The train tries to establish a radio connection while the train receives an order to terminate the radio connection and the train stops trying to connect. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR L2: FS, LS, OS, SR L3: FS, LS, OS, SR |  |  |
| Target of Test | The target of this test is to check that : • The train tries to establish a radio connection while the train receives an order to terminate the radio connection and the train stops trying to connect. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.8 |  |
|  |  | Subset-026-3.5.3.8 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • The Onboard has NO connection to the RBC • The train overpasses a Balise group with a session management package with Q_RBC = 1 • The train tries to establish a radio connection to the announced RBC • The train overpasses a Balise group with a session management package with Q_RBC = 0 • The train releases the connection |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | Radio Link is NOT CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 42 and packet 255 is received | I | BTM | Only if connection to RBC. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 133 and packet 255 is received | I | BTM | Only if connection to RIU. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | Train tries to set up a safe connection. | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only if connection to RBC. | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | Only if connection to RIU. | N/A | N/A | - |
| 11 | N/A | N/A | One Balise Group message containing packet 42 (Q_RBC = 0; NID_C = <same as Step 1>; NID_RBC = <same as Step 1>; NID_RADIO = <same as Step 1>) and packet 255 is received | I | BTM | Only if connection to RBC. | N/A | N/A | - |
| 12 | N/A | N/A | One Balise Group message containing packet 133 (Q_RIU = 0; NID_C = <same as Step 2>; NID_RBC = <same as Step 2>; NID_RADIO = <same as Step 2>) and packet 255 is received | I | BTM | Only if connection to RIU. | N/A | N/A | - |
| 13 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The communication session is not considered established for the on-board. | N/A | N/A | - |
| 15 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The on-board releases the safe connection | N/A | N/A | - |
| 16 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | When the indication status for the safe radio connection is “No Connection”, no symbol shall be displayed. | N/A | N/A | - |
| 17 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 8: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | Initiation of a Communication Session |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 11: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | Identity number of the country or region; same as NID_C of step 1 |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 12: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | Identity number of the country or region; same as NID_C of step 2 |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | - |
| D_INFILL | 15 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | Refers to the next main signal balise group (relevant only for the case of establishing a communication session). |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS, OS, SR, LS |
| Radio communication session | NOT ESTABLISHED | RELEASED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is NOT CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The on-board equipment does not terminate a communication session ordered by RBC in SL if Q_SLEEPSESSION = 0 |  | 4 |
| Applicable Mode/Level Combinations | L0: SL LNTC: SL L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that: • After receiving a condition requiring termination of the communication session from the RBC except for SL mode, the on-board equipment does not terminate the communication session if in SL mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 a) |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 24: “General Message”, including Packet 42 “Session Management”, is RECORDED. |
| Constraints | A communication session between onboard equipment and RBC is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | “Connection Up” symbol ST03 is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0; Q_SLEEPSESSION=0) is received | I | RTM | Q_RBC = 0 (Terminate communication session) Q_SLEEPSESSION = 0 (Ignore session establishment order) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DISCONNECT.Request is transmitted | O | RTM | The disconnection primitive is NOT sent. Check that no further messages are sent. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 0 | Ignore session establishment order |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The on-board equipment terminates a communication session ordered by RBC. |  | 5 |
| Applicable Mode/Level Combinations | L0: UN, PS, SL, SB, TR LNTC: PS, SL, SB, TR, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • After receiving a condition requiring termination of the communication session from the RBC, the onboard equipment terminates the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 a) |  |
|  |  | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.3.1.1 Acknowledgement of session termination |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-4.8.4.2 Acknowledgement of session termination |  |
|  |  | Subset-026-5.5.3.1.3 c) |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 24: “General Message”, including Packet 42 “Session Management”, is RECORDED. • Message 156: “Termination of a communication session” is RECORDED. • Message 39: “Acknowledgement of termination of a communication session” is RECORDED. |
| Constraints | A communication session between onboard equipment and RBC is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received | I | RTM | Q_RBC = 0 (Terminate communication session) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | Message 39: “Acknowledgement of termination of a communication session” is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 0 | Terminate communication session. |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | 1 | Order to contact RBC also in SL mode |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 + t’ | t’ > 0 |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | A termination of the communication session is required. The on-board equipment sends a radio message 156 “Termination of communication session” to the RBC. It receives from the RBC a radio message 24 “General message” requiring an acknowledgement. The message is ignored. On reception of the radio message 39 “Acknowledgement of termination of communication session” from the RBC, the onboard equipment considers the communication session terminated and releases the safe connection. |  | 6 |
| Applicable Mode/Level Combinations | L0: UN, SH, PS, SL, SB, TR, NL LNTC: SH, PS, SL, SB, TR, NL, SN L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that : • The onboard equipment ignores the messages received from the RBC after the sending of the message 156 with the exception of message 39: Acknowledgement of termination of communication session”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.6 |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.4.2 Session Management |  |
|  |  | Subset-026-4.8.3.1.1 Acknowledgement of session termination |  |
|  |  | Subset-026-4.8.4.2 Acknowledgement of session termination |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • A radio message 156 (Termination of communication session) is sent to the RBC. • A radio message 24 (General message) is received from the RBC. • A radio message 39 (Acknowledgement of termination of communication session) is received from the RBC. Check on the DMI that: • The driver is informed about the termination of the communication session Check on the JRU that: • The radio message 156 (Termination of communication session) sent by the on-board is recorded. • The radio message 24 (General message) received by the on-board is recorded. • No acknowledgement message 146 sent by the on-board is recorded. • The radio message 39 (Acknowledgement of termination of communication session) received by the onboard is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | “Connection Up” symbol ST03 is DISPLAYED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received | I | RTM | Q_RBC = 0 (Terminate communication session) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24, M_ACK = 1) containing packet 72 (X_TEXT(L_TEXT)='Test failed') is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Acknowledgement" (NID_MESSAGE=146) containing packet 72 is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | NOT The "plain text message" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | Radio link disconnection is sent. | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | When the indication status for the safe radio connection is “No Connection”, no symbol shall be displayed. | N/A | N/A | - |
| 14 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity : NID_C not relevant if NID_RBC has value "Contact last known RBC". |
| NID_RBC | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Not relevant if NID_RBC has value "Contact last known RBC". |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | Test failed |

| Step 10: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio link is DISCONNECTED |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | “Connection Up” symbol ST03 is REMOVED |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The on-board equipment terminates a communication session without any explicit trackside order. |  | 7 |
| Applicable Mode/Level Combinations | L0: SH, SB, UN, PS, TR, NL LNTC: SH, PS, SB, TR, SF, NL, SN L1: FS, LS, OS, SR, SH, PS, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SB, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that the termination of a communication session is initiated by the on-board, if a condition requiring the termination of the communication session is fulfilled without any explicit trackside order. Here not compatible systems versions between on-board and trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-4.8.3.1.1 Acknowledgement of session termination |  |
|  |  | Subset-026-4.8.4.2 Acknowledgement of session termination |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 156: “Termination of a communication session” is RECORDED. • Message 39: “Acknowledgement of termination of a communication session” is RECORDED. • Messages from the RBC received onboard after the Message 156: “Termination of a communication session” has been sent are ignored, with the exception of the Message 39: “Acknowledgement of termination of a communication session”. |
| Constraints | A communication session between onboard equipment and RBC is considered as established by trackside. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside not compatible" is displayed | O | DMI | Due to a communication error (e.g. ETCS system version incompatibility) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The onboard equipment detects an error condition that requires the termination of the communication session. It shall be checked that no more messages are sent If L1 message is sent to the RIU. If L2 or L3 message is sent to the RBC | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | If safe connection is set up with an RBC | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | If safe connection is set up with an RIU | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | Message 39: “Acknowledgement of termination of a communication session” is received from the RBC and the onboard equipment considers the communication session terminated. If L1 message is sent from the RIU. If L2 or L3 message is sent from the RBC | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | If safe connection is set up with an RBC | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | If safe connection is set up with an RIU | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The onboard tries to release the safe connection. | N/A | N/A | - |

| Step 1: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |

| Step 4: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 + t’ | t’ > 0 |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SB / TR / PT / NL / LS / SN / RV / PS |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| The system status message "Trackside not compatible" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The on-board equipment terminates a communication session without any explicit trackside order. |  | 8 |
| Applicable Mode/Level Combinations | L1: SL L2: SL L3: SL |  |  |
| Target of Test | The target of this test is to check that the termination of a communication session is initiated by the on-board, if a condition requiring the termination of the communication session is fulfilled without any explicit trackside order. Here not compatible systems versions between on-board and trackside. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-4.4.6.1.13 |  |
|  |  | Subset-026-4.8.3.1.1 Acknowledgement of session termination |  |
|  |  | Subset-026-4.8.4.2 Acknowledgement of session termination |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 156: “Termination of a communication session” is RECORDED. • Message 39: “Acknowledgement of termination of a communication session” is RECORDED. • Messages from the RBC received onboard after the Message 156: “Termination of a communication session” has been sent are ignored, with the exception of the Message 39: “Acknowledgement of termination of a communication session”. |
| Constraints | A communication session between onboard equipment and RBC is considered as established by trackside. The on-board has just sent Message 154 “No compatible version supported”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio Link is CONNECTED. |
| NOT RELEVANT | - | TIU | - |
| No display in Sleeping mode. | - | DMI | MASK=0000000000000000000000 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The on-board equipment detects an error condition that requires the termination of the communication session. It shall be checked that no more messages are sent | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | Message 39: “Acknowledgement of termination of a communication session” is received from the RBC and the on-board equipment considers the communication session terminated. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The onboard tries to release the safe connection. | N/A | N/A | - |

| Step 1: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |

| Step 3: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 + t’ | t’ > 0 |
| M_ACK | 1 | 0 | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 | SL |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio Link is DISCONNECTED. |
| NOT RELEVANT | - | TIU | - |
| No display in sleeping mode. | - | DMI | MASK=0000000000000000000000 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted . |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The driver closes the desk during the Start of Mission |  | 10 |
| Applicable Mode/Level Combinations | L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test case is to check that : - as soon as the driver closes the desk the on-board terminates the communication session. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-4.8.3.1.1 Acknowledgement of session termination |  |
|  |  | Subset-026-4.8.4.2 Acknowledgement of session termination |  |
|  |  | Subset-026-5.4.3.2.2 |  |
|  |  | Subset-026-8.6.14 Message 156: Termination of a communication session |  |
|  |  | Subset-026-8.7.17 Message 39: Acknowledgement of termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Closing of desk is RECORDED. • Message 156: “Termination of a communication session” is RECORDED. • Message 39: “Acknowledgement of termination of a communication session” is RECORDED. |
| Constraints | The on-board is inside the SoM procedure and a communication session is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Radio link is CONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Cab not active | I | TIU | The driver closes the desk. | N/A | N/A | - |
| 2 | N/A | N/A | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=0) is recorded | O | JRU | Closing of desk is RECORDED on the JRU. | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The on-board equipment detects the desk closed condition that requires the termination of the communication session. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | The on-board equipment considers the communication session terminated. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. | N/A | N/A | - |

| Step 3: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 |
| NID_ENGINE | 24 | FINITE VALUE | Engine ID of the train |

| Step 5: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | t0 + t’, t’ > 0 |
| M_ACK | 1 | FINITE VALUE | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | TERMINATED |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | Radio link is DISCONNECTED |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.5.5: Terminating a communication session |  | 3050500 |
| Test Case of Feature | The driver has made a shunting request. The on-board equipment sends to the RBC a radio message 130 “Request for shunting”. As it receives no answer, it repeats the shunting request message a defined number of times. After unanswered shunting request, the on-board equipment sends a radio message 156 “Termination of a communication session” to the RBC. It receives Message 39 (Acknowledgement of termination of a communication session), and it considers the communication session terminated and releases the safe connection. |  | 11 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT L3: FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is to check that : • The on-board repeats a defined number of times the sending of a radio message 130 “request for shunting” when no answer is received from the RBC. • The message is repeated with the fixed waiting time (15s) after each repetition • The termination of the communication session is initiated by the on-board equipment when no answer to the shunting request message is received. It sends a radio message 156 “Termination of a communication session” to the RBC. • No further message is sent by the on-board once the radio message 156 has been sent. • Message 39 is received, and the on-board equipment considers the communication session terminated and releases the safe connection. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.5.1 |  |
|  |  | Subset-026-3.5.5.1 b) |  |
|  |  | Subset-026-3.5.5.3 |  |
|  |  | Subset-026-3.5.5.2 |  |
|  |  | Subset-026-3.5.5.2 a) |  |
|  |  | Subset-026-3.5.5.2 c) |  |
|  |  | Subset-026-4.7.2 table - Output information Shunting request not answered by RBC |  |
|  |  | Subset-026-5.6.4.1.1 |  |
|  |  | Subset-026-5.6.4.1.2 |  |
|  |  | Subset-026-8.6.2 message 130: request for shunting |  |
|  |  | Subset-026-8.6.14 message 156: Termination of a communication session |  |
|  |  | Subset-026-A.3.1 - Repetition of radio messages (i.e. excluding the first sending) |  |
|  |  | Subset-026-8.6.14 message 156: Termination of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: • Four radio message 130 (Request for shunting) are sent to the RBC. • A radio message 156 (Termination of a communication session) is sent to the RBC. • A radio message 39 (Acknowledgement of termination of a communication session) is received from the RBC Check on the JRU that: • The four radio messages 130 (Request for shunting) sent by the on-board are recorded. • The radio message 156 (Termination of a communication session) sent by the on-board is recorded. • The radio message 39 (Acknowledgement of termination of a communication session) is recorded Check on the DMI that: • The system status message "SH request failed" is displayed. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "SHUNTING" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1011) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) containing packet 0 is transmitted | O | RTM | First shunting request message is sent to the RBC. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) containing packet 0 is transmitted | O | RTM | The first repetition of shunting request message is sent to the RBC after the fixed waiting time (15s). | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) containing packet 0 is transmitted | O | RTM | The second repetition of shunting request message is sent to the RBC after the fixed waiting time (15s). | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) containing packet 0 is transmitted | O | RTM | The third and last repetition of shunting request message is sent to the RBC after the fixed waiting time (15s). | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The system status message "SH request failed" is displayed | O | DMI | No answer is received from the RBC after the fixed waiting time (15s). | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit08=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | It shall be checked that no more RTM messages are sent. | N/A | N/A | - |
| 15 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | - | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | SA-DISCONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 19 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed | O | DMI | - | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0¦Bit41=0>) is recorded | O | JRU | - | N/A | N/A | - |

| Step 4: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0 | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 6: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0+1500 | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 8: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0+3000 | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 10: Radio Message 130 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 130 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0+4500 | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |

| Step 14: Radio Message 156 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 156 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | t0+6000 | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 16: Radio Message 39 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 39 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | always set to 0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The system status message "SH request failed" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |