| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110600 (3.11.6: Signalling related speed restrictions) Total: 8 Test case(s) |  |
| REF: | Subset-076-5-2-3110600 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 31.03.2013 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 24.06.2013 | All | Second version/ UNISIG review Comments included. Format Template modified; Dictionary steps used. | INECO |
| Version 2.9.2 06.08.2013 | All | Third version/ Lab internal version: Requirements blank-spaces corrected; Tool TCEditorv28 used. | INECO |
| Version 2.9.3 16.09.2013 | All | Fourth version/ UNISIG review comments from 28.08.2013 included. Based on Requirement field modified; TC8 created new. | INECO |
| Version 2.9.4 09.01.2014 | All | Fifth version/ Requirements blank-spaces corrected; Cross-reference checked; Old reference removed; TCEditor_v31 used | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | Supervision of signalling related speed restriction as soon as it received on-board. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment supervises the train movements against a signal related speed limit. Service brake is applied when supervised speed is higher than the permitted signal related speed limit. Service brake is released when speed is lower or equal to the permitted signal related speed limit. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.3 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. Permitted speed, equal to the speed define by the signal is DISPLAYED (if in FS mode and in OS mode only on driver request). Service Brake intervention indication is DISPLAYED Check on the JRU that: Reception of packet 12(optional packet 80) is RECORDED. Application of Service Brake is RECORDED. Release of Service Brake is RECORDED. |
| Constraints | The actual speed of the train is lower or equal to the speed permitted by MRSP which is V_MAIN (1). Track description (SSP and Gradients) covers the full length of the received Level 1 MA. The speed restriction set by the signal must be the most restrictive one. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Linking | STORED | Linking information to the main signal balise group |
| Movement Authority | STORED | Actual movement authority V_MAIN (1) Actual most restrictive speed value |
| Gradient Profile | STORED | - |
| Mode Profile | STORED | Only if in OS/LS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (1) (In OS only on driver request) (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 12 is received. | I | BTM | V_MAIN(2) becomes actual most restrictive speed value If the applicable mode is OS/LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests permitted speed | I | DMI | If in OS the supervised speed must be requested by the driver. | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | If in OS | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | CSM - Normal Status In FS or OS mode check that the permitted speed is updated to V_MAIN (2) If in LS mode: V_TRAIN < V_MAIN(2). | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = <V_MAIN(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED+10>) | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Status requested: Intervention status V_TRAIN > V_MAIN(2) | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = <V_MAIN(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 11 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Indication of warning due to over-speeding DMI Object : ST01 | N/A | N/A | - |
| 13 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 15 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 16 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The speed is lower or equal to the minimum of speed permitted by the signalling related speed. | N/A | N/A | - |
| 17 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 18 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Normal status If in FS or OS mode indication of permitted speed: V_MAIN (2) | N/A | N/A | - |
| 19 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = <V_MAIN(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Actual Most restrictive speed value |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | Actual movement authority V_MAIN (2) Actual most restrictive speed value |
| Gradient Profile | STORED | - |
| Mode Profile | STORED | Only if in OS/LS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAIN (2). Normal Status. In FS and OS mode V_PERMITTED = V_MAIN (2) In LS mode the permitted speed is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | Supervision of signalling related speed restriction when received from an infill balise group. In case of infill information the speed restriction shall be taken into account from the location reference of the balise group at the next main signal. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS / LS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment does not reduce the permitted speed near the main signal, because a new Infill MA is received. Due to the Infill information no Service brake is applied when reaching the vicinity of the main signal without a reduced speed. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.8.5.1 |  |
|  |  | Subset-026-3.8.5.1 b) |  |
|  |  | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.3 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. Permitted speed (V_MAIN) is DISPLAYED.(Only in FS mode) No Service Brake intervention indication is DISPLAYED. Check on the JRU that: An infill balise telegram, including Packet Number 136 and 12 is RECORDED. No Service Brake intervention is RECORDED. The actual speed (V_MAIN) is RECORDED. The telegram of the main signal balise group is RECORDED. |
| Constraints | The actual speed of the train is equal to the speed permitted by MRSP (V_MAIN). The speed restriction set by the signal must be the most restrictive one. The train must be equipped to receive Infill Information. It is assumed that Infill balises are linked and linking information to the main signal balise group and to the Infill balises are already stored on-board. V_MAIN already stored on-board must have the same value as the V_MAIN send in the eurobalise telegram including Packet Number 136 and 12. This definition is needed to make sure that there is no brake application due to any other reason than Infill (e.g. braking to a lower permitted speed). Infill Information that belongs to a main signal balise group already passed is rejected. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Linking information to the main signal balise group and to the Infill balises is already stored. |
| Movement Authority | STORED | Actual Movement Authority with EoA at the next main signal. V_MAIN = Actual most restrictive speed value in front of the main signal. V_EMA = 0 EoA exists at the next main signal. N_ITER=0 The train is located in the end section Q_DANGERPOINT=0 There is no danger point and release speed at the EoA Q_OVERLAP=0 No overlap is defined |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The train is located in the end section of the actual MA that defines an EoA with a stop at the next main signal. | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 136 and packet 12 is received. | I | BTM | This V_MAIN defines the most restrictive speed after the main signal. At the moment the train receives this value it recalculates the MRSP without a stop at the main signal. V_MAIN in front of and after the main signal are now the same. The non-appearance of a stop or a speed limit reduction can be tested. If the applicable mode is LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | The train reaches the vicinity of the main signal. | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS MA updated. The permitted speed near the signal remains the value V_MAIN. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS EoA location after main signal is updated. | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving. | I | INT | The driver approaches the signal with the permitted speed V_MAIN. | N/A | N/A | - |
| 8 | N/A | N/A | NOT Service brake commanded | O | TIU | In case of a Service Brake intervention the test fails, because the train did not handle the Infill information in the right way. | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS The actual speed (V_MAIN) is displayed. | N/A | N/A | - |
| 10 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS Speed and distance monitoring information has not changed | N/A | N/A | - |
| 11 | N/A | N/A | Balise Group message containing packet 12 is received. | I | BTM | The train passes the balise group at the main signal. This V_MAIN defines the most restrictive speed after the main signal. If the applicable mode is LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 12 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Optional step. Applies only if the applicable mode is FS Status requested: Normal Status. Indication of permitted speed: V_MAIN | N/A | N/A | - |
| 14 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS Speed and distance monitoring information has not changed | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. V_MAIN in front of and after the main signal are now the same. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile (If the applicable mode is LS) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | LS |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 11: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. Whether the V_MAIN variable in the starting conditions defines the most restrictive speed in front of the main signal. At the moment the train receives this value it recalculates the MRSP without a stop at the main signal. V_MAIN in front of and after the main signal are now the same. The non-appearance of a stop or a speed limit reduction can be tested. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | Actual Movement Authority behind the main signal. V_MAIN Actual most restrictive speed value. Same value as in the starting conditions. |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAIN. Normal Status V_PERMITTED = V_MAIN (in LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | Supervision of signalling related speed restriction when received from an infill Balise groups used as EOLMs. In case of infill information the speed restriction shall be taken into account from the location reference of the balise group at the next main signal. |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS / LS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment does not reduce the permitted speed near the main signal, because a new Infill MA from Euroloop is received. Due to the Infill information no Service brake is applied when reaching the vicinity of the main signal without a reduced speed. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.9.2.6 |  |
|  |  | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.3 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.30 Packet Number 134: EOLM Packet |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. Permitted speed (V_MAIN) is DISPLAYED.(only in FS mode) No Service Brake intervention indication is DISPLAYED. Check on the JRU that: Euroloop message, including Packet Number 136 and 12 is RECORDED. No Service Brake intervention is RECORDED. The actual speed (V_MAIN) is RECORDED. The telegram of the main signal balise group is RECORDED. |
| Constraints | The actual speed of the train is equal to the speed permitted by MRSP (V_MAIN). The speed restriction set by the signal must be the most restrictive one. The train must be equipped to receive Infill Information. Packet Number 134: EOLM Packet was received correctly. Linking information to the main signal balise group is already stored on-board. V_MAIN already stored on-board must have the same value as the V_MAIN send in the euroloop message including Packet Number 136 and 12. This definition is needed to make sure that there is no brake application due to any other reason than Infill (e.g. braking to a lower permitted speed). Infill Information that belongs to a main signal balise group already passed is rejected. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Linking information to the main signal balise group is already stored. |
| Movement Authority | STORED | Actual Movement Authority with EoA at the next main signal. V_MAIN= Actual most restrictive speed value in front of the main signal. V_EMA=0 EoA exists at the next main signal. N_ITER=0 The train is located in the end section Q_DANGERPOINT=0 There is no danger point and release speed at the EoA Q_OVERLAP=0 No overlap is defined |
| EOLM information | STORED | Information about the loop area and Identity |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The train is located in the end section of the actual MA that defines an EoA with a stop at the next main signal. | N/A | N/A | - |
| 2 | N/A | N/A | One Euroloop message (NID_C=<comment>) containing packet 136 and packet 12 (Q_DIR=1; V_RELEASEDP=3) is received | I | LTM | This V_MAIN defines the most restrictive speed after the main signal. V_MAIN in front of and after the main signal are now the same. If the applicable mode is LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | The train reaches the vicinity of the main signal. | Na | Na | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS MA updated. The permitted speed near the signal remains the value V_MAIN. | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS. EoA location after main signal are updated | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving. | I | INT | The driver approaches the signal with the permitted speed V_MAIN. | N/A | N/A | - |
| 8 | N/A | N/A | NOT Service brake commanded | O | TIU | In case of a Service Brake intervention the test fails, because the train did not handle the Infill information in the right way. | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS Status requested: Normal Status. The actual speed (V_MAIN) is displayed. | N/A | N/A | - |
| 10 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Speed and distance monitoring information has not changed | N/A | N/A | - |
| 11 | N/A | N/A | Balise Group message containing packet 12 is received. | I | BTM | The train passes the balise group at the main signal. This V_MAIN defines the most restrictive speed after the main signal. If the applicable mode is LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 12 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Optional step. Applies only if the applicable mode is FS Status requested: Normal Status. Indication of permitted speed: V_MAIN | N/A | N/A | - |
| 14 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS Speed and distance monitoring information has not changed | N/A | N/A | - |

| Step 2: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Up-link message (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region. |
| NID_LOOP | 14 | FINITE VALUE | Same value as in Packet 134 Identity of euroloop. Must match with NID_LOOP in Packet 134 already stored on-board. |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal. Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. Whether the V_MAIN variable in the starting conditions defines the most restrictive speed in front of the main signal. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile (If the applicable mode is LS) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | LS |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 11: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. Whether the V_MAIN variable in the starting conditions defines the most restrictive speed in front of the main signal. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile (If the applicable mode is LS) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | - |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | Actual Movement Authority behind the main signal. V_MAIN =Actual most restrictive speed value. Same value as in the starting conditions. |
| EOLM information | STORED | Information about the loop area and identity |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAIN. Normal Status V_PERMITTED = V_MAIN (in LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | Supervision of signalling related speed restriction when received from radio infill equipment (RIU). In case of infill information the speed restriction shall be taken into account from the location reference of the balise group at the next main signal. |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS / LS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment informs the RIU radio infill unit with Message 153 as soon as the location indicated in the order sent via balise groups is passed (i.e. entry of the train in the infill area) The ERTMS/ETCS on-board equipment does not reduce the permitted speed near the main signal, because a new Infill MA is received. Due to the Infill information no Service brake is applied when reaching the vicinity of the main signal without a reduced speed. The ERTMS/ETCS on-board equipment informs the RIU radio infill unit with Message 153 as soon as the next main signal balise group indicated in the order sent via balise groups is read. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 a) |  |
|  |  | Subset-026-3.9.3.10 b) |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 a) |  |
|  |  | Subset-026-3.9.3.11 b) |  |
|  |  | Subset-026-3.9.3.11 c) |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-7.4.2.29 Packet Number 133: Radio infill area information |  |
|  |  | Subset-026-8.7.15 Message 37: Infill MA |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. Permitted speed (V_MAIN) is DISPLAYED. (only in FS mode) No Service Brake intervention indication is DISPLAYED. Check on the JRU that: The Message 153 is RECORDED. The Messages 37 are RECORDED. The telegram of the main signal balise group is RECORDED. The Message 153 is RECORDED. No Service Brake intervention is RECORDED. The actual speed (V_MAIN) is RECORDED. |
| Constraints | The actual speed of the train is equal to the speed permitted by MRSP (V_MAIN). The speed restriction set by the signal must be the most restrictive one. The train must be equipped to receive Radio Infill Information. Linking information to the main signal balise group are already stored on-board (Packet 5). V_MAIN already stored on-board must have the same value as the V_MAIN send in step 6. Message 37: Infill MA. This definition is needed to make sure that there is no brake application due to any other reason than Infill (e.g. braking to a lower permitted speed). Infill Information that belongs to a main signal balise group already passed is rejected. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | with RIU |
| Linking | STORED | Linking information to the main signal balise group is already stored. |
| Movement Authority | STORED | Actual Movement Authority with EoA at the next main signal. V_MAIN=Actual most restrictive speed value in front of the main signal. V_EMA=0 EoA exists at the next main signal. Q_DANGERPOINT=0 There is no danger point and release speed at the EoA Q_OVERLAP=0 No overlap is defined |
| Radio Infill Area information | STORED | Packet 133 with Q_RIU = 1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The train reaches the infill area. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | Q_INFILL = 0 | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 (Q_DIR=1; V_MAIN=0) is received. | I | RTM | V_MAIN is set to 0 = trip order as long as the next main signal indicates an stop order The train receives cyclically Infill information Message 37: Infill MA with trip order at main signal. If the applicable mode is LS packet 80 should be included in the telegram. Other optional packet as 21/27 can be included. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 (Q_DIR=1; V_MAIN>0) is received. | I | RTM | V_MAIN ≥0 The train receives cyclically Infill information Message 37: Infill MA with a new MA beginning at the next main signal. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving. | I | INT | The train recalculates the MRSP, because the Infill MA revokes the stop command at the next main signal. The train reaches the vicinity of the main signal. | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS MA updated. The permitted speed near the signal remains the value V_MAIN. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS EoA location after main signal is updated | N/A | N/A | - |
| 11 | N/A | N/A | Train is moving. | I | INT | The driver approaches the signal with the permitted speed V_MAIN. | N/A | N/A | - |
| 12 | N/A | N/A | NOT Service brake commanded | O | TIU | In case of a Service Brake intervention the test fails, because the train did not handle the Infill information in the right way. | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS Status requested: Normal Status. The actual speed (V_MAIN) | N/A | N/A | - |
| 14 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS Speed and distance monitoring information has not changed | N/A | N/A | - |
| 15 | N/A | N/A | Balise Group message containing packet 133 is received. | I | BTM | The train passes the balise group at the main signal. Q_RIU = 0 | N/A | N/A | - |
| 16 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA. Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | Q_INFILL = 1 | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio-infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID (Identity of the train.) |
| NID_C | 10 | FINITE VALUE | Identifies the balise group at the next main signal. |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 0 | The train enters the infill area. |
| NID_PACKET | 8 | 0 | Position report (Position of train must be in the infill area). |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 4: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | V_MAIN is set to 0 = trip order as long as the next main signal indicates a stop order. |
| V_EMA | 7 | 0 | The next main signal gives the order to stop, so there is no LoA. |
| T_EMA | 10 | 0 | The next main signal gives the order to stop, so there is no LoA. |
| N_ITER | 5 | 0 | The next main signal gives the order to stop, so there are no sections. |
| L_SECTION(k) | 15 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| L_ENDSECTION | 15 | 0 | The next main signal gives the order to stop, so there are no sections. |
| Q_SECTIONTIMER | 1 | 0 | The next main signal gives the order to stop, so there are no sections. |
| T_SECTIONTIMER | 10 | FINITE VALUE | Not transmitted |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | Not transmitted |
| Q_ENDTIMER | 1 | 0 | The next main signal gives the order to stop, so there is no need for an end timer. |
| T_ENDTIMER | 10 | FINITE VALUE | Not transmitted |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | Not transmitted |
| Q_DANGERPOINT | 1 | 0 | The next main signal gives the order to stop, so there is no release speed. |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | 0 | The next main signal gives the order to stop, so there is no overlap. |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 6: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. Whether the V_MAIN variable in the starting conditions defines the most restrictive speed in front of the main signal. At the moment the train receives this value it recalculates the MRSP without a stop at the main signal. V_MAIN in front of and after the main signal are the same. The non-appearance of a stop or a speed limit reduction can be tested. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 15: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 133 | - |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_RIU | 1 | 0 | Order to terminate communication session. |
| NID_C | 10 | FINITE VALUE | Defines the radio Infill unit the train has connected. |
| NID_RIU | 14 | FINITE VALUE | - |
| NID_RADIO | 64 | FINITE VALUE | Radio subscriber number. |
| D_INFILL | 15 | FINITE VALUE | Distance to the end of the Infill area. May be 0, immediate disconnection after the main signal. If >0, the train rejects all Infill information after the main signal balise group. |
| NID_C | 10 | FINITE VALUE | Identifies the balise group at the main signal just passed. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Actual Movement Authority behind the main signal. V_MAIN=Actual most restrictive speed value Same value as in the starting conditions |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAIN. Normal Status V_PERMITTED = V_MAIN (in LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | If the ERTMS/ETCS on-board equipment switches from level 1 to level 2/3, the signalling related speed restriction shall remain valid until a L2/3 MA is accepted by the ERTMS/ETCS on-board equipment. |  | 5 |
| Applicable Mode/Level Combinations | L2 : FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The on-board supervises a signal related speed limit while the on-board equipment switches from level 1 to 2/3 The on-board equipment accepts the L2/3 MA (Message 3 containing packet 15, 21 and 27) and the L1 signalling speed restriction is not valid anymore. On DMI, the permitted speed, V_EMA and End of Movement Authority location are updated. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.6.3.1 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level symbol "Level 2" is displayed. Updated Permitted speed, V_EMA and End of Movement Authority location is DISPLAYED (Only in FS and in OS only on driver request). Check on the JRU that: Transition to Level 2 is recorded. Reception of Message 3 from RBC is recorded. Driver action is recorded. Speed and distance monitoring information is recorded. |
| Constraints | The actual speed of the train is lower or equal to the speed permitted by MRSP. The speed restriction set by the signal must be the most restrictive one. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Actual Movement Authority and covers at least the first section of the new level area V_MAIN = Actual more restrictive speed value |
| Gradient Profile | STORED | Covers at least the first section of the new level 2 area |
| International SSP | STORED | Covers at least the first section of the new level 2 area |
| Level Transition Order | STORED | M_LEVELTR=3 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In OS mode only on driver request) (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS / OS | Train is moving. | I | INT | The train is in vicinity of the level transition-boarder. | L1 | FS / LS / OS | - |
| 2 | L1 | FS / LS / OS | The on-board equipment performs transition to Level 2 | - | - | USE_FT5100100.37 | L2 | FS / LS / OS | - |
| 3 | L2 | FS / LS / OS | Train is moving. | I | INT | The full train length is inside level 2 area. | L2 | FS / LS / OS | - |
| 4 | L2 | FS / LS / OS | The driver requests permitted speed | I | DMI | Optional step. Applies only if the applicable mode is OS If in OS the supervised speed must be requested by the driver. | L2 | FS / LS / OS | - |
| 5 | L2 | FS / LS / OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L2 | FS / LS / OS | - |
| 6 | L2 | FS / LS / OS | The speed and distance monitoring information is displayed. | O | DMI | Status requested. Normal Status. If in FS or OS mode check that the permitted speed is still V_MAIN from level 1 If in LS mode: V_TRAIN < V_MAIN | L2 | FS / LS / OS | - |
| 7 | L2 | FS / LS / OS | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | The L1 MA is still in use, speed and distance monitoring information has not changed. | L2 | FS / LS / OS | - |
| 8 | L2 | FS / LS / OS | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21and packet 27 is received. | I | RTM | Level 2 MA is received with V_EMA less restrictive that V_MAIN of Level 1 MA. | L2 | FS / LS / OS | - |
| 9 | L2 | FS / LS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 | FS / LS / OS | - |
| 10 | L2 | FS / LS / OS | The speed and distance monitoring information is displayed. | O | DMI | L1 signalling speed restriction is not valid anymore. If in FS or OS mode the permitted speed, V_EMA and End of Movement Authority location are updated. If in LS mode the train overpass the L1 End of Movement Authority location to check that L1 signalling speed restriction has been updated. | L2 | FS / LS / OS | - |
| 11 | L2 | FS / LS / OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | End of Movement Authority location is updated. | L2 | FS / LS / OS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / LS / OS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | Level 2/3 Movement Authority New End of MA Location |
| Gradient Profile | STORED | Updated |
| International SSP | STORED | Updated |
| Level Transition Order | STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | New End of MA location (In LS mode the End of MA location is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | The on-board equipment receives an infill balise message with the signalling related speed to zero. The train trip information is ignored. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS / LS |  |  |
| Target of Test | The target of this test is to check that: The on-board ignores the train trip information coming from an infill balise. Telegram containing packet136 and packet 12 with V_MAIN set to 0 is received. Service brake is not applied and no transition to trip is performed. EoA Location is updated to the Main signal. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.5 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : At the balise passage the EoA Location is updated to the Main signal. Check on the TIU that: The emergency brakes are NOT APPLIED. |
| Constraints | The infill information also include an EOA at the next main signal. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Linking information to the main signal balise group and to the Infill balises is already stored. |
| Movement Authority | STORED | Actual movement authority V_MAIN = Actual most restrictive speed value |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only if in LS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 136 and packet 12 is received. | I | BTM | V_MAIN is set to 0 = trip order as long as the next main signal indicates an stop order | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS The EoA location is updated. | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS | N/A | N/A | - |
| 6 | N/A | N/A | NOT Service brake commanded | O | TIU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Same value as one of NID_BG in Packet 5. Identity of Balise. Must match with one of NID_BG in Packet 5 already stored on-board. |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1). Infill balises are linked. |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | Means “trip order” |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode Profile (If the applicable mode is LS) |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | LS |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | - |
| Movement Authority | STORED | The EoA location is update |
| Gradient Profile | STORED | - |
| Mode Profile | STORED | Only if in LS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT Service brake commanded | O | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | New EoA location (In LS mode the EoA location is not displayed) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | In case of a signal at danger the signalling related speed restriction shall have value zero, which shall be evaluated by the ERTMS/ETCS on-board equipment not as a speed limit but as a train trip order. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test is to check that: The on-board receives a balise group message with a signalling related speed restriction set to zero. The on-board equipment evaluates the signalling related speed restriction not as a speed limit but as a trip order. The train is tripped. The function “Supervise train movements against signalling related speed restriction when evaluated as a trip order” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.4 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Supervise signalling related speed restriction when evaluated as a trip order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The on-board receives a balise group message with a signal related speed to zero . Transition to Trip is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Linking | STORED | Linking information to the main signal balise group and to the Infill balises is already stored. |
| Movement Authority | STORED | Actual movement authority V_MAIN = Actual most restrictive speed value |
| Mode Profile | STORED | Only if in LS/OS mode |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In SR and OS only on driver request) (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / LS | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | L1 | FS / OS / SR / LS | - |
| 2 | L1 | FS / OS / SR / LS | The on-board commands train trip order. | - | - | USE_FT4060300.26: “Trip information received from balise and override is not active” V_MAIN is set to 0 = trip order as long as the next main signal indicates an stop order N_ITER = 0 | L1 | TR | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service Brake commanded | O | TIU | - |
| The Mode symbol "Trip" is displayed. The system status message "Unauthorized passing of EOA / LOA" is displayed The Status symbol "Service Brake or Emergency Brake intervention" is displayed. | O | DMI | DMI Object: Symbol MO05 Trip reason DMI Object : Symbol ST01 |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.6: Signalling related speed restrictions |  | 3110600 |
| Test Case of Feature | Supervision of signalling related speed restriction when received from radio infill equipment (RIU). In case of infill information the speed restriction shall be taken into account from the location reference of the balise group at the next main signal. The on-board equipment informs the RIU even if next main signal balise is missed. |  | 8 |
| Applicable Mode/Level Combinations | L1 : FS / LS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment informs the RIU radio infill unit with Message 153 as soon as the location indicated in the order sent via balise groups is passed (i.e. entry of the train in the infill area) The ERTMS/ETCS on-board equipment does not reduce the permitted speed near the main signal, because a new Infill MA is received. Due to the Infill information no Service brake is applied when reaching the vicinity of the main signal without a reduced speed. The ERTMS/ETCS on-board equipment informs the RIU radio infill unit with Message 153 as soon as the on-board equipment detects that the balise group at the main signal it is missed. Due to the linking information no reaction is performed by the on-board. The function “Determine Most Restrictive Speed Profile, based on Signalling related speed restriction” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.9.3.10 |  |
|  |  | Subset-026-3.9.3.10 a) |  |
|  |  | Subset-026-3.9.3.10 b) |  |
|  |  | Subset-026-3.9.3.11 |  |
|  |  | Subset-026-3.9.3.11 a) |  |
|  |  | Subset-026-3.9.3.11 b) |  |
|  |  | Subset-026-3.9.3.11 c) |  |
|  |  | Subset-026-3.9.3.11 d) |  |
|  |  | Subset-026-3.11.6.2 |  |
|  |  | Subset-026-3.11.6.3 |  |
|  |  | Subset-026-3.13.10.2.1 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - Signalling related speed restriction when evaluated as a speed limit |  |
|  |  | Subset-026-7.4.2.3 Packet Number 12: Level 1 Movement Authority |  |
|  |  | Subset-026-7.4.2.2 Packet Number 5: Linking |  |
|  |  | Subset-026-8.7.15 Message 37: Infill MA |  |
|  |  | Subset-026-8.6.11 Message 153: Radio infill request |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. Permitted speed (V_MAIN) is DISPLAYED. (only in FS mode) No Service Brake intervention indication is DISPLAYED. Check on the JRU that: The Message 153 is RECORDED. The Messages 37 are RECORDED. The telegram of the main signal balise group is NOT RECORDED. The Message 153 is RECORDED. No Service Brake intervention is RECORDED. The actual speed (V_MAIN) is RECORDED. |
| Constraints | The actual speed of the train is equal to the speed permitted by MRSP (V_MAIN). The speed restriction set by the signal must be the most restrictive one. The train must be equipped to receive Radio Infill Information. Linking information to the main signal balise group is already stored on-board with no set reaction. (Packet 5; Q_LINKREACTION=10). V_MAIN already stored on-board must have the same value as the V_MAIN send in step 6. Message 37: Infill MA. This definition is needed to make sure that there is no brake application due to any other reason than Infill (e.g. braking to a lower permitted speed). Infill Information that belongs to a main signal balise group already passed is rejected. It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | with RIU |
| Linking | STORED | Linking information to the main signal balise group is already stored Q_LINKREACTION=10 (no reaction) |
| Movement Authority | STORED | Actual Movement Authority with EoA at the next main signal. V_MAIN=Actual most restrictive speed value in front of the main signal. V_EMA=0 EoA exists at the next main signal. Q_DANGERPOINT=0 There is no danger point and release speed at the EoA Q_OVERLAP=0 No overlap is defined |
| Radio Infill Area information | STORED | Packet 133 with Q_RIU = 1 |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_PERMITTED = V_MAIN (In LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | V_MAIN |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | The train reaches the infill area. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | Q_INFILL = 0 | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 (Q_DIR=1; V_MAIN=0) is received. | I | RTM | V_MAIN is set to 0 = trip order as long as the next main signal indicates an stop order The train receives cyclically Infill information Message 37: Infill MA with trip order at main signal. If the applicable mode is LS packet 80 should be included in the telegram. Other optional packet as 21/27 can be included. | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 136 and packet 12 (Q_DIR=1; V_MAIN=0) is received. | I | RTM | V_MAIN ≥0 The train receives cyclically Infill information Message 37: Infill MA with a new MA beginning at the next main signal. If the applicable mode is LS packet 80 should be included in the telegram. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving. | I | INT | The train recalculates the MRSP, because the Infill MA revokes the stop command at the next main signal. The train reaches the vicinity of the main signal. | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS. Status requested Update MA is displayed The permitted speed near the signal remains the value V_MAIN. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Optional step. Applies only if the applicable mode is FS. EoA location after main signal are updated | N/A | N/A | - |
| 11 | N/A | N/A | Train is moving. | I | INT | The driver approaches the signal with the permitted speed V_MAIN. | N/A | N/A | - |
| 12 | N/A | N/A | NOT Service brake commanded. | O | TIU | In case of a Service Brake intervention the test fails, because the train did not handle the Infill information in the right way. | N/A | N/A | - |
| 13 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Optional step. Applies only if the applicable mode is FS. Status requested: Normal Status. The actual permitted speed (V_MAIN) is displayed. | N/A | N/A | - |
| 14 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Speed and distance monitoring information has not change | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA. Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted | O | RTM | The balise group at the main signal is missing Q_INFILL = 1 | N/A | N/A | - |
| 16 | N/A | N/A | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio-infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | ETCS-ID(Identity of the train.) |
| NID_C | 10 | FINITE VALUE | Identifies the balise group at the next main signal. |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 0 | The train enters the infill area. |
| NID_PACKET | 8 | 0 | Position report (Position of train must be in the infill area). |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 4: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | 0 | V_MAIN is set to 0 = trip order as long as the next main signal indicates a stop order. |
| V_EMA | 7 | 0 | The next main signal gives the order to stop, so there is no LoA. |
| T_EMA | 10 | 0 | The next main signal gives the order to stop, so there is no LoA. |
| N_ITER | 5 | 0 | The next main signal gives the order to stop, so there are no sections. |
| L_SECTION(k) | 15 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | Not transmitted because N_ITER = 0 |
| L_ENDSECTION | 15 | 0 | The next main signal gives the order to stop, so there are no sections. |
| Q_SECTIONTIMER | 1 | 0 | The next main signal gives the order to stop, so there are no sections. |
| T_SECTIONTIMER | 10 | FINITE VALUE | Not transmitted |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | Not transmitted |
| Q_ENDTIMER | 1 | 0 | The next main signal gives the order to stop, so there is no need for an end timer. |
| T_ENDTIMER | 10 | FINITE VALUE | Not transmitted |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | Not transmitted |
| Q_DANGERPOINT | 1 | 0 | The next main signal gives the order to stop, so there is no release speed. |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | 0 | The next main signal gives the order to stop, so there is no overlap. |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 6: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | Infill MA |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | Infill Location Reference |
| Q_DIR | 2 | FINITE VALUE | The packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | Identification of balise group at next main signal Can be verified by the train with the linking information already stored (Packet 5). |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | The Packet must be relevant for the train. |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | Same value as in the starting conditions. This V_MAIN defines the most restrictive speed after the main signal. Whether the V_MAIN variable in the starting conditions defines the most restrictive speed in front of the main signal. At the moment the train receives this value it recalculates the MRSP without a stop at the main signal. V_MAIN in front of and after the main signal are the same. The non-appearance of a stop or a speed limit reduction can be tested. |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| L_SECTION(k) | 15 | FINITE VALUE | In the case N_ITER>0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | In the case N_ITER=0: Value bigger than the braking distance of the train. To make sure there is no other cause to reduce the permitted speed at the main signal. |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |

| Step 15: Radio Message 153 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 153 | Radio-infill request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | Identity of the train. |
| NID_C | 10 | FINITE VALUE | Identifies the balise group at the main signal just missed |
| NID_BG | 14 | FINITE VALUE | Identity of the "target" main balise group |
| Q_INFILL | 1 | 1 | The train exits the Infill area. |
| NID_PACKET | 8 | 0 | Position report (Position of train must be behind the main signal). |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | ≠ NID_BG (main signal) |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | Actual Movement Authority behind the main signal. V_MAIN=Actual most restrictive speed value Same value as in the starting conditions |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAIN. Normal Status V_PERMITTED = V_MAIN (in LS mode the permitted speed is not displayed) |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |