| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180600 (5.18.6 Passing an “air tightness” area) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-5180600 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 02.12.2013 | All | First version/review | MULTITEL |
| Version 2.9.1 13.01.2014 | All | Apply of issues from Review1.0_Subset-076-5-2-5180600 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.6 Passing an “air tightness” area |  | 5180600 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of an air tightness section where is "Closing air conditioning intake" required. The on-board equipment displays to the driver a profile indicating the location where "Closing air conditioning intake" is required and where "Opening air conditioning intake" is permitted. |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that: - The on-board equipment displays to the driver the “Close air conditioning intake announcement” information according to the received track condition information - The on-board equipment updates the “Close air conditioning intake announcement” information according to train location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.1.2.1.3 |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Air tightness control |  |
|  |  | Subset-026-5.18.6.2 |  |
|  |  | Subset-026-5.18.6.2.1 |  |
|  |  | Subset-026-5.18.6.2.2 |  |
|  |  | Subset-026-5.18.6.2.2 a) |  |
|  |  | Subset-026-5.18.6.2.2 b) |  |
|  |  | Subset-026-5.18.6.3 |  |
|  |  | Subset-026-5.18.6.3 a) |  |
|  |  | Subset-026-5.18.6.3 b) |  |
|  |  | Subset-026-5.18.6.4 |  |
|  |  | Subset-026-5.18.6.4 a) |  |
|  |  | Subset-026-5.18.6.4 b) |  |
|  |  | Subset-026-5.18.6.4.1 |  |
|  |  | Subset-026-5.18.6.4.1 a) |  |
|  |  | Subset-026-5.18.6.4.1 b) |  |
|  |  | Subset-026-5.18.6.5 |  |
|  |  | Subset-026-5.20.4.2 |  |
|  |  | Subset-026-5.20.4.3 |  |
|  |  | Subset-026-5.20.4.3 a) |  |
|  |  | Subset-026-5.20.4.3 b) |  |
|  |  | Subset-026-5.20.4.4 |  |
|  |  | Subset-026-5.20.4.5 |  |
|  |  | Subset-026-A.3.1 - Time between minimum safe rear end of the train leaving a track condition area and on-board deleting the applicable indication |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: - “Close air conditioning intake announcement” information is displayed to the driver - “Air conditioning intake closed” information is updated according to train location - “Open air conditioning intake” information is displayed to the driver Check on TIU that: The track condition information is transmitted according to the received profile Check on JRU that: The symbols for: “Close air conditioning intake announcement”, “Air conditioning intake closed”, “Open air conditioning intake” are recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the Air tightness section L_TRACKCOND=FINITE VALUE, Length of the Air tightness section M_TRACKCOND=0101 Air tightness |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Close air conditioning intake announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC19) or manually (the driver is requested to act) (symbol TC21) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the “air tightness” area, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1¦Bit64=1>) is recorded | O | JRU | TC19 (automatic execution) or TC21 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the air tightness section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the air tightness section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 6 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing | N/A | N/A | - |
| 7 | N/A | N/A | "Air conditioning intake closed" replaces “Close air conditioning intake announcement” (symbol TC19 or TC21 is replaced by symbol TC19). | O | DMI | As soon as the train reaches the start location of the air tightness section with its max safe front end ( = D_TRACKCOND ) If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1&Bit64=0>) is recorded | O | JRU | If the related functionality is executed (application dependent) manually; otherwise, the symbols are identical | N/A | N/A | - |
| 9 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the air tightness section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 10 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded | O | JRU | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased | N/A | N/A | - |
| 11 | N/A | N/A | "Air conditioning intake closed" is REMOVED | O | DMI | As soon as the train reaches the end location of the air tightness section with its min safe rear end. | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | “Open air conditioning intake” is DISPLAYED It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC20) or manually (the driver is requested to act) (symbol TC22) | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=1¦Bit65=1) is recorded | O | JRU | TC20 (automatic execution) or TC22 (driver requested to act) | N/A | N/A | - |
| 16 | N/A | N/A | “Open air conditioning intake” is REMOVED | O | DMI | The “Open Air Conditioning intake” information shall remain displayed for a fixed time (5s, see Appendix A3.1) after the minimum safe rear end of the train has passed the end of the “Air tightness area”. | N/A | N/A | - |
| 17 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit63=0&Bit65=0) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.6 Passing an “air tightness” area |  | 5180600 |
| Test Case of Feature | The on-board equipment has received track condition information indicating the location of an air tightness section where is "Closing air conditioning intake" required. The on-board equipment displays to the driver a profile indicating the location where "Closing air conditioning intake" is required and where "Opening air conditioning intake" is permitted. Since the deletion of the air tightness section is requested by trackside, the ERTMS/ETCS on-board equipment stop indicating those locations and stop providing information related to this air tightness section to the ERTMS/ETCS external function. |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS, NL, TR, PT L2 : FS, LS, OS, NL, TR, PT L3 : FS, LS, OS, NL, TR, PT |  |  |
| Target of Test | The target of this test is to check that: - The on-board equipment displays to the driver the “Close air conditioning intake announcement” information according to the received track condition information - The on-board equipment updates the “Close air conditioning intake announcement” information according to train location. - After the trackside request to the deletion of the air tightness section, the ERTMS/ETCS on-board equipment stop providing information related to this section to the ERTMS/ETCS external function |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.4.4 |  |
|  |  | Subset-026-3.6.4.4 b) |  |
|  |  | Subset-026-3.7.3.2 |  |
|  |  | Subset-026-3.7.3.2 c) |  |
|  |  | Subset-026-3.12.1.5 |  |
|  |  | Subset-026-3.12.1.5 a) |  |
|  |  | Subset-026-3.12.1.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage Track Conditions excluding Sound Horn, Non Stopping areas, Tunnel Stopping areas and Big Metal Masses |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Air tightness control |  |
|  |  | Subset-026-5.18.6.2 |  |
|  |  | Subset-026-5.18.6.2.1 |  |
|  |  | Subset-026-5.18.6.2.2 |  |
|  |  | Subset-026-5.18.6.2.2 a) |  |
|  |  | Subset-026-5.18.6.2.2 b) |  |
|  |  | Subset-026-5.20.1.2 |  |
|  |  | Subset-026-5.20.4.2 |  |
|  |  | Subset-026-5.20.4.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: - “Close air conditioning intake announcement” information is displayed to the driver Check on TIU that: The track condition information is transmitted according to the received profile The track condition information is no more transmitted after the trackside request the deletion of the section Check on JRU that: The symbols for: “Close air conditioning intake announcement” is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| Track conditions excluding big metal masses | STORED | D_TRACKCOND=FINITE VALUE, Location of the Air tightness section L_TRACKCOND=FINITE VALUE, Length of the Air tightness secti M_TRACKCOND=0101 Air tightness |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | “Close air conditioning intake announcement” is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) automatically (symbol TC19) or manually (the driver is requested to act) (symbol TC21) | O | DMI | When the max safe front end of the train reaches a location in rear of the beginning of the “air tightness” area, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. | N/A | N/A | - |
| 2 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit62=1¦Bit64=1>) is recorded | O | JRU | TC19 (automatic execution) or TC21 (driver requested to act) | N/A | N/A | - |
| 3 | N/A | N/A | Track Conditions transmitted | O | TIU | The information related to the air tightness section is provided to the ERTMS/ETCS external function | N/A | N/A | - |
| 4 | N/A | N/A | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=2, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message containing packet 68 (Q_TRACKINIT = 1; D_TRACKINIT=0) is received | O | BTM | The information related to the air tightness section is deleted | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT Track Conditions transmitted | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded | O | JRU | - | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 68 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | 0 | Only if Q_TRACKINIT = 1 |
| D_TRACKCOND | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_TRACKCOND and the following variables follow |
| L_TRACKCOND | 15 | FINITE VALUE | - |
| M_TRACKCOND | 4 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_TRACKCOND(k) | 15 | FINITE VALUE | - |
| L_TRACKCOND(k) | 15 | FINITE VALUE | - |
| M_TRACKCOND(k) | 4 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 8 / 11 / 12 | FS / OS / TR / PT / NL / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |