| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5190300 (5.19.3: Limited Supervision is requested for a further location) Total: 5 Test case(s) |  |
| REF: | Subset-076-5-2-5190300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | ALL | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 03.10.2013 | All | Modifications requested by issues 13-14 of Review002_Subset076-5-2-5190300_v291 | MULTITEL |
| Version 2.9.3 05.11.2013 | All | Response to Review002_Subset-076-5-2-5190300_v291 | MULTITEL |
| Version 2.9.4 04.12.2013 | TC2 | Apply issue 16 of Review003_Subset-076-5-2-5190300_v291 | MULTITEL |
| Version 2.9.5 09.01.2014 | TC2 | Apply issue 1 of Review001_Subset-076-5-2-5190300_v294 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.3: Limited Supervision is requested for a further location |  | 5190300 |
| Test Case of Feature | Limited Supervision is requested for a further location when running in FS. An acknowledgement request for LS mode is displayed to the driver. The driver acknowledges LS mode. Mode is changed to Limited Supervision |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that: After receiving a Limited Supervision request for a further location a request for acknowledgement is DISPLAYED to the driver and that the beginning of the Limited Supervision area shall be considered as both the EOA and SvL (instead of the EOA/LOA and SvL derived from the MA) with no release speed, until the driver acknowledges When the driver acknowledges, the ERTMS/ETCS on-board equipment shall immediately switch to LS mode. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.6.2 row 6 column 5 |  |
|  |  | Subset-026-4.6.3 Condition Id [70] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision Mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.3.2 |  |
|  |  | Subset-026-5.19.3.2 a) |  |
|  |  | Subset-026-5.19.3.2 b) |  |
|  |  | Subset-026-5.19.3.2 c) |  |
|  |  | Subset-026-5.19.3.5 |  |
|  |  | Subset-026-5.19.3.5 b) |  |
|  |  | Subset-026-5.19.3.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Request for acknowledgement is DISPLAYED to the driver The mode initially DISPLAYED on the DMI changes to LS when the driver acknowledges the LS mode Check on the JRU that: The Driver Acknowledgement is RECORDED The transition to LS mode is RECORDED |
| Constraints | The request for acknowledgement shall be displayed to the driver, when the distance between the estimated front end of the train and the beginning of Limited Supervision area is shorter than a value, contained in the mode profile. The speed is lower than the LS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED | available from the front end of the train |
| International SSP | STORED | available from the front end of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L 1 / L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS, OS | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; Q_MAMODE=1) is received | I | BTM | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) D_MAMODE=<FINITE VALUE (<>0)>; L_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>; Q_MAMODE=1 <Begin of LS area to be considered as the SvL> | L1 / L2 / L3 | FS, OS | - |
| 2 | L1 / L2 / L3 | FS, OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The steps 1,2 or 3,4 will be chosen for testing | L1 / L2 / L3 | FS, OS | - |
| 3 | L1 / L2 / L3 | FS, OS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2; Q_MAMODE=1) is received | I | RTM | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) D_MAMODE=<FINITE VALUE (<>0)>; L_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>; Q_MAMODE=1 <Begin of LS area to be considered as the SvL> | L1 / L2 / L3 | FS, OS | - |
| 4 | L1 / L2 / L3 | FS, OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The steps 1,2 or 3,4 will be chosen for testing | L1 / L2 / L3 | FS, OS | - |
| 5 | L1 / L2 / L3 | FS, OS | Service brake commanded | O | TIU | Trains starts braking to LS area border as if it was both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed Otherwise EB will be applied (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS, OS | - |
| 6 | L1 / L2 / L3 | FS, OS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS, OS | - |
| 7 | L1 / L2 / L3 | FS, OS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | (Only if necessary for EOA supervision) | L1 / L2 / L3 | FS, OS | - |
| 8 | L1 / L2 / L3 | FS, OS | The target distance is updated | O | DMI | - | L1 / L2 / L3 | FS, OS | - |
| 9 | L1 / L2 / L3 | FS, OS | The target distance is updated is RECORDED | O | JRU | - | L1 / L2 / L3 | FS, OS | - |
| 10 | L1 / L2 / L3 | FS, OS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | Distance between estimated front end and beginning of Limited Supervision Area < L_ACKMAMODE (stored Packet 80) AND The speed is lower than the Limited Supervision mode permitted speed. V < V_NVLIMSUPERV | L1 / L2 / L3 | FS, OS | - |
| 11 | L1 / L2 / L3 | FS, OS | The driver acknowledges "Limited Supervision" | I | DMI | Before reaching the LS area | L1 / L2 / L3 | FS, OS | - |
| 12 | L1 / L2 / L3 | FS, OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1101) is recorded | O | JRU | - | L1 / L2 / L3 | FS, OS | - |
| 13 | L1 / L2 / L3 | FS, OS | The Mode symbol "Limited Supervision" is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded | O | JRU | - | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | Service brake not commanded | O | TIU | Only if it was applied for EOA supervision | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | Only if it was applied for EOA supervision | L1 / L2 / L3 | LS | - |
| 17 | L1 / L2 / L3 | LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Only if it was applied for EOA supervision | L1 / L2 / L3 | LS | - |
| 18 | L1 / L2 / L3 | LS | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet X is transmitted | O | RTM | Only in L2 / L3. The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | L1 / L2 / L3 | LS | - |
| 19 | L1 / L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2 / L3. Message 136: “Train Position report” is RECORDED. | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | Begin of LS area to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority. Should be sent also or previously stored. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | Begin of LS area to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 18: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | In L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.3: Limited Supervision is requested for a further location |  | 5190300 |
| Test Case of Feature | Limited Supervision is requested for a further location when running in FS or SR mode. An acknowledgement request for LS mode is displayed to the driver and the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, SR L2: FS, SR L3: FS, SR |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS a request for acknowledgement shall be displayed to the driver Until the switch to LS mode, the beginning of the LS area is considered as the EOA (keeping the SvL derived from the MA) with no Release Speed And the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode, And the service brake is triggered. The command shall be released as soon as the driver acknowledges (unless the command was triggered also for other reasons) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.6.2 row 6 column 5 |  |
|  |  | Subset-026-4.6.2 row 6 column 7 |  |
|  |  | Subset-026-4.6.3 Condition Id [72] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.3.2 |  |
|  |  | Subset-026-5.19.3.2 a) |  |
|  |  | Subset-026-5.19.3.2 b) |  |
|  |  | Subset-026-5.19.3.2 c) |  |
|  |  | Subset-026-5.19.3.5 |  |
|  |  | Subset-026-5.19.3.5 a) |  |
|  |  | Subset-026-5.19.3.7 |  |
|  |  | Subset-026-5.19.3.8 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The SB indication is DISPLAYED (for the supervision of the beginning of the LS area as an EOA) Request for acknowledgement is DISPLAYED to the driver The mode initially DISPLAYED on the DMI changes to LS when the max safe front end of the train reaches the beginning of the LS area The SB indication is REMOVED after the mode change The SB indication is DISPLAYED The SB indication is REMOVED after Drivers Acknowledgement Check on the JRU that: Application of Service Brake is RECORDED. Transition to LS mode is RECORDED Release of Service Brake is RECORDED Application of Service Brake is RECORDED. Driver Acknowledgement is RECORDED Release of Service Brake is RECORDED |
| Constraints | To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. The request for acknowledgement shall be displayed to the driver, when the distance between the estimated front end of the train and the beginning of Limited Supervision area is shorter than a value, contained in the mode profile. The speed is lower than the LS mode permitted speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 2 | FS / SR |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| ERTMS/ETCS level | 2 / 3 / 4 | L 1 / L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS, SR | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; Q_MAMODE=0) is received | I | BTM | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train Q_MAMODE=0 <Begin of LS area considered as EOA, SvL derived from the MA> | L1 / L2 / L3 | FS, SR | - |
| 2 | L1 / L2 / L3 | FS, SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | The steps 1,2 or 3,4 will be chosen for testing | L1 / L2 / L3 | FS, SR | - |
| 3 | L1 / L2 / L3 | FS, SR | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2; Q_MAMODE=0) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train Q_MAMODE=0 <Begin of LS area considered as EOA, SvL derived from the MA> | L1 / L2 / L3 | FS, SR | - |
| 4 | L1 / L2 / L3 | FS, SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | The steps 1,2 or 3,4 will be chosen for testing | L1 / L2 / L3 | FS, SR | - |
| 5 | L1 / L2 / L3 | FS, SR | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | Only if L_ACKMAMODE received in packet 80 is > 0 When the train reached the acknowledgement window a request for acknowledgement of LS is displayed immediately to the driver | L1 / L2 / L3 | FS, SR | - |
| 6 | L1 / L2 / L3 | FS, SR | Service brake commanded | O | TIU | The Service Brake is applied because of the EOA Until the switch to LS, the beginning of the LS area is considered as an EOA keeping the SvL given by the MA, with no release speed. The train is allowed to enter in the LS area by a distance corresponding to the length of the confidence interval | L1 / L2 / L3 | FS, SR | - |
| 7 | L1 / L2 / L3 | FS, SR | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | FS, SR | - |
| 8 | L1 / L2 / L3 | FS, SR | The Mode symbol "Limited Supervision" is displayed | O | DMI | The ERTMS/ETCS on-board equipment switches to LS mode when the max safe front end of the train reaches the LS area. | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Service brake not commanded | O | TIU | The Service Brake is released | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | Release of Service Brake is RECORDED. | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded | O | JRU | Mode transition to LS is RECORDED | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | A request for acknowledgement of LS is displayed | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet X is transmitted | O | RTM | Only in L2 / L3. The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | Only in L2 / L3. Message 136: “Train Position report” is RECORDED. | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | NOT The driver acknowledges "Limited Supervision" | I | DMI | the driver acknowledgement time (refer to Appendix A3.1) Driver doesn’t acknowledges within 5 sec after the change to LS | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | Service brake commanded | O | TIU | - | L1 / L2 / L3 | LS | - |
| 17 | L1 / L2 / L3 | LS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | SB indication is DISPLAYED | L1 / L2 / L3 | LS | - |
| 18 | L1 / L2 / L3 | LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | L1 / L2 / L3 | LS | - |
| 19 | L1 / L2 / L3 | LS | The driver acknowledges "Limited Supervision" | I | DMI | the driver acknowledgement time (refer to Appendix A3.1) Driver acknowledges more than 5 sec after the change to LS | L1 / L2 / L3 | LS | - |
| 20 | L1 / L2 / L3 | LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1101) is recorded | O | JRU | Driver Acknowledgement of transition to LS mode is RECORDED | L1 / L2 / L3 | LS | - |
| 21 | L1 / L2 / L3 | LS | Service brake not commanded | O | TIU | - | L1 / L2 / L3 | LS | - |
| 22 | L1 / L2 / L3 | LS | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | L1 / L2 / L3 | LS | - |
| 23 | L1 / L2 / L3 | LS | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | L1 / L2 / L3 | LS | - |
| 24 | L1 / L2 / L3 | LS | The Mode symbol "Limited Supervision" is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 0 | The beginning of the mode profile is considered as the EOA, SvL derived from the MA |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority. Should be sent also or previously stored. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 0 | The beginning of the mode profile is considered as the EOA, SvL derived from the MA |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 13: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | In L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.3: Limited Supervision is requested for a further location |  | 5190300 |
| Test Case of Feature | Limited Supervision is requested for a further location when running in FS mode. An acknowledgement request for LS mode is not displayed to the driver while the train speed is higher than the LS mode permitted speed. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that: After receiving the order to change to LS for a further location, the ERTMS/ETCS on-board equipment will not request an acknowledgement for changing to LS mode until the train speed is lower than the LS mode permitted speed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.6.2 row 6 column 5 |  |
|  |  | Subset-026-4.6.3 Condition Id [70] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.3.2 |  |
|  |  | Subset-026-5.19.3.2 a) |  |
|  |  | Subset-026-5.19.3.2 b) |  |
|  |  | Subset-026-5.19.3.2 c) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Request for acknowledgement is NOT DISPLAYED to the driver while the train speed is higher than the LS mode permitted speed Check on the JRU that: Transition to LS mode is RECORDED |
| Constraints | The speed of the train must be higher than the LS mode permitted speed, at least when the train enters in the area between D_MAMODE – L_ACKMAMODE and D_MAMODE. The brakes will probably be applied while the speed is higher than V_MAMODE. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED | available from the front end of the train |
| International SSP | STORED | available from the front end of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L 1 / L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS, OS | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS, OS | - |
| 2 | L1 / L2 / L3 | FS, OS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS, OS | - |
| 3 | L1 / L2 / L3 | FS, OS | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L1 / L2 / L3 | FS, OS | - |
| 4 | L1 / L2 / L3 | FS, OS | The train reaches the acknowledgement window and V> V_MAMODE. The request for acknowledgement is not displayed. | O | DMI | - | L1 / L2 / L3 | FS, OS | - |
| 5 | L1 / L2 / L3 | FS, OS | The train speed decreases to a value lower than V_MAMODE and the acknowledgement request is displayed. | O | DMI | - | L1 / L2 / L3 | FS, OS | - |
| 6 | L1 / L2 / L3 | FS, OS | The driver acknowledges the transition to LS mode. | I | DMI | - | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority. Should be sent also or previously stored. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 8: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | In L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.3: Limited Supervision is requested for a further location |  | 5190300 |
| Test Case of Feature | A Mode profile defining an LS area is available on-board. The train passes the beginning of the Limited Supervision area with its front end and the ERTMS/ETCS level switches to 1, 2 or 3. Then the on-board switches to LS mode. |  | 4 |
| Applicable Mode/Level Combinations | L0 : UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: on-board switches to LS mode as soon as the ETCS level switches to 1, 2 or 3 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.12.4.2 |  |
|  |  | Subset-026-4.6.2 row 6 column 11 |  |
|  |  | Subset-026-4.6.2 row 6 column 16 |  |
|  |  | Subset-026-4.6.3 Condition Id [71] |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.1.2 |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.3.7 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver acknowledges the LS mode The mode initially DISPLAYED on the DMI changes to LS Check on the JRU that: The acknowledgement of LS mode is recorded. The transition to LS mode is RECORDED |
| Constraints | There is a Mode Profile stored on-board For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. The train passes the beginning of the Limited Supervision area with its front end |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | Established | If entering in level 2/3 |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| Level Transition Order | STORED | Transition to levels L1, L2 or L3 |
| Mode Profile | STORED | Mode profile for LS (packet 80) is stored On-board, with M_MAMODE indicating LS (value 10) |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | If entering in level 2/3 |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | NOT The Mode symbol "Limited Supervision" is displayed | O | DMI | Mode transition was not valuated before the level change | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | The train level changes | - | - | Use_FT5100100.13,15,17,20,21,23,26 | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | The Level symbol "Level <L1 / L2 / L3>" is displayed | O | DMI | Mode transition was not valuated before the level change | L 1 / L 2 / L 3 | LS | - |
| 4 | L 1 / L 2 / L 3 | LS | Mode transition to LS is RECORDED | O | JRU | - | L 1 / L 2 / L 3 | LS | - |
| 5 | L 1 / L 2 / L 3 | LS | LS mode indication is DISPLAYED | O | DMI | - | L 1 / L 2 / L 3 | LS | - |
| 6 | L 1 / L 2 / L 3 | LS | A request for acknowledgement of LS mode is DISPLAYED to the driver | O | DMI | - | L 1 / L 2 / L 3 | LS | - |
| 7 | L 1 / L 2 / L 3 | LS | Driver acknowledges LS mode | I | DMI | - | L 1 / L 2 / L 3 | LS | - |
| 8 | L 1 / L 2 / L 3 | LS | Driver Acknowledgement of transition to LS mode is RECORDED | O | JRU | - | L 1 / L 2 / L 3 | LS | - |
| 9 | L 1 / L 2 / L 3 | LS | The on-board informs trackside about mode change Message 136: “Train Position Report” is SENT. | O | RTM | Only in L2 / L3. | L 1 / L 2 / L 3 | LS | - |
| 10 | L 1 / L 2 / L 3 | LS | Message 136: “Train position report” is RECORDED. | O | JRU | Only in L2 / L3. | L 1 / L 2 / L 3 | LS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | if in level 2 or 3 |
| Gradient Profile | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED UNCHANGED | Shall be at least available from the FRONT END of the train |
| Mode Profile | STORED UNCHANGED | Mode profile is stored On-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L 1 / L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | if in level 2 or 3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.3: Limited Supervision is requested for a further location |  | 5190300 |
| Test Case of Feature | Limited Supervision is requested for a further location when running in FS or OS. An acknowledgement request for LS mode is displayed to the driver. The train leaves the "rectangle of acknowledgement" and the acknowledgement request is not taken back. The driver acknowledges LS mode. Mode is changed to LS. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, OS L2: FS, OS L3: FS, OS |  |  |
| Target of Test | The target of this test is to check that: After receiving a Limited Supervision request for a further location, once the acknowledgement request is displayed, it is not taken back if the train leaves the “rectangle of acknowledgement”. The driver acknowledges LS mode. Mode is changed to LS. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.19.3.2 |  |
|  |  | Subset-026-5.19.3.2 a) |  |
|  |  | Subset-026-5.19.3.2 b) |  |
|  |  | Subset-026-5.19.3.2 c) |  |
|  |  | Subset-026-5.19.3.4 |  |
|  |  | Subset-026-5.19.3.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The request for acknowledgement is displayed to the driver when the train enters in the rectangle of acknowledgement. The acknowledgement request is not taken back when the train exits the rectangle of acknowledgement (the train accelerates and it overpasses the LS mode speed limit). The driver acknowledges LS mode. Check on the JRU that: The display of the request for acknowledgement is recorded The driver acknowledgement is recorded The transition to LS mode is recorded |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 | FS / OS |
| Radio communication session | ESTABLISHED | In L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | In L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | FS / OS | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS / OS | - |
| 2 | L1 / L2 / L3 | FS / OS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS / OS | - |
| 3 | L1 / L2 / L3 | FS / OS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=12; NID_PACKET=80; M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is recorded | O | JRU | One of the steps 3 or 4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS / OS | - |
| 4 | L1 / L2 / L3 | FS / OS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3; NID_PACKET=15; NID_PACKET=80; M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is recorded | O | JRU | One of the steps 3 or 4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) | L1 / L2 / L3 | FS / OS | - |
| 5 | L1 / L2 / L3 | FS / OS | Train is moving (V_TRAIN=<lower than the LS mode speed limit>) | I | INT | - | L1 / L2 / L3 | FS / OS | - |
| 6 | L1 / L2 / L3 | FS / OS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | Distance between estimated front end and beginning of Limited Supervision Area smaller than L_ACKMAMODE | L1 / L2 / L3 | FS / OS | - |
| 7 | L1 / L2 / L3 | FS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS | - |
| 8 | L1 / L2 / L3 | FS / OS | Train is moving (V_TRAIN=<higher than the LS mode speed limit>) | I | INT | - | L1 / L2 / L3 | FS / OS | - |
| 9 | L1 / L2 / L3 | FS / OS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | The acknowledgement is not taken back if the train leaves the “rectangle of acknowledgement” | L1 / L2 / L3 | FS / OS | - |
| 10 | L1 / L2 / L3 | FS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS | - |
| 11 | L1 / L2 / L3 | FS / OS | Train is moving (V_TRAIN=<lower than the LS mode speed limit>) | I | INT | - | L1 / L2 / L3 | FS / OS | - |
| 12 | L1 / L2 / L3 | FS / OS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | - | L1 / L2 / L3 | FS / OS | - |
| 13 | L1 / L2 / L3 | FS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS | - |
| 14 | L1 / L2 / L3 | FS / OS | The driver acknowledges "Limited Supervision" | I | DMI | Before reaching the LS area | L1 / L2 / L3 | FS / OS | - |
| 15 | L1 / L2 / L3 | FS / OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=13) is recorded | O | JRU | - | L1 / L2 / L3 | FS / OS | - |
| 16 | L1 / L2 / L3 | FS / OS | Any message (NID_MESSAGE_JRU=ALL; M_MODE=12) is recorded | O | JRU | The onboard equipment switches to LS mode | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | Begin of LS area to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 movement authority. Should be sent also or previously stored. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 15 | 1 | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | 1 | Begin of LS area to be considered as the SvL |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | In L2 / L3 |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | In L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |