| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110400 (3.11.4: Axle Load Speed Profile) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3110400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 05.10.2012 | All | First version/ review for Baseline 3 | INECO |
| Version 2.9.1 31.03.2013 | All | Second version/ UNISIG remarks from 12/03/2013 included (M_VERSION variable corrected); TCEditor tool and Dictionary used. | INECO |
| Version 2.9.2 28.05.2013 | All | Third version / UNISIG review comments from 13-05-2013 (INV) and comments from ALS included; TC4 created new. | INECO |
| Version 2.9.3 01.08.2013 | All | Fourth version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.4 11.12.2013 | All | Fifth version/TC_Editorv31; Requirements related with table 4.8.3 and 4.8.4 added. | INECO |
| Version 2.9.5 16.12.2013 | All | Sixth version/ Additional modification to TC3 mentioned in review sheet 005; Requirement 4.4.19.1.4 related with LS mode added. | INECO |
| Version 2.9.6 26.12.2013 | All | Seventh version/ Internal version: Cross-reference checked; old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.4: Axle Load Speed Profile |  | 3110400 |
| Test Case of Feature | Supervision of line speed depending on axle load profile. The train axle load category is greater than, or equal to the received axle load profile. Speed restriction shall be considered due to axle load. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment supervises train movements against an axle load profile. When the train reach the distance D_AXLELOAD the speed restriction given by the Axle Load speed Profile (packet 51- V_AXLELOAD) is applied by the on-board equipment because the axle load category of the train ≥ M_AXLELOADCAT(n) The ASP function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - ASP |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: Axle load Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the train reaches the distance D_AXLELOAD, permitted speed equal to axle load speed is DISPLAYED. (if in FS mode) When the train reaches the distance D_AXLELOAD , permitted speed requested by the driver and equal to axle load speed is DISPLAYED (if in OS mode) For LS mode, when the train reaches the distance D_AXLELOAD , the DMI aspect coherent with Overspeed Status( train speed a bit over the V_AXLELOAD) is displayed. Check on JRU that: The correct speed information is recorded. |
| Constraints | The train axle load category is greater than, or equal to the received axle load profile. The actual speed of the train is lower or equal to the permitted speed by MRSP. The speed restriction set by the axle load profile shall be considered in the MRSP, V_AXLELOAD lower than any other speed to be taken into account as MRSP. For LS mode: The permitted speed shall be lower than both the LS mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Axle load speed profile | NOT STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 51 (Q_DIR=1; V_AXLELOAD; D_AXLELOAD;M_AXLELOADCAT) is received | I | BTM | Train axle load category ≥ M_AXLELOADCAT(n) V_AXLELOAD becomes MRSP | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_AXLELOAD | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | If in FS or OS mode Permitted Speed is updated with the value of V_AXLELOAD The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section V_TRAIN< V_AXLELOAD If in LS mode the train speed will be a bit over the V_AXLELOAD. The DMI aspect is coherent with Overspeed Status. V_TRAIN > V_AXLELOAD | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_AXLELOAD>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_AXLELOAD Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train (M_AXLELOADCAT(n)) |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m)) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Axle load speed profile | STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed monitoring information is displayed | O | DMI | If in FS and OS mode Permitted speed = V_AXLELOAD If in LS mode train speed is greater than V_AXLELOAD. (Overspeed Status) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.4: Axle Load Speed Profile |  | 3110400 |
| Test Case of Feature | The train axle load category is lower than the received axle load category. Speed restriction shall not be considered due to axle load. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: The Speed restriction given by the Axle Load speed Profile (packet 51- V_AXLELOAD) is not applied by the on-board equipment because the axle load category of the train < M_AXLELOADCAT(n) When train reaches the distance D_AXLELOAD, the train speed remains constant. The “ASP function” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.4.4 |  |
|  |  | Subset-026-4.5.2.1 table - ASP |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle load speed profile |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: Axle load Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the train reaches the distance D_AXLELOAD, Permitted speed displayed on DMI different from V_AXLELOAD (if in FS mode) When the train reaches the distance D_AXLELOAD , Permitted speed requested by the driver different from V_AXLELOAD is DISPLAYED (if in OS mode) For LS mode, when the train reaches the distance D_AXLELOAD , the DMI aspect coherent with Normal Status (the train speed will be over the V_AXLELOAD) is displayed . Check on JRU that: The correct speed information is recorded. |
| Constraints | The train axle load category is lower than the received axle load category. The actual speed of the train is lower or equal to the permitted speed by MRSP. The speed restriction set by the axle load profile shall be considered in the MRSP, V_AXLELOAD lower than any other speed to be taken into account as MRSP. For LS mode: The permitted speed shall be lower than both the LS mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Axle load speed profile | NOT STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 51 (Q_DIR=1; V_AXLELOAD; D_AXLELOAD;M_AXLELOADCAT) is received | I | BTM | Train axle load category < M_AXLELOADCAT(n) V_AXLELOAD shall not apply | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_AXLELOAD | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | The speed information is displayed. | O | DMI | If in FS or OS mode Permitted Speed is NOT updated with the value of V_AXLELOAD The DMI aspect is coherent with Normal Status. If in LS mode the train speed will be a bit over the V_AXLELOAD. The DMI aspect is coherent with Normal Status. | N/A | N/A | - |
| 7 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded. | O | JRU | No change on speed and distance information - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n)) |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Axle load speed profile | STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed monitoring information is displayed | O | DMI | If in FS and OS modes Permitted speed ≠ V_AXLELOAD If in LS mode train speed is greater than V_AXLELOAD. Normal Status |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.4: Axle Load Speed Profile |  | 3110400 |
| Test Case of Feature | The initial state for Axle Load Speed Profile shall be “No restriction due to axle load”. Empty profile is received. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS/ LS/ OS L2: FS/ LS/ OS L3: FS/ LS/ OS |  |  |
| Target of Test | The target of the test is to check that: The train runs inside a section with ASP at a speed restricted due to axle load train category.(V_AXLELOAD) The train receives a balise telegram containing packet 51 where the variable Q_TRACKINIT is set to 1: “Empty profile, initial state to be resumed”. When the train reaches the distance D_TRACKINIT, the permitted speed is updated but different from stored V_AXLELOAD. The “ASP function” shall be active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.4.5 |  |
|  |  | Subset-026-4.5.2.1 table - ASP |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load Speed Profile |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: Axle load Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the train reaches the distance D_TRACKINIT, Permitted speed related to the track description (different from stored V_AXLELOAD) is DISPLAYED. (if in FS mode) When the train reaches the distance D_TRACKINIT, Permitted speed requested by driver and related to the track description (different from stored V_AXLELOAD) is DISPLAYED. (if in OS mode) For LS mode, when the train reaches the distance D_AXLELOAD , the DMI aspect coherent with Normal Status( train speed a bit over the V_AXLELOAD) is displayed.Check on JRU that: The correct speed information is recorded. |
| Constraints | An Axle load speed profile stored and actually in use The train axle load category is higher than the received axle load category. The variable Q_TRACKINIT state shall be set to 1; “Empty profile, initial state to be resumed." For LS mode: The permitted speed shall be lower than both the LS mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Axle load speed profile | STORED | Q_TRACKINIT set to 0 |
| Level Transition Order | STORED | If TC tested in L2/L3 a level transition order to Level 1at a further location should be stored |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2/ 3 /4 | L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message containing packet 51 (Q_DIR=1; Q_TRACKINIT=1) is received | I | BTM | Empty Profile is sent. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Immediate level transition to L1from L2/ L3 is performed. (Border Balise Group) | - | - | Optional step: only if applicable Level is L2/ L3 use: USE_FT5100100.32 for L2 Or USE_FT5100100.28 for L3 | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_TRACKINIT | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 1011) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Speed restriction due to axle load no more in use. If in FS or OS mode ,.Permitted speed updated but ≠ from V_AXLELOAD. The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section . If in LS mode the train speed will be greater than V_AXLELOAD. The DMI aspect is coherent with Normal Status. | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM≠ V_AXLELOAD; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 51 | Axle Load Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 1 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| Axle load speed profile | NOT STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed monitoring information is displayed | O | DMI | If in FS and OS modes Permitted speed ≠V_AXLELOAD If in LS mode train speed is greater than V_AXLELOAD. (Normal Status) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.4: Axle Load Speed Profile |  | 3110400 |
| Test Case of Feature | Supervision of line speed depending on axle load profile in Level 2/3. The train axle load category is greater than, or equal to the received axle load profile. Speed restriction shall be considered due to axle load. |  | 4 |
| Applicable Mode/Level Combinations | L2: FS/ LS/ OS L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: The ERTMS/ETCS on-board equipment supervises train movements against an axle load profile. When train reaches the distance D_AXLELOAD the speed restriction given by the Axle Load speed Profile (packet 51- V_AXLELOAD) is applied by the on-board equipment because the axle load category of the train ≥ M_AXLELOADCAT(n) The “ASP function” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - ASP |  |
|  |  | Subset-026-4.8.3.1.1 Axle Load speed profile |  |
|  |  | Subset-026-4.8.4.2 Axle Load speed profile |  |
|  |  | Subset-026-7.4.2.13 Packet Number 51: Axle load Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: When the train reaches the distance D_AXLELOAD, permitted train speed equal to axle load speed is DISPLAYED. (if in FS mode) When the train reaches the distance D_AXLELOAD , permitted speed requested by the driver and equal to axle load speed is DISPLAYED (if in OS mode) For LS mode, when the train reaches the distance D_AXLELOAD , the DMI aspect coherent with Overspeed Status( train speed a bit over the V_AXLELOAD) is displayed. Check on JRU that: The correct speed information is recorded. |
| Constraints | The train axle load category is greater than the received axle load profile. The actual speed of the train is lower or equal to the speed permitted by MRSP. The speed restriction set by the axle load profile shall be considered in the MRSP, V_AXLELOAD lower than any other speed to be taken into account as MRSP. “Train data” information transmitted to RBC and ACK received from RBC. For LS mode: The permitted speed shall be lower than both the LS mode speed limit and the maximum train speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Axle load speed profile | NOT STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| Train Data | STORED | ACK received from RBC |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET- UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 51 (Q_DIR=1) is received | I | RTM | Train axle load category ≥ M_AXLELOADCAT(n) V_AXLELOAD becomes MRSP | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_AXLELOAD | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | If in FS or OS mode Permitted Speed is updated with the value of V_AXLELOAD. The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section V_TRAIN <= V_AXLELOAD If in LS mode the train speed will be a bit over the V_AXLELOAD. The DMI aspect is coherent with Overspeed Status. V_TRAIN > V_AXLELOAD | N/A | N/A | - |
| 7 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_AXLELOAD>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_AXLELOAD Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 51 | Axle Load Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | 0 | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1 |
| D_AXLELOAD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_AXLELOAD and the following variables follow. |
| L_AXLELOAD | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(n) | 7 | FINITE VALUE | - |
| V_AXLELOAD(n) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(n). |
| N_ITER | 5 | FINITE VALUE | - |
| D_AXLELOAD(k) | 15 | FINITE VALUE | - |
| L_AXLELOAD(k) | 15 | FINITE VALUE | - |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| M_AXLELOADCAT(k,m) | 7 | FINITE VALUE | - |
| V_AXLELOAD(k,m) | 7 | FINITE VALUE | Speed restriction to be applied if the axle load category of the train ( M_AXLELOADCAT(k,m). |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Axle load speed profile | STORED | - |
| Mode Profile | STORED | Only applicable for LS/OS modes |
| Train Data | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed monitoring information is displayed | O | DMI | If in FS and LS modes Permitted speed = V_AXLELOAD If in LS mode train speed is greater than V_AXLELOAD. (Overspeed Status) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |