| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110300 (3.11.3: Static Speed Profile (SSP)) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-3110300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 31.03.2013 | All | Creation | DLR |
| Version 2.9.0 31.03.2013 | All | Second version/review for Baseline 3 | INECO |
| Version 2.9.1 31.05.2013 | All | Third version/ UNISIG review comments from 01/03/2013 included. All TCs changed. | INECO |
| Version 2.9.2 01.08.2013 | All | Fourth version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.3 26.12.2013 | All | Fifth version/ Internal modification: Cross-reference checked; old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.3: Static Speed Profile (SSP) |  | 3110300 |
| Test Case of Feature | Supervision of static speed profile depending on train categories. |  | 1 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment supervises the train movements against the speed limit set by "Cant Deficiency“ SSP category. Balise message containing packet 27 is received. Service brake is applied when train speed is higher than the permitted speed defined by Cant Deficiency“ SSP category. Service brake is released when speed is lower or equal to the permitted speed defined by Cant Deficiency“ SSP category. The function “Determine Most Restrictive Speed profile, based on SSP” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.3.2.3 |  |
|  |  | Subset-026-3.11.3.2.3 a) |  |
|  |  | Subset-026-3.11.3.2.5 |  |
|  |  | Subset-026-3.11.3.2.6 |  |
|  |  | Subset-026-3.11.3.2.6 a) |  |
|  |  | Subset-026-3.11.3.3.2 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. In FS mode, the correct permitted speed equal to the speed define by Cant Deficiency Category has to be shown on the DMI/. In OS mode, the permitted speed define by Cant Deficiency Category is shown on driver request. In LS mode, the DMI aspect coherent with Overspeed Status (train speed greater than speed define by Cant Deficiency Category) is displayed. Check on the JRU that: Application of Service Brake is RECORDED. Release of Service Brake is RECORDED. Speed and distance monitoring information is RECORDED. |
| Constraints | Service Brake Indication is DISPLAYED and REMOVED. There are no temporary speed restrictions. One category of the train must match with one stored in packet 27: “Cant Deficiency” SSP (NC_CDDIFF must match with the train, NC_CDTRAIN). The speed restriction set by “Cant Deficiency” SSP category in packet 27 must be more restrictive than the generic restriction one. Packet 27 Shall contain other speed restrictions for other train categories, more and less restrictive that the one applicable to the “Cant Deficiency” category of the train. There shouldn’t exist any other speed restriction lower than the ones in packet 27 It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | VALID | Train knows its cant deficiency category (NC_CDTRAIN) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | In FS mode Permitted Speed is displayed In OS Permitted Speed Limit is only shown at driver request |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 27 is received. | I | BTM | 0<V_STATIC<121 N_ITER must be > 0 to define different speed depending on train category. Q_DIFF (n)=0 NC_CDDIFF(n) is the specific SSP category for which a different value of the static line speed exists. V_DIFF(n) value must be the most restrictive in the SSP to ensure that the restriction is not rejected by the train. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_STATIC. | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests permitted speed | I | DMI | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | If in FS or OS mode, permitted speed is the one defined by the train category in the SSP.(V_DIFF) V_TRAIN < V_DIFF The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section If in LS mode the DMI aspect is coherent with Overspeed status in a ceiling speed monitoring section. V_TRAIN > V_DIFF | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_DIFF Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED+10>) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Status requested: Intervention status | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V_DIFF> ; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 12 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Indication of warning due to over-speeding | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | Train is moving | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The speed is lower or equal to the minimum of speed permitted by the SSP. | N/A | N/A | - |
| 18 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode Indication of permitted speed: V_DIFF Status requested: Normal status If in LS mode the V_TRAIN < V_DIFF. The DMI aspect is coherent with Normal Status. | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | 0<V_STATIC<121 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | must be > 0 |
| Q_DIFF(n) | 2 | 0 | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | = NC_CDTRAIN |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | Must be the MRSP |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| International SSP | STORED | - |
| Train Data | VALID | NC_CDTRAIN |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN <V_DIFF. Normal Status In FS or OS mode indication of permitted speed V_DIFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.3: Static Speed Profile (SSP) |  | 3110300 |
| Test Case of Feature | Supervision of specific line speed depending on train categories. |  | 2 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: FS / LS/ OS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment supervises the train movements against the speed limit set by the cant deficiency SSP, when there is not a value of cant deficiency SSP that matches its cant deficiency train category. Balise message containing packet 27 is received. The on-board equipment selects the “Cant Deficiency” SSP with the highest Cant Deficiency value below the value of its “Cant Deficiency” train category Service brake is applied when train speed is higher than the permitted speed defined by the selected cant deficiency SSP. Service brake is released when speed is lower or equal to the permitted speed defined by the selected cant deficiency SSP. The function “Determine Most Restrictive Speed profile, based on SSP” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.3.2.3 |  |
|  |  | Subset-026-3.11.3.2.3 b) |  |
|  |  | Subset-026-3.11.3.2.5 |  |
|  |  | Subset-026-3.11.3.3.2 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. In FS mode, the correct permitted speed equal to the speed define by Cant Deficiency Category has to be shown on the DMI/ In OS mode, the permitted speed define by Cant Deficiency Category is shown on driver request. In LS mode, the DMI aspect coherent with Overspeed Status (train speed greater than speed define by Cant Deficiency Category) is displayed. Check on the JRU that: Application of Service Brake is RECORDED. Release of Service Brake is RECORDED. Speed and distance monitoring information is RECORDED. |
| Constraints | Service Brake Indication is DISPLAYED and REMOVED. There are no temporary speed restrictions. The train does not correspond with any of the cant deficiency SSP defined in packet 27, it will be supervised according the highest Cant Deficiency value below the value of its “Cant Deficiency” train category.(at least 3SSP profiles are sent with values of NC_CDDIFF equal to a, b and d; the relation between categories is a < b < c < d where “c” is NC_CDTRAIN) The speed restriction set by international train category in packet 27 must be more restrictive than the generic restriction one. Packet 27 Shall contain other speed restrictions for other train categories, more and less restrictive that the one applicable to the category of the train. There shouldn’t exist any other speed restriction lower than the ones in packet 27 It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | VALID | Train knows its cant deficiency category (NC_CDTRAIN= c) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | In FS Permitted Speed is displayed. In OS Permitted Speed Limit is only shown at driver request. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 27 is received. | I | BTM | 0<V_STATIC<121 N_ITER must be > 2 to define different speed depending on train category. Q_DIFF (n)=0 NC_CDDIFF(n=a, b, d) = finite values; Specific SSP category, different from NC_CDTRAIN (c) a<b<c<d | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_STATIC. The on-board equipment selects the “Cant Deficiency” SSP with the highest Cant Deficiency value below the value of its “Cant Deficiency” train category | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests permitted speed | I | DMI | Optional step. Applies only if the applicable mode is OS. | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS. | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: V_DIFF(2) If in LS mode the train speed will be a bit over the V_DIFF(2). The DMI aspect is coherent with Overspeed Status. V_TRAIN>V_DIFF(2) | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS: Check that V_PERM is updated with V_DIFF(2) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED+10>) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Status requested: Intervention status | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 12 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Indication of warning due to over-speeding | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | Train is moving | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The speed is lower or equal to the minimum of speed permitted by the SSP. | N/A | N/A | - |
| 18 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Normal status If in FS or OS mode indication of permitted speed: V_DIFF(2) If in LS mode the V_TRAIN < V_DIFF(2). The DMI aspect is coherent with Normal Status. | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(2)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | 0<V_STATIC<121 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | must be ≥ 3 |
| Q_DIFF(1) | 2 | 0 | - |
| NC_CDDIFF(1) | 4 | a | < NC_CDTRAIN (c) |
| NC_DIFF(1) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(1) | 7 | FINITE VALUE | - |
| Q_DIFF(2) | 2 | 0 | - |
| NC_CDDIFF(2) | 4 | b | < NC_CDTRAIN (c) |
| NC_DIFF(2) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(2) | 7 | FINITE VALUE | - |
| Q_DIFF(3) | 2 | 0 | - |
| NC_CDDIFF(3) | 4 | D | > NC_CDTRAIN (c) |
| NC_DIFF(3) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(3) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| International SSP | STORED | - |
| Train Data | VALID | Train knows its cant deficiency category |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_DIFF(2). Normal Status In FS or OS mode indication of permitted speed: V_DIFF(2) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.3: Static Speed Profile (SSP) |  | 3110300 |
| Test Case of Feature | Supervision of specific line speed depending on train categories. |  | 3 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment supervises the train movements when no specific SSPs related to its cant deficiency train categories are given by the trackside. Balise message containing packet 27 is received. When the train enters inside the SSP area, the permitted speed is updated to V_STATIC. A level transition to L2/L3 is performed. The ERTMS/ETCS on-board equipment supervises the train movements when no specific SSPs related to its cant deficiency train categories are given by the trackside. The function “Determine Most Restrictive Speed profile, based on SSP” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.1.2 |  |
|  |  | Subset-026-3.11.3.2.3 |  |
|  |  | Subset-026-3.11.3.2.3 c) |  |
|  |  | Subset-026-3.11.3.2.6 |  |
|  |  | Subset-026-3.11.3.2.6 a) |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that:. If in FS or OS mode, in level 1, the permitted speed, equal to the speed define by Basic SSP , is DISPLAYED. If in FS or OS mode, in level 2/3, the permitted speed, equal to the speed define by Basic SSP , is DISPLAYED. In LS mode, the DMI aspect coherent with Overspeed Status (Train speed greater than the speed define by Basic SSP) is displayed. Check on the JRU that: A level transition to L2/L3 is RECORDED. An updated of the permitted speed is recorded. Speed and distance monitoring information is RECORDED. |
| Constraints | Packet 27 shall contain speed restrictions for the Basic SSP and should cover full length of L2/L3 MA. There shouldn’t exist any other speed restriction lower than the ones in packet 27 It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | VALID | Train knows its category and its cant deficiency category (NC_TRAIN and NC_CDTRAIN) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | In FS Permitted Speed is displayed. In OS Permitted Speed Limit is shown only at driver request. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / LS | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | L1 | FS / OS / LS | - |
| 2 | L1 | FS / OS / LS | Balise Group message containing packet 27 is received. | I | BTM | 0<V_STATIC<121and More restrictive than V_DIFF Q_DIFF (n)=2 | L1 | FS / OS / LS | - |
| 3 | L1 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | FS / OS / LS | - |
| 4 | L1 | FS / OS / LS | Train is moving. | I | INT | Train runs the distance D_STATIC. | L1 | FS / OS / LS | - |
| 5 | L1 | FS / OS / LS | The driver requests permitted speed | I | DMI | Optional step. Applies only if the applicable mode is OS. The supervised speed must be requested by the driver. | L1 | FS / OS / LS | - |
| 6 | L1 | FS / OS / LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS | L1 | FS / OS / LS | - |
| 7 | L1 | FS / OS / LS | The speed and distance monitoring information is displayed. | O | DMI | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: (V_STATIC) If in LS mode the train speed will be a bit over the V_STATIC. The DMI aspect is coherent with Overspeed Status. V_TRAIN>V_STATIC. | L1 | FS / OS / LS | - |
| 8 | L1 | FS / OS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_STATIC>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in FS/OS: Check that V_PERM is updated with V_STATIC Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | L1 | FS / OS / LS | - |
| 9 | L1 | FS / OS / LS | The on-board equipment performs an immediate Level transition from L1 to L2/L3. | - | - | USE_ FT5100100.37 (from L1 to L2) Or USE_ FT5100100.36 (from L1 to L3) | L2/L3 | FS / OS / LS | - |
| 10 | L2/L3 | FS / OS / LS | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | L2/L3 | FS / OS / LS | - |
| 11 | L2/L3 | FS / OS / LS | The speed and distance monitoring information is displayed. | O | DMI | The actual train speed is equal or lower than the allowed speed in the SSP. The DMI aspect is coherent with Normal Status. If in FS or OS mode indication of permitted speed: V_STATIC No modification on permitted speed value occurred If in LS mode the train speed will be a bit over the V_STATIC. The DMI aspect is coherent with NORMAL Status. | L2/L3 | FS / OS / LS | - |
| 12 | L2/L3 | FS / OS / LS | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TRAIN> <V_STATIC>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded. | O | JRU | No change on speed and distance information after the level transition.- | L2/L3 | FS / OS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Telegram counter (M _MCOUNT) – 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | More restrictive than V_DIFF |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | must be > 0 |
| Q_DIFF(n) | 2 | 2 | Other specific category, does not replace that Cant Deficiency SSP |
| NC_CDDIFF(n) | 4 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | > V_STATIC |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| International SSP | STORED | - |
| Level Transition Order | STORED | - |
| Train Data | VALID | Train knows its category |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_STATIC. Normal Status In FS or OS mode indication of permitted speed: V_STATIC |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.3: Static Speed Profile (SSP) |  | 3110300 |
| Test Case of Feature | Supervision of static speed profile depending on train categories. |  | 4 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of test is to check that: The ERTMS/ETCS on-board equipment supervises the train movements against the speed limit set by „other specific“ SSP category. Balise message containing packet 27 is received. Service brake is applied when train speed is higher than the permitted speed defined by “other specific“ SSP category. Service brake is released when speed is lower or equal to the permitted speed defined by “other specific“ SSP category. The function “Determine Most Restrictive Speed profile, based on SSP” is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.3.2.5 |  |
|  |  | Subset-026-3.11.3.2.6 |  |
|  |  | Subset-026-3.11.3.2.6 b) |  |
|  |  | Subset-026-3.11.3.3.2 |  |
|  |  | Subset-026-4.4.9.1.5 |  |
|  |  | Subset-026-4.4.12.1.3 |  |
|  |  | Subset-026-4.4.19.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Current train speed is DISPLAYED. In FS mode, the correct permitted speed equal to the speed define by “Other specific” SSP category has to be shown on the DMI/ In OS mode, the permitted speed define by “Other specific” SSP category is shown on driver request. In LS mode, the DMI aspect coherent with Overspeed Status (train speed greater than the speed define by “Other specific” SSP category) is displayed. Check on the JRU that: Application of Service Brake is RECORDED. Release of Service Brake is RECORDED. Speed and distance monitoring information is RECORDED. |
| Constraints | Service Brake Indication is DISPLAYED and REMOVED. There are no temporary speed restrictions. One category of the train must match with one stored in packet 27: “other specific” SSP (NC_DIFF must match with the train, NC_TRAIN). The speed restriction set by “other specific” SSP category in packet 27 must be more restrictive than the generic restriction one. Packet 27 shall contain other speed restrictions for other train categories, more and less restrictive that the one applicable to the “other specific” category of the train. There shouldn’t exist any other speed restriction lower than the ones in packet 27 It is assumed, that the train uses the service brake for speed supervision. If there is no service brake or it doesn’t react, the emergency brake will be used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | VALID | Train knows its category. (NC_TRAIN) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | In FS mode Permitted Speed is displayed In OS Permitted Speed Limit is only shown at driver request |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message containing packet 27 is received. | I | BTM | 0 < V_STATIC < 121 N_ITER>0 to define different speed depending on train category. Q_DIFF(n)=1 NC_DIFF(n)= It is the specific SSP category for which a different value of the static line speed exists.. V_DIFF(n) must be the most restrictive in the SSP to ensure that the restriction is not rejected by the train. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving. | I | INT | Train runs the distance D_STATIC. | N/A | N/A | - |
| 5 | N/A | N/A | The driver requests permitted speed | I | DMI | Optional step. Applies only if the applicable mode is OS. The supervised speed must be requested by the driver. | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS. | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | If in FS or OS mode check that the permitted speed is the one defined by the train category in the SSP. (V_DIFF(n)) If in LS mode the train speed will be a bit over the V_DIFF(n). The DMI aspect is coherent with Overspeed Status. V_TRAIN > V_DIFF(n) | N/A | N/A | - |
| 8 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(n)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS: Check that V_PERM is updated with V_DIFF(n) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) | N/A | N/A | - |
| 9 | N/A | N/A | Train is moving. | I | INT | (V_TRAIN=<V_PERMITTED+10>) | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | Indication of permitted speed: V_DIFF(n) Status requested: Intervention status | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V_DIFF(n)> ; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 12 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 13 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | Indication of warning due to over-speeding | N/A | N/A | - |
| 14 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | Train is moving | I | INT | (V_TRAIN=<V_PERMITTED>) | N/A | N/A | - |
| 16 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 17 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | The speed is lower or equal to the minimum of speed permitted by the SSP. | N/A | N/A | - |
| 18 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Status requested: Normal status If in FS or OS mode indication of permitted speed: V_DIFF(n) If in LS mode the V_TRAIN < V_DIFF(n). The DMI aspect is coherent with Normal Status. | N/A | N/A | - |
| 20 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM =<V_DIFF(n)>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | 0<V_STATIC<121 |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | must be > 0 |
| Q_DIFF(n) | 2 | 1 | Other specific category, does replace the Cant Deficiency SSP |
| NC_CDDIFF(n) | 4 | FINITE VALUE | - |
| NC_DIFF(n) | 4 | FINITE VALUE | Must match NC_TRAIN |
| V_DIFF(n) | 7 | FINITE VALUE | Must be the MRSP |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| International SSP | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_DIFF(n). Normal Status In FS or OS mode indication of permitted speed: V_DIFF(n) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |