| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5170200 (5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure) Total: 12 Test case(s) |  |
| REF: | Subset-076-5-2-5170200 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 04.10.2013 | TC2, TC3, TC6, TC9 | Modifications requested by issues 22 to 23 of Review002_Subset-076-5-2-5170200v291 | MULTITEL |
| Version 2.9.3 26.11.2013 | All | Response to Review004_Subset-076-5-2-5170200_v291 | MULTITEL |
| Version 2.9.4 20.01.2014 | TC5, TC7, TC9, TC10, TC11 | Response to Review001_Subset-076-5-2-5170200_v293 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and different of Train category, Axle load, Loading gauge or Traction system is changed |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT L2: FS, LS, OS, SR, SB, TR, PT L3: FS, LS, OS, SR, SB, TR, PT |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and different from train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 b) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED. New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. Train data is different from Train category, Axle load, Loading gauge or Traction system. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data is different from Train category, Axle load, Loading gauge or Traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / TR / PT / LS / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | When the on-board equipment is in SB or PT mode, Train Data not requiring validation and concerning one of: Train category, Axle load, Loading gauge or Traction system is changed |  | 2 |
| Applicable Mode/Level Combinations | L1: SB, PT L2: SB, PT L3: SB, PT |  |  |
| Target of Test | The target of test is to check: When the on-board equipment is in SB, PT mode, the steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and concerning train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 b) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED. New Train Data is RECORDED. |
| Constraints | The operational mode is one of SB, PT. There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 | SB / PT |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 / 8 UNCHANGED | SB / PT |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is TR |  | 3 |
| Applicable Mode/Level Combinations | L0: TR LNTC: TR L1: TR L2: TR L3: TR |  |  |
| Target of Test | The target of test is to check that: The Train Data is recorded when affected by changes of the input information in the ERTMS/ETCS on-board external device not requiring validation and concerning one of train category, axle load, traction system or loading gauge when the on-board equipment is in TR mode. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 c) |  |
|  |  | Subset-026-5.17.2.2 D5 |  |
|  |  | Subset-026-5.17.2.2 D5 b) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED. New Train Data is RECORDED. |
| Constraints | The operational mode is TR. There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. MA and track description, received from RBC not stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Movement Authority | NOT STORED | No MA or Track description from RBC stored on-board |
| RBC Transition Order | NOT STORED | No MA or Track description from RBC stored on-board |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L0 / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 UNCHANGED | TR |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 UNCHANGED | L0 / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is FS, LS or OS and train is at standstill |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and concerning train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board when the on-board is in FS, LS or OS mode and train is at standstill. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 a) |  |
|  |  | Subset-026-5.17.2.2 D7 |  |
|  |  | Subset-026-5.17.2.2 D7 a) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED. New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. The operational mode is one of FS, LS or OS. Train is at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is FS, LS or OS and train is NOT at standstill |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and concerning train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board when mode is FS. LS or OS and train is NOT at standstill. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.7.2 |  |
|  |  | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 a) |  |
|  |  | Subset-026-5.17.2.2 D7 |  |
|  |  | Subset-026-5.17.2.2 D7 b) |  |
|  |  | Subset-026-5.17.2.2 S2 |  |
|  |  | Subset-026-5.17.2.2 S3 |  |
|  |  | Subset-026-5.17.2.2 A5 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Service Brake indication is DISPLAYED Request to ACK the brake command is DISPLAYED (when train is at standstill) Service Brake indication is REMOVED Check on the TIU that: Service Brake is APPLIED. Service Brake is RELEASED Check on the JRU that : The Service Brake intervention is RECORDED. Information displayed to the driver is RECORDED. Driver’s acknowledgement is RECORDED Release of Service Brake is RECORDED New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. The operational mode is one of FS, LS or OS. Train is not at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | V_TRAIN<>0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | Service Brake is APPLIED. | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Service Brake intervention is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Service Brake intervention is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Request for acknowledgement is DISPLAYED | O | DMI | When the train reach standstill | N/A | N/A | - |
| 6 | N/A | N/A | Driver acknowledges the brake command | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | Service Brake is RELEASED. | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | Service Brake indication is REMOVED. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | Service Brake release is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The complete set of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is UN, SR or SN and train is at standstill |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN L1 : SR L2 : SR L3 : SR LNTC: SN |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and concerning train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board when mode is UN, SR or SN. MA and track description from RBC are stored on-board and train is at standstill. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 c) |  |
|  |  | Subset-026-5.17.2.2 D5 |  |
|  |  | Subset-026-5.17.2.2 D5 a) |  |
|  |  | Subset-026-5.17.2.2 D7 |  |
|  |  | Subset-026-5.17.2.2 D7 a) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED New Train Data is RECORDED |
| Constraints | There is a complete set of Train Data stored on-board The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge The operational mode is one of UN, SN or SR MA and Track description (received from RBC) are stored on-board Train is at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Movement Authority | STORED | Level 2/3 transition or RBC transition for further location |
| RBC Transition Order | STORED | Level 2/3 transition or RBC transition for further location |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 UNCHANGED | SR / UN / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is UN, SN, or SR and train is NOT at standstill |  | 7 |
| Applicable Mode/Level Combinations | L0 : UN L1 : SR L2 : SR L3 : SR LNTC: SN |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface not requiring validation and concerning train category, axle load, traction system or loading gauge until the effective Train Data change by the ERTMS on-board when mode is UN, SR or SN, there are MA and track description stored on-board and the train is not at standstill The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.7.2 |  |
|  |  | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 c) |  |
|  |  | Subset-026-5.17.2.2 D5 |  |
|  |  | Subset-026-5.17.2.2 D5 a) |  |
|  |  | Subset-026-5.17.2.2 D7 |  |
|  |  | Subset-026-5.17.2.2 D7 b) |  |
|  |  | Subset-026-5.17.2.2 S2 |  |
|  |  | Subset-026-5.17.2.2 S3 |  |
|  |  | Subset-026-5.17.2.2 A5 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Service Brake indication is DISPLAYED Request to ack the brake command is DISPLAYED (when train is at standstill) Service Brake indication is REMOVED Check on the TIU that: Service Brake is APPLIED. Service Brake is RELEASED Check on the JRU that : The Service Brake intervention is RECORDED. Information displayed to the driver is RECORDED. Driver’s acknowledgement is RECORDED Release of Service Brake is RECORDED New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. The operational mode is one of UN, SN or SR MA and Track description (received from RBC) are stored on-board Train is not at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | UN / SR / SN |
| Movement Authority | STORED | Level 2/3 transition or RBC transition for further location |
| RBC Transition Order | STORED | Level 2/3 transition or RBC transition for further location |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | V_TRAIN<>0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | Service Brake is APPLIED. | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Service Brake intervention is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Service Brake intervention is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Request for acknowledgement is DISPLAYED | O | DMI | When the train reach standstill | N/A | N/A | - |
| 6 | N/A | N/A | Driver acknowledges the brake command | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | Service Brake is RELEASED. | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | Service Brake Indication is REMOVED. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | Service Brake Release is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | The complete set of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 UNCHANGED | SR/ UN / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data not requiring validation and concerning one of Train category, Axle load, Loading gauge or Traction system is changed when mode is UN, SR or SN and there are not MA and track description from RBC stored on-board |  | 8 |
| Applicable Mode/Level Combinations | L0 : UN L1 : SR L2 : SR L3 : SR LNTC : SN |  |  |
| Target of Test | The target of test is to check that: The Train Data is recorded when affected by changes of the input information in the ERTMS/ETCS on-board external device not requiring validation and concerning one of train category, axle load, traction system or loading gauge when the on-board equipment is in UN, SN or SR mode and there are not MA and track description from RBC stored on-board. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 b) |  |
|  |  | Subset-026-5.17.2.2 D1 |  |
|  |  | Subset-026-5.17.2.2 D1 a) |  |
|  |  | Subset-026-5.17.2.2 D3 |  |
|  |  | Subset-026-5.17.2.2 D3 c) |  |
|  |  | Subset-026-5.17.2.2 D5 |  |
|  |  | Subset-026-5.17.2.2 D5 b) |  |
|  |  | Subset-026-5.17.2.2 A1 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |
|  |  | Subset-026-4.7.2 table - Output information Notification of Train Data change from source different from the driver |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed that Train Data has been changed. Check on the JRU that : Information displayed to the driver is RECORDED. New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and not requiring validation. The Train Data changed is one of train category, axle load, traction system or loading gauge. The operational mode is UN, SR or SN. No MA or Track description from RBC stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 | SR / UN / SN |
| Movement Authority | NOT STORED | No MA or Track description from RBC stored on-board |
| RBC Transition Order | NOT STORED | No MA or Track description from RBC stored on-board |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | The impacted Train Data concerns one of train category, axle load, loading gauge or traction system. | N/A | N/A | - |
| 2 | N/A | N/A | The driver is informed that Train Data has been changed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | Information displayed to the driver is RECORDED | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The complete set of Train Data is recorded on the JRU. | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 / 4 / 13 UNCHANGED | SR / UN / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data requiring validation when mode is FS, LS, OS, SR, SB, SN or UN and train is NOT as standstill |  | 9 |
| Applicable Mode/Level Combinations | L0: UN, SB LNTC: SB, SN L1: FS, LS, OS, SR, SB L2: FS, LS, OS, SR, SB L3: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface requiring validation until the effective Train Data change by the ERTMS on-board when mode is FS. LS OS, SR, SB, SN or UN and train is NOT at standstill. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.7.2 |  |
|  |  | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 a) |  |
|  |  | Subset-026-5.17.2.2 D2 |  |
|  |  | Subset-026-5.17.2.2 D2 a) |  |
|  |  | Subset-026-5.17.2.2 D9 |  |
|  |  | Subset-026-5.17.2.2 D9 b) |  |
|  |  | Subset-026-5.17.2.2 S4 |  |
|  |  | Subset-026-5.17.2.2 S5 |  |
|  |  | Subset-026-5.17.2.2 A6 |  |
|  |  | Subset-026-5.17.2.2 S6 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Service Brake indication is DISPLAYED Request to ACK the brake command is DISPLAYED (when train is at standstill) Service Brake indication is REMOVED Request to re-enter/re-validate Train data is DISPLAYED Check on the TIU that: Service Brake is APPLIED. Service Brake is RELEASED Check on the JRU that : The Service Brake intervention is RECORDED. Information displayed to the driver is RECORDED. Driver’s acknowledgement is RECORDED Release of Service Brake is RECORDED Driver validation of Train Data is RECORDED New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and requiring validation. The operational mode is one of FS, LS OS, SR, SB, SN or UN. Train is not at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | V_TRAIN<>0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | Service Brake is APPLIED. | O | TIU | - | N/A | N/A | - |
| 3 | N/A | N/A | The Service Brake intervention is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Service Brake intervention is RECORDED. | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | Request for acknowledgement is DISPLAYED | O | DMI | When the train reach standstill | N/A | N/A | - |
| 6 | N/A | N/A | Driver acknowledges the brake command | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | Service Brake is RELEASED. | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | Service Brake indication is REMOVED. | O | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | Service Brake release is RECORDED. | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | ERTMS/ETCS on-board equipment requests the driver to re-enter or re-validate the Train Data. | O | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The driver validates the Train Data | I | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | The driver validation of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | The complete set of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data requiring validation when mode is FS, LS, OS, SR, SB, SN or UN and train is as standstill |  | 10 |
| Applicable Mode/Level Combinations | L0 : UN, SB LNTC : SB, SN L1 : FS, LS, OS, SR, SB L2 : FS, LS, OS, SR, SB L3 : FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface requiring validation until the effective Train Data change by the ERTMS on-board when mode is FS. LS OS, SR, SB, SN or UN and train is at standstill. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 a) |  |
|  |  | Subset-026-5.17.2.2 D2 |  |
|  |  | Subset-026-5.17.2.2 D2 a) |  |
|  |  | Subset-026-5.17.2.2 D9 |  |
|  |  | Subset-026-5.17.2.2 D9 a) |  |
|  |  | Subset-026-5.17.2.2 S6 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Request to re-enter/re-validate Train data is DISPLAYED Check on the JRU that : Driver validation of Train Data is RECORDED New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and requiring validation. The operational mode is one of FS, LS OS, SR, SB, SN or UN. Train is at standstill |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | ERTMS/ETCS on-board equipment requests the driver to re-enter or re-validate the Train Data. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver validates the Train Data | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver validation of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The complete set of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 12 / 13 UNCHANGED | FS / OS / SR / UN / SB / LS / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data requiring validation when mode is TR or PT and Train Trip Procedure exits in a mode different from SH |  | 11 |
| Applicable Mode/Level Combinations | L0 : TR LNTC : TR L1: TR, PT L2: TR, PT L3: TR, PT |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board since the detection of an input information change on an external interface requiring validation until the effective Train Data change by the ERTMS on-board when mode is TR or PT and Train Trip Procedure exits in a mode different from SH. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 a) |  |
|  |  | Subset-026-5.17.2.2 D2 |  |
|  |  | Subset-026-5.17.2.2 D2 b) |  |
|  |  | Subset-026-5.17.2.2 S1 |  |
|  |  | Subset-026-5.17.2.2 D4 |  |
|  |  | Subset-026-5.17.2.2 D4 a) |  |
|  |  | Subset-026-5.17.2.2 S6 |  |
|  |  | Subset-026-5.17.2.2 A7 |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Request to re-enter/re-validate Train data is DISPLAYED Check on the JRU that : Driver validation of Train Data is RECORDED New Train Data is RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and requiring validation. The operational mode is one of TR or PT. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 8 | TR / PT |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT5110200.3,11-12: Use a TC related to the exit from Train Trip procedure in a mode different from SH | - | - | - | N/A | N/A | - |
| 3 | N/A | N/A | ERTMS/ETCS on-board equipment requests the driver to re-enter or re-validate the Train Data. | O | DMI | Once the Train Trip procedure exited | N/A | N/A | - |
| 4 | N/A | N/A | The driver validates the Train Data | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver validation of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The complete set of Train Data is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 CHANGED | FS / OS / SR / UN / LS / SN |
| Train Data | STORED | New Train Data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.17.2: Table of requirements for “Changing Train Data from sources different from the driver” procedure |  | 5170200 |
| Test Case of Feature | Train Data requiring validation when mode is TR or PT and Train Trip Procedure exits in SH |  | 12 |
| Applicable Mode/Level Combinations | L0: TR LNTC: TR L1: TR, PT L2: TR, PT L3: TR, PT |  |  |
| Target of Test | The target of test is to check: The steps performed by the on-board when detecting an input information change on an external interface requiring validation when mode is TR or PT and Train Trip Procedure exits in SH. In this TC, there is no effective Train Data change because of the switch to SH. The “Manage change of Train Data from external sources” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.17.2.2 S0 |  |
|  |  | Subset-026-5.17.2.2 D0 |  |
|  |  | Subset-026-5.17.2.2 D0 a) |  |
|  |  | Subset-026-5.17.2.2 D2 |  |
|  |  | Subset-026-5.17.2.2 D2 b) |  |
|  |  | Subset-026-5.17.2.2 S1 |  |
|  |  | Subset-026-5.17.2.2 D4 |  |
|  |  | Subset-026-5.17.2.2 D4 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage change of Train Data from external sources |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : Request to re-enter/re-validate Train data is NOT DISPLAYED Check on the JRU that : New Train Data is NOT RECORDED. |
| Constraints | There is a complete set of Train Data stored on-board. The Train Data is changed from sources different from the driver and requiring validation. The operational mode is one of TR or PT. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 8 | TR / PT |
| Train Data | STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | ERTMS/ETCS on-board external interface detects a change of input information | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT5110200.6 Use a TC related to the exit from Train Trip procedure in SH | - | - | - | N/A | N/A | - |
| 3 | N/A | N/A | ERTMS/ETCS on-board equipment does NOT request the driver to re-enter or re-validate the Train Data. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The new complete set of Train Data is NOT RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 CHANGED | SH |
| Train Data | STORED UNCHANGED | Same Train data |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 UNCHANGED | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |