| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3180400 (3.18.4 Additional Data) Total: 13 Test case(s) |  |
| REF: | Subset-076-5-2-3180400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool, Answer to Review001 | MULTITEL |
| Version 2.9.2 10.02.7655 | TC6, TC7 | Modifications requested by issues 1-3, 6, 8 from Review001_Subset076-5-2-3180400_v291 | MULTITEL |
| Version 2.9.3 13.12.2013 | TC3-4, TC7 | Modifications requested by 4-5 and 7 from Review001_Subset076-5-2-3180400_v291 | MULTITEL |
| Version 2.9.4 20.12.2013 | TC7 | Add the requirement "7.4.2.36 Packet Number 140: Train running number from RBC" | MULTITEL |
| Version 2.9.5 09.01.2014 | TC6 | Modifications requested by issue 1 of Review001_Subset076-5-2-3180400_v294 | MULTITEL |
| Version 2.9.6 22.01.2014 | TC8 | Apply of issue 1 from Review001_Subset076-5-2-3180400_v295.doc | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Entry / modification and validation of a set of train data for trains without radio connection to RBC |  | 3 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL |  |  |
| Target of Test | The target of this test case is to check that the driver is allowed to modify the train running number. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.5.2 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the driver can select ‘Train Data Entry’, . the driver is allowed to modify the train running number Check on the JRU that : The driver‘s selection action is recorded. |
| Constraints | This test case takes place within the scope of the Start of Mission procedure. The status of the Train Data is UNKNOWN or INVALID i.e each train data may be either unknown or stored in the On-Board equipment; in that case it has to be validated by the driver. The Desk is OPEN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Train Data | UNKNOWN / INVALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| No train data displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/L1 | NL | The driver can select ‘Train running number’ on 'Main Window" | O | DMI | Starting conditions | L0/LNTC/L1 | NL | - |
| 2 | L0/LNTC/L1 | NL | The driver selects ‘Train running number’ | I | DMI | - | L0/LNTC/L1 | NL | - |
| 3 | L0/LNTC/L1 | NL | The “Train running number” is displayed | O | DMI | - | L0/LNTC/L1 | NL | - |
| 4 | L0/LNTC/L1 | NL | The driver enters the "Train Running Number" | I | DMI | The driver is allowed to modify the train running number | L0/LNTC/L1 | NL | - |
| 5 | L0/LNTC/L1 | NL | The driver confirms the "Train Running Number" | I | DMI | The driver validates the new train running number | L0/LNTC/L1 | NL | - |
| 6 | L0/LNTC/L1 | NL | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | New data is RECORDED | L0/LNTC/L1 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Train Data | VALID | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry is enabled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Entry / modification and validation of set of train data for trains with radio connection (it means session opened) to RBC |  | 4 |
| Applicable Mode/Level Combinations | L2: NL L3: NL |  |  |
| Target of Test | The target of this test case is to check that the driver is allowed to modify the train running number. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.5.2 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the driver can select ‘Train Data Entry’, . the driver is allowed to modified the train running number Check on the JRU that : The driver‘s selection action is recorded The modified Train Running Number is recorded upon validation by the driver. |
| Constraints | In this test case, the driver is expected to modify the train running number |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3/4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| THE TRAIN DATA REQUEST IS ALLOWED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | NL | The driver can select ‘Train running number’ on 'Main Window" | O | DMI | Starting conditions | L2/L3 | NL | - |
| 2 | L2/L3 | NL | The driver selects ‘Train running number’ | I | DMI | - | L2/L3 | NL | - |
| 3 | L2/L3 | NL | The “Train running number” is displayed | O | DMI | The current set of Train Data is displayed in the window. | L2/L3 | NL | - |
| 4 | L2/L3 | NL | The driver enters the "Train Running Number" | I | DMI | The driver is allowed to modify the train running number | L2/L3 | NL | - |
| 5 | L2/L3 | NL | The driver confirms the "Train Running Number" | I | DMI | The driver validates the modified train running number . | L2/L3 | NL | - |
| 6 | L2/L3 | NL | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | Driver’s validation action is recorded on the JRU. The Train Running Number entered by the driver is recorded | L2/L3 | NL | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | session opened |
| Train Data | VALID | Train running number is new |
| ERTMS/ETCS level | 3/4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry is enabled | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | It is possible to enter driver ID in a non-leading engine. |  | 5 |
| Applicable Mode/Level Combinations | L0: NL LNTC: NL L1: NL L2: NL L3: NL |  |  |
| Target of Test | The target of this test case is to check that it is possible to enter driver ID in a non-leading engine. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.1.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The on-board equipment allows the driver to modify the Driver Id. Check on the JRU that : The modified driver ID is recorded and is identical to the data entered by the driver. |
| Constraints | The train is at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver enters the "Driver ID" | I | DMI | Entered Driver ID: DRIVER_ID_1 (different from DRIVER_ID_0) | N/A | N/A | - |
| 4 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=<DRIVER_ID_1>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |
| Driver ID | VALID | DRIVER_ID=< DRIVER_ID _1> CHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train is at standstill | I | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | If the driver selects “Use of EIRENE short number” to contact the RBC and the communication session is successfully established, the ERTMS/ETCS on-board equipment shall store as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. |  | 6 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SB, PT, NL L3: FS, LS, OS, SR, SB, PT, NL |  |  |
| Target of Test | The target of this test case is to check that the ERTMS/ETCS on-board equipment stores as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.4 |  |
|  |  | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 d) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.5.3.11 |  |
|  |  | Subset-026-7.4.3.3 Packet Number 2: Onboard supported system versions |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The RBC identity and telephone number are the RBC identity reported by EURORADIO and the EIRENE short number, respectively. Check on the RTM that: Message 136 “Train Position Report” including packet 5 “Train Running Number” is sent Check on the JRU that : Message 136 “Train Position Report” including packet 5 “Train Running Number” is RECORDED |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| Radio communication session | ESTABLISHED | At least one Mobile Terminal registered to a Radio Network |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | INVALID | - |
| Train Running Number | VALID | The train running number has been provided by the driver |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "Level" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver selects "Level 2" or “Level 3” | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The RBC Contact Window is displayed | O | DMI | Because no valid RBC contact info is stored on-board | N/A | N/A | - |
| 6 | N/A | N/A | The driver selects "Use short number" | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) is recorded | O | JRU | MASK=0000000000020000000000 | N/A | N/A | - |
| 9 | N/A | N/A | The on-board tries to set up a connection with the short phone number defined by EIRENE | O | RTM | According to EURORADIO specifications This attempt shall be repeated until successful or a defined number of times | N/A | N/A | - |
| 10 | N/A | N/A | Setting up of a safe connection is successful. | I | RTM | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) | O | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | Message 155: “Initiation of a communication session“ is RECORDED. | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions are compatible. | N/A | N/A | - |
| 14 | N/A | N/A | Message 32: “RBC/RIU System Version Configuration Determination” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 15 | N/A | N/A | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted | O | RTM | Communication session established for on-board. | N/A | N/A | - |
| 16 | N/A | N/A | Message 159: “Session established” is RECORDED. | O | JRU | - | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0, packet 5 is transmitted | O | RTM | Packet 1 must be used instead of Packet 0 if position report is based on 2 BG | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136; NID_PACKET=0; NID_PACKET=5) is recorded | O | JRU | - | N/A | N/A | - |
| 19 | N/A | N/A | Main Window is DISPLAYED | O | DMI | - | N/A | N/A | - |
| 20 | N/A | N/A | The driver selects "Level" | I | DMI | - | N/A | N/A | - |
| 21 | N/A | N/A | The driver selects "Level 2" or “Level 3” | I | DMI | - | N/A | N/A | - |
| 22 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | The main Windows is displayed instead of the RBC Contact Window. This proofs that the on-board has stored as valid RBC identity and telephone number, the RBC identity reported by EURORADIO and the EIRENE short number, respectively. | N/A | N/A | - |
| 24 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit_and[DMI_SYMB_STATUS,MASK]=MASK) is recorded | O | JRU | MASK=0000000000020000000000 | N/A | N/A | - |

| Step 11: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| Step 13: Radio Message 32 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 32 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 15: Radio Message 159 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 159 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 2 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| N_ITER | 5 | FINITE VALUE | - |
| M_VERSION(k) | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |

| Step 17: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 5 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 11 / 12 | FS / OS / SR / SB / PT / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| RBC ID/Phone Number | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Main window with all buttons disabled. The hour glass symbol ST05 is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | It shall be possible to change the train running number while running from the RBC |  | 7 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / SB / NL L3: FS / LS / OS / SR / SB / NL |  |  |
| Target of Test | The target of this test case is to check that : The RBC can modify the Train Running number when the train is running Note: The test case can be used in SB mode but outside the scope of the Start of mission procedure |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.4.1 |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.36 Packet Number 140: Train running number from RBC |  |
|  |  | Subset-026-4.7.2 table - Input information Train running number |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Train Running is NID_OPERATIONAL_1 as received in packet 140 (step 1) Check on the JRU that : The radio message 3 with the included packets received from the RBC is recorded. Driver’s selection of ” Train Running Number” is recorded |
| Constraints | In this test case, the driver is expected to attempt to modify the Train Running Number. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 11 / 12 | FS / OS / SR / SB / NL / LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Default Window is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=24) containing packet 140 is received | I | RTM | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) The on-board equipment has received the radio message 24 “General Message” with packet 140 “Train running number from RBC” | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 5 is transmitted | O | RTM | Check that the on-board does NOT sent a message 136 “position report” with packet 5. This is to test req. 3.18.4.5.4.1 | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects “Main” | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver selects “Train Running Number” | I | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The Train Running Number Window is displayed | O | DMI | The Train Running is now NID_OPERATIONAL_1 received in packet 140 (step 1) | N/A | N/A | - |
| 8 | N/A | N/A | The driver closes the " Train Running Number " window | I | DMI | - | N/A | N/A | - |
| 9 | N/A | N/A | The Main window is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver closes the "Main" window | I | DMI | - | N/A | N/A | - |
| 11 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 140 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_OPERATIONAL | 32 | NID_OPERATIONAL_1 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_1> |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Train Running Number Window is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | The on-board equipment, when the train is NOT at standstill, allows the driver to modify the train running number and the Driver ID (if allowed by national value). The driver modifies the train running number. |  | 8 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS / SR / NL L3: FS / LS / OS / SR / NL |  |  |
| Target of Test | The target of this test case is to check that : When the train is running, the driver is permitted to modify the train running number and the Driver ID (if allowed by national value). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.1.2 |  |
|  |  | Subset-026-3.18.4.5.3 |  |
|  |  | Subset-026-3.18.4.5.4 |  |
|  |  | Subset-026-3.18.4.5.5 |  |
|  |  | Subset-026-A.3.11 Train running number |  |
|  |  | Subset-026-4.7.2 table - Input information Train running number |  |
|  |  | Subset-026-7.4.3.4.1 Packet Number 5: Train running number |  |
|  |  | Subset-026-8.4.4.4.2 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The Train Running Number is now NID_OPERATIONAL_1 entered by the driver Check on the JRU that : Message 136 including packet 5 is RECORDED |
| Constraints | The current Train Running Number has been provided by the driver. The system version number is not X = 1. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_0> |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The train is moving. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects "Main" | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The "Driver ID" button is enabled | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The "Train running number" button is enabled | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses “Train Running Number” | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) | N/A | N/A | - |
| 7 | N/A | N/A | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 5 is transmitted | O | RTM | Message 136 “Train Position Report” including packet 5 “Train Running Number” is sent NID_OPERATIONAL in packet 5 is equal to NID_OPERATIONAL_1 entered by the driver in step 6. | N/A | N/A | - |
| 9 | N/A | N/A | Message 136 “Train Position Report” including packet 5 “Train Running Number” is RECORDED | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 11 / 12 / 13 | FS / OS / SR / UN / NL / LS / SN |
| National Values | STORED / NOT STORED | If STORED: M_NVDERUN=1 (Change of driver ID permitted while running) |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Train Running Number | STORED | NID_OPERATIONAL=<NID_OPERATIONAL_1> |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is NOT at standstill | I | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | If, following a change of National Values, the update of the adhesion factor is no more permitted to the driver, the adhesion factor previously modified by the driver to slippery rail shall immediately be reset to non slippery rail. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that : - a message including new national values is received (Q_NVDRIVER_ADHES = 0, update of the adhesion factor is no more permitted to the driver); - the driver hasn’t the possibility to modify the track adhesion factor. - the track adhesion factor is reset to non slippery rail - the driver is informed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : - the adhesion factor is reset from slippery to non slippery - the driver is not permitted to change the adhesion factor - the driver is informed Check on the JRU that - the national values are received |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES = 1; The driver is allowed to modify the adhesion factor. |
| Adhesion factor | VALID | Adhesion factor = 100 % (Default value) |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 and L3 |
| NOT RELEVANT | - | TIU | - |
| Default Window is Displayed | O | DMI | The Status symbol "Adhesion factor - slippery rail" is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Special” | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The Special window is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects “Adhesion” | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The <Adhesion> window is displayed. | O | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 7 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 00100000b) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST_02 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The Special window is displayed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | The driver closes the "Special" window | I | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | The Default window is displayed | O | DMI | - | N/A | N/A | - |
| 14 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 15 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | BTM | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 16 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; N_TOTAL=1; Q_NVDRIVER_ADHES=0) is recorded | O | JRU | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 17 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) with Packet 3 (Q_NVDRIVER_ADHES = 0) is received. | I | RTM | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 18 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24; NID_PACKET=3; Q_NVDRIVER_ADHES = 0) is recorded | O | JRU | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 19 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is removed | O | DMI | ST_02 removed The onboard resets the adhesion factor from slippery to not slippery and the driver should be informed. | N/A | N/A | - |
| 20 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=0>) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The driver is not allowed to modify the adhesion factor. Use_FT9990200.43 | - | - | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | Q_NVDRIVER_ADHES = 0; The driver is not allowed to modify the adhesion factor. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Default Window is displayed | O | DMI | The Status symbol "Adhesion factor - slippery rail" is not displayed |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | Any trackside adhesion profile is not affected, when the driver update is reset due to reception of new national values with variable Q_NVDRIVER_ADHES set to 0. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR L2: FS / LS / OS / SR L3: FS / LS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that : - Any trackside adhesion profiles are not affected due to reset of the driver update of adhesion profile. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.6.2.1 |  |
|  |  | Subset-026-7.4.2.1.1 Packet Number 3: National Values |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - the adhesion factor is not reset to 100% - the driver is not permitted to change the adhesion factor Check on the JRU that - the national values are received |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | Only in L2 and L3 |
| National Values | STORED | Q_NVDRIVER_ADHES = 1; The driver is allowed to modify the adhesion factor. |
| Adhesion factor | VALID | Adhesion factor = 100 % (Default value) |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only in L2 and L3 |
| NOT RELEVANT | - | TIU | - |
| The Track Adhesion factor is Displayed | O | DMI | Default value = 100% |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is stopped (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | The <Adhesion> window is displayed. | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver modifies "Adhesion Factor" | I | DMI | The driver selects Slippery rail | N/A | N/A | - |
| 4 | N/A | N/A | The driver confirms "Adhesion Factor" | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 00100000b) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | ST_02 | N/A | N/A | - |
| 7 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit39=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | Train is moving (V_TRAIN=<V_PERMITTED>) | I | INT | - | N/A | N/A | - |
| 9 | N/A | N/A | The train accepts a new adhesion factor send by a balise group with an M_ADHESION = 0 (70%). -Use_FT4080402.1 for L2 or L2 -Use_FT4080402.3 for L1 | - | - | - | N/A | N/A | - |
| 10 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | BTM | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 11 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; N_TOTAL=1; Q_NVDRIVER_ADHES=0) is recorded | O | JRU | Only for L1. Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 12 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) with Packet 3 (Q_NVDRIVER_ADHES=0) is received. | I | RTM | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 13 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24; NID_PACKET=3; Q_NVDRIVER_ADHES = 0) is recorded | O | JRU | Only for L2 /L3 Q_NVDRIVER_ADHES = 0 (driver is not allowed to modify Adhesion Factor) | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Adhesion factor - slippery rail" is displayed | O | DMI | The slippery rail icon ST_02 remains displayed. The onboard does NOT reset the adhesion factor from 70% to 100%. | N/A | N/A | - |
| 15 | N/A | N/A | The driver is not allowed to modify the adhesion factor. Use_FT9990200.43 | - | - | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | Q_NVDRIVER_ADHES = 0; The driver is not allowed to modify the adhesion factor. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | During SoM, a Mobile Terminal is registered to a Radio Network and the RBC Identity + Phone Number Status is Invalid. The on-board offers the possibility to the driver to select the RBC contact information. The driver inserts a RBC ID number different from the previous and validates it. The driver completes the SoM and the on-board sets up safe Radio connection with the correct RBC. |  | 11 |
| Applicable Mode/Level Combinations | L1: SB L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that: the ERTMS/ETCS on-board equipment stores one valid RBC identity and telephone number at a time, obtained from the last driver data entry. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-3.18.4.3.2 |  |
|  |  | Subset-026-3.18.4.3.5 |  |
|  |  | Subset-026-A.3.11 RBC ID |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC-id |  |
|  |  | Subset-026-4.7.2 table - Input information RBC contact information - RBC phone number |  |
|  |  | Subset-026-5.4.3.2 S3 |  |
|  |  | Subset-026-5.4.3.2 S3 e) |  |
|  |  | Subset-026-5.4.3.2 A31 |  |
|  |  | Subset-026-5.4.3.2 D31 |  |
|  |  | Subset-026-5.4.3.3 - Following D31: session has been successfully opened |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI: The driver enters a new RBC ID Number; Check on JRU: The messages to RBC are recorded during the phase of connection: the RBC ID Number shall be the one inserted by the driver in this TC. |
| Constraints | Train is at standstill; RBC IDENTITY + PHONE NUMBER STATUS is INVALID; The train is in the overlapping area of the previous RBC and of the new RBC (ID Number inserted by the driver). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Table of priority of trackside supported levels | STORED | - |
| Driver ID | VALID | Entered after the desk is open |
| RBC ID/Phone Number | INVALID | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The “RBC contact window” is displayed with “Use short number”, “Enter RBC data” and “Radio network ID” enabled | O | DMI | The window is displayed during the SoM procedure |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses the ‘Enter RBC data’ button | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver modifies the existing RBC ID/Phone number. | I | DMI | The new RBC ID/Phone number shall be different from the displayed ones. | N/A | N/A | - |
| 4 | N/A | N/A | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. | N/A | N/A | - |
| 7 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | The RBC ID Number shall be the one inserted by the driver in step 3 | N/A | N/A | - |
| 11 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | No communication session shall be established with the previously stored RBC | N/A | N/A | - |

| Step 9: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | VALID | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| RBC ID/Phone Number | VALID | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | O | RTM | Connection set up with the RBC whose RBC ID/Phone number were inserted by the driver |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | The on-board has a previous RBC ID Number stored and no connection is set up An order to set up a safe radio communication session with a new RBC is received from a Balise Group and the on-board sets up the communication. |  | 12 |
| Applicable Mode/Level Combinations | L1: FS, OS, SR, LS L2: FS, OS, SR, LS L3: FS, OS, SR, LS |  |  |
| Target of Test | The target of this test is to check that: the ERTMS/ETCS on-board equipment stores one valid RBC identity and telephone number at a time, obtained from the last received order to establish a session with an RBC (excluding RBC transition orders) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.5.3.4 |  |
|  |  | Subset-026-3.5.3.4 b) |  |
|  |  | Subset-026-3.5.3.7 |  |
|  |  | Subset-026-3.5.3.7 a) |  |
|  |  | Subset-026-3.5.3.7 b) |  |
|  |  | Subset-026-3.18.4.3.1 |  |
|  |  | Subset-026-8.6.13 Message 155: Initiation of a communication session |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on JRU: The messages to RBC are recorded during the phase of connection: the RBC ID Number shall be the one received by the balise |
| Constraints | Train is moving. The train is in the overlapping area of the previous RBC and of the new RBC (received by the balise). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | NOT ESTABLISHED | - |
| Train Data | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message (NID_RBC≠16383) containing packet 42 (NID_RADIO ≠ FFFF FFFF FFFF FFFF) is received | I | BTM | Not use last known RBC and don’t use short number. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-CONNECT.Request is transmitted | O | RTM | The on-board tries to set up a connection with the phone number included in the session management packet. | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | The RBC ID Number shall be the one received from the Balise Step 2 | N/A | N/A | - |
| 8 | N/A | N/A | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded | O | JRU | No communication session shall be established with the previously stored RBC | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | Relevant direction for the train |
| L_PACKET | 13 | 113 | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | RBC ETCS identity |
| NID_RBC | 14 | ≠ 16383 | NID_RBC ≠ 16383 (Contact last known RBC) Onboard will initiate communication to a different RBC from the last known |
| NID_RADIO | 64 | ≠ FFFF FFFF FFFF FFFF | NOT use short number |
| Q_SLEEPSESSION | 1 | FINITE VALUE | Order to contact RBC also in SL mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 155 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 155 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.18.4 Additional Data |  | 3180400 |
| Test Case of Feature | At standstill, the ERTMS/ETCS on-board equipment shall offer the possibility to the driver to modify the Radio Network ID. |  | 13 |
| Applicable Mode/Level Combinations | L0: SB, NL, UN LNTC: SB, NL, SN L1: SB, FS, LS, SR, OS, NL, PT L2: SB, FS, LS, SR, OS, NL, PT L3: SB, FS, LS, SR, OS, NL, PT |  |  |
| Target of Test | The target of this test is to check that, at standstill, the ERTMS/ETCS on-board equipment offers the possibility to the driver to modify the Radio Network ID. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.18.4.3.6 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI: The on-board allows the driver to open the “Radio data” window and to modify the Radio Network ID. Check on JRU: NID_MESSAGE_JRU=48 with the Radio Network ID selected by the driver is recorded |
| Constraints | Train is at standstill. Train is in an area with radio coverage. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SB / PT / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver presses “Radio data” | I | DMI | - | N/A | N/A | - |
| 2 | N/A | N/A | The Radio data window is displayed | O | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver presses “Radio Network ID” | I | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The Radio Network ID window is displayed | O | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | The driver selects a Radio Network ID | I | DMI | It is possible that the driver has to wait the onboard completes the alphanumeric list of available and allowed Radio Networks | N/A | N/A | - |
| 6 | N/A | N/A | RADIO NETWORK ID ENTERED BY THE DRIVER (NID_MESSAGE_JRU=48) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 8 / 11 / 12 / 13 | FS / OS / SR / UN / SB / PT / NL / LS / SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |