| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3060500 (Position Reporting to the RBC) Total: 24 Test case(s) |  |
| REF: | Subset-076-5-2-3060500 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 22.10.2013 | All | review | DLR |
| Version 2.9.2 05.12.2013 | All | review | DLR |
| Version 2.9.3 13.12.2013 | All | fixed use steps and TC 9 which was defective (Excel sheet) | DLR |
| Version 2.9.4 08.01.2014 | All | review | DLR |
| Version 2.9.5 22.01.2014 | TC15 TC11 TC12 TC13 TC14 TC16 TC17 TC18 TC19 TC37 | “Preceeded step” changed to “Use_” “Followed by” step changed to “Use_” | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | Supervision of train integrity entered by driver. Driver input of train integrity is only permitted at standstill. The driver confirms train integrity (Q_LENGTH = 2). |  | 1 |
| Applicable Mode/Level Combinations | L3 : FS, LS, OS, SR, SB, PT |  |  |
| Target of Test | The target of this test is: • The driver enters train integrity information when the train is at standstill. This data entry is RECORDED in the JRU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 c) |  |
|  |  | Subset-026-3.6.5.2.1 |  |
|  |  | Subset-026-3.6.5.2.2 |  |
|  |  | Subset-026-3.6.5.2.3 |  |
|  |  | Subset-026-3.6.5.2.3 a) |  |
|  |  | Subset-026-3.6.5.2.3 a) a) |  |
|  |  | Subset-026-3.6.5.2.3 a) c) |  |
|  |  | Subset-026-3.6.5.2.3 b) |  |
|  |  | Subset-026-3.6.5.2.4 |  |
|  |  | Subset-026-3.6.5.2.5 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train integrity confirmed by driver |  |
|  |  | Subset-026-4.7.2 table - Input information Train integrity confirmation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that : • the driver enters train integrity information. Check on JRU that: • the driver action is RECORDED |
| Constraints | It has to be recognised that a Position Report shall be send by on-board to track-side. Driver input of train integrity shall only be permitted at standstill. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Train Position | STORED | Refered to the LRBG Q_LENGTH = 0; no train integrity available |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 255 is received | I | BTM | Train passes LRBG at instant T1 | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Check the time stamp value of instant T1 | N/A | N/A | - |
| 3 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Transmission of a position report. | N/A | N/A | - |
| 5 | N/A | N/A | The driver presses "Train Integrity" | I | DMI | The train integrity is confirmed by the driver at instant T2 | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTION=0001 1010) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | - | N/A | N/A | - |
| 8 | N/A | N/A | At the Special Window, the "Train Integrity" button is disabled | O | DMI | Check that train integrity status confirmation is not possible when the train is not at standstill. | N/A | N/A | - |
| 9 | N/A | N/A | At the Main Window, the "Train Data" button is disabled | O | DMI | Check that entering new train length is not possible when train is not at standstill | N/A | N/A | - |
| 10 | N/A | N/A | Train is in standstill (V_TRAIN=0) | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | T3: Transmission of a position report. Check that L_TRAININT = (train length) + L_DOUBTOVER and Q_LENGTH = 2. | N/A | N/A | - |
| 12 | N/A | N/A | The driver presses "Train Integrity" | I | DMI | T4: T3 + 5 sec | N/A | N/A | - |
| 13 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1010) is recorded | O | JRU | Train integrity confirmation is recorded | N/A | N/A | - |
| 14 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Transmission of a position report. Check that L_TRAININT = (train length) + L_DOUBTOVER and Q_LENGTH = 2. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BGa, incuded in the list of expected balises |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| Train Position | STORED | - |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | - | INT | V_TRAIN=0 |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | Supervision of train integrity evaluated by train integrity device. The external monitoring device confirms train integrity (Q_LENGTH = 1). |  | 2 |
| Applicable Mode/Level Combinations | L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is: • The external monitoring device evaluates train integrity. Status of train integrity is checked on TIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.2.1 |  |
|  |  | Subset-026-3.6.5.2.3 |  |
|  |  | Subset-026-3.6.5.2.3 a) |  |
|  |  | Subset-026-3.6.5.2.3 a) b) |  |
|  |  | Subset-026-3.6.5.2.4 |  |
|  |  | Subset-026-3.6.5.2.5 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that : • Train integrity is confirmed. |
| Constraints | An external integrity-monitoring device provides train integrity information to the on-board equipment via TIU. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / SB / TR / PT / LS / RV |
| Position Report parameters | STORED | T_CYCLOC available. |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train integrity confirmed | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | As soon as T_CYCLOC expires. Check that Q_LENGTH=1 | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Train integrity information to the on-board equipment is provided via TIU. | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | Supervision of train integrity evaluated by train integrity device. The external monitoring informs the onboard equipment that train integrity is lost (Q_LENGTH = 3). |  | 3 |
| Applicable Mode/Level Combinations | L3 : FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is: • The external monitoring device evaluates train integrity. Status of train integrity is checked on TIU. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.2.1 |  |
|  |  | Subset-026-3.6.5.2.3 |  |
|  |  | Subset-026-3.6.5.2.3 a) |  |
|  |  | Subset-026-3.6.5.2.3 a) d) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 d) |  |
|  |  | Subset-026-3.6.5.1.4.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on TIU that : • Train integrity is lost. |
| Constraints | An external integrity-monitoring device provides train integrity information to the on-board equipment via TIU. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / SB / TR / PT / LS / RV |
| ERTMS/ETCS level | 4 | L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train integrity lost | I | TIU | - | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3060500.4/5 : The on-board equipment sends a position report by means of Packet 0/1 Position report | - | - | Check that Q_LENGTH=3. Check that L_TRAININT = (train length) + L_DOUBTOVER. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 7 / 8 / 12 / 14 | FS / OS / SR / SB / TR / PT / LS / RV |
| Train Position | STORED | Train integrity lost (Q_LENGTH = 3) |
| ERTMS/ETCS level | 4 | L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Train integrity information to the on-board equipment is provided via TIU. | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report by means of Packet 0: “Position report”. |  | 4 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, TR, SF, NL LNTC: SH, SL, SB, TR, SF, NL, SN L1: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |
|  |  | Subset-026-3.6.5.1.1 |  |
|  |  | Subset-026-3.6.5.1.2 |  |
|  |  | Subset-026-3.6.5.1.2 a) |  |
|  |  | Subset-026-3.6.5.1.2 b) |  |
|  |  | Subset-026-3.6.5.1.2 c) |  |
|  |  | Subset-026-3.6.5.1.2 d) |  |
|  |  | Subset-026-3.6.5.1.2 e) |  |
|  |  | Subset-026-3.6.5.1.2 f) |  |
|  |  | Subset-026-3.6.5.1.2 g) |  |
|  |  | Subset-026-3.6.5.1.2 h) |  |
|  |  | Subset-026-3.6.5.1.8 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 136: “Train position report” is RECORDED. It shall include at least the following data: • Distance between the LRBG and the estimated front-end of the train (D_LRBG). • The confidence interval to this distance (L_DOUBTOVER and L_DOUBTUNDER). • Location reference Identity (NID_LRBG). • The orientation in relation to the LRBG orientation (Q_DIRLRBG). • The position of the front-end in relation to the LRBG (Q_DLRBG). • Train speed (V_TRAIN). • Train integrity information (Q_LENGTH and L_TRAININT). • Direction of train movement in relation to the LRBG orientation. • Mode (M_MODE). • Level (M_LEVEL). |
| Constraints | The train reports its position by means of Packet 0: “Position report” after passing : • A linked Balise Group (BGa or SBGa) when linking information is available onboard. OR • An unlinked Balise Group BGa. This action will be performed : a) When reaching standstill, if applicable to the current mode. b) On executing a mode change. c) When the driver has entered train integrity information. d) If loss of train integrity has been detected. e) When the train has passed a RBC/RBC border with its min safe rear end. f) If the train has changed its orientation (operational requirement: already covered by mode change in b) above). g) On executing a level transition. h) When the on-board equipment establishes a session with the RBC. i) As requested by the RBC specified in the position report parameters. j) If no position report parameters have been given from the RBC, the on-board equipment shall report the train position at every passage of an LRBG compliant balise group. k) When the train has passed a RBC/RBC border with its max safe front end. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| M_LEVEL | 3 | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”. |  | 5 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, TR, SF, NL LNTC: SH, SL, SB, TR, SF, NL, SN L1: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |
|  |  | Subset-026-3.6.5.1.1 |  |
|  |  | Subset-026-3.6.5.1.2 |  |
|  |  | Subset-026-3.6.5.1.2 a) |  |
|  |  | Subset-026-3.6.5.1.2 b) |  |
|  |  | Subset-026-3.6.5.1.2 c) |  |
|  |  | Subset-026-3.6.5.1.2 d) |  |
|  |  | Subset-026-3.6.5.1.2 e) |  |
|  |  | Subset-026-3.6.5.1.2 f) |  |
|  |  | Subset-026-3.6.5.1.2 g) |  |
|  |  | Subset-026-3.6.5.1.2 h) |  |
|  |  | Subset-026-3.6.5.1.2 i) |  |
|  |  | Subset-026-3.6.5.1.8 |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 136: “Train position report” is RECORDED. It shall include at least the following data: • Distance between the LRBG and the estimated front-end of the train (D_LRBG). • The confidence interval to this distance (L_DOUBTOVER and L_DOUBTUNDER). • Location reference Identity (NID_LRBG). • The orientation in relation to the LRBG orientation (Q_DIRLRBG). • The position of the front-end in relation to the LRBG (Q_DLRBG). • Train speed (V_TRAIN). • Train integrity information (Q_LENGTH and L_TRAININT). • Direction of train movement in relation to the LRBG orientation. • Optionally, the previous LRBG • Mode (M_MODE). • Level (M_LEVEL). |
| Constraints | When no linking information is available onboard, if the train crosses a single balise group SBGb after having passed another balise group BGa (single or not), it reports its position by means of Packet 1: “Position report based on two balise groups”. The train does not change its orientation between these two balise groups. This action will be performed : a) When reaching standstill, if applicable to the current mode. b) On executing a mode change. c) When the driver has entered train integrity information. d) If loss of train integrity has been detected. e) When the train has passed a RBC/RBC border with its min safe rear end. f) If the train has changed its orientation (operational requirement: already covered by mode change in b) above). g) On executing a level transition. h) When the on-board equipment establishes a session with the RBC. i) As requested by the RBC specified in the position report parameters. j) If no position report parameters have been given from the RBC, the on-board equipment shall report the train position at every passage of an LRBG compliant balise group. k) When the train has passed a RBC/RBC border with its max safe front end. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 1 is transmitted | O | RTM | The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PRVLRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | Train orientation according to reference direction. |
| Q_DLRBG | 2 | FINITE VALUE | Train front position according to reference direction. |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | Actual running direction according to reference direction. |
| M_MODE | 4 | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| M_LEVEL | 3 | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment is in NL mode in L2/L3 and sends position reports as requested by RBC, immediately. |  | 6 |
| Applicable Mode/Level Combinations | L2: NL L3: NL |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment in NL mode in L2/L3, shall report its position to the RBC immediately, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.4.15.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) immediately as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor periodically in space, nor at every LRBG compliant balise group passage, nor at a specified location. |
| Constraints | Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=32767; M_LOC=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT immediately. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 11 | NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment shall not report the transition PS to SH mode. |  | 7 |
| Applicable Mode/Level Combinations | L2: PS L3: PS |  |  |
| Target of Test | The target of this test is to check that: - The on-board equipment performs mode change from PS to SH - The onboard shall not try to established a safe radio connection and a radio communication session. - The train does not sends a position report after a mode change from PS to SH mode. In SH mode the function report train position when mode changes is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 b) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when mode changes to… note 2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU interface that: - After a mode change to SH from PS, the onboard equipment does not sends a position report to inform trackside about it. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | PS | The mode change from PS to SH is performed | - | - | Use_FT4060300.62 | L2/ L3 | SH | - |
| 2 | L2/ L3 | SH | NOT SA-CONNECT. Request is transmitted | O | RTM | The on-board equipment shall not try to initiate a safe radio connection. | L2/ L3 | SH | - |
| 3 | L2/ L3 | SH | NOT SA-DATA .Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) containing packet X is transmitted | O | RTM | The on-board equipment shall not try to establishes a radio communication session. | L2/ L3 | SH | - |
| 4 | L2/ L3 | SH | NOT SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | Exception[2] from table 4.5.2.1 | L2/ L3 | SH | - |
| 5 | L2/ L3 | SH | NOT MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/ L3 | SH | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SH |
| Radio communication session | NOT ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| Cab active | I | TIU | - |
| The Mode symbol "Shunting" is displayed | O | DMI | DMI Object: symbol MO01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report after the train has passed a RBC/RBC border with its min safe rear end and max safe front end . |  | 8 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, TR L3: FS, LS, OS, SR, TR |  |  |
| Target of Test | The target of this test is to check that: • The train sends a position report after passing a RBC/RBC border with its min safe rear end and max safe front end (only level 2/3). • The function Report train position when train front/rear passes an RBC/RBC border is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 k) |  |
|  |  | Subset-026-3.6.5.1.4 e) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train front/rear passes an RBC/RBC border (only level 2/3) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • After passing a RBC/RBC border with its min safe rear end, the train sends a position report to inform trackside about it. • After passing a RBC/RBC border with its max safe front end, the train sends a position report to inform trackside about it Once the Handing Over RBC has received the position report and detected that the minimum safe rear end of the train has crossed the border, it shall send a disconnection order to the on-board equipment. |
| Constraints | After passing the RBC/RBC border with its min safe rear end and Max safe front end, the train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4: The on-board equipment sends a position report by means of Packet 0: “Position report”. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5: The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train passes a RBC/RBC border with its max safe front end. | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train passes a RBC/RBC border with its min safe rear end. | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 / 12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report when the train passes with its min safe rear end a level transition border which led to a transition from level 2/3 to level 0, NTC or 1. |  | 9 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, TR L3: FS, LS, OS, SR, TR |  |  |
| Target of Test | The target of this test is to check that: • The train passes with its min safe rear end a level transition border which led to a transition from level 2/3 to level 0, NTC or 1. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 f) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train rear passes a level transition border (from level 2/3 to 0, NTC, 1) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • After train the rear passes a level transition border (from level 2/3 to 0, NTC, 1), the onboard equipment sends a position report to inform trackside about it. |
| Constraints | When the train rear passes a level transition border (from level 2/3 to 0, NTC, 1), the train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4: The on-board equipment sends a position report by means of Packet 0: “Position report”. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5: The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”. A communication session has already been established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 7 /12 | FS / OS / SR / TR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / OS / SR / TR / LS | The level transition order to L0/LNTC/L1 is performed at the current position: USE_FT5100100.11: Order to switch immediately from L2 to L0 ; USE_FT5100100.29: From L3 to L0 ; USE_FT5100100.30: From L2 to LNTC; USE_FT5100100.25: From L3 to LNTC; USE_FT5100100.32: From L2 to L1 or USE_FT5100100.28: From L3 to L1. | - | - | The level switch must be from level 2/3 to 0, NTC, 1 | L0 / LNTC / L1 | FS / OS / SR / UN / TR / LS / SN | - |
| 2 | L0 / LNTC / L1 | FS / OS / SR / UN / TR / LS / SN | Train is moving. The train rear passes the level transition border | I | INT | - | L0 / LNTC / L1 | FS / OS / SR / UN / TR / LS / SN | - |
| 3 | L0 / LNTC / L1 | FS / OS / SR / UN / TR / LS / SN | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | L0 / LNTC / L1 | FS / OS / SR / UN / TR / LS / SN | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 /12 / 13 | FS / OS / SR / UN / TR / LS / SN |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The train sends a position report at every linked balise passage when no linking information is stored. |  | 10 |
| Applicable Mode/Level Combinations | L0: UN, NL LNTC: TR, NL, SN L1: FS, LS, OS, SR, TR, PT, NL, RV L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report after a linked Balise Group passage. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 j) |  |
|  |  | Subset-026-4.5.2.1 table - … or Report train position at every passage of an LRBG compliant balise group |  |
|  |  | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Balise group message is RECORDED. |
| Constraints | Packet 58: “Position Report Parameters” has not been previously received or the position report parameters have been deleted. The linked Balise Group is composed of 1 or more balises. The train reports its position by means of Packet 0: “Position report” after passing a linked Balise Group (BGa or SBGa) when linking information is available onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (Q_LINK=0) containing packet 255 is received | I | BTM | The train overpasses a non linked balise group. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | NO Train Position Report is SENT. | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 255 is received | I | BTM | The train overpasses a linked balise group. | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The train passes an LRBG compliant Balise group |  | 11 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV L3: FS, LS, OS, SR, SH, SL, SB, TR, PT, SF, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 j) |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Message 136: “Train position report” is RECORDED. It shall include at least the following data: • Distance between the LRBG and the estimated front-end of the train (D_LRBG). • The confidence interval to this distance (L_DOUBTOVER and L_DOUBTUNDER). • Location reference Identity (NID_LRBG). • The orientation in relation to the LRBG orientation (Q_DIRLRBG). • The position of the front-end in relation to the LRBG (Q_DLRBG). • Train speed (V_TRAIN). • Train integrity information (Q_LENGTH and L_TRAININT). • Direction of train movement in relation to the LRBG orientation. • Mode (M_MODE). • Level (M_LEVEL). |
| Constraints | The train has no train position report parameters stored Onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 14 | FS / OS / SR / SH / SL / SB / TR / PT / SF / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 255 is received | I | BTM | LRBG compliant balise group. Balise groups marked as linked and contained in the previously received linking information, if linking information is used on-board. OR The last balise group not marked as unlinked, if no linking was used when this balise group was passed. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Transmission of a position report. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Balise groups marked as linked (Q_LINK=1) and contained in the previously received linking information, if linking information is used on-board. OR The last balise group not marked as unlinked (Q_LINK=0), if no linking was used when this balise group was passed. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 5 / 6 / 7 / 8 / 9 / 11 / 12 / 14 | FS / OS / SR / SH / SL / SB / TR / PT / SF / NL / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The train passes a linked Balise Group (BGa or SBGa) not included in the linking information stored onboard. The onboard will not send a position report. |  | 12 |
| Applicable Mode/Level Combinations | L0: UN, NL LNTC: TR, NL, SN L1: FS, LS, OS, SR, TR, PT, NL, RV L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment does not send a position report after a linked Balise Group not included in the linking info is passed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 j) |  |
|  |  | Subset-026-4.5.2.1 table - … or Report train position at every passage of an LRBG compliant balise group |  |
|  |  | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Balise group (BGa or SBGa) Message is RECORDED. Check on the RTM that: • No position report (message 136) is sent. |
| Constraints | Packet 58: “Position Report Parameters” has not been previously received. The linked Balise Group (BGa or SBGa) is composed of 1 or more balises. No event requires sending a message 136 at the location where this TC is used. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Linked Balise Group (BGa or SBGa) is NOT included. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (Q_LINK=1) containing packet 255 is received | I | BTM | The train overpasses a balise group not included in the linking info. | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | NO Train Position Report is SENT. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group. Not included in the linking information |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report after a mode change. |  | 13 |
| Applicable Mode/Level Combinations | L2: FS, OS, SR, SH, UN, SL, SB, TR, PT, SF, NL, LS, RV L3: FS, OS, SR, SH, UN, SL, SB, TR, PT, SF, NL, LS, RV |  |  |
| Target of Test | The target of this test is to check that: • The train sends a position report after a mode change. The message includes the new mode value. • The function report train position when mode changes to FS, OS, SR, SH, UN, SL, SB, TR, PT, SF, NL, LS or RV is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 b) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when mode changes to… |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check that: • After a mode change, the onboard equipment sends a position report to inform trackside about it. |
| Constraints | The train has previously changed of mode. After the mode change, the train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4: The on-board equipment sends a position report by means of Packet 0: “Position report”. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5: The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”. A communication session has already been established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 9 / 11 /12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Transmission of a position report. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 9 / 11 /12 / 13 / 14 | FS / OS / SR / SH / UN / SL / SB / TR / PT / SF / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, periodically in time. |  | 15 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC periodically in time, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) periodically in time as specified in Packet 58: “Position Report Parameters”. It shall include at least the following data: • Message 136: “Train position report” is not SENT periodically in space, nor at a specified location, nor at every LRBG compliant balise group passage. In order to set a reference for the next messages, the first train position report is requested to be sent immediately (M_LOC=0). |
| Constraints | A cab needs to be active (SB Mode) Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4 OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5 Train speed must not be constant during the test to avoid confusion between location reports periodically sent in space and periodically sent in time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=1 to 254; D_CYCLOC=32767; M_LOC=0; N_ITER=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving | I | INT | No action until T_CYCLOC has expired. Speed shall not be constant during steps 4 to 6 | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |
| 6 | N/A | N/A | Repeat steps 4 to 5 until new Position Reports Parameters (Packet 58) is RECEIVED. | I | INT | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | FINITE VALUE | Between 1 and 254 |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, periodically in space. |  | 16 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC periodically in space, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) periodically in space as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor at a specified location, nor at every LRBG compliant balise group passage. In order to set a reference for the next messages, the first train position report is requested to be sent immediately (M_LOC=0). |
| Constraints | A cab needs to be active (SB Mode) Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4 OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5 Train speed must not be constant during the test to avoid confusion between location reports periodically sent in space and periodically sent in time. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=1 to 32766; M_LOC=0; N_ITER=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving | I | INT | No action until train moves distance D_CYCLOC Speed shall not be constant during steps 4 to 6 | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |
| 6 | N/A | N/A | Repeat steps 4 to 5 until new Position Reports Parameters (Packet 58) is RECEIVED. | - | - | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 1 to 32766 | - |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, at a specified location. |  | 17 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC at a specified location when the Max Safe Front End has overpassed the location, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) at a specified location as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor periodically in space, nor at every LRBG compliant balise group passage, nor immediately. |
| Constraints | A cab needs to be active (SB Mode) Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4 OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=32767; D_LOC > 0;M_LOC=2; Q_LGTLOC=1) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | No action until train moves distance D_LOC | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | When distance D_LOC has been reached, Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 2 | Do not send position report on passage of LRBG compliant balise group |
| N_ITER | 5 | 1 | - |
| D_LOC(1) | 15 | FINITE VALUE | - |
| Q_LGTLOC(1) | 1 | 1 | Max safe front end |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, at a specified location. |  | 18 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS, OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC at a specified location when the Min Safe Rear End has overpassed the location, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) at a specified location as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor periodically in space, nor at every LRBG compliant balise group passage, nor immediately. |
| Constraints | A cab needs to be active (SB Mode) Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4 OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=32767; D_LOC > 0; M_LOC=2; Q_LGTLOC=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | No action until train moves distance D_LOC | N/A | N/A | - |
| 4 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | When distance D_LOC has been reached, Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 2 | Do not send position report on passage of LRBG compliant balise group |
| N_ITER | 5 | 1 | - |
| D_LOC(1) | 15 | FINITE VALUE | - |
| Q_LGTLOC(1) | 1 | 0 | Min safe front end |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, at every passage of an LRBG compliant balise group. |  | 19 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, TR, PT, RV L2: FS, LS, OS, SR, TR, PT, RV L3: FS, LS; OS, SR, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC at every passage of an LRBG compliant balise group, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) at every passage of an LRBG compliant balise group as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor periodically in space, nor at a specified location, nor immediately. |
| Constraints | Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4 OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=32767; M_LOC=1) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | No action until next LRBG compliant balise group has been reached | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL>0) containing packet 255 is received | I | BTM | The train ovepasses an LRBG compliant balise group | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 1 | At every LRBG compliant balise group |
| N_ITER | 5 | 0 | - |

| Step 4: Eurobalise Telegram (balise 1/>0) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | >0 | No single balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends position reports as requested by RBC, immediately. |  | 20 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS; OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The ERTMS/ETCS on-board equipment shall report its position to the RBC immediately, according to the previously received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) immediately as specified in Packet 58: “Position Report Parameters”. • Message 136: “Train position report” is not SENT periodically in time, nor periodically in space, nor at every LRBG compliant balise group passage, nor at a specified location. |
| Constraints | A cab needs to be active. (SB Mode) Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; D_CYCLOC=32767; M_LOC=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT immediately. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No ack. required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | Position report parameters |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | Not periodically in time |
| D_CYCLOC | 15 | 32767 | - |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The position report parameters are valid until new parameters are given from the RBC. |  | 21 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, LS, OS, SR, SB, TR, PT, RV L2: FS, LS, OS, SR, SB, TR, PT, RV L3: FS, LS; OS, SR, SB, TR, PT, RV |  |  |
| Target of Test | The target of this test is to check that: • The position report parameters are valid until new parameters are given from the RBC. • The ERTMS/ETCS on-board equipment shall report its position to the RBC according to the received parameters. • The on-board function “Report train position as requested by RBC…“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.7 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position as requested by RBC… |  |
|  |  | Subset-026-7.4.2.15 Packet Number 58: Position report parameters |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Train position reports (Message 136) are SENT (and RECORDED) immediately as specified in Packet 58: “Position Report Parameters”. |
| Constraints | Packet 58: “Position Report Parameters” has been previously received. The train reports its position by means of: • Packet 0: “Position report”, as described in FT3060500.4. OR • Packet 1: “Position report based on two single balise groups“, as described in FT3060500.5. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=60; D_CYCLOC=32767; M_LOC=0) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | At t=t0, message 136: “Train Position Report” is SENT. The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 4 | N/A | N/A | Train is moving (V_TRAIN=40; TIME=t0+60) | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | At t=t0+60, message 136: “Train Position Report” is SENT. The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 6 | N/A | N/A | Train is moving (V_TRAIN=40; TIME=t0+120) | I | INT | - | N/A | N/A | - |
| 7 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | At t=t0+120, message 136: “Train Position Report” is SENT. The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 58 (T_CYCLOC=255; 0 < D_CYCLOC < 32767; M_LOC=0) is received | I | RTM | T=t0+160; D=d0 | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | Train is moving (V_TRAIN=40; TIME=t0+180) | I | INT | - | N/A | N/A | - |
| 11 | N/A | N/A | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | At time t=t0+180, no position report is sent | N/A | N/A | - |
| 12 | N/A | N/A | Train is moving (V_TRAIN=40; D=d0+D_CYCLOC) | I | INT | - | N/A | N/A | - |
| 13 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | At t=t0+120, message 136: “Train Position Report” is SENT. The position refers to the front end of the respective engine with regards to the train orientation. | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 60 | - |
| D_CYCLOC | 15 | 32767 | Not periodicaly in distance |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| Step 8: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 58 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| T_CYCLOC | 8 | 255 | - |
| D_CYCLOC | 15 | dx | 0 < dx < 32767 |
| M_LOC | 3 | 0 | Now |
| N_ITER | 5 | 0 | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The on-board equipment sends a position report after the train has reached standstill. |  | 22 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, RV L3: FS, LS, OS, SR, RV |  |  |
| Target of Test | The target of this test is to check that: • The train sends a position report after reaching standstill. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 a) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when train reaches standstill |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.2 Packet Number 1: Position Report based on two balise groups |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: • Speed value equal to zero is RECORDED. |
| Constraints | After reaching standstill, the train reports its position by means of: • Packet 0: “Position report”, as described in TC1 of Feature 184: The on-board equipment sends a position report by means of Packet 0: “Position report”. OR • Packet 1: “Position report based on two single balise groups“, as described in TC2 of Feature 184: The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 / 14 | FS / OS / SR / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 / 14 | FS / OS / SR / LS / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The train passes a linked Balise Group (BGa or SBGa) included in the linking information stored onboard. |  | 23 |
| Applicable Mode/Level Combinations | L0: UN, NL LNTC: TR, NL, SN L1: FS, LS, OS, SR, TR, PT, NL, RV L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report after a linked Balise Group passage. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 j) |  |
|  |  | Subset-026-4.5.2.1 table - … or Report train position at every passage of an LRBG compliant balise group |  |
|  |  | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Balise group (BGa or SBGa) Message is RECORDED. |
| Constraints | Packet 58: “Position Report Parameters” has not been previously received or they have been deleted. The linked Balise Group (BGa or SBGa) is composed of 1 or more balises. The train reports its position by means of Packet 0: “Position report” after passing a linked Balise Group (BGa or SBGa) when linking information is available onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Balise Group overpassed in Step 1 is included. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 255 (Q_LINK=1) is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.4 "The on-board equipment sends a position report by means of Packet 0: “Position report”" or Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Balise Group overpassed in Step 1 is included. |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | Position Reporting to the RBC |  | 3060500 |
| Test Case of Feature | The train passes two linked Single Balise Groups SBGa and SBGb but there is no linking information available onboard. |  | 24 |
| Applicable Mode/Level Combinations | L0: UN, NL LNTC: TR, NL, SN L1: FS, LS, OS, SR, TR, PT, NL, RV L2: FS, LS, OS, SR, TR, PT, NL, RV L3: FS, LS, OS, SR, TR, PT, NL, RV |  |  |
| Target of Test | The target of this test is to check that: • The on-board equipment sends a position report after passing two linked Single Balise Groups when no linking information is available onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 j) |  |
|  |  | Subset-026-4.5.2.1 table - … or Report train position at every passage of an LRBG compliant balise group |  |
|  |  | Subset-026-3.6.2.2.2 |  |
|  |  | Subset-026-3.6.2.2.2 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : • Single Balise group SBGa Message is RECORDED. • Single Balise group SBGb Message is RECORDED. • Message 136: “Train position report” is RECORDED. It shall include at least the following data: • Distance between the LRBG and the estimated front-end of the train (D_LRBG). • The confidence interval to this distance (L_DOUBTOVER and L_DOUBTUNDER). • Location reference Identity (NID_LRBG and NID_PRVBG). • The orientation in relation to the LRBG orientation (Q_DIRLRBG). • The position of the front-end in relation to the LRBG (Q_DLRBG). • Train speed (V_TRAIN). • Train integrity information (Q_LENGTH and L_TRAININT). • Direction of train movement in relation to the LRBG orientation. • Mode (M_MODE). • Level (M_LEVEL). |
| Constraints | Linking information is NOT available onboard. Packet 58: “Position Report Parameters” has not been previously received or they have been deleted. The train reports its position by means of Packet 1: “Position report based on two balise groups” after passing two linked Single Balise Groups (SBGa and SBGb) when no linking information is available onboard. The train does not change its orientation between these two balise groups. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| Linking | UNKNOWN | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 (Q_LINK=1) is received | I | BTM | The train overpasses a Single balise group SBGa | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |
| 4 | N/A | N/A | One Balise Group message (N_TOTAL=0) containing packet 255 (Q_LINK=1) is received | I | BTM | The train overpasses another Single balise group SBGb | N/A | N/A | - |
| 5 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Use_FT3060500.5 "The on-board equipment sends a position report by means of Packet 1: “Position report based on two balise groups”". | - | - | Message 136: “Train Position Report” is SENT. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | SBGa | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 0 | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | SBGb | Identity of the balise group |
| Q_LINK | 1 | 1 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / TR / PT / NL / LS / SN / RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |