| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3140300 (3.14.3: Reverse Movement Protection) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-3140300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 13.09.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.3: Reverse Movement Protection |  | 3140300 |
| Test Case of Feature | Reverse movement protection against given MA direction |  | 1 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / PT / RV L2: FS / LS / OS / SR / PT / RV L3: FS / LS / OS / SR / PT / RV |  |  |
| Target of Test | The target of test is to check that: • the brake command shall be triggered after a distance specified by the national value when a reverse movement is detected. • an indication is given to the driver showing when the RMP is commanding the brakes. • the brake command is released at standstill and after driver acknowledgement. • after the revocation of the brake command the RMP will be reinitialised using the current position of the train as the new reference location. • The function Reverse Movement Protection is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.5 |  |
|  |  | Subset-026-3.14.3.1 |  |
|  |  | Subset-026-3.14.3.2 |  |
|  |  | Subset-026-3.14.3.4 |  |
|  |  | Subset-026-3.14.3.5 |  |
|  |  | Subset-026-4.5.2.1 table - Reverse Movement Protection |  |
|  |  | Subset-026-4.7.2 table - Output information Brake reason |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn for Reverse Movement Protection |  |
|  |  | Subset-026-4.4.11.1.3 e) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • the brake indication is DISPLAYED • after releasing of the brake the indication of the brake is REMOVED • the driver acknowledgement is DISPLAYED and REMOVED after the driver has acknowledged Check on the TIU that: • the brake is APPLIED • the brake is RELEASED Check on the JRU that: • the brake intervention is RECORDED • the driver acknowledgement is RECORDED • the brake release is RECORDED |
| Constraints | The train had previously received a valid MA. For RV mode the train had previously received a valid MA, before it switched to RV mode. The MA is deleted on entering RV mode and the train is supposed to move in the direction opposite to the one that had been given by the former MA. If national values are not available the default value of 2m shall be considered for D_NVROLL. To check the re-initialisation of the reverse movement protection, this test case has to be done twice. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 12/ 14 | FS / OS / SR / PT / LS / RV |
| Movement Authority | STORED | Except for RV, SR, PT. The MA includes the permitted direction. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train moves in the opposite direction to the permitted one for more than a distance specified in the national value D_NVROLL. For the modes FS / LS / OS / SR this means, the train is moving backwards. In PT the “normally allowed movement” if backwards, thus in PT the train moves forwards. In RV the train moves forwards in the direction of the last valid MA, because in RV the “normally allowed movement” is backwards. | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | The brake command is triggered at Reverse Movement Protection (RMP) after a distance specified by the national value (default value = 2m) | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | The brake indication and the information that the RMP is commanding the brakes is DISPLAYED. | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | The brake intervention is RECORDED. | N/A | N/A | - |
| 5 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 6 | N/A | N/A | The request "Acknowledge Reverse Movement Protection" is displayed | O | DMI | - | N/A | N/A | - |
| 7 | N/A | N/A | The driver acknowledges | I | DMI | Ackn for Reverse Movement Protection | N/A | N/A | - |
| 8 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | The driver acknowledgement is RECORDED. | N/A | N/A | - |
| 9 | N/A | N/A | The request "Acknowledge brake command" is removed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | Service brake not commanded | O | TIU | Current mode is UNCHANGED | N/A | N/A | - |
| 11 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | The brake release is RECORDED. | N/A | N/A | - |
| 13 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train moves backwards 1 meter and the RMP does not command the brakes | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 12/ 14 | FS / OS / SR / PT / LS / RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT AFFECTED | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.14.3: Reverse Movement Protection |  | 3140300 |
| Test Case of Feature | Reverse movement protection against given MA direction |  | 2 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS / SR / PT / RV L2: FS / LS / OS / SR / PT / RV L3: FS / LS / OS / SR / PT / RV |  |  |
| Target of Test | The target of test is to check that: • the brake command shall be triggered after a distance specified by the national value when a reverse movement is detected. • an indication is given to the driver showing when the RMP is commanding the brakes. • the brake command is released at standstill and after driver acknowledgement. • a balise telegram has been received by the on-board equipment during reverse movement. • information received from balises during reverse movement are ignored. • The function Reverse Movement Protection is active |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.14.1.5 |  |
|  |  | Subset-026-3.14.3.1 |  |
|  |  | Subset-026-3.14.3.2 |  |
|  |  | Subset-026-3.14.3.4 |  |
|  |  | Subset-026-3.14.3.5 |  |
|  |  | Subset-026-3.14.3.6 |  |
|  |  | Subset-026-A.3.3.2 |  |
|  |  | Subset-026-4.5.2.1 table - Reverse Movement Protection |  |
|  |  | Subset-026-4.7.2 table - Output information Brake reason |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn for Reverse Movement Protection |  |
|  |  | Subset-026-4.8.2.1 |  |
|  |  | Subset-026-4.8.2.1 e) |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: • the brake indication is DISPLAYED • after releasing of the brake the indication of the brake is REMOVED • the driver acknowledgement is DISPLAYED and REMOVED after the driver has acknowledged • the system status message “Trackside malfunction” from the BG is NOT DISPLAYED Check on the TIU that: • the brake is APPLIED • the brake is RELEASED Check on the JRU that: • the brake intervention is RECORDED • the driver acknowledgement is RECORDED • the brake release is RECORDED • the balise telegram is RECORDED • the balise telegram is ignored |
| Constraints | The train had previously received a valid MA. For RV mode the train had previously received a valid MA, before it switched to RV mode. The MA is deleted on entering RV mode and the train is supposed to move in the direction opposite to the one that had been given by the former MA. If national values are not available the default value of 2m shall be considered for D_NVROLL. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 12/ 14 | FS / OS / SR / PT / LS / RV |
| Movement Authority | STORED | Except for RV, SR, PT. The MA includes the permitted direction. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is moving (V_TRAIN=40) | I | INT | The train moves in the opposite direction to the permitted one for more than a distance specified in the national value D_NVROLL. For the modes FS / LS / OS / SR this means, the train is moving backwards. In PT the “normally allowed movement” if backwards, thus in PT the train moves forwards. In RV the train moves forwards in the direction of the last valid MA, because in RV the “normally allowed movement” is backwards. | N/A | N/A | - |
| 2 | N/A | N/A | Service brake commanded | O | TIU | The brake command is triggered at Reverse Movement Protection (RMP) after a distance specified by the national value (default value = 2m) | N/A | N/A | - |
| 3 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 254 (), packet 255 () is received | I | BTM | The balise contains the default balise message as in the example in A.3.3.3. | N/A | N/A | - |
| 6 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT The system status message "Trackside malfunction" is displayed | O | DMI | The balise telegram has to be ignored during reverse movement. | N/A | N/A | - |
| 8 | N/A | N/A | Train is at standstill | I | INT | - | N/A | N/A | - |
| 9 | N/A | N/A | The request "Acknowledge brake command" is displayed | O | DMI | - | N/A | N/A | - |
| 10 | N/A | N/A | The driver acknowledges | I | DMI | Ackn for Reverse Movement Protection | N/A | N/A | - |
| 11 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The request "Acknowledge brake command" is removed | O | DMI | - | N/A | N/A | - |
| 13 | N/A | N/A | Service brake not commanded | O | TIU | Current mode is UNCHANGED | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | N/A | N/A | - |
| 15 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | Use_FT3040400.6: “3.4.4: Linking” | - | - | Testing the positive for A.3.3.2 and 4.8.2.1 e) by receiving the same BGas in Step 5 in forward movement. | N/A | N/A | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 254 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 8 / 12/ 14 | FS / OS / SR / PT / LS / RV |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT AFFECTED | - | RTM | - |
| NOT AFFECTED | - | TIU | - |
| NOT AFFECTED | - | DMI | - |
| NOT AFFECTED | - | BTM | - |
| NOT AFFECTED | - | INT | - |