| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080418 (4.8.4: Accepted Information depending on the modes - International SSP) Total: 19 Test case(s) |  |
| REF: | Subset-076-5-2-4080418 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.4 19.12.2013 | All | First version/review | DLR |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 28.10.2013 | All | General review | CEDEX |
| Version 2.9.3 29.11.2013 | TC 5 and 22 | Update of "Use_FT" steps | CEDEX |
| Version 2.9.4 09.12.2013 | TCs 2, 3, 4, 20 | Review | CEDEX |
| Version 2.9.5 10.12.2013 | All | General review | CEDEX |
| Version 2.9.6 19.12.2013 | All | Editorial review | CEDEX |
| Version 2.9.7 09.01.2014 | All | Update of step comment | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 14.01.2015 | All | Test cases renumbering (TC 7 has been deleted) | CEDEX |
| Version 3.0.0 10.03.2015 | All | Test cases renumbering (TC 7 has been deleted) | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The trackside generates and sends a message including packet 27(SSP) to a train. |  | 1 |
| Applicable Mode/Level Combinations | L2: SB, FS, LS, OS, SR, PT L3: SB, FS, LS, OS, SR, PT |  |  |
| Target of Test | The target of this test is to check that The message including packet 27 is accepted and stored on-board. The SSP is correctly calculated |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-3.6.3.2.2 a) |  |
|  |  | Subset-026-3.6.3.2.2 b) |  |
|  |  | Subset-026-3.6.3.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.2 d) |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Message including packet 27 SSP is recorded. The received SSP corresponds to site data. One of the last eight LRBG:s is used as reference for the SSP |
| Constraints | The value of V_STATIC given in packet 27 will have to be the MRSP after accepting the radio message. In PT mode the information “Recognition of Exit from TR mode” has to be received before A cab needs to be active and the Train Data must be valid (SB Mode) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS / OS / SR / SB / PT / LS | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 27 (D_STATIC=0;V_STATIC=<V1>), packet 21 is received | I | RTM | Complete message is accepted V1 will be the MRSP after accepting this message. | L2/L3 | FS / OS / SR / SB / PT / LS | - |
| 2 | L2/L3 | FS / OS / SR / SB / PT / LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Complete message is recorded | L2/L3 | FS / OS / SR / SB / PT / LS | - |
| 3 | L2/L3 | FS / OS / SR / SB / PT / LS | The Mode symbol "Full Supervision" is displayed | O | DMI | Only if not already in FS mode. | L2/L3 | FS | - |
| 4 | L2/L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | Only if not already in FS mode. | L2/L3 | FS | - |
| 5 | L2/L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is updated to the value V1. | L2/L3 | FS | - |
| 6 | L2/L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;V_PERM=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L2/L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The trackside generates and sends a message including packet 27(SSP) to a train which rejects the SSP because train data has been sent to the RBC and the acknowledgement of train data has not been received. |  | 2 |
| Applicable Mode/Level Combinations | L2: SB, FS, LS, OS, SR, PT L3: SB, FS, LS, OS, SR, PT |  |  |
| Target of Test | The target of this test is to check that Packet 27 is rejected |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Packet 27 SSP is recorded. |
| Constraints | For SB mode, only if valid Train Data are stored on-board and a cab is active. For PT mode, only if following the reception of the information “Recognition of Exit from TR mode” with a more recent time stamp |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | Validated train data | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 27 (D_STATIC=0;V_STATIC=<V1>), packet 21 is received | I | RTM | SSP is rejected. The value V1 will be the lowest one among all the values currently managed by the on-board equipment. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | SSP is recorded | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. Since the Permitted Speed has not changed (packet 27 was rejected), this message should not be recorded at this time. | N/A | N/A | - |

| Step 1: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 3: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |

| Step 5: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 8 / 12 | FS / OS / SR / SB / PT / LS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The trackside generates and sends a message including packet 27(SSP) to a train which accepts the SSP, although an emergency stop, that has been accepted, not has been revoked or deleted. |  | 3 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that Packet 27 is accepted on-board, when it is received between the reception of message 15 ( "Conditional Emergency Stop") and message 18 ("Revocation of Emergency Stop"). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.2.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-3.6.3.2.2 a) |  |
|  |  | Subset-026-3.6.3.2.2 b) |  |
|  |  | Subset-026-3.6.3.2.2 c) |  |
|  |  | Subset-026-3.6.3.2.2 d) |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Packet 27 SSP is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only if in OS mode. | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | - | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | The system status message "Emergency stop" is displayed | O | DMI | Only if Q_EMERGENCYSTOP=0,1 | N/A | N/A | - |
| 7 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | RTM | SSP is accepted. V1 will be the MRSP after accepting this message. | N/A | N/A | - |
| 8 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | SSP recorded | N/A | N/A | - |
| 9 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode: The PS changes. If in LS mode: The supervision status will change from NoS to OvS and V_TRAIN will be checked instead of V_PERM. | N/A | N/A | - |
| 10 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;V_PERM=<V1>) is recorded | O | JRU | If in FS or OS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. If in LS mode: The supervision status will change from NoS to OvS and V_TRAIN will be checked instead of V_PERM. | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) is received | I | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 4: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 7: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 11: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The onboard receives a message including packet 27(SSP). The message is REJECTED. |  | 4 |
| Applicable Mode/Level Combinations | L2: TR L3: TR |  |  |
| Target of Test | The target of this test is to check that Packet 27 Static Speed Profile is REJECTED. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Packet 27 SSP is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2 / L3 | TR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | RTM | SSP is rejected | L1 / L2 / L3 | TR | - |
| 2 | L1 / L2 / L3 | TR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | SSP recorded | L1 / L2 / L3 | TR | - |
| 3 | L2 / L3 | TR | Use_FT5110200.13 to change from TR mode to PT mode (Transition [7] in 4.6.3 table) | - | - | - | L2 / L3 | PT | - |
| 4 | L2 / L3 | PT | SA-DATA.Indication with Euroradio Message "Recognition of exit from TRIP mode" (NID_MESSAGE=6) is received | I | RTM | New Messages are accepted after ack. from RBC and release of EB. | L2 / L3 | PT | - |
| 5 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=6) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 6 | L2 / L3 | PT | The driver selects "Start" | I | DMI | To request RBC for a New MA for L2/L3 | L2 / L3 | PT | - |
| 7 | L2 / L3 | PT | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded | O | JRU | Start selected | L2 / L3 | PT | - |
| 8 | L2 / L3 | PT | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) is transmitted | O | RTM | - | L2 / L3 | PT | - |
| 9 | L2 / L3 | PT | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 10 | L2 / L3 | PT | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21 is received | I | RTM | No Packet 27 is included in this message, in order to check the rejection of this packet before. | L2 / L3 | PT | - |
| 11 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 12 | L2 / L3 | PT | Not the Mode symbol "Full Supervision" is displayed | O | DMI | - | L2 / L3 | PT | - |
| 13 | L2 / L3 | PT | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | - | L2 / L3 | PT | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| Step 4: Radio Message 6 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 6 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 8: Radio Message 132 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 132 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_MARQSTREASON | 5 | FINITE VALUE | - |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| ERTMS/ETCS level | 3 / 4 | L 2 / L 3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Transmission of Static Speed Profile via radio infill device and RECEIVED by the on-board. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | To verify that the Static Speed Profile transmitted via radio infill device is accepted by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-3.9.1.3 |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill location reference |  |
|  |  | Subset-026-8.7.15 Message 37: Infill MA |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-8.4.4.4 |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: The message sent from the RIU containing the packet 27 ‘International Static Speed Profile’ has been RECEIVED by on-board |
| Constraints | Before execution of test case the following preconditions have to be insured: The on-board system has be able to establish a radio communication session. The infill information for a certain main signal BG has been received by the on-board system. The main BG is not read when the train reaches the correct location, but no reaction occurs. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the main BG |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Infill MA" (NID_MESSAGE=37) containing packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | RTM | V1 will be the MRSP after the infill information is used on-board (after passing the location of the main BG). | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is updated to the value V1. | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed is equal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increase its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | N/A | N/A | - |

| Step 1: Radio Message 37 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 37 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The onboard equipment receives a speed profile (ISSP) from Radio. The message is REJECTED. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS OS, SR, LS LNTC: SN L0:UN |  |  |
| Target of Test | The target of this test is to check that the packet 27 is rejected. The onboard rejects the message, because no Level 2/3 transition order is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The message including packet 27 (ISSP) is recorded. The received Speed Profile corresponds to site data Check on the DMI that: The PS is not updated after changing to L2/3 |
| Constraints | No Level 2 / 3 transition order is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | No level transition to L2/3 is stored on-board. |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only for OS mode and if this information is not already displayed. | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 2 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | The speed and distance monitoring information is displayed | O | DMI | If not in LS mode: The Permitted Speed is higher than V1, which will be received afterwards. If in LS mode: The train speed is higher than V1. | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 3 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 27 (V_STATIC=V1; D_STATIC=0) is received | I | RTM | Complete message is rejected. V1 will be lower than the MRSP after changing to L2/3, but it will not be displayed. | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 4 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=24) is recorded | O | JRU | Complete message is recorded | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 5 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | USE_FT4060300.19 OR USE_FT5100100.36,37 | - | - | The Train changes to Level 2/3. If Previous Level is L0 or LNTC, the Mode also changes to FS. If Previous Level is L1, there is no change of Mode. | L2 / L3 | FS, SR, OS, LS | - |
| 6 | L2 / L3 | FS, SR, OS, LS | USE_FT4060300.21,46 OR USE_FT5190300.2 | - | - | Only if previous Mode is SR | L2 / L3 | FS, OS, LS | - |
| 7 | L2 / L3 | FS, OS, LS | The speed and distance monitoring information is displayed | O | DMI | If not in LS mode: The Permitted Speed is higher than V1, which was received at the beginning of this TC. If in LS mode: The train speed is higher than V1 and the train is in Normal Status. | L2 / L3 | FS, OS, LS | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | 0 | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Reception of telegram with Static Speed Profile packet (27). Message is ACCEPTED. |  | 8 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS, SR, SB |  |  |
| Target of Test | The target of this test is to check that The Balise telegram, including packet 27 is accepted and stored on-board. The SSP is correctly calculated |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Check on the JRU that The balise telegram including packet 27 (SSP )is recorded. The received SSP corresponds to site data The BG that sends the message is used as reference for the SSP |
| Constraints | A cab needs to be active and the Train Data must be valid (SB Mode). The value of V_STATIC given in packet 27 will have to be the MRSP after accepting the radio message. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 6 / 12 | FS / OS / SR / SB / LS |
| ERTMS/ETCS level | 2 | L 1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / OS / SR / SB / LS | One Balise Group message containing packet 12, packet 21, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | BTM | Complete message is accepted V1 will be the MRSP after accepting this message. | L1 | FS / OS / SR / SB / LS | - |
| 2 | L1 | FS / OS / SR / SB / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Complete message is recorded | L1 | FS / OS / SR / SB / LS | - |
| 3 | L1 | FS / OS / SR / SB / LS | The Mode symbol "Full Supervision" is displayed | O | DMI | Only if not already in FS mode. | L1 | FS | - |
| 4 | L1 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded | O | JRU | Only if not already in FS mode. | L1 | FS | - |
| 5 | L1 | FS | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is updated to the value V1. | L1 | FS | - |
| 6 | L1 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20;V_PERM=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Reception of a message including packet 27(SSP). The message is REJECTED because the level is L2/L3 and no transition to L1 has been received. |  | 9 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that The Balise telegram, including packet 27 is rejected. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : There are no changes Check on the JRU that : Check on the JRU that The balise telegram including packet 27 (SSP )is recorded. |
| Constraints | No level transition to L1 must be stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 4 / 12 / 13 | FS / OS / UN / LS / SN |
| Level Transition Order | NOT STORED | No level transition to L1 must be stored on-board. |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step: only for OS mode. The Target Speed and the Permitted Speed are requested, only if it is necessary, i.e., if it is not displayed already. | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 2 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | The speed and distance monitoring information is displayed | O | DMI | If not in LS mode: The Permitted Speed is higher than V1, which will be received afterwards. If in LS mode: The train speed is higher than V1. | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 3 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | One Balise Group message containing packet 27 (D_STATIC=0; V_STATIC=<V1>) is received | I | BTM | If the on-board is in L0(UN) or LNTC(SN), the packets 12 and 21 will also be received in this message. | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 4 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Complete message is recorded | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 5 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=2) is received | I | BTM | - | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 6 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 7 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / LS / OS / UN /SN | - |
| 8 | L1 | FS / LS / OS / UN /SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / LS / OS / UN /SN | - |
| 9 | L1 | FS / LS / OS | The speed and distance monitoring information is displayed | O | DMI | Only for FS or OS mode. The Target Speed is NOT updated to the value V1. | L1 | FS / LS / OS | - |
| 10 | L1 | FS / LS / OS | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS or OS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS / OS | - |
| 11 | L1 | FS / LS / OS | The speed and distance monitoring information is displayed | O | DMI | If in FS or OS mode: The Permitted Speed is NOT updated to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). If in LS mode: The train speed is higher than V1 and the train is in Normal Status. | L1 | FS / LS / OS | - |
| 12 | L1 | FS / LS / OS | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V1>) is recorded | O | JRU | Only for FS ot OS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS / OS | - |
| 13 | L1 | UN / SN | USE_FT4060300.28 for changing to TR mode | - | - | Only if the on-board is in L0(UN) or LNTC(SN). | L1 | TR | - |

| Step 3: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| - | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 7 / 12 | FS / OS / TR / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Transmission of Static Speed Profile via infill Balise Group and RECEIVED by the on-board. |  | 10 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | To verify that the Static Speed Profile transmitted via infill Balise Group is accepted by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill location reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Balise group message with packet 27 ‘International Static Speed Profile’ has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the main BG |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 136 (Infill location reference) and packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | BTM | Message is ACCEPTED. V1 will be the MRSP after the infill information is used on-board (after passing the location of the main BG). | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is updated to the value V1. | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed is equal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increases its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Reception of Static Speed Profile via infill Balise Group. The message is ACCEPTED, because a Level 1 transition order is stored onboard. |  | 11 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | To verify that the Static Speed Profile transmitted via infill Balise Group is accepted by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill location reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Balise group message with packet 27 ‘International Static Speed Profile’ has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 4 / 12 / 13 | FS / UN / LS / SN |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the main BG |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Level transtion order to L1 |
| ERTMS/ETCS level | 0 / 1 / 3 / 4 | L0 / LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is above the value V1, which will be received in the following message | L0 / LNTC / L2 / L3 | FS / LS / OS / UN /SN | - |
| 2 | L0 / LNTC / L2 / L3 | FS / LS / UN /SN | One Balise Group message containing packet 136 (Infill location reference) containing packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | BTM | Message is ACCEPTED. V1 will be the MRSP after the infill information is used on-board (after passing the location of the main BG). | L0 / LNTC / L2 / L3 | FS / LS / UN /SN | - |
| 3 | L0 / LNTC / L2 / L3 | FS / LS / UN /SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC / L2 / L3 | FS / LS / UN /SN | - |
| 4 | L0 / LNTC / L2 / L3 | FS / LS / UN /SN | The Level symbol "Level 1" is displayed | O | DMI | The train reaches the previously announced location. | L1 | FS / LS | - |
| 5 | L1 | FS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | The train reaches the previously announced location. | L1 | FS / LS | - |
| 6 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is updated to the value V1. | L1 | FS / LS | - |
| 7 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS | - |
| 8 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed is equal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increases its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | L1 | FS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Reception of Static Speed Profile via infill Balise Group. The message is REJECTED, because the current mode is not FS nor LS. |  | 12 |
| Applicable Mode/Level Combinations | L1: OS |  |  |
| Target of Test | To verify that the Static Speed Profile transmitted via infill Balise Group is rejected by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill location reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Balise group message with packet 27 ‘International Static Speed Profile’ has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the BG that will act as the main BG |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. The Target Speed and the Permitted Speed are requested, only if it is necessary, i.e., if it is not displayed already. | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is above the value V1, which will be received in the following message | N/A | N/A | - |
| 3 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 136, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | BTM | - | N/A | N/A | - |
| 4 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target Speed is NOT updated to the value V1. | N/A | N/A | - |
| 6 | N/A | N/A | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is NOT updated to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). | N/A | N/A | - |
| 8 | N/A | N/A | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Reception of Static Speed Profile via infill Balise Group. The message is REJECTED, because there is no L1 transition order stored on-board. |  | 13 |
| Applicable Mode/Level Combinations | L2: FS, LS L3: FS, LS |  |  |
| Target of Test | To verify that the Static Speed Profile transmitted via infill Balise Group is rejected by the on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.6.3.2.2 |  |
|  |  | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill location reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Balise group message with packet 27 ‘International Static Speed Profile’ has been RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the BG that will act as the main BG |
| Level Transition Order | NOT STORED | To level 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | If not in LS mode: The Permitted Speed is higher than V1, which will be received afterwards. If in LS mode: The train speed is higher than V1. | L2 / L3 | FS / LS | - |
| 2 | L2 / L3 | FS / LS | One Balise Group message (N_TOTAL=1) containing packet 136, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | BTM | - | L2 / L3 | FS / LS | - |
| 3 | L2 / L3 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / LS | - |
| 4 | L2 / L3 | FS / LS | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=2; Q_LINK=0) is received | I | BTM | This is an unlinked BG | L2 / L3 | FS / LS | - |
| 5 | L2 / L3 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / LS | - |
| 6 | L2 / L3 | FS / LS | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / LS | - |
| 7 | L1 | FS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / LS | - |
| 8 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is NOT updated to the value V1. | L1 | FS / LS | - |
| 9 | L1 | FS / LS | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS | - |
| 10 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | If in FS mode: The Permitted Speed is NOT updated to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). If in LS mode: The train speed is higher than V1 and the train is in Normal Status. | L1 | FS / LS | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | 0 | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The train receives a message including packet 27 (SSP) that is transmitted via Euroloop. The message is accepted, because a level 1 transition order is stored onboard |  | 14 |
| Applicable Mode/Level Combinations | L2: FS, LS L3: FS, LS |  |  |
| Target of Test | The target of this test is to check that The Euroloop telegram, including packet 27 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill Location Reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : Check on the JRU that The Euroloop telegram including packet 27 (SSP) is recorded. The received SSP corresponds to site data One balise from LRBG (EOLM) is used as reference for the SSP |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the main BG |
| Level Transition Order | STORED | To level 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS | One Euroloop message containing packet 136, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | LTM | The message is ACCEPTED | L2 / L3 | FS / LS | - |
| 2 | L2 / L3 | FS / LS | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L2 / L3 | FS / LS | - |
| 3 | L2 / L3 | FS / LS | The Level symbol "Level 1" is displayed | O | DMI | The train reaches the previously announced location. | L1 | FS / LS | - |
| 4 | L1 | FS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | The train reaches the previously announced location. | L1 | FS / LS | - |
| 5 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is updated to the value V1. | L1 | FS / LS | - |
| 6 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS | - |
| 7 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed isequal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increases its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | L1 | FS / LS | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The train receives a message including packet 27 (SSP) that is transmitted via Euroloop. The message is REJECTED, because the current mode is not FS nor LS. |  | 15 |
| Applicable Mode/Level Combinations | L1: OS |  |  |
| Target of Test | The target of this test is to check that The Euroloop telegram, including packet 27 is rejected and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill Location Reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The Euroloop telegram including packet 27 (SSP) is recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the BG that will act as the main BG |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Optional step. The Target Speed and the Permitted Speed are requested, only if it is necessary, i.e., if it is not displayed already. | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is above the value V1, which will be received in the following message | N/A | N/A | - |
| 3 | N/A | N/A | One Euroloop message containing packet 136, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | LTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Target Speed is NOT updated to the value V1. | N/A | N/A | - |
| 6 | N/A | N/A | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |
| 7 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is NOT updated to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). | N/A | N/A | - |
| 8 | N/A | N/A | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=<V1>) is recorded | O | JRU | This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | N/A | N/A | - |

| Step 3: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The train receives a message including packet 27 (SSP) that is transmitted via Euroloop. The message is REJECTED, because there is no L1 transition order stored on-board. |  | 16 |
| Applicable Mode/Level Combinations | L2: FS, LS L3: FS, LS |  |  |
| Target of Test | The target of this test is to check that The Euroloop telegram, including packet 27 is rejected and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill Location Reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU: Check on the JRU that The Euroloop telegram including packet 27 (SSP) is recorded. Check that on the DMI: Internation SSP packet is not used and displayed. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the BG that will act as the main BG |
| Level Transition Order | NOT STORED | To level 1 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | If in FS mode: The Permitted Speed is higher than V1, which will be received afterwards. If in LS mode: The train speed is higher than V1. | L2 / L3 | FS / LS | - |
| 2 | L2 / L3 | FS / LS | One Euroloop message containing packet 136, packet 27 (D_STATIC=0; V_STATIC=<V1>) is received | I | LTM | - | L2 / L3 | FS / LS | - |
| 3 | L2 / L3 | FS / LS | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L2 / L3 | FS / LS | - |
| 4 | L2 / L3 | FS / LS | One Balise Group message containing packet 41 (D_LEVELTR=0; M_LEVELTR=2) is received | I | BTM | - | L2 / L3 | FS / LS | - |
| 5 | L2 / L3 | FS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS / LS | - |
| 6 | L2 / L3 | FS / LS | The Level symbol "Level 1" is displayed | O | DMI | - | L1 | FS / LS | - |
| 7 | L1 | FS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | - | L1 | FS / LS | - |
| 8 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode. The Target Speed is NOT updated to the value V1. | L1 | FS / LS | - |
| 9 | L1 | FS / LS | Not SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode. This message would be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS | - |
| 10 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed is equal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increases its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | L1 | FS / LS | - |

| Step 2: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 2 | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | Intentionally deleted |  | Intentionally deleted |
| Applicable Mode/Level Combinations | Intentionally deleted |  |  |
| Target of Test | Intentionally deleted |  |  |
| Version | Intentionally deleted | Intentionally deleted |  |
| Author | Intentionally deleted |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Intentionally deleted |  |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | A message including packet 27 is transmitted from RBC and is received by the onboard equipment The onboard ACCEPTS the message, because a Level 2 /3 transition order is stored onboard. |  | 18 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS, SR, LS, OS |  |  |
| Target of Test | The target of this test is to check that the packet 27 is accepted and stored on-board. The onboard ACCEPTS the message, because a Level 2/3 transition order is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The message including packet 27 (ISSP) is recorded. The received Speed Profile corresponds to site data |
| Constraints | A Level 2 / 3 transition order is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 13 | FS / OS / SR / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | A level transition to L2/3 is stored on-board. |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27 (V_STATIC=<V1>) is received | I | RTM | Complete message is accepted V1 will be the MRSP after changing to L2/3 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 2 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=3) is recorded | O | JRU | Complete message is recorded | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | - |
| 3 | L0/LNTC/ L1 | FS, OS, SR, UN, LS, SN | USE_FT4060300.19 OR USE_FT5100100.36,37 | - | - | The Train changes to Level 2/3. If Previous Level is L0 or LNTC, the Mode also changes to FS. If Previous Level is L1, there is no change of Mode. | L2 / L3 | FS, SR, OS, LS | - |
| 4 | L2 / L3 | FS, SR, OS, LS | USE_FT4060300.21,46 OR USE_FT5190300.2 | - | - | Only if previous Mode is SR | L2 / L3 | FS, OS, LS | - |
| 5 | L2 / L3 | FS, SR, OS, LS | USE_FT4060300.23 OR USE_FT4060300.59 | - | - | Only if previous Mode is OS or LS | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | The Permitted Speed is V1, which was received at the beginning of this TC. | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM=V1) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | Both | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - International SSP |  | 4080418 |
| Test Case of Feature | The train receives a message including packet 27 (SSP) that is transmitted via Euroloop. The message is accepted due to the current level (L1) |  | 19 |
| Applicable Mode/Level Combinations | L1: FS, LS |  |  |
| Target of Test | The target of this test is to check that The Euroloop telegram, including packet 27 is accepted and stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 International SSP |  |
|  |  | Subset-026-4.8.3.1.1 Infill Location Reference |  |
|  |  | Subset-026-4.8.4.1.1 |  |
|  |  | Subset-026-4.8.4.2 International SSP |  |
|  |  | Subset-026-4.8.4.2 Infill Location Reference |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.32 Packet Number 136: Infill location reference |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The Euroloop telegram including packet 27 (SSP) is recorded. The received SSP corresponds to site data One balise from LRBG (EOLM) is used as reference for the SSP |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| Linking | STORED | Q_LINKREACTION=10 (No reaction) for the main BG |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS / LS | One Euroloop message containing packet 136, packet 27 (D_STATIC=0;V_STATIC=<V1>) is received | I | LTM | The message is ACCEPTED | L1 | FS / LS | - |
| 2 | L1 | FS / LS | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7) is recorded | O | JRU | - | L1 | FS / LS | - |
| 3 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Target Speed is updated to the value V1. | L1 | FS / LS | - |
| 4 | L1 | FS / LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET=<V1>) is recorded | O | JRU | Only if in FS mode: This message will be recorded when any of the speed and distance monitoring information changes, according to SUBSET-027v300. | L1 | FS / LS | - |
| 5 | L1 | FS / LS | The speed and distance monitoring information is displayed | O | DMI | Only if in FS mode: The Permitted Speed isequal to the value V1, when the train reaches the location of the previously announced main BG (the main BG is not read, but no reaction occurs). Only if in LS mode: The train increases its speed until it overpasses the new PS and the supervision status changes to Overspeed Status. | L1 | FS / LS | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Defines the type of media: Loop (1) |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 136 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY = 1. |
| NID_BG | 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 12 | FS / LS |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |