| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5160000 (5.16 : Procedure passing a non protected Level Crossing) Total: 7 Test case(s) |  |
| REF: | Subset-076-5-2-5160000 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 03.10.2013 | TC1, TC2, TC3, TC4 | Modifications requested by issue 10 of Review001_Subset076-5-2-5160000_v291 | MULTITEL |
| Version 2.9.3 28.10.2013 | All | Response to Review001_Subset-076-5-2-5160000_v291 | MULTITEL |
| Version 2.9.4 10.12.2013 | TC2 | Response to issue 4 from Review003_Subset-076-5-2-5160000_v293 | MULTITEL |
| Version 2.9.5 20.12.2013 | TC1 | Add a second packet 88 to cover 8.4.1.4.5 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | The train receives two packets 88 related to non protected LX. When approaching a non protected LX, the on-board equipment shall inform the driver about the status of the LX, as soon as either the EOA or the SvL related to the LX start location becomes the Most Relevant Displayed Target (see 3.13.10.4.2). |  | 1 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The on-board equipment manages messages with several occurrences packet 88. The on-board equipment shall inform the driver about the status of the LX, as soon as either the EOA or the SvL related to the LX start location becomes the Most Relevant Displayed Target |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.4 |  |
|  |  | Subset-026-5.16.1.4 a) |  |
|  |  | Subset-026-4.7.2 table - Output information LX status "not protected" |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The on-board equipment receives two packets 88. The driver is informed of the status of the LX as soon as either the EOA or the SvL related to the LX start location becomes the Most Relevant Displayed Target. |
| Constraints | The Most Relevant Displayed Target is related to the current MA. If in LS mode, the LSSMA is displayed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | If in FS mode, the Permitted Speed related to the current MA is displayed If in OS, the driver has requested to show supervision limits If in LS mode, the LSSMA is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | One Balise Group message containing packet 88 (NID_LX=<NID_LX_1>; Q_LXSTATUS=1), packet 88 (NID_LX=<NID_LX_2 different from NID_LX_1>; Q_LXSTATUS=1) is received. | I | BTM | The train receives two packets 88 about not protected LX located before the end of the MA. | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=88; NID_LX=<NID_LX_1>; Q_LXSTATUS=1; NID_PACKET=88; NID_LX=<NID_LX_2 different from NID_LX_1>; Q_LXSTATUS=1) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | First LX (NID_LX=NID_LX_1) The ERTMS/ETCS on-board equipment shall temporarily supervise the LX start location as both the EOA and SvL (instead of the EOA/LOA and SvL derived from the MA), with no release speed. The EOA or the SvL related to the LX start location becomes the Most Relevant Displayed Target. The Permitted Speed or LSSMA is reduced to reflect the new Most Relevant Displayed Target. | N/A | N/A | - |
| 5 | N/A | N/A | The symbol „LX not protected“ (LX01) is displayed | O | DMI | First LX (NID_LX=NID_LX_1) The on-board equipment shall inform the driver about the status of the LX | N/A | N/A | - |
| 6 | N/A | N/A | The “DMI SYMBOL STATUS” message is RECORDED with the Bit 86 (LX01) = 1 (displayed) | O | JRU | First LX (NID_LX=NID_LX_1) | N/A | N/A | - |
| 7 | N/A | N/A | Use_FT5160000.4 | - | - | First LX (NID_LX=NID_LX_1) Indication given to the driver shall remain displayed until the train has passed the LX end location with its min safe front end. | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Second LX (NID_LX=NID_LX_2) The ERTMS/ETCS on-board equipment shall temporarily supervise the LX start location as both the EOA and SvL (instead of the EOA/LOA and SvL derived from the MA), with no release speed. The EOA or the SvL related to the LX start location becomes the Most Relevant Displayed Target. The Permitted Speed or LSSMA is reduced to reflect the new Most Relevant Displayed Target. | N/A | N/A | - |
| 9 | N/A | N/A | The symbol „LX not protected“ (LX01) is displayed | O | DMI | Second LX (NID_LX=NID_LX_2) The on-board equipment shall inform the driver about the status of the LX | N/A | N/A | - |
| 10 | N/A | N/A | The “DMI SYMBOL STATUS” message is RECORDED with the Bit 86 (LX01) = 1 (displayed) | O | JRU | Second LX (NID_LX=NID_LX_2) | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | NID_LX_1 |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | Not protected LX |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 88 | Second packet 88 |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | NID_LX_2 different from NID_LX_1 |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | Not protected LX |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01) is displayed | O | DMI | Driver is informed of the status of the second LX (NID_LX=NID_LX_2) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | When approaching a non protected LX, the on-board equipment shall inform the driver about the status of the LX, as soon as the ERTMS/ETCS on-board equipment substitutes the supervision of the LX start location as both the EOA and SvL by the inclusion of the LX speed restriction in the MRSP. |  | 2 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: the on-board equipment shall inform the driver about the status of the LX, as soon as the ERTMS/ETCS on-board equipment substitutes the supervision of the LX start location as both the EOA and SvL by the inclusion of the LX speed restriction in the MRSP. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.4 |  |
|  |  | Subset-026-5.16.1.4 b) |  |
|  |  | Subset-026-4.7.2 table - Output information LX status "not protected" |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The driver is informed of the status of the LX as soon as the ERTMS/ETCS on-board equipment substitutes the supervision of the LX start location as both the EOA and SvL by the inclusion of the LX speed restriction in the MRSP. |
| Constraints | A packet 88 about a not protected LX located before the end of the current MA has been received. Before reaching the stopping area, V_PERM (or LSSMA) > V_TRAIN > V_LX. This message requires to stop the train in rear of the non protected LX. L_STOPLX is high enough to ensure that neither the EOA nor the SvL related to the LX start location have become the Most Relevant Displayed Target (see 3.13.10.4.2). If in LS mode, the LSSMA is displayed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected Q_STOPLX=1 Stop required |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01) is not displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The EOA nor the SvL related to the LX start location have not yet become the Most Relevant Displayed Target (see 3.13.10.4.2). If in OS, the driver has requested to show supervision limits. If in LS mode, the LSSMA is displayed. If in FS mode, the Permitted Speed related to the current MA is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | The train stopped with its estimated front end inside the stopping area (D_LX - L_STOPLX , D_LX) V_TRAIN = 0 | O | DMI | The ERTMS/ETCS on-board equipment substitutes the supervision of the LX start location as both the EOA and SvL by the inclusion of the LX speed restriction in the MRSP | N/A | N/A | - |
| 3 | N/A | N/A | The symbol „LX not protected“ (LX01) is displayed | O | DMI | The on-board equipment shall inform the driver about the status of the LX | N/A | N/A | - |
| 4 | N/A | N/A | The “DMI SYMBOL STATUS” message is RECORDED with the Bit 86 (LX01) = 1 (displayed) | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The MRSP is updated to V_LX | N/A | N/A | - |
| 6 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | with V_PERM = V_LX | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01) is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | Indication given to the driver shall remain displayed until information “LX is protected” is received on-board. |  | 3 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: Indication given to the driver shall remain displayed until information “LX is protected” is received on-board |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.5 |  |
|  |  | Subset-026-5.16.1.5 a) |  |
|  |  | Subset-026-4.7.2 table - Output information LX status "not protected" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The LX status is no longer displayed after the information “LX is protected” is received on-board. |
| Constraints | The train is approaching a non protected Level Crossing and the DMI informs the driver about the LX status. The Protected LX described in packet 88 has the NID_LX than the not protected one in order to revoke it. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01)is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 88 (Q_LXSTATUS=0) is received. | I | BTM | The train receives a packet 88 about a protected LX | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The symbol „LX not protected“ (LX01) is NOT displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The “DMI SYMBOL STATUS” message is RECORDED with the Bit 86 (LX01) = 0 (not displayed) | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 0 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 88 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 0 | Protected LX |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 1 | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01)is no longer displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | Indication given to the driver shall remain displayed until the train has passed the LX end location with its min safe front end. |  | 4 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: Indication given to the driver shall remain displayed until the train has passed the LX end location with its min safe front end. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.5 |  |
|  |  | Subset-026-5.16.1.5 b) |  |
|  |  | Subset-026-4.7.2 table - Output information LX status "not protected" |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The LX status is no longer displayed after the train has passed the LX end location with its min safe front end. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01)is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The symbol „LX not protected“ (LX01) is NOT displayed any more | O | DMI | Train has passed the LX end location with its min safe front end. | N/A | N/A | - |
| 2 | N/A | N/A | The “DMI SYMBOL STATUS” message is RECORDED with the Bit 86 (LX01) = 0 (not displayed) | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The symbol „LX not protected“ (LX01) is no longer displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | The train passes a non protected Level Crossing that requires to stop in rear of the LX start location. |  | 5 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The train stops in rear of the LX start location as required. The MRSP is updated to V_LX. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.2 |  |
|  |  | Subset-026-5.16.1.3 |  |
|  |  | Subset-026-5.16.2.1 |  |
|  |  | Subset-026-4.5.2.1 table - LX speed |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check the speed of the train inside the stopping area (D_LX - L_STOPLX , D_LX) and the update of V_PERM after the stop. |
| Constraints | A packet 88 about a not protected LX has been received. This message requires to stop the train in rear of the non protected LX. If in LS, V_LX lower than V_NVLIMSUPERV (default value = 100 km/h) and V_MAXTRAIN and the LSSMA is displayed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected Q_STOPLX=1 Stop required |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train approaches the LX start location, the MRSP decreases as the ERTMS/ETCS on-board equipment supervises the LX start location as both the EOA and SvL with no release speed. If in FS mode, the Permitted Speed is displayed If in OS, the driver has requested to show supervision limits If in LS mode, the LSSMA is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | Train is at standstill (V_TRAIN=0) | I | INT | The train stopped with its estimated front end inside the stopping area (D_LX - L_STOPLX , D_LX) | N/A | N/A | - |
| 3 | N/A | N/A | GENERAL MESSAGE (NID_MESSAGE_JRU=1, V_TRAIN=0) is recorded | O | JRU | The stop of the train is RECORDED | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The ERTMS/ETCS on-board equipment shall no longer supervise the LX start location as both the EOA and SvL (i.e. the EOA/LOA and SvL derived from the MA shall be again supervised) and shall immediately include the LX speed restriction in the MRSP, starting from the estimated train front end The MRSP (permitted speed or LSSMA, depending of the mode) is updated to V_LX | N/A | N/A | - |
| 5 | N/A | N/A | The “SPEED AND DISTANCE MONITORING INFORMATION” message is RECORDED | O | JRU | V_PERM = V_LX | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |
| Level Crossing information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | O | INT | V_TRAIN=0 |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | The train passes a non protected Level Crossing that doesn’t require to stop in rear of the LX start location. |  | 6 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: When the train reaches the location of the braking to target Permitted speed supervision limit calculated for the LX speed, the MRSP is updated to V_LX |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.5.2.1 table - LX speed |  |
|  |  | Subset-026-4.5.2.1 table - Speed and Distance Monitoring based on MRSP, MA, release speed, gradient, mode profile, non protected LX start location, and route unsuitability location |  |
|  |  | Subset-026-5.16.1.1 |  |
|  |  | Subset-026-5.16.1.2 |  |
|  |  | Subset-026-5.16.1.3 |  |
|  |  | Subset-026-5.16.3.1 |  |
|  |  | Subset-026-5.16.3.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check V_PERM stops decreasing at the location of the braking to target speed supervision limit calculated for the LX speed. The test case shall consider the train data, the configuration of the special brakes, service brake and traction cut-off interfaces which are not affected by the Train Data acquisition, the service brake application and the special brakes statuses which are continuously acquired on the Train Interface, the fixed values and the National Values (either a valid Packet 3 or the Default Values are used) as preconditions to be known before setting the appropriate values for the rest of the trackside info and the speed profile. The required observables (speed supervision limits) should be checked using a suitable tool that performs fully the “speed and distance monitoring" functionality (by default use ERA Braking curves tool, which can be downloaded from this website: http://www.era.europa.eu/Core-Activities/ERTMS/Pages/Braking-Curves-Simulation-Tool.aspx) |
| Constraints | A packet 88 about a not protected LX has been received. This message doesn’t require to stop the train in rear of the non protected LX. The train approaches the LX with V_TRAIN < V_PERM . If in LS, V_LX lower than V_NVLIMSUPERV (default value = 100 km/h) and V_MAXTRAIN and the LSSMA is displayed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected Q_STOPLX=0 No stop required |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only in OS (The Permitted Speed and the Target Distance shall be displayed until the driver requests to stop this. See the req. 4.4.12.1.4) | N/A | N/A | - |
| 2 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train approaches the LX start location, the MRSP decreases as the ERTMS/ETCS on-board equipment supervises the LX start location as both the EOA and SvL with no release speed. If in FS mode, the Permitted Speed related to the current MA is displayed If in OS, the driver has requested to show supervision limits If in LS mode, the LSSMA is displayed. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The DMI aspect is coherent with Indication Status in a Target speed Monitoring section | N/A | N/A | - |
| 4 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train reaches with the estimated or the max safe front end (depending whether the SBI at LX speed is the SBI1 or the SBI2) the location of the braking to target Permitted speed supervision limit calculated for the LX speed with V_TRAIN < V_PERM The Permitted Speed stops decreasing. V_PERM remains set to the value V_LX The ERTMS/ETCS on-board equipment shall no longer supervise the LX start location as both the EOA and SvL and shall immediately include the LX speed restriction in the MRSP | N/A | N/A | - |
| 5 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE= 0; M_SDMSUPSTAT=0; V_PERM=<V_LX>; D_TARGET=<the EOA/LOA and the SvL (if any) derived from the MA>) | O | JRU | V_PERM = V_LX The DMI aspect is changed to Ceiling speed Monitoring section (see the Condition [3] in 3.13.10.6.1). The Target Distance (for OS and FS mode only) changes: the EOA/LOA and the SvL (if any) derived from the MA shall be again supervised | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |
| Level Crossing information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.16 : Procedure passing a non protected Level Crossing |  | 5160000 |
| Test Case of Feature | The train reaches a LX end location. |  | 7 |
| Applicable Mode/Level Combinations | L1: FS, LS, OS L2: FS, LS, OS L3: FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that: The end location of the LX speed restriction shall be the LX end location. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-5.16.1.3 |  |
|  |  | Subset-026-4.5.2.1 table - LX speed |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Test the update of the MRSP after the LX end location. |
| Constraints | After the LX end location, the MRSP is different from V_LX. If in LS mode, the LSSMA is displayed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| National Values | NOT AVAILABLE | No national values are received or available on-board. |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Level Crossing information | STORED | Q_LXSTATUS=1 LX is not protected V_PERM=V_LX |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has NOT passed the LX end location, the MRSP is V_LX. If in FS mode, the Permitted Speed related to the current MA is displayed If in OS, the driver has requested to show supervision limits If in LS mode, the LSSMA is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | The “SPEED AND DISTANCE MONITORING INFORMATION” message is RECORDED | O | JRU | V_PERM=V_LX | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed the LX end location. V_LX is no longer valid, the MRSP is updated with V_PERM different from V_LX. | N/A | N/A | - |
| 4 | N/A | N/A | The “SPEED AND DISTANCE MONITORING INFORMATION” message is RECORDED with the V_PERM variable different from V_LX. | O | JRU | V_PERM is not equal to V_LX | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |