| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3110800 (3.11.8: Train related speed restriction) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-3110800 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 09.07.2013 | All | Second version/ Step 5 from TC1 changed; UNISIG review comments from 25.06.2013 included. | INECO |
| Version 2.9.2 01.08.2013 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 2.9.3 20.12.2013 | All | Fourth version/ Dictionary description and Requirements removed; Corrections according to review sheet 008. | INECO |
| Version 2.9.4 23.12.2013 | All | Fifth version/ Corrections according to review sheet 009; Cross reference check; Old reference removed. | INECO |
| Version 2.9.5 09.01.2013 | All | Sixth version/ New steps added for monitoring speed restrictions. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.11.8: Train related speed restriction |  | 3110800 |
| Test Case of Feature | It is possible to define the maximum train speed related to the actual performance and configuration of the train. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN L1: FS / LS / OS / SR / RV L2: FS / LS / OS / SR / RV L3: FS / LS / OS / SR / RV |  |  |
| Target of Test | The target of this test is to check that: Train comes to standstill. The driver modifies the maximum train speed. The maximum train speed becomes the permitted speed Train is moving again. The supervision with the exceeding of V_MAXTRAIN which should cause an application of brakes. The “Train related speed restriction” function is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.11.8.1 |  |
|  |  | Subset-026-3.13.2.2.13.1 |  |
|  |  | Subset-026-3.18.3.2.3 |  |
|  |  | Subset-026-A.3.11 Train Data: Maximum train speed |  |
|  |  | Subset-026-4.5.2.1 table - Train related speed restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The driver modifies the maximum train speed at standstill. Permitted speed displayed on driver request (only for OS and SR modes) and equal to Train related speed restriction, V_MAXTRAIN is displayed. (not possible to be checked in LS mode). The speed and distance monitoring information is displayed (Intervention status) Status symbol “ST01” is displayed and removed Check on the JRU that: TRAIN DATA CHANGED is recorded. TYPE OF TRAIN DATA is recorded. Permitted speed equal to train related speed restriction, V_MAXTRAIN is recorded.(not recorded in LS mode) SPEED MONITORING INFORMATION is recorded Exceeding of speed limit V_ MAXTRAIN is recorded. Service brake commanded state (commanded and release) is recorded. |
| Constraints | Train is moving and the actual V_MAXTRAIN is not the MRSP. At standstill, when the driver modifies the value of V_MAXTRAIN, V_MAXTRAIN becomes the most restrictive speed. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 14 | FS / OS / SR / UN / LS / RV |
| Train Data | STORED | V_MAXTRAIN different from V_MRSP |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L 0 / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Reversing Area Information | STORED | Only applicable for RV mode |
| Reversing Supervision Information | STORED | Only applicable for RV mode |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | V_TRAIN |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train is at standstill. | I | INT | V_TRAIN=0 | N/A | N/A | - |
| 2 | N/A | N/A | The driver selects “Main”. | I | DMI | - | N/A | N/A | - |
| 3 | N/A | N/A | The <Main> window is displayed. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The driver selects "Train Data" window. | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 0100) is recorded | O | JRU | Train Data Entry Request | N/A | N/A | - |
| 6 | N/A | N/A | The driver modifies “Maximum Speed “and validates the "Flexible Train Data" | I | DMI | V_MAXTRAIN becomes the most restrictive speed limit Characteristics about data unit, range and resolution are described in Table A.3.11 | N/A | N/A | - |
| 7 | N/A | N/A | The “Train data validation” window is displayed | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver validates the Train Data | I | DMI | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered | N/A | N/A | - |
| 9 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 0101) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | When train data are changed | N/A | N/A | - |
| 11 | N/A | N/A | TYPE OF TRAIN DATA (NID_MESSAGE_JRU=41; M_TRAIN_DATA_ENTRY=1) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed. | I | DMI | Optional step. Applies only if the applicable mode is OS/ SR | N/A | N/A | - |
| 13 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0001 1011) is recorded | O | JRU | Optional step. Applies only if the applicable mode is OS / SR | N/A | N/A | - |
| 14 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN < V_MAXTRAIN. If in FS, OS , SR or RV mode ; Permitted Speed is updated with the value of V_MAXTRAIN (N/A in UN mode) Status requested: Normal status | N/A | N/A | - |
| 15 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = V_MAXTRAIN; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded. | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 16 | N/A | N/A | Train is moving. | I | INT | - | N/A | N/A | - |
| 17 | N/A | N/A | The speed and distance monitoring information is displayed. | O | DMI | V_TRAIN >V_MAXTRAIN. Status requested: Intervention status | N/A | N/A | - |
| 18 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = V_MAXTRAIN; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded. | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) | N/A | N/A | - |
| 19 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 20 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 21 | N/A | N/A | The Status symbol "Service Brake is displayed. | O | DMI | Start DMI Object: ST01 | N/A | N/A | - |
| 22 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 23 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | V_TRAIN < V_MAXTRAIN. Status requested: Normal status | N/A | N/A | - |
| 24 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = V_MAXTRAIN; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded | O | JRU | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) | N/A | N/A | - |
| 25 | N/A | N/A | Service brake not commanded | O | TIU | - | N/A | N/A | - |
| 26 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded | O | JRU | - | N/A | N/A | - |
| 27 | N/A | N/A | The Status symbol "Service Brake is removed. | O | DMI | Stop DMI Object: ST01 | N/A | N/A | - |
| 28 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded | O | JRU | - | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 12 / 14 | FS / OS / SR / UN / LS / RV |
| Train Data | STORED | CHANGED: V_MAXTRAIN is the permitted speed |
| ERTMS/ETCS level | 0 / 2 / 3 / 4 | L 0 / L1 / L2 / L3 |
| Driver ID | VALID | - |
| Reversing Area Information | STORED | Only applicable for RV mode |
| Reversing Supervision Information | STORED | Only applicable for RV mode |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake not commanded | I | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | CSM: V_TRAIN < V_MAXTRAIN. Normal Status |
| NOT RELEVANT | - | BTM | - |
| Train is moving. | I | INT | - |