| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080300 (4.8.3: Accepted information depending on the level and transmission media) Total: 52 Test case(s) |  |
| REF: | Subset-076-5-2-4080300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 31.03.2013 | All | First version/review for Baseline 3 | INECO/CEDEX |
| Version 2.9.1 01.08.2013 | All | Second version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO/CEDEX |
| Version 2.9.2 15.11.2013 | All | Third version/ UNISIG Review from October 2013 included; TCEditor_v31 tool used | INECO |
| Version 2.9.3 09.12.2013 | All | Forth version / Modified after bilateral meeting; Review sheet 004 from 02.01.2014; FT renumbered. | INECO |
| Version 2.9.4 02.01.2014 | All | Fifth version/ Internal version: Cross-reference checked; Old reference removed. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The SR distance information from loop shall be accepted by on-board equipment according to 4.8.3 table. Tested action: A from LTM and A[6] for SR |  | 1 |
| Applicable Mode/Level Combinations | L1: SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a loop with packet SR distance information from loop. (Message accepted) The received Information is used by the on-board equipment. The target distance is updated to D_SR When the distance is covered, service brakes is commanded. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 SR distance information from loop |  |
|  |  | Subset-026-4.8.4.2 SR distance information from loop |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: The Target distance is updated to SR distance. Check on the TIU that: Service brake are commanded when the distance is covered. Check on the JRU that: Reception of packet 13 is RECORDED. |
| Constraints | The on-board is prepared to receive messages from Euroloop. Override is not active. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The override symbol is not displayed. | O | DMI | Absence of symbol MO03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message containing packet 13 (D_SR=<D_Max>) is received | I | LTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7; NID_PACKET=13) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The target distance is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | Check that D_TARGET update to D_SR | N/A | N/A | - |
| 5 | N/A | N/A | Train is moving | I | INT | The train covers the D_SR | N/A | N/A | - |
| 6 | N/A | N/A | Emergency brake commanded | O | TIU | - | N/A | N/A | - |
| 7 | N/A | N/A | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Loop |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 13 | SR distance Information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 8 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG (k) | 14 | FINITE VALUE | Reference balise |
| D_SR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Service brake commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The SR distance information from loop shall be rejected by on-board equipment according to 4.8.3 and 4.8.4.2 tables. Tested action: R by mode |  | 2 |
| Applicable Mode/Level Combinations | L1: TR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a loop with packet SR distance information from loop duet to the mode. The information is not used by the on-board equipment. The target distance not updated to SR distance. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 SR distance information from loop |  |
|  |  | Subset-026-4.8.4.2 SR distance information from loop |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Mode symbol SR is displayed. SR distance National/default value is displayed. Information from packet 13 is not used by the on-board equipment. Check on the JRU that: Reception of packet 13 from Loop is RECORDED. Mode change to SR. SR Speed and Distance National/Default values are recorded. |
| Constraints | The on-board is prepared to receive messages from Euroloop. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| National Values | STORED | D_NVSTFF |
| EOLM information | STORED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | TR | One Euroloop message containing packet 13 (D_SR=<D_Max>)is received | I | LTM | The packet is rejected by on-board equipment due to mode. Packet 13 (D_SR ≠ D_NVSTFF) | L1 | TR | - |
| 2 | L1 | TR | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7; NID_PACKET=13) is recorded | O | JRU | - | L1 | FS/ LS/ OS | - |
| 3 | L1 | TR | The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. | - | - | USE_FT5110200.13 | L1 | PT | - |
| 4 | L1 | PT | The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start“, because the train data are available on-board. | - | - | USE_FT5110200.7 | L1 | PT | - |
| 5 | L1 | PT | Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. | - | - | USE_FT4060300.49 | L1 | SR | - |
| 6 | L1 | SR | The driver presses “Special” | I | DMI | - | L1 | SR | - |
| 7 | L1 | SR | The driver confirms “SR speed/ distance” | I | DMI | Here, check that D_SR is not the one from packet 13 received from trackside. National/Default value is used. | L1 | SR | - |
| 8 | L1 | SR | SR SPEED/DISTANCE ENTERED BY THE DRIVER (NID_MESSAGE_JRU=25) is recorded. | O | JRU | - | L1 | SR | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Loop |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 13 | SR distance Information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 8 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | ≠ D_NVSTFF |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG (k) | 14 | FINITE VALUE | Reference balise |
| D_SR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| National Values | STORED | - |
| EOLM information | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The target distance is displayed | O | DMI | D_NVSTFF |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Inhibition of revocable TSRs from balises in L2/3 shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action : R[2] for packet 64 and A[8] for packet 65 |  | 3 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from RBC with packet 64 Inhibition of revocable TSRs from balises in level different from 2 or 3 because a level 2/3 transition is stored onboard.. The on-board rejects the message from RBC with packet 65 TSRs from balises in level 2 because the Inhibition of revocable TSRs from RTM in L2/L3 is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.3.1.1 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.4.2 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.3.1.1 Exception [8] |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.16.1 Packet Number 64: Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The permitted speed is not updated to V_TSR. Information is not used by the on-board equipment Check on the JRU that: Reception of packet 64 is recorded. Reception of packet 65 is recorded. The TSR is not applied when the D_TSR distance is reached. V_TSR is not recorded. |
| Constraints | An order to switch to L2/3 at a further location has been received. If coming from L0 Un and switch to L2 SR the information regarding Inhibition of revocable TSRs from balises in L2/3 is DELETED according to table 4.10.1.3. The TC shall not apply to this combination. For LS mode, V_TRAIN shall be used to check that the speed restriction is not apply, since V_PERM is not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/ 12 / 13 | FS/ OS/ SR/ UN / LS /SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | M_LEVELTR=3/4 |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ OS/ SR/ UN / LS /SN | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 (Q_DIR=1) is received | I | RTM | The packet is accepted. | L0/ LNTC/ L1 | FS/ OS/ SR/ UN / LS /SN | - |
| 2 | L0/ LNTC/ L1 | FS/ OS/ SR/ UN / LS /SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ OS/ SR/ UN / LS /SN | - |
| 3 | L0/ LNTC/ L1 | FS/ OS/ SR/ UN / LS /SN | Level transition to L2 is executed | - | - | USE_FT5100100.37: Transition from L1 to L2 or USE_FT5100100.21: Transition from LNTC to L2 or USE_FT5100100.13: Transition from L0 to L2 | L2 | FS/OS /SR/ LS | - |
| 4 | L2 | FS/OS /SR/ LS | One Balise Group message (N_TOTAL=1) containing packet 65 (Q_DIR=1) is received | I | BTM | The packet is rejected. | L2 | FS/OS /SR/ LS | - |
| 5 | L2 | FS/OS /SR/ LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 | FS/OS /SR/ LS | - |
| 6 | L2 | FS/OS /SR/ LS | Train is moving. | I | INT | The train drives inside TSR area with it maximum safe front end. | L2 | FS/OS /SR/ LS | - |
| 7 | L2 | FS/OS /SR/ LS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS and SR modes | L2 | FS/OS /SR/ LS | - |
| 8 | L2 | FS/OS /SR/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS and SR modes | L2 | FS/OS /SR/ LS | - |
| 9 | L2 | FS/OS /SR/ LS | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V_TSR in stored message will not be used by the on-board equipment. For FS/OS/SR: Permitted speed does not change to the TSR speed. For LS mode: V_TRAIN will be used instead of V_PERM and should be greater than V_TSR. | L2 | FS/OS /SR/ LS | - |
| 10 | L2 | FS/OS /SR/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= < other than V_TSR >) is recorded | O | JRU | For LS mode: Check that V_TRAIN>V_TSR. V_PERM will not be recorded on the JRU, since it is not displayed on the DMI. | L2 | FS/OS /SR/ LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable TSRs from balises in level 2 or 3. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | 0-126 | Value reserved for non RBC transmission (balise, loop or radio Infill) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 12 | FS / OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 | L2 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Inhibition of revocable TSRs from balises in L2/3 shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action : R |  | 4 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1: FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with packet 64 Inhibition of revocable TSRs from balises in level different from 2 or 3 because a level 2/3 transition is not stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.4.2 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.16.1 Packet Number 64: Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The permitted speed is updated to V_TSR. Information from packet 65 is used by the on-board equipment. Check on the JRU that: Reception of packet 64 is RECORDED. Reception of packet 65 is recorded. Speed and distance monitoring information is recorded. |
| Constraints | An order to switch to L2/3 at a further location has not been received. For LS mode the update of speed with the correct value shall be checked with an Overspeed status, since V_PERM is not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/12 / 13 | FS/ OS/ SR/ UN/ LS /SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN /SN | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 (Q_DIR=1) is received | I | RTM | The packet is rejected | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN /SN | - |
| 2 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN /SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN /SN | - |
| 3 | L0/ LNTC/ L1 | FS/ LS/ OS/ SR/ UN/ SN | Level transition to L2 is executed immediately. | - | - | USE_FT5100100.37: Transition from L1 to L2 Or USE_FT5100100.21: Transition from LNTC to L2 Or USE_FT5100100.13: Transition from L0 to L2 | L2 | FS/LS/OS/SR | - |
| 4 | L2 | FS/ LS/ OS/ SR | One Balise Group message (N_TOTAL=1) containing packet 65 (Q_DIR=1) is received | I | BTM | The packet is accepted | L2 | FS/ LS/ OS/ SR | - |
| 5 | L2 | FS/LS/OS/SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 | FS/ LS/ OS/ SR | - |
| 6 | L2 | FS/ LS/ OS/ SR | Train is moving. | I | INT | The train drives inside TSR area with it maximum safe front end. | L2 | FS/ LS/ OS/ SR | - |
| 7 | L2 | FS/ LS/ OS/ SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS and SR modes | L2 | FS/ LS/ OS/ SR | - |
| 8 | L2 | FS/ LS/ OS/ SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS and SR modes | L2 | FS/ LS/ OS/ SR | - |
| 9 | L2 | FS/ LS/ OS/ SR | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V_TSR in stored message 65 will be used by the on-board equipment. Only If in FS/OS/SR : Status request: Normal status The DMI aspect is coherent with Normal status in a Ceiling speed monitoring section . Only if in LS mode the requested Status is : Overspeed status. The DMI aspect is coherent with Overspeed status in a ceiling speed monitoring section. | L2 | FS/ LS/ OS/ SR | - |
| 10 | L2 | FS/ LS/ OS/ SR | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20 ; M_SDMTYPE=0; V_PERM= < V_TSR >) is recorded | O | JRU | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Check that V_PERM is updated with V_TSR Only if in LS mode : M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | L2 | FS/ LS/ OS/ SR | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable TSRs from balises in level 2 or 3. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | 0-126 | Value reserved for non RBC transmission (balise, loop or radio Infill) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1 / 2 / 12 | FS/ OS/ SR / LS |
| Radio communication session | ESTABLISHED | - |
| Temporary Speed Restrictions | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Stop Shunting on desk opening shall be accepted in all levels according to 4.8.3 table and accepted or rejected by on-board equipment according to 4.8.4 table. Tested action: A in all levels / A in PS mode |  | 5 |
| Applicable Mode/Level Combinations | L0 : PS LNTC : PS L1 / L2 / L3: PS |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a BG containing packet 135 “Stop Shunting on desk opening” The on-board equipment switch to SB mode |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Stop Shunting on desk opening |  |
|  |  | Subset-026-4.8.4.2 Stop Shunting on desk opening |  |
|  |  | Subset-026-7.4.2.31 Packet Number 135: Stop Shunting on desk opening |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 135 is recorded. The mode change to SB mode is recorded. |
| Constraints | Packet 135 Stop Shunting on desk opening is received and accepted. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 15 | PS |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab not active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | PS | One Balise Group message (N_TOTAL=1) containing packet 135 is received | I | BTM | The packet 135 "Stop Shunting on desk opening" is accepted | L0/ LNTC/ L1/ L2/ L3 | PS | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | PS | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | PS | The driver open the desk. On-board equipment switches to SB mode | - | - | USE_FT4060300.61 | L0/ LNTC/ L1/ L2/ L3 | SB | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 135 | Stop Shunting on desk opening |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Cab active | I | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 6

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Stop Shunting on desk opening shall be accepted in all levels according to 4.8.3 table and accepted or rejected by on-board equipment according to 4.8.4 table. Tested action: A in all levels/ R in specific modes |  | 6 |
| Applicable Mode/Level Combinations | L0 : UN / TR LNTC : TR / SN L1: FS/ LS/ OS/ SR/ TR L2 / L3: FS/ LS/ OS/ SR/ TR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a BG with Stop Shunting on desk opening. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Stop Shunting on desk opening |  |
|  |  | Subset-026-4.8.4.2 Stop Shunting on desk opening |  |
|  |  | Subset-026-7.4.2.31 Packet Number 135: Stop Shunting on desk opening |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 135 is RECORDED. The mode change to SH is recorded. Mode change to SB mode is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/ 7/ 12/ 13 | FS/ OS/ SR/ UN/ TR/ LS/ SN |
| Stop Shunting on desk opening | NOT STORED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN/ TR/ SN | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 135 is received | I | BTM | The packet 135 "Stop Shunting on desk opening" is rejected | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN/ TR/ SN | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN/ TR/ SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN/ TR/ SN | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ LS/ OS/ SR/ UN/ TR/ SN | The on-board equipment switch to SH. | - | - | USE_FT5060200.5: From FS to SH or USE_FT5060200.5: From OS to SH or USE_FT5060200.5: From SR to SH or USE_FT5060200.5: From UN to SH or USE_FT5110200.6: From TR to SH or USE_FT5060200.5: From LS to SH or USE_FT5060200.5: From SN to SH | L0/ LNTC/ L1/ L2/ L3 | SH | - |
| 4 | L0/ LNTC/ L1/ L2/ L3 | SH | The on-board equipment switch to PS. | - | - | USE_FT5120400.1 | L0/ LNTC/ L1/ L2/ L3 | PS | - |
| 5 | L0/ LNTC/ L1/ L2/ L3 | PS | The driver open the desk. On- board equipment switches to SH mode | - | - | USE_FT4060300.62 | L0/ LNTC/ L1/ L2/ L3 | SH | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 135 | Stop Shunting on desk opening |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| Stop Shunting on desk opening | NOT STORED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 7

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R and R[1] from BTM |  | 7 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a BG with Permitted Braking distance information or accepted it, if a Level1 transition order is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: Speed and distance is displayed correctly according to the acceptance or rejection of information from packet 52 by the on-board equipment. Check on the JRU that: Reception of packet 52 is RECORDED. Check the rejection or acceptation of Packet 52. |
| Constraints | D_PBD should give the MRSP when received. For LS mode the monitoring of Permitted braking distance information shall be checked with an Overspeed status, since V_PERM and V_TARGET are not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 | L0/ LNTC |
| Permitted Braking Distance Information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN/SN | One Balise Group message (N_TOTAL=1) containing packet 52 (Q_DIR=1) is received | I | BTM | The packet is rejected | L0 / LNTC | UN/SN | - |
| 2 | L0 / LNTC | UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN/SN | - |
| 3 | L0 / LNTC | UN/SN | The speed and distance monitoring information is displayed | O | DMI | monitoring type =CSM and the status requested=Normal. No modification of MRSP | L0 / LNTC | UN/SN | - |
| 4 | L0 / LNTC | UN/SN | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | L0 / LNTC | UN/SN | - |
| 5 | L0 / LNTC | UN/SN | One Balise Group message (N_TOTAL=1) containing packet 41 containing packet 12, packet 21 and 27 is received | I | BTM | - | L0 / LNTC | UN/SN | - |
| 6 | L0 / LNTC | UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN/SN | - |
| 7 | L0 / LNTC | UN/SN | One Balise Group message (N_TOTAL=1) containing packet 52 (Q_DIR=1) is received | I | BTM | The packet is accepted | L0 / LNTC | UN/SN | - |
| 8 | L0 / LNTC | UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 9 | L0 / LNTC | UN/SN | On-board equipment executes a Level transition order to level 1. | - | - | USE_FT5100100.15: from L0 to L1 or USE_FT5100100.23: from LNTC to L1 | L1 | UN / SN | - |
| 10 | L1 | UN / SN | On-board equipment executes a mode change. | - | - | USE_FT4060300.19 (Transition to FS) Or USE_FT4060300.55 (Transition to OS) Or USE_FT5190300.4 (Transition to LS) | L1 | FS / LS /OS | - |
| 11 | L1 | FS / LS /OS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS mode | L1 | FS / LS /OS | - |
| 12 | L1 | FS / LS /OS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS mode | L1 | FS / LS /OS | - |
| 13 | L1 | FS /LS/ OS | The speed and distance monitoring information is displayed | O | DMI | Only if not in LS mode: The calculated speed restriction is shown in the Planning Area Speed Profile (PASP). V_TARGET is displayed | L1 | FS / LS/ OS | - |
| 14 | L1 | FS/ LS/ OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET= <V_MRSP>) is recorded | O | JRU | Only if not in LS mode: V_TARGET= <V_MRSP> is the speed restriction calculated to ensure the permitted braking distance given in packet 52 | L1 | FS/ LS/ OS | - |
| 15 | L1 | FS / LS /OS | The speed and distance monitoring information is displayed | O | DMI | The train has passed the start location (D_PBSR) of the speed restriction to ensure permitted braking distance Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling speed monitoring section and the permitted speed, which is given by the speed restriction to ensure the permitted braking distance, is shown. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling speed monitoring section. The train speed will be a bit over the PS. | L1 | FS / LS /OS | - |
| 16 | L1 | FS / LS /OS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; V_PERM=<V_MRSP>) is recorded | O | JRU | V_MRSP is the speed restriction calculated to ensure the permitted braking distance given in packet 52. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if not in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | L1 | FS / LS /OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | 2 | To level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS /LS |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Permitted Braking Distance Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 8

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested action: A from BTM and R from RTM |  | 8 |
| Applicable Mode/Level Combinations | L1: FS / OS / SR / LS |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a BG with Permitted Braking distance information. The on-board rejects the message from RBC with Permitted Braking distance information because an order to switch to L2/L3 is not stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 52 is RECORDED. Check that Permitted Braking distance information is monitoring |
| Constraints | D_PBD should give the MRSP. For LS mode the monitoring of Permitted braking distance information shall be checked with an Overspeed status, since V_PERM and V_TARGET are not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | To level 2 / 3 at a further location |
| ERTMS/ETCS level | 2 | L1 |
| Permitted Braking Distance Information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 (Q_DIR=1) is received | I | RTM | The packet is rejected | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS and SR modes | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS and SR modes | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Monitoring type =CSM and the status requested=Normal. No modification of MRSP | N/A | N/A | - |
| 6 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 52 (Q_DIR=1) is received | I | BTM | The packet is accepted | N/A | N/A | - |
| 7 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if not in LS mode: V_TARGET is displayed. The calculated speed restriction is shown in the Planning Area Speed Profile (PASP). | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET= <V_MRSP>) is recorded | O | JRU | Only if not in LS mode: V_TARGET= <V_MRSP> is the speed restriction calculated to ensure the permitted braking distance given in packet 52 | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed the start location (D_PBSR) of the speed restriction to ensure permitted braking distance Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling speed monitoring section and the permitted speed, which is given by the speed restriction to ensure the permitted braking distance, is shown. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling speed monitoring section. The train speed will be a bit over the PS. | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; V_PERM=<V_MRSP>) is recorded | O | JRU | V_MRSP is the speed restriction calculated to ensure the permitted braking distance given in packet 52. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if not in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| Step 6: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Permitted Braking Distance Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 9

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested action: R from BTM and A from RTM |  | 9 |
| Applicable Mode/Level Combinations | L2 / L3 : FS / LS / OS/ SR / SB / PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a BG with Permitted Braking distance information due to no level transition ordered stored. The on-board accepts the message from RBC with Permitted Braking distance information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.8.4.2 [4] |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI interface that: Speed and distance monitoring is displayed correctly according to the acceptance or rejection of information from packet 52 by the on-board equipment. Check on the JRU that: Reception of packet 52 is RECORDED. |
| Constraints | SB mode: Cab is active and Valid Train data are stored on-board D_PBD shall give the MRSP. In level 2/ 3 and PT mode, “Recognition of Exit from TR mode” has been received with a more recent time stamp. Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received. For LS mode the monitoring of Permitted braking distance information shall be checked with an Overspeed status, since V_PERM and V_TARGET are not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 8/ 12 | FS/ OS/ SR/ SB/ PT/ LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | For L1 at further location |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 52 (Q_DIR=1) is received | I | BTM | The packet is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS and SR modes | N/A | N/A | - |
| 4 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS and SR modes | N/A | N/A | - |
| 5 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | monitoring type =CSM and the status requested=Normal. No modification of MRSP | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 (Q_DIR=1) is received | I | RTM | The packet is accepted | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Only if not in LS mode: V_TARGET is displayed. The calculated speed restriction is shown in the Planning Area Speed Profile (PASP). | N/A | N/A | - |
| 9 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET= <V_MRSP>) is recorded | O | JRU | Only if not in LS mode: V_TARGET= <V_MRSP> is the speed restriction calculated to ensure the permitted braking distance given in packet 52 | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | The train has passed the start location (D_PBSR) of the speed restriction to ensure permitted braking distance Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling speed monitoring section and the permitted speed, which is given by the speed restriction to ensure the permitted braking distance, is shown. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling speed monitoring section. The train speed will be a bit over the PS. | N/A | N/A | - |
| 11 | N/A | N/A | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; V_PERM=<V_MRSP>) is recorded | O | JRU | V_MRSP is the speed restriction calculated to ensure the permitted braking distance given in packet 52. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if not in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 52 | Permitted Braking distance information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT=1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT=0, D_PBD and the following variables |
| Q_GDIR | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking Distance information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 8/ 12 | FS/ OS/ SR/ SB/ PT/ LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Permitted Braking Distance Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 10

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested actions: R from BTM & A from BTM |  | 10 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts or rejects the message from a BG with Level Crossing information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The first LX information is not supervised, LX symbol is not displayed The second LX information is supervised, LX symbol is displayed Check on the JRU that: Reception of packets 88 is RECORDED. Two telegrams containing Level Crossing information are RECEIVED. First packet is rejected and second one is accepted. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | For level 2 and 3 |
| ERTMS/ETCS level | 0 / 1 | L0/ LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | BTM | The packet is rejected | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PAQUET=88) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | Train is moving | I | INT | The train reaches the D_LX and no supervision of the LX start location is effective. | L0 / LNTC | UN / SN | - |
| 4 | L0 / LNTC | UN / SN | NOT The Level symbol "Level Crossing" is displayed | O | DMI | - | L0 / LNTC | UN / SN | - |
| 5 | L0 / LNTC | UN / SN | On-board equipment receives a Level transition order to level 1, 2 or 3. Packets 12, 21, 27 and 80 (for OS/LS mode) shall be added if MA and track description not received before. | - | - | USE_FT5100100.15: from L0 to L1 USE_FT5100100.23: from LNTC to L1 or USE_FT5100100.13: from L0 to L2 USE_FT5100100.21: from LNTC to L2 or USE_FT5100100.26: from L0 to L3 USE_FT5100100.20: from LNTC to L3 | L1/ L2 / L3 | FS / OS / LS | - |
| 6 | L1/ L2 / L3 | FS / OS / LS | Mode change is performed | - | - | USE_FT4060300 corresponding TC´s | L1/ L2 / L3 | FS / OS / LS | - |
| 7 | L1/ L2 / L3 | FS / OS / LS | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | BTM | The packet is accepted | L1/ L2 / L3 | FS / OS / LS | - |
| 8 | L1/ L2 / L3 | FS / OS / LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PAQUET=88) is recorded | O | JRU | - | L1/ L2 / L3 | FS / OS / LS | - |
| 9 | L1/ L2 / L3 | FS / OS / LS | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | L1/ L2 / L3 | FS / OS / LS | - |
| 10 | L1/ L2 / L3 | FS / OS / LS | The Level symbol "Level Crossing" is displayed | O | DMI | Symbol LX01 | L1/ L2 / L3 | FS / OS / LS | - |
| 11 | L1/ L2 / L3 | FS / OS / LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | L1/ L2 / L3 | FS / OS / LS | - |
| 12 | L1/ L2 / L3 | FS / OS / LS | Train is at standstill | I | INT | The train stops in rear of the non-protected LX | L1/ L2 / L3 | FS / OS / LS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 88 | Level crossing information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 7: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 88 | Level crossing information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | For level 2 and 3 |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 11

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing information shall be accepted or rejected by on-board equipment according to 4.8.3 and 4.8.4 table. Tested Action: R from RTM |  | 11 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1 : FS/ LS/ OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with Level Crossing information because of no level transition order is stored. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing information |  |
|  |  | Subset-026-4.8.4.2 Level Crossing information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LX information is not supervised, LX symbol is not displayed. Information from packet 88 is not used by the on-board equipment. Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. No level transition to L2/L3 is stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4 / 12 / 13 | FS/ OS/ SR/ UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | M_LEVELTR= 3 / 4 |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |
| Level Crossing information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | RTM | The packet is rejected | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PAQUET=88) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | The train reaches the D_LX and no supervision of the LX start location is effective. | N/A | N/A | - |
| 4 | N/A | N/A | NOT The Level symbol "Level Crossing" is displayed | O | DMI | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level Crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | 1 | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4 / 12 / 13 | FS/ OS/ SR/ UN / LS/ SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC / L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 12

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level crossing information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: A from RTM |  | 12 |
| Applicable Mode/Level Combinations | L2 / L3 : FS/ LS/ OS |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a RBC with Level crossing information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LX information is supervised, LX symbol is displayed. Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | RTM | The packet is accepted | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PAQUET=88) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | N/A | N/A | - |
| 4 | N/A | N/A | The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 | N/A | N/A | - |
| 5 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | Train is at standstill | I | INT | The train stops in rear of the non-protected LX | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 13

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Plain text Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested action: A and A [12] from RTM |  | 13 |
| Applicable Mode/Level Combinations | L2/ L3: FS/ LS/ OS/ SR/ SB/ TR/ PT/ RV |  |  |
| Target of Test | The target of this test case is to check that: A Plain text Information is rejected if the text message is sent with a request for report of driver acknowledgement with the same text message identifier as a previously received text message, which the driver has not yet acknowledged. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Plain Text information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [12] |  |
|  |  | Subset-026-4.8.4.2 Plain Text information |  |
|  |  | Subset-026-7.4.2.23 Packet Number 72: Packet for sending plain text messages |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver does not acknowledges the first plaint text message and the second plain text message is not displayed because the second packet 72 is rejected by the on-board. Check on the JRU that: Reception of packets 72 are RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | The packet is accepted. The text message is sent with a request for report of driver acknowledgement | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The "plain text message" is displayed | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | NOT The driver acknowledges "plain text message" | I | DMI | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 72 is received | I | RTM | The packet is rejected The text message has the same text message identifier as a previously received text message. | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET=72) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | <> from 0 |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM <> 0 |
| Q_TEXTREPORT | 1 | 1 | If Q_TEXTCONFIRM <> 0 |
| NID_TEXTMESSAGE | 8 | 0-255 | Only if Q_TEXTREPORT=1 |
| NID_C | 10 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| NID_RBC | 14 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT (L_TEXT) | 8 | FINITE VALUE | - |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | Plain Text Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | <>0 |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM <> 0 |
| Q_TEXTREPORT | 1 | 1 | If Q_TEXTCONFIRM <> 0 |
| NID_TEXTMESSAGE | 8 | 0-255 | Only if Q_TEXTREPORT=1 The same value than RTM message in step 3 |
| NID_C | 10 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| NID_RBC | 14 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT (L_TEXT) | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "plain text message" is displayed | O | DMI | The first plain text received |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 14

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Fixed text Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: A and A [12] from RTM |  | 14 |
| Applicable Mode/Level Combinations | L2/ L3: FS/ LS/ OS/ SR/ SB/ TR/ PT/ RV |  |  |
| Target of Test | The target of this test case is to check that: A Fixed text Information is rejected if the text message is sent with a request for report of driver acknowledgement with the same text message identifier as a previously received text message, which the driver has not yet acknowledged. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Fixed Text information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [12] |  |
|  |  | Subset-026-4.8.4.2 Fixed Text information |  |
|  |  | Subset-026-7.4.2.24 Packet Number 76: Packet for sending fixed text messages |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The second plain text message is not displayed Check on the JRU that: Reception of packets 76 are RECORDED. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received | I | RTM | The packet is accepted. The text message is sent with a request for report of driver acknowledgement | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET=76) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The text message is displayed to the driver. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT The driver acknowledges "fixed text message" | I | DMI | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received | I | RTM | The packet is rejected The text message has the same text message identifier as a the previously received text message. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET=76) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | <>0 |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM <> 0 |
| Q_TEXTREPORT | 1 | 1 | If Q_TEXTCONFIRM <> 0 |
| NID_TEXTMESSAGE | 8 | 0-255 | Only if Q_TEXTREPORT=1 |
| NID_C | 10 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| NID_RBC | 14 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| Q_TEXT | 8 | FINITE VALUE | - |

| Step 5: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 76 | Fixed Text Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event |
| M_LEVELTEXTDISPLAY | 3 | 3/ 4/ 5 | Level 2/ Level 3 / Not be limit at the level |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY=1 (NTC) |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | <>0 |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM <> 0 |
| Q_TEXTREPORT | 1 | 1 | If Q_TEXTCONFIRM <> 0 |
| NID_TEXTMESSAGE | 8 | 0-255 | Only if Q_TEXTREPORT=1 The same value than RTM message in step 1 |
| NID_C | 10 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| NID_RBC | 14 | FINITE VALUE | Only if Q_TEXTREPORT=1 |
| Q_TEXT | 8 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The "fixed text message" is displayed | O | DMI | The first fixed text received |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 15

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Danger for SH information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R & A from BTM |  | 15 |
| Applicable Mode/Level Combinations | L0 : SH LNTC : SH |  |  |
| Target of Test | The target of this test case is to check that: A Danger for SH information shall be rejected if it is not received together with an immediate level transition order to L1/ L2 / L3 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Danger for SH information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [13] |  |
|  |  | Subset-026-4.8.4.2 Danger for SH information |  |
|  |  | Subset-026-7.4.2.28 Packet Number 132: Danger for Shunting information |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 132 is RECORDED. |
| Constraints | Immediate Level transition order to L1/2/3 is not stored on-board |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 | SH |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | SH | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 132 is received | I | BTM | The packet "Danger for SH" is rejected | L0 / LNTC | SH | - |
| 2 | L0 / LNTC | SH | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0 / LNTC | SH | - |
| 3 | L0 / LNTC | SH | Train is moving | I | INT | Information from BG is rejected. No transition to TR mode | L0 / LNTC | SH | - |
| 4 | L0 / LNTC | SH | USE_FT4080408.1 | - | - | The train is in SH mode in L1,L2 or L3 and overpasses a BG with packets#132# (danger for SH information) with variable Q_ASPECT=0 (the train will be tripped if it´s in SH mode)- The packets are accepted. | L1 / L2 / L3 | TR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 132 | Danger for SH information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_ASPECT | 1 | 0 | Stop if in SH mode |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 | TR |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 16

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Conditional Level Transition Order shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: A[11] from BTM |  | 16 |
| Applicable Mode/Level Combinations | L0 : SH/ UN/ PS/ SL/ TR / NL LNTC : SH/ PS/ SL/ TR / NL/ PS L1 : FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a BG with conditional level transition order if a previous level transition order has announced a level transition still to be executed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.3.1.1 Exception [11] |  |
|  |  | Subset-026-7.4.2.11.2 Packet Number 46: Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 46 is RECORDED. |
| Constraints | All levels are available in on-board equipment. In SH and PS mode, the level transition is not performed, only stored onboard. For Packet 46 , the current level shall be contained in the priority list of the conditional level transition order. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 7/ 11/12/ 13/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS |
| Radio communication session | ESTABLISHED | Only L2 / L3 |
| Level Transition Order | STORED | D_LEVELTR ≠now M_LEVETR ≠ from the actual one |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only for L2 / L3 |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <>" is displayed. | O | DMI | Current level |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 46 is received | I | BTM | The order received in packet 46 is to same level but the immediate level transition is not executed because the information received in this packet is rejected. (The level transition announced still to be executed) | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 2 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 3 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | Train is moving. | I | INT | The distance D_LEVELTR received in packet 41 is reached. | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 4 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=Y) is recorded | O | JRU | Triggering event: When the level changes This step not applies to SH and PS modes | LY | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 5 | LY | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | The Level symbol "Level Y" is displayed. | O | DMI | The level indication changes This step not applies to SH and PS modes | LY | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 6 | LY | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<BitXX=1>) is recorded | O | JRU | This step not applies to SH and PS modes | LY | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 46 | Conditional level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | LY (Order to same level as the one contained in packet 41) |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | (k) = LY (contained in the priority list) |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 7/ 8/ 11/12/ 13/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ PS |
| Radio communication session | ESTABLISHED | Only L2 / L3 |
| Level Transition Order | STORED | Only in SH and PS mode |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Y" is displayed | - | DMI | New level |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 17

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Conditional Level Transition Order shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: A[11] from BTM |  | 17 |
| Applicable Mode/Level Combinations | L0 : SH/ UN/ PS/ SL/ TR / NL LNTC : SH/ PS/ SL/ TR / NL/ PS L1 : FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL L2 / L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects conditional level transition order from a BG with a level transition order received in the same message. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.4.2 Level Transition Order and Conditional Level Transition Order |  |
|  |  | Subset-026-4.8.3.1.1 Exception [11] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.11.2 Packet Number 46: Conditional Level Transition Order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 41 and 46 transmitted in the same message is RECORDED. |
| Constraints | All levels are available in on-board equipment. In SH and PS mode, the level transition is not performed, only stored onboard. For Packet 46 , the current level shall be contained in the priority list of the conditional level transition order. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 7/ 11/12/ 13/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS |
| Radio communication session | ESTABLISHED | Only L2 / L3 |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <>" is displayed | O | DMI | Current level |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 and packet 46 is received | I | BTM | The packet 46 is rejected because a level transition order is received in the same BG. Packet 41: M_LEVETR= LX Packet 46: M_LEVELTR= LY | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 3 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=X) is recorded | O | JRU | The level transition order to LY (packet 46) is not executed. | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 4 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | The Level symbol "Level X" is displayed. | O | DMI | The level indication changes | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |
| 5 | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<BitXX=1>) is recorded | O | JRU | - | LX | FS/ OS/ SR/ SH/ UN/ SL/ TR/ NL/ LS/ SN/ PS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 46 | Conditional level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| M_LEVELTR | 3 | FINITE VALUE | = LY |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | LY contained in the priority list |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| NID_PACKET | 8 | 41 | Level transition order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | FINITE VALUE | = LX |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC) |
| L_ACKLEVELTR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 3/ 4/ 5/ 7/ 8/ 11/12/ 13/ 15 | FS/ OS/ SR/ SH/ UN/ SL/ TR/ PT/ NL/ LS/ SN/ PS |
| Radio communication session | ESTABLISHED | Only L2 / L3 |
| Level Transition Order | STORED | Only for SH and PH mode |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level X" is displayed | O | DMI | New level |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 18

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The SR distance information from loop shall be rejected by on-board equipment according to 4.8.4.2 table. Exception [6] Tested Action: A[6] by mode |  | 18 |
| Applicable Mode/Level Combinations | L1: FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a loop with packet SR distance information from loop due to override activation. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 SR distance information from loop |  |
|  |  | Subset-026-4.8.4.2 SR distance information from loop |  |
|  |  | Subset-026-4.8.4.2 [6] |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 13 from Loop is RECORDED. Check on the TIU interfaces that: After pass to SR mode the brake is not apply when the D_SR is covered. The packet 13 is rejected. |
| Constraints | The on-board is prepared to receive messages from Euroloop. The Override is triggered. Train speed shall be less than V_NVSUPOVTRP. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS/ OS/ LS |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | speed must be less than V_NVSUPOVTRP |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/ LS/ OS | The driver selects "Override EoA" | I | DMI | - | L1 | SR | - |
| 2 | L1 | SR | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1110) is recorded | O | JRU | - | L1 | SR | - |
| 3 | L1 | SR | The Mode symbol "Override EOA is active" is displayed | O | DMI | - | L1 | SR | - |
| 4 | L1 | SR | The Mode symbol "Staff Responsible" is displayed | O | DMI | - | L1 | SR | - |
| 5 | L1 | SR | Train is moving | I | INT | - | L1 | SR | - |
| 6 | L1 | SR | One Euroloop message (NID_C=<comment>) containing packet 13 (D_SR=<D_Max>)is received | I | LTM | The packet is rejected by on-board equipment because override function is active | L1 | SR | - |
| 7 | L1 | SR | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7; NID_PACKET=13) is recorded | O | JRU | - | L1 | SR | - |
| 8 | L1 | SR | Train is moving | I | INT | THE TRAIN COVER THE SR DISTANCE D_SR=<D_Max> | L1 | SR | - |
| 9 | L1 | SR | Emergency brake not commanded | O | TIU | - | L1 | SR | - |

| Step 6: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Loop |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 13 | SR distance Information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 8 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG (k) | 14 | FINITE VALUE | Reference balise |
| D_SR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| Emergency brake not commanded | O | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 19

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Session management with neighbouring RIU shall be rejected by on-board equipment according to 4.8.4. table. Tested Action: R by modes |  | 19 |
| Applicable Mode/Level Combinations | L1: SH/ PS / SL / NL / TR / PT / RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the information received from BG containing packet 143: Session management with neighbouring RIU . No communication session with RIU is established by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Session Management with neighbouring RIU |  |
|  |  | Subset-026-4.8.4.2 Session Management with neighbouring RIU |  |
|  |  | Subset-026-7.4.2.37.1 Packet Number 143: Session Management with neighbouring Radio Infill Unit |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 143 is RECORDED. No radio link connection is recorded. |
| Constraints | Communication session with RIU is not established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 8 / 11/ 14 /15 | SH/ SL /NL / TR/ PT / RV/ PS |
| Radio communication session | ESTABLISHED | With RIU(A) |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With RIU(A) |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 143 (Q_DIR=1; Q_RIU=1) is received | I | RTM | The packet is rejected by on-board equipment. The packet 143 is send by RIU (A)” to connect with RIU(B) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 143 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | RIU B is a value different from the value for RIU A |
| NID_RADIO | 64 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 /5/ 7/ 8 / / 11/ 14 /15 | PS /SH/ SL /NL / TR/ PT /RV |
| Radio communication session | ESTABLISHED | With RIU(A) |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | - | RTM | With RIU(A) |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 20

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be rejected by on-board equipment according to 4.8.4 table. |  | 20 |
| Applicable Mode/Level Combinations | L1 : SH/ PS/ SL/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from BG with Permitted Braking distance information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO/CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The speed and distance monitoring information is displayed but not updated. Check on the JRU that: Reception of packet 52 is RECORDED. Check the Permitted Braking distance information is not apply |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 11 / 14 / 15 | SH / SL / TR/ NL/ RV/ PS |
| ERTMS/ETCS level | 2 | L1 |
| Permitted Braking Distance Information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 52 (Q_DIR=1) is received | I | BTM | The packet is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | monitoring type =CSM and the status requested=Normal. No modification of MRSP | N/A | N/A | - |
| 4 | N/A | N/A | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 52 | Permitted Braking Distance Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT = 1. |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT = 0, D_PBD and the following variables follow. |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill, 1 = uphill. |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation. |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 11 / 14 / 15 | SH / SL / TR/ NL/ RV/ PS |
| ERTMS/ETCS level | 2 | L1 |
| Permitted Braking Distance Information | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 21

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing information shall be rejected by on-board equipment according to 4.8.4 table. Tested Action: R by modes |  | 21 |
| Applicable Mode/Level Combinations | L1 : SH/ PS/ SL/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from BG with Level Crossing information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing information |  |
|  |  | Subset-026-4.8.4.2 Level Crossing information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Level symbol "Level Crossing" is not displayed Check on the JRU that: Reception of packet 88 is RECORDED. Check the Level Crossing information is not apply. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 11 / 14 / 15 | SH / SL / NL / TR/ RV / PS |
| ERTMS/ETCS level | 2 | L1 |
| Level Crossing information | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_DIR=0) is received | I | BTM | The packet is rejected | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PAQUET=88) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | The train reaches the D_LX and no supervision of the LX start location is effective. | N/A | N/A | - |
| 4 | N/A | N/A | NOT The Level symbol "Level Crossing" is displayed | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 88 | Level Crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | FINITE VALUE | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 3 / 5 / 7 / 11 / 14 / 15 | SH / SL / TR/ NL/ RV / PS |
| ERTMS/ETCS level | 2 | L1 |
| Level Crossing information | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 22

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The SR distance information from loop shall be REJECTED by on-board equipment according to 4.8.3 table. |  | 22 |
| Applicable Mode/Level Combinations | L2 / L3: SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from a loop with packet SR distance information from loop. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO/CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 SR distance information from loop |  |
|  |  | Subset-026-4.8.4.2 SR distance information from loop |  |
|  |  | Subset-026-7.4.2.3.1 Packet Number 13: Staff Responsible distance information from loop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 13 from Loop is RECORDED. |
| Constraints | The on-board is prepared to receive messages from Euroloop. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Euroloop message (NID_C=<comment>) containing packet 13 (D_SR=<D_Max>)is received | I | LTM | The packet is rejected by on-board equipment. | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM EUROLOOP (NID_MESSAGE_JRU=7; NID_PACKET=13) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | Train is moving | I | INT | THE TRAIN COVER THE SR DISTANCE D_SR=<D_Max> | N/A | N/A | - |
| 4 | N/A | N/A | Emergency brake not commanded | O | TIU | - | N/A | N/A | - |

| Step 1: Euroloop Message |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 1 | Loop |
| NID_C | 10 | FINITE VALUE | - |
| NID_LOOP | 14 | FINITE VALUE | Identity of Euroloop |
| NID_PACKET | 8 | 13 | SR distance Information from loop |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 8 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Main signal balise group |
| Q_NEWCOUNTRY | 1 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG | 14 | FINITE VALUE | Reference balise |
| D_SR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_NEWCOUNTRY (k) | 1 | FINITE VALUE | - |
| NID_C (k) | 10 | FINITE VALUE | If Q_NEWCOUNTRY=1 |
| NID_BG (k) | 14 | FINITE VALUE | Reference balise |
| D_SR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 2 | SR |
| EOLM information | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 23

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level crossing information shall be accepted by on-board equipment according to 4.8.4.2 table. Tested Action: A in PT mode |  | 23 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from RBC with Level crossing information in level 2 or 3 and mode PT because "Recognition of Exit from TR mode" has been received . |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level crossing information |  |
|  |  | Subset-026-4.8.4.2 Level Crossing information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LX information is supervised, LX symbol is displayed Check on the JRU that: Reception of packet 88 is RECORDED. Mode change to FS is recorded. |
| Constraints | In level 2/ 3 "Recognition of Exit from TR mode" has been received. Train data has been sent and “Acknowledgment of Train Data " has been received. (Message 8 ) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | Message 8 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 88 (Q_DIR=1) is received | I | RTM | The packet is accepted. | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PAQUET=88) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | The on-board performs a mode change to FS | - | - | USE_FT4060300.20 | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | The Level symbol "Level Crossing" is displayed | O | DMI | Symbol LX01 | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | Train is at standstill | I | INT | The train stops in rear of the non-protected LX | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level Crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q_LXSTATUS = 1. |
| Q_STOPLX | 1 | 1 | Only if Q_LXSTATUS = 1. |
| L_STOPLX | 15 | FINITE VALUE | Only if Q_STOPLX = 1. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | - |

## Test case 24

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Session management with neighbouring RIU shall be rejected by on-board equipment according to 4.8.3 table. Tested Action: R by levels |  | 24 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2 / L3: FS/ LS/ OS /SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the telegram from BG containing packet 143: Session management with neighbouring RIU . No communication session is established by the on-board equipment. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Session Management with neighbouring RIU |  |
|  |  | Subset-026-4.8.4.2 Session Management with neighbouring RIU |  |
|  |  | Subset-026-7.4.2.37.1 Packet Number 143: Session Management with neighbouring Radio Infill Unit |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 143 is recorded. No communication session established with RIIU (B) is recorded. |
| Constraints | Communication session with RIU (A) is established. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2/ 4/11 /13 | FS/ OS/ SR/ UN/ LS/ SN |
| Radio communication session | ESTABLISHED | with RIU (A) |
| ERTMS/ETCS level | 0/ 1/ 3/ 4 | L0/LNTC/L2/L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | with RIU (A) |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 143 (Q_DIR=1; Q_RIU=1) is received | I | RTM | The packet is rejected by on-board equipment. The packet 143 is send by RIU (A)” to connect with RIU(B) | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 143 | Session Management with neighbouring Radio Infill Unit |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RIU | 1 | 1 | - |
| NID_C | 10 | FINITE VALUE | RIU ETCS identity. |
| NID_RIU | 14 | FINITE VALUE | RIU B is a value different from the value for RIU A |
| NID_RADIO | 64 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2/ 4/11 /13 | FS/ OS/ SR/ UN/ LS/ SN |
| Radio communication session | ESTABLISHED | with RIU (A) |
| ERTMS/ETCS level | 0/ 1/ 3/ 4 | L0/LNTC/L2/L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | with RIU (A) |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 25

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R[1] from BTM |  | 25 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a BG with Level Crossing information because of level transition to Level 1 is stored. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The LX information is supervised, LX symbol is displayed Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. Level transition order to L1 stored onboard before receiving packet 88. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to Level 1 at a further location |
| Mode Profile | STORED | Only for LS/ OS mode |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | BTM | The packet is accepted | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PAQUET=88) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | On-board equipment executes both, level transition to L1 and mode change. | - | - | USE_FT4060300.19: Transition to FS or USE_FT4060300.55: Transition to OS or USE_FT5190300.4: Transition to LS | L1 | FS / LS / OS | - |
| 4 | L1 | FS / LS / OS | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | L1 | FS / LS / OS | - |
| 5 | L1 | FS / LS / OS | The Level symbol "Level Crossing" is displayed | O | DMI | Symbol LX01 | L1 | FS / LS / OS | - |
| 6 | L1 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | L1 | FS / LS / OS | - |
| 7 | L1 | FS / LS / OS | Train is at standstill | I | INT | The train stops in rear of the non-protected LX | L1 | FS / LS / OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 88 | Level crossing information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | Only for LS/ OS mode |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 26

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R[2] from BTM |  | 26 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a BG with Level Crossing information because level transition order to Level 2/3 is stored. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The LX information is supervised, LX symbol is displayed. Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. Level transition order to L2/L3 stored onboard before receiving packet 88. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to L2 / L3 at a further location |
| Mode Profile | STORED | Only for LS / OS mode |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message (N_TOTAL=1) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1) is received | I | BTM | The packet is accepted | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PAQUET=88) is recorded | O | JRU | - | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | On-board equipment executes both, level transition to L2/ L3 and mode change. | - | - | USE_FT4060300.19: Transition to FS or USE_FT4060300.55: Transition to OS or USE_FT5190300.4: Transition to LS | L2 / L3 | FS / LS / OS | - |
| 4 | L2 / L3 | FS / LS / OS | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | L2 / L3 | FS / LS / OS | - |
| 5 | L2 / L3 | FS / LS / OS | The Level symbol "Level Crossing" is displayed | O | DMI | Symbol LX01 | L2 / L3 | FS / LS / OS | - |
| 6 | L2 / L3 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | L2 / L3 | FS / LS / OS | - |
| 7 | L2 / L3 | FS / LS / OS | Train is at standstill | I | INT | The train stops in rear of the non-protected LX | L2 / L3 | FS / LS / OS | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 88 | Level crossing information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | Only for LS/ OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 27

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level Crossing Information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R[2] from RTM |  | 27 |
| Applicable Mode/Level Combinations | L0 : UN LNTC : SN L1: FS / LS / OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from a RTM with Level Crossing information because of level transition to Level 2/3 is stored. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The LX information is supervised, LX symbol is displayed Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. Level transition order to L2 / L3 stored onboard before receiving packet 88. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4 / 12 / 13 | FS/ OS/ SR/ UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | STORED | Transition to L2 / L3 at a further location |
| Mode Profile | STORED | Only for LS / OS mode |
| ERTMS/ETCS level | 0/ 1 / 2 | L0/ LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 | FS/ LS / OS/ SR/ UN SN | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 88 (Q_LXSTATUS=1;Q_STOPLX=1)is received | I | RTM | The packet is accepted | L0 / LNTC / L1 | FS/ LS / OS/ SR/ UN SN | - |
| 2 | L0 / LNTC / L1 | FS/ LS / OS/ SR/ UN SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PAQUET=88) is recorded | O | JRU | - | L0 / LNTC / L1 | FS/ LS / OS/ SR/ UN SN | - |
| 3 | L0 / LNTC / L1 | FS/ LS / OS/ SR/ UN SN | On-board equipment receives a Level transition order to level 2 or 3. | - | - | For L0/NTC: Use_FT4060300.19: Transition to L2/3 in FS Or Use_FT4060300.55: Transition to L2/3 in OS Or Use_FT5190300.4: Transition to L2/3 in LS For L1: Use_FT5100100.37 transition to L2 Or Use_FT5100100.36 transition to L3 | L2/ L3 | FS / LS / OS | - |
| 4 | L2/ L3 | FS / LS / OS | Train is moving | I | INT | The train supervises the LX start location as an EOA and SVL | L2/ L3 | FS / LS / OS | - |
| 5 | L2/ L3 | FS / LS / OS | The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 | L2/ L3 | FS / LS / OS | - |
| 6 | L2/ L3 | FS / LS / OS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit86=1>) is recorded | O | JRU | - | L2/ L3 | FS / LS / OS | - |
| 7 | L2/ L3 | FS / LS / OS | Train is at standstill | I | INT | The train stops in rear of the non protected LX | L2/ L3 | FS / LS / OS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Level Transition Order | NOT STORED | - |
| Mode Profile | STORED | Only for LS/ OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level Crossing" is displayed | O | DMI | DMI Object: Symbol LX01 |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

## Test case 28

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Level crossing information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: A [3] from RTM |  | 28 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the Level crossing information received from RBC because no acknowledgement of train data has been received yet. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Level Crossing Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-3.18.3.2.3 |  |
|  |  | Subset-026-A.3.11 Train Data: Train length |  |
|  |  | Subset-026-4.8.4.2 Level Crossing Information |  |
|  |  | Subset-026-7.4.2.4 Packet Number 15: Level 2/3 Movement Authority |  |
|  |  | Subset-026-7.4.2.6 Packet Number 21: Gradient Profile |  |
|  |  | Subset-026-7.4.2.7 Packet Number 27: International Static Speed Profile |  |
|  |  | Subset-026-7.4.2.26.1 Packet Number 88: Level Crossing information |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.5 Packet Number 11: Validated train data |  |
|  |  | Subset-026-8.6.1 Message 129: Validated Train Data |  |
|  |  | Subset-026-8.7.2 Message 3: Movement Authority |  |
|  |  | Subset-026-8.7.4 Message 8: Acknowledgement of Train Data |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LX information is not supervised, LX symbol is not displayed. Information from packet 88 is not used by the on-board equipment. Check on the JRU that: Reception of packet 88 is RECORDED. |
| Constraints | Level crossing shall be non-protected and the train must stop in rear of the LX. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only for LS / OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is at standstill | I | INT | V_TRAIN=0 |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver modifies the "Length" and validates the "Flexible Train Data" | I | DMI | Driver modifies the length of the train Note: The unit , range and resolution of the train length shall be in line with table A.3.11 | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 0101) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_PAQUET=11) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 88 (Q_LXSTATUS=1; Q_STOPLX=1)is received | I | RTM | The packet is rejected Acknowledgement of train data not yet received | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PAQUET=88) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | Acknowledgement of train data is received | N/A | N/A | - |
| 9 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 10 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DIR=1), packet 21 (Q_DIR=1) and packet 27 is received | I | RTM | Packet 80 shall be added if the applicable mode is LS / OS | N/A | N/A | - |
| 11 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 12 | N/A | N/A | Train is moving | I | INT | The train does not supervise the LX start location as an EOA and SVL | N/A | N/A | - |
| 13 | N/A | N/A | NOT The Level symbol "Level Crossing" is displayed | O | DMI | - | N/A | N/A | - |

| Step 4: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 88 | Level crossing Information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LX | 8 | FINITE VALUE | - |
| D_LX | 15 | FINITE VALUE | - |
| L_LX | 15 | FINITE VALUE | - |
| Q_LXSTATUS | 1 | 1 | - |
| V_LX | 7 | FINITE VALUE | Only if Q _LXSTATUS=1 |
| Q_STOPLX | 1 | 1 | Only if Q _LXSTATUS=1 |
| L_STOPLX | 15 | FINITE VALUE | Only if Q _LXSTATUS=1 |

| Step 8: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | Acknowledgement of Train Data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| Step 10: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | New |
| Gradient Profile | STORED | New |
| International SSP | STORED | New |
| Mode Profile | STORED | Only for LS/ OS mode |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| Train is moving | I | INT | - |

## Test case 29

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Inhibition of revocable TSRs from balises in L2/3 shall be accepted or rejected by on-board equipment according to 4.8.4 table. Tested action: A and R of Packet 64 by mode |  | 29 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with packet 64 Inhibition of revocable TSRs from balises in level 2 or 3 and mode PT because "Recognition of Exit from TR mode" has not been received The on-board equipment change to FS mode. The permitted speed to be displayed is V_TSR. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.16.1 Packet Number 64: Inhibition of revocable TSRs from balises in L2/3 |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Mode symbol for FS is displayed. Permitted speed is displayed and is V_TSR. Information from packet 65 is used by on-board equipment. Check on the JRU that: Reception of packet 64 is recorded. Mode change to FS is recorded. Reception of packet 65 is recorded Permitted speed , V_TSR is recorded. |
| Constraints | In level 2/3 "Recognition of Exit from TR mode" has not been received yet. V_TSR shall be most restrictive than V_MAIN |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 (Q_DIR=1) is received | I | RTM | The packet is rejected. | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | Train trip procedure to enter FS mode | - | - | USE_FT5110200.5 According to table 4.10.1.3 the Inhibition of TSRs info remains unchanged after mode change to FS | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | Train is moving | I | INT | V_MAIN | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | One Balise Group message (N_TOTAL=1) containing packet 65 (Q_DIR=1) is received | I | BTM | The packet is accepted. | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2 / L3 | FS | - |
| 7 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V_TSR in stored message will be used by the on-board equipment packet 64 has been rejected in PT mode. Permitted speed change to the V_TSR | L2 / L3 | FS | - |
| 8 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <V_TSR>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 64 | Inhibition of revocable TSRs from balises in level 2 or 3. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | 0-126 | Value reserved for non RBC transmission (balise, loop or radio Infill) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_TSR |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 30

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Temporary speed Restriction shall be rejected by on-board equipment according to 4.8.4 table. Tested action: Packet 65 rejected by mode |  | 30 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with packet 65 because "Recognition of Exit from TR mode" has not been received The on-board equipment change to FS mode. The permitted speed to be displayed is not V_TSR. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Temporary Speed restriction |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-4.8.4.2 Temporary Speed restriction |  |
|  |  | Subset-026-7.4.2.17 Packet Number 65: Temporary Speed Restriction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Mode symbol changes to FS Permitted speed is displayed and is not V_TSR. Information from packet 65 is not used by the on-board equipment. Check on the JRU that: Reception of packet 65 is recorded . Mode change to FS is recorded. Permitted speed , V_MAIN is recorded. |
| Constraints | In level 2/3 "Recognition of Exit from TR mode" has not been received yet. Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received. V_TSR shall be most restrictive than V_MAIN. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 (Q_DIR=1) is received | I | RTM | The packet is rejected. Recognition of Exit from TR mode not yet received. | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | Train trip procedure to enter FS mode | - | - | USE_FT5110200.5 According to table 4.10.1.3 the Temporary Speed Restriction remains unchanged after mode change to FS | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | Train is moving | I | INT | V_MAIN | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | Permitted speed not updated to V_TSR | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <V_MAIN>) is recorded | O | JRU | - | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 65 | Temporary Speed Restriction |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_TSR | 8 | 0-126 | Value reserved for non RBC transmission (balise, loop or radio Infill) |
| D_TSR | 15 | FINITE VALUE | - |
| L_TSR | 15 | FINITE VALUE | - |
| Q_FRONT | 1 | FINITE VALUE | - |
| V_TSR | 7 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed | O | DMI | V_MAIN |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 31

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.4 table. Tested Action: R in PT mode |  | 31 |
| Applicable Mode/Level Combinations | L2: PT L3: PT |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with Permitted Braking distance information in level 2 or 3 and mode PT because "Recognition of Exit from TR mode" has not been received. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.4.2 [1] |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Speed and distance monitoring information is not updated. Information from packet 52 is not used by the on-board equipment. Check on the JRU that: Reception of packet 52 is recorded . |
| Constraints | In level 2/ 3 "Recognition of Exit from TR mode" has not been received. Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has been received. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | Ack. received (Message 8) |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | PT | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 (Q_DIR=1) is received | I | RTM | The packet is rejected | L2 / L3 | PT | - |
| 2 | L2 / L3 | PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | PT | - |
| 3 | L2 / L3 | PT | The on-board performs a mode change to FS | - | - | USE_FT4060300.20 | L2 / L3 | FS | - |
| 4 | L2 / L3 | FS | Train is moving. | I | INT | - | L2 / L3 | FS | - |
| 5 | L2 / L3 | FS | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V from braking distance in stored message will not be used by the on-board equipment. No modification of MRSP Permitted speed does not changed. | L2 / L3 | FS | - |
| 6 | L2 / L3 | FS | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded | O | JRU | JRU message is not recorded | L2 / L3 | FS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking distance information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT=1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT=0, D_PBD and the following variables |
| Q_GDIR | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Radio communication session | ESTABLISHED | - |
| Train Data | VALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Permitted Braking Distance Information | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 32

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested Action: R[2] from RTM |  | 32 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / LS / OS |  |  |
| Target of Test | The target of this test case is to check that: The on-board accepts the message from RBC with Permitted Braking distance information because an order to L2/L3 at a further location has been received |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |
|  |  | Subset-026-7.4.2.13.1 Packet Number 52: Permitted Braking Distance Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The speed and distance monitoring information is displayed. Information from packet 52 is used by the on-board equipment. Check on the JRU that: Reception of packet 52 is RECORDED. |
| Constraints | Packet 52 accepted only if an order to L2/L3 at a further location has been received D_PBD should give the MRSP For LS mode the monitoring of Permitted braking distance information shall be checked with an Overspeed status, since V_PERM and V_TARGET are not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 4/ 12/ 13 | FS / OS / UN / LS / SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | To level 2/3 at further location |
| ERTMS/ETCS level | 0 / 1 / 2 | L0 / LNTC/ L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC/ L1 | FS/ OS/ UN/ LS/ SN | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 (Q_DIR=1) is received | I | RTM | The packet is accepted | L0 / LNTC/ L1 | FS/OS/UN/LS/SN | - |
| 2 | L0 / LNTC/ L1 | FS/ OS/ UN/ LS/ SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0 / LNTC/ L1 | FS/OS/UN/LS/SN | - |
| 3 | L0 / LNTC/ L1 | FS/ OS/ UN/ LS/ SN | The on-board performs a level transition to L2 | - | - | USE_FT5100100.37: Transition from L1 to L2 or USE_FT5100100.21: Transition from LNTC to L2 Or USE_FT5100100.13: Transition from L0 to L2 | L2 | FS/ OS/ LS | - |
| 4 | L2 | FS/ OS/ LS | Train is moving. | I | INT | - | L2 | FS/ OS/ LS | - |
| 5 | L2 | FS/ OS/ LS | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS and SR modes | L2 | FS/ OS/ LS | - |
| 6 | L2 | FS/ OS/ LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS and SR modes | L2 | FS/ OS/ LS | - |
| 7 | L2 | FS/ OS/ LS | The speed and distance monitoring information is displayed | O | DMI | Only if not in LS mode: The calculated speed restriction is shown in the Planning Area Speed Profile (PASP). V_TARGET is displayed | L2 | FS/ OS/ LS | - |
| 8 | L2 | FS/ OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_TARGET= <V_MRSP>) is recorded | O | JRU | Only if not in LS mode: V_TARGET= <V_MRSP> is the speed restriction calculated to ensure the permitted braking distance given in packet 52 | L2 | FS/ OS/ LS | - |
| 9 | L2 | FS/ OS/ LS | The speed and distance monitoring information is displayed | O | DMI | The train has passed the start location (D_PBSR) of the speed restriction to ensure permitted braking distance Only if not in LS mode: The DMI aspect is coherent with Normal Status in a Ceiling speed monitoring section and the permitted speed, which is given by the speed restriction to ensure the permitted braking distance, is shown. Only if in LS mode: The DMI aspect is coherent with Overspeed Status in a Ceiling speed monitoring section. The train speed will be a bit over the PS. | L2 | FS/ OS/ LS | - |
| 10 | L2 | FS/ OS/ LS | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; V_PERM=<V_MRSP>) is recorded | O | JRU | V_MRSP is the speed restriction calculated to ensure the permitted braking distance given in packet 52. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if not in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V_TRAIN will be used instead of V_PERM | L2 | FS/ OS/ LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking distance information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT=1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT=0, D_PBD and the following variables |
| Q_GDIR | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS / OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Permitted Braking Distance Information | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 33

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Permitted Braking distance information shall be accepted or rejected by on-board equipment according to 4.8.3 table. Tested action: A[3] from RTM |  | 33 |
| Applicable Mode/Level Combinations | L2: FS/ LS/ OS / SR L3: FS/ LS/ OS / SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board rejects the message from RBC with Permitted Braking distance information in level 2 or 3 because “ Acknowledgment of Train Data " has not been received. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Permitted Braking Distance Information |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |
|  |  | Subset-026-4.8.4.2 Permitted Braking Distance Information |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The Speed and distance monitoring information is not updated. Information from packet 52 is not used by the on-board equipment. Check on the JRU that: Reception of packet 52 is RECORDED. |
| Constraints | Train data has been sent and no “Acknowledgment of Train Data " has been received.(Message 8 not received) D_PBD should give the MRSP For LS mode, V_TRAIN shall be used to check that the speed restriction is not apply, since V_PERM is not recorded. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver modifies the "Length" and validates the "Flexible Train Data" | I | DMI | Driver modifies the length of the train | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0001 0101) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_PAQUET=11) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 52 (Q_DIR=1) is received | I | RTM | The packet is rejected because no “Acknowledgment of Train Data " has been received yet | N/A | N/A | - |
| 7 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 8 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | Only applicable for OS/ SR modes | N/A | N/A | - |
| 9 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only applicable for OS/ SR modes | N/A | N/A | - |
| 10 | N/A | N/A | The speed and distance monitoring information is displayed | O | DMI | Speed value defined as V from braking distance in stored message will not be used by the on-board equipment. For FS/OS/SR: Actual speed does NOT change. For LS mode: V_TRAIN will be used instead of V_TARGET and shall be greater than any given speed restriction to ensure the permitted braking distance. | N/A | N/A | - |

| Step 4: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | Validated train data |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 11 | Validated Train Data |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 52 | Permitted Braking distance information |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TRACKINIT | 1 | FINITE VALUE | - |
| D_TRACKINIT | 15 | FINITE VALUE | Only if Q_TRACKINIT=1 |
| D_PBD | 15 | FINITE VALUE | Only if Q_TRACKINIT=0, D_PBD and the following variables |
| Q_GDIR | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR | 1 | FINITE VALUE | - |
| D_PBDSR | 15 | FINITE VALUE | - |
| L_PBDSR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_PBD(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0=downhill, 1=uphill |
| G_PBDSR(k) | 8 | FINITE VALUE | Gradient value to be used for the calculation |
| Q_PBDSR(k) | 1 | FINITE VALUE | - |
| D_PBDSR(k) | 15 | FINITE VALUE | - |
| L_PBDSR(k) | 15 | FINITE VALUE | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS / SR / LS |
| Radio communication session | ESTABLISHED | - |
| Train Data | INVALID | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Permitted Braking Distance Information | NOT STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 34

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Generic LS function marker and LSSMA display toggle on order shall be accepted by on-board equipment according to 4.8.4.2 table. Tested Action: A[9] from BTM for LSSMA display toggle on information and A from BTM for Generic LS function marker information. |  | 34 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L2 / L3: FS/ OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board ACCEPTS the message from BTM with Generic LS function marker information in all levels and modes and ACCEPTS LSSMA display toggle on order information in level 0/ NTC/ 2/ 3 because the condition "the max safe front of the train is inside de LS area of a mode profile received in the same message" is fulfilled. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.2 |  |
|  |  | Subset-026-4.4.19.1.4.2 c) |  |
|  |  | Subset-026-4.4.19.1.4.3 |  |
|  |  | Subset-026-4.4.19.1.4.6 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 Generic LS function marker |  |
|  |  | Subset-026-4.8.4.2 Generic LS function marker |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-4.8.4.2 [9] |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA is displayed. Check on the JRU that:. Mode change to LS is recorded. Level transition to L1 is recorded. |
| Constraints | For the on-board equipment to accepts the information “LSSMA display toggle on order“ an order to switch to L1 at a further location shall be stored. LS mode is requested for current location. (D_MAMODE= 0) For LSSMA to be displayed by the onboard, T_LSSMA shall be greater that the time-out value given by trackside (passage over the Balise group) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/ 13 | FS/ OS/ SR/ UN/ SN |
| Level Transition Order | STORED | L1 at further location |
| ERTMS/ETCS level | 0/ 1/ 3 / 4 | L0/ LNTC / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | Train is moving. | I | INT | Train location: in rear of L1 area | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | - |
| 2 | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | One Balise Group message (N_TOTAL=1) containing packet 12, packet 80, packet 41, packet 180 and packet 181 is received. | I | BTM | Information from packet 180 is Accepted. Information from packet 181 is Accepted. | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | - |
| 3 | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | - |
| 4 | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | Train is moving. | I | INT | Train location: For LS, SR; UN and SN the max safe front end of the train is inside the LS area as defined in packet 80. For OS the max safe front end of the train is inside the OS area and the estimate front end of the train is not inside an LS Ack area. | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | - |
| 5 | L0/ LNTC/ L2/ L3 | FS/ OS/SR/UN/SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12; M_LEVEL=2) is recorded | O | JRU | Transition to L1 and Mode change to LS | L1 | LS | - |
| 6 | L1 | LS | The symbol “Acknowledgment for Limited supervision” is displayed | O | DMI | Yellow DMI Object: MO22 | L1 | LS | - |
| 7 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L1 | LS | - |
| 8 | L1 | LS | The driver acknowledges “Limited Supervision” | I | DMI | The driver acknowledges the request of LS mode within the driver acknowledgment time (5 s). | L1 | LS | - |
| 9 | L1 | LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1101) is recorded | O | JRU | - | L1 | LS | - |
| 10 | L1 | LS | The Mode symbol “Limited Supervision” is displayed | O | DMI | Grey DMI Object: MO21 | L1 | LS | - |
| 11 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit36=1>) is recorded | O | JRU | - | L1 | LS | - |
| 12 | L1 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | DMI Object: Symbol LS01 (The on-board has changed to LS mode since T_LSSMA) | L1 | LS | - |
| 13 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 | LS | - |
| 14 | L1 | LS | The LSSMA number is displayed. | O | DMI | The EVC displays the LSSMA speed value | L1 | LS | - |
| 15 | L1 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | Level 1 Movement Authority |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to “no time-out”. |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | - |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | Level Transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 0 | - |
| M_LEVELTR | 3 | 2 | To level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | FINITE VALUE | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 181 | Generic LS function Marker |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 35

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be accepted by on-board equipment according to 4.8.3 table. Tested Action: R[2]= A from RBC for LSSMA display toggle on information |  | 35 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS/ OS/ SR |  |  |
| Target of Test | The target of this test case is to check that: The on-board ACCEPTS the LSSMA display toggle on order information in level 0/ NTC/ 1 because the condition "an order to switch to L2/L3 at a further location has been received " is fulfilled. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.2 |  |
|  |  | Subset-026-4.4.19.1.4.2 c) |  |
|  |  | Subset-026-4.4.19.1.4.3 |  |
|  |  | Subset-026-4.4.19.1.4.6 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-4.8.4.2 [9] |  |
|  |  | Subset-026-7.4.2.9 Packet Number 41: Level Transition Order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA is displayed. Check on the JRU that:. Mode change to LS is recorded. Level transition to L2/ L3 is recorded. |
| Constraints | For the on-board equipment to accepts the information “LSSMA display toggle on order“ in the applicable modes the condition “the max safe front of the train is inside de LS area of a mode profile received in the same message” shall be fulfilled. LS mode is requested for current location. (D_MAMODE= 0) For LSSMA to be displayed by the onboard, T_LSSMA shall be greater that the time-out value given by trackside (passage over the Balise group) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 4/ 13 | FS/ OS/ SR/ UN/ SN |
| Radio communication session | ESTABLISHED | - |
| Level Transition Order | STORED | L2/ L3 at further location |
| ERTMS/ETCS level | 0/ 1/ 2 | L0/ LNTC / L1 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | Train is moving. | I | INT | Train location: in rear of L2/L3 area | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 2 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 (Q_DIR=1), packet 21, packet 27, packet 80 (D_MAMODE=0) and packet 180 (Q_LSSMA=1) is received. | I | RTM | Information from packet 180 is Accepted (Toggled on order) | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 3 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 4 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | Train is moving. | I | INT | Train location: For LS, SR; UN and SN the max safe front end of the train is inside the LS area as defined in packet 80. For OS the max safe front end of the train is inside the OS area and the estimate front end of the train is not inside an LS Ack. area. | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 5 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | One Balise Group message (N_TOTAL=1) containing packet 41(D_LEVELTR=0) is received. | I | BTM | Level transition to L2/L3 is executed | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 6 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | - |
| 7 | L0/ LNTC/ L1 | FS/ OS/SR/UN/SN | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12; M_LEVEL=3/4) is recorded | O | JRU | Transition to L2/L3 and Mode change to LS | L2 / L3 | LS | - |
| 8 | L2 / L3 | LS | The symbol “Acknowledgment for Limited supervision” is displayed | O | DMI | Yellow DMI Object: MO22 | L2 / L3 | LS | - |
| 9 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 10 | L2 / L3 | LS | The driver acknowledges “Limited Supervision” | I | DMI | The driver acknowledges the request of LS mode within the driver acknowledgment time (5 s). | L2 / L3 | LS | - |
| 11 | L2 / L3 | LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 1101) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 12 | L2 / L3 | LS | The Mode symbol “Limited Supervision” is displayed | O | DMI | Grey DMI Object: MO21 | L2 / L3 | LS | - |
| 13 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit36=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 14 | L2 / L3 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | DMI Object: Symbol LS01 (The on-board has changed to LS mode since T_LSSMA) | L2 / L3 | LS | - |
| 15 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 16 | L2 / L3 | LS | The LSSMA number is displayed. | O | DMI | The EVC displays the LSSMA speed value | L2 / L3 | LS | - |
| 17 | L2 / L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2 / L3 | LS | - |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Movement Authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Level 2/3 Movement Authority |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | Gradient Profile |
| Q_DIR | 2 | 1 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | International Static Speed Profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | - |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | - |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | Level transition Order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | 32767 | Now |
| M_LEVELTR | 3 | 3 / 4 | L2/ L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR=1 |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR (k) | 3 | FINITE VALUE | - |
| NID_NTC (k) | 8 | FINITE VALUE | If M_LEVELTR=1 |
| L_ACKLEVELTR (k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 36

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Generic LS function marker and LSSMA display toggle on order shall be accepted by on-board equipment according to tables 4.8.3 and 4.8.4.2. Tested Action: A from BTM for LSSMA display toggle on information and A from BTM for Generic LS function marker information. |  | 36 |
| Applicable Mode/Level Combinations | L1: LS |  |  |
| Target of Test | The target of this test case is to check that: The on-board ACCEPTS the message from BTM with Generic LS function marker information and with LSSMA display toggle on order information |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.2 |  |
|  |  | Subset-026-4.4.19.1.4.2 b) |  |
|  |  | Subset-026-4.4.19.1.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 Generic LS function marker |  |
|  |  | Subset-026-4.8.4.2 Generic LS function marker |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA speed and Symbol LS01 are displayed. Check on the JRU that:. Reception of packet 181 from BTM is recorded. Reception of packet 180 from BTM is recorded. LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (LSSMA) is recorded. |
| Constraints | For LSSMA to be displayed by the on-board equipment , the delay timer to toggle on the LSSMA display (T_LSSMA) shall be greater that the time-out value given by trackside (passage over the first encountered balise of the Balise group giving the order) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 181 is received. | I | BTM | Information from packet 181 is Accepted. | L1 | LS | - |
| 2 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | LS | - |
| 3 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1) is received. | I | BTM | Information from packet 180 is Accepted. Toggle on order | L1 | LS | - |
| 4 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | LS | - |
| 5 | L1 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | DMI Object: Symbol LS01 (The on-board pass over the first balise of the BG containing packet 180 since T_LSSMA) | L1 | LS | - |
| 6 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 | LS | - |
| 7 | L1 | LS | The LSSMA number is displayed. | O | DMI | The EVC displays the LSSMA speed value | L1 | LS | - |
| 8 | L1 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 181 | Generic LS function Marker |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 3: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 37

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be Accepted by on-board equipment according to tables 4.8.3. and 4.8.4.2. Tested Action: A from RBC for LSSMA display toggle on information |  | 37 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment ACCEPTS the message from RBC with LSSMA display toggle on order information |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.2 |  |
|  |  | Subset-026-4.4.19.1.4.2 a) |  |
|  |  | Subset-026-4.4.19.1.4.3 |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA speed and Symbol LS01 are displayed. Check on the JRU that: Reception of packet 180 from RTM is recorded. LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (LSSMA) is recorded. |
| Constraints | Acknowledgment of the Train Data (Message 8) has been received and stored on-board. No pending Emergency Stop(s) stored on-board. For LSSMA to be displayed by the on-board equipment , the delay timer to toggle on the LSSMA display (T_LSSMA) shall be greater that the time-out value given by trackside (for RBC the value of the time stamp of the message including the order) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | Information from packet 180 is Accepted. (Toggled on order) | L2 / L3 | LS | - |
| 2 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 3 | L2 / L3 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | Delay timer value becomes greater than the time-out value given by trackside so, DMI Object: Symbol LS01 (The on-board pass over the first balise of the BG containing packet 180 since T_LSSMA) | L2 / L3 | LS | - |
| 4 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 5 | L2 / L3 | LS | The LSSMA number is displayed. | O | DMI | The EVC displays the LSSMA speed value . (The LSSMA number shall overlay the symbol LS01) | L2 / L3 | LS | - |
| 6 | L2 / L3 | LS | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2 / L3 | LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 38

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle off order shall be accepted by on-board equipment according to tables 4.8.3 and 4.8.4.2. Tested Action: A from BTM for LSSMA display toggle off information |  | 38 |
| Applicable Mode/Level Combinations | L1: LS |  |  |
| Target of Test | The target of this test case is to check that: The on-board ACCEPTS the message from BTM with LSSMA display toggle off order information |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.5 |  |
|  |  | Subset-026-4.4.19.1.4.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle off order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle off order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA display is removed. Absence of symbol LS01 Check on the JRU that:. LSSMA toggle off order is recorded. |
| Constraints | The Generic LS function marker information shall be stored onboard. The delay timer to toggle on the LSSMA display (T_LSSMA) shall be greater that the time-out value given by trackside (passage over the first encountered balise of the Balise group giving the order) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | DMI Object: Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | LS | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 0) is received. | I | BTM | Information from packet 180 is Accepted. Toggle off order | L1 | LS | - |
| 2 | L1 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L1 | LS | - |
| 3 | L1 | LS | The Limited Supervision symbol "Limited Supervision frame" is removed. | O | DMI | DMI Object: Absence of Symbol | L1 | LS | - |
| 4 | L1 | LS | The LSSMA number is removed. | O | DMI | DMI Object: Absence of Symbol | L1 | LS | - |
| 5 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=0>) is recorded | O | JRU | - | L1 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGb |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of symbol |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 39

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle off order shall be accepted by on-board equipment according to tables 4.8.3 and 4.8.4.2. Tested Action: A from RBC for LSSMA display toggle off information |  | 39 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment ACCEPTS the message from RBC with LSSMA display toggle off order information |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.4.5 |  |
|  |  | Subset-026-4.4.19.1.4.5 b) |  |
|  |  | Subset-026-4.5.2.1 table - Manage LSSMA display to the driver |  |
|  |  | Subset-026-4.8.3.1.1 LSSMA display toggle off order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle off order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: LSSMA display is removed. (Absence of symbol LS01) Check on the JRU that:. Reception of a telegram with packet 180 containing an toggle off order is recorded. |
| Constraints | The Generic LS function marker information shall be stored onboard. The delay timer to toggle on the LSSMA display (T_LSSMA) shall be greater that the time-out value given by trackside (passage over the first encountered balise of the Balise group giving the order) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | DMI Object: Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=0) is received | I | RTM | Information from packet 180 is Accepted. (Toggled off order) | L2 / L3 | LS | - |
| 2 | L2 / L3 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2 / L3 | LS | - |
| 3 | L2 / L3 | LS | The Limited Supervision symbol "Limited Supervision frame" is removed. | O | DMI | DMI Object: Absence of Symbol | L2 / L3 | LS | - |
| 4 | L2 / L3 | LS | The LSSMA number is removed. | O | DMI | DMI Object: Absence of Symbol | L2 / L3 | LS | - |
| 5 | L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=0>) is recorded | O | JRU | - | L2 / L3 | LS | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | Toggle off |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | Absence of symbol |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 40

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Generic LS function marker shall be accepted by on-board equipment according to tables 4.8.3 and 4.8.4.2. Tested Action: A from BTM in all modes and levels |  | 40 |
| Applicable Mode/Level Combinations | L0: UN/ TR LNTC: SN/ TR L1: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL/ RV L2/ L3: FS/ LS/ OS/ SR/ SH/ PS/ SL/ TR/ NL/ RV |  |  |
| Target of Test | The target of this test case is to check that: The on-board ACCEPTS the message from BTM with Generic LS function marker information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Generic LS function marker |  |
|  |  | Subset-026-4.8.4.2 Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that:. Reception of Generic LS function marker from BTM is recorded. |
| Constraints | The TC description tests only that the information is received and stored by the onboard but doesn´t covers the usage of this information by the on-board equipment. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 /3 / 4/ 5/ 7/ 11/ 12/13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/TR/ NL/ LS/ SN/ PS/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ PS/ RV | One Balise Group message (N_TOTAL=1) containing packet 181 is received. | I | BTM | Information from packet 181 is Accepted. | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ PS/ RV | - |
| 2 | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ PS/ RV | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L0/ LNTC/ L1/ L2/ L3 | FS/ OS/ SR/ SH/ UN/ SL/ SB/ TR/ PT/ NL/ LS/ SN/ PS/ RV | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 181 | Generic LS function Marker |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | i / n |
| N_TOTAL | 3 | FINITE VALUE | n balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | - |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 /3 / 4/ 5/ 7/ 11/ 12/13/ 14/ 15 | FS/ OS/ SR/ SH/ UN/ SL/TR/ NL/ LS/ SN/ PS/ RV |
| ERTMS/ETCS level | 0/ 1/ 2/ 3/ 4 | L0/ LNTC/ L1/ L2/ L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 41

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when the information is received via BG, according to the 4.8.3.1 table. Tested Action: R[1] from BTM |  | 41 |
| Applicable Mode/Level Combinations | L2 / L3: FS / OS / SR / LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored onboard, it REJECTS the LSSMA display toggle on order information in level 2/3 if it is received via BG, because no announcement to change to L1 is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (after changing to L1 and LS mode, if this is necessary): - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that (after changing to L1 and LS mode, if this is necessary): - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | For the on-board equipment to REJECT the information “LSSMA display toggle on order“ an order to switch to L1 at a further location cannot be stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Level Transition Order | NOT STORED | L1 at further location |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS/ OS/SR/LS | Train is moving. | I | INT | - | L2/ L3 | FS/ OS/SR/LS | - |
| 2 | L2/ L3 | FS/ OS/SR/LS | One Balise Group message containing packet 12, packet 21, packet 27, packet 80, packet 181, packet 180 (Q_LSSMA=1) is received. | I | BTM | The information from packet 180 is Rejected. | L2/ L3 | FS/ OS/SR/LS | - |
| 3 | L2/ L3 | FS/ OS/SR/LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/ L3 | FS/ OS/SR/LS | - |
| 4 | L2/ L3 | FS/ OS/SR/LS | USE_FT5190300.1 (for Transition [70]) or USE_FT5190300.2 (for Transition [72]) or USE_FT5190200.6 (for Transition [74]) | - | - | If not in LS mode: the on-board equipment changes to LS mode. | L2/ L3 | LS | - |
| 5 | L2/ L3 | FS/ OS/SR/LS | USE_FT5100100.32 (if in L2) USE_FT5100100.28 (if in L3) | - | - | The on-board equipment changes to L1. | L1 | LS | - |
| 6 | L1 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L1 | LS | - |
| 7 | L1 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 | LS | - |
| 8 | L1 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L1 | LS | - |
| 9 | L1 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1 | LS | - |

| Step 2: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 12 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 181 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | 0 | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 42

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle off order shall be rejected by the on-board equipment when the information is received via BG, according to the 4.8.3.1 table. |  | 42 |
| Applicable Mode/Level Combinations | L2 / L3: LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board and the LSSMA is already displayed on the DMI, it REJECTS the LSSMA display toggle off order information in level 2/3 if it is received via BG. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle off order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle off order |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | LS | Train is moving. | I | INT | - | L2/ L3 | LS | - |
| 2 | L2/ L3 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is displayed. No JRU message is recorded because the symbol was displayed before this location. | L2/ L3 | LS | - |
| 3 | L2/ L3 | LS | The LSSMA number is displayed. | O | DMI | The EVC displaies the LSSMA speed value. No JRU message is recorded because the value was displayed before this location. | L2/ L3 | LS | - |
| 4 | L2/ L3 | LS | One Balise Group message containing packet 41 (M_LEVELTR=2), packet 180 (Q_LSSMA=0) is received. | I | BTM | The toggle off order from packet 180 is Rejected (even if, as in this case, it is received together with an order to switch to L1). No Limited Supervision mode profile is sent in this message, because the generic LS function marker should be deleted, according to 4.4.19.1.4.8. | L2/ L3 | LS | - |
| 5 | L2/ L3 | LS | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/ L3 | LS | - |
| 6 | L2/ L3 | LS | The Level symbol "Level 1" is displayed | O | DMI | This will happen when the train reaches the announced location. | L1 | LS | - |
| 7 | L1 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03=1>) is recorded | O | JRU | The displayed new symbol is recorded. | L1 | LS | - |
| 8 | L1 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The on-board equipment does not toggle off the LSSMA symbol. The DMI Object: Symbol LS01 is displayed. No JRU message is recorded because the symbol was displayed before this location. | L1 | LS | - |
| 9 | L1 | LS | The LSSMA number is displayed. | O | DMI | The on-board equipment does not toggle off the LSSMA number. The EVC displaies the LSSMA speed value. No JRU message is recorded because the value was displayed before this location. | L1 | LS | - |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | Greater than zero. |
| M_LEVELTR | 3 | 2 | Change to Level 1 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | Toggle off |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 43

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when the information is received via RBC, according to the 4.8.3.1 table. |  | 43 |
| Applicable Mode/Level Combinations | L1: FS / OS / SR / LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board, it REJECTS the LSSMA display toggle on order information in level 1 if it is received via radio (RBC), because no announcement to change to L2/3 is stored on-board. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that (after changing to L1 and LS mode, if this is necessary): - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that (after changing to L1 and LS mode, if this is necessary): - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | For the on-board equipment to REJECT the information “LSSMA display toggle on order“ an order to switch to L2/3 at a further location cannot be stored. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| Level Transition Order | NOT STORED | L2/3 at further location |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | FS/ OS/SR/LS | USE_FT4080300.40 | - | - | The on-board recives and accepts the Generic LS function marker (Packet 181) | L1 | FS/ OS/SR/LS | - |
| 2 | L1 | FS/ OS/SR/LS | Train is moving. | I | INT | - | L1 | FS/ OS/SR/LS | - |
| 3 | L1 | FS/ OS/SR/LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | The information from packet 180 is Rejected. | L1 | FS/ OS/SR/LS | - |
| 4 | L1 | FS/ OS/SR/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | FS/ OS/SR/LS | - |
| 5 | L1 | FS/ OS/SR/LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27, packet 80 (M_MAMODE=10), packet 180 (Q_LSSMA=1) is received | I | RTM | Only if NOT in LS mode. The information from packet 180 is Rejected. The toggle on order would be rejected for FS, OS and SR mode if the max safe front end of the train was not inside the Limited Supervision area of a mode profile received in the same message (see the exception [9] in 4.8.4.2) | L1 | FS/ OS/SR/LS | - |
| 6 | L1 | FS/ OS/SR/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if NOT in LS mode. | L1 | FS/ OS/SR/LS | - |
| 7 | L1 | FS/ OS/SR/LS | USE_FT5190300.1 (for Transition [70]) or USE_FT5190300.2 (for Transition [72]) or USE_FT5190200.6 (for Transition [74]) | - | - | If not in LS mode: the on-board equipment changes to LS mode. The generic LS function marker (Packet 181) will be sent in the same BG message (see the requirement 4.4.19.1.4.8). | L1 | LS | - |
| 8 | L1 | FS/ OS/SR/LS | USE_FT5100100.37 (for changing to L2) USE_FT5100100.36 (or changing to L3) | - | - | The on-board equipment changes to L2/3. | L2/ L3 | LS | - |
| 9 | L2/ L3 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L2/ L3 | LS | - |
| 10 | L2/ L3 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2/ L3 | LS | - |
| 11 | L2/ L3 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L2/ L3 | LS | - |
| 12 | L2/ L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2/ L3 | LS | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 5: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | 10 | LS |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Mode Profile | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 44

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle off order shall be rejected by the on-board equipment when the information is received via radio (RBC), according to the 4.8.3.1 table. |  | 44 |
| Applicable Mode/Level Combinations | L1: LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board and the LSSMA is already displayed on the DMI, it REJECTS the LSSMA display toggle off order information in level 1 if it is received via radio (RBC). |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle off order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle off order |  |
|  |  | Subset-026-7.4.2.37.4 Packet Number 181: Generic LS function marker |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 2 | L1 |
| Generic LS function marker | STORED | - |
| LSSMA display toggle on order | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 | LS | Train is moving. | I | INT | - | L1 | LS | - |
| 2 | L1 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is displayed. No JRU message is recorded because the symbol was displayed before this location. | L1 | LS | - |
| 3 | L1 | LS | The LSSMA number is displayed. | O | DMI | The EVC displays the LSSMA speed value. No JRU message is recorded because the value was displayed before this location. | L1 | LS | - |
| 4 | L1 | LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 41 (M_LEVELTR=3¦4), packet 180 (Q_LSSMA=0) is received | I | RTM | The toggle off order from the packet 180 is Rejected (even if, as in this case, it is received together with an order to switch to L2/3). | L1 | LS | - |
| 5 | L1 | LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L1 | LS | - |
| 6 | L1 | LS | The Level symbol "Level 2" is displayed | O | DMI | This will happen when the train reaches the announced location. "Level 3" instead of "Level 2" will be displayed if M_LEVELTR=4 before. | L2/ L3 | LS | - |
| 7 | L2/ L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit04=1>) is recorded | O | JRU | The displayed new symbol is recorded. Bit05=1 instead of Bit04=1 will be recorded if the previous packet 41 was sent with M_LEVELTR=4. | L2/ L3 | LS | - |
| 8 | L2/ L3 | LS | The Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The on-board equipment does not toggle off the LSSMA symbol. The DMI Object: Symbol LS01 is displayed. No JRU message is recorded because the symbol was displayed before this location. | L2/ L3 | LS | - |
| 9 | L2/ L3 | LS | The LSSMA number is displayed. | O | DMI | The on-board equipment does not toggle off the LSSMA number. The EVC displays the LSSMA speed value. No JRU message is recorded because the value was displayed before this location. | L2/ L3 | LS | - |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 41 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_LEVELTR | 15 | FINITE VALUE | - |
| M_LEVELTR | 3 | 3 | This value can be 4 (L3) instead. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTR = 1 (NTC). |
| L_ACKLEVELTR | 15 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_LEVELTR(k) | 3 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | If M_LEVELTR(k) = 1 (NTC). |
| L_ACKLEVELTR(k) | 15 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 0 | Toggle off |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 3/4 | L2/L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is displayed. | O | DMI | Symbol LS01 toggled on |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 45

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when the Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has not been received yet. |  | 45 |
| Applicable Mode/Level Combinations | L2: FS / OS / SR / LS L3: FS / OS / SR / LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board, it REJECTS the LSSMA display toggle on order information in level 2/3 if it is received via radio (RBC), because the Train Data has been sent to the RBC and the “Acknowledgement of Train Data” has not been received yet. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |
|  |  | Subset-026-4.8.3.1.1 Exception [3] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 12 | FS/ OS/ SR/ LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS/ OS/SR/LS | Train is moving. | I | INT | - | L2/ L3 | FS/ OS/SR/LS | - |
| 2 | L2/ L3 | FS/ OS/SR/LS | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - | L2/ L3 | FS/ OS/SR/LS | - |
| 3 | L2/ L3 | FS/ OS/SR/LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/ L3 | FS/ OS/SR/LS | - |
| 4 | L2/ L3 | FS/ OS/SR/LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | Only if in LS mode. The information from packet 180 is Rejected. | L2/ L3 | FS/ OS/SR/LS | - |
| 5 | L2/ L3 | FS/ OS/SR/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if in LS mode. | L2/ L3 | FS/ OS/SR/LS | - |
| 6 | L2/ L3 | FS/ OS/SR/LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27, packet 80 (M_MAMODE=10), packet 180 (Q_LSSMA=1) is received | I | RTM | Only if NOT in LS mode. The information from packet 180 is Rejected. The toggle on order would be rejected for FS, OS and SR mode if the max safe front end of the train was not inside the Limited Supervision area of a mode profile received in the same message (see the exception [9] in 4.8.4.2) | L2/ L3 | FS/ OS/SR/LS | - |
| 7 | L2/ L3 | FS/ OS/SR/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if NOT in LS mode. | L2/ L3 | FS/ OS/SR/LS | - |
| 8 | L2/ L3 | N/A | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - | L2/ L3 | FS/ OS/SR/LS | - |
| 9 | L2/ L3 | FS/ OS/SR/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/ L3 | FS/ OS/SR/LS | - |
| 10 | L2/ L3 | FS/ OS/SR/LS | USE_FT5190300.1 (for Transition [70]) or USE_FT5190300.2 (for Transition [72]) or USE_FT5190200.6 (for Transition [74]) | - | - | If not in LS mode: the on-board equipment changes to LS mode. | L2/ L3 | LS | - |
| 11 | L2/ L3 | FS/ OS/SR/LS | USE_FT5100100.37 (for changing to L2) USE_FT5100100.36 (or changing to L3) | - | - | The on-board equipment changes to L2/3. | L2/ L3 | LS | - |
| 12 | L2/ L3 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L2/ L3 | LS | - |
| 13 | L2/ L3 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2/ L3 | LS | - |
| 14 | L2/ L3 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L2/ L3 | LS | - |
| 15 | L2/ L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L2/ L3 | LS | - |

| Step 2: Radio Message 129 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 129 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 11 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NC_CDTRAIN | 4 | FINITE VALUE | - |
| NC_TRAIN | 15 | FINITE VALUE | - |
| L_TRAIN | 12 | FINITE VALUE | - |
| V_MAXTRAIN | 7 | FINITE VALUE | - |
| M_LOADINGGAUGE | 8 | FINITE VALUE | - |
| M_AXLELOADCAT | 7 | FINITE VALUE | - |
| M_AIRTIGHT | 2 | FINITE VALUE | - |
| N_AXLE | 10 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| M_VOLTAGE(k) | 4 | FINITE VALUE | Identity of the traction system. |
| NID_CTRACTION(k) | 10 | FINITE VALUE | NID_CTRACTION(k) given only if M_VOLTAGE(k) ≠ 0. |
| N_ITER | 5 | FINITE VALUE | - |
| NID_NTC(k) | 8 | FINITE VALUE | Type of National System available. |

| Step 4: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 6: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | 10 | LS |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | Toggle on |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 8: Radio Message 8 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 8 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | Reference to received train data message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 46

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when an emergency stop has been accepted and it is not yet revoked or deleted onboard. |  | 46 |
| Applicable Mode/Level Combinations | L2: FS / OS / LS L3: FS / OS / LS |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board, it REJECTS the LSSMA display toggle on order information in level 2/3 if it is received via radio (RBC), because an emergency stop message has been accepted and it is not yet revoked or deleted onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |
|  |  | Subset-026-4.8.3.1.1 Exception [5] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/12 | FS/ OS/ LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/ L3 | FS/ OS/LS | Train is moving. | I | INT | - | L2/ L3 | FS/ OS/LS | - |
| 2 | L2/ L3 | FS/ OS/LS | SA-DATA.Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received | I | RTM | - | L2/ L3 | FS/ OS/LS | - |
| 3 | L2/ L3 | FS/ OS/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/ L3 | FS/ OS/LS | - |
| 4 | L2/ L3 | FS/ OS/LS | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147; Q_EMERGENCYSTOP=0¦1) with packet 0, is transmitted | O | RTM | This message tests that conditional emergency stop information has been accepted independently of the value of Q_EMERGENCYSTOP. Packet 1 must be sent instead of Packet 0, if the position report is based on 2 BG | L2/ L3 | FS/ OS/LS | - |
| 5 | L2/ L3 | FS/ OS/LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | L2/ L3 | FS/ OS/LS | - |
| 6 | L2/ L3 | FS/ OS/LS | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | Only if in LS mode. The information from packet 180 is Rejected. | L2/ L3 | FS/ OS/LS | - |
| 7 | L2/ L3 | FS/ OS/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if in LS mode. | L2/ L3 | FS/ OS/LS | - |
| 8 | L2/ L3 | FS/ OS/LS | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 21, packet 27, packet 80 (M_MAMODE=10), packet 180 (Q_LSSMA=1) is received | I | RTM | Only if NOT in LS mode. The information from packet 180 is Rejected. The toggle on order would be rejected for FS, OS and SR mode if the max safe front end of the train was not inside the Limited Supervision area of a mode profile received in the same message (see the exception [9] in 4.8.4.2) | L2/ L3 | FS/ OS/LS | - |
| 9 | L2/ L3 | FS/ OS/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if NOT in LS mode. | L2/ L3 | FS/ OS/LS | - |
| 10 | L2/ L3 | FS/ OS/LS | SA-DATA.Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) is received | I | RTM | - | L2/ L3 | FS/ OS/LS | - |
| 11 | L2/ L3 | FS/ OS/LS | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | L2/ L3 | FS/ OS/LS | - |
| 12 | L2/ L3 | FS/ OS/LS | USE_FT5190300.1 (for Transition [70]) or USE_FT5190300.2 (for Transition [72]) or USE_FT5190200.6 (for Transition [74]) | - | - | If not in LS mode: the on-board equipment changes to LS mode. | L2/ L3 | LS | - |
| 13 | L2/ L3 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L2/ L3 | LS | - |
| 14 | L2/ L3 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L2/ L3 | LS | - |
| 15 | L2/ L3 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L2/ L3 | LS | - |
| 16 | L2/ L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44) is recorded | O | JRU | - | L2/ L3 | LS | - |

| Step 2: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_REF | 16 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 4: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | 0 ¦ 1 | Conditional Emergency Stop accepted, with update of current EOA/LOA (ref 3.10.2.2) ¦ Conditional Emergency Stop accepted, with no update of current EOA/LOA (ref 3.10.2.2) |
| NID_PACKET | 8 | 0 | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 6: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 8: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 21 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_GRADIENT | 15 | FINITE VALUE | - |
| Q_GDIR | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A | 8 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_GRADIENT(k) | 15 | FINITE VALUE | - |
| Q_GDIR(k) | 1 | FINITE VALUE | 0 = downhill 1= uphill. |
| G_A(k) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 27 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_STATIC | 15 | FINITE VALUE | - |
| V_STATIC | 7 | FINITE VALUE | Basic SSP |
| Q_FRONT | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| Q_DIFF(n) | 2 | FINITE VALUE | - |
| NC_CDDIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 0. |
| NC_DIFF(n) | 4 | FINITE VALUE | If Q_DIFF(n) = 1 or 2. |
| V_DIFF(n) | 7 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_STATIC(k) | 15 | FINITE VALUE | - |
| V_STATIC(k) | 7 | FINITE VALUE | Basic SSP. |
| Q_FRONT(k) | 1 | FINITE VALUE | - |
| N_ITER(k) | 5 | FINITE VALUE | - |
| Q_DIFF(k,m) | 2 | FINITE VALUE | - |
| NC_CDDIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 0. |
| NC_DIFF(k,m) | 4 | FINITE VALUE | If Q_DIFF(k,m) = 1 or 2. |
| V_DIFF(k,m) | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | FINITE VALUE | - |
| M_MAMODE | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | 10 | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 10: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 47

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when the max safe front end of the train is not inside the Limited Supervision area of a mode profile received in the same message. |  | 47 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN L1: FS / OS / SR L2: FS / OS / SR L3: FS / OS / SR |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board, it REJECTS the LSSMA display toggle on order information, because the max safe front end of the train is not inside the Limited Supervision area of a mode profile received in the same message. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-4.4.19.1.4.6 |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2 / 4 / 13 | FS/ OS/ SR /UN /SN |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | LNTC / L0 / L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only for L2/L3 |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | USE_FT4080300.40 | - | - | The on-board receives and accepts the Generic LS function marker (Packet 181) | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 2 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | Train is moving. | I | INT | - | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 3 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | Only if in L2/3. The information from packet 180 is Rejected. | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 4 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if in L2/3. | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 5 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | One Balise Group message containing packet 180 (Q_LSSMA=1) is received | I | BTM | Only if in L0/LNTC/L1 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 6 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if in L0/LNTC/L1 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | - |
| 7 | L0 / LNTC / L1 / L2/ L3 | UN/SN/FS/ OS/SR | USE_FT5190300.1 (for Transition [70]) or USE_FT5190300.4 (for Transition [71]) or USE_FT5190300.2 (for Transition [72]) or USE_FT5190200.6 (for Transition [74]) | - | - | The train changes to LS mode. The generic LS function marker (Packet 181) will be sent in the same BG message (see the requirement 4.4.19.1.4.8). | L1 / L2/ L3 | LS | - |
| 8 | L1 / L2/ L3 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L1 / L2/ L3 | LS | - |
| 9 | L1 / L2/ L3 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 / L2/ L3 | LS | - |
| 10 | L1 / L2/ L3 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L1 / L2/ L3 | LS | - |
| 11 | L1 / L2/ L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1 / L2/ L3 | LS | - |

| Step 3: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 5: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only for L2/3 |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 48

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | LSSMA display toggle on order shall be rejected by the on-board equipment when it is not in the appropriate mode. |  | 48 |
| Applicable Mode/Level Combinations | L1: TR / PT L2: TR / PT L3: TR / PT |  |  |
| Target of Test | The target of this test case is to check that: When the on-board equipment has the Generic LS function marker information stored on-board, it REJECTS the LSSMA display toggle on order information, because the current mode leads to reject the information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | CEDEX |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 LSSMA display toggle on order |  |
|  |  | Subset-026-4.8.4.2 LSSMA display toggle on order |  |
|  |  | Subset-026-4.4.19.1.4.6 |  |
|  |  | Subset-026-7.4.2.37.3 Packet Number 180: LSSMA display toggle order |  |
|  |  | Subset-026-4.7.2 table - Output information LSSMA |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: - The LSSMA is not displayed. - The LSSMA number is not displayed. Check on the JRU that: - The LSSMA is not recorded. - The LSSMA number is not recorded. |
| Constraints | - |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 7 / 8 | TR / PT |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only for L2/L3 |
| NOT RELEVANT | - | TIU | - |
| Current mode and level is displayed. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L1 / L2/ L3 | TR / PT | Train is moving. | I | INT | - | L1 / L2/ L3 | TR / PT | - |
| 2 | L1 / L2/ L3 | TR / PT | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 180 (Q_LSSMA=1) is received | I | RTM | Only if in L2/3. The information from packet 180 is Rejected. | L1 / L2/ L3 | TR / PT | - |
| 3 | L1 / L2/ L3 | TR / PT | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | Only if in L2/3. | L1 / L2/ L3 | TR / PT | - |
| 4 | L1 / L2/ L3 | TR / PT | One Balise Group message containing packet 180 (Q_LSSMA=1) is received | I | BTM | Only if in L1. The information from packet 180 is Rejected. | L1 / L2/ L3 | TR / PT | - |
| 5 | L1 / L2/ L3 | TR / PT | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | Only if in L1 | L1 / L2/ L3 | TR / PT | - |
| 6 | L1 / L2/ L3 | TR / PT | USE_FT5110200.13 and USE_FT5110200.5 | - | - | Only if in TR mode: the train changes to PT mode and then to LS mode. | L1 / L2/ L3 | PT | - |
| 7 | L1 / L2/ L3 | PT | USE_FT4060300.60 (for Transition [70]) | - | - | The train changes to LS mode. The generic LS function marker (Packet 181) will be sent in the same BG message (see the requirement 4.4.19.1.4.8). | L1 / L2/ L3 | LS | - |
| 8 | L1 / L2/ L3 | LS | Not the Limited Supervision symbol "Limited Supervision frame" is displayed. | O | DMI | The DMI Object: Symbol LS01 is not displayed | L1 / L2/ L3 | LS | - |
| 9 | L1 / L2/ L3 | LS | Not DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded | O | JRU | - | L1 / L2/ L3 | LS | - |
| 10 | L1 / L2/ L3 | LS | Not the LSSMA number is displayed. | O | DMI | The EVC does not display the LSSMA speed value | L1 / L2/ L3 | LS | - |
| 11 | L1 / L2/ L3 | LS | Not LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded | O | JRU | - | L1 / L2/ L3 | LS | - |

| Step 2: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 180 | LSSMA display toggle order |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |

| Step 4: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 180 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_LSSMA | 1 | 1 | - |
| T_LSSMA | 8 | FINITE VALUE | Only if Q_LSSMA = 1 |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |
| Generic LS function marker | STORED | - |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only for L2/3 |
| NOT RELEVANT | - | TIU | - |
| The LSSMA is not displayed. | O | DMI | Symbol LS01 toggled off |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 49

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Temporary Speed Restriction Revocation shall be accepted by on-board equipment according to 4.8.3 table. Tested Action: R[1] from BTM |  | 49 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives and accepts the packet 66 (transmitted by balise) when a Level 1 announcement is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.3.1.1 Exception [1] |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that Reception of packet 66 is recorded. The on-board equipment changes to L1, and possibly to FS, LS, OS or SR. Check on the DMI that The permitted speed is higher than the previously stored value of V_TSR (for the location of TSR) |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. A level 1 transition order is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Transition to Level 1 at a further location |
| Temporary Speed Restrictions | STORED | TSR revocable |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | One Balise Group message (N_TOTAL=1) containing packet 66 is received | I | BTM | - | LNTC | SN | - |
| 2 | LNTC | SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=66) is recorded | O | JRU | The packet is accepted | LNTC | SN | - |
| 3 | LNTC | SN | On-board equipment executes a Level transition order to level 1. | - | - | USE_FT5100100.23: from LNTC to L1 | L1 | SN | - |
| 4 | L1 | SN | On-board equipment executes a mode change. | - | - | USE_FT4060300.19 (Transition to FS) Or USE_FT4060300.55 (Transition to OS) Or USE_FT5190300.4 (Transition to LS) Or USE_FT4060300.52 (Transition to SR) | L1 | FS / LS / OS / SR | - |
| 5 | L1 | FS / LS / OS / SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is OS or SR. | L1 | FS / LS / OS / SR | - |
| 6 | L1 | FS / LS / OS / SR | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode: The PS is higher than the previously received value of V_TSR. The train is located in the area of the previously stored TSR, given in packet 65. If in LS mode: The train speed is higher than the previously received value of V_TSR and the supervision status is NoS. | L1 | FS / LS / OS / SR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Temporary Speed Restrictions | STORED | TSR revocable |
| ERTMS/ETCS level | 2 | L1 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 50

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | Temporary Speed Restriction Revocation shall be accepted by on-board equipment according to 4.8.3 table. Tested Action: R[2] from BTM |  | 50 |
| Applicable Mode/Level Combinations | LNTC: SN |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment receives and accepts the packet 66 (transmitted by balise) when a Level 2/3 announcement is stored onboard. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Temporary Speed Restriction Revocation |  |
|  |  | Subset-026-4.8.3.1.1 Exception [2] |  |
|  |  | Subset-026-7.4.2.18 Packet Number 66: Temporary Speed Restriction Revocation |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that Reception of packet 66 is recorded. The on-board equipment changes to L2/L3, and possibly to FS, LS, OS or SR. Check on the DMI that The permitted speed is higher than the previously stored value of V_TSR (for the location of TSR) |
| Constraints | The data of the stored TSR (packet 65) has to be the most restrictive speed profile (for the location of TSR) and the Temporary Speed Restriction is not marked as non-revocable. A level 2/3 transition order is stored onboard. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 13 | SN |
| Level Transition Order | STORED | Transition to Level 2/3 at a further location |
| Temporary Speed Restrictions | STORED | TSR revocable |
| ERTMS/ETCS level | 1 | LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | LNTC | SN | One Balise Group message (N_TOTAL=1) containing packet 66 is received | I | BTM | - | LNTC | SN | - |
| 2 | LNTC | SN | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=66) is recorded | O | JRU | The packet is accepted | LNTC | SN | - |
| 3 | LNTC | SN | On-board equipment executes a Level transition order to level 2/3. | - | - | USE_FT5100100.21: from LNTC to L2 Or USE_FT5100100.20: from LNTC to L3 | L2/L3 | SN | - |
| 4 | L2/L3 | SN | On-board equipment executes a mode change. | - | - | USE_FT4060300.19 (Transition to FS) Or USE_FT4060300.55 (Transition to OS) Or USE_FT5190300.4 (Transition to LS) Or USE_FT4060300.53 (Transition to SR) | L2/L3 | FS / LS / OS / SR | - |
| 5 | L2/L3 | FS / LS / OS / SR | The driver requests the speed and distance monitoring information to be displayed | I | DMI | If the current mode is OS or SR. | L2/L3 | FS / LS / OS / SR | - |
| 6 | L2/L3 | FS / LS / OS / SR | The speed and distance monitoring information is displayed | O | DMI | If in FS, OS or SR mode: The PS is higher than the previously received value of V_TSR. The train is located in the area of the previously stored TSR, given in packet 65. If in LS mode: The train speed is higher than the previously received value of V_TSR and the supervision status is NoS. | L2/L3 | FS / LS / OS / SR | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 66 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_TSR | 8 | FINITE VALUE | Identity of TSR to be revoked. |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 12 | FS / OS / SR / LS |
| Temporary Speed Restrictions | STORED | TSR revocable |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 51

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Session Management from balises in L2/3 shall be rejected by on-board equipment when it is an order to establish a communication session with an RBC referred to in an RBC transition order currently stored according to 4.8.3 table. Tested Action : A[14] from BTM |  | 51 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment handles only one communication session. The on-board rejects the Session Management information from BG to establish a communication session with the same RBC referred to in an RBC transition order currently stored. The on-board equipment does not terminate the existing communication with Handing RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.3.1.1 Exception [14] |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 42 is recorded. |
| Constraints | The on-board equipment can handle only one communication session. Packet 42: Session Management is received before the RBC transition Border. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SL / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | With Handing RBC |
| RBC Transition Order | STORED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | One Balise Group message (N_TOTAL=1) containing packet 42 (Q_RBC=1; NID_RBC=<Accepting RBC>) is received | I | BTM | Packet 42 is rejected. | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |
| 2 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |
| 3 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The existing communication session with Handing RBC shall not be terminated. | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SL / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | With Handing RBC |
| RBC Transition Order | STORED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 52

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.3: Accepted information depending on the level and transmission media |  | 4080300 |
| Test Case of Feature | The Session Management from balises in L2/3 shall be rejected by on-board equipment when it is an order to establish a communication session with an RBC referred to in an RBC transition order received in the same message according to 4.8.3 table. Tested Action : A[14] from BTM |  | 52 |
| Applicable Mode/Level Combinations | L2: FS, LS, OS, SR, SL, SB, TR, PT, NL L3: FS, LS, OS, SR, SL, SB, TR, PT, NL |  |  |
| Target of Test | The target of this test case is to check that: The on-board equipment handles only one communication session. The on-board rejects the Session Management information from BG to establish a communication session with the same RBC referred to in an RBC transition order received in the same message. The on-board equipment does not terminate the existing communication with Handing RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Session Management |  |
|  |  | Subset-026-4.8.3.1.1 Exception [14] |  |
|  |  | Subset-026-7.4.2.10 Packet Number 42: Session management |  |
|  |  | Subset-026-7.4.2.27 Packet Number 131: RBC transition order |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that: Reception of packet 131 and packet 42 are recorded. |
| Constraints | The on-board equipment can handle only one communication session. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SL / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | With Handing RBC |
| RBC Transition Order | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | One Balise Group message (N_TOTAL=1) containing packet 131 (D_RBCTR ≠ 0; NID_RBC=<Accepting RBC>) and packet 42 (Q_RBC=1; NID_RBC=<Accepting RBC>) is received | I | BTM | Packet 42 is rejected. | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |
| 2 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |
| 3 | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | NOT SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The existing communication session with Handing RBC shall not be terminated. | L2/L3 | FS/LS/OS, SR/SL/SB, TR/PT/NL | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | FINITE VALUE | - |
| N_TOTAL | 3 | FINITE VALUE | 1 ( N ( 7 i.e. from 2 to 8 balises in the group |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Identical Value for all balises of the group |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | ID of Balise Group |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 131 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_RBCTR | 15 | FINITE VALUE | ≠ 0 |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | Accepting. RBC radio subscriber number. |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 42 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_RBC | 1 | 1 | Establish communication session |
| NID_C | 10 | FINITE VALUE | - |
| NID_RBC | 14 | FINITE VALUE | Accepting RBC |
| NID_RADIO | 64 | FINITE VALUE | - |
| Q_SLEEPSESSION | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 5 / 6 / 7 / 8 / 11 / 12 | FS / OS / SR / SL / SB / TR / PT / NL / LS |
| Radio communication session | ESTABLISHED | With Handing RBC |
| RBC Transition Order | STORED | With Accepting RBC |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | With Handing RBC |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |