| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5180800 (5.18.8: Advising a tunnel stopping area) Total: 5 Test case(s) |  |
| REF: | Subset-076-5-2-5180800 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 17.12.2012 | All | First version/review | MULTITEL |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 05.07.2013 | All | Template standardised with a dedicated tool | MULTITEL |
| Version 2.9.2 01.08.2013 | All | Answer to Review002_Subset-076-5-2-5180800_v291 | MULTITEL |
| Version 2.9.3 29.11.2013 | TC2, TC3, TC4 | Answer to Review001_Subset-076-5-2-5180800_v292 | MULTITEL |
| Version 2.9.4 12.12.2013 | TC2, TC3, TC4 | Answer to issue 22 from Review005_Subset-076-5-2-5180800_v293 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.8: Advising a tunnel stopping area |  | 5180800 |
| Test Case of Feature | On driver request, the ERTMS/ETCS on-board equipment enables the display of the tunnel stopping area related information |  | 1 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, on driver request, the ERTMS/ETCS on-board equipment enables the display of the tunnel stopping area related information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.8.2 |  |
|  |  | Subset-026-5.18.8.3 |  |
|  |  | Subset-026-5.18.8.3 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | After the driver request, the ERTMS/ETCS on-board equipment enables the diplay on the DMI of the the tunnel stopping area related information. |
| Constraints | The display of the tunnel stopping area related information is in its initial state (DISABLED). There are tunnel stopping areas stored on-board and the train front end is in rear of the Permitted supervision limit (Subset-026-5.18.8.3). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | There are tunnel stopping areas stored on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Toggling function for tunnel stopping area is displayed (DR05) | O | DMI | For a touch screen DMI, the symbol DR05 is displayed in C2/C3/C4. For a soft key DMI, the symbol DR05 is displayed in F6. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver toggles on the display of the tunnel stopping area (DR05). | I | DMI | On driver request, the ERTMS/ETCS on-board equipment enables the display of the tunnel stopping area related information. | N/A | N/A | - |
| 2 | N/A | N/A | The driver’s request is RECORDED. | O | JRU | M_DRIVERACTIONS, Request to show tunnel stopping area information recorded | N/A | N/A | - |
| 3 | N/A | N/A | The tunnel stopping area information is displayed (TC36 or TC37). | O | DMI | “Tunnel stopping area” or “Tunnel stopping area announcement” is displayed | N/A | N/A | - |
| 4 | N/A | N/A | The “tunnel stopping area” information displayed is RECORDED | O | JRU | DMI_SYMB_STATUS, TC36 or TC37 (bit 79 or 80) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Symbol TC36, TC37 is shown in C2/C3/C4 to indicate respectively "Tunnel stopping area" and "Tunnel stopping area announcement". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.8: Advising a tunnel stopping area |  | 5180800 |
| Test Case of Feature | On driver request, the ERTMS/ETCS on-board equipment disables the display of the tunnel stopping area related information |  | 2 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, on driver request, the ERTMS/ETCS on-board equipment disables the display of the tunnel stopping area related information. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.8.2 |  |
|  |  | Subset-026-5.18.8.3 |  |
|  |  | Subset-026-5.18.8.3 a) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | After the driver request, the ERTMS/ETCS on-board equipment disables the diplay on the DMI of the the tunnel stopping area related information. |
| Constraints | The display of the tunnel stopping area related information is ENABLED. There are tunnel stopping areas stored on-board and the train front end is in rear of the Permitted supervision limit (Subset-026-5.18.8.3). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | There are tunnel stopping areas stored on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Symbol TC36, TC37 is shown in C2/C3/C4 to indicate respectively "Tunnel stopping area" and "Tunnel stopping area announcement". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver toggles off the display of the tunnel stopping area. | I | DMI | On driver request, the ERTMS/ETCS on-board equipment disables the display of the tunnel stopping area related information. | N/A | N/A | - |
| 2 | N/A | N/A | The driver’s request is RECORDED. | O | JRU | M_DRIVERACTIONS, Request to hide tunnel stopping area information | N/A | N/A | - |
| 3 | N/A | N/A | The tunnel stopping area information is no longer displayed (TC36 or TC37). | O | DMI | “Tunnel stopping area” or “Tunnel stopping area announcement” is hidden | N/A | N/A | - |
| 4 | N/A | N/A | The “tunnel stopping area” information shown off is RECORDED | O | JRU | DMI_SYMB_STATUS, TC36 or TC37 (bit 79 or 80) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS UNCHANGED |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 UNCHANGED |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Toggling function for tunnel stopping area is displayed (DR05) | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.8: Advising a tunnel stopping area |  | 5180800 |
| Test Case of Feature | The train is advising a tunnel stopping area with its train front end is in advance of the Permitted supervision limit |  | 3 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, when the train is advising a tunnel stopping area with its train front end in advance of the Permitted supervision limit, no information related to this tunnel stopping area shall be displayed and the next tunnel stopping area stored on-board, shall be checked. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.8.3 |  |
|  |  | Subset-026-5.18.8.3 b) |  |
|  |  | Subset-026-5.18.8.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train front end becomes in advance of the Permitted supervision limit, the information related to this tunnel stopping area is replaced with the indication of the remaining distance to the start location of the next tunnel stopping area. |
| Constraints | The display of the tunnel stopping area related information is ENABLED. There are, at least, 2 tunnel stopping areas stored on-board and the train front end is in rear of the Permitted supervision limit (Subset-026-5.18.8.3) of the first. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | There are tunnel stopping areas stored on-board |
| ERTMS/ETCS level | 0 / 1 / 12 | FS / OS / LS |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Symbol TC36, TC37 is shown in C2/C3/C4 to indicate respectively "Tunnel stopping area" and "Tunnel stopping area announcement". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The tunnel stopping area information is displayed (TC36 or TC37). | O | DMI | The train front end is in rear of the Permitted supervision limit, “Tunnel stopping area” or “Tunnel stopping area announcement” is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | The tunnel stopping area information (TC37) is UPDATED with the next tunnel stopping area. | O | DMI | The train front end is in advance of the Permitted supervision limit. The indication of the remaining distance to the start location of the tunnel stopping area is UPDATED. | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.8: Advising a tunnel stopping area |  | 5180800 |
| Test Case of Feature | The train is advising a tunnel stopping area with its train front end is in advance of the Permitted supervision limit |  | 4 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, when the train is advising the last tunnel stopping area with its train front end in advance of the Permitted supervision limit, no information related to tunnel stopping areas shall be displayed. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.8.3 |  |
|  |  | Subset-026-5.18.8.3 b) |  |
|  |  | Subset-026-5.18.8.4 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train front end became in advance of the Permitted supervision limit, the information related to this tunnel stopping area is deleted and not replaced. |
| Constraints | The display of the tunnel stopping area related information is ENABLED. There is only one tunnel stopping area stored on-board and the train front end is in rear of the Permitted supervision limit (Subset-026-5.18.8.3). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | There are tunnel stopping areas stored on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Symbol TC36, TC37 is shown in C2/C3/C4 to indicate respectively "Tunnel stopping area" and "Tunnel stopping area announcement". | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The tunnel stopping area information is displayed (TC36 or TC37). | O | DMI | The train front end is in rear of the Permitted supervision limit, “Tunnel stopping area” or “Tunnel stopping area announcement” is displayed. | N/A | N/A | - |
| 2 | N/A | N/A | The tunnel stopping area information (TC36 or TC37) is REMOVED. | O | DMI | The train front end is in advance of the Permitted supervision limit. | N/A | N/A | - |
| 3 | N/A | N/A | The “tunnel stopping area information” shown off is RECORDED. | O | JRU | DMI_SYMB_STATUS, TC36 or TC37 (bit 79 or 80) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.18.8: Advising a tunnel stopping area |  | 5180800 |
| Test Case of Feature | The train updates the message displayed about the train stopping area when entering in the tunnel stopping area. |  | 5 |
| Applicable Mode/Level Combinations | L1 : FS, LS, OS L2 : FS, LS, OS L3 : FS, LS, OS |  |  |
| Target of Test | The target of this test is to check that, when the train front end reaches the start location of the tunnel stopping area, the “Tunnel stopping area” information is UPDATED. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.7.2 table - Output information Track conditions sound horn, non stopping areas, tunnel stopping areas |  |
|  |  | Subset-026-5.18.8.2 |  |
|  |  | Subset-026-5.18.8.3 |  |
|  |  | Subset-026-5.18.8.3 a) |  |
|  |  | Subset-026-5.18.8.4 |  |
|  |  | Subset-026-5.18.8.5 |  |
|  |  | Subset-026-5.18.8.5 a) |  |
|  |  | Subset-026-5.18.8.5 b) |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | When the train front end reaches the start location of the tunnel stopping area, the “Tunnel stopping area announcement” is replaced whith “Tunnel stopping area” information. |
| Constraints | The display of the tunnel stopping area related information is in its initial state (DISABLED). There are tunnel stopping areas stored on-board and the train front end is in rear of the Permitted supervision limit. The tunnel stopping area is longer enough for the train to reach it in rear of the Permitted supervision limit. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Track conditions excluding big metal masses | STORED | There are tunnel stopping areas stored on-board |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Toggling function for tunnel stopping area is displayed (DR05) | O | DMI | For a touch screen DMI, the symbol DR05 is displayed in C2/C3/C4. For a soft key DMI, the symbol DR05 is displayed in F6. |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver toggles on the display of the tunnel stopping area (DR05). | I | DMI | On driver request, the ERTMS/ETCS on-board equipment enables the display of the tunnel stopping area related information. | N/A | N/A | - |
| 2 | N/A | N/A | The driver’s request is RECORDED. | O | JRU | M_DRIVERACTIONS, Request to show tunnel stopping area information recorded | N/A | N/A | - |
| 3 | N/A | N/A | “Tunnel stopping area announcement” is DISPLAYED (TC37). | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The “Tunnel stopping area announcement” displayed is RECORDED | O | JRU | DMI_SYMB_STATUS, TC37 (bit 80) | N/A | N/A | - |
| 5 | N/A | N/A | “Tunnel stopping area announcement” is REMOVED | O | DMI | The train front end reaches start location of tunnel stopping area | N/A | N/A | - |
| 6 | N/A | N/A | The “Tunnel stopping area announcement” show off is RECORDED | O | JRU | DMI_SYMB_STATUS, TC37 (bit 80) | N/A | N/A | - |
| 7 | N/A | N/A | “Tunnel stopping area” is DISPLAYED (TC 36). | O | DMI | - | N/A | N/A | - |
| 8 | N/A | N/A | The “Tunnel stopping area ” displayed is RECORDED | O | JRU | DMI_SYMB_STATUS, TC36 (bit 79) | N/A | N/A | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 UNCHANGED | FS / OS / LS |
| ERTMS/ETCS level | 2 / 3 / 4 UNCHANGED | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |