| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5190600 (5.19.6: Exit of Limited Supervision mode) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5190600 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | MULTITEL |
| Version 2.9.1 09.10.2013 | TC1 | Update with the current guidelines | MULTITEL |
| Version 2.9.2 12.11.2013 | TC1 | Response to Review002_Subset-076-5-2-5190600_v291 | MULTITEL |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.6: Exit of Limited Supervision mode |  | 5190600 |
| Test Case of Feature | The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  | 1 |
| Applicable Mode/Level Combinations | L2: LS L3: LS |  |  |
| Target of Test | The target of this test case is to check that on-board switches to FS mode when the train reaches the end of the LS area . |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.6.2 row 5 column 6 |  |
|  |  | Subset-026-4.6.3 Condition Id [31] |  |
|  |  | Subset-026-5.19.6.1.1 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : the mode initially displayed changes to FS. Check on the JRU that : the new current mode FS is recorded. |
| Constraints | The Limited Supervision area ends before the EOA/LOA of the current MA |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | After he end of the Limited Supervision area, no specific mode is required by a Mode Profile. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| LS mode and level is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L2 / L3 | LS | The Mode symbol "Full Supervision" is displayed | O | DMI | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. | L2 / L3 | FS | - |
| 2 | L2 / L3 | FS | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded | O | JRU | The mode change to FS is stored. | L2 / L3 | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| FS mode is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |