| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 5190400 (5.19.4: Limited Supervision from Unfitted or SN mode) Total: 1 Test case(s) |  |
| REF: | Subset-076-5-2-5190400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 15.11.2016 | All | First version | MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 5.19.4: Limited Supervision from Unfitted or SN mode |  | 5190400 |
| Test Case of Feature | On-board is in UN or SN. A transition to Limited Supervision mode occurs at a transition of level: from level 0 or NTC to level 1 or 2 or 3. The driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  | 1 |
| Applicable Mode/Level Combinations | L0: UN LNTC: SN |  |  |
| Target of Test | The target of this test is to check that: A transition to Limited Supervision mode occurs at a transition of level: from level 0 or NTC to level 1 or 2 or 3. A request for acknowledgement shall be displayed to the driver And the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode, And the service brake is triggered. The command shall be released as soon as the driver acknowledges (unless the command was triggered also for other reasons) |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.4.19.1.2 |  |
|  |  | Subset-026-4.7.2 table - Input information Ackn of Limited Supervision mode |  |
|  |  | Subset-026-5.19.2.3 |  |
|  |  | Subset-026-5.19.2.4 |  |
|  |  | Subset-026-5.19.4.2 |  |
|  |  | Subset-026-7.4.2.26 Packet Number 80: Mode Profile |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that : The transition from level 0/NTC to level 1/2/3 occurs A Request for Acknowledgement is DISPLAYED The SB indication is DISPLAYED The SB indication is REMOVED after Drivers Acknowledgement Check on the JRU that : The transition from level 0/NTC to level 1/2/3 is RECORDED Transition to LS mode is RECORDED Application of Service Brake is RECORDED Driver Acknowledgement is RECORDED Release of Service Brake is RECORDED |
| Constraints | In this TC the train will be running under the permitted speed in LS Mode before reaching the LS area If the Service Brake were not available for any reason, Emergency Brake shall be used. For LS mode, the mode profile shall define the entry and the length of the Limited Supervision area. To be in Limited Supervision mode, SSP and gradient shall be at least available from the FRONT END of the train. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 4 / 13 | UN / SN |
| Radio communication session | ESTABLISHED | Only if level transition order to L2/L3 is stored |
| Gradient Profile | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| International SSP | STORED / NOT STORED | Shall be at least available from the FRONT END of the train If not stored, it shall be sent in this TC together with packet 12/15 and packet 80 |
| Level Transition Order | STORED | M_LEVELTR = <2 / 3 / 4> |
| ERTMS/ETCS level | 0 / 1 | L0 / LNTC |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if level transition order to L2/L3 is stored |
| NOT RELEVANT | - | TIU | - |
| The Level symbol "Level <L1 / L2 / L3> announcement" is displayed | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | L0 / LNTC | UN / SN | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received | I | BTM | One of the steps 1 or 2 will be chosen for testing The train receives a message including a packet 80 Mode profile Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L0 / LNTC | UN / SN | - |
| 2 | L0 / LNTC | UN / SN | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received | I | RTM | One of the steps 1 or 2 will be chosen for testing The train receives a message including a packet 80 Mode profile Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train | L0 / LNTC | UN / SN | - |
| 3 | L0 / LNTC | UN / SN | Complete message including Packet 80: Mode profile is RECORDED | O | JRU | - | L0 / LNTC | UN / SN | - |
| 4 | L0 / LNTC | UN / SN | The Level symbol "Level <L1 / L2 / L3>" is displayed | O | DMI | The estimated front end passes the location given in the level transition announcement | L0 / LNTC | UN / SN | - |
| 5 | L0 / LNTC | UN / SN | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit3=1¦Bit4=1¦Bit5=1>) is recorded | O | JRU | - | L1 / L2 / L3 | UN / SN | - |
| 6 | L1 / L2 / L3 | UN / SN | Mode transition to LS is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 7 | L1 / L2 / L3 | LS | The driver acknowledges Level Announcement <L1 / L2 / L3> | I | DMI | Only if in SN | L1 / L2 / L3 | LS | - |
| 8 | L1 / L2 / L3 | LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11 ; M_DRIVERACTIONS=<7 / 8 / 9>) is recorded | O | JRU | Only if in SN | L1 / L2 / L3 | LS | - |
| 9 | L1 / L2 / L3 | LS | The Mode symbol "Acknowledge Limited Supervision" is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |
| 10 | L1 / L2 / L3 | LS | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded | O | JRU | - | L1 / L2 / L3 | LS | - |
| 11 | L1 / L2 / L3 | LS | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 12 | L1 / L2 / L3 | LS | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded | O | JRU | Only in L2 / L3. | L1 / L2 / L3 | LS | - |
| 13 | L1 / L2 / L3 | LS | Service brake commanded | O | TIU | Driver doesn’t acknowledges within 5 sec after the change to LS (the driver acknowledgement time (refer to Appendix A3.1)) | L1 / L2 / L3 | LS | - |
| 14 | L1 / L2 / L3 | LS | The Status symbol "Service Brake or Emergency Brake intervention" is displayed | O | DMI | - | L1 / L2 / L3 | LS | - |
| 15 | L1 / L2 / L3 | LS | Service Brake Intervention is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |
| 16 | L1 / L2 / L3 | LS | The driver acknowledges "Limited Supervision" | I | DMI | Driver acknowledges more than 5 sec after the change to LS (the driver acknowledgement time (refer to Appendix A3.1)) | L1 / L2 / L3 | LS | - |
| 17 | L1 / L2 / L3 | LS | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11 ; M_DRIVERACTIONS=13) is recorded | O | JRU | - | L1 / L2 / L3 | LS | - |
| 18 | L1 / L2 / L3 | LS | Service brake not commanded | O | TIU | - | L1 / L2 / L3 | LS | - |
| 19 | L1 / L2 / L3 | LS | The Status symbol "Service Brake or Emergency Brake intervention" is removed | O | DMI | - | L1 / L2 / L3 | LS | - |
| 20 | L1 / L2 / L3 | LS | Service Brake Release is RECORDED | O | JRU | - | L1 / L2 / L3 | LS | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 0 | 1 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode change is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 12 | A Level 1 movement authority, shall be sent together. |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_MAIN | 7 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out". |
| N_ITER | 5 | FINITE VALUE | Set to zero if V_MAIN = 0 or if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Uplink |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Balise |
| N_PIG | 3 | 1 | 2 of N |
| N_TOTAL | 3 | 1 | N balises |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | - |
| NID_C | 10 | FINITE VALUE | - |
| NID_BG | 14 | FINITE VALUE | BGa |
| Q_LINK | 1 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Radio Message 3 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 3 | Message 3: Movement authority |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 10 + 14 | FINITE VALUE | - |
| NID_PACKET | 8 | 15 | Packet no 15: Level 2/3 MA |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| V_EMA | 7 | FINITE VALUE | - |
| T_EMA | 10 | FINITE VALUE | Can be set to "no time-out" |
| N_ITER | 5 | FINITE VALUE | Set to zero if only one section in the MA. |
| L_SECTION(k) | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER(k) | 1 | FINITE VALUE | - |
| T_SECTIONTIMER(k) | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC(k) | 15 | FINITE VALUE | - |
| L_ENDSECTION | 15 | FINITE VALUE | - |
| Q_SECTIONTIMER | 1 | FINITE VALUE | - |
| T_SECTIONTIMER | 10 | FINITE VALUE | - |
| D_SECTIONTIMERSTOPLOC | 15 | FINITE VALUE | - |
| Q_ENDTIMER | 1 | FINITE VALUE | - |
| T_ENDTIMER | 10 | FINITE VALUE | - |
| D_ENDTIMERSTARTLOC | 15 | FINITE VALUE | - |
| Q_DANGERPOINT | 1 | FINITE VALUE | - |
| D_DP | 15 | FINITE VALUE | - |
| V_RELEASEDP | 7 | FINITE VALUE | - |
| Q_OVERLAP | 1 | FINITE VALUE | - |
| D_STARTOL | 15 | FINITE VALUE | - |
| T_OL | 10 | FINITE VALUE | - |
| D_OL | 15 | FINITE VALUE | - |
| V_RELEASEOL | 7 | FINITE VALUE | - |
| NID_PACKET | 8 | 80 | Packet number 80: Mode profile |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| D_MAMODE | 15 | 0 | The mode transition is required for the current location |
| M_MAMODE | 2 | 10 | LS |
| V_MAMODE | 7 | FINITE VALUE | - |
| L_MAMODE | 15 | FINITE VALUE | - |
| L_ACKMAMODE | 15 | FINITE VALUE | - |
| Q_MAMODE | 1 | FINITE VALUE | - |
| N_ITER | 5 | FINITE VALUE | - |
| D_MAMODE(k) | 15 | FINITE VALUE | - |
| M_MAMODE(k) | 2 | FINITE VALUE | OS, LS, SH. |
| V_MAMODE(k) | 7 | FINITE VALUE | - |
| L_MAMODE(k) | 15 | FINITE VALUE | - |
| L_ACKMAMODE(k) | 15 | FINITE VALUE | - |
| Q_MAMODE(k) | 1 | FINITE VALUE | - |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train position report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 10+14 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = 1 or 2 |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | 12 | LS |
| M_LEVEL | 3 | 3 / 4 | L2 / L3 |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 12 | LS |
| Radio communication session | ESTABLISHED | Only in L2 / L3. |
| Gradient Profile | STORED | Shall be at least available from the FRONT END of the train |
| International SSP | STORED | Shall be at least available from the FRONT END of the train |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | Only if in L2 / L3 |
| SB is RELEASED | O | TIU | - |
| LS Mode Indication is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |