| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3100200_01_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3100200_01_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 20:19 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | MLT |

# Sequence description

Tested Feature: 3.10.2 Emergency Stop

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 50.00 |  | L2 |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | SR |  | - |  |
|  | 44 |  | 2 |  |  | 50.00 |  | L2 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 45 |  | 3 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | SR |  | - |  |
|  | 46 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 47 |  | 5 |  |  | 50.00 |  | L2 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 48 |  | 6 |  |  | 50.00 |  | L2 |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 49 |  | 7 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 50 |  | 8 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 51 |  | 9 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SR |  | - |  |
|  | 52 |  | 10 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 53 |  | 11 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 54 |  | 12 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | SR |  | - |  |
|  | 57 |  | 2 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 58 |  | 3 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 59 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 3: New Level 2/3 MA is ACCEPTED by the on board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 300.00 |  | L2 |  | SR |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received |  | I |  | RTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L2 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 300.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 300.00 |  | L2 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 63 |  | 4 |  |  | 300.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080447: 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop Test Case 1: Acceptance of Unconditional Emergency Stop information. A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC. The On-Board equipment switches immediately to the Trip mode. The train responds to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’, indicating that the train takes into account the Unconditional Emergency Stop. The data displayed on the DMI are updated, accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 500.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Unconditional Emergency Stop" (NID_MESSAGE=16) is received |  | I |  | RTM |  | The message is accepted. |  |  | L2 |  |  | FS |  | - |  |
|  | 65 |  | 2 |  |  | 500.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9;NID_MESSAGE=16) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3100200: 3.10.2 Emergency Stop Test Case 1: The train receives a Movement Authority which is REJECTED, because a valid emergency stop message is stored on-board. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 1 |  |  | 500.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15 is received |  | I |  | RTM |  | The message is REJECTED |  |  | L2 |  |  | FS |  | - |  |
|  | 67 |  | 2 |  |  | 500.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 68 |  | 3 |  |  | 500.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is not updated |  | O |  | DMI |  | Nothing changes on DMI |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080447: 4.8.4: Accepted Information depending on the modes - Unconditional Emergency Stop Test Case 1: Acceptance of Unconditional Emergency Stop information. A train is running with a valid Movement Authority including a danger point (optionally) and an overlap (optionally). The train receives the Message 16 ‘Unconditional Emergency Stop’ from the supervising RBC. The On-Board equipment switches immediately to the Trip mode. The train responds to the RBC by sending it a message 147 ‘Acknowledgement of Emergency Stop’, indicating that the train takes into account the Unconditional Emergency Stop. The data displayed on the DMI are updated, accordingly. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 3 |  |  | 500.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 70 |  | 4 |  |  | 500.00 |  | L2 |  | TR |  |  | The Mode symbol "Trip" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 71 |  | 5 |  |  | 500.00 |  | L2 |  | TR |  |  | The system status message "Emergency stop" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 72 |  | 6 |  |  | 500.00 |  | L2 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 73 |  | 7 |  |  | 500.00 |  | L2 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 74 |  | 8 |  |  | 500.00 |  | L2 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 75 |  | 9 |  |  | 500.00 |  | L2 |  | TR |  |  | SA-DATA.Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147;Q_EMERGENCYSTOP=2) is transmitted |  | O |  | RTM |  | Indicating that the train has received the Unconditional Emergency Stop. Check that the messages 147 is send over the normal priority channel. |  |  | L2 |  |  | TR |  | - |  |
|  | 76 |  | 10 |  |  | 500.00 |  | L2 |  | TR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10;NID_MESSAGE=147) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | Feature 3060500: Position Reporting to the RBC Test Case 4: The on-board equipment sends a position report by means of Packet 0: 'Position report'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 500.00 |  | L2 |  | TR |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | The position refers to the front end of the respective engine with regards to the train orientation. |  |  | L2 |  |  | TR |  | - |  |
|  | 79 |  | 2 |  |  | 500.00 |  | L2 |  | TR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | Feature 3060400: 3.6.4: Train Position Confidence Interval Test Case 3: The train receives from an unlinked BG a TSR message which starts behind the position of the next BG which is linked. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 92 |  | 13 |  |  | 500.00 |  | L2 |  | TR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - _____ MA is deleted |  |  | L2 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  | 1 |  |  | 510.00 |  | L2 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L2 |  |  | TR |  | - |  |
|  | 96 |  | 2 |  |  | 540.00 |  | L2 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L2 |  |  | TR |  | - |  |
|  | 97 |  | 3 |  |  | 540.00 |  | L2 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 98 |  | 4 |  |  | 540.00 |  | L2 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 99 |  | 5 |  |  | 540.00 |  | L2 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | TR |  | - |  |
|  | 100 |  | 6 |  |  | 540.00 |  | L2 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L2 |  |  | PT |  | - |  |
|  | 101 |  | 7 |  |  | 540.00 |  | L2 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 102 |  | 8 |  |  | 540.00 |  | L2 |  | PT |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | Inform trackside about mode change with existing communication session Only in level 2/3 |  |  | L2 |  |  | PT |  | - |  |
|  | 103 |  | 9 |  |  | 540.00 |  | L2 |  | PT |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Only in level 2/3 |  |  | L2 |  |  | PT |  | - |  |
|  | 104 |  | 10 |  |  | 540.00 |  | L2 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L2 |  |  | PT |  | - |  |
|  | 105 |  | 11 |  |  | 540.00 |  | L2 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 106 |  | 12 |  |  | 540.00 |  | L2 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 107 |  | 13 |  |  | 540.00 |  | L2 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 9: Train is at standstill in (L2/L3) and PT mode. The on-board has reported mode change to RBC. After receiving RBC acknowledgement and emergency stop revocation it is possible to accept new MA |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 1 |  |  | 540.00 |  | L2 |  | PT |  |  | RBC sends message 6: 'Recognition of exist from TR' acknowledging the mode change. |  | I |  | RTM |  | New Messages are accepted after ack. from RBC and release of EB. |  |  | L2 |  |  | PT |  | - |  |
|  | 109 |  | 2 |  |  | 540.00 |  | L2 |  | PT |  |  | Radio message 6 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 110 |  | 3 |  |  | 540.00 |  | L2 |  | PT |  |  | The RBC sends message 18 to revoke a pending Emergency Stop. |  | I |  | RTM |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 111 |  | 4 |  |  | 540.00 |  | L2 |  | PT |  |  | Radio message 18 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 112 |  | 5 |  |  | 540.00 |  | L2 |  | PT |  |  | The on-board offers the possibility to the driver to select ' start' |  | O |  | DMI |  | (only if train data has been previously entered) |  |  | L2 |  |  | PT |  | - |  |
|  | 113 |  | 6 |  |  | 540.00 |  | L2 |  | PT |  |  | Driver selects 'Start' |  | I |  | DMI |  | To request RBC for a New MA for L2/L3 |  |  | L2 |  |  | PT |  | - |  |
|  | 114 |  | 7 |  |  | 540.00 |  | L2 |  | PT |  |  | Driver action is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 115 |  | 8 |  |  | 540.00 |  | L2 |  | PT |  |  | The on-board requests to RBC for a New MA by sending message 132 'MA request' |  | O |  | RTM |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 116 |  | 9 |  |  | 540.00 |  | L2 |  | PT |  |  | Radio message 132 is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 117 |  | 10 |  |  | 540.00 |  | L2 |  | PT |  |  | The on-board equipment waits for RBC to answer. Hour glass symbol (ST05) is DISPLAYED. |  | O |  | DMI |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 118 |  | 11 |  |  | 540.00 |  | L2 |  | PT |  |  | The displayed symbol/ ST05 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | PT |  | - |  |
|  | 119 |  | 12 |  |  | 540.00 |  | L2 |  | PT |  |  | Radio message 3: New MA with optional packets as SSP/ GP and Mode Profile are received and accepted from RBC |  | I |  | RTM |  | Message 3: 'Movement Authority' with packet 15 and optional packets as 21/ 27/ 80 and 49.(for OS/ LS/ SH) |  |  | L2 |  |  | PT |  | - |  |
|  | 120 |  | 13 |  |  | 540.00 |  | L2 |  | PT |  |  | Radio message 3 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 121 |  | 14 |  |  | 540.00 |  | L2 |  | FS |  |  | New MA/ Track description and Mode Profile are RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 32: The train receives an order to switch immediately from L2 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 |  | 1 |  |  | 650.00 |  | L2 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. • D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  | I |  | BTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 125 |  | 2 |  |  | 650.00 |  | L2 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 126 |  | 3 |  |  | 650.00 |  | L2 |  | FS |  |  | Level 1 is DISPLAYED. |  | O |  | DMI |  | The On-board equipment switches to Level 1. |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 4 |  |  | 650.00 |  | L1 |  | FS |  |  | New Level 1 is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  |  | 650.00 |  | L1 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L1 |  |  | FS |  | - |  |
|  | 129 |  | 2 |  |  | 650.00 |  | L1 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 130 |  | 3 |  |  | 650.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L1 |  |  | FS |  | - |  |
|  | 131 |  | 4 |  |  | 650.00 |  | L1 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 132 |  | 5 |  |  | 650.00 |  | L1 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L1 |  |  | FS |  | - |  |
|  | 133 |  | 6 |  |  | 650.00 |  | L1 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L1 |  |  | FS |  | - |  |
|  | 134 |  | 7 |  |  | 650.00 |  | L1 |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080445: 4.8.4: Accepted Information depending on the modes - Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses Test Case 7: Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 850.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 68 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 136 |  | 2 |  |  | 850.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 2: The train is running in L1 with an MA, track description, and a stored track condition. The train switches to TR mode due to a balise group (BGa) sending a message with an incompatible ETCS version. During the TR mode, it receives an new MA which should be rejected and a new track condition which should be accepted. The track conditions received during the TR mode and before the switch to TR mode are applied during the TR mode and before it reaches standstill. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 142 |  | 3 |  |  | 850.00 |  | L1 |  | FS |  |  | The Track Condition symbol "Close air conditioning intake announcement" is displayed. |  | O |  | DMI |  | D1: The max safe front end of the train reaches a location in rear of the beginning of the 'air tightness' area. The symbol TC19 shall be used to indicate an automatic execution/ the symbol TC21 when the driver is requested to act. |  |  | L1 |  |  | FS |  | - |  |
|  | 143 |  | 4 |  |  | 850.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=1¦Bit64=1) is recorded. |  | O |  | JRU |  | D1 |  |  | L1 |  |  | FS |  | - |  |
|  | 144 |  | 5 |  |  | 850.00 |  | L1 |  | FS |  |  | Close air tightness. |  | O |  | TIU |  | D1. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual and the driver action will be required. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 36: The train is in level 1, 2 or 3 and passes over a balise group (BGa) sending a message with an incompatible ETCS version. The train is tripped and an indication is given to the driver. Transition Condition Id [65]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 146 |  | 1 |  |  | 900.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) is received |  | I |  | BTM |  | The On-board equipment notices that M_VERSION in BGa message is greater than the highest M_VERSION supported by the on-board equipment, so the information from this balise shall be ignored. (M _VERSION is different to M_VERSION 000xxxxx ) |  |  | L1 |  |  | FS |  | - |  |
|  | 147 |  | 2 |  |  | 900.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 148 |  | 3 |  |  | 900.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 150 |  | 1 |  |  | 900.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 151 |  | 2 |  |  | 900.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 36: The train is in level 1, 2 or 3 and passes over a balise group (BGa) sending a message with an incompatible ETCS version. The train is tripped and an indication is given to the driver. Transition Condition Id [65]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 152 |  | 5 |  |  | 900.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 153 |  | 6 |  |  | 900.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit09=1>) is recorded |  | O |  | JRU |  | Trackside not compatible |  |  | L1 |  |  | TR |  | - |  |
|  | 154 |  | 7 |  |  | 900.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 155 |  | 8 |  |  | 900.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 156 |  | 9 |  |  | 900.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 157 |  | 10 |  |  | 900.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 2: The train is running in L1 with an MA, track description, and a stored track condition. The train switches to TR mode due to a balise group (BGa) sending a message with an incompatible ETCS version. During the TR mode, it receives an new MA which should be rejected and a new track condition which should be accepted. The track conditions received during the TR mode and before the switch to TR mode are applied during the TR mode and before it reaches standstill. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 158 |  | 7 |  |  | 900.00 |  | L1 |  | TR |  |  | EB is applied (until standstill and acknowledgement by driver) when entering TR mode. |  | O |  | TIU |  | D2. |  |  | L1 |  |  | TR |  | - |  |
|  | 159 |  | 8 |  |  | 900.00 |  | L1 |  | TR |  |  | The planning information area (D area) becomes empty because the MA and the track description data are deleted. |  | O |  | DMI |  | D3. Only if the on-board equipment was in the FS mode before switching to the TR mode. |  |  | L1 |  |  | TR |  | - |  |
|  | 160 |  | 9 |  |  | 900.00 |  | L1 |  | TR |  |  | The speed and distance monitoring information is removed |  | O |  | DMI |  | D3. Verify that the distance to target digital is removed. |  |  | L1 |  |  | TR |  | - |  |
|  | 161 |  | 10 |  |  | 915.00 |  | L1 |  | TR |  |  | The on-board is in TR mode. A new MA and track description covering all the MA length are read but rejected because of TR mode. A powerless section with pantograph to be lowered is received. |  | I |  | BTM |  | D4. A powerless section with pantograph to be lowered is received with : M_TRACKCOND = 3 D_TRACKCOND and L_TRACKCOND set in order to define this area as soon as possible and before the switch to PT. |  |  | L1 |  |  | TR |  | - |  |
|  | 162 |  | 11 |  |  | 915.00 |  | L1 |  | TR |  |  | The received BTM telegram is RECORDED. |  | O |  | JRU |  | D4. |  |  | L1 |  |  | TR |  | - |  |
|  | 163 |  | 12 |  |  | 915.00 |  | L1 |  | TR |  |  | EB is still applied. |  | O |  | TIU |  | D4. The received MA is not taken into account. |  |  | L1 |  |  | TR |  | - |  |
|  | 164 |  | 13 |  |  | 915.00 |  | L1 |  | TR |  |  | The EB application icon is still DISPLAYED (ST01 icon in the C area). |  | O |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 165 |  | 14 |  |  | 915.00 |  | L1 |  | TR |  |  | The newly received MA should not be displayed in the planning area. |  | O |  | DMI |  | The received MA is not taken into account. |  |  | L1 |  |  | TR |  | - |  |
|  | 166 |  | 15 |  |  | 915.00 |  | L1 |  | TR |  |  | The newly received track conditions should not be displayed in the planning area. |  | O |  | DMI |  | If 'Air conditioning intake closed' is displayed at this distance/ the track condition newly received has not been rejected. |  |  | L1 |  |  | TR |  | - |  |
|  | 167 |  | 16 |  |  | 915.00 |  | L1 |  | TR |  |  | No EB revocation is RECORDED. |  | O |  | JRU |  | The received MA is not taken into account. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5180200: 5.18.2: Passing a powerless section with pantograph to be lowered Test Case 1: The on-board equipment has received track condition information indicating the location of a powerless section where lowering of the pantograph is required. The on-board equipment displays to the driver a profile indicating the location where lowering of the pantograph is required and where raising of the pantograph is permitted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 169 |  | 1 |  |  | 915.00 |  | L1 |  | TR |  |  | 'Lower pantograph announcement' is DISPLAYED. It is DISPLAYED if the related functionality is executed (application dependent) -automatically (symbol TC02) -manually (the driver is requested to act) (symbol TC03) |  | O |  | DMI |  | When the max safe front end of the train reaches a location in rear of the beginning of the powerless section, taking into account the time necessary for performing the required actions (see Subset-076-6-8) and the current train speed. |  |  | L1 |  |  | TR |  | - |  |
|  | 170 |  | 2 |  |  | 915.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=1¦Bit46=1>) is recorded |  | O |  | JRU |  | TC02 (automatic execution) or TC03 (driver requested to act) |  |  | L1 |  |  | TR |  | - |  |
|  | 171 |  | 3 |  |  | 940.00 |  | L1 |  | TR |  |  | 'Lower pantograph announcement' (symbol TC02 or symbol TC03) is REMOVED. |  | O |  | DMI |  | As soon as the train reaches the start location of the powerless section with its max safe front end ( = D_TRACKCOND ) |  |  | L1 |  |  | TR |  | - |  |
|  | 172 |  | 4 |  |  | 940.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit45=0&Bit46=0>) is recorded |  | O |  | JRU |  | TC02 (automatic execution) TC03 (driver requested to act) |  |  | L1 |  |  | TR |  | - |  |
|  | 173 |  | 5 |  |  | 940.00 |  | L1 |  | TR |  |  | 'Lowered pantograph' (symbol TC01) is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 174 |  | 6 |  |  | 940.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit44=1>) is recorded |  | O |  | JRU |  | TC01 |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 2: The train is running in L1 with an MA, track description, and a stored track condition. The train switches to TR mode due to a balise group (BGa) sending a message with an incompatible ETCS version. During the TR mode, it receives an new MA which should be rejected and a new track condition which should be accepted. The track conditions received during the TR mode and before the switch to TR mode are applied during the TR mode and before it reaches standstill. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 175 |  | 18 |  |  | 950.00 |  | L1 |  | TR |  |  | The Track Condition symbol "Close air conditioning intake announcement" is removed. |  | O |  | DMI |  | D5: The train reaches the distance of the track condition area received before the TRIP procedure started. Only if the related functionality is manually executed by the driver. Symbol is TC21. |  |  | L1 |  |  | TR |  | - |  |
|  | 176 |  | 19 |  |  | 950.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit64=0) is recorded. |  | O |  | JRU |  | D5. Only if the related functionality is manually executed by the driver. |  |  | L1 |  |  | TR |  | - |  |
|  | 177 |  | 20 |  |  | 950.00 |  | L1 |  | TR |  |  | The Track Condition symbol "Air conditioning intake closed" is displayed. |  | O |  | DMI |  | D5. Only if the related functionality is manually executed by the driver. Symbol is TC19. |  |  | L1 |  |  | TR |  | - |  |
|  | 178 |  | 21 |  |  | 950.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=1) is recorded. |  | O |  | JRU |  | D5. Only if the related functionality is manually executed by the driver. |  |  | L1 |  |  | TR |  | - |  |
|  | 179 |  | 22 |  |  | 1060.00 |  | L1 |  | TR |  |  | The Track Condition symbol "Air conditioning intake closed" is removed. |  | O |  | DMI |  | D6: The min safe rear end of the train reaches the end location of the air tightness area. Symbol is TC19. |  |  | L1 |  |  | TR |  | - |  |
|  | 180 |  | 23 |  |  | 1060.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit62=0) is recorded. |  | O |  | JRU |  | D6 |  |  | L1 |  |  | TR |  | - |  |
|  | 181 |  | 24 |  |  | 1060.00 |  | L1 |  | TR |  |  | The Track Condition symbol "Open air conditioning intake" is displayed. |  | O |  | DMI |  | D6. The symbol TC20 shall be used to indicate an automatic execution/ the symbol TC22 when the driver is requested to act. |  |  | L1 |  |  | TR |  | - |  |
|  | 182 |  | 25 |  |  | 1060.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=1¦Bit65=1) is recorded. |  | O |  | JRU |  | D6 |  |  | L1 |  |  | TR |  | - |  |
|  | 183 |  | 26 |  |  | 1060.00 |  | L1 |  | TR |  |  | Close air tightness. |  | O |  | TIU |  | D6. Automatic only if the EVC is capable of commanding the air tightness system. Otherwise it is manual/ the driver action will be required. |  |  | L1 |  |  | TR |  | - |  |
|  | 184 |  | 27 |  |  | 1060.00 |  | L1 |  | TR |  |  | The Track Condition symbol "Open air conditioning intake" is removed. |  | O |  | DMI |  | D6 + 5 seconds. The symbol TC20 shall be used to indicate an automatic execution/ the symbol TC22 when the driver is requested to act. |  |  | L1 |  |  | TR |  | - |  |
|  | 185 |  | 28 |  |  | 1060.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21;Bit63=0¦Bit65=0) is recorded. |  | O |  | JRU |  | D6 + 5 seconds. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 186 |  | 1 |  |  | 1080.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 187 |  | 2 |  |  | 1090.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 188 |  | 3 |  |  | 1090.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 189 |  | 4 |  |  | 1090.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 190 |  | 5 |  |  | 1090.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 191 |  | 6 |  |  | 1090.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 192 |  | 7 |  |  | 1090.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 195 |  | 10 |  |  | 1090.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 196 |  | 11 |  |  | 1090.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 197 |  | 12 |  |  | 1090.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 198 |  | 13 |  |  | 1090.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 7: The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start', because the train data are available on-board (new TC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 199 |  | 1 |  |  | 1090.00 |  | L1 |  | PT |  |  | The On-Board equipment waits for the driver to select 'Start'. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 200 |  | 2 |  |  | 1090.00 |  | L1 |  | PT |  |  | The Driver selects 'Start' |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 201 |  | 3 |  |  | 1090.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 202 |  | 4 |  |  | 1090.00 |  | L1 |  | PT |  |  | The Mode symbol "Acknowledge Staff Responsible" is displayed. |  | O |  | DMI |  | DMI Object: Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 203 |  | 5 |  |  | 1090.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 49: Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 204 |  | 1 |  |  | 1090.00 |  | L1 |  | PT |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 205 |  | 2 |  |  | 1090.00 |  | L1 |  | PT |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 206 |  | 3 |  |  | 1090.00 |  | L1 |  | PT |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 207 |  | 4 |  |  | 1090.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | Symbol MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 208 |  | 5 |  |  | 1090.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 216 |  | 1 |  |  | 1150.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 217 |  | 2 |  |  | 1150.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 51 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 50.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100000 b |  |  |

| - Step 53 : Message 159 - Session Established (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 56 : Message 129 - Validated Train Data (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 38 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0010 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 110 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 44 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 d |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 58 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 50.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 60 : Message 3 - Movement Authority (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 64 : Message 16 - Unconditional Emergency Stop (RBC n? 1) - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 16 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_EM | 4 | 11 d |  |  |

| - Step 66 : Message 3 - Movement Authority (RBC n? 1) - Distance 500.00 m Delay : 2.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 75 : Message 147 - Acknowledgement of Emergency Stop (RBC n? 1) - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 147 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_EM | 4 | 11 d |  |  |
| Q_EMERGENCYSTOP | 2 | 10 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 450 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 78 : Message 136 - Train Position Report (RBC n? 1) - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 450 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0111 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 102 : Message 136 - Train Position Report (RBC n? 1) - Distance 540.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 490 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 108 : Message 6 - Recognition of Exit From TRIP Mode (RBC n? 1) - Distance 540.00 m Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 6 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 110 : Message 18 - Revocation of Emergency Stop (RBC n? 1) - Distance 540.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 18 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_EM | 4 | 11 d |  |  |

| - Step 115 : Message 132 - MA Request (RBC n? 1) - Distance 540.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 0 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 490 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 119 : Message 3 - Movement Authority (RBC n? 1) - Distance 540.00 m Delay : 15.00 s Conditional Message Id : 132 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 124 : Balise Group BG 5 d 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 124 : Balise Group BG 5 d 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 128 : Message 24 - General Message (RBC n? 1) - Distance 650.00 m Delay : 4.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 130 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 132 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 650.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |

| - Step 135 : Balise Group BG 06 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 d |  |  |
| D_TRACKCOND | 15 | 100 d |  |  |
| L_TRACKCOND | 15 | 110 d |  |  |
| M_TRACKCOND | 4 | 0101 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Balise Group BG 06 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 146 : Balise Group BG 77 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0110000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 77 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 146 : Balise Group BG 77 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0110000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 77 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 161 : Balise Group BG 88 d 1/2 - Distance 915.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 88 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 99 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 b |  |  |
| D_TRACKCOND | 15 | 25 d |  |  |
| L_TRACKCOND | 15 | 300 d |  |  |
| M_TRACKCOND | 4 | 0011 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_TRACKCOND | 15 | 10 d |  |  |
| L_TRACKCOND | 15 | 110 d |  |  |
| M_TRACKCOND | 4 | 5 h |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 161 : Balise Group BG 88 d 2/2 - Distance 920.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 88 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 30 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |