| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3120300_02_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3120300_02_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 21:01 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | MLT |

# Sequence description

3.12.3 Text Transmission

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  | Dist. (m) |  | Previous |  |  |  |  |  | Description of Events |  |  | I/O |  | Interface |  |  | Comments |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  | 0.00 |  | N/A |  |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  |  | I |  | INT |  |  | The on-board equipment changes to SB mode. |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  | 0.00 |  | N/A |  |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | Driver opens desk |  |  | I |  | TIU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The actual mode SB is DISPLAYED |  |  | O |  | DMI |  |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 25: During a start of mission, the driver shall have the opportunity to set a new VBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. |  |  | O |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver chooses to set/remove a Virtual Balise Cover |  |  | I |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver sets and validate a new VBC. |  |  | I |  | DMI |  |  | - _____ The driver sets a VBC with VBC code 69633 This code corresponds to: - NID_VBCMK=1 - NID_C=64 - T_VBC=1 |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  | 0.00 |  | N/A |  |  | SB |  |  | The new VBC is RECORDED. |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 5: During a start of mission, the driver shall have the opportunity to set a new VBC with the same VBC identity as an already stored VBC. The new VBC shall replace the previous one, including its validity period. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. |  |  | O |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver chooses to set/remove a Virtual Balise Cover |  |  | I |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver sets and validate a new VBC with the same VBC identity as an already stored VBC and a different validity period |  |  | I |  | DMI |  |  | The VBC has the same NID_VBCMK and NID_C as an already stored and a different T_VBC ERA_ERTMS_015560 v330: When converting the entered ‘VBC set code’ into binary format, the 6 less significant bits shall form the variable NID_VBCMK, the next 10 bits shall form the variable NID_C and the remaining bits shall form the variable T_VBC _____ The driver sets a VBC with VBC code 135169 This code corresponds to: - NID_VBCMK=1 - NID_C=64 - T_VBC=2 |  | N/A |  |  | SB |  | - |  |
|  | 13 |  | 4 |  | 0.00 |  | N/A |  |  | SB |  |  | The new VBC is RECORDED with the correct validity period. |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 23: During a start of mission, the driver shall have the opportunity to remove an existing VBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 14 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. |  |  | O |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 15 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver chooses to set/remove a Virtual Balise Cover |  |  | I |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 16 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver removes an existing VBC. |  |  | I |  | DMI |  |  | - _____ The driver removes a VBC with VBC code 135169 by entering 1088 in remove VBC This code corresponds to: - NID_VBCMK=1 - NID_C=64 - T_VBC=2 |  | N/A |  |  | SB |  | - |  |
|  | 17 |  | 4 |  | 0.00 |  | N/A |  |  | SB |  |  | The deletion of VBC is RECORDED. |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  |  | O |  | DMI |  |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  |  | SB |  | - |  |
|  | 19 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  |  | O |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 20 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver enters and validates a correct Driver ID |  |  | I |  | DMI |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 21 |  | 4 |  | 0.00 |  | N/A |  |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  |  | O |  | JRU |  |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 22 |  | 1 |  | 0.00 |  | N/A |  |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  |  | I |  | DMI |  |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  | N/A |  |  | SB |  | - |  |
|  | 23 |  | 2 |  | 0.00 |  | N/A |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  |  | O |  | JRU |  |  | - |  | N/A |  |  | SB |  | - |  |
|  | 24 |  | 3 |  | 0.00 |  | N/A |  |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  |  | O |  | DMI |  |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver selects ‘Train Data Entry’ |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | A Train data window is is displayed on the DMI. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 28 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver’s selection is recorded on the JRU. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  |  | O |  | DMI |  |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver enters each Train data unknown on-board |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver is allowed to validate the train data |  |  | O |  | DMI |  |  | A complete set of valid Train Data is available |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver validates the train data. |  |  | I |  | DMI |  |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 5 |  | 0.00 |  | L1 |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  |  | O |  | JRU |  |  | Triggered by : The driver validation. |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 6 |  | 0.00 |  | L1 |  |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  |  | O |  | JRU |  |  | Triggered by : Data Entry completed |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 37 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver is requested to enter the Train running number. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 38 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  |  | I |  | DMI |  |  | Entered Train Running Number: NID_OPERATIONAL_1 |  | L1 |  |  | SB |  | - |  |
|  | 39 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 1 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver selects 'Start'. |  |  | I |  | DMI |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 42 |  | 2 |  | 0.00 |  | L1 |  |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  |  | O |  | JRU |  |  | Triggered by: The driver acts on the on-board DMI. |  | L1 |  |  | SB |  | - |  |
|  | 43 |  | 3 |  | 0.00 |  | L1 |  |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  |  | O |  | DMI |  |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  | L1 |  |  | SB |  | - |  |
|  | 44 |  | 4 |  | 0.00 |  | L1 |  |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 45 |  | 5 |  | 0.00 |  | L1 |  |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  |  | I |  | DMI |  |  | Symbol MO10 |  | L1 |  |  | SB |  | - |  |
|  | 46 |  | 6 |  | 0.00 |  | L1 |  |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SB |  | - |  |
|  | 47 |  | 7 |  | 0.00 |  | L1 |  |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 48 |  | 8 |  | 0.00 |  | L1 |  |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  |  | O |  | DMI |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 49 |  | 9 |  | 0.00 |  | L1 |  |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 50 |  | 10 |  | 0.00 |  | L1 |  |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  | 1 |  | 50.00 |  | L1 |  |  | SR |  |  | One Balise Group message containing packet 72 is received |  |  | I |  | BTM |  |  | If in stand by mode the balise group consists of only one balise. |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 2 |  | 50.00 |  | L1 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  | 100.00 |  | L1 |  |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  |  | I |  | BTM |  |  | Packet 80 not included for current location |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 2 |  | 100.00 |  | L1 |  |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | SR |  | - |  |
|  | 58 |  | 3 |  | 100.00 |  | L1 |  |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  |  | O |  | JRU |  |  | The on-board equipment switches immediately to FS mode |  | L1 |  |  | FS |  | - |  |
|  | 59 |  | 4 |  | 100.00 |  | L1 |  |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  |  | O |  | DMI |  |  | DMI Object: Symbol MO11 |  | L1 |  |  | FS |  | - |  |
|  | 60 |  | 5 |  | 100.00 |  | L1 |  |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 3: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  | 2 |  | 100.00 |  | L1 |  |  | FS |  |  | The "plain text message" is displayed |  |  | O |  | DMI |  |  | The end conditions are evaluated. The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED |  | L1 |  |  | FS |  | - |  |
|  | 62 |  | 3 |  | 100.00 |  | L1 |  |  | FS |  |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  |  | O |  | JRU |  |  | The display of the message is RECORDED |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 3: Limited Supervision is requested for a further location when running in FS mode. An acknowledgement request for LS mode is not displayed to the driver while the train speed is higher than the LS mode permitted speed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  | 200.00 |  | L1 |  |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  |  | I |  | BTM |  |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) |  | L1 |  |  | FS |  | - |  |
|  | 66 |  | 3 |  | 200.00 |  | L1 |  |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  |  | O |  | JRU |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 67 |  | 4 |  | 250.00 |  | L1 |  |  | FS |  |  | The train reaches the acknowledgement window and V> V_MAMODE. The request for acknowledgement is not displayed. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 68 |  | 5 |  | 350.00 |  | L1 |  |  | FS |  |  | The train speed decreases to a value lower than V_MAMODE and the acknowledgement request is displayed. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 69 |  | 6 |  | 400.00 |  | L1 |  |  | FS |  |  | The driver acknowledges the transition to LS mode. |  |  | I |  | DMI |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 70 |  | 7 |  | 400.00 |  | L1 |  |  | LS |  |  | Mode transition to LS is RECORDED |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 3: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 5 |  | 400.00 |  | L1 |  |  | LS |  |  | The "plain text message" is removed |  |  | O |  | DMI |  |  | The plain text message ‘X_TEXT(L_TEXT)’ is REMOVED |  | L1 |  |  | LS |  | - |  |
|  | 74 |  | 6 |  | 400.00 |  | L1 |  |  | LS |  |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  |  | O |  | JRU |  |  | The deletion of the message is RECORDED |  | L1 |  |  | LS |  | - |  |
|  | Feature 4080414: 4.8.4: Accepted Information depending on the modes - Fixed Text Information Test Case 3: A fixed text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  | 1 |  | 450.00 |  | L1 |  |  | LS |  |  | One Balise Group message containing packet 76 is received |  |  | I |  | BTM |  |  | If in standby mode the balise group consists of only one balise. |  | L1 |  |  | LS |  | - |  |
|  | 76 |  | 2 |  | 450.00 |  | L1 |  |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  | 1 |  | 500.00 |  | L1 |  |  | LS |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  |  | I |  | BTM |  |  | Packet 80 not included for current location |  | L1 |  |  | LS |  | - |  |
|  | 81 |  | 2 |  | 500.00 |  | L1 |  |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 82 |  | 3 |  | 500.00 |  | L1 |  |  | LS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  |  | O |  | JRU |  |  | The on-board equipment switches immediately to FS mode |  | L1 |  |  | FS |  | - |  |
|  | 83 |  | 4 |  | 500.00 |  | L1 |  |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  |  | O |  | DMI |  |  | DMI Object: Symbol MO11 |  | L1 |  |  | FS |  | - |  |
|  | 84 |  | 5 |  | 500.00 |  | L1 |  |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 11: The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 85 |  | 2 |  | 500.00 |  | L1 |  |  | FS |  |  | The "fixed text message" is displayed |  |  | O |  | DMI |  |  | The fixed text message ‘Q_TEXT’ is DISPLAYED in the language selected by the driver |  | L1 |  |  | FS |  | - |  |
|  | 86 |  | 3 |  | 500.00 |  | L1 |  |  | FS |  |  | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded |  |  | O |  | JRU |  |  | The display of the message is RECORDED |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 3: Limited Supervision is requested for a further location when running in FS mode. An acknowledgement request for LS mode is not displayed to the driver while the train speed is higher than the LS mode permitted speed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  | 1 |  | 600.00 |  | L1 |  |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  |  | I |  | BTM |  |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 3 |  | 600.00 |  | L1 |  |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  |  | O |  | JRU |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 91 |  | 4 |  | 650.00 |  | L1 |  |  | FS |  |  | The train reaches the acknowledgement window and V> V_MAMODE. The request for acknowledgement is not displayed. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 92 |  | 5 |  | 750.00 |  | L1 |  |  | FS |  |  | The train speed decreases to a value lower than V_MAMODE and the acknowledgement request is displayed. |  |  | O |  | DMI |  |  | - |  | L1 |  |  | FS |  | - |  |
|  | 93 |  | 6 |  | 800.00 |  | L1 |  |  | FS |  |  | The driver acknowledges the transition to LS mode. |  |  | I |  | DMI |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 94 |  | 7 |  | 800.00 |  | L1 |  |  | LS |  |  | Mode transition to LS is RECORDED |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 11: The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified mode. The end condition is limited to a condition on exit from a specified mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 5 |  | 800.00 |  | L1 |  |  | LS |  |  | The "fixed text message" is removed |  |  | O |  | DMI |  |  | The fixed text message ‘Q_TEXT’ is REMOVED |  | L1 |  |  | LS |  | - |  |
|  | 98 |  | 6 |  | 800.00 |  | L1 |  |  | LS |  |  | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded |  |  | O |  | JRU |  |  | The deletion of the message is RECORDED |  | L1 |  |  | LS |  | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  | 1 |  | 850.00 |  | L1 |  |  | LS |  |  | One Balise Group message containing packet 72 is received |  |  | I |  | BTM |  |  | If in stand by mode the balise group consists of only one balise. |  | L1 |  |  | LS |  | - |  |
|  | 100 |  | 2 |  | 850.00 |  | L1 |  |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 104 |  | 1 |  | 900.00 |  | L1 |  |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  |  | I |  | BTM |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 105 |  | 2 |  | 900.00 |  | L1 |  |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 106 |  | 3 |  | 900.00 |  | L1 |  |  | LS |  |  | Level 2 is DISPLAYED. |  |  | O |  | DMI |  |  | (Except in SL mode) The On-board equipment switches to Level 2. |  | L2 |  |  | LS |  | - |  |
|  | 107 |  | 4 |  | 900.00 |  | L2 |  |  | LS |  |  | New Level 2 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 4: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 2 |  | 900.00 |  | L2 |  |  | LS |  |  | The "plain text message" is displayed |  |  | O |  | DMI |  |  | The end conditions are evaluated. The plain text message ‘X_TEXT(L_TEXT)’ is DISPLAYED |  | L2 |  |  | LS |  | - |  |
|  | 109 |  | 3 |  | 900.00 |  | L2 |  |  | LS |  |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 32: The train receives an order to switch immediately from L2 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  | 950.00 |  | L2 |  |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. • D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  |  | I |  | BTM |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | 112 |  | 2 |  | 950.00 |  | L2 |  |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | 113 |  | 3 |  | 950.00 |  | L2 |  |  | LS |  |  | Level 1 is DISPLAYED. |  |  | O |  | DMI |  |  | The On-board equipment switches to Level 1. |  | L1 |  |  | LS |  | - |  |
|  | 114 |  | 4 |  | 950.00 |  | L1 |  |  | LS |  |  | New Level 1 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 4: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 5 |  | 950.00 |  | L1 |  |  | LS |  |  | The "plain text message" is removed |  |  | O |  | DMI |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 116 |  | 6 |  | 950.00 |  | L1 |  |  | LS |  |  | STOP DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=19) is recorded |  |  | O |  | JRU |  |  | The deletion of the message is RECORDED |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 1 |  | 1000.00 |  | L1 |  |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  |  | I |  | BTM |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 119 |  | 2 |  | 1000.00 |  | L1 |  |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 120 |  | 3 |  | 1000.00 |  | L1 |  |  | LS |  |  | Level 2 is DISPLAYED. |  |  | O |  | DMI |  |  | (Except in SL mode) The On-board equipment switches to Level 2. |  | L2 |  |  | LS |  | - |  |
|  | 121 |  | 4 |  | 1000.00 |  | L2 |  |  | LS |  |  | New Level 2 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 4: The on-board equipment has received information about plain text messages. The plain text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  | 8 |  | 1000.00 |  | L2 |  |  | LS |  |  | The "plain text message" is NOT displayed |  |  | O |  | DMI |  |  | The plain text message ‘X_TEXT(L_TEXT)’ is NOT displayed. The text message was already displayed and the end condition was already fulfilled, the start location shall not be re-evaluated. |  | L2 |  |  | LS |  | - |  |
|  | Feature 4080414: 4.8.4: Accepted Information depending on the modes - Fixed Text Information Test Case 3: A fixed text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  | 1 |  | 1050.00 |  | L2 |  |  | LS |  |  | One Balise Group message containing packet 76 is received |  |  | I |  | BTM |  |  | If in standby mode the balise group consists of only one balise. |  | L2 |  |  | LS |  | - |  |
|  | 124 |  | 2 |  | 1050.00 |  | L2 |  |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 32: The train receives an order to switch immediately from L2 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  | 1100.00 |  | L2 |  |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. • D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  |  | I |  | BTM |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | 129 |  | 2 |  | 1100.00 |  | L2 |  |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | 130 |  | 3 |  | 1100.00 |  | L2 |  |  | LS |  |  | Level 1 is DISPLAYED. |  |  | O |  | DMI |  |  | The On-board equipment switches to Level 1. |  | L1 |  |  | LS |  | - |  |
|  | 131 |  | 4 |  | 1100.00 |  | L1 |  |  | LS |  |  | New Level 1 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 12: The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  | 2 |  | 1100.00 |  | L1 |  |  | LS |  |  | The "fixed text message" is displayed |  |  | O |  | DMI |  |  | The fixed text message ‘Q_TEXT’ is DISPLAYED in the language selected by the driver. |  | L1 |  |  | LS |  | - |  |
|  | 133 |  | 3 |  | 1100.00 |  | L1 |  |  | LS |  |  | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded |  |  | O |  | JRU |  |  | The display of the message is RECORDED |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  | 1150.00 |  | L1 |  |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  |  | I |  | BTM |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 136 |  | 2 |  | 1150.00 |  | L1 |  |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  |  | O |  | JRU |  |  | - |  | L1 |  |  | LS |  | - |  |
|  | 137 |  | 3 |  | 1150.00 |  | L1 |  |  | LS |  |  | Level 2 is DISPLAYED. |  |  | O |  | DMI |  |  | (Except in SL mode) The On-board equipment switches to Level 2. |  | L2 |  |  | LS |  | - |  |
|  | 138 |  | 4 |  | 1150.00 |  | L2 |  |  | LS |  |  | New Level 2 is RECORDED. |  |  | O |  | JRU |  |  | - |  | L2 |  |  | LS |  | - |  |
|  | Feature 3120300: 3.12.3: Text Transmission Test Case 12: The on-board equipment has received information about fixed text messages. The fixed text message is displayed according to the start / end conditions. The start condition is limited to a condition on entry in a specified level. The end condition is limited to a condition on exit from a specified level. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 |  | 5 |  | 1150.00 |  | L2 |  |  | LS |  |  | The "fixed text message" is removed |  |  | O |  | DMI |  |  | The fixed text message ‘Q_TEXT’ is REMOVED |  | L2 |  |  | LS |  | - |  |
|  | 140 |  | 6 |  | 1150.00 |  | L2 |  |  | LS |  |  | STOP DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=17) is recorded |  |  | O |  | JRU |  |  | The deletion of the message is RECORDED |  | L2 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  | 1200.00 |  | L2 |  |  | LS |  |  | The ERTMS/ETCS on-board unit is powered down |  |  | I |  | INT |  |  | - _____ The train reaches standstill. |  | L2 |  |  | NP |  | - |  |
|  | 142 |  | 2 |  | 1200.00 |  | L2 |  |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  |  | O |  | JRU |  |  | There are no messages recorded in the JRU during a period of time |  | L2 |  |  | NP |  | - |  |
| - Step 51 : Balise Group BG 0 d 1/2 - Distance 50.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Variable |  |  |  |  |  |  |  | Length |  |  | Value |  |  | Comment |  |  |  |  | User comment |  |  |  |  |  |  |  |  |  |  |
| Q_UPDOWN |  |  |  |  |  |  |  | 1 |  |  | 1 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_VERSION |  |  |  |  |  |  |  | 7 |  |  | 0100000 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_MEDIA |  |  |  |  |  |  |  | 1 |  |  | 0 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N_PIG |  |  |  |  |  |  |  | 3 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| N_TOTAL |  |  |  |  |  |  |  | 3 |  |  | 1 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_DUP |  |  |  |  |  |  |  | 2 |  |  | 00 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_MCOUNT |  |  |  |  |  |  |  | 8 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_C |  |  |  |  |  |  |  | 10 |  |  | 64 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_BG |  |  |  |  |  |  |  | 14 |  |  | 0 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_LINK |  |  |  |  |  |  |  | 1 |  |  | 1 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_PACKET |  |  |  |  |  |  |  | 8 |  |  | 72 d |  |  | Packet for sending plain text messages |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_DIR |  |  |  |  |  |  |  | 2 |  |  | 01 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L_PACKET |  |  |  |  |  |  |  | 13 |  |  | 244 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_SCALE |  |  |  |  |  |  |  | 2 |  |  | 01 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_TEXTCLASS |  |  |  |  |  |  |  | 2 |  |  | 00 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_TEXTDISPLAY |  |  |  |  |  |  |  | 1 |  |  | 0 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| D_TEXTDISPLAY |  |  |  |  |  |  |  | 15 |  |  | 32767 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_MODETEXTDISPLAY |  |  |  |  |  |  |  | 4 |  |  | 0000 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_LEVELTEXTDISPLAY |  |  |  |  |  |  |  | 3 |  |  | 101 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L_TEXTDISPLAY |  |  |  |  |  |  |  | 15 |  |  | 300 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| T_TEXTDISPLAY |  |  |  |  |  |  |  | 10 |  |  | 1023 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_MODETEXTDISPLAY |  |  |  |  |  |  |  | 4 |  |  | 0000 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| M_LEVELTEXTDISPLAY |  |  |  |  |  |  |  | 3 |  |  | 101 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Q_TEXTCONFIRM |  |  |  |  |  |  |  | 2 |  |  | 00 b |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| L_TEXT |  |  |  |  |  |  |  | 8 |  |  | 19 d |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | e |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | s |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | t |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | o |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | f |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | F |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | T |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | . |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| X_TEXT |  |  |  |  |  |  |  | 8 |  |  | 3 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NID_PACKET |  |  |  |  |  |  |  | 8 |  |  | 255 d |  |  | End of information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| - Step 51 : Balise Group BG 0 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 64 : Balise Group BG 2 d 1/2 - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 1000 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1000 d |  |  |
| L_ACKMAMODE | 15 | 950 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 64 : Balise Group BG 2 d 2/2 - Distance 205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 75 : Balise Group BG 3 d 1/2 - Distance 450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 76 d | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 92 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 0000 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 0000 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| Q_TEXT | 8 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 75 : Balise Group BG 3 d 2/2 - Distance 455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 80 : Balise Group BG 4 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 80 : Balise Group BG 4 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 88 : Balise Group BG 5 d 1/2 - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 1000 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1000 d |  |  |
| L_ACKMAMODE | 15 | 950 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 88 : Balise Group BG 5 d 2/2 - Distance 605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 6 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 244 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 111111111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 3 d |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 3 d |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 19 d |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | f |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | F |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 1 |  |  |
| X_TEXT | 8 | 2 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 3 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | 0 |  |  |
| X_TEXT | 8 | . |  |  |
| X_TEXT | 8 | 4 |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 6 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 104 : Balise Group BG 7 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 104 : Balise Group BG 7 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 111 : Balise Group BG 8 d 1/2 - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 111 : Balise Group BG 8 d 2/2 - Distance 955.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 118 : Balise Group BG 9 d 1/2 - Distance 1000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 118 : Balise Group BG 9 d 2/2 - Distance 1005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 123 : Balise Group BG 10 d 1/2 - Distance 1050.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 76 d | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 92 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 1 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 2 d |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 15 d |  |  |
| M_LEVELTEXTDISPLAY | 3 | 2 d |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| Q_TEXT | 8 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 123 : Balise Group BG 10 d 2/2 - Distance 1055.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 128 : Balise Group BG 11 d 1/2 - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 128 : Balise Group BG 11 d 2/2 - Distance 1105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Balise Group BG 12 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Balise Group BG 12 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |