| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3080200_06_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3080200_06_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 19:27 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | DLR |

# Sequence description

TC7: The train is running in ERTMS level 2 or 3 and reaches the location to issue the first MA request. The on-board equipment then cyclically requests the RBC for a new MA. The train reaches the end of the current MA and stops since no new MA is received from the RBC.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | SB |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 100.00 |  | L2 |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | SR |  | - |  |
|  | 44 |  | 2 |  |  | 100.00 |  | L2 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 45 |  | 3 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | SR |  | - |  |
|  | 46 |  | 4 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 47 |  | 5 |  |  | 100.00 |  | L2 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 48 |  | 6 |  |  | 100.00 |  | L2 |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 49 |  | 7 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 50 |  | 8 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 51 |  | 9 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SR |  | - |  |
|  | 52 |  | 10 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 53 |  | 11 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 54 |  | 12 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | SR |  | - |  |
|  | 57 |  | 2 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 58 |  | 3 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 59 |  | 4 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 21: The train is in level 2 or 3 in SR and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train The system switches to FS immediately. Transition Condition Id [31]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 650.00 |  | L2 |  | SR |  |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | The track description does not cover the full length of the train. _____ Includes Packet 57 |  |  | L2 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 650.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 650.00 |  | L2 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode even if the track description is not available for the whole train in this moment. |  |  | L2 |  |  | FS |  | - |  |
|  | 63 |  | 4 |  |  | 650.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 64 |  | 5 |  |  | 650.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3080200: 3.8.2: MA request to the RBC Test Case 7: The train is running in ERTMS level 2 or 3 and reaches the location to issue the first MA request. The on-board equipment then cyclically requests the RBC for a new MA. The train reaches the end of the current MA and stops since no new MA is received from the RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  | 1 |  |  | 690.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | The train is running and reaches the location to issue the first MA request : T = t’. At T = t’ + T_CYCRQST second MA request is issued. Check that the difference of time between two consecutive MA requests is effectively equal to T_CYCRQST (10 sec). Steps 1 and 2 are executed repeatedly every T_CYCRQST seconds N times (N is chosen by the tester but shall be at least 2). |  |  | L2 |  |  | FS |  | - |  |
|  | 68 |  | 2 |  |  | 690.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 69 |  | 3 |  |  | 750.00 |  | L2 |  | FS |  |  | Train is at standstill (V_TRAIN=0; TIME= 60) |  | I |  | INT |  | The train is at standstill at the end of the current MA |  |  | L2 |  |  | FS |  | - |  |
|  | 70 |  | 4 |  |  | 750.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | The train informs the RBC about standstill. |  |  | L2 |  |  | FS |  | - |  |
|  | 71 |  | 5 |  |  | 750.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 72 |  | 6 |  |  | 750.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | The on-board equipment carries on requesting the RBC for a MA every T_CYCRQST seconds until the driver presses 'Override EoA' button Steps 6 and 7 are executed repeatedly until the driver presses 'Override EoA' button. |  |  | L2 |  |  | FS |  | - |  |
|  | 73 |  | 7 |  |  | 750.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  |  | 750.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 75 |  | 2 |  |  | 750.00 |  | L2 |  | FS |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | FS |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 01 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 01 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 43 : Balise Group BG 2 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 43 : Balise Group BG 2 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 51 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |

| - Step 53 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 56 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 2 d |  |  |
| M_LEVEL | 3 | 3 |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 58 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 60 : Message 3 - Movement Authority (RBC n? 1) - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 51 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1400 d |  |  |
| Q_SECTIONTIMER | 1 | 1 |  |  |
| T_SECTIONTIMER | 10 | 70 d |  |  |
| D_SECTIONTIMERSTOPLOC | 15 | 900 d |  |  |
| Q_ENDTIMER | 1 | 0 |  |  |
| Q_DANGERPOINT | 1 | 1 |  |  |
| D_DP | 15 | 0 d |  |  |
| V_RELEASEDP | 7 | 2 d |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 5000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 11111111 b |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 16 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 5000 d |  |  |
| V_STATIC | 7 | 1111111 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 57 d | Movement authority request parameters |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 49 d |  |  |
| T_MAR | 8 | 3 d |  |  |
| T_TIMEOUTRQST | 10 | 1111111111 b |  |  |
| T_CYCRQST | 8 | 10 d |  |  |

| - Step 67 : Message 132 - MA Request (RBC n? 1) - Distance 690.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 00010 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 600 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 70 : Message 136 - Train Position Report (RBC n? 1) - Distance 750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 650 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 10 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 72 : Message 132 - MA Request (RBC n? 1) - Distance 750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 00010 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 650 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 10 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 |  |  |