| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3110500_02_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3110500_02_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 20:30 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | RINA |

# Sequence description

Speed supervision of Temporary Speed Restriction in L2. Tested TC´s are 4, 6, 7, 8 and 9.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver selects L1. [59.-1.1] |  |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 50.00 |  | L2 |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | SR |  | - |  |
|  | 44 |  | 2 |  |  | 50.00 |  | L2 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 45 |  | 3 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | SR |  | - |  |
|  | 46 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 47 |  | 5 |  |  | 50.00 |  | L2 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 48 |  | 6 |  |  | 50.00 |  | L2 |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 49 |  | 7 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 50 |  | 8 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 51 |  | 9 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SR |  | - |  |
|  | 52 |  | 10 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 53 |  | 11 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 54 |  | 12 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | SR |  | - |  |
|  | 57 |  | 2 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 58 |  | 3 |  |  | 50.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 59 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 21: The train is in level 2 or 3 in SR and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train The system switches to FS immediately. Transition Condition Id [31]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 300.00 |  | L2 |  | SR |  |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | The track description does not cover the full length of the train. |  |  | L2 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 300.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 300.00 |  | L2 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode even if the track description is not available for the whole train in this moment. |  |  | L2 |  |  | FS |  | - |  |
|  | 63 |  | 4 |  |  | 300.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 64 |  | 5 |  |  | 300.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 10: Inhibition of revocable Temporary Speed Restriction from balises in L2/L3. When ERTMS/ETCS on-board equipment has accepted an order to reject revocable TSRs from balises, this inhibition shall be stored and shall be effective immediately, but only for revocable TSRs received from balises thereafter. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  | 1 |  |  | 350.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 64 is received |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 68 |  | 2 |  |  | 350.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 69 |  | 3 |  |  | 500.00 |  | L2 |  | FS |  |  | Balise Group message containing packet 65 is received |  | I |  | BTM |  | Packet 65 with NID_TSR = 0-126 (revocable TSR) |  |  | L2 |  |  | FS |  | - |  |
|  | 70 |  | 4 |  |  | 500.00 |  | L2 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 71 |  | 5 |  |  | 579.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | The train drives inside TSR area with it maximum safe front end. |  |  | L2 |  |  | FS |  | - |  |
|  | 74 |  | 8 |  |  | 579.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Speed value defined as V_TSR in stored message will not be used by the on-board equipment. If in FS or OS mode Permitted speed does not change to the TSR speed. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. |  |  | L2 |  |  | FS |  | - |  |
|  | 75 |  | 9 |  |  | 579.00 |  | L2 |  | FS |  |  | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <other than V_TSR>) is recorded |  | O |  | JRU |  | Speed and distance monitoring information does not change. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 77 |  | 1 |  |  | 1100.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | FS |  | - |  |
|  | 78 |  | 2 |  |  | 1100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 79 |  | 3 |  |  | 1100.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | FS |  | - |  |
|  | 80 |  | 4 |  |  | 1100.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 81 |  | 5 |  |  | 1100.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | FS |  | - |  |
|  | 82 |  | 6 |  |  | 1100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | FS |  | - |  |
|  | 83 |  | 7 |  |  | 1100.00 |  | L2 |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 11: Deletion of the inhibition of revocable Temporary Speed Restriction from balises in L2/L3 due to termination of the communication session with the RBC that order the inhibition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 2 |  |  | 1400.00 |  | L2 |  | FS |  |  | Balise Group message containing packet 65 is received. |  | I |  | BTM |  | Packet 65 with NID_TSR = 0-126 (revocable TSR), D_TSR=0 V_TSR > actual train speed |  |  | L2 |  |  | FS |  | - |  |
|  | 85 |  | 3 |  |  | 1400.00 |  | L2 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 86 |  | 4 |  |  | 1450.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | The train drives inside TSR area with its maximum safe front end. Speed value defined as V_TSR in stored message will become the permitted speed. |  |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 7 |  |  | 1450.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | CSM - Normal Status- V_TSR If in FS or OS mode: The Permitted Speed is updated with the value of V_TSR. If in LS mode: V_TRAIN < V_TSR |  |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 8 |  |  | 1450.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <TSR speed km/h>>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 91 |  | 9 |  |  | 1650.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Intervention status, so V_TSR < V_TRAIN |  |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 10 |  |  | 1650.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <TSR speed km/h>; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L2 |  |  | FS |  | - |  |
|  | 93 |  | 11 |  |  | 1650.00 |  | L2 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 94 |  | 12 |  |  | 1650.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 95 |  | 13 |  |  | 1650.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is displayed. |  | O |  | DMI |  | Start DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 96 |  | 14 |  |  | 1650.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 97 |  | 15 |  |  | 1675.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Normal status V_TRAIN < V_TSR |  |  | L2 |  |  | FS |  | - |  |
|  | 98 |  | 16 |  |  | 1675.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20, V_PERM= <same value as V_TSR>; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 99 |  | 17 |  |  | 1675.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 100 |  | 18 |  |  | 1675.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 101 |  | 19 |  |  | 1675.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is removed. |  | O |  | DMI |  | Stop DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 102 |  | 20 |  |  | 1675.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 27: Establishing a radio communication initiated by RBC to onboard using NID_RADIO (known by RBC); connection indication is received from the RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 |  | 1 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-CONNECT.Indication is received |  | I |  | RTM |  | Receive of an connection indication from RTM Parameter: Called ETCS-ID, Calling ETCS-ID, application type. The on-board equipment has to compare the called ETCS-ID with ist own ETCS-ID. |  |  | L2 |  |  | FS |  | - |  |
|  | 104 |  | 2 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-CONNECT.Response is transmitted |  | O |  | RTM |  | A safe connection is now set-up |  |  | L2 |  |  | FS |  | - |  |
|  | 105 |  | 3 |  |  | 2000.00 |  | L2 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 106 |  | 4 |  |  | 2000.00 |  | L2 |  | FS |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 107 |  | 5 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=38) is received |  | I |  | RTM |  | Communication session established for on-board |  |  | L2 |  |  | FS |  | - |  |
|  | 108 |  | 6 |  |  | 2000.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 109 |  | 7 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 110 |  | 8 |  |  | 2000.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 111 |  | 9 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 112 |  | 10 |  |  | 2000.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 113 |  | 1 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | FS |  | - |  |
|  | 114 |  | 2 |  |  | 2000.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 115 |  | 3 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 116 |  | 4 |  |  | 2000.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 6: A new Temporary Speed Restriction shall not replace a previously received Temporary Speed Restriction with another identity. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 117 |  | 1 |  |  | 2300.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received |  | I |  | RTM |  | NID_TSR (B) ? NID_TSR (A) D_TSR(B) < D_TSR (A) + L_TSR(A) V_TSR (B) > V_TSR (A) This TSR partially overlaps the first one, and only the most restrictive speed is taken into account in the overlapped area. |  |  | L2 |  |  | FS |  | - |  |
|  | 118 |  | 2 |  |  | 2300.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 119 |  | 3 |  |  | 2500.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | Train location: Inside overlap area |  |  | L2 |  |  | FS |  | - |  |
|  | 122 |  | 6 |  |  | 2500.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Check that the first Temporary Speed Restriction is taken into account for the train speed supervision No modification on permitted speed value occurred _____ V_TSR(A) |  |  | L2 |  |  | FS |  | - |  |
|  | 123 |  | 7 |  |  | 2500.00 |  | L2 |  | FS |  |  | NOT SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | V_TSR(A) is still taken into account and speed and distance monitoring information has not changed. |  |  | L2 |  |  | FS |  | - |  |
|  | 124 |  | 8 |  |  | 2700.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | V_TRAIN > V_TSR(A) Status requested: Intervention status |  |  | L2 |  |  | FS |  | - |  |
|  | 125 |  | 9 |  |  | 2700.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L2 |  |  | FS |  | - |  |
|  | 126 |  | 10 |  |  | 2700.00 |  | L2 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 127 |  | 11 |  |  | 2700.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 128 |  | 12 |  |  | 2700.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is displayed. |  | O |  | DMI |  | Start DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 129 |  | 13 |  |  | 2700.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 130 |  | 14 |  |  | 2725.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | V_TRAIN < V_TSR(A) Status requested: Normal status |  |  | L2 |  |  | FS |  | - |  |
|  | 131 |  | 15 |  |  | 2725.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 132 |  | 16 |  |  | 2725.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 133 |  | 17 |  |  | 2725.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 134 |  | 18 |  |  | 2725.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is removed. |  | O |  | DMI |  | Stop DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 135 |  | 19 |  |  | 2725.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 136 |  | 20 |  |  | 3040.00 |  | L2 |  | FS |  |  | Train is moving |  | I |  | INT |  | Train location: Full train length has left TSR (A) area. |  |  | L2 |  |  | FS |  | - |  |
|  | 137 |  | 21 |  |  | 3040.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Check that the second Temporary Speed Restriction is taken into account for the train speed supervision. If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Overspeed Status. |  |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 22 |  |  | 3040.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20, V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_TSR(B) Only if in LS mode :M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 7: An update for current TSR is transmitted via RBC and is received by the on board ERTMS/ETCS System, & older TSR is revocable. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 |  | 1 |  |  | 3200.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received |  | I |  | RTM |  | NID_TSR ( B) = NID_TSR (A) V_TSR (B) > V_TSR (A) |  |  | L2 |  |  | FS |  | - |  |
|  | 140 |  | 2 |  |  | 3200.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 5 |  |  | 3300.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | CSM - Normal Status- V_TSR(B) If in FS, OS or SR Permitted Speed is updated with the value of V_TSR(B) V_TRAIN < V_TSR (B) |  |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 6 |  |  | 3300.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 145 |  | 7 |  |  | 3300.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | Train location: Inside TSR area |  |  | L2 |  |  | FS |  | - |  |
|  | 146 |  | 8 |  |  | 3900.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Intervention status, so V_TSR(B) < V_TRAIN |  |  | L2 |  |  | FS |  | - |  |
|  | 147 |  | 9 |  |  | 3900.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L2 |  |  | FS |  | - |  |
|  | 148 |  | 10 |  |  | 3900.00 |  | L2 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 149 |  | 11 |  |  | 3900.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 150 |  | 12 |  |  | 3900.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is displayed. |  | O |  | DMI |  | Start DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 151 |  | 13 |  |  | 3900.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 152 |  | 14 |  |  | 4000.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | V_TRAIN < V_TSR(B) Status requested: Normal status If in FS ,OS or SR mode: Indication of permitted speed as V_TSR(B) If in LS mode V_TRAIN is displayed. |  |  | L2 |  |  | FS |  | - |  |
|  | 153 |  | 15 |  |  | 4000.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 154 |  | 16 |  |  | 4000.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 155 |  | 17 |  |  | 4000.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 156 |  | 18 |  |  | 4000.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is removed. |  | O |  | DMI |  | Stop DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 157 |  | 19 |  |  | 4000.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 4: The Temporary Speed Restrictions shall only be revoked on request from the trackside. Reception and acceptance of Temporary Speed Restriction Revocation (packet 66) by on-board if Level 2 or Level 3 is active. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 158 |  | 1 |  |  | 4100.00 |  | L2 |  | FS |  |  | SA-DATA .Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 66 is received |  | I |  | RTM |  | Reception of TSR Revocation order NID_TSR = NID_TSR (A) |  |  | L2 |  |  | FS |  | - |  |
|  | 159 |  | 2 |  |  | 4100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 160 |  | 3 |  |  | 4100.00 |  | L2 |  | FS |  |  | Train is moving |  | I |  | INT |  | The train is located in the area of TSR, given in packet 65. |  |  | L2 |  |  | FS |  | - |  |
|  | 163 |  | 6 |  |  | 4100.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Permitted Speed is less restrictive immediately after the revocation of the TSR, i.e., the PS is higher than the previously received value of V_TSR. If in LS mode train speed will be a bit over the V_TSR. The DMI aspect is Normal Status. |  |  | L2 |  |  | FS |  | - |  |
|  | 164 |  | 7 |  |  | 4100.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= <other than V_TSR>; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080440: 4.8.4: Accepted Information depending on the modes - Temporary Speed Restriction Test Case 4: The trackside generates and sends a Telegram including packet 65(TSR) to a train. The onboard ACCEPTS the message. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 1 |  |  | 4200.00 |  | L2 |  | FS |  |  | One Balise Group message containing packet 65 is received |  | I |  | BTM |  | Complete Telegram is ACCEPTED and stored |  |  | L2 |  |  | FS |  | - |  |
|  | 166 |  | 2 |  |  | 4200.00 |  | L2 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6;NID_PACKET=65) is recorded |  | O |  | JRU |  | Complete Telegram is recorded _____ NID_TSR(1)=255 |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 1: Train receives TSR message where speed < maximum train speed and any other previously stored static speed restriction |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 179 |  | 3 |  |  | 4250.00 |  | L2 |  | FS |  |  | The train runs an optional distance greater than zero but doesn't drive in section defined in TSR. The PS will decrease until it reaches the value of the TSR. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 180 |  | 4 |  |  | 4250.00 |  | L2 |  | FS |  |  | Permitted speed recorded on JRU |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 181 |  | 5 |  |  | 4374.00 |  | L2 |  | FS |  |  | The train drives in TSR area. The maximum safe front end of the train reaches the beginning of the TSR area. Speed value defined as TSR in stored message will be shown to driver on DMI |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 182 |  | 6 |  |  | 4374.00 |  | L2 |  | FS |  |  | Permitted speed recorded on JRU |  | O |  | JRU |  | - _____ NID_TSR(1)=255 V_TSR = 75 km/h |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 8: An update for current TSR is transmitted via RBC and is received by the on board ERTMS/ETCS System, & older TSR is non-revocable. The new TSR is considered as an additional TSR. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 183 |  | 1 |  |  | 4850.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 65 is received |  | I |  | RTM |  | TSR (2) with the same identity as an already received TSR (1); V_TSR (B) > V_TSR (A) |  |  | L2 |  |  | FS |  | - |  |
|  | 184 |  | 2 |  |  | 4850.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 185 |  | 3 |  |  | 5000.00 |  | L2 |  | FS |  |  | Train is moving |  | I |  | INT |  | Train location : Inside overlap area of both TSR´s. |  |  | L2 |  |  | FS |  | - |  |
|  | 188 |  | 6 |  |  | 5000.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | CSM - Normal Status- V_TSR(A) If in FS; OS or SR Permitted Speed is the value of V_TSR(A) and NOT V_TSR (B) If in LS mode; V_TRAIN < V_TSR (A) |  |  | L2 |  |  | FS |  | - |  |
|  | 189 |  | 7 |  |  | 5550.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Status requested: Intervention status, so V_TSR(A) < V_TRAIN |  |  | L2 |  |  | FS |  | - |  |
|  | 190 |  | 8 |  |  | 5550.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=4) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 4 (Intervention status) |  |  | L2 |  |  | FS |  | - |  |
|  | 191 |  | 9 |  |  | 5550.00 |  | L2 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 192 |  | 10 |  |  | 5550.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 193 |  | 11 |  |  | 5550.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is displayed. |  | O |  | DMI |  | Start DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 194 |  | 12 |  |  | 5550.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 195 |  | 13 |  |  | 5600.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | V_TRAIN < V_TSR(A) Status requested: Normal status |  |  | L2 |  |  | FS |  | - |  |
|  | 196 |  | 14 |  |  | 5600.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= A km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) |  |  | L2 |  |  | FS |  | - |  |
|  | 197 |  | 15 |  |  | 5600.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 198 |  | 16 |  |  | 5600.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 199 |  | 17 |  |  | 5600.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake is removed. |  | O |  | DMI |  | Stop DMI Object: ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 200 |  | 18 |  |  | 5600.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 201 |  | 19 |  |  | 6040.00 |  | L2 |  | FS |  |  | Train is moving |  | I |  | INT |  | Train location: Full train length has left TSR (A) area. |  |  | L2 |  |  | FS |  | - |  |
|  | 202 |  | 20 |  |  | 6040.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | If in FS, OS or SR Permitted Speed is the value of V_TSR(B) CSM - Normal Status- V_TSR(B) If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Overspeed Status. |  |  | L2 |  |  | FS |  | - |  |
|  | 203 |  | 21 |  |  | 6040.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM = B km/h; M_SDMTYPE = 0; M_SDMSUPSTAT=0/2) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) Only if in FS/OS/SR: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal status) Check that V_PERM is updated with V_TSR(B) Only if in LS mode: M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed status) |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 8: The driver closes the desk. The on-board equipment immediately switches to SB mode and the end of mission procedure is triggered (there is a radio link with RBC). Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 205 |  | 1 |  |  | 6500.00 |  | L2 |  | FS |  |  | CAB not active |  | I |  | TIU |  | The driver closed the desk. The on-board equipment switches immediately to SB mode. _____ [delay=2000] |  |  | L2 |  |  | FS |  | - |  |
|  | 206 |  | 2 |  |  | 6500.00 |  | L2 |  | FS |  |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 207 |  | 3 |  |  | 6500.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5050300: 5.5.3: End of Mission Procedure Test Case 3: Inform track-side about End of Mission procedure. A communication session with RBC exists. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 209 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 or packet 1 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 210 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 212 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | SB |  | - |  |
|  | 213 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 214 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | SB |  | - |  |
|  | 215 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 216 |  | 5 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | SB |  | - |  |
|  | 217 |  | 6 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | SB |  | - |  |
|  | 218 |  | 7 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 9: In case the train has changed its orientation any Temporary Speed Restriction shall be deleted (operational requirement: will be executed due to the mode change). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 219 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | Cab B active |  | I |  | TIU |  | The driver opens the desk (cab B). Train orientation is change. _____ <event><action target="cabBButton" toggle="True"/>/></event> [delay=3000] |  |  | L2 |  |  | SB |  | - |  |
|  | 220 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | CAB STATUS (NID_MESSAGE_JRU=38) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 223 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L2 |  |  | SB |  | - |  |
|  | 224 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L2 |  |  | SB |  | - |  |
|  | 225 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 22: On-board Equipment reports its valid position to RBC at start up |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 227 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 228 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 229 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 230 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=155) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 231 |  | 5 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SB |  | - |  |
|  | 232 |  | 6 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=32) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 233 |  | 7 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 234 |  | 8 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=159) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 235 |  | 9 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157, Q_STATUS=1) containing packet 0 is transmitted |  | O |  | RTM |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  |  | SB |  | - |  |
|  | 236 |  | 10 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=157; Q_STATUS=1; NID_PACKET=0) is recorded |  | O |  | JRU |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 237 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L2 |  |  | SB |  | - |  |
|  | 238 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 239 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 240 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 242 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L2 |  |  | SB |  | - |  |
|  | 243 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L2 |  |  | SB |  | - |  |
|  | 244 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L2 |  |  | SB |  | - |  |
|  | 245 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 247 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L2 |  |  | SB |  | - |  |
|  | 248 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 249 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 250 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 251 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 252 |  | 5 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 253 |  | 6 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 254 |  | 7 |  |  | 6500.00 |  | L2 |  | SB |  |  | The DMI allows the driver to select ‘START’. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 19: Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3: The driver selects 'Start'. The on-board equipment requests the RBC for a Movement Authority and a Movement Authority for FS mode is received from the RBC together with a Train running number. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 255 |  | 1 |  |  | 6500.00 |  | L2 |  | SB |  |  | T = t’ The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 256 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 257 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted |  | O |  | RTM |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  |  | SB |  | - |  |
|  | 258 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded |  | O |  | JRU |  | Packet 1 must be used if position report is based on 2 BG |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 7: The onboard equipment receives a L2/L3 MA with shifted location reference that is transmitted from RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 268 |  | 2 |  |  | 6500.00 |  | L2 |  | SB |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 269 |  | 3 |  |  | 6500.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "MA with Shifted Location Reference" (NID_MESSAGE=33) containing packet 15, (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | RTM |  | For FS mode, Packet 80: Mode Profile is not included. For OS mode, Packet 80: Mode Profile is included. Other packets can be included. New track description (or stored one if not sent) covers the movement authority distance |  |  | L2 |  |  | SB |  | - |  |
|  | 270 |  | 4 |  |  | 6500.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 271 |  | 5 |  |  | 6500.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The information is updated |  |  | L2 |  |  | FS |  | - |  |
|  | 272 |  | 6 |  |  | 6500.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3110500: 3.11.5: Temporary Speed Restrictions Test Case 9: In case the train has changed its orientation any Temporary Speed Restriction shall be deleted (operational requirement: will be executed due to the mode change). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 273 |  | 6 |  |  | 5900.00 |  | L2 |  | FS |  |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | The train moves again inside the TSR area. |  |  | L2 |  |  | FS |  | - |  |
|  | 276 |  | 9 |  |  | 5900.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Check that the Temporary Speed Restriction is not taken into account for the train speed supervision. If in FS, OS or SR Permitted Speed is NOT V_TSR CSM - Normal Status If in LS mode train speed will be a bit over the V_TSR(B). The DMI aspect is Normal Status. |  |  | L2 |  |  | FS |  | - |  |
|  | 277 |  | 10 |  |  | 5900.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; V_PERM= < another than V_TSR>) is recorded |  | O |  | JRU |  | The speed value is a permitted speed equal to the maximum train speed or any lower static speed restriction (not the TSR). |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 278 |  | 1 |  |  | 5000.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 279 |  | 2 |  |  | 5000.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 01 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 01 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 51 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 50.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| M_VERSION | 7 | 0100000 b |  |  |

| - Step 53 : Message 159 - Session Established (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 56 : Message 129 - Validated Train Data (RBC n? 1) - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 2 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_CTRACTION | 10 | 2 d |  |  |
| NID_CTRACTION | 10 | 3 d |  |  |

| - Step 58 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 50.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 60 : Message 3 - Movement Authority (RBC n? 1) - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 8000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 67 : Message 24 - General Message (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 13 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 64 d | Inhibition of revocable TSRs from balises in L2/3 |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 23 d |  |  |

| - Step 69 : Balise Group BG 2 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 01 d |  |  |
| D_TSR | 15 | 100 d |  |  |
| L_TSR | 15 | 400 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 10 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 69 : Balise Group BG 2 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 77 : Message 24 - General Message (RBC n? 1) - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 79 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 81 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 1100.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |

| - Step 84 : Balise Group BG 3 d 1/2 - Distance 1400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 1 d |  |  |
| D_TSR | 15 | 0 d |  |  |
| L_TSR | 15 | 600 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 10 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 84 : Balise Group BG 3 d 2/2 - Distance 1405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 107 : Message 38 - Initiation of a Communication Session (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 38 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |

| - Step 109 : Message 159 - Session Established (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 111 : Message 136 - Train Position Report (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 600 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 20 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 113 : Message 129 - Validated Train Data (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 600 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 10 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_CTRACTION | 10 | 2 d |  |  |
| NID_CTRACTION | 10 | 3 d |  |  |

| - Step 115 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 2000.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 117 : Message 24 - General Message (RBC n? 1) - Distance 2300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 19 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 127 d |  |  |
| D_TSR | 15 | 800 d |  |  |
| L_TSR | 15 | 1800 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 15 d |  |  |

| - Step 139 : Message 24 - General Message (RBC n? 1) - Distance 3200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 19 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 127 d |  |  |
| D_TSR | 15 | 1900 d |  |  |
| L_TSR | 15 | 2000 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 20 d |  |  |

| - Step 158 : Message 24 - General Message (RBC n? 1) - Distance 4100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 66 d | Temporary speed restriction revocation |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 31 d |  |  |
| NID_TSR | 8 | 127 d |  |  |

| - Step 165 : Balise Group BG 04 d 1/2 - Distance 4200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 255 d |  |  |
| D_TSR | 15 | 200 d |  |  |
| L_TSR | 15 | 1600 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 15 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 165 : Balise Group BG 04 d 2/2 - Distance 4205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 183 : Message 24 - General Message (RBC n? 1) - Distance 4850.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 19 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 255 d |  |  |
| D_TSR | 15 | 800 d |  |  |
| L_TSR | 15 | 1500 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 20 d |  |  |

| - Step 209 : Message 150 - End of Mission (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 150 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 0 d |  |  |
| D_LRBG | 15 | 2200 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 212 : Message 24 - General Message (RBC n? 1) - Distance 6500.00 m Delay : 10.00 s Conditional Message Id : 150 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 214 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 216 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 6500.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |

| - Step 229 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 231 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 6500.00 m Delay : 5.00 s Conditional Message Id : 159 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| M_VERSION | 7 | 0100000 b |  |  |

| - Step 233 : Message 159 - Session Established (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 235 : Message 157 - SoM Position Report (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 01 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 2300 d |  |  |
| Q_DIRLRBG | 2 | 00 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 00 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 250 : Message 129 - Validated Train Data (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 2300 d |  |  |
| Q_DIRLRBG | 2 | 00 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 00 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_CTRACTION | 10 | 2 d |  |  |
| NID_CTRACTION | 10 | 3 d |  |  |

| - Step 252 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 6500.00 m Delay : 20.00 s Conditional Message Id : 129 Back Delay : 20.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 257 : Message 132 - MA Request (RBC n? 1) - Distance 6500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 0 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 2300 d |  |  |
| Q_DIRLRBG | 2 | 00 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 00 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 269 : Message 33 - MA with Shifted Location Reference (RBC n? 1) - Distance 6500.00 m Delay : 10.00 s Conditional Message Id : 132 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 33 d |  |  |
| L_MESSAGE | 10 | 34 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 2400 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |