| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3050300_12_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3050300_12_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 18:03 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | DLR |

# Sequence description

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Row |  | Dist. (m) |  | Previous |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  | Next |  |  |  | Test Result |  |  |
|  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 |  | 1 |  | 0.00 |  | N/A |  | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  | N/A |  | NP |  | - |  |
| 2 |  | 2 |  | 0.00 |  | N/A |  | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 3 |  | 1 |  | 0.00 |  | N/A |  | SB |  | Driver opens desk |  | I |  | TIU |  | - |  | N/A |  | SB |  | - |  |
| 4 |  | 2 |  | 0.00 |  | N/A |  | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  | N/A |  | SB |  | - |  |
| 5 |  | 3 |  | 0.00 |  | N/A |  | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  | 1 |  | 0.00 |  | N/A |  | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  | SB |  | - |  |
| 7 |  | 2 |  | 0.00 |  | N/A |  | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  | N/A |  | SB |  | - |  |
| 8 |  | 3 |  | 0.00 |  | N/A |  | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  | N/A |  | SB |  | - |  |
| 9 |  | 4 |  | 0.00 |  | N/A |  | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  | 1 |  | 0.00 |  | N/A |  | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Level 2 [19.-1.1] |  | L2 |  | SB |  | - |  |
| 11 |  | 2 |  | 0.00 |  | L2 |  | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 12 |  | 3 |  | 0.00 |  | L2 |  | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 38: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to enter the RBC-ID/phone number if its status is 'unknown'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 |  | 1 |  | 0.00 |  | L2 |  | SB |  | The on-board equipment displays the RBC contact window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 14 |  | 2 |  | 0.00 |  | L2 |  | SB |  | The driver presses the ‘Enter RBC data’ button |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 16 |  | 4 |  | 0.00 |  | L2 |  | SB |  | The driver enters valid RBC ID/Phone number. |  | I |  | DMI |  | - _____ RBC_ID = 5768683; NID_RADIO = 003265342101FFFF |  | L2 |  | SB |  | - |  |
| 17 |  | 5 |  | 0.00 |  | L2 |  | SB |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 18 |  | 6 |  | 0.00 |  | L2 |  | SB |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). |  | I |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 19 |  | 7 |  | 0.00 |  | L2 |  | SB |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  | L2 |  | SB |  | - |  |
| Feature 3050300: 3.5.3: Establishing a communication session Test Case 24: Within the Start of Mission procedure, system version are not compatible and session is not established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 |  | 1 |  | 0.00 |  | L2 |  | SB |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or a defined number of times. The onboard tries to set up a connection with the phone number entered in the „preceded by' test case. |  | L2 |  | SB |  | - |  |
| 21 |  | 2 |  | 0.00 |  | L2 |  | SB |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  | L2 |  | SB |  | - |  |
| 22 |  | 3 |  | 0.00 |  | L2 |  | SB |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  | L2 |  | SB |  | - |  |
| 23 |  | 4 |  | 0.00 |  | L2 |  | SB |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 24 |  | 5 |  | 0.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 25 |  | 6 |  | 0.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 26 |  | 7 |  | 0.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are NOT compatible. Communication session established for on-board. |  | L2 |  | SB |  | - |  |
| 27 |  | 8 |  | 0.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 28 |  | 9 |  | 0.00 |  | L2 |  | SB |  | The system status message "Trackside not compatible" is displayed |  | O |  | DMI |  | Communication error is ETCS system version incompatibility |  | L2 |  | SB |  | - |  |
| 29 |  | 10 |  | 0.00 |  | L2 |  | SB |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit 09>) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| 30 |  | 11 |  | 0.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "No compatible version supported" (NID_MESSAGE=154) is transmitted |  | O |  | RTM |  | - |  | L2 |  | SB |  | - |  |
| 31 |  | 12 |  | 0.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  | L2 |  | SB |  | - |  |
| Feature 3050500: 3.5.5: Terminating a communication session Test Case 7: The on-board equipment terminates a communication session due to error. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 33 |  | 1 |  | 0.00 |  | L2 |  | SB |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | The onboard equipment detects an error condition that requires the termination of the communication session. It shall be checked that no more messages are sent If L1 message is sent to the RIU. If L2 or L3 message is sent to the RBC |  | L2 |  | SB |  | - |  |
| 34 |  | 2 |  | 0.00 |  | L2 |  | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  | L2 |  | SB |  | - |  |
| 36 |  | 4 |  | 0.00 |  | L2 |  | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is received from the RBC and the onboard equipment considers the communication session terminated. If L1 message is sent from the RIU. If L2 or L3 message is sent from the RBC |  | L2 |  | SB |  | - |  |
| 37 |  | 5 |  | 0.00 |  | L2 |  | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  | L2 |  | SB |  | - |  |
| 39 |  | 7 |  | 0.00 |  | L2 |  | SB |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The onboard tries to release the safe connection. |  | L2 |  | SB |  | - |  |
| Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 40 |  | 1 |  | 0.00 |  | L2 |  | SB |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  | L2 |  | NP |  | - |  |
| 41 |  | 2 |  | 0.00 |  | L2 |  | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  | L2 |  | NP |  | - |  |

| Step 10: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=2 | 0 s |

| - Step 24 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 26 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 011 0000 b |  |  |

| - Step 30 : Message 154 - No compatible version supported (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 154 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 33 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 36 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |