| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3050700_01_RIU_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3050700_01_RIU_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 18/04/17 18:28 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 18.04.2017 | All |  | RINA |

# Sequence description

Safe Radio Connection Indication

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The driver selects Level 1. [59.-1.1] |  |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 43 |  | 5 |  |  | 100.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 4: The ERTMS/ETCS shall execute the transition between 'No connection' status and 'Connection Lost / Set-up failed' status if the 'connection status' timer expires. Condition Id [2]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 200.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=1) is received. |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 45 |  | 2 |  |  | 200.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 46 |  | 3 |  |  | 200.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | Connection status timer starts as soon as the first request is sent. |  |  | L1 |  |  | FS |  | - |  |
|  | 47 |  | 4 |  |  | 350.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The safe connection is not confirmed. Repetition of the request after. |  |  | L1 |  |  | FS |  | - |  |
|  | 48 |  | 5 |  |  | 500.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The safe connection is not confirmed. Repetition of the request after. |  |  | L1 |  |  | FS |  | - |  |
|  | 49 |  | 6 |  |  | 650.00 |  | L1 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. |  | O |  | DMI |  | After 45s from the first request to set up the safe connection. DMI object: Symbol ST04 |  |  | L1 |  |  | FS |  | - |  |
|  | 50 |  | 7 |  |  | 650.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 7: The ERTMS/ETCS shall execute the transition between 'Connection Lost / Set-up failed' status and 'No Connection' status if no Start of Mission procedure is going and the requests to set-up a safe radio connection are stopped with the relevant RIU for reason other than the successful set-up. Condition Id [3]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  | 1 |  |  | 800.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=1) is received. |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 52 |  | 2 |  |  | 800.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 53 |  | 3 |  |  | 800.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The on-board equipment sends 3 times, until the 'connection status' timer expired, a request to establish a new safe radio connection with the RIU. Connection status timer starts as soon as the first request is sent. |  |  | L1 |  |  | FS |  | - |  |
|  | 54 |  | 4 |  |  | 1250.00 |  | L1 |  | FS |  |  | The Status symbol 'Safe radio connection - Connection Lost/Set-Up failed' is DISPLAYED. |  | O |  | DMI |  | After 45s from the first request to set up the safe connection. Start DMI Object: ST04 |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 5 |  |  | 1250.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 6 |  |  | 1400.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=0) is received. |  | I |  | BTM |  | The request to set-up a safe connection is stopped for reason other than the successful set-up. |  |  | L1 |  |  | FS |  | - |  |
|  | 57 |  | 7 |  |  | 1400.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 58 |  | 8 |  |  | 1400.00 |  | L1 |  | FS |  |  | The Status symbol 'Safe radio connection - Connection Lost/Set-Up failed' is REMOVED. |  | O |  | DMI |  | Stop DMI Object ST04 |  |  | L1 |  |  | FS |  | - |  |
|  | 59 |  | 9 |  |  | 1400.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 60 |  | 10 |  |  | 1400.00 |  | L1 |  | FS |  |  | NO status symbol 'No Connection ' is displayed. |  | O |  | DMI |  | DMI object: Absence of Symbol |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 16: The ERTMS/ETCS shall execute the transition between 'No connection' status and 'Connection Up' status if the safe radio connection is set-up. Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  | 1 |  |  | 1600.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=1) is received. |  | I |  | BTM |  | Packet 'Radio infill area information' |  |  | L1 |  |  | FS |  | - |  |
|  | 62 |  | 2 |  |  | 1600.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 63 |  | 3 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 64 |  | 4 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-CONNECT. Confirm is received |  | I |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 65 |  | 5 |  |  | 1600.00 |  | L1 |  | FS |  |  | The Status symbol 'Safe radio connection - Connection Up' is displayed. |  | O |  | DMI |  | DMI object: ST03 |  |  | L1 |  |  | FS |  | - |  |
|  | 66 |  | 6 |  |  | 1600.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3090300: 3.9.3: Infill by radio Test Case 1: Starting event is received from the balise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  | 4 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 71 |  | 5 |  |  | 1600.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 72 |  | 6 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | The trackside ETCS system version is accepted since it is compatible with at least one of the ETCS versions stored on-board. At this point, the on- board equipment considers the communication session with the RBC as established. |  |  | L1 |  |  | FS |  | - |  |
|  | 73 |  | 7 |  |  | 1600.00 |  | L1 |  | FS |  |  | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 74 |  | 8 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 75 |  | 9 |  |  | 1600.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | Communication session established for trackside. Only for information ' not relevant for on-board equipment. |  |  | L1 |  |  | FS |  | - |  |
|  | 76 |  | 10 |  |  | 1600.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted |  | O |  | RTM |  | The on-board unit shall inform the radio infill unit using message 153 as soon as the location received during preceding TC FT4080429.3 is passed |  |  | L1 |  |  | FS |  | - |  |
|  | 77 |  | 11 |  |  | 1600.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 5: The ERTMS/ETCS shall execute the transition between 'Connection Up' status and 'Connection Lost / Set-up failed' status if the 'connection status' timer expires. Condition Id [2]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 2000.00 |  | L1 |  | FS |  |  | SA-DISCONNECT. Indication is received |  | I |  | RTM |  | The connection is lost. |  |  | L1 |  |  | FS |  | - |  |
|  | 79 |  | 2 |  |  | 2000.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | Connection status timer starts as soon as the first request is sent. |  |  | L1 |  |  | FS |  | - |  |
|  | 80 |  | 3 |  |  | 2150.00 |  | L1 |  | FS |  |  | SA-DISCONNECT. Indication is received |  | I |  | RTM |  | The connection is lost. |  |  | L1 |  |  | FS |  | - |  |
|  | 81 |  | 4 |  |  | 2150.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The safe connection is not confirmed. Repetition of the request after. |  |  | L1 |  |  | FS |  | - |  |
|  | 82 |  | 5 |  |  | 2300.00 |  | L1 |  | FS |  |  | SA-DISCONNECT. Indication is received |  | I |  | RTM |  | The connection is lost. |  |  | L1 |  |  | FS |  | - |  |
|  | 83 |  | 6 |  |  | 2300.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The safe connection is not confirmed. Repetition of the request after. |  |  | L1 |  |  | FS |  | - |  |
|  | 84 |  | 7 |  |  | 2300.00 |  | L1 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is removed. |  | O |  | DMI |  | DMI object: ST03 |  |  | L1 |  |  | FS |  | - |  |
|  | 85 |  | 8 |  |  | 2300.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 86 |  | 9 |  |  | 2450.00 |  | L1 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. |  | O |  | DMI |  | After 45s from the first request to set up the safe connection DMI object: ST04 |  |  | L1 |  |  | FS |  | - |  |
|  | 87 |  | 10 |  |  | 2450.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 56: Driver selects 'override', train speed is under or equal to the speed limit for triggering the 'override' function and the mode is FS, LS and OS. Transition Condition Id [37]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  | 1 |  |  | 2800.00 |  | L1 |  | FS |  |  | The driver selects 'Override EoA' |  | I |  | DMI |  | - _____ Train is at standstill |  |  | L1 |  |  | FS |  | - |  |
|  | 89 |  | 2 |  |  | 2800.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 3 |  |  | 2800.00 |  | L1 |  | FS |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 91 |  | 4 |  |  | 2800.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 92 |  | 5 |  |  | 2800.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 93 |  | 6 |  |  | 2800.00 |  | L1 |  | SR |  |  | The Mode symbol 'Override EOA is active' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 94 |  | 7 |  |  | 2800.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 95 |  | 8 |  |  | 2800.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 96 |  | 9 |  |  | 2800.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The permitted speed V_NVSUPOVTRP is indicated to the driver. |  |  | L1 |  |  | SR |  | - |  |
|  | 97 |  | 10 |  |  | 2800.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 17: The ERTMS/ETCS shall execute the transition between 'Connection Lost / Set-up failed' status and 'Connection Up' status if the safe radio connection is set-up. Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 98 |  | 1 |  |  | 3000.00 |  | L1 |  | SR |  |  | Balise Group message containing packet 133 (Q_RIU=1) is received. |  | I |  | BTM |  | Packet 'Radio infill area information' |  |  | L1 |  |  | SR |  | - |  |
|  | 99 |  | 2 |  |  | 3000.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 100 |  | 3 |  |  | 3000.00 |  | L1 |  | SR |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 101 |  | 4 |  |  | 3000.00 |  | L1 |  | SR |  |  | SA-CONNECT. Confirm is received |  | I |  | RTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 102 |  | 5 |  |  | 3000.00 |  | L1 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is removed. |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 103 |  | 6 |  |  | 3000.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 104 |  | 7 |  |  | 3000.00 |  | L1 |  | SR |  |  | The Status symbol 'Safe radio connection - Connection Up' is displayed. |  | O |  | DMI |  | DMI object: ST03 |  |  | L1 |  |  | SR |  | - |  |
|  | 105 |  | 8 |  |  | 3000.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 |  | 1 |  |  | 3500.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  |  | SR |  | - |  |
|  | 107 |  | 2 |  |  | 3500.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 108 |  | 3 |  |  | 3500.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 4 |  |  | 3500.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 5 |  |  | 3500.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 10: The ERTMS/ETCS shall execute the transition between 'Connection Up' status and 'No Connection' status if the RIU safe radio connection is released. Condition Id [5]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  |  | 3700.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=0; D_INFILL=0) is received. |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 112 |  | 2 |  |  | 3700.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 113 |  | 3 |  |  | 3700.00 |  | L1 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted. |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 114 |  | 4 |  |  | 3700.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 115 |  | 5 |  |  | 3700.00 |  | L1 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 116 |  | 6 |  |  | 3700.00 |  | L1 |  | FS |  |  | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 117 |  | 7 |  |  | 3700.00 |  | L1 |  | FS |  |  | SA-DISCONNECT. Request is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 118 |  | 8 |  |  | 3700.00 |  | L1 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is REMOVED. |  | O |  | DMI |  | Stop DMI Object ST03 |  |  | L1 |  |  | FS |  | - |  |
|  | 119 |  | 9 |  |  | 3700.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 120 |  | 10 |  |  | 3700.00 |  | L1 |  | FS |  |  | NO status symbol "No Connection " is displayed. |  | O |  | DMI |  | DMI object: Absence of Symbol |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080429: 4.8.4: Accepted Information depending on the modes - Radio Infill Area information Test Case 3: Transmission of radio infill area information for establishing a communication session with RIU via BG |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  | 1 |  |  | 4500.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1) containing packet 133 (Q_RIU=1) is received |  | I |  | BTM |  | Q_RIU=1 means "Establish communication session" |  |  | L1 |  |  | FS |  | - |  |
|  | 123 |  | 2 |  |  | 4500.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3090300: 3.9.3: Infill by radio Test Case 1: Starting event is received from the balise. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 |  | 2 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 126 |  | 3 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Confirmation of the vital connection from RTM |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 4 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 128 |  | 5 |  |  | 4500.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 129 |  | 6 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | The trackside ETCS system version is accepted since it is compatible with at least one of the ETCS versions stored on-board. At this point, the on- board equipment considers the communication session with the RBC as established. |  |  | L1 |  |  | FS |  | - |  |
|  | 130 |  | 7 |  |  | 4500.00 |  | L1 |  | FS |  |  | MESSAGE FROM RADIO INFILL UNIT (NID_MESSAGE_JRU=8) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 131 |  | 8 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) is transmitted |  | O |  | RTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 132 |  | 9 |  |  | 4500.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | Communication session established for trackside. Only for information ' not relevant for on-board equipment. |  |  | L1 |  |  | FS |  | - |  |
|  | 133 |  | 10 |  |  | 4500.00 |  | L1 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Radio infill request" (NID_MESSAGE=153) containing packet 0 is transmitted |  | O |  | RTM |  | The on-board unit shall inform the radio infill unit using message 153 as soon as the location received during preceding TC FT4080429.3 is passed |  |  | L1 |  |  | FS |  | - |  |
|  | 134 |  | 11 |  |  | 4500.00 |  | L1 |  | FS |  |  | MESSAGE TO RADIO INFILL UNIT (NID_MESSAGE_JRU=5) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050700: 3.5.7: Safe Radio Connection Indication Test Case 12: The ERTMS/ETCS shall execute the transition between 'Connection Up' status and 'No Connection' status if the safe radio connection is lost and the requests to set-up a safe radio connection are stopped with the relevant RIU for reason other than the successful set-up. Condition Id [6]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 5000.00 |  | L1 |  | FS |  |  | SA-DISCONNECT. Indication is received |  | I |  | RTM |  | The connection is lost |  |  | L1 |  |  | FS |  | - |  |
|  | 136 |  | 2 |  |  | 5000.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The request to connect is sent |  |  | L1 |  |  | FS |  | - |  |
|  | 137 |  | 3 |  |  | 5150.00 |  | L1 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | The safe connection is not confirmed. Repetition of the request. |  |  | L1 |  |  | FS |  | - |  |
|  | 138 |  | 4 |  |  | 5200.00 |  | L1 |  | FS |  |  | Balise Group message containing packet 133 (Q_RIU=0) is received. |  | I |  | BTM |  | Before the connection status timer expires. |  |  | L1 |  |  | FS |  | - |  |
|  | 139 |  | 5 |  |  | 5200.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 140 |  | 6 |  |  | 5200.00 |  | L1 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is REMOVED. |  | O |  | DMI |  | Stop DMI Object : ST03 |  |  | L1 |  |  | FS |  | - |  |
|  | 141 |  | 7 |  |  | 5200.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 142 |  | 8 |  |  | 5200.00 |  | L1 |  | FS |  |  | NO status symbol "No Connection " is displayed. |  | O |  | DMI |  | DMI object: Absence of Symbol |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 143 |  | 1 |  |  | 5600.00 |  | L1 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 144 |  | 2 |  |  | 5600.00 |  | N/A |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 4500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 1/2 - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 2 d 2/2 - Distance 205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 3 d 1/2 - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 3 d 2/2 - Distance 805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 4 d 1/2 - Distance 1400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 4 d 2/2 - Distance 1405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 61 : Balise Group BG 5 d 1/2 - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 61 : Balise Group BG 5 d 2/2 - Distance 1605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 70 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 72 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 1600.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |

| - Step 74 : Message 159 - Session Established (RBC n? 1) - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 76 : Message 153 - Radio infill Request (RBC n? 1) - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 153 d |  |  |
| L_MESSAGE | 10 | 27 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_INFILL | 1 | 0 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 010 b |  |  |

| - Step 98 : Balise Group BG 6 d 1/2 - Distance 3000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 98 : Balise Group BG 6 d 2/2 - Distance 3005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 106 : Balise Group BG 7 d 1/2 - Distance 3500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 106 : Balise Group BG 7 d 2/2 - Distance 3505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 111 : Balise Group BG 8 d 1/2 - Distance 3700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 111 : Balise Group BG 8 d 2/2 - Distance 3705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 113 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 3700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 115 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 3700.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048584 d |  |  |

| - Step 122 : Balise Group BG 9 d 1/2 - Distance 4500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 122 : Balise Group BG 9 d 2/2 - Distance 4505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 127 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 4500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 129 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 4500.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |

| - Step 131 : Message 159 - Session Established (RBC n? 1) - Distance 4500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 133 : Message 153 - Radio infill Request (RBC n? 1) - Distance 4500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 153 d |  |  |
| L_MESSAGE | 10 | 27 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_INFILL | 1 | 0 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048585 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 010 b |  |  |

| - Step 138 : Balise Group BG 10 d 1/2 - Distance 5200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 133 d | Radio infill area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 153 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_RIU | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RIU | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| D_INFILL | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 138 : Balise Group BG 10 d 2/2 - Distance 5205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |