| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990400_47_v310.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990400_47_v310.mdb |
| ISSUE : | 3.1.0 |
| DATE : | 19/04/17 14:27 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.1.0 19.04.2017 | All |  | DLR |

# Sequence description

Accepted information depending on the level and transmission media Tested here "LSSMA display toggle on/off order"

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1. [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | ADDITIONAL DATA (NID_MESSAGE_JRU=24; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 43 |  | 5 |  |  | 100.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 1: Limited Supervision is requested for a further location when running in FS. An acknowledgement request for LS mode is displayed to the driver. The driver acknowledges LS mode. Mode is changed to Limited Supervision |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 200.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; Q_MAMODE=1) is received |  | I |  | BTM |  | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) D_MAMODE=<FINITE VALUE (<>0)>; L_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>; Q_MAMODE=1 <Begin of LS area to be considered as the SvL> |  |  | L1 |  |  | FS |  | - |  |
|  | 45 |  | 2 |  |  | 200.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | The steps 1,2 or 3,4 will be chosen for testing |  |  | L1 |  |  | FS |  | - |  |
|  | 51 |  | 8 |  |  | 200.00 |  | L1 |  | FS |  |  | The target distance is updated |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 52 |  | 9 |  |  | 200.00 |  | L1 |  | FS |  |  | The target distance is updated is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 53 |  | 10 |  |  | 250.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | Distance between estimated front end and beginning of Limited Supervision Area < L_ACKMAMODE (stored Packet 80) AND The speed is lower than the Limited Supervision mode permitted speed. V < V_NVLIMSUPERV |  |  | L1 |  |  | FS |  | - |  |
|  | 54 |  | 11 |  |  | 300.00 |  | L1 |  | FS |  |  | The driver acknowledges "Limited Supervision" |  | I |  | DMI |  | Before reaching the LS area _____ The driver acknowledges 'Limited Supervision' [43.-1.1] |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 12 |  |  | 300.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1101) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 13 |  |  | 300.00 |  | L1 |  | FS |  |  | The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 57 |  | 14 |  |  | 300.00 |  | L1 |  | LS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4080300: 4.8.3: Accepted information depending on the level and transmission media Test Case 36: Generic LS function marker and LSSMA display toggle on order shall be accepted by on-board equipment according to tables 4.8.3 and 4.8.4.2. Tested Action: A from BTM for LSSMA display toggle on information and A from BTM for Generic LS function marker information. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 1 |  |  | 800.00 |  | L1 |  | LS |  |  | One Balise Group message (N_TOTAL=1) containing packet 181 is received. |  | I |  | BTM |  | Information from packet 181 is Accepted. |  |  | L1 |  |  | LS |  | - |  |
|  | 64 |  | 2 |  |  | 800.00 |  | L1 |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 65 |  | 3 |  |  | 900.00 |  | L1 |  | LS |  |  | One Balise Group message (N_TOTAL=1) containing packet 180 (Q_LSSMA= 1) is received. |  | I |  | BTM |  | Information from packet 180 is Accepted. Toggle on order _____ T_LSSMA = 0 |  |  | L1 |  |  | LS |  | - |  |
|  | 66 |  | 4 |  |  | 900.00 |  | L1 |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 67 |  | 5 |  |  | 900.00 |  | L1 |  | LS |  |  | The Limited Supervision symbol "Limited Supervision frame" is displayed. |  | O |  | DMI |  | DMI Object: Symbol LS01 (The on-board pass over the first balise of the BG containing packet 180 since T_LSSMA) |  |  | L1 |  |  | LS |  | - |  |
|  | 68 |  | 6 |  |  | 900.00 |  | L1 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit87=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 69 |  | 7 |  |  | 900.00 |  | L1 |  | LS |  |  | The LSSMA number is displayed. |  | O |  | DMI |  | The EVC displays the LSSMA speed value |  |  | L1 |  |  | LS |  | - |  |
|  | 70 |  | 8 |  |  | 900.00 |  | L1 |  | LS |  |  | LOWEST SUPERVISED SPEED WITHIN THE MOVEMENT AUTHORITY (NID_MESSAGE_JRU=44; <V_LSSMA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 3160200: 3.16.2: Balises Test Case 2: Plausibility check for variables transmitted by balises. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 1 |  |  | 1100.00 |  | L1 |  | LS |  |  | One Balise Group message (N_TOTAL=1; M_DUP=11;Q_LINK=0) is received |  | I |  | BTM |  | The train finds a BG marked as unlinked A variable of the BG message has an error: M_DUP=11 is value is not in the specified value range. _____ Includes Packet 72 |  |  | L1 |  |  | LS |  | - |  |
|  | 72 |  | 2 |  |  | 1100.00 |  | L1 |  | LS |  |  | Balise group message is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 73 |  | 3 |  |  | 1100.00 |  | L1 |  | LS |  |  | Error indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 74 |  | 4 |  |  | 1100.00 |  | L1 |  | LS |  |  | Error indication is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 76 |  | 6 |  |  | 1100.00 |  | L1 |  | LS |  |  | The balise group message is rejected |  | I |  | INT |  | The BG passed should contain a packet that makes possible to test this step |  |  | L1 |  |  | LS |  | - |  |
|  | 77 |  | 7 |  |  | 1100.00 |  | L1 |  | LS |  |  | SB is APPLIED. |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 78 |  | 8 |  |  | 1100.00 |  | L1 |  | LS |  |  | SB Indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 79 |  | 9 |  |  | 1100.00 |  | L1 |  | LS |  |  | SB intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 80 |  | 10 |  |  | 1200.00 |  | L1 |  | LS |  |  | Train reaches standstill. |  | I |  | INT |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 81 |  | 11 |  |  | 1200.00 |  | L1 |  | LS |  |  | SB is RELEASED |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 82 |  | 12 |  |  | 1200.00 |  | L1 |  | LS |  |  | SB indication is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 83 |  | 13 |  |  | 1200.00 |  | L1 |  | LS |  |  | SB revocation is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 9990400: A3.4: Handling of Stored Information in specific Situations Test Case 64: A.3.4.1.2 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 85 |  | 2 |  |  | 1200.00 |  | L1 |  | LS |  |  | LSSMA symbol is displayed |  | O |  | DMI |  | Object: Symbol LS01 There is no change in the display status as the. |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 56: Driver selects 'override', train speed is under or equal to the speed limit for triggering the 'override' function and the mode is FS, LS and OS. Transition Condition Id [37]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 1200.00 |  | L1 |  | LS |  |  | The driver selects 'Override EoA' |  | I |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 87 |  | 2 |  |  | 1200.00 |  | L1 |  | LS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 88 |  | 3 |  |  | 1200.00 |  | L1 |  | LS |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 89 |  | 4 |  |  | 1200.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 90 |  | 5 |  |  | 1200.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 91 |  | 6 |  |  | 1200.00 |  | L1 |  | SR |  |  | The Mode symbol 'Override EOA is active' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 92 |  | 7 |  |  | 1200.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 93 |  | 8 |  |  | 1200.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 94 |  | 9 |  |  | 1200.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The permitted speed V_NVSUPOVTRP is indicated to the driver. |  |  | L1 |  |  | SR |  | - |  |
|  | 95 |  | 10 |  |  | 1200.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 5: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 96 |  | 1 |  |  | 1235.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | Complete message including Mode Profile is accepted |  |  | L1 |  |  | SR |  | - |  |
|  | 97 |  | 2 |  |  | 1235.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  | 1 |  |  | 1270.00 |  | L1 |  | LS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  |  | NP |  | - |  |
|  | 100 |  | 2 |  |  | 1270.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 01 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 60 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2100 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 350 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 01 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 02 d 1/2 - Distance 200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 60 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 400 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1600 d |  |  |
| L_ACKMAMODE | 15 | 350 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 02 d 2/2 - Distance 205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 1/2 - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 181 d | Generic LS function marker |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 23 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 2/2 - Distance 805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 65 : Balise Group BG 04 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 180 d | LSSMA display toggle order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 32 d |  |  |
| Q_LSSMA | 1 | 1 |  |  |
| T_LSSMA | 8 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 65 : Balise Group BG 04 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 99 d 1/2 - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 0 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 99 d 2/2 - Distance 1105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 11 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 0 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 268 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 0 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 5 |  |  |
| L_TEXTDISPLAY | 15 | 50 d |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 5 |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 22 d |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | k |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | B |  |  |
| X_TEXT | 8 | G |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | w |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | h |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | r |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | r |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 96 : Balise Group BG 98 d 1/2 - Distance 1235.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 98 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 60 d |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Gradient profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 600 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 96 : Balise Group BG 98 d 2/2 - Distance 1240.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 98 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |