| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3130700_10_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3130700_10_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 21:11 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | CEDEX |

# Sequence description

The main objective is to test the TCs 6 and 7 ofFT3130700. The NVs are sent at 100m. A Lambda train is used for testing.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | SB | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] Select Level 1 |  |  | L1 |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 6: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 0.00 |  | L1 | SR |  | Train is moving (V_TRAIN=<below V1>) |  | I |  | INT |  | See the meaning of V1 in the following step. |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 | SR |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | This action will be done once, since after it the displayed information will not disappear unless the driver requests it. Only inSR mode. |  |  | L1 |  | SR | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 | SR |  | One Balise Group message containing packet 65 (D_TSR=0, V_TSR = <V1>, Q_FRONT = 1), packet 3 (V_NVSUPOVTRP = V_NVALLOWOVTRP = <V2>, V_NVSTFF=<V3>, V_NVUNFIT = <V3>) is received. |  | I |  | BTM |  | In SR mode, the length of the given distance to run in SR shall be long enough to prevent the on-board equipment from braking to the end of that given distance during the execution of this TC (i.e., the on-board equipment will always be in a Ceiling Speed Monitoring section). V4 = maximum train speed. The relation between the values of the speed limits is: V1 < V2 < V3 < V4. This location is at distance D0. Inside Packet 3, the value of T_NVOVTRP has to be high enough in order to ensure the Override status ends due to the train having passed the distance D_NVOVTRP. |  |  | L1 |  | SR | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 43 |  | 5 |  |  | 100.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM = <V1>) |  | O |  | DMI |  | The PS is equal to <V1> (the Temporary Speed Restriction). Only in SR mode. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. NOT IN UN MODE _____ V1=25km/h |  |  | L1 |  | SR | - |  |
|  | 44 |  | 6 |  |  | 100.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) _____ V1=25km/h |  |  | L1 |  | SR | - |  |
|  | 45 |  | 7 |  |  | 170.00 |  | L1 | SR |  | Train is moving (V_TRAIN=<V1-10>) |  | I |  | INT |  | The train speed is lightly > V1 |  |  | L1 |  | SR | - |  |
|  | 46 |  | 8 |  |  | 170.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM = <V1>) |  | O |  | DMI |  | The PS is equal to <V1> (the Temporary Speed Restriction). The DMI aspect is coherent with Over speed Status in a Ceiling Speed Monitoring section. _____ dV_warning_min=4km/h, so the train speed is set to 28km/h in the Speed Profile |  |  | L1 |  | SR | - |  |
|  | 47 |  | 9 |  |  | 170.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Over Speed Status) |  |  | L1 |  | SR | - |  |
|  | 48 |  | 10 |  |  | 220.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM = <V1>) |  | O |  | DMI |  | The PS is equal to <V1> (the Temporary Speed Restriction). Only in SR mode. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. NOT IN UN MODE |  |  | L1 |  | SR | - |  |
|  | 49 |  | 11 |  |  | 220.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | SR | - |  |
|  | 50 |  | 12 |  |  | 350.00 |  | L1 | SR |  | The driver selects "Override EoA" |  | I |  | DMI |  | This action is done before reaching the location: D0 + L_TSR. Since V_TRAIN < V_NVALLOWOVTRP, it is possible to select "Override EOA". This location is at D1. |  |  | L1 |  | SR | - |  |
|  | 51 |  | 13 |  |  | 350.00 |  | L1 | SR |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1110) is recorded |  | O |  | JRU |  | M_DRIVERACTIONS=0000 1110 (Override selected) |  |  | L1 |  | SR | - |  |
|  | 52 |  | 14 |  |  | 350.00 |  | L1 | SR |  | The Mode symbol "Override EOA is active" is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 53 |  | 15 |  |  | 350.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM =<V1> ) |  | O |  | DMI |  | The PS is equal to <V1> (the Temporary Speed Restriction), since V_TSR < V_NVSUPOVTRP. The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. |  |  | L1 |  | SR | - |  |
|  | 54 |  | 16 |  |  | 500.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM =<V2> ) |  | O |  | DMI |  | This location is at distance D0 + L_TSR. The PS is equal to <V2> (= V_NVSUPOVTRP). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V2=30km/h |  |  | L1 |  | SR | - |  |
|  | 55 |  | 17 |  |  | 500.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded |  | O |  | JRU |  | V2 = V_NVSUPOVTRP M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | SR | - |  |
|  | 56 |  | 18 |  |  | 600.00 |  | L1 | SR |  | The Mode symbol "Override EOA is active" is removed |  | O |  | DMI |  | This location is at distance D1 + D_NVOVTRP. |  |  | L1 |  | SR | - |  |
|  | 57 |  | 19 |  |  | 600.00 |  | L1 | SR |  | The speed and distance monitoring information is displayed (V_PERM =<V3> ) |  | O |  | DMI |  | The PS is equal to <V3> (=V_NVSTFF if in SR mode, or V_NVUNFIT if in UN mode). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V3=40km/h |  |  | L1 |  | SR | - |  |
|  | 58 |  | 20 |  |  | 600.00 |  | L1 | SR |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded |  | O |  | JRU |  | V3 = V_NVSTFF, if in SR mode. V3 = V_NVUNFIT if in UN mode. M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) _____ V3=40km/h |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 59 |  | 1 |  |  | 650.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | 60 |  | 2 |  |  | 650.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 61 |  | 3 |  |  | 650.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | FS | - |  |
|  | 62 |  | 4 |  |  | 650.00 |  | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 63 |  | 5 |  |  | 650.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4080434: 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information Test Case 2: On-board receives a BG message or a radio message from the RBC including Packet 139 ( Reversing supervision information) via balise group.The Reversing Supervision Information is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 650.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Only if the onboard is in RV mode. D_REVERSE_ini is the value of D_REVERSE, which is being supervised at the beginning of this TC, when this starts in RV mode |  |  | L1 |  | FS | - |  |
|  | 65 |  | 2 |  |  | 650.00 |  | L1 | FS |  | Train is moving (V_TRAIN < permitted) |  | I |  | INT |  | If the current level is L1, choose the following two steps. If the current level is L2/L3, choose the steps 5 and 6. D_REVERSE_ini is the value of D_REVERSE, which is stored on-board at the beginning of this TC, when this does not start in RV mode |  |  | L1 |  | FS | - |  |
|  | 66 |  | 3 |  |  | 750.00 |  | L1 | FS |  | One Balise Group message containing packet 139 (Q_DIR=1), packet 138 is received |  | I |  | BTM |  | The value of D_REVERSE in packet 139 has to be different from the value D_REVERSE_ini. |  |  | L1 |  | FS | - |  |
|  | 67 |  | 4 |  |  | 750.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 70 |  | 7 |  |  | 750.00 |  | L1 | FS |  | Train is moving (V_TRAIN < permitted) |  | I |  | INT |  | The on-board equipment changes to either FS, LS or OS mode (only if the current mode is different from RV mode) |  |  | L1 |  | FS | - |  |
|  | 71 |  | 8 |  |  | 950.00 |  | L1 | FS |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | The train has to be in the reversing area defined in the previously stored packet 138. |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 22: The ETCS on-board equipment switches to RV Mode after the driver has acknowledged reversing. The train is at standstill. Transition Condition Id [59]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 1 |  |  | 950.00 |  | L1 | FS |  | Driver selects Reversing. |  | I |  | TIU |  | (e.g. from a direction controller in reverse position: the current position of the direction controller is backwards) |  |  | L1 |  | FS | - |  |
|  | 74 |  | 2 |  |  | 950.00 |  | L1 | FS |  | DIRECTION CONTROLLER POSITION (NID_MESSAGE_JRU=39; M_DIRECTION_CONTROLLER=01) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 75 |  | 3 |  |  | 950.00 |  | L1 | FS |  | The Mode symbol 'Acknowledge Reversing' is displayed. |  | O |  | DMI |  | DMI Object: Symbol MO15 |  |  | L1 |  | FS | - |  |
|  | 76 |  | 4 |  |  | 950.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit30=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 77 |  | 5 |  |  | 950.00 |  | L1 | FS |  | The driver acknowledges 'Reversing' |  | I |  | DMI |  | - |  |  | L1 |  | FS | - |  |
|  | 78 |  | 6 |  |  | 950.00 |  | L1 | FS |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0101) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 79 |  | 7 |  |  | 950.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=14) is recorded |  | O |  | JRU |  | The on-board equipment switches to RV mode. |  |  | L1 |  | RV | - |  |
|  | 80 |  | 8 |  |  | 950.00 |  | L1 | RV |  | The Mode symbol 'Reversing' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO14 |  |  | L1 |  | RV | - |  |
|  | 81 |  | 9 |  |  | 950.00 |  | L1 | RV |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit29=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | RV | - |  |
|  | Feature 4080434: 4.8.4: Accepted Information depending on the modes - Reversing Supervision Information Test Case 2: On-board receives a BG message or a radio message from the RBC including Packet 139 ( Reversing supervision information) via balise group.The Reversing Supervision Information is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  | 10 |  |  | 950.00 |  | L1 | RV |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance is displayed and it is different to the value D_REVERSE_ini. The target distance matches the value of D_REVERSE that was received in the step 3 or 5. |  |  | L1 |  | RV | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 7: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 83 |  | 1 |  |  | 950.00 |  | L1 | RV |  | Train is moving (V_TRAIN=<below V1>) |  | I |  | INT |  | See the meaning of V1 in the following steps. |  |  | L1 |  | RV | - |  |
|  | 84 |  | 2 |  |  | 950.00 |  | L1 | RV |  | The speed and distance monitoring information to be displayed |  | o |  | DMI |  | - |  |  | L1 |  | RV | - |  |
|  | 85 |  | 3 |  |  | 900.00 |  | L1 | RV |  | One Balise Group message containing packet 139 (V_REVERSE = <V1>) is received. |  | I |  | BTM |  | Only in L1. The length of the given distance to run in RV (D_REVERSE) shall be long enough to prevent the on-board equipment from braking to the end of that given distance during the execution of this TC (i.e., the on-board equipment will always be in a Ceiling Speed Monitoring section). V2 = maximum train speed. The relation between the values of the speed limits is: V1 < V2 . This location is at distance D0. |  |  | L1 |  | RV | - |  |
|  | 86 |  | 4 |  |  | 900.00 |  | L1 | RV |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only in L1. |  |  | L1 |  | RV | - |  |
|  | 89 |  | 7 |  |  | 900.00 |  | L1 | RV |  | The speed and distance monitoring information is displayed (V_PERM = <V1>) |  | O |  | DMI |  | V1 = V_REVERSE _____ V1=30km/h |  |  | L1 |  | RV | - |  |
|  | 90 |  | 8 |  |  | 900.00 |  | L1 | RV |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_REVERSE M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) _____ V1=30km/h |  |  | L1 |  | RV | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 91 |  | 1 |  |  | 850.00 |  | L1 | RV |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  | RV | - |  |
|  | 92 |  | 2 |  |  | 850.00 |  | L1 | RV |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | RV | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  | 1 |  |  | 800.00 |  | L1 | RV |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  | NP | - |  |
|  | 95 |  | 2 |  |  | 800.00 |  | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 41 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 0 d |  |  |
| D_TSR | 15 | 0 d |  |  |
| L_TSR | 15 | 400 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| V_TSR | 7 | 5 d |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 286 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 6 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 250 d |  |  |
| T_NVOVTRP | 8 | 100 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 1 |  |  |
| Q_NVKVINTSET | 2 | 0 |  |  |
| V_NVKVINT | 7 | 0 d |  |  |
| M_NVKVINT | 7 | 48 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_NVKRINT | 5 | 0 d |  |  |
| M_NVKRINT | 5 | 20 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| L_NVKRINT | 5 | 10 d |  |  |
| M_NVKRINT | 5 | 18 d |  |  |
| M_NVKTINT | 5 | 20 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 41 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 59 : Balise Group BG 2 d 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 10 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 10 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 59 : Balise Group BG 2 d 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 66 : Balise Group BG 3 d 1/2 - Distance 750.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 138 d | Reversing area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 55 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STARTREVERSE | 15 | 0 d |  |  |
| L_REVERSEAREA | 15 | 1000 d |  |  |
| NID_PACKET | 8 | 139 d | Reversing supervision information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 47 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REVERSE | 15 | 1000 d |  |  |
| V_REVERSE | 7 | 8 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 66 : Balise Group BG 3 d 2/2 - Distance 755.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 85 : Balise Group BG 4 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 139 d | Reversing supervision information |  |
| Q_DIR | 2 | 10 b |  |  |
| L_PACKET | 13 | 47 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REVERSE | 15 | 1000 d |  |  |
| V_REVERSE | 7 | 6 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 85 : Balise Group BG 4 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 91 : Balise Group BG 5 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 10 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 91 : Balise Group BG 5 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |