| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3131030_15_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3131030_15_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 21:36 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | CEDEX |

# Sequence description

Gamma train. TC8

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | SB | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] Select Level 1 |  |  | L1 |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 18 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 19 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 20 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 21 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 24 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 25 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | SR | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 | SR |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3131030: Requirements for Ceiling speed monitoring (Tables 5, 6 and 7) Test Case 8: The SBI speed is displayed when the supervision status is Overspeed. Triggering condition t2 of Table 5 (Overspeed Status). Revoking condition r1 of Table 6 (Normal Status). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 80.00 |  | L1 | FS |  | Train is moving (V_TRAIN = VMRSP) |  | I |  | INT |  | - |  |  | L1 |  | FS | - |  |
|  | 45 |  | 2 |  |  | 80.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The DMI aspect is coherent with Normal Status . Permitted speed is displayed. (As there is no change in the DMI, there is not JRU step with Normal Status recorded) |  |  | L1 |  | FS | - |  |
|  | 46 |  | 3 |  |  | 300.00 |  | L1 | FS |  | Train is moving (V_TRAIN > VMRSP) |  | I |  | INT |  | - |  |  | L1 |  | FS | - |  |
|  | 47 |  | 4 |  |  | 300.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | When the V_TRAIN (estimated speed) > Vmrsp, then the supervision status changes to "Overspeed Status" (t2 in 3.13.10.3.3 table 5 of the SRS). So, the DMI aspect is coherent with Overspeed Status in a Ceiling Speed Monitoring section. The DMI shall display the SBI Speed (3.13.10.3.2). |  |  | L1 |  | FS | - |  |
|  | 48 |  | 5 |  |  | 300.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=2; V_SBI=<V1>) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 2 (Overspeed Status) V1 will be the corresponding value for V_SBI. |  |  | L1 |  | FS | - |  |
|  | 49 |  | 6 |  |  | 400.00 |  | L1 | FS |  | Train is moving (V_TRAIN = VMRSP) |  | I |  | INT |  | - |  |  | L1 |  | FS | - |  |
|  | 50 |  | 7 |  |  | 400.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | When the V_TRAIN (estimated speed) = Vmrsp, then the supervision status changes to "Normal Status" (r1 in 3.13.10.3.3 table 6 of the SRS). |  |  | L1 |  | FS | - |  |
|  | 51 |  | 8 |  |  | 400.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0) is recorded |  | O |  | JRU |  | M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 650.00 |  | L1 | FS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  | NP | - |  |
|  | 53 |  | 2 |  |  | 650.00 |  | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 01 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 00 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 44 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 10 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 01 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 00 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |