| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_3130700_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_3130700_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 07/04/17 20:54 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 07.04.2017 | All |  | CEDEX |

# Sequence description

The main objective is to test the TCs 1 and 5 ofFT3130700. The NVs are sent at 50m. A Lambda train is used for testing.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  | Description of Events | I/O |  | Interface |  | Comments |  |  | Next |  |  |  | Test Result |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A | NP |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  | SB | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A | SB |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  | SB | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  | SB | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  | SB | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A | SB |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  | SB | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] Select Level 1. |  |  | L1 |  | SB | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  | SB | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 | SB |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 17 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  | SB | - |  |
|  | 18 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 19 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  | SB | - |  |
|  | 20 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  | SB | - |  |
|  | 21 |  | 5 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  | SB | - |  |
|  | 22 |  | 6 |  |  | 0.00 |  | L1 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 23 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 24 |  | 2 |  |  | 0.00 |  | L1 | SB |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  | SB | - |  |
|  | 25 |  | 3 |  |  | 0.00 |  | L1 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 | SB |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 | SB |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  | SB | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 | SB |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  | SB | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  | SB | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 | SB |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 | SB |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  | SB | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SB | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 | SR |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  | SR | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 | SR |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 | SR |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location |  |  | L1 |  | SR | - |  |
|  | Feature 4080420: 4.8.4: Accepted Information depending on the modes - Linking Test Case 3: A telegram including packet 5 Linking Information is transmitted via balise group and is received by the onboard equipment. No linking data were received previously. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 50.00 |  | L1 | SR |  | One Balise Group message containing packet 5 is received |  | I |  | BTM |  | Complete message is accepted |  |  | L1 |  | SR | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 2 |  |  | 50.00 |  | L1 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | SR | - |  |
|  | 42 |  | 3 |  |  | 50.00 |  | L1 | SR |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  | FS | - |  |
|  | 43 |  | 4 |  |  | 50.00 |  | L1 | FS |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  | FS | - |  |
|  | 44 |  | 5 |  |  | 50.00 |  | L1 | FS |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 1: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 50.00 |  | L1 | FS |  | Train is moving (V_TRAIN=<below V5>) |  | I |  | INT |  | See the meaning of V5 in the following step. |  |  | L1 |  | FS | - |  |
|  | 49 |  | 2 |  |  | 800.00 |  | L1 | FS |  | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=0, D_STATIC(1)=<L_AXLELOAD+L_0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=0), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=0), packet 88 (D_LX=0, V_LX=<V4>), packet 52 (D_PBDSR=0), packet 5, packet 12 (V_MAIN = <V7>) and packet 21 is received. |  | I |  | BTM |  | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L_0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC = <V1> and V_MAIN = <V7>. V5 = the speed restriction to ensure permitted braking distance (D_PBD will be set so as to ensure V5 is the minimum of all the speed restrictions). V6 = maximum train speed. The relationship between the values of the speed limits is: V5 < V4 < V3 < V2 < V1 < V7 < V6 < Vmax. The relationships between the relevant length values are the following ones: L_PBDSR < L_LX < L_TSR L_TSR +L_TRAIN < L_AXLELOAD L_TRAIN < L_0 This location is at distance D0. _____ V5=70km/h, according to the ERA tool for the calculation of the braking curves. |  |  | L1 |  | FS | - |  |
|  | 50 |  | 3 |  |  | 800.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 51 |  | 4 |  |  | 800.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM = <V5>) |  | O |  | DMI |  | The PS is equal to <V5> (the speed restriction to ensure permitted braking distance). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V5=70km/h, according to the ERA tool for the calculation of the braking curves. |  |  | L1 |  | FS | - |  |
|  | 52 |  | 5 |  |  | 800.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V5>) is recorded |  | O |  | JRU |  | V5 = the speed restriction to ensure permitted braking distance M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 53 |  | 6 |  |  | 1000.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V4> ) |  | O |  | DMI |  | This location is at distance D0 + L_PBDSR. The PS is equal to <V4> (Permitted speed for the LX speed restriction). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V4=100km/h |  |  | L1 |  | FS | - |  |
|  | 54 |  | 7 |  |  | 1000.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0, V_PERM = <V4>) is recorded |  | O |  | JRU |  | V4 = Permitted speed for the LX speed restriction M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 55 |  | 8 |  |  | 1200.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V3> ) |  | O |  | DMI |  | This location is at distance D0 + L_LX. The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V3=120km/h |  |  | L1 |  | FS | - |  |
|  | 56 |  | 9 |  |  | 1200.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded |  | O |  | JRU |  | V3 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | Feature 3060400: 3.6.4: Train Position Confidence Interval Test Case 1: The train passes linked Balise Group BGa. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 57 |  | 1 |  |  | 1200.00 |  | L1 | FS |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | The train passes the Balise Group BGa. |  |  | L1 |  | FS | - |  |
|  | 58 |  | 2 |  |  | 1800.00 |  | L1 | FS |  | One Balise Group message (NID_BG=<BGa>)containing packet 255 is received |  | I |  | BTM |  | Packets inside telegram are not relevant. |  |  | L1 |  | FS | - |  |
|  | 59 |  | 3 |  |  | 1800.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Balise Group Message is RECORDED. |  |  | L1 |  | FS | - |  |
|  | 60 |  | 4 |  |  | 1800.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded |  | O |  | JRU |  | L_DOUBTUNDER and L_DOUBTOVER have reduced their values after having recorded the Balise Group BGa. Check that value of L_DOUBTUNDER and L_DOUBTOVER in position report is equal to Q_LOCACC if linking info is available, to Q_NVLOCACC if not, or 12 m if neither linking info nor national values are available. The variables L_DOUBTUNDER and L_DOUBTOVER are stored in each JRU record. |  |  | L1 |  | FS | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 1: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 10 |  |  | 2150.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V2> ) |  | O |  | DMI |  | This location is at distance D0 + L_TSR+L_TRAIN. The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V2=140km/h |  |  | L1 |  | FS | - |  |
|  | 64 |  | 11 |  |  | 2150.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded |  | O |  | JRU |  | V2 = V_AXLELOAD M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | Feature 3060400: 3.6.4: Train Position Confidence Interval Test Case 1: The train passes linked Balise Group BGa. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 2150.00 |  | L1 | FS |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | The train passes the Balise Group BGa. |  |  | L1 |  | FS | - |  |
|  | 66 |  | 2 |  |  | 2950.00 |  | L1 | FS |  | One Balise Group message (NID_BG=<BGa>)containing packet 255 is received |  | I |  | BTM |  | Packets inside telegram are not relevant. |  |  | L1 |  | FS | - |  |
|  | 67 |  | 3 |  |  | 2950.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Balise Group Message is RECORDED. |  |  | L1 |  | FS | - |  |
|  | 68 |  | 4 |  |  | 2950.00 |  | L1 | FS |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1) is recorded |  | O |  | JRU |  | L_DOUBTUNDER and L_DOUBTOVER have reduced their values after having recorded the Balise Group BGa. Check that value of L_DOUBTUNDER and L_DOUBTOVER in position report is equal to Q_LOCACC if linking info is available, to Q_NVLOCACC if not, or 12 m if neither linking info nor national values are available. The variables L_DOUBTUNDER and L_DOUBTOVER are stored in each JRU record. |  |  | L1 |  | FS | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 1: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements being compensated by the train length when requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 12 |  |  | 3100.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V1> ) |  | O |  | DMI |  | This location is at distance D0 + L_AXLELOAD + L_TRAIN. The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V1=160km/h |  |  | L1 |  | FS | - |  |
|  | 72 |  | 13 |  |  | 3100.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_STATIC M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 73 |  | 14 |  |  | 4050.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V7> ) |  | O |  | DMI |  | This location is at distance D0 + L_AXLELOAD + L_0 + L_TRAIN. The PS is equal to <V7> (=V_MAIN ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V7=180km/h |  |  | L1 |  | FS | - |  |
|  | 74 |  | 15 |  |  | 4050.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V7>) is recorded |  | O |  | JRU |  | V7 = V_MAIN M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 75 |  | 16 |  |  | 4250.00 |  | L1 | FS |  | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V8>), packet 12 (V_MAIN = <V9>) and packet 21 is received. |  | I |  | BTM |  | The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. V6 = maximum train speed. The relation between the values of the speed limits is: V6 < V8 < V9 This location is at distance D1. _____ V6=200km/h |  |  | L1 |  | FS | - |  |
|  | 76 |  | 17 |  |  | 4250.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | 77 |  | 18 |  |  | 4250.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM = <V6>) |  | O |  | DMI |  | The PS is equal to V6 (maximum train speed). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V6=200km/h |  |  | L1 |  | FS | - |  |
|  | 78 |  | 19 |  |  | 4250.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V6>) is recorded |  | O |  | JRU |  | V6 = maximum train speed M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | Feature 3130700: 3.13.7: Determination of Most Restrictive Speed Profile (MRSP) Test Case 5: The Most Restrictive Speed Profile shall be computed from all speed restrictions by selecting the most restrictive parts of each element, some elements not being compensated by the train length because it is not requested by trackside. Besides, the MRSP shall be recalculated when any of the elements it is built of is changed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 |  | 1 |  |  | 4250.00 |  | L1 | FS |  | Train is moving (V_TRAIN=<below V3>) |  | I |  | INT |  | See the meaning of V3 in the following step. |  |  | L1 |  | FS | - |  |
|  | 84 |  | 6 |  |  | 4450.00 |  | L1 | FS |  | One Balise Group message containing packet 27 (D_STATIC=0, V_STATIC=<V1>, Q_FRONT=1, D_STATIC(1)=<L_AXLELOAD+L0>, V_STATIC(1)=<Vmax>), packet 51 (D_AXLELOAD=0, V_AXLELOAD=<V2>, Q_FRONT=1), packet 65 (D_TSR=0, V_TSR=<V3>, Q_FRONT=1), packet 5, packet 12 (V_MAIN = <V5>) and packet 21 is received. |  | I |  | BTM |  | Only in L1. This location is at distance D1. The length of the MA shall be long enough to prevent the on-board equipment from braking to the EoA/SvL or LoA during the execution of this TC. The length value "L0" will be a positive value and V_STATIC(1)=<Vmax> will be the highest speed value of all speed limits, in order to be able to see the supervision of V_STATIC=<V1> and the maximum train speed (<V4>). V4 = maximum train speed. The relation between the values of the speed limits is: V3 < V2 < V1 < V4 < V5 < Vmax. The relation between the relevant length values is: L_TSR < L_AXLELOAD. |  |  | L1 |  | FS | - |  |
|  | 85 |  | 7 |  |  | 4450.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Only in L1. |  |  | L1 |  | FS | - |  |
|  | 86 |  | 8 |  |  | 4450.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V3> ) |  | O |  | DMI |  | The PS is equal to <V3> (=V_TSR). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V3=80km/h |  |  | L1 |  | FS | - |  |
|  | 87 |  | 9 |  |  | 4450.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V3>) is recorded |  | O |  | JRU |  | V3 = V_TSR M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 88 |  | 10 |  |  | 4650.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V2> ) |  | O |  | DMI |  | In L1, this location is at distance D1 + L_TSR. In L2/L3, this location is at distance D0 + L_TSR. The PS is equal to <V2> (=V_AXLELOAD). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V2=100km/h |  |  | L1 |  | FS | - |  |
|  | 89 |  | 11 |  |  | 4650.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V2>) is recorded |  | O |  | JRU |  | V2 = V_AXLELOAD M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 90 |  | 12 |  |  | 4850.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V1> ) |  | O |  | DMI |  | In L1, this location is at distance D1 + L_AXLELOAD. In L2/L3, this location is at distance D0 + L_AXLELOAD. The PS is equal to <V1> (=V_STATIC). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V1=120km/h |  |  | L1 |  | FS | - |  |
|  | 91 |  | 13 |  |  | 4850.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V1>) is recorded |  | O |  | JRU |  | V1 = V_STATIC M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | 92 |  | 14 |  |  | 5050.00 |  | L1 | FS |  | The speed and distance monitoring information is displayed (V_PERM =<V4> ) |  | O |  | DMI |  | In L1, this location is at distance D1 + L_AXLELOAD + L_0. In L2/L3, this location is at distance D0 + L_AXLELOAD + L_0. The PS is equal to <V4> (=maximum train speed ). The DMI aspect is coherent with Normal Status in a Ceiling Speed Monitoring section. _____ V4=200km/h |  |  | L1 |  | FS | - |  |
|  | 93 |  | 15 |  |  | 5050.00 |  | L1 | FS |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; M_SDMTYPE=0; M_SDMSUPSTAT=0; V_PERM = <V4>) is recorded |  | O |  | JRU |  | V4 = maximum train speed M_SDMTYPE (Speed and distance monitoring type) = 0 (Ceiling speed monitoring) M_SDMSUPSTAT (Speed and distance monitoring supervision status) = 0 (Normal Status) |  |  | L1 |  | FS | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  | 1 |  |  | 5150.00 |  | L1 | FS |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  | FS | - |  |
|  | 95 |  | 2 |  |  | 5150.00 |  | L1 | FS |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  | FS | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 5400.00 |  | L1 | FS |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  | NP | - |  |
|  | 98 |  | 2 |  |  | 5400.00 |  | L1 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  | NP | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  | Data |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 20 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 10000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 264 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 750 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| N_ITER | 5 | 5 d |  |  |
| D_LINK | 15 | 1000 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| D_LINK | 15 | 1150 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| D_LINK | 15 | 1300 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| D_LINK | 15 | 200 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| D_LINK | 15 | 700 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 10 d |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 286 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 1 |  |  |
| Q_NVKVINTSET | 2 | 0 |  |  |
| V_NVKVINT | 7 | 0 d |  |  |
| M_NVKVINT | 7 | 48 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_NVKRINT | 5 | 0 d |  |  |
| M_NVKRINT | 5 | 20 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| L_NVKRINT | 5 | 10 d |  |  |
| M_NVKRINT | 5 | 18 d |  |  |
| M_NVKTINT | 5 | 20 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Balise Group BG 2 d 1/2 - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 52 d | Permitted Braking Distance Information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_PBD | 15 | 660 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_PBDSR | 8 | 0 d |  |  |
| Q_PBDSR | 1 | 0 b |  |  |
| D_PBDSR | 15 | 0 d |  |  |
| L_PBDSR | 15 | 200 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 88 d | Level Crossing information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 72 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LX | 8 | 0 d |  |  |
| D_LX | 15 | 0 d |  |  |
| L_LX | 15 | 400 d |  |  |
| Q_LXSTATUS | 1 | 1 |  |  |
| V_LX | 7 | 20 d |  |  |
| Q_STOPLX | 1 | 0 b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 0 d |  |  |
| D_TSR | 15 | 0 d |  |  |
| L_TSR | 15 | 600 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| V_TSR | 7 | 24 d |  |  |
| NID_PACKET | 8 | 51 d | Axle load speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 81 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_AXLELOAD | 15 | 0 d |  |  |
| L_AXLELOAD | 15 | 1550 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_AXLELOADCAT | 7 | 0000001 b |  |  |
| V_AXLELOAD | 7 | 28 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 32 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 2500 d |  |  |
| V_STATIC | 7 | 120 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 36 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 20000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Balise Group BG 2 d 2/2 - Distance 805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 58 : Balise Group BG 3 d 1/2 - Distance 1800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 58 : Balise Group BG 3 d 2/2 - Distance 1805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 66 : Balise Group BG 4 d 1/2 - Distance 2950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 66 : Balise Group BG 4 d 2/2 - Distance 2955.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 75 : Balise Group BG 5 d 1/2 - Distance 4250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 10 b |  |  |
| V_MAIN | 7 | 120 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 4000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 100 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 75 : Balise Group BG 5 d 2/2 - Distance 4255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 84 : Balise Group BG 6 d 1/2 - Distance 4450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 65 d | Temporary speed restriction |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_TSR | 8 | 0 d |  |  |
| D_TSR | 15 | 0 d |  |  |
| L_TSR | 15 | 200 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| V_TSR | 7 | 16 d |  |  |
| NID_PACKET | 8 | 51 d | Axle load speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 81 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 |  |  |
| D_AXLELOAD | 15 | 0 d |  |  |
| L_AXLELOAD | 15 | 400 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_AXLELOADCAT | 7 | 0000001 b |  |  |
| V_AXLELOAD | 7 | 20 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 24 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 600 d |  |  |
| V_STATIC | 7 | 120 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 10 b |  |  |
| V_MAIN | 7 | 100 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 84 : Balise Group BG 6 d 2/2 - Distance 4455.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 94 : Balise Group BG 7 d 1/2 - Distance 5150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 10100 b |  |  |
| A_NVMAXREDADH2 | 6 | 1110 b |  |  |
| A_NVMAXREDADH3 | 6 | 1110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 94 : Balise Group BG 7 d 2/2 - Distance 5155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |