| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_6060304_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_6060304_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 14:26 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

6.6.3.4 Messages transmitted to RBC/RIU

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - _____ The driver sets: - Train category to TILT2: * NC_CDTRAIN=5 (Cant Deficiency 180mm ) * NC_TRAIN=4 (Passenger train) - Loading gauge to G1 (M_LOADINGAUGE=1) - Axle load category to D2 (M_AXLELOADCAT=7) |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 _____ The driver sets the Train running number to 12345678 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 44: The train switches in FS mode, because a MA and Track Description are stored on-board and no specific mode is required by a Mode Profile and No Trip order is given by balise. Transition Condition Id [32]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 , packet 21 and packet 27 is received |  | I |  | BTM |  | Packet 80 not included for current location _____ T_LOA=60s (used to test FT6060304.2) T0: The timer starts |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 100.00 |  | L1 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 45 |  | 2 |  |  | 100.00 |  | L1 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 |  | 1 |  |  | 100.00 |  | L1 |  | FS |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L1 |  |  | FS |  | - |  |
|  | 47 |  | 2 |  |  | 100.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 3 |  |  | 100.00 |  | L1 |  | FS |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | FS |  | - |  |
|  | 49 |  | 4 |  |  | 100.00 |  | L2 |  | FS |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 3 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | FS |  | - |  |
|  | 51 |  | 4 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 52 |  | 5 |  |  | 100.00 |  | L2 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 53 |  | 6 |  |  | 100.00 |  | L2 |  | FS |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 54 |  | 7 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 55 |  | 8 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 56 |  | 9 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board _____ M_VERSION = 001 0000 b (Version 1.0) |  |  | L2 |  |  | FS |  | - |  |
|  | 57 |  | 10 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 58 |  | 11 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 59 |  | 12 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 6: Transmission and translation of packet 11: 'Validated Train data' with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  | 1 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. |  | O |  | RTM |  | For Message 129 no translation is necessary. For Packet 11 the translation is: NID_OPERATIONAL=NID_OPERTAIONAL1 NC_CDTRAIN translated to NC_TRAIN and set to xxx xxxx x1xx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0101101 N_ITER set to 0 _____ Details about the values in packet 11: - NID_OPERATIONAL= 1234 5678 bcd - NC_TRAIN=000 1000 0100 0000 b - N_ITER=0 for the first occurence of N_ITER |  |  | L2 |  |  | FS |  | - |  |
|  | 62 |  | 2 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 63 |  | 3 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | The RBC understands and acknowledges this set of Train Data. |  |  | L2 |  |  | FS |  | - |  |
|  | 64 |  | 4 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080423: 4.8.4: Accepted Information depending on the modes - MA Request Parameters Test Case 2: The train receives and accepts a message including Packet 57 (MA request parameters) that is transmitted from RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 100.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 57 is received |  | I |  | RTM |  | Complete message is accepted _____ T_TIMEOUTRQST=25s (used to test FT6060304.2) |  |  | L2 |  |  | FS |  | - |  |
|  | 66 |  | 2 |  |  | 100.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Complete message is recorded |  |  | L2 |  |  | FS |  | - |  |
|  | 68 |  | 4 |  |  | 100.00 |  | L2 |  | FS |  |  | 'Start' is selected |  | I |  | DMI |  | Only level 2/3 |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 2: Transmission of the message 132: 'MA Request' by the on-board equipment to an RBC certified to the system version number X = 1.Reception of the message 3:' Movement Authority' from the X=1 RBC. Tested action: Data is translated: T[4] & data is unchanged U[1] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 1 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 1 is transmitted to a system version X=1 RBC. |  | O |  | RTM |  | Q_TRACKDEL=0 For Message132, the on-board effectues the translation to system version X=1 as: Q_MARQSTREASON is replaced with Q_TRACKDEL And Value diferent from x1xxx is set to 0 (no track description deleted) _____ T1=T0+T_LOA-T_TIMEOUTRQST=T0+35s |  |  | L2 |  |  | FS |  | - |  |
|  | 72 |  | 2 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 73 |  | 3 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) |  | I |  | RTM |  | No data translation is necesary.Data from message and packet remains unchanged. _____ packets 21 and 27 added |  |  | L2 |  |  | FS |  | - |  |
|  | 74 |  | 4 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 75 |  | 5 |  |  | 400.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed. |  | O |  | DMI |  | MA and track description for level 2/3 is used. |  |  | L2 |  |  | FS |  | - |  |
|  | 76 |  | 6 |  |  | 400.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080420: 4.8.4: Accepted Information depending on the modes - Linking Test Case 1: The trackside generates and sends a message including packet 5 (Linking Information) via RBC to a train. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 77 |  | 1 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received |  | I |  | RTM |  | Complete message is accepted |  |  | L2 |  |  | FS |  | - |  |
|  | 78 |  | 2 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Complete message is recorded |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 1: Transmission of the message 132: 'MA Request' by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ' T[4]' & data is unchanged 'U[1]' |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 80 |  | 1 |  |  | 550.00 |  | L2 |  | FS |  |  | No linked balise group message is received |  | I |  | BTM |  | The balise group missed was linked with service brake reaction |  |  | L2 |  |  | FS |  | - |  |
|  | 81 |  | 2 |  |  | 550.00 |  | L2 |  | FS |  |  | Service brake commanded. |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 82 |  | 3 |  |  | 550.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake " is displayed. |  | O |  | DMI |  | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 |  |  | L2 |  |  | FS |  | - |  |
|  | 83 |  | 4 |  |  | 550.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 84 |  | 5 |  |  | 550.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 85 |  | 6 |  |  | 600.00 |  | L2 |  | FS |  |  | Train is at standstill (V_TRAIN=0; TIME= 60) |  | I |  | INT |  | - _____ In this sequence, the duration of the stop is 20s, not 60s |  |  | L2 |  |  | FS |  | - |  |
|  | 86 |  | 7 |  |  | 600.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is removed |  | O |  | DMI |  | - _____ The MA is shortened to the current position |  |  | L2 |  |  | FS |  | - |  |
|  | 87 |  | 8 |  |  | 600.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. |  | O |  | RTM |  | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) |  |  | L2 |  |  | FS |  | - |  |
|  | 88 |  | 9 |  |  | 600.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 10 |  |  | 600.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21 and packet 27 is received. |  | I |  | RTM |  | The onboard should take into account exception (3) requirement 6.6.3.2.3 |  |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 11 |  |  | 600.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080420: 4.8.4: Accepted Information depending on the modes - Linking Test Case 1: The trackside generates and sends a message including packet 5 (Linking Information) via RBC to a train. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 91 |  | 1 |  |  | 600.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 5 is received |  | I |  | RTM |  | Complete message is accepted |  |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 2 |  |  | 600.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Complete message is recorded |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 32: The train receives an order to switch immediately from L2 to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  | 1 |  |  | 650.00 |  | L2 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. • D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  | I |  | BTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 95 |  | 2 |  |  | 650.00 |  | L2 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 96 |  | 3 |  |  | 650.00 |  | L2 |  | FS |  |  | Level 1 is DISPLAYED. |  | O |  | DMI |  | The On-board equipment switches to Level 1. |  |  | L1 |  |  | FS |  | - |  |
|  | 97 |  | 4 |  |  | 650.00 |  | L1 |  | FS |  |  | New Level 1 is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 5: No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged 'U[2]'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 98 |  | 1 |  |  | 725.00 |  | L1 |  | FS |  |  | Balise Group 1 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received |  | I |  | BTM |  | The Balise group is passed before D_LINK |  |  | L1 |  |  | FS |  | - |  |
|  | 99 |  | 2 |  |  | 725.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 100 |  | 3 |  |  | 725.00 |  | L1 |  | FS |  |  | The distance monitoring information is displayed |  | O |  | DMI |  | TDM and MA length are updated. |  |  | L1 |  |  | FS |  | - |  |
|  | 101 |  | 4 |  |  | 725.00 |  | L1 |  | FS |  |  | DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 102 |  | 5 |  |  | 775.00 |  | L1 |  | FS |  |  | Balise Group 2 message (N_TOTAL=1; NID_C=<country/region>) containing packet 16 is received |  | I |  | BTM |  | The on-board antenna hasn’t left the expectation window of the BG1. BG2 found satisfied the same criteria as BG1 |  |  | L1 |  |  | FS |  | - |  |
|  | 103 |  | 6 |  |  | 775.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 7 |  |  | 775.00 |  | L1 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 8 |  |  | 775.00 |  | L1 |  | FS |  |  | Service brake commanded. |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 9 |  |  | 775.00 |  | L1 |  | FS |  |  | The Status symbol "Service Brake " is displayed. |  | O |  | DMI |  | Symbol ST01 |  |  | L1 |  |  | FS |  | - |  |
|  | 107 |  | 10 |  |  | 775.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 108 |  | 11 |  |  | 775.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 12 |  |  | 825.00 |  | L1 |  | FS |  |  | Train reaches standstill. Train is at standstill. |  | I |  | INT |  | V_MAIN = 0 |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 13 |  |  | 825.00 |  | L1 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. |  | O |  | RTM |  | Packet 4 is deleted because M_ERROR = 8 (X=2). The RBC not informed about the double repositioning error. |  |  | L1 |  |  | FS |  | - |  |
|  | 111 |  | 14 |  |  | 825.00 |  | L1 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 112 |  | 15 |  |  | 825.00 |  | L1 |  | FS |  |  | The new speed and distance monitoring information is displayed. |  | O |  | DMI |  | MA and Track description are shortened to the current train position. Driver is informed. |  |  | L1 |  |  | FS |  | - |  |
|  | 113 |  | 16 |  |  | 825.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 114 |  | 17 |  |  | 825.00 |  | L1 |  | FS |  |  | Service brake not commanded. |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 115 |  | 18 |  |  | 825.00 |  | L1 |  | FS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 116 |  | 19 |  |  | 825.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 117 |  | 20 |  |  | 825.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 1 |  |  | 825.00 |  | L1 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L1 |  |  | NP |  | - |  |
|  | 119 |  | 2 |  |  | 825.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 0 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 40 d |  |  |
| V_EMA | 7 | 0001010 b |  |  |
| T_EMA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 0 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 44 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 56 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0010000 b |  |  |

| - Step 58 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 21 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 3 d |  |  |
| L_PACKET | 13 | 90 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |

| - Step 61 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 150 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NID_OPERATIONAL | 32 | 12345678 h |  |  |
| NC_TRAIN | 15 | 000100010000000 b |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000000 b |  |  |
| M_AXLELOAD | 7 | 0101101 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_STM | 8 | 2 d |  |  |
| NID_STM | 8 | 3 d |  |  |

| - Step 63 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 65 : Message 24 - General Message (RBC n? 1) - Distance 100.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 16 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 57 d | Movement authority request parameters |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 49 d |  |  |
| T_MAR | 8 | 11111111 b |  |  |
| T_TIMEOUTRQST | 10 | 25 d |  |  |
| T_CYCRQST | 8 | 11111111 b |  |  |

| - Step 71 : Message 132 - MA Request (RBC n? 1) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_TRACKDEL | 1 | 0 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 300 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 73 : Message 3 - Movement Authority (RBC n? 1) - Distance 400.00 m Delay : 5.00 s Conditional Message Id : 132 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 15 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 77 : Message 24 - General Message (RBC n? 1) - Distance 400.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 18 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 69 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 450 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 01 b |  |  |
| Q_LOCACC | 6 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 87 : Message 132 - MA Request (RBC n? 1) - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_TRACKDEL | 1 | 1 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 500 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 89 : Message 3 - Movement Authority (RBC n? 1) - Distance 600.00 m Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 91 : Message 24 - General Message (RBC n? 1) - Distance 600.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 23 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 108 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 550 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_LINK | 15 | 100 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 11111111111111 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 50 d |  |  |

| - Step 94 : Balise Group BG 3 d 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 94 : Balise Group BG 3 d 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 98 : Balise Group BG 4 d 1/2 - Distance 725.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0010000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 d |  |  |
| NID_PACKET | 8 | 16 d | Transmission of the update of an MA section |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 40 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| L_SECTION | 15 | 2000 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 98 : Balise Group BG 4 d 2/2 - Distance 730.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0010000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 102 : Balise Group BG 5 d 1/2 - Distance 775.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0010000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 d |  |  |
| NID_PACKET | 8 | 16 d | Transmission of the update of an MA section |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 40 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| L_SECTION | 15 | 2000 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 102 : Balise Group BG 5 d 2/2 - Distance 780.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0010000 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 110 : Message 136 - Train Position Report (RBC n? 1) - Distance 825.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 175 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 2 d |  |  |