| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5190300_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5190300_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 14:04 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Req 5.19.3: Limited Supervision is requested for a further location.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L0 |  |  | L0 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L0 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L0 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 12: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to possibilities offered at S10 |  |  | L0 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L0 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | The Mode symbol "Acknowledge Unfitted" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver acknowledges "Unfitted" |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L0 |  | UN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded |  | O |  | JRU |  | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode |  |  | L0 |  |  | UN |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L0 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L0 |  |  | UN |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L0 |  | UN |  |  | The Mode symbol "Unfitted" is displayed |  | O |  | DMI |  | DMI Object: Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 39 |  | 11 |  |  | 0.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 19: The on-board equipment is in unfitted mode. Then it has to be checked that the UN mode is permanently displayed to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | UN mode is permanently DISPLAYED to the driver when the driver is in UN mode. |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L0 |  |  | UN |  | - |  |
|  | 41 |  | 2 |  |  | 0.00 |  | L0 |  | UN |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 34: There is no cab signalling (only the current train speed is displayed) to the driver as long as the current level is Level 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 42 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | No cab signalling is displayed to the driver while the system is in Level 0 |  | O |  | DMI |  | No permitted speed (except in SH if conditions are fulfilled), no target speed, no target distance are displayed to the driver. Only the current train speed is displayed. |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 35: An indication of Level 0 is displayed to the driver as long as the current ETCS level is 0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | Permanent Indication of Level 0 to the driver |  | O |  | DMI |  | The level indication may not be permanent if the driver uses the menus. |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 3: On-board equipment supervises train movements against the unfitted speed defined by a National value. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 500.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<V_PERMITTED>) |  | I |  | INT |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 45 |  | 2 |  |  | 750.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<V_PERMITTED+6>) |  | I |  | INT |  | - _____ At this step, the train speed is limited to 105km/h to avoid the brake application. |  |  | L0 |  |  | UN |  | - |  |
|  | 46 |  | 3 |  |  | 750.00 |  | L0 |  | UN |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L0 |  |  | UN |  | - |  |
|  | 47 |  | 4 |  |  | 750.00 |  | L0 |  | UN |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 48 |  | 5 |  |  | 850.00 |  | L0 |  | UN |  |  | One Balise Group message (N_TOTAL=0) containing packet 3 (V_NVUNFIT=110) is received |  | I |  | BTM |  | Received value for V_NVUNFIT is higher than the default value. |  |  | L0 |  |  | UN |  | - |  |
|  | 49 |  | 6 |  |  | 850.00 |  | L0 |  | UN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 50 |  | 7 |  |  | 950.00 |  | L0 |  | UN |  |  | The speed and distance monitoring information is removed |  | O |  | DMI |  | Indication of warning due to over-speeding is removed |  |  | L0 |  |  | UN |  | - |  |
|  | 51 |  | 8 |  |  | 950.00 |  | L0 |  | UN |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 1150.00 |  | L0 |  | UN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. |  |  | L0 |  |  | UN |  | - |  |
|  | 53 |  | 2 |  |  | 1150.00 |  | L0 |  | UN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 54 |  | 3 |  |  | 1150.00 |  | L0 |  | UN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 55 |  | 4 |  |  | 1150.00 |  | L0 |  | UN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 12: A L1 MA is transmitted via balise group and is received by a train that have received announcement of level transition to level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 1150.00 |  | L0 |  | UN |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | For FS mode, Packet 80: Mode Profile is not included. For OS mode, Packet 80: Mode Profile is included. Other packets can be included. |  |  | L0 |  |  | UN |  | - |  |
|  | 57 |  | 2 |  |  | 1150.00 |  | L0 |  | UN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 4: A Mode profile defining an LS area is available on-board. The train passes the beginning of the Limited Supervision area with its front end and the ERTMS/ETCS level switches to 1, 2 or 3. Then the on-board switches to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 62 |  | 1 |  |  | 1350.00 |  | L0 |  | UN |  |  | NOT The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | Mode transition was not valuated before the level change |  |  | L0 |  |  | UN |  | - |  |
|  | 64 |  | 3 |  |  | 1550.00 |  | L0 |  | UN |  |  | The Level symbol "Level <L1 / L2 / L3>" is displayed |  | O |  | DMI |  | Mode transition was not valuated before the level change |  |  | L1 |  |  | LS |  | - |  |
|  | 65 |  | 4 |  |  | 1550.00 |  | L1 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 66 |  | 5 |  |  | 1550.00 |  | L1 |  | LS |  |  | LS mode indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 67 |  | 6 |  |  | 1550.00 |  | L1 |  | LS |  |  | A request for acknowledgement of LS mode is DISPLAYED to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 68 |  | 7 |  |  | 1550.00 |  | L1 |  | LS |  |  | Driver acknowledges LS mode |  | I |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 69 |  | 8 |  |  | 1550.00 |  | L1 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  | 1 |  |  | 1750.00 |  | L1 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  |  | 1800.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | FS |  | - |  |
|  | 75 |  | 2 |  |  | 1800.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 76 |  | 3 |  |  | 1800.00 |  | L1 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 77 |  | 4 |  |  | 1800.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 1: Limited Supervision is requested for a further location when running in FS. An acknowledgement request for LS mode is displayed to the driver. The driver acknowledges LS mode. Mode is changed to Limited Supervision |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 2250.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; Q_MAMODE=1) is received |  | I |  | BTM |  | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) D_MAMODE=<FINITE VALUE (<>0)>; L_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>; Q_MAMODE=1 <Begin of LS area to be considered as the SvL> |  |  | L1 |  |  | FS |  | - |  |
|  | 79 |  | 2 |  |  | 2250.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | The steps 1,2 or 3,4 will be chosen for testing |  |  | L1 |  |  | FS |  | - |  |
|  | 82 |  | 5 |  |  | 2250.00 |  | L1 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | Trains starts braking to LS area border as if it was both the EOA and SvL (instead of the EOA and SvL given by the MA), with no Release Speed Otherwise EB will be applied (Only if necessary for EOA supervision) |  |  | L1 |  |  | FS |  | - |  |
|  | 83 |  | 6 |  |  | 2250.00 |  | L1 |  | FS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | (Only if necessary for EOA supervision) |  |  | L1 |  |  | FS |  | - |  |
|  | 84 |  | 7 |  |  | 2250.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | (Only if necessary for EOA supervision) |  |  | L1 |  |  | FS |  | - |  |
|  | 85 |  | 8 |  |  | 2250.00 |  | L1 |  | FS |  |  | The target distance is updated |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 86 |  | 9 |  |  | 2250.00 |  | L1 |  | FS |  |  | The target distance is updated is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 87 |  | 10 |  |  | 2500.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | Distance between estimated front end and beginning of Limited Supervision Area < L_ACKMAMODE (stored Packet 80) AND The speed is lower than the Limited Supervision mode permitted speed. V < V_NVLIMSUPERV |  |  | L1 |  |  | FS |  | - |  |
|  | 88 |  | 11 |  |  | 2500.00 |  | L1 |  | FS |  |  | The driver acknowledges "Limited Supervision" |  | I |  | DMI |  | Before reaching the LS area |  |  | L1 |  |  | FS |  | - |  |
|  | 89 |  | 12 |  |  | 2500.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1101) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 13 |  |  | 2700.00 |  | L1 |  | FS |  |  | The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 91 |  | 14 |  |  | 2700.00 |  | L1 |  | LS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 92 |  | 15 |  |  | 2700.00 |  | L1 |  | LS |  |  | Service brake not commanded |  | O |  | TIU |  | Only if it was applied for EOA supervision |  |  | L1 |  |  | LS |  | - |  |
|  | 93 |  | 16 |  |  | 2700.00 |  | L1 |  | LS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | Only if it was applied for EOA supervision |  |  | L1 |  |  | LS |  | - |  |
|  | 94 |  | 17 |  |  | 2700.00 |  | L1 |  | LS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | Only if it was applied for EOA supervision |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 3000.00 |  | L1 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 2: Limited Supervision is requested for a further location when running in FS or SR mode. An acknowledgement request for LS mode is displayed to the driver and the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  | 1 |  |  | 3100.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; Q_MAMODE=0) is received |  | I |  | BTM |  | The steps 1,2 or 3,4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train Q_MAMODE=0 <Begin of LS area considered as EOA, SvL derived from the MA> |  |  | L1 |  |  | FS |  | - |  |
|  | 100 |  | 2 |  |  | 3100.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | The steps 1,2 or 3,4 will be chosen for testing |  |  | L1 |  |  | FS |  | - |  |
|  | 103 |  | 5 |  |  | 3300.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | Only if L_ACKMAMODE received in packet 80 is > 0 When the train reached the acknowledgement window a request for acknowledgement of LS is displayed immediately to the driver |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 6 |  |  | 3300.00 |  | L1 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | The Service Brake is applied because of the EOA Until the switch to LS, the beginning of the LS area is considered as an EOA keeping the SvL given by the MA, with no release speed. The train is allowed to enter in the LS area by a distance corresponding to the length of the confidence interval |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 7 |  |  | 3300.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 8 |  |  | 3500.00 |  | L1 |  | FS |  |  | The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | The ERTMS/ETCS on-board equipment switches to LS mode when the max safe front end of the train reaches the LS area. |  |  | L1 |  |  | LS |  | - |  |
|  | 107 |  | 9 |  |  | 3500.00 |  | L1 |  | LS |  |  | Service brake not commanded |  | O |  | TIU |  | The Service Brake is released |  |  | L1 |  |  | LS |  | - |  |
|  | 108 |  | 10 |  |  | 3500.00 |  | L1 |  | LS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | Release of Service Brake is RECORDED. |  |  | L1 |  |  | LS |  | - |  |
|  | 109 |  | 11 |  |  | 3500.00 |  | L1 |  | LS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=12) is recorded |  | O |  | JRU |  | Mode transition to LS is RECORDED |  |  | L1 |  |  | LS |  | - |  |
|  | 110 |  | 12 |  |  | 3500.00 |  | L1 |  | LS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | A request for acknowledgement of LS is displayed |  |  | L1 |  |  | LS |  | - |  |
|  | 113 |  | 15 |  |  | 3500.00 |  | L1 |  | LS |  |  | NOT The driver acknowledges "Limited Supervision" |  | I |  | DMI |  | the driver acknowledgement time (refer to Appendix A3.1) Driver doesn’t acknowledges within 5 sec after the change to LS |  |  | L1 |  |  | LS |  | - |  |
|  | 114 |  | 16 |  |  | 3500.00 |  | L1 |  | LS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 115 |  | 17 |  |  | 3500.00 |  | L1 |  | LS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | SB indication is DISPLAYED |  |  | L1 |  |  | LS |  | - |  |
|  | 116 |  | 18 |  |  | 3500.00 |  | L1 |  | LS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 117 |  | 19 |  |  | 3500.00 |  | L1 |  | LS |  |  | The driver acknowledges "Limited Supervision" |  | I |  | DMI |  | the driver acknowledgement time (refer to Appendix A3.1) Driver acknowledges more than 5 sec after the change to LS |  |  | L1 |  |  | LS |  | - |  |
|  | 118 |  | 20 |  |  | 3500.00 |  | L1 |  | LS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1101) is recorded |  | O |  | JRU |  | Driver Acknowledgement of transition to LS mode is RECORDED |  |  | L1 |  |  | LS |  | - |  |
|  | 119 |  | 21 |  |  | 3500.00 |  | L1 |  | LS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 120 |  | 22 |  |  | 3500.00 |  | L1 |  | LS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 121 |  | 23 |  |  | 3500.00 |  | L1 |  | LS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 122 |  | 24 |  |  | 3500.00 |  | L1 |  | LS |  |  | The Mode symbol "Limited Supervision" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  | 1 |  |  | 3800.00 |  | L1 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 5: Limited Supervision is requested for a further location when running in FS or OS. An acknowledgement request for LS mode is displayed to the driver. The train leaves the "rectangle of acknowledgement" and the acknowledgement request is not taken back. The driver acknowledges LS mode. Mode is changed to LS. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 |  | 1 |  |  | 3900.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is received |  | I |  | BTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 3 |  |  | 3900.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=12; NID_PACKET=80; M_MAMODE=2; D_MAMODE=<FINITE VALUE (<>0)>; L_ACKMODE=<FINITE VALUE (<>0)>) is recorded |  | O |  | JRU |  | One of the steps 3 or 4 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) |  |  | L1 |  |  | FS |  | - |  |
|  | 129 |  | 5 |  |  | 3900.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=<lower than the LS mode speed limit>) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 130 |  | 6 |  |  | 3900.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | Distance between estimated front end and beginning of Limited Supervision Area smaller than L_ACKMAMODE |  |  | L1 |  |  | FS |  | - |  |
|  | 131 |  | 7 |  |  | 3900.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 132 |  | 8 |  |  | 4000.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=<higher than the LS mode speed limit>) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 133 |  | 9 |  |  | 4000.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | The acknowledgement is not taken back if the train leaves the 'rectangle of acknowledgement' |  |  | L1 |  |  | FS |  | - |  |
|  | 134 |  | 10 |  |  | 4000.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 135 |  | 11 |  |  | 4150.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=<lower than the LS mode speed limit>) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 136 |  | 12 |  |  | 4150.00 |  | L1 |  | FS |  |  | The Mode symbol "Acknowledge Limited Supervision" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 137 |  | 13 |  |  | 4150.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit37=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 138 |  | 14 |  |  | 4200.00 |  | L1 |  | FS |  |  | The driver acknowledges "Limited Supervision" |  | I |  | DMI |  | Before reaching the LS area |  |  | L1 |  |  | FS |  | - |  |
|  | 139 |  | 15 |  |  | 4200.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=13) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 140 |  | 16 |  |  | 4200.00 |  | L1 |  | FS |  |  | Any message (NID_MESSAGE_JRU=ALL; M_MODE=12) is recorded |  | O |  | JRU |  | The onboard equipment switches to LS mode |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  |  | 4300.00 |  | L1 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 143 |  | 1 |  |  | 4400.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ V_NVLIMSUPERV=11d (55km/h) |  |  | L1 |  |  | FS |  | - |  |
|  | 144 |  | 2 |  |  | 4400.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5190300: 5.19.3: Limited Supervision is requested for a further location Test Case 3: Limited Supervision is requested for a further location when running in FS mode. An acknowledgement request for LS mode is not displayed to the driver while the train speed is higher than the LS mode permitted speed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 146 |  | 1 |  |  | 4400.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  | I |  | BTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at further location) |  |  | L1 |  |  | FS |  | - |  |
|  | 148 |  | 3 |  |  | 4400.00 |  | L1 |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 149 |  | 4 |  |  | 4500.00 |  | L1 |  | FS |  |  | The train reaches the acknowledgement window and V> V_MAMODE. The request for acknowledgement is not displayed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 150 |  | 5 |  |  | 4600.00 |  | L1 |  | FS |  |  | The train speed decreases to a value lower than V_MAMODE and the acknowledgement request is displayed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 151 |  | 6 |  |  | 4600.00 |  | L1 |  | FS |  |  | The driver acknowledges the transition to LS mode. |  | I |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 152 |  | 7 |  |  | 4600.00 |  | L1 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 39: On-board supervision of Limited Supervision speed defined by national / default value. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 155 |  | 1 |  |  | 5000.00 |  | L1 |  | LS |  |  | The driver over-passes maximum Limited Supervision speed (Mode Related Speed Restriction). |  | I |  | INT |  | The ERTMS/ETCS on-board equipment supervises train speed versus a mode related speed restriction defined by national / default value. |  |  | L1 |  |  | LS |  | - |  |
|  | 156 |  | 2 |  |  | 5000.00 |  | L1 |  | LS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 157 |  | 3 |  |  | 5000.00 |  | L1 |  | LS |  |  | The service brake intervention indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 158 |  | 4 |  |  | 5000.00 |  | L1 |  | LS |  |  | The service brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 159 |  | 5 |  |  | 5200.00 |  | L1 |  | LS |  |  | The speed is lower or equal to maximum Limited Supervision speed (Mode Related Speed Restriction). |  | I |  | INT |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 160 |  | 6 |  |  | 5200.00 |  | L1 |  | LS |  |  | Service brake is RELEASED. |  | O |  | TIU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 161 |  | 7 |  |  | 5200.00 |  | L1 |  | LS |  |  | The service brake intervention indication is REMOVED. |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 162 |  | 8 |  |  | 5200.00 |  | L1 |  | LS |  |  | The release of service brake is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 163 |  | 1 |  |  | 5250.00 |  | L1 |  | LS |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ Reset of the National Values |  |  | L1 |  |  | LS |  | - |  |
|  | 164 |  | 2 |  |  | 5250.00 |  | L1 |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 166 |  | 1 |  |  | 5300.00 |  | L1 |  | LS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 167 |  | 2 |  |  | 5300.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=0 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 48 : Balise Group BG 999 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 999 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 22 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 1 d |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 48 : Balise Group BG 999 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 999 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 100 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 100 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 010 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 16 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1200 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 26 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 200 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 8 d |  |  |
| L_MAMODE | 15 | 400 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 100 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 100 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 74 : Balise Group BG 1 d 1/2 - Distance 1800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 26 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 74 : Balise Group BG 1 d 2/2 - Distance 1805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 78 : Balise Group BG 2 d 1/2 - Distance 2250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 400 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 350 d |  |  |
| L_ACKMAMODE | 15 | 150 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 4550 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 78 : Balise Group BG 2 d 2/2 - Distance 2255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 3 d 1/2 - Distance 3100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 400 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 200 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3650 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 99 : Balise Group BG 3 d 2/2 - Distance 3105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 31 d 1/2 - Distance 3900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 31 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3750 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 100 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 100 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 31 d 2/2 - Distance 3905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 31 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 143 : Balise Group BG 4 d 1/2 - Distance 4400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 200 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 1111111 b |  |  |
| L_MAMODE | 15 | 1000 d |  |  |
| L_ACKMAMODE | 15 | 100 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 26 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2750 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 11 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00000 b |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 143 : Balise Group BG 4 d 2/2 - Distance 4405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 163 : Balise Group BG 5 d 1/2 - Distance 5250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 163 : Balise Group BG 5 d 2/2 - Distance 5255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |