| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5040300_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5040300_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 11:39 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

5.4.3 Procedure Start of mission

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080427: 4.8.4: Accepted Information depending on the modes - Plain Text Information Test Case 3: A plain text message sent via balise group is received by the on-board equipment and recorded in the JRU. The message is ACCEPTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 72 is received |  | I |  | BTM |  | If in stand by mode the balise group consists of only one balise. _____ The aim of this balise group is to set a determined LRBG: NID_C=64 NID_BG=1 --> NID_LRBG=1048577 |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 100.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reaches standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 44 |  | 2 |  |  | 100.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 |  | 1 |  |  | 100.00 |  | L1 |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | L1 |  |  | NP |  | - |  |
|  | 46 |  | 2 |  |  | 100.00 |  | L1 |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 47 |  | 1 |  |  | 100.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | L1 |  |  | SB |  | - |  |
|  | 48 |  | 2 |  |  | 100.00 |  | L1 |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 49 |  | 3 |  |  | 100.00 |  | L1 |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 50 |  | 4 |  |  | 100.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 51 |  | 1 |  |  | 100.00 |  | L1 |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L2 |  |  | L1 |  |  | SB |  | - |  |
|  | 52 |  | 2 |  |  | 100.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - _____ The Driver selects L2 |  |  | L1 |  |  | SB |  | - |  |
|  | 53 |  | 3 |  |  | 100.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 40: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to re-enter the RBC-ID/phone number if its status is 'invalid' or "valid". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 55 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver presses the ‘Enter RBC data’ button |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 59 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver modifies "RBC Data" |  | I |  | DMI |  | The Driver set a RBC ID value in [0-16777214] _____ The driver sets (even if the initial values are the same): - the RBC ID to 1050091 - the RBC phone number to 003265342103 |  |  | L2 |  |  | SB |  | - |  |
|  | 61 |  | 8 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver modifies the existing RBC Phone number. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 62 |  | 9 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 63 |  | 10 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 64 |  | 11 |  |  | 100.00 |  | L2 |  | SB |  |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 26: Within the scope of the Start of Mission procedure in SB mode, the procedure ends as soon as the desk is closed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver closes the desk |  | I |  | TIU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 67 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | 'CAB STATUS' closed is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 68 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver opens the desk |  | I |  | TIU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 69 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | 'CAB STATUS' open is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L2 |  |  | SB |  | - |  |
|  | 72 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L2 |  |  | SB |  | - |  |
|  | 73 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 11: The on-board equipment allows the driver to validate an existing ERTMS/ETCS Level data within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver validates the ERMS/ETCS level. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 75 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The initial ERTMS/ETCS Level is still displayed on the DMI. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 76 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | Driver’s validation action is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 40: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to re-enter the RBC-ID/phone number if its status is 'invalid' or "valid". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 79 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver presses the ‘Enter RBC data’ button |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 80 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). |  | O |  | DMI |  | - _____ The current data are: - RBC ID=1050091 - RBC phone number=003265342103 |  |  | L2 |  |  | SB |  | - |  |
|  | 81 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver modifies "RBC Data" |  | I |  | DMI |  | The Driver tries to set a RBC ID value out of [0-16777214] |  |  | L2 |  |  | SB |  | - |  |
|  | 82 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | NOT The Driver validates the new RBC ID |  | I |  | DMI |  | The validation of RBC ID has no effect. |  |  | L2 |  |  | SB |  | - |  |
|  | 83 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver modifies "RBC Data" |  | I |  | DMI |  | The Driver set a RBC ID value in [0-16777214] _____ The driver sets: - the RBC ID to 1050192 - the RBC phone number to 003265342104 |  |  | L2 |  |  | SB |  | - |  |
|  | 84 |  | 7 |  |  | 100.00 |  | L2 |  | SB |  |  | The Driver validates the new RBC ID |  | I |  | DMI |  | The validation of RBC ID enable the "RBC data entry complete? Yes" |  |  | L2 |  |  | SB |  | - |  |
|  | 85 |  | 8 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver modifies the existing RBC Phone number. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 86 |  | 9 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 87 |  | 10 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 88 |  | 11 |  |  | 100.00 |  | L2 |  | SB |  |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded |  | O |  | JRU |  | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 90 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  |  | L2 |  |  | SB |  | - |  |
|  | 91 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  |  | L2 |  |  | SB |  | - |  |
|  | 92 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 93 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 94 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 95 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 96 |  | 7 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SB |  | - |  |
|  | 97 |  | 8 |  |  | 100.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 98 |  | 9 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 99 |  | 10 |  |  | 100.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 31: On-board equipment reports to RBC 'invalid train position' and RBC confirms train position |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | On-board reports invalid position to RBC (A34) |  | O |  | RTM |  | MESSAGE 157, SoM position report |  |  | L2 |  |  | SB |  | - |  |
|  | 103 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | Message recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 104 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | RBC is able to confirm position (D33), it reports valid position (A35) |  | I |  | RTM |  | Message 43. SoM position report confirmed by RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 105 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | Message RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L2 |  |  | SB |  | - |  |
|  | 107 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 108 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 109 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L2 |  |  | SB |  | - |  |
|  | 112 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 113 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L2 |  |  | SB |  | - |  |
|  | 114 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L2 |  |  | SB |  | - |  |
|  | 115 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L2 |  |  | SB |  | - |  |
|  | 116 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 119 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L2 |  |  | SB |  | - |  |
|  | 120 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 121 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 122 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 123 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 124 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 125 |  | 7 |  |  | 100.00 |  | L2 |  | SB |  |  | The DMI allows the driver to select ‘START’. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 21: Within the scope of the Start of Mission procedure in SB or PT mode and in ERTMS Level 2 or 3, the on-board equipment cyclically requests the RBC for a Movement Authority from the time the driver requests the 'Start of Mission' until the RBC responds to the MA request. No MA request parameters were received before and the default values are used |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 126 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | Action performed within Use_FT5040300.17-19 . |  |  | L2 |  |  | SB |  | - |  |
|  | 127 |  | 2 |  |  | 100.00 |  | L2 |  | SB |  |  | T = t’ The on-board equipment sends the radio message 132 'MA request' to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 128 |  | 3 |  |  | 100.00 |  | L2 |  | SB |  |  | The radio message 132 'MA request' sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 129 |  | 4 |  |  | 100.00 |  | L2 |  | SB |  |  | T = t’ + T_ CYCRQSTD The on-board equipment sends the radio message 132 'MA request' to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | RTM |  | Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD = 60s (default value) |  |  | L2 |  |  | SB |  | - |  |
|  | 130 |  | 5 |  |  | 100.00 |  | L2 |  | SB |  |  | The radio message 132 'MA request' sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 131 |  | 6 |  |  | 100.00 |  | L2 |  | SB |  |  | t’ + T_ CYCRQSTD < T < t’ + N * T_ CYCRQSTD The radio message 132 'MA request' is sent cyclically to the RBC every T_ CYCRQSTD seconds (= 60s ) (+ recorded on the JRU). The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | RTM |  | Radio message 132 is sent to the RBC through the RTM (step i) Radio message 132 sent to the RBC is recorded on the JRU (step i+1) N ( 2 and K = 2*N + 1 (N is chosen by the tester) ( The MA request is performed at least twice. Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD. |  |  | L2 |  |  | SB |  | - |  |
|  | 132 |  | 7 |  |  | 100.00 |  | L2 |  | SB |  |  | The radio message 132 'MA request' sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 133 |  | 8 |  |  | 100.00 |  | L2 |  | SB |  |  | T = t’ + N * T_ CYCRQSTD The on-board equipment sends the radio message 132 'MA request' to the RBC. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | RTM |  | Check that the difference of time between two consecutive MA requests is effectively equal to T_ CYCRQSTD = 60s (default value) |  |  | L2 |  |  | SB |  | - |  |
|  | 134 |  | 9 |  |  | 100.00 |  | L2 |  | SB |  |  | The radio message 132 'MA request' sent to the RBC is recorded. The variable Q_MARQSTREASON=xxxx1 (Start selected by driver). |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 100.00 |  | L2 |  | SB |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 136 |  | 2 |  |  | 100.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 00000000000039 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 01001000 b | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011101100 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 000000111110100 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 111111111111111 b |  |  |
| T_TEXTDISPLAY | 10 | 1111111111 b |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 18 d |  |  |
| X_TEXT | 8 | 01000010 b |  |  |
| X_TEXT | 8 | 01000111 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 00101000 b |  |  |
| X_TEXT | 8 | 00110110 b |  |  |
| X_TEXT | 8 | 00110100 b |  |  |
| X_TEXT | 8 | 00101100 b |  |  |
| X_TEXT | 8 | 00110001 b |  |  |
| X_TEXT | 8 | 00101001 b |  |  |
| X_TEXT | 8 | 00100000 b |  |  |
| X_TEXT | 8 | 01110010 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01100011 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01101001 b |  |  |
| X_TEXT | 8 | 01110110 b |  |  |
| X_TEXT | 8 | 01100101 b |  |  |
| X_TEXT | 8 | 01100100 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 39 : Balise Group BG 00000000000039 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000001 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| Step 51: DMI Events Distance 100.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=2 | 10 s |

| - Step 94 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10011011 b |  |  |
| L_MESSAGE | 10 | 0000001010 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 96 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 173.00 s Conditional Message Id : 155 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 00100000 b |  |  |
| L_MESSAGE | 10 | 0000001011 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 98 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 102 : Message 157 - SoM Position Report (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 01 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 32767 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 104 : Message 43 - SoM Position Report Confirmed by RBC (RBC n? 1) - Distance 100.00 m Delay : 8.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 43 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |

| - Step 121 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 10000001 b |  |  |
| L_MESSAGE | 10 | 0000100100 b |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 00000000 b | Position report |  |
| L_PACKET | 13 | 0000001110010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 000100000000000000000001 b |  |  |
| D_LRBG | 15 | 000000000110010 b |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 000000000000000 b |  |  |
| L_DOUBTUNDER | 15 | 000000000000000 b |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0000000 b |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 00001011 b | Validated train data |  |
| L_PACKET | 13 | 0000001100000 b |  |  |
| NC_CDTRAIN | 4 | 0000 b |  |  |
| NC_TRAIN | 15 | 000000000000000 b |  |  |
| L_TRAIN | 12 | 000000000000 b |  |  |
| V_MAXTRAIN | 7 | 0000000 b |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 0000000 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 123 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 20.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 127 : Message 132 - MA Request (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 50 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 129 : Message 132 - MA Request (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 50 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 131 : Message 132 - MA Request (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 50 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 133 : Message 132 - MA Request (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 50 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 3 d |  |  |