| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5100400_05_LNTC_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5100400_05_LNTC_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:11 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | INECO |

# Sequence description

Level transitions from LNTC to L2 and viceversa are tested with driver ack before or after new level area.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects LNTC |  |  | LNTC |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | LNTC |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | LNTC |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | LNTC |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | LNTC |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | LNTC |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | LNTC |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | LNTC |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | LNTC |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | LNTC |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | LNTC |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 15: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level NTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU. |  | O |  | JRU |  | Triggered by : The driver acts on the on-board DMI. |  |  | LNTC |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 51: The ETCS on-board equipment switches to SN Mode within the Start of Mission procedure. Transition Condition Id [58]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 32 |  | 1 |  |  | 0.00 |  | LNTC |  | SB |  |  | The Mode symbol 'Acknowledge National System' is displayed |  | O |  | DMI |  | DMI Object: MO20 |  |  | LNTC |  |  | SB |  | - |  |
|  | 33 |  | 2 |  |  | 0.00 |  | LNTC |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit35=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 34 |  | 3 |  |  | 0.00 |  | LNTC |  | SB |  |  | The driver acknowledges 'National System' |  | I |  | DMI |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 35 |  | 4 |  |  | 0.00 |  | LNTC |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11: M_DRIVERACTIONS= ) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SB |  | - |  |
|  | 36 |  | 5 |  |  | 0.00 |  | LNTC |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 37 |  | 6 |  |  | 0.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 Or USE_FT4070201.12: Indication of NTC National mode to the driver. |  |  | LNTC |  |  | SN |  | - |  |
|  | 38 |  | 7 |  |  | 0.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 39 |  | 8 |  |  | 0.00 |  | LNTC |  | SN |  |  | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. |  | O |  | JRU |  | Optional step: The status of the Train position information will change to 'Unknown' only if it was 'Invalid'. |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 100.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L2 Annouced |  |  | LNTC |  |  | SN |  | - |  |
|  | 41 |  | 2 |  |  | 100.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 42 |  | 3 |  |  | 100.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 43 |  | 4 |  |  | 100.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 6: Transition from Level NTC to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the boarder location, Train Data shall be sent to the RBC (which acknowledges the data). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 1 |  |  | 100.00 |  | LNTC |  | SN |  |  | Train is moving |  | I |  | INT |  | Train location: in rear of the border of level transition location |  |  | LNTC |  |  | SN |  | - |  |
|  | 45 |  | 2 |  |  | 150.00 |  | LNTC |  | SN |  |  | Balise Group message containing packet 42 is received . |  | I |  | BTM |  | Packet 42: Q_RBC = 1 Q_SLEEPSESSION=1 The packet 42 is received on-board via balise group in rear of the border location |  |  | LNTC |  |  | SN |  | - |  |
|  | 46 |  | 3 |  |  | 150.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 3 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | LNTC |  |  | SN |  | - |  |
|  | 51 |  | 4 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 52 |  | 5 |  |  | 150.00 |  | LNTC |  | SN |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 53 |  | 6 |  |  | 150.00 |  | LNTC |  | SN |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 54 |  | 7 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 55 |  | 8 |  |  | 150.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 56 |  | 9 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | LNTC |  |  | SN |  | - |  |
|  | 57 |  | 10 |  |  | 150.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 58 |  | 11 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 59 |  | 12 |  |  | 150.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100300: 5.10.3: Specific Additional Requirements Test Case 6: Transition from Level NTC to level 2/3 area .When the ERTMS/ETCS communication session is open in rear of the boarder location, Train Data shall be sent to the RBC (which acknowledges the data). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 |  | 5 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted |  | O |  | RTM |  | This message is not sent in NL or SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 62 |  | 6 |  |  | 150.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 63 |  | 7 |  |  | 150.00 |  | LNTC |  | SN |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. |  | I |  | RTM |  | The RBC acknowledges train data. This message is not sent in NL or SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 64 |  | 8 |  |  | 150.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 30: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L2. The driver acknowledges before entering the L2 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 450.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 2 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE12 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 66 |  | 2 |  |  | 450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 67 |  | 3 |  |  | 450.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 2 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE13 Except in SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 68 |  | 4 |  |  | 450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 69 |  | 5 |  |  | 450.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 2 announcement" |  | I |  | DMI |  | Except in SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 70 |  | 6 |  |  | 450.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded |  | O |  | JRU |  | Except in SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 71 |  | 7 |  |  | 450.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 2 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE13 Except in SL mode |  |  | LNTC |  |  | SN |  | - |  |
|  | 72 |  | 8 |  |  | 450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 73 |  | 9 |  |  | 450.00 |  | LNTC |  | SN |  |  | The Level symbol " Level 2 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE12 |  |  | LNTC |  |  | SN |  | - |  |
|  | 74 |  | 10 |  |  | 450.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit12=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080422: 4.8.4: Accepted Information depending on the modes - Location Identity (NID_C + NID_BG transmitted in the balise telegram) Test Case 1: The Location Identity (NID_C + NID_BG) of the BG message is accepted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 75 |  | 1 |  |  | 460.00 |  | LNTC |  | SN |  |  | One Balise Group message containing packet 255 is received |  | I |  | BTM |  | The train overpasses all the balises in the BG. The BG is marked as linked. No message consistency problem: All the telegrams are correctly received. |  |  | LNTC |  |  | SN |  | - |  |
|  | 76 |  | 2 |  |  | 460.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 77 |  | 3 |  |  | 460.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; NID_LRBG=<equal to the identity of the previous BG>) is recorded |  | O |  | JRU |  | The variable TRAIN_POSITION is recorded in the header of the JRU message with the correct value of NID_LRBG (i.e., the value of the Location Identity -NID_C + NID_BG-, given in the BG) |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 18: The train receives a message that is transmitted via RBC. The message is accepted because a level 2 /3 transition is stored onborad |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 460.00 |  | LNTC |  | SN |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 79 |  | 2 |  |  | 460.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 19: Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 500.00 |  | LNTC |  | SN |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 is received |  | I |  | BTM |  | At the level border the order to switch to ETCS level 1,2 or 3 |  |  | LNTC |  |  | SN |  | - |  |
|  | 87 |  | 2 |  |  | 500.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 88 |  | 3 |  |  | 500.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded |  | O |  | JRU |  | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode |  |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 4 |  |  | 500.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is displayed. |  | O |  | DMI |  | Displayed to the driver until SSP and gradients are known for the whole length of the train. |  |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 5 |  |  | 500.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 91 |  | 6 |  |  | 500.00 |  | L2 |  | FS |  |  | The Level symbol 'Level <L1/ L2/ L3>' is displayed |  | O |  | DMI |  | - _____ DMI Symbol LE04 |  |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 7 |  |  | 500.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03/ 04/ 05=1>) is recorded |  | O |  | JRU |  | - _____ Bit 04 |  |  | L2 |  |  | FS |  | - |  |
|  | 93 |  | 8 |  |  | 500.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 94 |  | 9 |  |  | 500.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 95 |  | 10 |  |  | 500.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is removed. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 96 |  | 11 |  |  | 500.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 30: The train receives an order to switch immediately from L2 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 98 |  | 1 |  |  | 700.00 |  | L2 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC |  | I |  | BTM |  | 'The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' _____ NID_NTC (1)= 3 |  |  | L2 |  |  | FS |  | - |  |
|  | 99 |  | 2 |  |  | 700.00 |  | L2 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 100 |  | 3 |  |  | 700.00 |  | L2 |  | FS |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L2 |  |  | FS |  | - |  |
|  | 101 |  | 4 |  |  | 700.00 |  | L2 |  | FS |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 |  | 2 |  |  | 700.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 103 |  | 3 |  |  | 700.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 104 |  | 4 |  |  | 700.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 17: The train has passed over the Level Transition border and the On-board equipment has switched from L2 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 105 |  | 1 |  |  | 700.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 106 |  | 2 |  |  | 700.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 107 |  | 3 |  |  | 700.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 108 |  | 4 |  |  | 700.00 |  | LNTC |  | SN |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 109 |  | 5 |  |  | 700.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 110 |  | 6 |  |  | 700.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 111 |  | 7 |  |  | 950.00 |  | LNTC |  | SN |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 112 |  | 8 |  |  | 950.00 |  | LNTC |  | SN |  |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 113 |  | 9 |  |  | 950.00 |  | LNTC |  | SN |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 114 |  | 10 |  |  | 950.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 115 |  | 11 |  |  | 950.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 116 |  | 12 |  |  | 950.00 |  | LNTC |  | SN |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 117 |  | 13 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 118 |  | 14 |  |  | 950.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 119 |  | 15 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 120 |  | 16 |  |  | 950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 121 |  | 1 |  |  | 1100.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ Announcemment of Level 2 |  |  | LNTC |  |  | SN |  | - |  |
|  | 122 |  | 2 |  |  | 1100.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 123 |  | 3 |  |  | 1100.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 124 |  | 4 |  |  | 1100.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080422: 4.8.4: Accepted Information depending on the modes - Location Identity (NID_C + NID_BG transmitted in the balise telegram) Test Case 1: The Location Identity (NID_C + NID_BG) of the BG message is accepted |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 125 |  | 1 |  |  | 1150.00 |  | LNTC |  | SN |  |  | One Balise Group message containing packet 255 is received |  | I |  | BTM |  | The train overpasses all the balises in the BG. The BG is marked as linked. No message consistency problem: All the telegrams are correctly received. |  |  | LNTC |  |  | SN |  | - |  |
|  | 126 |  | 2 |  |  | 1150.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 127 |  | 3 |  |  | 1150.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; NID_LRBG=<equal to the identity of the previous BG>) is recorded |  | O |  | JRU |  | The variable TRAIN_POSITION is recorded in the header of the JRU message with the correct value of NID_LRBG (i.e., the value of the Location Identity -NID_C + NID_BG-, given in the BG) |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 18: The train receives a message that is transmitted via RBC. The message is accepted because a level 2 /3 transition is stored onborad |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 128 |  | 1 |  |  | 1150.00 |  | LNTC |  | SN |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 129 |  | 2 |  |  | 1150.00 |  | LNTC |  | SN |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 19: Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 |  | 1 |  |  | 1200.00 |  | LNTC |  | SN |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 is received |  | I |  | BTM |  | At the level border the order to switch to ETCS level 1,2 or 3 _____ Order to L2 |  |  | LNTC |  |  | SN |  | - |  |
|  | 137 |  | 2 |  |  | 1200.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 138 |  | 3 |  |  | 1200.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded |  | O |  | JRU |  | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode |  |  | L2 |  |  | FS |  | - |  |
|  | 139 |  | 4 |  |  | 1200.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is displayed. |  | O |  | DMI |  | Displayed to the driver until SSP and gradients are known for the whole length of the train. |  |  | L2 |  |  | FS |  | - |  |
|  | 140 |  | 5 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 141 |  | 6 |  |  | 1200.00 |  | L2 |  | FS |  |  | The Level symbol 'Level <L1/ L2/ L3>' is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 142 |  | 7 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03/ 04/ 05=1>) is recorded |  | O |  | JRU |  | - _____ L2 Symbol LE04 |  |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 8 |  |  | 1200.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 9 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 145 |  | 10 |  |  | 1200.00 |  | L2 |  | FS |  |  | The system status message 'Entering FS' is removed. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 146 |  | 11 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 32: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 2, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 147 |  | 1 |  |  | 1200.00 |  | L2 |  | FS |  |  | The Level symbol "Acknowledge Level 2 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE13 |  |  | L2 |  |  | FS |  | - |  |
|  | 148 |  | 2 |  |  | 1200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 149 |  | 3 |  |  | 1350.00 |  | L2 |  | FS |  |  | NOT The driver acknowledges "Level 2 announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | L2 |  |  | FS |  | - |  |
|  | 150 |  | 4 |  |  | 1350.00 |  | L2 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 151 |  | 5 |  |  | 1350.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 152 |  | 6 |  |  | 1350.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 153 |  | 7 |  |  | 1450.00 |  | L2 |  | FS |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 154 |  | 8 |  |  | 1450.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 155 |  | 9 |  |  | 1450.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=136) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 156 |  | 10 |  |  | 1450.00 |  | L2 |  | FS |  |  | The driver acknowledges "Level 2 announcement" |  | I |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 157 |  | 11 |  |  | 1450.00 |  | L2 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1000) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 158 |  | 12 |  |  | 1450.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 159 |  | 13 |  |  | 1450.00 |  | L2 |  | FS |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 160 |  | 14 |  |  | 1450.00 |  | L2 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 161 |  | 15 |  |  | 1450.00 |  | L2 |  | FS |  |  | The Level symbol "Acknowledge Level 2 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE13 |  |  | L2 |  |  | FS |  | - |  |
|  | 162 |  | 16 |  |  | 1450.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit13=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 163 |  | 1 |  |  | 1700.00 |  | L2 |  | FS |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC annoucement |  |  | L2 |  |  | FS |  | - |  |
|  | 164 |  | 2 |  |  | 1700.00 |  | L2 |  | FS |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') |  |  | L2 |  |  | FS |  | - |  |
|  | 165 |  | 3 |  |  | 1700.00 |  | L2 |  | FS |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L2 |  |  | FS |  | - |  |
|  | 166 |  | 4 |  |  | 1700.00 |  | L2 |  | FS |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 15: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L2 to LNTC. The driver acknowledges before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 167 |  | 1 |  |  | 2200.00 |  | L2 |  | FS |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED.. |  |  | L2 |  |  | FS |  | - |  |
|  | 168 |  | 2 |  |  | 2200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 169 |  | 3 |  |  | 2200.00 |  | L2 |  | FS |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L2 |  |  | FS |  | - |  |
|  | 170 |  | 4 |  |  | 2200.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 171 |  | 5 |  |  | 2300.00 |  | L2 |  | FS |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 172 |  | 6 |  |  | 2300.00 |  | L2 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 173 |  | 7 |  |  | 2300.00 |  | L2 |  | FS |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L2 |  |  | FS |  | - |  |
|  | 174 |  | 8 |  |  | 2300.00 |  | L2 |  | FS |  |  | The Level symbol "Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE08 |  |  | L2 |  |  | FS |  | - |  |
|  | 175 |  | 9 |  |  | 2300.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 176 |  | 1 |  |  | 2500.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 177 |  | 2 |  |  | 2500.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=STM |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 40 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 40 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 2 d 1/2 - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101 FFFFh |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 45 : Balise Group BG 2 d 2/2 - Distance 155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 54 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 56 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 150.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 58 : Message 159 - Session Established (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 61 : Message 129 - Validated Train Data (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 41 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 122 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1101 b |  |  |
| M_LEVEL | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 63 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 150.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 75 : Balise Group BG 3 d 1/2 - Distance 460.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 75 : Balise Group BG 3 d 2/2 - Distance 465.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 78 : Message 3 - Movement Authority (RBC n? 1) - Distance 460.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 50 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 86 : Balise Group BG 4 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 86 : Balise Group BG 4 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 98 : Balise Group BG 5 d 1/2 - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 98 : Balise Group BG 5 d 2/2 - Distance 705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 112 : Message 136 - Train Position Report (RBC n? 1) - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 122 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048581 d |  |  |
| D_LRBG | 15 | 250 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1101 b |  |  |
| M_LEVEL | 3 | 1 |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 121 : Balise Group BG 6 d 1/2 - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 100 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 121 : Balise Group BG 6 d 2/2 - Distance 1105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 6 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 7 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 7 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 7 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 128 : Message 3 - Movement Authority (RBC n? 1) - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048583 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 50 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 136 : Balise Group BG 8 d 1/2 - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 136 : Balise Group BG 8 d 2/2 - Distance 1205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 8 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 154 : Message 136 - Train Position Report (RBC n? 1) - Distance 1450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048584 d |  |  |
| D_LRBG | 15 | 250 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 |  |  |

| - Step 163 : Balise Group BG 9 d 1/2 - Distance 1700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 1000 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 500 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 163 : Balise Group BG 9 d 2/2 - Distance 1705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 9 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |