| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5040300_04_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5040300_04_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 11:42 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

FT5040300: Table of requirements for "Start of Mission" procedure

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  | Dist. (m) | Previous |  |  | Description of Events | I/O | Interface | Comments |  | Next |  |  | Test Result |
|  |  |  |  |  | Levels | Mode |  |  |  |  |  |  | Levels |  | Mode |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 2 | 1 |  | 0.00 | N/A | NP |  | ERTMS/ETCS on-board unit is powered up | I | INT | The on-board equipment changes to SB mode. |  | N/A |  | NP | - |
|  | 3 | 2 |  | 0.00 | N/A | NP |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded | O | JRU | - |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 5 | 1 |  | 0.00 | N/A | SB |  | Driver opens desk | I | TIU | - |  | N/A |  | SB | - |
|  | 6 | 2 |  | 0.00 | N/A | SB |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). | O | JRU | - |  | N/A |  | SB | - |
|  | 7 | 3 |  | 0.00 | N/A | SB |  | The actual mode SB is DISPLAYED | O | DMI | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 8 | 1 |  | 0.00 | N/A | SB |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed | O | DMI | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  | N/A |  | SB | - |
|  | 9 | 2 |  | 0.00 | N/A | SB |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. | O | DMI | - |  | N/A |  | SB | - |
|  | 10 | 3 |  | 0.00 | N/A | SB |  | The driver enters and validates a correct Driver ID | I | DMI | - |  | N/A |  | SB | - |
|  | 11 | 4 |  | 0.00 | N/A | SB |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. | O | JRU | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | N/A |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 50: Following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 | 4 |  | 0.00 | N/A | SB |  | The driver selects "Level 2/3" and enters "RBC Data" | I | DMI | The status of the ERTMS/ETCS Level is set to valid. The status of the RBC ID/Phone Number is set to valid. _____ The Driver selects L2 The Driver enters: - RBC ID = 1050091 (NID_C=64 and NID_RBC=1515) - RBC phone number = 003265342101 (NID_RADIO) |  | N/A |  | SB | - |
|  | 14 | 5 |  | 0.00 | N/A | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<36¦37>) is recorded | O | JRU | The driver selects Level |  | L2 |  | SB | - |
|  | 15 | 6 |  | 0.00 | L2 | SB |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The driver enters "RBC Data" |  | L2 |  | SB | - |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 | 1 |  | 0.00 | L2 | SB |  | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  | L2 |  | SB | - |
|  | 19 | 2 |  | 0.00 | L2 | SB |  | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. |  | L2 |  | SB | - |
|  | 20 | 3 |  | 0.00 | L2 | SB |  | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - |  | L2 |  | SB | - |
|  | 21 | 4 |  | 0.00 | L2 | SB |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. | O | JRU | - |  | L2 |  | SB | - |
|  | 22 | 5 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 23 | 6 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 24 | 7 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  | SB | - |
|  | 25 | 8 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 26 | 9 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 27 | 10 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 28 | 11 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 29 | 12 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 50: Following D2, if the stored position is "unknown", the status of the ERTMS/ETCS Level and of the RBC ID/Phone Number information is invalidated. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 | 9 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Train Accepted" (NID_MESSAGE=41) is received | I | RTM | - |  | L2 |  | SB | - |
|  | 31 | 10 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=41) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 32 | 11 |  | 0.00 | L2 | SB |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded | O | JRU | If the status of the Train Position is invalid, it is set to unknown |  | L2 |  | SB | - |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 10: The driver closes the desk during the Start of Mission |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34 | 1 |  | 0.00 | L2 | SB |  | Cab not active | I | TIU | The driver closes the desk. |  | L2 |  | SB | - |
|  | 35 | 2 |  | 0.00 | L2 | SB |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_A_STATUS=0) is recorded | O | JRU | Closing of desk is RECORDED on the JRU. |  | L2 |  | SB | - |
|  | 36 | 3 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted | O | RTM | The on-board equipment detects the desk closed condition that requires the termination of the communication session. |  | L2 |  | SB | - |
|  | 37 | 4 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 38 | 5 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received | I | RTM | The on-board equipment considers the communication session terminated. |  | L2 |  | SB | - |
|  | 39 | 6 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 40 | 7 |  | 0.00 | L2 | SB |  | SA-DISCONNECT.Request is transmitted | O | RTM | The train tries to release the safe connection. |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 26: Within the scope of the Start of Mission procedure in SB mode, the procedure ends as soon as the desk is closed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 | 4 |  | 0.00 | L2 | SB |  | The driver opens the desk | I | TIU | - |  | L2 |  | SB | - |
|  | 45 | 5 |  | 0.00 | L2 | SB |  | 'CAB STATUS' open is RECORDED | O | JRU | - |  | L2 |  | SB | - |
|  | 46 | 6 |  | 0.00 | L2 | SB |  | The on-board equipment requires the driver to enter a Driver ID | O | DMI | The SoM procedure starts from the beginning |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 9: The on-board equipment allows the driver to modify an existing Driver ID before validating it within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is INVALID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 | 1 |  | 0.00 | L2 | SB |  | There is an initial Driver ID displayed. | O | DMI | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  | L2 |  | SB | - |
|  | 57 | 2 |  | 0.00 | L2 | SB |  | The driver modifies the Driver ID (e.g. a new valid data is input). | I | DMI | The on-board equipment allows the driver to modify an existing Driver ID. The new Driver ID must be "1234567890azbycx". |  | L2 |  | SB | - |
|  | 58 | 3 |  | 0.00 | L2 | SB |  | The modified Driver ID is displayed on the DMI | O | DMI | The on-board equipment has clearly authorised the driver to enter a new Driver ID. |  | L2 |  | SB | - |
|  | 59 | 4 |  | 0.00 | L2 | SB |  | The driver validates the modified Driver ID. | I | DMI | The on-board equipment authorises the driver to validate a modified Driver ID. |  | L2 |  | SB | - |
|  | 60 | 5 |  | 0.00 | L2 | SB |  | The new Driver ID entered by the driver is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. | O | JRU | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 61 | 1 |  | 0.00 | L2 | SB |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" | I | DMI | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The driver selects L2. |  | L2 |  | SB | - |
|  | 62 | 2 |  | 0.00 | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 63 | 3 |  | 0.00 | L2 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 39: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is 'invalid' or "valid". |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 | 1 |  | 0.00 | L2 | SB |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. | O | DMI | - |  | L2 |  | SB | - |
|  | 66 | 2 |  | 0.00 | L2 | SB |  | The driver presses the ‘Enter RBC data’ button | I | DMI | - |  | L2 |  | SB | - |
|  | 67 | 3 |  | 0.00 | L2 | SB |  | The on-board equipment displays the RBC data window with an initial value for the RBC ID/Phone number (since the status of this information is INVALID). | O | DMI | - |  | L2 |  | SB | - |
|  | 68 | 4 |  | 0.00 | L2 | SB |  | The driver is allowed to validate the RBC data (the ‘Yes’ button related to the ‘RBC data entry complete?’ question is enabled). | O | DMI | - |  | L2 |  | SB | - |
|  | 69 | 5 |  | 0.00 | L2 | SB |  | The driver validates the RBC data (presses the ‘Yes’ button related to the ‘RBC data entry complete?’ question). | I | DMI | - |  | L2 |  | SB | - |
|  | 70 | 6 |  | 0.00 | L2 | SB |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=2) is recorded | O | JRU | The variables NID_C, NID_RBC and NID_RADIO are identical to the data entered by the driver. |  | L2 |  | SB | - |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 | 1 |  | 0.00 | L2 | SB |  | SA-CONNECT.Request is transmitted | O | RTM | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  | L2 |  | SB | - |
|  | 73 | 2 |  | 0.00 | L2 | SB |  | SA-CONNECT.Confirm is received | I | RTM | Setting up of a safe connection is successful. |  | L2 |  | SB | - |
|  | 74 | 3 |  | 0.00 | L2 | SB |  | The Status symbol "Safe radio connection - Connection Up" is displayed | O | DMI | - |  | L2 |  | SB | - |
|  | 75 | 4 |  | 0.00 | L2 | SB |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. | O | JRU | - |  | L2 |  | SB | - |
|  | 76 | 5 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 77 | 6 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 78 | 7 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received | I | RTM | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  | L2 |  | SB | - |
|  | 79 | 8 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 80 | 9 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 81 | 10 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 48: On-board reports unknown position and RBC accepts the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 | 3 |  | 0.00 | L2 | SB |  | Onboard reports unknown position to RBC | O | RTM | - |  | L2 |  | SB | - |
|  | 85 | 4 |  | 0.00 | L2 | SB |  | SoM position report RECORDED | O | JRU | - |  | L2 |  | SB | - |
|  | 86 | 5 |  | 0.00 | L2 | SB |  | RBC is Not able to confirm position (D33), it accepts the train (D22), it reports train accepted (A23) | I | RTM | message 41, with NID_LRBG set to unknown |  | L2 |  | SB | - |
|  | 87 | 6 |  | 0.00 | L2 | SB |  | message 41, with NID_LRBG set to unknown RECORDED | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 | 1 |  | 0.00 | L2 | SB |  | The driver can select either SH or NL or ‘Train Data Entry’ | O | DMI | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  | L2 |  | SB | - |
|  | 89 | 2 |  | 0.00 | L2 | SB |  | The driver selects ‘Train Data Entry’ | I | DMI | - |  | L2 |  | SB | - |
|  | 90 | 3 |  | 0.00 | L2 | SB |  | A Train data window is is displayed on the DMI. | O | DMI | - |  | L2 |  | SB | - |
|  | 91 | 4 |  | 0.00 | L2 | SB |  | The driver’s selection is recorded on the JRU. | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 93 | 1 |  | 0.00 | L2 | SB |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. | O | DMI | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  | L2 |  | SB | - |
|  | 94 | 2 |  | 0.00 | L2 | SB |  | The driver enters each Train data unknown on-board | I | DMI | - |  | L2 |  | SB | - |
|  | 95 | 3 |  | 0.00 | L2 | SB |  | The driver is allowed to validate the train data | O | DMI | A complete set of valid Train Data is available |  | L2 |  | SB | - |
|  | 96 | 4 |  | 0.00 | L2 | SB |  | The driver validates the train data. | I | DMI | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  | L2 |  | SB | - |
|  | 97 | 5 |  | 0.00 | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Triggered by : The driver validation. |  | L2 |  | SB | - |
|  | 98 | 6 |  | 0.00 | L2 | SB |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded | O | JRU | Triggered by : Data Entry completed |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 | 1 |  | 0.00 | L2 | SB |  | The driver is requested to enter the Train running number. | O | DMI | - |  | L2 |  | SB | - |
|  | 101 | 2 |  | 0.00 | L2 | SB |  | The driver enters the "Train Running Number" [MAIN] | I | DMI | Entered Train Running Number: NID_OPERATIONAL_1 |  | L2 |  | SB | - |
|  | 102 | 3 |  | 0.00 | L2 | SB |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 4: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 2/3 (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. Radio communication session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 | 3 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted | O | RTM | - |  | L2 |  | SB | - |
|  | 104 | 4 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 105 | 5 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received | I | RTM | - |  | L2 |  | SB | - |
|  | 106 | 6 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 107 | 7 |  | 0.00 | L2 | SB |  | The DMI allows the driver to select ‘START’. | O | DMI | - |  | L2 |  | SB | - |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 17: Within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 2 or 3: Driver presses start, an MA request is sent and the On-board receives an SR authorisation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 | 1 |  | 0.00 | L2 | SB |  | T = t’ The driver selects 'Start'. | I | DMI | - |  | L2 |  | SB | - |
|  | 109 | 2 |  | 0.00 | L2 | SB |  | Driver’s selection of 'Start' is recorded on the JRU. | O | JRU | - |  | L2 |  | SB | - |
|  | 110 | 3 |  | 0.00 | L2 | SB |  | SA-DATA.Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted | O | RTM | Packet 1 must be used if position report is based on 2 BG |  | L2 |  | SB | - |
|  | 111 | 4 |  | 0.00 | L2 | SB |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=132; NID_PACKET=0) is recorded | O | JRU | Packet 1 must be used if position report is based on 2 BG |  | L2 |  | SB | - |
|  | 112 | 5 |  | 0.00 | L2 | SB |  | SA-DATA.Indication with Euroradio Message "SR Authorisation" (NID_MESSAGE=2) is received | I | RTM | - |  | L2 |  | SB | - |
|  | 113 | 6 |  | 0.00 | L2 | SB |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=2) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 114 | 7 |  | 0.00 | L2 | SB |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI | O | DMI | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode It shall also be checked that there is no more request to the RBC for a MA. |  | L2 |  | SB | - |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 50: Acknowledgement for SR mode is requested in L2/ L3 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 116 | 1 |  | 0.00 | L2 | SB |  | The driver acknowledges 'Staff Responsible' | I | DMI | Symbol MO10 |  | L2 |  | SB | - |
|  | 117 | 2 |  | 0.00 | L2 | SB |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded | O | JRU | - |  | L2 |  | SB | - |
|  | 118 | 3 |  | 0.00 | L2 | SB |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 119 | 4 |  | 0.00 | L2 | SR |  | The Mode symbol 'Staff Responsible' is displayed | O | DMI | Symbol MO09 |  | L2 |  | SR | - |
|  | 120 | 5 |  | 0.00 | L2 | SR |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 3060100: 3.6.1: Location Principles and Train Position and Train Orientation Test Case 7: The driver acknowledges SR mode in L2 or L3, with connection to the RBC established. The train overpasses unlinked Balise Groups which should not be used for location determination. When the train overpasses a Balise Group which is marked as linked the train must use it as reference location. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 121 | 2 |  | 0.00 | L2 | SR |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | NID_LRBG = 16777215 (UNKNOWN) |  | L2 |  | SR | - |
|  | 122 | 3 |  | 0.00 | L2 | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 29: When the train is NOT at standstill, the driver is NOT allowed by the on-board equipment to modify the current Driver ID when such modification is NOT permitted (national value M_NVDERUN set to 0). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 | 1 |  | 0.00 | L2 | SR |  | Train is moving | I | INT | - |  | L2 |  | SR | - |
|  | Feature 3060100: 3.6.1: Location Principles and Train Position and Train Orientation Test Case 7: The driver acknowledges SR mode in L2 or L3, with connection to the RBC established. The train overpasses unlinked Balise Groups which should not be used for location determination. When the train overpasses a Balise Group which is marked as linked the train must use it as reference location. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 126 | 5 |  | 50.00 | L2 | SR |  | One Balise Group message (Q_LINK=0) containing packet 255 is received | I | BTM | A unlinked BG is passed _____ NID_C=64 and NID_BG=1 |  | L2 |  | SR | - |
|  | 127 | 6 |  | 50.00 | L2 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 128 | 7 |  | 50.00 | L2 | SR |  | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | - |  | L2 |  | SR | - |
|  | 129 | 8 |  | 100.00 | L2 | SR |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24; NID_LRBG=16777215) containing packet 72 (Q_DIR=2) is received | I | RTM | NID_LRBG = 16777215 (UNKNOWN); X_TEXT = ‘Test is OK’ |  | L2 |  | SR | - |
|  | 130 | 9 |  | 100.00 | L2 | SR |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 131 | 10 |  | 100.00 | L2 | SR |  | The "plain text message" is displayed | O | DMI | X_TEXT is displayed |  | L2 |  | SR | - |
|  | 132 | 11 |  | 100.00 | L2 | SR |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 133 | 12 |  | 150.00 | L2 | SR |  | One Balise Group message (Q_LINK=1) containing packet 255 is received | I | BTM | A BG marked as linked is passed _____ NID_C=64 and NID_BG=2 |  | L2 |  | SR | - |
|  | 134 | 13 |  | 150.00 | L2 | SR |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | 135 | 14 |  | 150.00 | L2 | SR |  | SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted | O | RTM | NID_LRBG = LRBG of the passed BG _____ NID_LRBG=1048578 (NID_C=64 and NID_BG=2) |  | L2 |  | SR | - |
|  | 136 | 15 |  | 150.00 | L2 | SR |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - |  | L2 |  | SR | - |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 137 | 1 |  | 200.00 | L2 | SR |  | The ERTMS/ETCS on-board unit is powered down | I | INT | - _____ The train reaches standstill. |  | L2 |  | NP | - |
|  | 138 | 2 |  | 200.00 | L2 | NP |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded | O | JRU | There are no messages recorded in the JRU during a period of time |  | L2 |  | NP | - |
| Step 13: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  | Event Name |  |  |  | Data |  |  |  |  | Delay |  |  |  |  |
| 29 |  |  | Request Change Level |  |  |  | LEVEL=2 |  |  |  |  | 10 s |  |  |  |  |

| - Step 22 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 24 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 26 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 28 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 00 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 0 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |

| - Step 30 : Message 41 - Train Accepted (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 157 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 41 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |

| - Step 36 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 38 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 0.00 m Delay : 10.00 s Conditional Message Id : 156 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |

| Step 61: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=2 | 10 s |

| - Step 76 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 78 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 80 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 84 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 10 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |

| - Step 86 : Message 41 - Train Accepted (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 157 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 41 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |

| - Step 103 : Message 129 - Validated Train Data (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 36 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 0 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 96 d |  |  |
| NC_CDTRAIN | 4 | 0 d |  |  |
| NC_TRAIN | 15 | 0 d |  |  |
| L_TRAIN | 12 | 0 d |  |  |
| V_MAXTRAIN | 7 | 0 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 0 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 105 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 0.00 m Delay : 40.00 s Conditional Message Id : 129 Back Delay : 40.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 110 : Message 132 - MA Request (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_MARQSTREASON | 5 | 1 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |

| - Step 112 : Message 2 - SR Authorisation (RBC n? 1) - Distance 0.00 m Delay : 10.00 s Conditional Message Id : 132 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 2 d |  |  |
| L_MESSAGE | 10 | 12 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_SR | 15 | 10000 d |  |  |

| - Step 121 : Message 136 - Train Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |

| - Step 126 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 126 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 129 : Message 24 - General Message (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 31 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 10 b |  |  |
| L_PACKET | 13 | 172 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 10 d |  |  |
| X_TEXT | 8 | T |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | O |  |  |
| X_TEXT | 8 | K |  |  |

| - Step 133 : Balise Group BG 2 d 1/2 - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 133 : Balise Group BG 2 d 2/2 - Distance 155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Message 136 - Train Position Report (RBC n? 1) - Distance 150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |