| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5190200_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5190200_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:58 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Req 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip)

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 32: Indication of Level 1 is displayed to the driver while the current Level is 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 0.00 |  | L1 |  | SR |  |  | Permanent Indication of Level 1 to the driver |  | O |  | DMI |  | The level indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 5: Texts displayed on DMI, fixed text messages and the data entry dialogs are displayed in language selected by the driver |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver selects "Settings" [MAIN] |  | I |  | DMI |  | Language=<LANGUAGE_A> |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 2 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver modifies "Language" |  | I |  | DMI |  | Selection of <LANGUAGE_B> |  |  | L1 |  |  | SR |  | - |  |
|  | 42 |  | 3 |  |  | 0.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 43 |  | 4 |  |  | 0.00 |  | L1 |  | SR |  |  | The Settings window is displayed |  | O |  | DMI |  | All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_B> chosen by the driver SRS 3.12.3.1.4.1 Note : In case of plain text messages the trackside selects the language in which the message is displayed. |  |  | L1 |  |  | SR |  | - |  |
|  | 44 |  | 5 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver modifies "Language" |  | I |  | DMI |  | Selection of <LANGUAGE_A> |  |  | L1 |  |  | SR |  | - |  |
|  | 45 |  | 6 |  |  | 0.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=29) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 7 |  |  | 0.00 |  | L1 |  | SR |  |  | The Settings window is displayed |  | O |  | DMI |  | All labels, all FIXED text messages and ALL data entry dialogs are displayed in <LANGUAGE_A> chosen by the driver SRS 3.12.3.1.4.1 Note : In case of plain text messages the trackside selects the language in which the message is displayed. |  |  | L1 |  |  | SR |  | - |  |
|  | 47 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver closes the "Settings" window |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 6: When the train is at standstill, the on-board equipment allows the driver to modify the current Driver ID at any time. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver selects "Main" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 49 |  | 2 |  |  | 0.00 |  | L1 |  | SR |  |  | The "Driver ID" button is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 50 |  | 3 |  |  | 0.00 |  | L1 |  | SR |  |  | The driver enters the "Driver ID" |  | I |  | DMI |  | Entered Driver ID: DRIVER_ID_1 (different from DRIVER_ID_0) |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 4 |  |  | 0.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; DRIVER_ID=<DRIVER_ID_1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 1: Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  | I |  | BTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 55 |  | 4 |  |  | 50.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 56 |  | 5 |  |  | 50.00 |  | L1 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 59 |  | 8 |  |  | 50.00 |  | L1 |  | LS |  |  | Driver acknowledges within 5 sec after the change to LS |  | I |  | DMI |  | The driver acknowledgement time (refer to Appendix A3.1) |  |  | L1 |  |  | LS |  | - |  |
|  | 60 |  | 9 |  |  | 50.00 |  | L1 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 61 |  | 10 |  |  | 50.00 |  | L1 |  | LS |  |  | Service brake is NOT APPLIED |  | O |  | TIU |  | Negative Testing |  |  | L1 |  |  | LS |  | - |  |
|  | 62 |  | 11 |  |  | 50.00 |  | L1 |  | LS |  |  | NO service Brake Indication is DISPLAYED |  | O |  | DMI |  | Negative Testing |  |  | L1 |  |  | LS |  | - |  |
|  | 63 |  | 12 |  |  | 50.00 |  | L1 |  | LS |  |  | NO Service Brake Intervention is RECORDED |  | O |  | JRU |  | Negative Testing |  |  | L1 |  |  | LS |  | - |  |
|  | 64 |  | 13 |  |  | 50.00 |  | L1 |  | LS |  |  | LS Mode indication is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 7: Limited Supervision mode is permanently displayed to the driver when the on-board equipment is in Limited Supervision mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 50.00 |  | L1 |  | LS |  |  | Permanent Indication of LS mode to the driver |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 7: In normal operation during a mission the driver has no possibility to modify the current ERTMS/ETCS Level while the train is running. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 1 |  |  | 150.00 |  | L1 |  | LS |  |  | The driver selects "Main" |  | I |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 67 |  | 2 |  |  | 150.00 |  | L1 |  | LS |  |  | The "Level" button is not enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 2: The on-board equipment is already in LS mode, when receiving the LS mode profile. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 68 |  | 1 |  |  | 350.00 |  | L1 |  | LS |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  | I |  | BTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) |  |  | L1 |  |  | LS |  | - |  |
|  | 70 |  | 3 |  |  | 350.00 |  | L1 |  | LS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 71 |  | 4 |  |  | 350.00 |  | L1 |  | LS |  |  | NO Driver acknowledgement is requested |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 72 |  | 5 |  |  | 350.00 |  | L1 |  | LS |  |  | LS Mode indication is still displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 73 |  | 6 |  |  | 550.00 |  | L1 |  | LS |  |  | The length of LS area is updated |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  |  | 700.00 |  | L1 |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - _____ The train reached the end of L_MAMODE. The mode changes to FS |  |  | L1 |  |  | FS |  | - |  |
|  | 75 |  | 2 |  |  | 700.00 |  | L1 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  | 1 |  |  | 700.00 |  | L1 |  | FS |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L1 |  |  | FS |  | - |  |
|  | 77 |  | 2 |  |  | 700.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3040400: 3.4.4: Linking Test Case 4: A telegram including packet 5 Linking Information announcing a repositioning balise group BG2 is transmitted via balise group BG1 and is received by the on-board equipment. A balise telegram containing repositioning information is received afterwards from BG2 and accepted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 700.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1) containing packet 5 (NID_BG=16383) is received |  | I |  | BTM |  | - _____ In this sequence, NID_BG=4 in packet 5. |  |  | L1 |  |  | FS |  | - |  |
|  | 79 |  | 2 |  |  | 700.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 81 |  | 3 |  |  | 700.00 |  | L1 |  | FS |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | FS |  | - |  |
|  | 82 |  | 4 |  |  | 700.00 |  | L2 |  | FS |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 83 |  | 3 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | FS |  | - |  |
|  | 84 |  | 4 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 85 |  | 5 |  |  | 700.00 |  | L2 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 86 |  | 6 |  |  | 700.00 |  | L2 |  | FS |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 87 |  | 7 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 88 |  | 8 |  |  | 700.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 9 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 10 |  |  | 700.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 91 |  | 11 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 12 |  |  | 700.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 94 |  | 1 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | FS |  | - |  |
|  | 95 |  | 2 |  |  | 700.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 96 |  | 3 |  |  | 700.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 97 |  | 4 |  |  | 700.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 31: There is a correct cab signalling displayed to the driver when the current mode is FS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 98 |  | 1 |  |  | 700.00 |  | L2 |  | FS |  |  | In FS mode and Level 1/2/3 there is a permanent and correct cab signalling displayed which is: Permitted speed is displayed always Target speed is displayed always Target distance is displayed always Release speed is displayed if the conditions to be displayed are fulfilled |  | O |  | DMI |  | The speed and distance indications may not be permanent if the driver uses the menus. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 3: Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. Overspeed Case. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 2 |  |  | 1200.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received |  | I |  | RTM |  | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train |  |  | L2 |  |  | FS |  | - |  |
|  | 101 |  | 3 |  |  | 1200.00 |  | L2 |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 102 |  | 4 |  |  | 1200.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 103 |  | 5 |  |  | 1200.00 |  | L2 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 104 |  | 6 |  |  | 1200.00 |  | L2 |  | LS |  |  | The on-board informs trackside about mode change Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 105 |  | 7 |  |  | 1200.00 |  | L2 |  | LS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 106 |  | 8 |  |  | 1200.00 |  | L2 |  | LS |  |  | Service brake is APPLIED |  | O |  | TIU |  | Due to over-speeding |  |  | L2 |  |  | LS |  | - |  |
|  | 107 |  | 9 |  |  | 1200.00 |  | L2 |  | LS |  |  | Service Brake Indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 108 |  | 10 |  |  | 1200.00 |  | L2 |  | LS |  |  | Service Brake Intervention is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 109 |  | 11 |  |  | 1200.00 |  | L2 |  | LS |  |  | Driver acknowledges within 5 sec after the change to LS |  | I |  | DMI |  | The driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | LS |  | - |  |
|  | 110 |  | 12 |  |  | 1200.00 |  | L2 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 111 |  | 13 |  |  | 1360.00 |  | L2 |  | LS |  |  | Service Brake is RELEASED |  | O |  | TIU |  | The speed becomes lower than or equal to permitted speed in LS mode |  |  | L2 |  |  | LS |  | - |  |
|  | 112 |  | 14 |  |  | 1360.00 |  | L2 |  | LS |  |  | Service Brake Indication is REMOVED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 113 |  | 15 |  |  | 1360.00 |  | L2 |  | LS |  |  | Service Brake Release is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 114 |  | 16 |  |  | 1360.00 |  | L2 |  | LS |  |  | LS mode indication is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 1 |  |  | 1500.00 |  | L2 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 116 |  | 2 |  |  | 1500.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The mode change to FS is stored. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 4: Limited Supervision is requested for the current location and the driver doesn’t acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 2 |  |  | 1600.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received |  | I |  | RTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train |  |  | L2 |  |  | FS |  | - |  |
|  | 119 |  | 3 |  |  | 1600.00 |  | L2 |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 120 |  | 4 |  |  | 1600.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 121 |  | 5 |  |  | 1600.00 |  | L2 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 122 |  | 6 |  |  | 1600.00 |  | L2 |  | LS |  |  | The on-board informs trackside about mode change Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 123 |  | 7 |  |  | 1600.00 |  | L2 |  | LS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 124 |  | 8 |  |  | 1600.00 |  | L2 |  | LS |  |  | Driver doesn’t acknowledges within 5 sec after the change to LS |  | I |  | DMI |  | the driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | LS |  | - |  |
|  | 125 |  | 9 |  |  | 1710.00 |  | L2 |  | LS |  |  | Service Brake is APPLIED |  | O |  | TIU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 126 |  | 10 |  |  | 1710.00 |  | L2 |  | LS |  |  | Service Brake Indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 127 |  | 11 |  |  | 1710.00 |  | L2 |  | LS |  |  | Service Brake Intervention is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 128 |  | 12 |  |  | 1850.00 |  | L2 |  | LS |  |  | Driver acknowledges more than 5 sec after the change to LS |  | I |  | DMI |  | the driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | LS |  | - |  |
|  | 129 |  | 13 |  |  | 1850.00 |  | L2 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 130 |  | 14 |  |  | 1850.00 |  | L2 |  | LS |  |  | Service Brake is RELEASED |  | O |  | TIU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 131 |  | 15 |  |  | 1850.00 |  | L2 |  | LS |  |  | Service Brake Indication is REMOVED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 132 |  | 16 |  |  | 1850.00 |  | L2 |  | LS |  |  | Service Brake Release is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 133 |  | 17 |  |  | 1850.00 |  | L2 |  | LS |  |  | LS mode indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 |  | 1 |  |  | 1950.00 |  | L2 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 135 |  | 2 |  |  | 1950.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The mode change to FS is stored. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 5: Limited Supervision is requested for the current location and the driver doesn’t acknowledges |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 137 |  | 2 |  |  | 2000.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received |  | I |  | RTM |  | One of the steps 2 or 3 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train |  |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 3 |  |  | 2000.00 |  | L2 |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 139 |  | 4 |  |  | 2000.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 140 |  | 5 |  |  | 2000.00 |  | L2 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 141 |  | 6 |  |  | 2000.00 |  | L2 |  | LS |  |  | The on-board informs trackside about mode change Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 142 |  | 7 |  |  | 2000.00 |  | L2 |  | LS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 143 |  | 8 |  |  | 2130.00 |  | L2 |  | LS |  |  | DRIVER NEVER ACKNOWLEDGES |  | I |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 144 |  | 9 |  |  | 2130.00 |  | L2 |  | LS |  |  | Service Brake is APPLIED |  | O |  | TIU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 145 |  | 10 |  |  | 2130.00 |  | L2 |  | LS |  |  | Service Brake Indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 146 |  | 11 |  |  | 2130.00 |  | L2 |  | LS |  |  | Service Brake Intervention is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 147 |  | 12 |  |  | 2290.00 |  | L2 |  | LS |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 148 |  | 13 |  |  | 2290.00 |  | L2 |  | LS |  |  | Inform Trackside about standstill Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 149 |  | 14 |  |  | 2290.00 |  | L2 |  | LS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 150 |  | 15 |  |  | 2290.00 |  | L2 |  | LS |  |  | Driver acknowledges the change to LS |  | I |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 151 |  | 16 |  |  | 2290.00 |  | L2 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 152 |  | 17 |  |  | 2290.00 |  | L2 |  | LS |  |  | Service Brake is RELEASED |  | O |  | TIU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 153 |  | 18 |  |  | 2290.00 |  | L2 |  | LS |  |  | Service Brake Indication is REMOVED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 154 |  | 19 |  |  | 2290.00 |  | L2 |  | LS |  |  | Service Brake Release is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 155 |  | 20 |  |  | 2290.00 |  | L2 |  | LS |  |  | LS mode indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 3060400: 3.6.4: Train Position Confidence Interval Test Case 7: The train passes Balise Group BGa. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 159 |  | 4 |  |  | 2400.00 |  | L2 |  | LS |  |  | One Balise Group message (N_TOTAL=1; Q_LINK=1) containing packet 255 is received |  | I |  | BTM |  | Packets inside telegram are not relevant. _____ To reset the odometry. |  |  | L2 |  |  | LS |  | - |  |
|  | 160 |  | 5 |  |  | 2400.00 |  | L2 |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 5190600: 5.19.6: Exit of Limited Supervision mode Test Case 1: The train is in LS mode and it reaches the end of the LS area (The end of the LS area is located before the EOA/LOA of the MA). The system switches to Full Supervision Mode immediately |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 162 |  | 1 |  |  | 2400.00 |  | L2 |  | LS |  |  | The Mode symbol "Full Supervision" is displayed |  | O |  | DMI |  | When the train reaches the end of the LS area with its min. safe front end. The on-board equipment switches to FS mode. The driver is informed on the DMI of the change to FS mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 163 |  | 2 |  |  | 2400.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The mode change to FS is stored. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5090200: 5.9.2: On Sight is requested for current location (from modes different from Stand By and Post Trip) Test Case 1: On Sight is requested for the current location and the driver doesn’t acknowledges |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 2 |  |  | 2450.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=0) is received |  | I |  | RTM |  | One of the steps 1 or 2 will be chosen for testing The train receives a message including a packet 80 Mode profile Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train |  |  | L2 |  |  | FS |  | - |  |
|  | 166 |  | 3 |  |  | 2450.00 |  | L2 |  | FS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 167 |  | 4 |  |  | 2450.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board equipment shall switch immediately to OS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 168 |  | 5 |  |  | 2450.00 |  | L2 |  | OS |  |  | Mode transition to OS is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 169 |  | 6 |  |  | 2450.00 |  | L2 |  | OS |  |  | The on-board informs trackside about mode change Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 170 |  | 7 |  |  | 2450.00 |  | L2 |  | OS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 171 |  | 8 |  |  | 2450.00 |  | L2 |  | OS |  |  | Driver doesn’t acknowledges within 5 sec after the change to OS |  | I |  | DMI |  | the driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | OS |  | - |  |
|  | 172 |  | 9 |  |  | 2450.00 |  | L2 |  | OS |  |  | Service Brake is APPLIED |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 173 |  | 10 |  |  | 2450.00 |  | L2 |  | OS |  |  | Service Brake Indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 174 |  | 11 |  |  | 2450.00 |  | L2 |  | OS |  |  | Service Brake Intervention is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 175 |  | 12 |  |  | 2460.00 |  | L2 |  | OS |  |  | Inform Trackside about standstill Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. As soon as the train reaches standstill |  |  | L2 |  |  | OS |  | - |  |
|  | 176 |  | 13 |  |  | 2460.00 |  | L2 |  | OS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | OS |  | - |  |
|  | 177 |  | 14 |  |  | 2460.00 |  | L2 |  | OS |  |  | Driver acknowledges the change to OS |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 178 |  | 15 |  |  | 2460.00 |  | L2 |  | OS |  |  | Driver Acknowledgement of transition to OS mode is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 179 |  | 16 |  |  | 2460.00 |  | L2 |  | OS |  |  | Service Brake is RELEASED |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 180 |  | 17 |  |  | 2460.00 |  | L2 |  | OS |  |  | Service Brake Indication is REMOVED |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 181 |  | 18 |  |  | 2460.00 |  | L2 |  | OS |  |  | Service Brake Release is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 182 |  | 19 |  |  | 2460.00 |  | L2 |  | OS |  |  | OS mode indication is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 30: There is a correct cab signalling displayed to the driver when the current mode is OS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 183 |  | 1 |  |  | 2460.00 |  | L2 |  | OS |  |  | The Train Speed is displayed. There is no indication to the driver, until request it, of Permitted speed, Target speed, Target distance, Release speed, Gradient, MRSP, First Indication location and EOA/LOA. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 184 |  | 2 |  |  | 2460.00 |  | L2 |  | OS |  |  | The driver requests to see the supervision limits in OS mode |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 185 |  | 3 |  |  | 2460.00 |  | L2 |  | OS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1011) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 186 |  | 4 |  |  | 2460.00 |  | L2 |  | OS |  |  | The correct cab signalling driver request in OS mode is displayed: Train Speed, Permitted speed, Target speed, Target distance, Release speed Gradient, MRSP, First Indication location, EOA/LOA |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 187 |  | 5 |  |  | 2460.00 |  | L2 |  | OS |  |  | The driver requests to hide supervision limits |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 188 |  | 6 |  |  | 2460.00 |  | L2 |  | OS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 1001) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 189 |  | 7 |  |  | 2460.00 |  | L2 |  | OS |  |  | The Train Speed is displayed. There is no indication to the driver of Permitted speed, Target speed, Target distance, Release speed, Gradient, MRSP, First Indication location and EOA/LOA. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 3: On-board supervision of on sight speed defined by trackside. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 190 |  | 1 |  |  | 2460.00 |  | L2 |  | OS |  |  | Driver requests display of Permitted speed at Driver’s Request |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 191 |  | 2 |  |  | 2460.00 |  | L2 |  | OS |  |  | Driver request is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 192 |  | 3 |  |  | 2460.00 |  | L2 |  | OS |  |  | Permitted speed is DISPLAYED on the DMI on Driver’s Request |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 193 |  | 4 |  |  | 2460.00 |  | L2 |  | OS |  |  | The ERTMS/ETCS on-board equipment supervises train speed versus a mode related speed restriction given by trackside (V_MAMODE). |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 4: On-board supervision of on sight speed defined by national / default value. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 196 |  | 1 |  |  | 2750.00 |  | L2 |  | OS |  |  | Train is moving (V_TRAIN=<V_NVONSIGHT>) |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 199 |  | 4 |  |  | 2750.00 |  | L2 |  | OS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of permitted speed |  |  | L2 |  |  | OS |  | - |  |
|  | 200 |  | 5 |  |  | 2750.00 |  | L2 |  | OS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 201 |  | 6 |  |  | 2800.00 |  | L2 |  | OS |  |  | Train is moving (V_TRAIN=<V_NVONSIGHT+10>) |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 202 |  | 7 |  |  | 2800.00 |  | L2 |  | OS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | overspeeding |  |  | L2 |  |  | OS |  | - |  |
|  | 203 |  | 8 |  |  | 2800.00 |  | L2 |  | OS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 204 |  | 9 |  |  | 2800.00 |  | L2 |  | OS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 205 |  | 10 |  |  | 2800.00 |  | L2 |  | OS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is displayed |  | O |  | DMI |  | Indication of warning due to over-speeding |  |  | L2 |  |  | OS |  | - |  |
|  | 206 |  | 11 |  |  | 2800.00 |  | L2 |  | OS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 207 |  | 12 |  |  | 2860.00 |  | L2 |  | OS |  |  | Train is moving (V_TRAIN=<V_NVONSIGHT>) |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 208 |  | 13 |  |  | 2900.00 |  | L2 |  | OS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 209 |  | 14 |  |  | 2900.00 |  | L2 |  | OS |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 210 |  | 15 |  |  | 2900.00 |  | L2 |  | OS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 211 |  | 16 |  |  | 2900.00 |  | L2 |  | OS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Indication of permitted speed |  |  | L2 |  |  | OS |  | - |  |
|  | 212 |  | 17 |  |  | 2900.00 |  | L2 |  | OS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 6: Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 214 |  | 2 |  |  | 2915.00 |  | L2 |  | OS |  |  | SA-DATA.Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE=2) is received |  | I |  | RTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) |  |  | L2 |  |  | OS |  | - |  |
|  | 215 |  | 3 |  |  | 2915.00 |  | L2 |  | OS |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 216 |  | 4 |  |  | 2915.00 |  | L2 |  | OS |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 217 |  | 5 |  |  | 2915.00 |  | L2 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 218 |  | 6 |  |  | 2915.00 |  | L2 |  | LS |  |  | The on-board informs trackside about mode change Message 136: 'Train Position Report' is SENT. |  | O |  | RTM |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 219 |  | 7 |  |  | 2915.00 |  | L2 |  | LS |  |  | Message 136: 'Train position report' is RECORDED. |  | O |  | JRU |  | Only in L2 / L3. |  |  | L2 |  |  | LS |  | - |  |
|  | 220 |  | 8 |  |  | 2915.00 |  | L2 |  | LS |  |  | Driver acknowledges within 5 sec after the change to LS |  | I |  | DMI |  | The driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | LS |  | - |  |
|  | 221 |  | 9 |  |  | 2915.00 |  | L2 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 222 |  | 10 |  |  | 2975.00 |  | L2 |  | LS |  |  | Service brake is NOT APPLIED |  | O |  | TIU |  | Negative Testing |  |  | L2 |  |  | LS |  | - |  |
|  | 223 |  | 11 |  |  | 2975.00 |  | L2 |  | LS |  |  | NO service Brake Indication is DISPLAYED |  | O |  | DMI |  | Negative Testing |  |  | L2 |  |  | LS |  | - |  |
|  | 224 |  | 12 |  |  | 2975.00 |  | L2 |  | LS |  |  | NO Service Brake Intervention is RECORDED |  | O |  | JRU |  | Negative Testing |  |  | L2 |  |  | LS |  | - |  |
|  | 225 |  | 13 |  |  | 2975.00 |  | L2 |  | LS |  |  | LS Mode indication is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 6: On-board equipment allows the driver to select override request only when the speed is 0 Km/h (V_NVALLOWOVTRP: default value) AND Validated Train Data is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 226 |  | 1 |  |  | 2975.00 |  | L2 |  | LS |  |  | Train is moving (V_TRAIN>0) |  | I |  | INT |  | - _____ V_TRAIN=30 (150km/h) is not requested in this sequence. |  |  | L2 |  |  | LS |  | - |  |
|  | 227 |  | 2 |  |  | 2975.00 |  | L2 |  | LS |  |  | The driver presses "Override" |  | I |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 228 |  | 3 |  |  | 2975.00 |  | L2 |  | LS |  |  | The <EOA> button in the <Override> window is disabled |  | O |  | DMI |  | - _____ See TC Bug 1561 |  |  | L2 |  |  | LS |  | - |  |
|  | 229 |  | 4 |  |  | 3150.00 |  | L2 |  | LS |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 230 |  | 5 |  |  | 3150.00 |  | L2 |  | LS |  |  | The <EOA> button in the <Override> window is enabled |  | O |  | DMI |  | As soon as standstill is reached |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 4: The on-board equipment allows the driver to modify the current set of Train Data when the train is NOT in the process of SoM if the train is at standstill. The driver modifies the train category. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 231 |  | 1 |  |  | 3150.00 |  | L2 |  | LS |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - _____ The train is at standstill (V_TRAIN=0) |  |  | L2 |  |  | LS |  | - |  |
|  | 232 |  | 2 |  |  | 3150.00 |  | L2 |  | LS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 233 |  | 3 |  |  | 3150.00 |  | L2 |  | LS |  |  | The 'Train data window' is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 234 |  | 4 |  |  | 3150.00 |  | L2 |  | LS |  |  | The driver modifies the "Train Category" and validates the "Flexible Train Data" |  | I |  | DMI |  | The on-board equipment does not authorize the driver to let the 'Train category' value in blank before the validation of the Train Data. The driver sets the "Train Category" such that: <NC_CDTRAIN_2> different from <NC_CDTRAIN_1> <NC_TRAIN_2> different from <NC_TRAIN_1> |  |  | L2 |  |  | LS |  | - |  |
|  | 235 |  | 5 |  |  | 3150.00 |  | L2 |  | LS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=21) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 236 |  | 6 |  |  | 3150.00 |  | L2 |  | LS |  |  | TRAIN DATA (NID_MESSAGE_JRU=2; NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 237 |  | 7 |  |  | 3150.00 |  | L2 |  | LS |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 (NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is transmitted |  | O |  | RTM |  | Only if a communication session is established |  |  | L2 |  |  | LS |  | - |  |
|  | 238 |  | 8 |  |  | 3150.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129; NID_PACKET=11; NC_CDTRAIN=<NC_CDTRAIN_2>; NC_TRAIN=<NC_TRAIN_2>) is recorded |  | O |  | JRU |  | Only if a communication session is established |  |  | L2 |  |  | LS |  | - |  |
|  | 239 |  | 9 |  |  | 3150.00 |  | L2 |  | LS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | Only if a communication session is established |  |  | L2 |  |  | LS |  | - |  |
|  | 240 |  | 10 |  |  | 3150.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | Only if a communication session is established |  |  | L2 |  |  | LS |  | - |  |
|  | 241 |  | 11 |  |  | 3150.00 |  | L2 |  | LS |  |  | The Main window is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 21: The train is in level 2 or 3 in SR and a new MA, SSP and Gradient Profile is received onboard. No specific mode is required by a Mode Profile. The track description is available at least from the Front End of the train The system switches to FS immediately. Transition Condition Id [31]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 242 |  | 1 |  |  | 3150.00 |  | L2 |  | LS |  |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 is received |  | I |  | RTM |  | The track description does not cover the full length of the train. |  |  | L2 |  |  | LS |  | - |  |
|  | 243 |  | 2 |  |  | 3150.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 249 |  | 1 |  |  | 3300.00 |  | L2 |  | LS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | FS |  | - |  |
|  | 250 |  | 2 |  |  | 3300.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 251 |  | 3 |  |  | 3300.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | FS |  | - |  |
|  | 252 |  | 4 |  |  | 3300.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 253 |  | 5 |  |  | 3300.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | FS |  | - |  |
|  | 254 |  | 6 |  |  | 3300.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | FS |  | - |  |
|  | 255 |  | 7 |  |  | 3300.00 |  | L2 |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4070200: 4.7.2: DMI versus Mode Table - Input Test Case 3: The on-board equipment, when the train is NOT at standstill, allows the driver to modify the train running number and the Driver ID (if allowed by national value). The driver modifies the train running number. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 256 |  | 1 |  |  | 3500.00 |  | L2 |  | FS |  |  | Train is moving (V_TRAIN=<different from zero>) |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 257 |  | 2 |  |  | 3500.00 |  | L2 |  | FS |  |  | The driver selects "Main" |  | I |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 258 |  | 3 |  |  | 3500.00 |  | L2 |  | FS |  |  | The "Driver ID" button is enabled |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 259 |  | 4 |  |  | 3500.00 |  | L2 |  | FS |  |  | The "Train running number" button is enabled |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 260 |  | 5 |  |  | 3500.00 |  | L2 |  | FS |  |  | The driver presses 'Train Running Number' |  | I |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 261 |  | 6 |  |  | 3500.00 |  | L2 |  | FS |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 (different from NID_OPERATIONAL_0) |  |  | L2 |  |  | FS |  | - |  |
|  | 262 |  | 7 |  |  | 3500.00 |  | L2 |  | FS |  |  | The Main Window is DISPLAYED |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 263 |  | 8 |  |  | 3500.00 |  | L2 |  | FS |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=<NID_OPERATIONAL_1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 43: Modification of adhesion factor by driver defined by default national value |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 264 |  | 1 |  |  | 3560.00 |  | L2 |  | FS |  |  | The driver selects 'Special Window' |  | I |  | DMI |  | - _____ The train reached standstill |  |  | L2 |  |  | FS |  | - |  |
|  | 265 |  | 2 |  |  | 3560.00 |  | L2 |  | FS |  |  | The Special Window is DISPLAYED. Label „Adhesion' is DISABLED |  | O |  | DMI |  | Check that „Adhesion' cannot b eselected by driver |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 266 |  | 1 |  |  | 3560.00 |  | L2 |  | FS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 267 |  | 2 |  |  | 3560.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 52 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 24 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 24 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 9 d |  |  |
| L_MAMODE | 15 | 500 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 68 : Balise Group BG 2 d 1/2 - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 24 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 9 d |  |  |
| L_MAMODE | 15 | 350 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 68 : Balise Group BG 2 d 2/2 - Distance 355.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 74 : Balise Group BG 3 d 1/2 - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 011 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 74 : Balise Group BG 3 d 2/2 - Distance 705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 69 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 1700 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 12 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 87 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 89 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 700.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 91 : Message 159 - Session Established (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 94 : Message 129 - Validated Train Data (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 38 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 8 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0000 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 110 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 d |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 96 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 700.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 100 : Message 3 - Movement Authority (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 43 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2500 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 500 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 13 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 104 : Message 136 - Train Position Report (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 500 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 118 : Message 3 - Movement Authority (RBC n? 1) - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 29 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2100 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 900 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 350 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 122 : Message 136 - Train Position Report (RBC n? 1) - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 900 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 137 : Message 3 - Movement Authority (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 43 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 1300 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 400 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 141 : Message 136 - Train Position Report (RBC n? 1) - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 1300 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 148 : Message 136 - Train Position Report (RBC n? 1) - Distance 2290.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 1590 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 159 : Balise Group BG 4 d 1/2 - Distance 2400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 159 : Balise Group BG 4 d 2/2 - Distance 2405.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 255 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 165 : Message 3 - Movement Authority (RBC n? 1) - Distance 2450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 29 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 1750 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 400 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 169 : Message 136 - Train Position Report (RBC n? 1) - Distance 2450.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 1800 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 175 : Message 136 - Train Position Report (RBC n? 1) - Distance 2460.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 1860 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 214 : Message 3 - Movement Authority (RBC n? 1) - Distance 2915.00 m Delay : 2.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 29 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 2050 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 16 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 218 : Message 136 - Train Position Report (RBC n? 1) - Distance 2915.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 2050 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 16 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 237 : Message 129 - Validated Train Data (RBC n? 1) - Distance 3150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 38 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 110 d |  |  |
| NC_CDTRAIN | 4 | 0 d |  |  |
| NC_TRAIN | 15 | 1 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 18 d |  |  |
| M_LOADINGGAUGE | 8 | 00000001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 d |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 239 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 3150.00 m Delay : 60.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 242 : Message 3 - Movement Authority (RBC n? 1) - Distance 3150.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 32 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 120 d |  |  |
| Q_FRONT | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 249 : Message 24 - General Message (RBC n? 1) - Distance 3300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |

| - Step 251 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 3300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 253 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 3300.00 m Delay : 7.00 s Conditional Message Id : 156 Back Delay : 7.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |