| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5080200_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5080200_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 12:11 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Tested Feature: 5.8.2: Selection of Override

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 44 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 31: The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the train exceeds T_NVOVTRP, override status is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 56: Driver selects 'override', train speed is under or equal to the speed limit for triggering the 'override' function and the mode is FS, LS, SB, PT and OS. Transition Condition Id [37]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 50.00 |  | L1 |  | FS |  |  | The driver selects 'Override EoA' |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 49 |  | 2 |  |  | 50.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 50 |  | 3 |  |  | 50.00 |  | L1 |  | FS |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 4 |  |  | 50.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 5 |  |  | 50.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 6 |  |  | 50.00 |  | L1 |  | SR |  |  | The Mode symbol 'Override EOA is active' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 54 |  | 7 |  |  | 50.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 55 |  | 8 |  |  | 50.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 56 |  | 9 |  |  | 50.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The permitted speed V_NVSUPOVTRP is indicated to the driver. |  |  | L1 |  |  | SR |  | - |  |
|  | 57 |  | 10 |  |  | 50.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 31: The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the train exceeds T_NVOVTRP, override status is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 |  | 3 |  |  | 200.00 |  | L1 |  | SR |  |  | As soon as t>to +T_NVOVTRP the override status is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 59 |  | 4 |  |  | 200.00 |  | L1 |  | SR |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 250.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 250.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 250.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 63 |  | 4 |  |  | 250.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 30: The driver selects 'override request', the system switches to SR mode and 'override status' is displayed. As soon as the train exceeds D_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 1 |  |  | 275.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN< V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 56: Driver selects 'override', train speed is under or equal to the speed limit for triggering the 'override' function and the mode is FS, LS, SB, PT and OS. Transition Condition Id [37]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 |  | 1 |  |  | 275.00 |  | L1 |  | FS |  |  | The driver selects 'Override EoA' |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 67 |  | 2 |  |  | 275.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0000 1110) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 68 |  | 3 |  |  | 275.00 |  | L1 |  | FS |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 69 |  | 4 |  |  | 275.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 70 |  | 5 |  |  | 275.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 71 |  | 6 |  |  | 275.00 |  | L1 |  | SR |  |  | The Mode symbol 'Override EOA is active' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 72 |  | 7 |  |  | 275.00 |  | L1 |  | SR |  |  | The driver requests the speed and distance monitoring information to be displayed |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 73 |  | 8 |  |  | 275.00 |  | L1 |  | SR |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 74 |  | 9 |  |  | 275.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The permitted speed V_NVSUPOVTRP is indicated to the driver. |  |  | L1 |  |  | SR |  | - |  |
|  | 75 |  | 10 |  |  | 275.00 |  | L1 |  | SR |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 30: The driver selects 'override request', the system switches to SR mode and 'override status' is displayed. As soon as the train exceeds D_NVOVTRP, 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  | 3 |  |  | 475.00 |  | L1 |  | SR |  |  | The Mode symbol "Override EOA is active" is removed |  | O |  | DMI |  | As soon as d = d0 +D_NVOVTRP, d0 = is the position of triggering the 'EOA override' button |  |  | L1 |  |  | SR |  | - |  |
|  | 77 |  | 4 |  |  | 475.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=0>) is recorded |  | O |  | JRU |  | The cancellation of 'override status' is RECORDED |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 22: Timed Service brake intervention due to over-speeding when Supervised Speed Limit decreases. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 78 |  | 1 |  |  | 625.00 |  | L1 |  | SR |  |  | The driver over-passes the Service Brake Intervention limit (SBI). |  | I |  | INT |  | The driver doesn’t take into account the Warning Indication due to over-speeding. |  |  | L1 |  |  | SR |  | - |  |
|  | 79 |  | 2 |  |  | 625.00 |  | L1 |  | SR |  |  | An indication for driver intervention is given. |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 80 |  | 3 |  |  | 625.00 |  | L1 |  | SR |  |  | The Service Brake is APPLIED. |  | O |  | TIU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 81 |  | 4 |  |  | 625.00 |  | L1 |  | SR |  |  | The Service Brake intervention is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  | 1 |  |  | 725.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 83 |  | 2 |  |  | 725.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 84 |  | 3 |  |  | 725.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 85 |  | 4 |  |  | 725.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 37: Driver selects 'override', train speed is NOT below max. speed for override and the mode is FS, LS, OS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 745.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN> V_NVALLOWOVTRP) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 87 |  | 2 |  |  | 775.00 |  | L1 |  | FS |  |  | The selection of 'override request' is not possible for the driver |  | I |  | DMI |  | No possibility to select "override request". The override procedure can’t start. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 3: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and should be replaced when a new set of NVs is available. A previously received set of National Values not yet applicable should be deleted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 88 |  | 1 |  |  | 875.00 |  | L1 |  | FS |  |  | First message including Packet 3: National Values, Including V_NVALLOWOVTRP (40 km/h) is RECEIVED. |  | I |  | BTM |  | NID_C (000011) is different from initial balise (NID_C = (000001)). The received V_NVALLOWOVTRP is different than the initial value of V_NVALLOWOVTRP (30 km/h). |  |  | L1 |  |  | FS |  | - |  |
|  | 89 |  | 2 |  |  | 875.00 |  | L1 |  | FS |  |  | First message including Packet 3: National Values, Including V_NVALLOWOVTRP (40 km/h) is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 3 |  |  | 925.00 |  | L1 |  | FS |  |  | Second message including Packet 3: National Values, Including V_NVALLOWOVTRP (50 km/h) and Linking Information is RECEIVED. |  | I |  | BTM |  | NID_C (000011) is different from initial balise / new Region or Country is announced. |  |  | L1 |  |  | FS |  | - |  |
|  | 91 |  | 4 |  |  | 925.00 |  | L1 |  | FS |  |  | Second message including Packet 3: National Values, Including V_NVALLOWOVTRP (50 km/h) is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 92 |  | 5 |  |  | 980.00 |  | L1 |  | FS |  |  | The train receives a balise message including a NID_C = 000011 |  | I |  | BTM |  | New Region or Country is announced. |  |  | L1 |  |  | FS |  | - |  |
|  | 93 |  | 6 |  |  | 980.00 |  | L1 |  | FS |  |  | The train receives a balise message including a NID_C = 000011 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 94 |  | 7 |  |  | 980.00 |  | L1 |  | FS |  |  | The Train Speed is between 40 and 50 km/h |  | O |  | DMI |  | Second message including Packet 3: National Values including V_NVALLOWOVTRP (50 km/h) gets VALID. Initial set of NVs gets INVALID and is DELETED. |  |  | L1 |  |  | FS |  | - |  |
|  | 95 |  | 8 |  |  | 980.00 |  | L1 |  | FS |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | FS |  | - |  |
|  | 96 |  | 9 |  |  | 980.00 |  | L1 |  | FS |  |  | Override button IS ACTIVATED. |  | O |  | DMI |  | Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | FS |  | - |  |
|  | 97 |  | 10 |  |  | 1095.00 |  | L1 |  | FS |  |  | The train speed > 50 km/h |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 98 |  | 11 |  |  | 1095.00 |  | L1 |  | FS |  |  | The driver selects the right menu in order to see the override button, |  | I |  | DMI |  | (override request is not available in NL, SL and TR) |  |  | L1 |  |  | FS |  | - |  |
|  | 99 |  | 12 |  |  | 1095.00 |  | L1 |  | FS |  |  | Override button is NOT ACTIVATED. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 100 |  | 13 |  |  | 1225.00 |  | L1 |  | FS |  |  | The train receives a balise message including a NID_C = 000001 |  | I |  | BTM |  | The Message does NOT contain National Values. Due to the Fact that the last uses NV of Region 000001 should be deleted as invalid, train uses default Values. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 41: Application of national or default values depending of the availability of an appropriate set of NVs |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 102 |  | 1 |  |  | 1225.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN=V_PERMITTED+10) |  | I |  | INT |  | As there are no NV available the train ERTMS will manage the train according to default values. |  |  | L1 |  |  | FS |  | - |  |
|  | 103 |  | 2 |  |  | 1225.00 |  | L1 |  | FS |  |  | The driver selects "menu" to see the "override button" |  | I |  | DMI |  | The driver selects the right menu in order to see the override button, if the menu with override button is not activated. (optional step) (override request is not available in NL, SL and TR) |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 3 |  |  | 1225.00 |  | L1 |  | FS |  |  | Train is moving (V_TRAIN>=V_NVALLOWOVTRP) |  | I |  | INT |  | The train runs with speed higher than V_NVALLOWOVTRP (default value 0 km/h) (e.g. V_Train = 30km/h). The 'override request' is NOT ACTIVATED. |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 4 |  |  | 1225.00 |  | L1 |  | FS |  |  | The 'override request' is not activated |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 5 |  |  | 1375.00 |  | L1 |  | FS |  |  | Train is at standstill (V_TRAIN=0; TIME= 60) |  | I |  | INT |  | The train decelerates to standstill. When the train reached standstill then the 'override request' is ACTIVATED. |  |  | L1 |  |  | FS |  | - |  |
|  | 107 |  | 6 |  |  | 1375.00 |  | L1 |  | FS |  |  | The 'override request' is activated |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 1: The ERTMS/ETCS on-board equipment allows the driver to select 'Override'. The speed limit for triggering the 'override' function is a national value and should be replaced when a new set of NVs is available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 1 |  |  | 1425.00 |  | L1 |  | FS |  |  | The train runs with speed higher than V_NVALLOWOVTRP (included in the default value) (e.g. V_Train = 30km/h). The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 2 |  |  | 1425.00 |  | L1 |  | FS |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECEIVED. |  | I |  | BTM |  | Use the steps 2/3 or 4/5 for testing' Evaluating a balise group message, balise identity information shall be used to ensure that correct NV are used. (Compare NID_C) Balise group is taken as a new LRBG. New V_NVALLOWOVTRP > default value (0 km/h) |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 3 |  |  | 1425.00 |  | L1 |  | FS |  |  | Packet 3: National Values, Including V_NVALLOWOVTRP, D_VALIDNV & NID_C is RECORDED |  | O |  | JRU |  | Use the steps 2/3 or 4/5 for testing' |  |  | L1 |  |  | FS |  | - |  |
|  | 113 |  | 6 |  |  | 1425.00 |  | L1 |  | FS |  |  | The override button is activated |  | I |  | DMI |  | if the menu with override button is not activated, the driver selects the right menu in order to see the override button |  |  | L1 |  |  | FS |  | - |  |
|  | 114 |  | 7 |  |  | 1475.00 |  | L1 |  | FS |  |  | The train does not reach the location indicated in D_VALIDNV, D_Train < D_VALIDNV The 'override request' is NOT AVAILABLE |  | O |  | DMI |  | As long as the train has not reached the location indicated in D_VALIDNV, the national values shall still not be applicable After this one steps 8 or 9 will be chosen for testing depending on design conditions |  |  | L1 |  |  | FS |  | - |  |
|  | 115 |  | 8 |  |  | 1525.00 |  | L1 |  | FS |  |  | The train reaches the distance indicated in D_VALIDNV, D_Train > D_VALIDNV; And the speed of the train V_Train is less or equal to V_NVALLOWOVTRP, V_Train < V_NVALLOWOVTRP The 'override request' is AVAILABLE. |  | O |  | DMI |  | Only If NID_C of received NV is equal to country or region identifier of the LRBG, National values shall become applicable. Override request is not available in NL, SL, RV and TR |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4041500: 4.4.15: NON LEADING Test Case 1: The train Onboard System shall use a 'non leading input signal' from the train interface for entering to NL mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 1 |  |  | 1625.00 |  | L1 |  | FS |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 120 |  | 2 |  |  | 1625.00 |  | L1 |  | FS |  |  | The driver selects 'Main' |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 121 |  | 3 |  |  | 1625.00 |  | L1 |  | FS |  |  | The <Main> window is displayed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 122 |  | 4 |  |  | 1625.00 |  | L1 |  | FS |  |  | The Non Leading button in the Main window is disabled |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 123 |  | 5 |  |  | 1625.00 |  | L1 |  | FS |  |  | Non-leading permitted |  | I |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 124 |  | 6 |  |  | 1625.00 |  | L1 |  | FS |  |  | NON LEADING INPUT (NID_MESSAGE_JRU=32) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 125 |  | 7 |  |  | 1625.00 |  | L1 |  | FS |  |  | The Non Leading button in the Main window is enabled |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 126 |  | 8 |  |  | 1625.00 |  | L1 |  | FS |  |  | The driver selects "NON LEADING" |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 9 |  |  | 1625.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 13: Indication of Non Leading mode to the driver |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 129 |  | 1 |  |  | 1625.00 |  | L1 |  | NL |  |  | NL mode is permanently displayed to the driver |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | NL |  | - |  |
|  | Feature 4041500: 4.4.15: NON LEADING Test Case 1: The train Onboard System shall use a 'non leading input signal' from the train interface for entering to NL mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 130 |  | 11 |  |  | 1635.00 |  | L1 |  | NL |  |  | Train is moving (V_TRAIN=40) |  | I |  | INT |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | 131 |  | 12 |  |  | 1635.00 |  | L1 |  | NL |  |  | No supervision information is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | 132 |  | 13 |  |  | 1635.00 |  | L1 |  | NL |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | 133 |  | 14 |  |  | 1635.00 |  | L1 |  | NL |  |  | Emergency brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | Feature 5080200: 5.8.2: Selection of 'Override' Test Case 5: The "Override request" is not available in SB for L1, NL, SL, RV and TR modes. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 |  | 1 |  |  | 1725.00 |  | L1 |  | NL |  |  | The train runs with speed lower than V_NVALLOWOVTRP (included in the previous set of NV) (e.g. V_Train = 30km/h). The 'override request' is NOT AVAILABLE. |  | O |  | DMI |  | National values are applicable and used by the train. Override request is not available in SB for L1, NL, SL, RV and TR |  |  | L1 |  |  | NL |  | - |  |
|  | 135 |  | 2 |  |  | 1725.00 |  | L1 |  | NL |  |  | Onboard receives a balise group message. |  | I |  | BTM |  | Balise group is taken as a new LRBG. There is a mismatch between country or region identifier NID_C of the Balise Group and the corresponding identifier for which a set of national values is used on-board |  |  | L1 |  |  | NL |  | - |  |
|  | 136 |  | 3 |  |  | 1725.00 |  | L1 |  | NL |  |  | Balise group message is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | 137 |  | 4 |  |  | 1825.00 |  | L1 |  | NL |  |  | The train decelerates to standstill and when it has reached standstill the 'override request' is still NOT AVAILABLE. |  | O |  | DMI |  | Default Values shall be used as fall back values (e.g. V_Train = 30km/h). Override request is not available in SB for L1, NL, SL, RV and TR |  |  | L1 |  |  | NL |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 138 |  | 1 |  |  | 1875.00 |  | L1 |  | NL |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - _____ Reset NV |  |  | L1 |  |  | NL |  | - |  |
|  | 139 |  | 2 |  |  | 1875.00 |  | L1 |  | NL |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | NL |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  |  | 1950.00 |  | L1 |  | NL |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reaches standstill |  |  | L1 |  |  | NP |  | - |  |
|  | 142 |  | 2 |  |  | 1950.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 11 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 7 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 30 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 30 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 60 : Balise Group BG 00000000001060 1/2 - Distance 250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0011110 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 000001111101000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0011110 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 60 : Balise Group BG 00000000001060 2/2 - Distance 255.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 82 : Balise Group BG 00000000001182 1/2 - Distance 725.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00001100 b | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001001 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0011110 b |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 0000000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 000011111010000 b |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00010101 b | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000110110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 000000000000000 b |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 00000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 00011011 b | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000000111010 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 000000000000000 b |  |  |
| V_STATIC | 7 | 0011110 b |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 82 : Balise Group BG 00000000001182 2/2 - Distance 730.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000001100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 88 : Balise Group BG 00000000000088 1/2 - Distance 875.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00000011 b | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011100110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000000001010 b |  |  |
| NID_C | 10 | 0011111110 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0000110 b |  |  |
| V_NVSTFF | 7 | 0001011 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001000 b |  |  |
| V_NVSUPOVTRP | 7 | 0000110 b |  |  |
| D_NVOVTRP | 15 | 000000011001000 b |  |  |
| T_NVOVTRP | 8 | 00111100 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 88 : Balise Group BG 00000000000088 2/2 - Distance 880.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000010 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 90 : Balise Group BG 00000000000090 1/2 - Distance 925.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00000011 b | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000011100110 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000000001010 b |  |  |
| NID_C | 10 | 0011111110 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0000110 b |  |  |
| V_NVSTFF | 7 | 0001011 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001010 b |  |  |
| V_NVSUPOVTRP | 7 | 0000110 b |  |  |
| D_NVOVTRP | 15 | 000000011001000 b |  |  |
| T_NVOVTRP | 8 | 00111100 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 00000101 b | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 000000000110111 b |  |  |
| Q_NEWCOUNTRY | 1 | 1 b |  |  |
| NID_C | 10 | 0011111110 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 01 b |  |  |
| Q_LOCACC | 6 | 000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 90 : Balise Group BG 00000000000090 2/2 - Distance 930.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000000000011 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 92 : Balise Group BG 00000001101192 1/2 - Distance 980.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0011111110 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00000101 b | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 0000001001111 b |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 000000011110101 b |  |  |
| Q_NEWCOUNTRY | 1 | 1 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| NID_BG | 14 | 00000011011110 b |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 01 b |  |  |
| Q_LOCACC | 6 | 000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 92 : Balise Group BG 00000001101192 2/2 - Distance 985.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0011111110 b |  |  |
| NID_BG | 14 | 00000001101111 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 11111111 b | End of information |  |

| - Step 100 : Balise Group BG 000000110111100 1/2 - Distance 1225.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0000000001 b |  |  |
| NID_BG | 14 | 00000011011110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 100 : Balise Group BG 000000110111100 2/2 - Distance 1230.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0000000001 b |  |  |
| NID_BG | 14 | 00000011011110 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 109 : Balise Group BG 000000000001109 1/2 - Distance 1425.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 000 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0000000001 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 000000001100100 b |  |  |
| NID_C | 10 | 0001000000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 0000110 b |  |  |
| V_NVSTFF | 7 | 0001000 b |  |  |
| V_NVONSIGHT | 7 | 0000110 b |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 0010100 b |  |  |
| V_NVREL | 7 | 0001000 b |  |  |
| D_NVROLL | 15 | 000000000000010 b |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0001000 b |  |  |
| V_NVSUPOVTRP | 7 | 0000110 b |  |  |
| D_NVOVTRP | 15 | 000000011001000 b |  |  |
| T_NVOVTRP | 8 | 00111100 b |  |  |
| D_NVPOTRP | 15 | 000000011001000 b |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 11111111 b |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 111111111111111 b |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 001100 b |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 109 : Balise Group BG 000000000001109 2/2 - Distance 1430.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 001 b |  |  |
| N_TOTAL | 3 | 001 b |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 0000000001 b |  |  |
| NID_BG | 14 | 00000000000100 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Balise Group BG 000000000001135 1/2 - Distance 1725.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 254 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 135 : Balise Group BG 000000000001135 2/2 - Distance 1730.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 00000000 b |  |  |
| NID_C | 10 | 254 d |  |  |
| NID_BG | 14 | 00000000000101 b |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 138 : Balise Group BG 99 d 1/2 - Distance 1875.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 0010100 b |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00000 b |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 138 : Balise Group BG 99 d 2/2 - Distance 1880.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 99 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |