| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5080400_04_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5080400_04_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 12:27 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Tested Feature: 5.8.4: End of Override procedure.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 44 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 45 |  | 4 |  |  | 50.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 8: The driver selects override request, the system switches to SR mode and override status is displayed. As soon as the train receives a new MA from a balise group, the on-board switches to FS and the 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 46 |  | 1 |  |  | 150.00 |  | L1 |  | FS |  |  | The driver selects "Override EOA" |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 47 |  | 2 |  |  | 150.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=14) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 48 |  | 3 |  |  | 150.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 49 |  | 4 |  |  | 150.00 |  | L1 |  | SR |  |  | The Mode symbol "Override EOA is active" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 50 |  | 5 |  |  | 150.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit18=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 51 |  | 6 |  |  | 250.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12 (<V_MAIN different from 0>), packet 21, packet 27 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 52 |  | 7 |  |  | 250.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=12; <V_MAIN different from 0>; NID_PACKET=21; NID_PACKET=27) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 53 |  | 8 |  |  | 250.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | The train switches to FS |  |  | L1 |  |  | FS |  | - |  |
|  | 54 |  | 9 |  |  | 250.00 |  | L1 |  | FS |  |  | The override status is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 10 |  |  | 250.00 |  | L1 |  | FS |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 11 |  |  | 250.00 |  | L1 |  | FS |  |  | The former EOA/LOA, that was in use before the Override activation, has been replaced with the one of the new MA received |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 26: The train receives and uses trip information given by balise and override is not active. Transition Condition Id [18]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 |  | 1 |  |  | 300.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 12 (V_MAIN=0) is received |  | I |  | BTM |  | It contains the packet 12 'Level 1 Movement Authority' with V_MAIN=0, which means train trip |  |  | L1 |  |  | FS |  | - |  |
|  | 59 |  | 2 |  |  | 300.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 60 |  | 3 |  |  | 300.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 62 |  | 1 |  |  | 300.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 63 |  | 2 |  |  | 300.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 26: The train receives and uses trip information given by balise and override is not active. Transition Condition Id [18]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 64 |  | 5 |  |  | 300.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 65 |  | 6 |  |  | 300.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit12=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 66 |  | 7 |  |  | 300.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 67 |  | 8 |  |  | 300.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 68 |  | 9 |  |  | 300.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Symbol: ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 69 |  | 10 |  |  | 300.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  | 1 |  |  | 320.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 71 |  | 2 |  |  | 340.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 72 |  | 3 |  |  | 340.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 73 |  | 4 |  |  | 340.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 74 |  | 5 |  |  | 340.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 75 |  | 6 |  |  | 340.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 76 |  | 7 |  |  | 340.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 79 |  | 10 |  |  | 340.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 80 |  | 11 |  |  | 340.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 81 |  | 12 |  |  | 340.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 82 |  | 13 |  |  | 340.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 83 |  | 14 |  |  | 340.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 7: The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start', because the train data are available on-board (new TC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 84 |  | 1 |  |  | 340.00 |  | L1 |  | PT |  |  | The On-Board equipment waits for the driver to select 'Start'. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 85 |  | 2 |  |  | 340.00 |  | L1 |  | PT |  |  | The Driver selects 'Start' |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 86 |  | 3 |  |  | 340.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 87 |  | 4 |  |  | 340.00 |  | L1 |  | PT |  |  | The Mode symbol "Acknowledge Staff Responsible" is displayed. |  | O |  | DMI |  | DMI Object: Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 88 |  | 5 |  |  | 340.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 49: Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 89 |  | 1 |  |  | 340.00 |  | L1 |  | PT |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 90 |  | 2 |  |  | 340.00 |  | L1 |  | PT |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 91 |  | 3 |  |  | 340.00 |  | L1 |  | PT |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 92 |  | 4 |  |  | 340.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | Symbol MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 93 |  | 5 |  |  | 340.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 6: The driver selects override request, and override status is displayed. As soon as the train overpasses the SR distance supervised before overriding with its estimated front end, the 'override status' is removed so the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  | 1 |  |  | 450.00 |  | L1 |  | SR |  |  | V_TRAIN < V_NVALLOWOVTRP |  | I |  | INT |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 96 |  | 2 |  |  | 550.00 |  | L1 |  | SR |  |  | D=do The driver selects override and the override status is displayed |  | I |  | DMI |  | do is the position of the train when the driver selects override |  |  | L1 |  |  | SR |  | - |  |
|  | 97 |  | 3 |  |  | 550.00 |  | L1 |  | SR |  |  | The driver selection is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 98 |  | 4 |  |  | 690.00 |  | L1 |  | SR |  |  | The override status is REMOVED |  | O |  | DMI |  | Train overpasses the SR distance supervised before overriding _____ D_NVSTFF = 350m |  |  | L1 |  |  | SR |  | - |  |
|  | 99 |  | 5 |  |  | 690.00 |  | L1 |  | SR |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 100 |  | 6 |  |  | 690.00 |  | L1 |  | SR |  |  | Emergency brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 101 |  | 7 |  |  | 690.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | The train does not switch to TR Transition Condition Id [42] not executed because Override is active |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 35: The train is moving in SR mode. It passes over a balise group sending a message which contains packet 137 'Stop if in Staff Responsible' AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. Therefore, the train is tripped. Transition Condition Id [54]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 103 |  | 1 |  |  | 740.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 137 (Q_SRSTOP=0) is received |  | I |  | BTM |  | BGa telegram contains Packet 137: 'Stop if in Staff Responsible' with variable Q_SRSTOP = 0. |  |  | L1 |  |  | SR |  | - |  |
|  | 104 |  | 2 |  |  | 740.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 105 |  | 3 |  |  | 740.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 107 |  | 1 |  |  | 740.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 108 |  | 2 |  |  | 740.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 35: The train is moving in SR mode. It passes over a balise group sending a message which contains packet 137 'Stop if in Staff Responsible' AND no list of expected balise groups related to SR mode has been received or the list of expected balise groups related to SR mode does not include the identity of the over-passed balise group AND override is not active. Therefore, the train is tripped. Transition Condition Id [54]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 109 |  | 5 |  |  | 740.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 110 |  | 6 |  |  | 740.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit16=1>) is recorded |  | O |  | JRU |  | SR stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 111 |  | 7 |  |  | 740.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 112 |  | 8 |  |  | 740.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 113 |  | 9 |  |  | 740.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 114 |  | 10 |  |  | 740.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 1 |  |  | 760.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 116 |  | 2 |  |  | 780.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 117 |  | 3 |  |  | 780.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 118 |  | 4 |  |  | 780.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 119 |  | 5 |  |  | 780.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 120 |  | 6 |  |  | 780.00 |  | L1 |  | TR |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 121 |  | 7 |  |  | 780.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 124 |  | 10 |  |  | 780.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 125 |  | 11 |  |  | 780.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 126 |  | 12 |  |  | 780.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 127 |  | 13 |  |  | 780.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 128 |  | 14 |  |  | 780.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 7: The train is in PT mode and level 1.The ERTMS/ETCS on-board equipment shall offer the possibility to the driver to select „start', because the train data are available on-board (new TC) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 129 |  | 1 |  |  | 780.00 |  | L1 |  | PT |  |  | The On-Board equipment waits for the driver to select 'Start'. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 130 |  | 2 |  |  | 780.00 |  | L1 |  | PT |  |  | The Driver selects 'Start' |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 131 |  | 3 |  |  | 780.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 132 |  | 4 |  |  | 780.00 |  | L1 |  | PT |  |  | The Mode symbol "Acknowledge Staff Responsible" is displayed. |  | O |  | DMI |  | DMI Object: Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 133 |  | 5 |  |  | 780.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 49: Acknowledgement for SR mode is requested in level 1 and driver acknowledges. Transition Condition Id [8]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 |  | 1 |  |  | 780.00 |  | L1 |  | PT |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | PT |  | - |  |
|  | 135 |  | 2 |  |  | 780.00 |  | L1 |  | PT |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 136 |  | 3 |  |  | 780.00 |  | L1 |  | PT |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 137 |  | 4 |  |  | 780.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | Symbol MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 138 |  | 5 |  |  | 780.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080421: 4.8.4: Accepted Information depending on the modes - List of balises for SH Area Test Case 6: Transmission and acceptance of a list of balises for SH area together with a mode profile for shunting and a MA via balise group (independent of an empty or not empty list) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 |  | 1 |  |  | 850.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12, packet 21, packet 27, packet 49, packet 80 is received |  | I |  | BTM |  | Telegram is received by on-board |  |  | L1 |  |  | FS |  | - |  |
|  | 141 |  | 2 |  |  | 850.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Packets and telegram are recorded. |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 39: Acknowledgement for shunting is requested, driver acknowledges, the system switches to SH mode and an end of mission takes place (only in levels 2 and 3). Transition Condition Id [50]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 144 |  | 1 |  |  | 875.00 |  | L1 |  | FS |  |  | Train is moving. |  | I |  | INT |  | Train Location: The train reaches the acknowledgement window defined by the variable L_ACKMODE (in packet 80) with its maximum safe front end. V_TRAIN: The speed is lower than the maximum shunting speed: D_MAMODE>D>D_MAMODE–L_ACKMAMODE V<V_NVSHUNT |  |  | L1 |  |  | FS |  | - |  |
|  | 145 |  | 2 |  |  | 875.00 |  | L1 |  | FS |  |  | The Mode symbol 'Acknowledge Shunting' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO02 |  |  | L1 |  |  | FS |  | - |  |
|  | 146 |  | 3 |  |  | 875.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit17=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 147 |  | 4 |  |  | 900.00 |  | L1 |  | FS |  |  | The driver acknowledges 'Shunting' |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 148 |  | 5 |  |  | 900.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 149 |  | 6 |  |  | 900.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=3) is recorded |  | O |  | JRU |  | Triggering event: when the mode changes |  |  | L1 |  |  | SH |  | - |  |
|  | 150 |  | 7 |  |  | 900.00 |  | L1 |  | SH |  |  | The Mode symbol 'Mode <SH>' is displayed |  | O |  | DMI |  | DMI Object: MO01 or USE_FT4070201.20 |  |  | L1 |  |  | SH |  | - |  |
|  | 151 |  | 8 |  |  | 900.00 |  | L1 |  | SH |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit16=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 5080400: 5.8.4: End of Override procedure Test Case 10: The driver selects override request, the system does not change the mode. As soon as the train crosses a balise group not in the list of expected balises in SH mode, the train trip suppression is cancelled |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 154 |  | 1 |  |  | 1050.00 |  | L1 |  | SH |  |  | The driver select 'override request' |  | I |  | DMI |  | The 'override request' is available only when the train speed is below V_NVALLOWOVTRP (execute with no standstill) |  |  | L1 |  |  | SH |  | - |  |
|  | 155 |  | 2 |  |  | 1050.00 |  | L1 |  | SH |  |  | The selection of 'override request' is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 156 |  | 3 |  |  | 1050.00 |  | L1 |  | SH |  |  | The override status is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 157 |  | 4 |  |  | 1050.00 |  | L1 |  | SH |  |  | The display of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 158 |  | 5 |  |  | 1150.00 |  | L1 |  | SH |  |  | Train reads a message while crossing a balise group not in the list of expected balises. |  | I |  | BTM |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 159 |  | 6 |  |  | 1150.00 |  | L1 |  | SH |  |  | Received balise message is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 160 |  | 7 |  |  | 1150.00 |  | L1 |  | SH |  |  | The train mode is SH. The train does not switch to TR because the train trip is inhibited. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 161 |  | 8 |  |  | 1150.00 |  | L1 |  | SH |  |  | Emergency brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 162 |  | 9 |  |  | 1150.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | The train does not switch to TR Transition Condition Id [52] not executed because Override is active |  |  | L1 |  |  | SH |  | - |  |
|  | 163 |  | 10 |  |  | 1150.00 |  | L1 |  | SH |  |  | The override status is REMOVED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 164 |  | 11 |  |  | 1150.00 |  | L1 |  | SH |  |  | The cancellation of override status is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4080426: 4.8.4: Accepted Information depending on the modes - National Values Test Case 4: Transmission of National Values via balise group |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 166 |  | 1 |  |  | 1200.00 |  | L1 |  | SH |  |  | One Balise Group message containing packet 3 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 167 |  | 2 |  |  | 1200.00 |  | L1 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 34: Before entering the shunting area, the train receives a list of balises for shunting area. When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied. Transition Condition Id [52]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 169 |  | 1 |  |  | 1200.00 |  | L1 |  | SH |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) is received |  | I |  | BTM |  | NID_BG variable in BGa message is different from NID_BG values in the list of balises for SH stored onboard OR the list of balises for SH is empty. |  |  | L1 |  |  | SH |  | - |  |
|  | 170 |  | 2 |  |  | 1200.00 |  | L1 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 171 |  | 3 |  |  | 1200.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 173 |  | 1 |  |  | 1200.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 174 |  | 2 |  |  | 1200.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 34: Before entering the shunting area, the train receives a list of balises for shunting area. When the train passes over the first balise group (BGa) not included in the list, it switches to TRIP mode and the emergency brakes are applied. Transition Condition Id [52]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 175 |  | 5 |  |  | 1200.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 176 |  | 6 |  |  | 1200.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded |  | O |  | JRU |  | SH stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 177 |  | 7 |  |  | 1200.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 178 |  | 8 |  |  | 1200.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 179 |  | 9 |  |  | 1200.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 180 |  | 10 |  |  | 1200.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 181 |  | 1 |  |  | 1250.00 |  | L1 |  | TR |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reached standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 182 |  | 2 |  |  | 1250.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 40 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 400 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 20 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 10 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 8 d |  |  |
| V_NVSUPOVTRP | 7 | 8 d |  |  |
| D_NVOVTRP | 15 | 300 d |  |  |
| T_NVOVTRP | 8 | 40 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 350 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 2 d 1/2 - Distance 250.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 10 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1400 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 10 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 51 : Balise Group BG 2 d 2/2 - Distance 260.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 58 : Balise Group BG 22 d 1/2 - Distance 300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 22 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 0 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1400 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 58 : Balise Group BG 22 d 2/2 - Distance 305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 22 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 103 : Balise Group BG 23 d 1/2 - Distance 740.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 23 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 137 d | Stop if in staff responsible |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 24 d |  |  |
| Q_SRSTOP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 103 : Balise Group BG 23 d 2/2 - Distance 745.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 23 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 140 : Balise Group BG 3 d 1/2 - Distance 850.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 20 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 100 d |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 75 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 49 d | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| N_ITER | 5 | 3 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 133 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 134 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 135 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 00 b |  |  |
| L_PACKET | 13 | 0 d |  |  |
| Q_SCALE | 2 | 00 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 0 d |  |  |
| L_MAMODE | 15 | 0 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 140 : Balise Group BG 3 d 2/2 - Distance 855.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 158 : Balise Group BG 4 d 1/2 - Distance 1150.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 00000000 b | Virtual Balise Cover Marker |  |
| NID_VBCMK | 6 | 000000 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 158 : Balise Group BG 4 d 2/2 - Distance 1155.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 166 : Balise Group BG 24 d 1/2 - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 24 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 240 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| NID_C | 10 | 352 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 2 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 d |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 0 d |  |  |
| M_NVEBCL | 4 | 1001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 166 : Balise Group BG 24 d 2/2 - Distance 1205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 24 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |