| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5180700_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5180700_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:44 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

Req 5.18.7: Inhibition of a defined type of brake.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 2: New Level 1 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 100.00 |  | L1 |  | SR |  |  | One Balise Group message (N_TOTAL=1) containing packet 12 (MA), packet 21 (Gradient) and packet 27 (SSP) is received |  | I |  | BTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 100.00 |  | L1 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 100.00 |  | L1 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | The Target Distance will match the length of the last received MA. |  |  | L1 |  |  | FS |  | - |  |
|  | 42 |  | 4 |  |  | 100.00 |  | L1 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20; D_TARGET=<equal to the lenth of the last MA>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080445: 4.8.4: Accepted Information depending on the modes - Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses Test Case 7: Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 100.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 68 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 44 |  | 2 |  |  | 100.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 24: Once in Full Supervision mode, if SSP and gradient are not known for the whole length of the train, an indication 'ENTRY IN FULL SUPERVISION' shall be clearly displayed to the driver until SSP and gradient are known for the whole length of the train. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 48 |  | 1 |  |  | 100.00 |  | L1 |  | FS |  |  | "Entry in FS mode" is DISPLAYED to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 49 |  | 2 |  |  | 100.00 |  | L1 |  | FS |  |  | Indication of 'Entry in FS mode' information is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 50 |  | 3 |  |  | 140.00 |  | L1 |  | FS |  |  | The indication 'Entry in FS' is REMOVED as soon as the whole train is fully inside the area covered by the track description data . |  | O |  | DMI |  | Movement of the train: SSP + gradient are then known for the whole length of the train. |  |  | L1 |  |  | FS |  | - |  |
|  | 51 |  | 4 |  |  | 140.00 |  | L1 |  | FS |  |  | Deletion of 'Entry in FS mode' information is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5180700: 5.18.7: Inhibition of a defined type of brake Test Case 1: Passing the area forbidden for Regenerative braking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 525.00 |  | L1 |  | FS |  |  | 'Inhibition of regenerative brake announcement' is DISPLAYED It is DISPLAYED if the related functionality is executed: Automatically manually (the driver is requested to act) |  | O |  | DMI |  | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the regenerative brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. Distance from the reference balise (DLRBG)<D_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 53 |  | 2 |  |  | 525.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=1¦Bit61=1>) is recorded |  | O |  | JRU |  | TC17 (automatic execution) or TC18 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 54 |  | 3 |  |  | 525.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 55 |  | 4 |  |  | 525.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 56 |  | 5 |  |  | 575.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 57 |  | 6 |  |  | 575.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing |  |  | L1 |  |  | FS |  | - |  |
|  | 58 |  | 7 |  |  | 650.00 |  | L1 |  | FS |  |  | 'Inhibition of regenerative brake announcement' is REMOVED |  | O |  | DMI |  | The train reaches the max safe front end of the start location of 'brake inhibition area' |  |  | L1 |  |  | FS |  | - |  |
|  | 59 |  | 8 |  |  | 650.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit61=0>) is recorded |  | O |  | JRU |  | TC18 If the related functionality is executed automatically, there is no removal of the symbol TC18, because it it symbol TC17 that has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 60 |  | 9 |  |  | 650.00 |  | L1 |  | FS |  |  | 'Inhibition of regenerative brake' is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 61 |  | 10 |  |  | 650.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=1>) is recorded |  | O |  | JRU |  | TC17 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 62 |  | 11 |  |  | 650.00 |  | L1 |  | FS |  |  | Regenerative braking switched off |  | I |  | TIU |  | The On-board Equipment enters in the area forbidden for regenerative braking |  |  | L1 |  |  | FS |  | - |  |
|  | 63 |  | 12 |  |  | 650.00 |  | L1 |  | FS |  |  | REGENERATIVE BRAKE STATUS (NID_MESSAGE_JRU=33; M_RB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 64 |  | 13 |  |  | 650.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the regenerative brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 65 |  | 14 |  |  | 650.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=3, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased |  |  | L1 |  |  | FS |  | - |  |
|  | 66 |  | 15 |  |  | 990.00 |  | L1 |  | FS |  |  | 'Inhibition of regenerative brake' is REMOVED |  | O |  | DMI |  | The train reaches with its min safe rear end the end location of brake inhibition area _____ L_TRAIN=40 |  |  | L1 |  |  | FS |  | - |  |
|  | 67 |  | 16 |  |  | 990.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit60=0>) is recorded |  | O |  | JRU |  | TC17 |  |  | L1 |  |  | FS |  | - |  |
|  | 68 |  | 17 |  |  | 990.00 |  | L1 |  | FS |  |  | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 69 |  | 18 |  |  | 990.00 |  | L1 |  | FS |  |  | Regenerative braking switched on |  | I |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 70 |  | 19 |  |  | 990.00 |  | L1 |  | FS |  |  | REGENERATIVE BRAKE STATUS (NID_MESSAGE_JRU=33; M_RB_STATUS=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080445: 4.8.4: Accepted Information depending on the modes - Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses Test Case 7: Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 71 |  | 1 |  |  | 1190.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 68 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 72 |  | 2 |  |  | 1190.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5180700: 5.18.7: Inhibition of a defined type of brake Test Case 2: Passing the area forbidden for eddy currents braking for service brake. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 76 |  | 1 |  |  | 1365.00 |  | L1 |  | FS |  |  | 'Inhibition of eddy current brake announcement' is DISPLAYED It is DISPLAYED if the related functionality is executed: automatically manually (the driver is requested to act) |  | O |  | DMI |  | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the eddy currents brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. DLRBG<D_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 77 |  | 2 |  |  | 1365.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=1¦Bit59=1>) is recorded |  | O |  | JRU |  | TC15 (automatic execution) or TC16 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 78 |  | 3 |  |  | 1365.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 79 |  | 4 |  |  | 1365.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 80 |  | 5 |  |  | 1415.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 81 |  | 6 |  |  | 1415.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing |  |  | L1 |  |  | FS |  | - |  |
|  | 82 |  | 7 |  |  | 1490.00 |  | L1 |  | FS |  |  | 'Inhibition of eddy current brake announcement' is REMOVED |  | O |  | DMI |  | The train reaches the max safe front end of the start location of 'brake inhibition area' |  |  | L1 |  |  | FS |  | - |  |
|  | 83 |  | 8 |  |  | 1490.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit59=0>) is recorded |  | O |  | JRU |  | TC16 If the related functionality is executed automatically, there is no removal of the symbol TC16, because it it symbol TC15 that has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 84 |  | 9 |  |  | 1490.00 |  | L1 |  | FS |  |  | 'Inhibition of eddy current brake' is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 85 |  | 10 |  |  | 1490.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=1>) is recorded |  | O |  | JRU |  | TC15 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 86 |  | 11 |  |  | 1490.00 |  | L1 |  | FS |  |  | Eddy currents braking for service brake permission is switched off. |  | I |  | TIU |  | The On-board Equipment enters the area DLRBG>D_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 87 |  | 12 |  |  | 1490.00 |  | L1 |  | FS |  |  | EDDY CURRENT BRAKE STATUS (NID_MESSAGE_JRU=35; M_ECB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 88 |  | 13 |  |  | 1490.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the eddy currents brake inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 89 |  | 14 |  |  | 1490.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=6, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased |  |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 15 |  |  | 1830.00 |  | L1 |  | FS |  |  | 'Inhibition of eddy current brake' is REMOVED |  | O |  | DMI |  | The train reaches with its min safe rear end the end location of brake inhibition area _____ L_TRAIN=40 |  |  | L1 |  |  | FS |  | - |  |
|  | 91 |  | 16 |  |  | 1830.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit58=0>) is recorded |  | O |  | JRU |  | TC15 |  |  | L1 |  |  | FS |  | - |  |
|  | 92 |  | 17 |  |  | 1830.00 |  | L1 |  | FS |  |  | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 93 |  | 18 |  |  | 1830.00 |  | L1 |  | FS |  |  | Eddy currents braking for service brake permission is switched on |  | I |  | TIU |  | DLRBG-L_TRAIN>D_TRACKCOND + L_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 94 |  | 19 |  |  | 1830.00 |  | L1 |  | FS |  |  | EDDY CURRENT BRAKE STATUS (NID_MESSAGE_JRU=35; M_ECB_STATUS=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080445: 4.8.4: Accepted Information depending on the modes - Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses Test Case 7: Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 95 |  | 1 |  |  | 2000.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 68 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 96 |  | 2 |  |  | 2000.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5180700: 5.18.7: Inhibition of a defined type of brake Test Case 3: Going into the area forbidden for magnetic shoes braking. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 1 |  |  | 2175.00 |  | L1 |  | FS |  |  | 'Inhibition of magnetic shoe brake announcement' is DISPLAYED It is DISPLAYED if the related functionality is executed: automatically manually (the driver is requested to act) |  | O |  | DMI |  | The max safe front end of the train reaches a location (Point C) in rear of the beginning of the magnetic shoes brake inhibition area taking into account the time necessary for performing the required actions and the current train speed. DLRBG<D_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 101 |  | 2 |  |  | 2175.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=1¦Bit57=1>) is recorded |  | O |  | JRU |  | TC13 (automatic execution) or TC14 (driver requested to act) |  |  | L1 |  |  | FS |  | - |  |
|  | 102 |  | 3 |  |  | 2175.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 103 |  | 4 |  |  | 2175.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 5 |  |  | 2225.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 6 |  |  | 2225.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START>0, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START and D_MINSRE_TO_END are decreasing |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 7 |  |  | 2300.00 |  | L1 |  | FS |  |  | 'Inhibition of magnetic shoe brake announcement' is REMOVED |  | O |  | DMI |  | The train reaches the max safe front end of the start location of 'brake inhibition area' |  |  | L1 |  |  | FS |  | - |  |
|  | 107 |  | 8 |  |  | 2300.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit57=0>) is recorded |  | O |  | JRU |  | TC14 If the related functionality is executed automatically, there is no removal of the symbol TC14, because it it symbol TC13 that has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 108 |  | 9 |  |  | 2300.00 |  | L1 |  | FS |  |  | 'Inhibition of magnetic shoe brake' is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 10 |  |  | 2300.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=1>) is recorded |  | O |  | JRU |  | TC13 If the related functionality is executed automatically, there is no new display of the symbol, because it has been displayed since the announcement |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 11 |  |  | 2300.00 |  | L1 |  | FS |  |  | Track Conditions transmitted |  | O |  | TIU |  | The information related to the magnetic shoes inhibition area is provided to the ERTMS/ETCS external function |  |  | L1 |  |  | FS |  | - |  |
|  | 111 |  | 12 |  |  | 2300.00 |  | L1 |  | FS |  |  | TRACK CONDITIONS (NID_MESSAGE_JRU=45; M_TRACKCOND_TI=4, D_MAXSFE_TO_START=-32768, D_MINSRE_TO_END>0) is recorded |  | O |  | JRU |  | D_MAXSFE_TO_START not relevant and D_MINSRE_TO_END decreased |  |  | L1 |  |  | FS |  | - |  |
|  | 112 |  | 13 |  |  | 2300.00 |  | L1 |  | FS |  |  | Magnetic shoes braking permission is switched off. |  | I |  | TIU |  | The On-board Equipment enters in the area DLRBG>D_TRACKCOND |  |  | L1 |  |  | FS |  | - |  |
|  | 113 |  | 14 |  |  | 2300.00 |  | L1 |  | FS |  |  | MAGNETIC SHOE BRAKE STATUS (NID_MESSAGE_JRU=34; M_MSB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 114 |  | 15 |  |  | 2640.00 |  | L1 |  | FS |  |  | 'Inhibition of magnetic shoe brake' is REMOVED |  | O |  | DMI |  | The train reaches with its min safe rear end the end location of brake inhibition area _____ L_TRAIN=40 |  |  | L1 |  |  | FS |  | - |  |
|  | 115 |  | 16 |  |  | 2640.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit56=0>) is recorded |  | O |  | JRU |  | TC13 |  |  | L1 |  |  | FS |  | - |  |
|  | 116 |  | 17 |  |  | 2640.00 |  | L1 |  | FS |  |  | NOT TRACK CONDITIONS (NID_MESSAGE_JRU=45) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 117 |  | 18 |  |  | 2640.00 |  | L1 |  | FS |  |  | Magnetic shoe braking permission is switched on. |  | I |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 118 |  | 19 |  |  | 2640.00 |  | L1 |  | FS |  |  | MAGNETIC SHOE BRAKE STATUS (NID_MESSAGE_JRU=34; M_MSB_STATUS=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4080445: 4.8.4: Accepted Information depending on the modes - Track Conditions excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses Test Case 7: Acceptance of message including packets 68 (Track Condition) that is transmitted via Balise Group. Level 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 1 |  |  | 2800.00 |  | L1 |  | FS |  |  | One Balise Group message containing packet 68 is received |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 120 |  | 2 |  |  | 2800.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6; NID_PACKET=68) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 3120100: 3.12.1: Track Conditions Test Case 2: The on-board equipment receives from the balises a telegram containing a packet 68 (track conditions) to resume the track condition to initial state (the initial state shall be used by the on-board equipment) The on-board delete all track condition and continue run. The message is accepted and stored on-board The on-board equipment NOT displays anymore a profile indicating the location of previous received track conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 124 |  | 1 |  |  | 2925.00 |  | L1 |  | FS |  |  | The Track Condition symbol is displayed. |  | O |  | DMI |  | When the max safe front end of the train reaches a location in rear of the beginning of the track condition announced. |  |  | L1 |  |  | FS |  | - |  |
|  | 125 |  | 2 |  |  | 3000.00 |  | L1 |  | FS |  |  | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 68 (with No track condition) is received. |  | I |  | BTM |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 126 |  | 3 |  |  | 3000.00 |  | L1 |  | FS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 127 |  | 4 |  |  | 3000.00 |  | L1 |  | FS |  |  | NOT The Track Condition symbol announcement is displayed. |  | O |  | DMI |  | The train reaches the start location of the previous track condition section with its max safe front end ( = D_TRACKCOND ) |  |  | L1 |  |  | FS |  | - |  |
|  | 128 |  | 5 |  |  | 3050.00 |  | L1 |  | FS |  |  | NOT The Track Condition symbol is displayed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 9: Desks are closed. Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 129 |  | 1 |  |  | 3400.00 |  | L1 |  | FS |  |  | CAB not active |  | I |  | TIU |  | Driver closes the desk. _____ The train reached standstill. |  |  | L1 |  |  | FS |  | - |  |
|  | 130 |  | 2 |  |  | 3400.00 |  | L1 |  | FS |  |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 131 |  | 3 |  |  | 3400.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 4040700: 4.4.7: STAND BY Test Case 1: In Stand-By mode, the desk of the engine can be open or closed. No interaction with the driver shall be possible as long as the desk is closed, except isolation of the ERTMS/ETCS on-board equipment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 133 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | Cab active |  | I |  | TIU |  | The driver opens the desk |  |  | L1 |  |  | SB |  | - |  |
|  | 134 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | CAB STATUS (NID_MESSAGE_JRU=38) is recorded |  | O |  | JRU |  | M_CAB_A_STATUS = 1 |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  |  | SB |  | - |  |
|  | 142 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  |  | SB |  | - |  |
|  | 143 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 42: Entry of the Train running number through the Driver ID window when the Train running number is invalid |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 144 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | The on-board equipment offers the possibility to re-validate the Train running number |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 145 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver confirms the "Train Running Number" [DRIVER ID] |  | I |  | DMI |  | Train Running Number entered: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 146 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 26: Within the scope of the Start of Mission procedure in SB mode, the procedure ends as soon as the desk is closed |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 147 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 148 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver closes the desk |  | I |  | TIU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 149 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | 'CAB STATUS' closed is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 150 |  | 4 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver opens the desk |  | I |  | TIU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 151 |  | 5 |  |  | 3400.00 |  | L1 |  | SB |  |  | 'CAB STATUS' open is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 152 |  | 6 |  |  | 3400.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID |  | O |  | DMI |  | The SoM procedure starts from the beginning |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 153 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  |  | SB |  | - |  |
|  | 154 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  |  | SB |  | - |  |
|  | 155 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 156 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 157 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 158 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 159 |  | 4 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 3: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) The on-board equipment allows the driver to modify train data before the validation |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 160 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 161 |  | 2 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver modifies at least one of the train data. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 162 |  | 3 |  |  | 3400.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 163 |  | 4 |  |  | 3400.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 164 |  | 5 |  |  | 3400.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 1 |  |  | 3400.00 |  | L1 |  | SB |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train remained at standstill. |  |  | L1 |  |  | NP |  | - |  |
|  | 166 |  | 2 |  |  | 3400.00 |  | L1 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L1 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 40 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 5000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 b |  |  |
| D_TRACKCOND | 15 | 550 d |  |  |
| L_TRACKCOND | 15 | 300 d |  |  |
| M_TRACKCOND | 4 | 0110 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 2 d 1/2 - Distance 1190.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 d |  |  |
| D_TRACKCOND | 15 | 300 d |  |  |
| L_TRACKCOND | 15 | 300 d |  |  |
| M_TRACKCOND | 4 | 0111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 71 : Balise Group BG 2 d 2/2 - Distance 1195.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 95 : Balise Group BG 3 d 1/2 - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 d |  |  |
| D_TRACKCOND | 15 | 300 d |  |  |
| L_TRACKCOND | 15 | 300 d |  |  |
| M_TRACKCOND | 4 | 1000 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 95 : Balise Group BG 3 d 2/2 - Distance 2005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 119 : Balise Group BG 4 d 1/2 - Distance 2800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 d |  |  |
| D_TRACKCOND | 15 | 250 d |  |  |
| L_TRACKCOND | 15 | 300 d |  |  |
| M_TRACKCOND | 4 | 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 119 : Balise Group BG 4 d 2/2 - Distance 2805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 4 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 5 d 1/2 - Distance 3000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 41 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 1 d |  |  |
| D_TRACKINIT | 15 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 125 : Balise Group BG 5 d 2/2 - Distance 3005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |