| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990500_08_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990500_08_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 15:57 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | RINA |

# Sequence description

9990500_08

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 40 |  | 2 |  |  | 50.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L2 |  | SR |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L2 |  | SR |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 1 |  |  | 100.00 |  | L2 |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | SR |  | - |  |
|  | 44 |  | 2 |  |  | 100.00 |  | L2 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 45 |  | 3 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | SR |  | - |  |
|  | 46 |  | 4 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 47 |  | 5 |  |  | 100.00 |  | L2 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 48 |  | 6 |  |  | 100.00 |  | L2 |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 49 |  | 7 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 50 |  | 8 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 51 |  | 9 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SR |  | - |  |
|  | 52 |  | 10 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 53 |  | 11 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 54 |  | 12 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | SR |  | - |  |
|  | 57 |  | 2 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 58 |  | 3 |  |  | 100.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 59 |  | 4 |  |  | 100.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 48: A MA, SSP and Gradient Profile have been received on-board. The track description is available at least from the front end of the train. The system switches to FS. Transition Condition Id [10]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 400.00 |  | L2 |  | SR |  |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 21 and packet 27 is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 61 |  | 2 |  |  | 400.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 62 |  | 3 |  |  | 400.00 |  | L2 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=0) is recorded |  | O |  | JRU |  | Triggered event: When the mode changes |  |  | L2 |  |  | FS |  | - |  |
|  | 63 |  | 4 |  |  | 400.00 |  | L2 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed. |  | O |  | DMI |  | The driver is informed on the DMI of the change to FS mode. DMI Object: MO11 |  |  | L2 |  |  | FS |  | - |  |
|  | 64 |  | 5 |  |  | 400.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3150100: 3.15.1: RBC/RBC Handover Test Case 1: The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC; Send a position report to the Handing Over RBC when the maximum safe front end of the train passes the given location and Send a position report to the Handing Over RBC when the minimum safe rear end of the train passes the given location. Safe Radio Connection Indication is displayed continuously. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General Message" (NID_MESSAGE=24) containing packet 131 (D_RBCTR? 0) is received |  | I |  | RTM |  | from the Handing Over RBC D_RBCTR ? 0 The radio message contains a movement authority including the RBC transition border. |  |  | L2 |  |  | FS |  | - |  |
|  | 66 |  | 2 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 67 |  | 3 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 68 |  | 4 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-CONNECT. Confirm is received |  | I |  | RTM |  | A safe connection is established with the Accepting RBC |  |  | L2 |  |  | FS |  | - |  |
|  | 69 |  | 5 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 70 |  | 6 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 71 |  | 7 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | The Version of the ERTMS/ETCS language is valid for the train and M_VERSION ( { 000 XXXX, 111 1111}-) |  |  | L2 |  |  | FS |  | - |  |
|  | 72 |  | 8 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Communication session established for on-board. |  |  | L2 |  |  | FS |  | - |  |
|  | 73 |  | 9 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including Packet 2 is transmitted. |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 74 |  | 10 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Communication session established for trackside (Accepting RBC) |  |  | L2 |  |  | FS |  | - |  |
|  | 75 |  | 11 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 76 is received |  | I |  | RTM |  | Packet is stored onboard till handover is finished. Accepting RBC is the sender of the Fixed text message. |  |  | L2 |  |  | FS |  | - |  |
|  | 76 |  | 12 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 77 |  | 13 |  |  | 400.00 |  | L2 |  | FS |  |  | NOT "fixed text message" is displayed |  | O |  | DMI |  | At this location |  |  | L2 |  |  | FS |  | - |  |
|  | 78 |  | 14 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 0 and packet 11 is transmitted. |  | O |  | RTM |  | The radio message contains also position report data. This step does not apply for SL or NL mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 79 |  | 15 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | This step does not apply for SL or NL mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 80 |  | 16 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received. |  | I |  | RTM |  | This step does not apply for SL or NL mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 81 |  | 17 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | This step does not apply for SL or NL mode. |  |  | L2 |  |  | FS |  | - |  |
|  | 82 |  | 18 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | To both RBCs (Handing Over) with the use of the position report parameters valid for the Handing Over RBC |  |  | L2 |  |  | FS |  | - |  |
|  | 83 |  | 19 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 84 |  | 20 |  |  | 400.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | To both RBCs (Accepting) with the use of the position report parameters valid for the Handing Over RBC |  |  | L2 |  |  | FS |  | - |  |
|  | 85 |  | 21 |  |  | 400.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 86 |  | 22 |  |  | 560.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | Train location: Train maximum safe front end location = LRBG + D_RBCTR |  |  | L2 |  |  | FS |  | - |  |
|  | 87 |  | 23 |  |  | 560.00 |  | L2 |  | FS |  |  | The "fixed text message" is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 88 |  | 24 |  |  | 560.00 |  | L2 |  | FS |  |  | START DISPLAYING FIXED TEXT MESSAGE (NID_MESSAGE_JRU=16) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 89 |  | 25 |  |  | 560.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | To both RBCs (Handing Over) when the maximum safe front end of the train passes the location of the RBC transition. |  |  | L2 |  |  | FS |  | - |  |
|  | 90 |  | 26 |  |  | 560.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 91 |  | 27 |  |  | 560.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | To both RBCs (Accepting) when the maximum safe front end of the train passes the location of the RBC transition. |  |  | L2 |  |  | FS |  | - |  |
|  | 92 |  | 28 |  |  | 560.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 93 |  | 29 |  |  | 560.00 |  | L2 |  | FS |  |  | The indication to the driver is the same. The Status symbol "Safe radio connection - Connection Up" is displayed. |  | O |  | DMI |  | The switch of the indication status of the safe radio connection from the Handing Over RBC with the Accepting RBC is done here. DMI Object: ST03 |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 2: The on-board equipment shall ensure that actions related to RBC/RBC handover will be taken only once, although the train passes over a defined location more than once. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 106 |  | 5 |  |  | 615.00 |  | L2 |  | FS |  |  | The driver stops the train. |  | I |  | INT |  | The train will have to be stopped so that its max safe front end is very close to the location defined by D_LRBG + D_RBCTR, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL |  |  | L2 |  |  | FS |  | - |  |
|  | 107 |  | 6 |  |  | 530.00 |  | L2 |  | FS |  |  | The train moves backwards and overpasses the location defined by D_LRBG + D_RBCTR with its max safe front end again. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 108 |  | 7 |  |  | 530.00 |  | L2 |  | FS |  |  | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted |  | O |  | RTM |  | No position report is sent to the Handing over RBC. |  |  | L2 |  |  | FS |  | - |  |
|  | 109 |  | 8 |  |  | 530.00 |  | L2 |  | FS |  |  | No position report to the Handing over RBC is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 110 |  | 9 |  |  | 510.00 |  | L2 |  | FS |  |  | The driver stops the train. |  | I |  | INT |  | The brakes might have been applied. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 6: A L2/L3 MA is transmitted by radio and is received by the onboard equipment. The message is REJECTED. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 112 |  | 1 |  |  | 555.00 |  | L2 |  | FS |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received |  | I |  | RTM |  | The train receives Message 3: Movement Authority with, at least: Packet 15: Level 2/3 Movement Authority. Other packets can be included. SSP and gradient (new received or stored) cover the full length of the train The message is REJECTED as a consequence of the mode of the onboard is a mode where the MA is not accpeted o the level is a level where the RBC MA is rejected. |  |  | L2 |  |  | FS |  | - |  |
|  | 113 |  | 2 |  |  | 555.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 114 |  | 3 |  |  | 555.00 |  | L2 |  | FS |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | Nothing changes on the DMI |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 2: The on-board equipment shall ensure that actions related to RBC/RBC handover will be taken only once, although the train passes over a defined location more than once. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 11 |  |  | 555.00 |  | L2 |  | FS |  |  | The MA sent by the Handing over RBC is not displayed. |  | O |  | DMI |  | The MA has been rejected. |  |  | L2 |  |  | FS |  | - |  |
|  | 116 |  | 12 |  |  | 560.00 |  | L2 |  | FS |  |  | The train moves forward and overpasses the location defined by D_LRBG + D_RBCTR with its max safe front end again. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 117 |  | 13 |  |  | 560.00 |  | L2 |  | FS |  |  | NOT SA-DATA.Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted |  | O |  | RTM |  | No position report is sent to the Handing over RBC. |  |  | L2 |  |  | FS |  | - |  |
|  | 118 |  | 14 |  |  | 560.00 |  | L2 |  | FS |  |  | No position report to the Handing over RBC is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3150100: 3.15.1: RBC/RBC Handover Test Case 1: The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC; Send a position report to the Handing Over RBC when the maximum safe front end of the train passes the given location and Send a position report to the Handing Over RBC when the minimum safe rear end of the train passes the given location. Safe Radio Connection Indication is displayed continuously. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 38 |  |  | 696.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | Train min safe rear end location = LRBG + D_RBCTR The minimum safe rear end of the train passes the location of the RBC transition |  |  | L2 |  |  | FS |  | - |  |
|  | 120 |  | 39 |  |  | 696.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 is transmitted. |  | O |  | RTM |  | To the Handing Over RBC |  |  | L2 |  |  | FS |  | - |  |
|  | 121 |  | 40 |  |  | 696.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  | 1 |  |  | 696.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | FS |  | - |  |
|  | 124 |  | 2 |  |  | 696.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 125 |  | 3 |  |  | 696.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | FS |  | - |  |
|  | 126 |  | 4 |  |  | 696.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 127 |  | 5 |  |  | 696.00 |  | L2 |  | FS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | FS |  | - |  |
|  | 128 |  | 6 |  |  | 696.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | FS |  | - |  |
|  | 129 |  | 7 |  |  | 696.00 |  | L2 |  | FS |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 3150100: 3.15.1: RBC/RBC Handover Test Case 1: The on-board equipment is able to handle two communication sessions at the same time. When receiving an order to switch to another RBC at a given location, the on-board equipment shall establish the communication session with the Accepting RBC; Send a position report to the Handing Over RBC when the maximum safe front end of the train passes the given location and Send a position report to the Handing Over RBC when the minimum safe rear end of the train passes the given location. Safe Radio Connection Indication is displayed continuously. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 130 |  | 42 |  |  | 696.00 |  | L2 |  | FS |  |  | Train is moving. |  | I |  | INT |  | The RBC transition (handover) is performed |  |  | L2 |  |  | FS |  | - |  |
|  | 131 |  | 43 |  |  | 696.00 |  | L2 |  | FS |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed. |  | O |  | DMI |  | The indication to the driver is the same. Safe Connection indication of Accepting RBC |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 3: The on-board equipment shall ensure that actions related to track condition information, with the exception of big metal masses, will be taken only once, although the train passes over a defined location more than once. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 132 |  | 1 |  |  | 741.00 |  | L2 |  | FS |  |  | When the max safe front end of the train reaches the location defined by D_LRBG + D_TRACKCOND, the pantograph output of the TIU is activated (lower pantograph) |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 133 |  | 2 |  |  | 770.00 |  | L2 |  | FS |  |  | The driver stops the train. |  | I |  | INT |  | The train will have to be stopped so that its max safe front end is very close to the location defined by D_LRBG + D_TRACKCOND, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. _____ D_NVROLL set to 100 at step 65, so it is no problem to move backwards up to 100 m |  |  | L2 |  |  | FS |  | - |  |
|  | 134 |  | 3 |  |  | 741.00 |  | L2 |  | FS |  |  | The train moves backwards and overpasses the location defined by D_LRBG + D_TRACKCOND with its max safe front end. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 135 |  | 4 |  |  | 741.00 |  | L2 |  | FS |  |  | No new action is performed concerning the pantograph. |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 136 |  | 5 |  |  | 670.00 |  | L2 |  | FS |  |  | The driver stops the train. |  | I |  | INT |  | The brakes might have been applied. |  |  | L2 |  |  | FS |  | - |  |
|  | 137 |  | 6 |  |  | 741.00 |  | L2 |  | FS |  |  | The train moves forward and overpasses the location defined by D_LRBG + D_TRACKCOND with its max safe front end. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 7 |  |  | 741.00 |  | L2 |  | FS |  |  | No new action is performed concerning the pantograph. |  | O |  | TIU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 4: The on-board equipment shall ensure that the permission to initiate Reversing mode is only displayed once, right before passing over the end of the Reversing area, but it is not shown again after passing again over this location (due to backwards movement: initiated by the driver or due to roll-away, but not because the driver selects RV mode). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 |  | 1 |  |  | 830.00 |  | L2 |  | FS |  |  | The driver stops the train inside the area where reversing is permitted. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 140 |  | 2 |  |  | 830.00 |  | L2 |  | FS |  |  | The driver is informed that the reversing of movement is permitted. |  | O |  | DMI |  | The driver does not select RV mode |  |  | L2 |  |  | FS |  | - |  |
|  | 141 |  | 3 |  |  | 830.00 |  | L2 |  | FS |  |  | The information to the driver is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 142 |  | 4 |  |  | 830.00 |  | L2 |  | FS |  |  | The train starts moving again. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 5 |  |  | 930.00 |  | L2 |  | FS |  |  | The driver stops the train a bit further away from the end of the reversing area. |  | I |  | INT |  | The train will have to be stopped very close to the end of the reversing area, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL |  |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 6 |  |  | 900.00 |  | L2 |  | FS |  |  | The train moves backwards and overpasses the end of the reversing area again. |  | I |  | INT |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 145 |  | 7 |  |  | 870.00 |  | L2 |  | FS |  |  | The driver stops the train. |  | I |  | INT |  | The brakes might have been applied. |  |  | L2 |  |  | FS |  | - |  |
|  | 146 |  | 8 |  |  | 870.00 |  | L2 |  | FS |  |  | No information is displayed about the reversing of movement. |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 147 |  | 9 |  |  | 870.00 |  | L2 |  | FS |  |  | It is not possible to select RV mode. |  | O |  | DMI |  | - _____ The driver stops the train. |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 46: On Sight is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to OS mode. Transition Condition Id [40]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 150 |  | 3 |  |  | 950.00 |  | L2 |  | FS |  |  | SA-DATA. Indication with Euroradio Message 'Movement Authority' (NID_MESSAGE=3) containing packet 15, packet 80 (M_MAMODE = 00) is received |  | I |  | RTM |  | Only if in L2/L3 Reception of the mode transition announcement (mode transition order at current location) NID_PACKET = 80: Mode Profile M_MAMODE = 00: On sight |  |  | L2 |  |  | FS |  | - |  |
|  | 151 |  | 4 |  |  | 950.00 |  | L2 |  | FS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_PACKET= 80) is recorded |  | O |  | JRU |  | Only if in L2/L3 |  |  | L2 |  |  | FS |  | - |  |
|  | 152 |  | 5 |  |  | 950.00 |  | L2 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 153 |  | 6 |  |  | 950.00 |  | L2 |  | FS |  |  | The Mode symbol 'On Sight' is displayed |  | O |  | DMI |  | DMI Object: MO07 Or USE_FT4070201.23 |  |  | L2 |  |  | FS |  | - |  |
|  | 154 |  | 7 |  |  | 950.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit22=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  |  |  |
|  | 155 |  | 8 |  |  | 950.00 |  | L2 |  | FS |  |  | The Mode symbol 'Acknowledge On Sight' is displayed |  | O |  | DMI |  | DMI Object: MO08 |  |  | L2 |  |  | FS |  | - |  |
|  | 156 |  | 9 |  |  | 950.00 |  | L2 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 157 |  | 10 |  |  | 950.00 |  | L2 |  | FS |  |  | SA-DATA. Request with Euroradio message (NID_MESSAGE=136) containing packet 0 is transmitted |  | O |  | RTM |  | Message 136: 'Train Position Report' Only in L2 / L3 |  |  | L2 |  |  | FS |  | - |  |
|  | 158 |  | 11 |  |  | 950.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 159 |  | 12 |  |  | 950.00 |  | L2 |  | FS |  |  | The driver acknowledges 'On Sight' |  | I |  | DMI |  | Driver acknowledges within 5 sec after the change to OS The driver acknowledgement time (refer to Appendix A3.1) |  |  | L2 |  |  | FS |  | - |  |
|  | 160 |  | 13 |  |  | 950.00 |  | L2 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0000) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 161 |  | 14 |  |  | 950.00 |  | L2 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | Negative Testing |  |  | L2 |  |  | FS |  | - |  |
|  | 162 |  | 15 |  |  | 950.00 |  | L2 |  | FS |  |  | NOT The Status symbol 'Service Brake intervention' is displayed |  | O |  | DMI |  | Negative Testing |  |  | L2 |  |  | OS |  | - |  |
|  | 163 |  | 16 |  |  | 950.00 |  | L2 |  | OS |  |  | NOT SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | Negative Testing |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3060100: 3.6.1: Location Principles and Train Position and Train Orientation Test Case 5: An error in the radio messages sequence has been detected. The onboard equipment sends a radio message 136 'Train Position Report' containing the packet 4 'Error reporting' to the RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 166 |  | 3 |  |  | 1000.00 |  | L2 |  | OS |  |  | SA-DATA.Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received |  | I |  | RTM |  | A message 34 is received from the RBC with T_TRAIN 2; The time stamp of this message is smaller than the time stamp of the first one: T_TRAIN 2<T_TRAIN_1 |  |  | L2 |  |  | OS |  | - |  |
|  | 167 |  | 4 |  |  | 1000.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 9990500: A3.5: Handling of Actions in Specific Situations Test Case 5: The on-board equipment shall ensure that the information about track ahead free request is displayed only once, although the train passes over the start location of the area more than once. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 171 |  | 1 |  |  | 1100.00 |  | L2 |  | OS |  |  | ‘Track Ahead Free Request’ is displayed to the driver, according to the D_LRBG + D_TAFDISPLAY. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 172 |  | 2 |  |  | 1100.00 |  | L2 |  | OS |  |  | Display of ‘Track Ahead Free Request’ is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 173 |  | 3 |  |  | 1130.00 |  | L2 |  | OS |  |  | The driver acknowledges the ‘Track Ahead Free Request’ |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 174 |  | 4 |  |  | 1130.00 |  | L2 |  | OS |  |  | The driver acknowledgement is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 175 |  | 5 |  |  | 1130.00 |  | L2 |  | OS |  |  | SA-DATA.Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 176 |  | 6 |  |  | 1130.00 |  | L2 |  | OS |  |  | Radio message 149 is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 177 |  | 7 |  |  | 1130.00 |  | L2 |  | OS |  |  | ‘Track Ahead Free Request’ is removed |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 178 |  | 8 |  |  | 1130.00 |  | L2 |  | OS |  |  | Removal ‘Track Ahead Free Request’ is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 179 |  | 9 |  |  | 1160.00 |  | L2 |  | OS |  |  | The driver stops the train. |  | I |  | INT |  | The train will have to be stopped very close to the location defined by D_LRBG + D_TAFDISPLAY, since it will pass over this location in backwards direction. Backwards movements are limited by the variable D_NVROLL. |  |  | L2 |  |  | OS |  | - |  |
|  | 180 |  | 10 |  |  | 1100.00 |  | L2 |  | OS |  |  | The train moves backwards and overpasses the location defined by D_LRBG + D_TAFDISPLAY again. |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 181 |  | 11 |  |  | 1100.00 |  | L2 |  | OS |  |  | No ‘Track Ahead Free Request’ is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 182 |  | 12 |  |  | 1100.00 |  | L2 |  | OS |  |  | No ‘Track Ahead Free Request’ is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 183 |  | 13 |  |  | 1080.00 |  | L2 |  | OS |  |  | The driver stops the train. |  | I |  | INT |  | The brakes might have been applied. |  |  | L2 |  |  | OS |  | - |  |
|  | 184 |  | 14 |  |  | 1100.00 |  | L2 |  | OS |  |  | The train moves forward and overpasses the location defined by D_LRBG + D_TAFDISPLAY again. |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 185 |  | 15 |  |  | 1100.00 |  | L2 |  | OS |  |  | No ‘Track Ahead Free Request’ is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 186 |  | 16 |  |  | 1100.00 |  | L2 |  | OS |  |  | No ‘Track Ahead Free Request’ is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3160200: 3.16.2: Balises Test Case 17: No linking information is used on-board -A BG marked as unlinked is found -No message consistency problem The message is accepted. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 187 |  | 1 |  |  | 1200.00 |  | L2 |  | OS |  |  | One Balise Group message (N_TOTAL=1;Q_LINK=0) containing packet 72, packet 255 is received |  | I |  | BTM |  | No linking information is used on-board The train overpasses a BG marked as unlinked No message consistency problem |  |  | L2 |  |  | OS |  | - |  |
|  | 188 |  | 2 |  |  | 1200.00 |  | L2 |  | OS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 189 |  | 3 |  |  | 1200.00 |  | L2 |  | OS |  |  | The "plain text message" is displayed |  | O |  | DMI |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). The BG message is accepted. |  |  | L2 |  |  | OS |  | - |  |
|  | 190 |  | 4 |  |  | 1200.00 |  | L2 |  | OS |  |  | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded |  | O |  | JRU |  | Only if mode not PS, SH, SL or PT (with exception of L2/L3 and following reception of 'Recognition of Exit from TR mode' with a more recent time stamp). |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 193 |  | 1 |  |  | 1225.00 |  | L2 |  | OS |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 194 |  | 2 |  |  | 1225.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 5 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 5 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 5 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 43 : Balise Group BG 2 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 43 : Balise Group BG 2 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 49 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 51 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 53 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 56 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 12 d |  |  |
| L_DOUBTUNDER | 15 | 12 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 2 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 58 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 60 : Message 3 - Movement Authority (RBC n? 1) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 111 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 8 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 3000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 4000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 255 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 8 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 4000 d |  |  |
| V_STATIC | 7 | 127 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 131 d | RBC transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 129 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_RBCTR | 15 | 500 d |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RBC | 14 | 1616 d |  |  |
| NID_RADIO | 64 | 003265342102FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 3 d | National values |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 230 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_VALIDNV | 15 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| V_NVSHUNT | 7 | 6 d |  |  |
| V_NVSTFF | 7 | 8 d |  |  |
| V_NVONSIGHT | 7 | 6 d |  |  |
| V_NVLIMSUPERV | 7 | 20 d |  |  |
| V_NVUNFIT | 7 | 20 d |  |  |
| V_NVREL | 7 | 8 d |  |  |
| D_NVROLL | 15 | 100 d |  |  |
| Q_NVSBTSMPERM | 1 | 1 b |  |  |
| Q_NVEMRRLS | 1 | 0 b |  |  |
| Q_NVGUIPERM | 1 | 0 b |  |  |
| Q_NVSBFBPERM | 1 | 0 b |  |  |
| Q_NVINHSMICPERM | 1 | 0 b |  |  |
| V_NVALLOWOVTRP | 7 | 0 d |  |  |
| V_NVSUPOVTRP | 7 | 6 d |  |  |
| D_NVOVTRP | 15 | 200 d |  |  |
| T_NVOVTRP | 8 | 60 d |  |  |
| D_NVPOTRP | 15 | 200 d |  |  |
| M_NVCONTACT | 2 | 10 b |  |  |
| T_NVCONTACT | 8 | 255 d |  |  |
| M_NVDERUN | 1 | 1 b |  |  |
| D_NVSTFF | 15 | 32767 d |  |  |
| Q_NVDRIVER_ADHES | 1 | 0 b |  |  |
| A_NVMAXREDADH1 | 6 | 010100 b |  |  |
| A_NVMAXREDADH2 | 6 | 001110 b |  |  |
| A_NVMAXREDADH3 | 6 | 001110 b |  |  |
| Q_NVLOCACC | 6 | 12 d |  |  |
| M_NVAVADH | 5 | 00001 b |  |  |
| M_NVEBCL | 4 | 0001 b |  |  |
| Q_NVKINT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 58 d | Position report parameters |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 56 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| T_CYCLOC | 8 | 255 d |  |  |
| D_CYCLOC | 15 | 32767 d |  |  |
| M_LOC | 3 | 001 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 68 d | Track condition |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 65 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TRACKINIT | 1 | 0 b |  |  |
| D_TRACKCOND | 15 | 700 d |  |  |
| L_TRACKCOND | 15 | 100 d |  |  |
| M_TRACKCOND | 4 | 0011 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 138 d | Reversing area information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 55 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STARTREVERSE | 15 | 700 d |  |  |
| L_REVERSEAREA | 15 | 100 d |  |  |
| NID_PACKET | 8 | 139 d | Reversing supervision information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 47 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REVERSE | 15 | 32767 d |  |  |
| V_REVERSE | 7 | 40 d |  |  |

| - Step 69 : Message 155 - Initiation of a Communication Session (RBC n? 2) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 71 : Message 32 - RBC/RIU System Version (RBC n? 2) - Distance 400.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 73 : Message 159 - Session Established (RBC n? 2) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 75 : Message 24 - General Message (RBC n? 2) - Distance 400.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 21 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 76 d | Packet for sending fixed text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 92 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| Q_TEXT | 8 | 0 d |  |  |

| - Step 78 : Message 129 - Validated Train Data (RBC n? 2) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 300 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 80 : Message 8 - Acknowledgement of Train Data (RBC n? 2) - Distance 400.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 82 : Message 136 - Train Position Report (RBC n? 1) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 300 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 84 : Message 136 - Train Position Report (RBC n? 2) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 300 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 89 : Message 136 - Train Position Report (RBC n? 1) - Distance 560.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 460 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 12 d |  |  |
| L_DOUBTUNDER | 15 | 12 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 8 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 91 : Message 136 - Train Position Report (RBC n? 2) - Distance 560.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 26 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 129 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 460 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 12 d |  |  |
| L_DOUBTUNDER | 15 | 12 d |  |  |
| Q_LENGTH | 2 | 1 d |  |  |
| L_TRAININT | 15 | 40 d |  |  |
| V_TRAIN | 7 | 8 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 112 : Message 3 - Movement Authority (RBC n? 1) - Distance 555.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 39 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 8 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 78 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 2000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_GRADIENT | 15 | 2000 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 255 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 86 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 8 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| D_STATIC | 15 | 2000 d |  |  |
| V_STATIC | 7 | 127 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 120 : Message 136 - Train Position Report (RBC n? 1) - Distance 696.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 596 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 123 : Message 24 - General Message (RBC n? 1) - Distance 696.00 m Delay : 2.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 352 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |

| - Step 125 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 696.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 127 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 696.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 15.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |

| - Step 150 : Message 3 - Movement Authority (RBC n? 2) - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 29 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 1023 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 127 d |  |  |
| L_MAMODE | 15 | 2000 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 157 : Message 136 - Train Position Report (RBC n? 2) - Distance 950.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 850 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 166 : Message 34 - Track Ahead Free Request (RBC n? 2) - Distance 1000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 34 d |  |  |
| L_MESSAGE | 10 | 16 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_REF | 16 | 0 d |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_TAFDISPLAY | 15 | 1000 d |  |  |
| L_TAFDISPLAY | 15 | 100 d |  |  |

| - Step 175 : Message 149 - Track Ahead Free Granted (RBC n? 2) - Distance 1130.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 149 d |  |  |
| L_MESSAGE | 10 | 26 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 129 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048578 d |  |  |
| D_LRBG | 15 | 1030 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 12 d |  |  |
| L_DOUBTUNDER | 15 | 12 d |  |  |
| Q_LENGTH | 2 | 1 d |  |  |
| L_TRAININT | 15 | 40 d |  |  |
| V_TRAIN | 7 | 8 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 187 : Balise Group BG 999 d 1/2 - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 999 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 72 d | Packet for sending plain text messages |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 276 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_TEXTCLASS | 2 | 00 b |  |  |
| Q_TEXTDISPLAY | 1 | 0 b |  |  |
| D_TEXTDISPLAY | 15 | 32767 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| L_TEXTDISPLAY | 15 | 32767 d |  |  |
| T_TEXTDISPLAY | 10 | 1023 d |  |  |
| M_MODETEXTDISPLAY | 4 | 1111 b |  |  |
| M_LEVELTEXTDISPLAY | 3 | 101 b |  |  |
| Q_TEXTCONFIRM | 2 | 00 b |  |  |
| L_TEXT | 8 | 23 d |  |  |
| X_TEXT | 8 | N |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | i |  |  |
| X_TEXT | 8 | o |  |  |
| X_TEXT | 8 | n |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | v |  |  |
| X_TEXT | 8 | a |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | u |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | s |  |  |
| X_TEXT | 8 |  |  |  |
| X_TEXT | 8 | d |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | l |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | t |  |  |
| X_TEXT | 8 | e |  |  |
| X_TEXT | 8 | d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 187 : Balise Group BG 999 d 2/2 - Distance 1205.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 999 d |  |  |
| NID_BG | 14 | 999 d |  |  |
| Q_LINK | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |