| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5040500_03_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5040500_03_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 11:50 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | INECO |

# Sequence description

Degraded Situation of Procedure Start of mission

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L0 [74.-1.1] |  |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L0 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L0 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 3: Within the scope of the Start of Mission procedure in SB mode, the selection to re-enter the Level shall be possible at S10 and S20. Then the process shall go to S2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Level> button in the <Main> window is enabled |  | O |  | DMI |  | In addition to all possibilities offered at S10 and S20. |  |  | L0 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects from the main window list to re-enter the ETCS level. |  | I |  | DMI |  | The process goes to S2 inside the SoM-procedure |  |  | L0 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The <ERTMS/ETCS Level> window is displayed. |  | O |  | DMI |  | The on-board allows the driver to re-enter the ETCS level. |  |  | L0 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects "Level <X>" |  | I |  | DMI |  | The driver re-enters the ERTMS/ETCS level. X= different or can be the same as the previous one. _____ The Driver selects L1 [59.-1.1] |  |  | L0 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110) is recorded |  | O |  | JRU |  | Check that M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110 Is one of this value |  |  | L0 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=X) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | The Level symbol "Level X" is displayed. |  | O |  | DMI |  | DMI Object: LEXX Depends on the driver level selection _____ The process go to S10 |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 11: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 36 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to all possibilities offered at S10 |  |  | L1 |  |  | SB |  | - |  |
|  | 37 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 38 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L1 |  |  | SB |  | - |  |
|  | 39 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 40 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The Mode symbol "Acknowledge Staff Responsible" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode. DMI Object: MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 41 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit25=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 42 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges "Staff Responsible" |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 43 |  | 8 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | Ack of Staff Responsible mode As soon as the driver acknowledges the on-board equipment switches to SR mode |  |  | L1 |  |  | SB |  | - |  |
|  | 44 |  | 9 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L1 |  |  | SR |  | - |  |
|  | 45 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol "Staff Responsible" is displayed |  | O |  | DMI |  | . DMI Object: Symbol MO09 |  |  | L1 |  |  | SR |  | - |  |
|  | 46 |  | 11 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 9: Desks are closed. Transition Condition Id [28]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 47 |  | 1 |  |  | 0.00 |  | L1 |  | SR |  |  | CAB not active |  | I |  | TIU |  | Driver closes the desk. |  |  | L1 |  |  | SR |  | - |  |
|  | 48 |  | 2 |  |  | 0.00 |  | L1 |  | SR |  |  | CAB STATUS (NID_MESSAGE_JRU=38; M_CAB_STATUS=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 49 |  | 3 |  |  | 0.00 |  | L1 |  | SR |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 50 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 51 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 52 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 25: During a start of mission, the driver shall have the opportunity to set a new VBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 53 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment offers the possibility to set/remove a Virtual Balise Cover. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 54 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver chooses to set/remove a Virtual Balise Cover |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 55 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver sets and validate a new VBC. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 56 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The new VBC is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 57 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  |  | SB |  | - |  |
|  | 58 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  |  | SB |  | - |  |
|  | 59 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 61 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 62 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 63 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 66 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L1 |  |  | SB |  | - |  |
|  | 67 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 68 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 70 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 71 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 72 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - _____ The process is at S20 |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 7: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment shall offer the driver the possibility to set/remove a Virtual Balise Cover, only at S10 and S20. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 74 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The <Set VBC> button or <Remove VBC> button in the <Setting> window is enabled |  | O |  | DMI |  | In addition to all possibilities listed in the main window. |  |  | L1 |  |  | SB |  | - |  |
|  | 75 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects one of the option 'Set VBC' or 'Remove VBC' |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 76 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment allows the driver to set or to remove the VBC. |  | O |  | DMI |  | Depending on driver selection window 'Set VBC' or 'Remove VBC' is display |  |  | L1 |  |  | SB |  | - |  |
|  | 77 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver sets a VBC and validates "Set VBC" |  | I |  | DMI |  | The driver enters a new VBC code. See paragraph 11.3.12.5 for Set VBC code and 11.3.13.5 for Remove VBC code ( ERA-ERTMS-015560 v3.3.0) |  |  | L1 |  |  | SB |  | - |  |
|  | 78 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | VIRTUAL BALISE COVER SET BY THE DRIVER (NID_MESSAGE_JRU=28) is recorded |  | O |  | JRU |  | - _____ The process go to S20 |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 3: Within the scope of the Start of Mission procedure in SB mode, the selection to re-enter the Level shall be possible at S10 and S20. Then the process shall go to S2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The <Level> button in the <Main> window is enabled |  | O |  | DMI |  | In addition to all possibilities offered at S10 and S20. |  |  | L1 |  |  | SB |  | - |  |
|  | 80 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects from the main window list to re-enter the ETCS level. |  | I |  | DMI |  | The process goes to S2 inside the SoM-procedure |  |  | L1 |  |  | SB |  | - |  |
|  | 81 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The <ERTMS/ETCS Level> window is displayed. |  | O |  | DMI |  | The on-board allows the driver to re-enter the ETCS level. |  |  | L1 |  |  | SB |  | - |  |
|  | 82 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects "Level <X>" |  | I |  | DMI |  | The driver re-enters the ERTMS/ETCS level. X= different or can be the same as the previous one. _____ The Driver selects L0 [74.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 83 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110) is recorded |  | O |  | JRU |  | Check that M_DRIVERACTIONS = 0010 0010/ 0010 0011/ 0010 0100/ 0010 0101/ 0010 0110 Is one of this value |  |  | L1 |  |  | SB |  | - |  |
|  | 84 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=X) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 85 |  | 7 |  |  | 0.00 |  | L0 |  | SB |  |  | The Level symbol "Level X" is displayed. |  | O |  | DMI |  | DMI Object: LEXX Depends on the driver level selection _____ The process go to S10 |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 12: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to possibilities offered at S10 |  |  | L0 |  |  | SB |  | - |  |
|  | 87 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 88 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L0 |  |  | SB |  | - |  |
|  | 89 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 90 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | The Mode symbol "Acknowledge Unfitted" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 91 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 92 |  | 7 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver acknowledges "Unfitted" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 93 |  | 8 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded |  | O |  | JRU |  | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode |  |  | L0 |  |  | SB |  | - |  |
|  | 94 |  | 9 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L0 |  |  | UN |  | - |  |
|  | 95 |  | 10 |  |  | 0.00 |  | L0 |  | UN |  |  | The Mode symbol "Unfitted" is displayed |  | O |  | DMI |  | DMI Object: Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 96 |  | 11 |  |  | 0.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 98 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=0 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| Step 32: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=1 | 0 s |

| Step 82: DMI Events Distance 0.00 m |  |  |  |
| --- | --- | --- | --- |
| Event Id. | Event Name | Data | Delay |
| 29 | Request Change Level | LEVEL=0 | 0 s |