| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5060200_02_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5060200_02_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 11:59 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | RINA |

# Sequence description

Req 5.6.2: Table of requirements for "Shunting Initiated by Driver" procedure.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | NP |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ Driver select Level 1 [59.-1.1] |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 2: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An ongoing mission shall NOT exist. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver presses 'Shunting' button. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver selection is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The SH symbol, MO01 is DISPLAYED. |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SH |  |  | The displayed symbol for SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4080408: 4.8.4: Accepted Information depending on the modes - Danger for SH information Test Case 1: The train is in shunting mode and overpasses a balise with the packet number 132 (danger for shunting information). The train will be tripped if it’s in shunting mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 34 |  | 1 |  |  | 100.00 |  | L1 |  | SH |  |  | One Balise Group message (N_TOTAL=0; NID_C=<comment>) containing packet 132 "Danger for shunting information" (Q_ASPETC = 0) is received |  | I |  | BTM |  | One of the balises in BGa has a packet number 132 (Danger for shunting information) with the variable Q_ASPECT set to zero. If in L0/LNTC an immediate level transition to level 1/2/3 shall be also included (packet 41) |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 2 |  |  | 100.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=7) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 17: Indication of Trip mode and the reason to the driver. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 1 |  |  | 100.00 |  | L1 |  | TR |  |  | The Trip mode (TR) is permanently DISPLAYED |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | TR |  | - |  |
|  | 42 |  | 2 |  |  | 100.00 |  | L1 |  | TR |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The Trip Reason is related to the conditions (SRS 4.6.3) that lead to the transition to TR. |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 32: The train is in Shunting mode in L1, L2, or L3, and overpasses a balise with the packet number 132 (danger for shunting information) with the variable Q_ASPECT=0 ( which means :Stop if in SH mode)) AND (override is not active). Transition Condition Id [49]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 43 |  | 4 |  |  | 100.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit19=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 44 |  | 5 |  |  | 100.00 |  | L1 |  | TR |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit15=1>) is recorded |  | O |  | JRU |  | SH stop order |  |  | L1 |  |  | TR |  | - |  |
|  | 45 |  | 6 |  |  | 100.00 |  | L1 |  | TR |  |  | Emergency brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 46 |  | 7 |  |  | 100.00 |  | L1 |  | TR |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 47 |  | 8 |  |  | 100.00 |  | L1 |  | TR |  |  | The Status symbol ' Emergency Brake ' is displayed |  | O |  | DMI |  | DMI Object: Symbol ST01 |  |  | L1 |  |  | TR |  | - |  |
|  | 48 |  | 9 |  |  | 100.00 |  | L1 |  | TR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 49 |  | 1 |  |  | 110.00 |  | L1 |  | TR |  |  | No Request for driver acknowledgement to Train Trip is DISPLAYED |  | O |  | DMI |  | The train is still moving |  |  | L1 |  |  | TR |  | - |  |
|  | 50 |  | 2 |  |  | 154.00 |  | L1 |  | TR |  |  | The request for driver acknowledgement to Train Trip/ symbol MO05 is DISPLAYED |  | O |  | DMI |  | As soon as the train has come to standstill |  |  | L1 |  |  | TR |  | - |  |
|  | 51 |  | 3 |  |  | 154.00 |  | L1 |  | TR |  |  | The displayed symbol MO05is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 52 |  | 4 |  |  | 154.00 |  | L1 |  | TR |  |  | The driver acknowledges the TR Mode |  | I |  | DMI |  | - |  |  | L1 |  |  | TR |  | - |  |
|  | 53 |  | 5 |  |  | 154.00 |  | L1 |  | TR |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 54 |  | 6 |  |  | 154.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is DISPLAYED |  | O |  | DMI |  | The on-board equipment switches to PT mode. |  |  | L1 |  |  | PT |  | - |  |
|  | 55 |  | 7 |  |  | 154.00 |  | L1 |  | PT |  |  | The new mode PT/ symbol MO06 is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 16: An indication of Post Trip mode (PT) is displayed to the driver as long as the current mode is PT. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 154.00 |  | L1 |  | PT |  |  | The on-board equipment has just changed to PT. The Post Trip mode (PT) is permanently DISPLAYED to the driver. |  | O |  | DMI |  | Indication of mode PT to the driver while the system is in mode PT. |  |  | L1 |  |  | PT |  | - |  |
|  | 57 |  | 2 |  |  | 154.00 |  | L1 |  | PT |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 13: Before the train reaches standstill, no message to acknowledge Train Trip is displayed. The train reaches standstill in level 1, 2 or 3 and TR mode. The train/engine performs transition to PT after Driver acknowledgement of the TRIP mode. In level 2 or level 3, a communication session is opened and the On-Board sends a position report to the track-side when the mode changes Transition Id [7]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 10 |  |  | 154.00 |  | L1 |  | PT |  |  | The reason (Trip Reason) is DISPLAYED |  | O |  | DMI |  | The trip reason is displayed while the train is in PT mode |  |  | L1 |  |  | PT |  | - |  |
|  | 61 |  | 11 |  |  | 154.00 |  | L1 |  | PT |  |  | Emergency brake not commanded |  | O |  | TIU |  | Release of emergency brake, because the train is at standstill and the driver has acknowledged the trip. |  |  | L1 |  |  | PT |  | - |  |
|  | 62 |  | 12 |  |  | 154.00 |  | L1 |  | PT |  |  | EMERGENCY BRAKE COMMAND STATE (NID_MESSAGE_JRU=3; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 63 |  | 13 |  |  | 154.00 |  | L1 |  | PT |  |  | The Status symbol "Service Brake or Emergency Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 64 |  | 14 |  |  | 154.00 |  | L1 |  | PT |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5110200: 5.11.2: Table of requirements for 'Train Trip' procedure Test Case 8: In PT mode, the ERTMS/ETCS on-board equipment shall NOT offer the possibility to the driver to select „start', because the train data is not available. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 65 |  | 1 |  |  | 154.00 |  | L1 |  | PT |  |  | The <START> button in the <MAIN> window is disabled. |  | O |  | DMI |  | The On-Board equipment does not allows the driver to select 'Start' because no valid Train data available on-board. |  |  | L1 |  |  | PT |  | - |  |
|  | 66 |  | 2 |  |  | 154.00 |  | L1 |  | PT |  |  | The <SHUNTING> button in the <MAIN> window is enable. |  | O |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 5: Driver selects 'Shunting', train is at standstill (Transition [5]) and the system switches to SH mode. An ongoing mission exists.'End of Mission' procedure triggered. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  |  | 154.00 |  | L1 |  | PT |  |  | The driver presses 'shunting' button |  | I |  | DMI |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 70 |  | 2 |  |  | 154.00 |  | L1 |  | PT |  |  | Driver selection is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | PT |  | - |  |
|  | 71 |  | 3 |  |  | 154.00 |  | L1 |  | PT |  |  | The SH symbol is DISPLAYED |  | O |  | DMI |  | The system switches to SH SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). |  |  | L1 |  |  | SH |  | - |  |
|  | 72 |  | 4 |  |  | 154.00 |  | L1 |  | SH |  |  | The transition to SH mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 73 |  | 5 |  |  | 154.00 |  | L1 |  | SH |  |  | The change of permitted speed is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 74 |  | 6 |  |  | 154.00 |  | L1 |  | SH |  |  | Driver requests the permitted speed |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 75 |  | 7 |  |  | 154.00 |  | L1 |  | SH |  |  | Driver request is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 76 |  | 8 |  |  | 154.00 |  | L1 |  | SH |  |  | The permitted speed is DISPLAYED |  | O |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 18: The on-board is in SH mode and the driver selects 'exit shunting'. The on-board equipment immediately switches to SB mode if the train is at standstill. Transition Condition Id [19]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 77 |  | 1 |  |  | 300.00 |  | L1 |  | SH |  |  | The Train is at standstill |  | I |  | INT |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 78 |  | 2 |  |  | 300.00 |  | L1 |  | SH |  |  | The driver selects 'EXIT SHUNTING' |  | I |  | DMI |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 79 |  | 3 |  |  | 300.00 |  | L1 |  | SH |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0001) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SH |  | - |  |
|  | 80 |  | 4 |  |  | 300.00 |  | L1 |  | SH |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | The on-board equipment switches to SB mode. |  |  | L1 |  |  | SB |  | - |  |
|  | 81 |  | 5 |  |  | 300.00 |  | L1 |  | SB |  |  | The Mode symbol 'Stand By' is displayed |  | O |  | DMI |  | DMI Object: symbol MO13 |  |  | L1 |  |  | SB |  | - |  |
|  | 82 |  | 6 |  |  | 300.00 |  | L1 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit28=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 8: The on-board equipment allows the driver to validate an existing Driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 83 |  | 1 |  |  | 300.00 |  | L1 |  | SB |  |  | There is an initial Driver ID displayed. The on-board equipment allows the driver to simply revalidate it. |  | O |  | DMI |  | A stored on-board Driver ID is displayed to the driver since the status of the Driver ID is INVALID. |  |  | L1 |  |  | SB |  | - |  |
|  | 84 |  | 2 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver validates the Driver Id. |  | I |  | DMI |  | The on-board equipment allows the driver to simply revalidate an existing Driver ID without entering a new Driver ID. |  |  | L1 |  |  | SB |  | - |  |
|  | 85 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 86 |  | 1 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 87 |  | 2 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 88 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 89 |  | 4 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 2: The on-board equipment requests the driver to validate an existing complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is invalid (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 91 |  | 1 |  |  | 300.00 |  | L1 |  | SB |  |  | A complete set of Train Data is displayed to the driver The driver is allowed to validate the train data. |  | O |  | DMI |  | The driver might be requested to validate the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but not the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 92 |  | 2 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to simply validate the existing set of Train Data. |  |  | L1 |  |  | SB |  | - |  |
|  | 93 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 94 |  | 4 |  |  | 300.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 45: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'invalid', the ERTMS/ETCS on-board equipment requests the driver to re-validate the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 96 |  | 1 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver is requested to re-validate the Train running number. |  | O |  | DMI |  | Displayed Train Running Number: NID_OPERATIONAL_1 |  |  | L1 |  |  | SB |  | - |  |
|  | 97 |  | 2 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver confirms the "Train Running Number" [MAIN] |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 98 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 1 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 101 |  | 2 |  |  | 300.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 102 |  | 3 |  |  | 300.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 103 |  | 4 |  |  | 300.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 104 |  | 5 |  |  | 300.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 105 |  | 6 |  |  | 300.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 106 |  | 7 |  |  | 300.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 107 |  | 8 |  |  | 300.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 108 |  | 9 |  |  | 300.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 109 |  | 10 |  |  | 300.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 4070201: 4.7.2: DMI versus Mode Table - Output Test Case 21: An indication of SR is displayed to the driver as long as the current mode is SR. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 110 |  | 1 |  |  | 300.00 |  | L1 |  | SR |  |  | Permanent Indication of mode SR to the driver |  | O |  | DMI |  | The mode indication may not be permanent if the driver uses the menus. |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 27: The on-board equipment allows the driver to view the current Driver ID at any time. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  |  | 320.00 |  | L1 |  | SR |  |  | The driver presses "Data view" |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 112 |  | 2 |  |  | 320.00 |  | L1 |  | SR |  |  | The data from "Data View" window are displayed |  | O |  | DMI |  | The displayed data must contain the Driver ID. |  |  | L1 |  |  | SR |  | - |  |
|  | 113 |  | 3 |  |  | 320.00 |  | L1 |  | SR |  |  | The driver closes the "Data View" window |  | I |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 114 |  | 1 |  |  | 350.00 |  | L1 |  | SR |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 115 |  | 2 |  |  | 350.00 |  | L1 |  | SR |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 116 |  | 3 |  |  | 350.00 |  | L1 |  | SR |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | SR |  | - |  |
|  | 117 |  | 4 |  |  | 350.00 |  | L2 |  | SR |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 1 |  |  | 350.00 |  | L2 |  | SR |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | SR |  | - |  |
|  | 119 |  | 2 |  |  | 350.00 |  | L2 |  | SR |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 120 |  | 3 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | SR |  | - |  |
|  | 121 |  | 4 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 122 |  | 5 |  |  | 350.00 |  | L2 |  | SR |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 123 |  | 6 |  |  | 350.00 |  | L2 |  | SR |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 124 |  | 7 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 125 |  | 8 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 126 |  | 9 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SR |  | - |  |
|  | 127 |  | 10 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 128 |  | 11 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 129 |  | 12 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080401: 4.8.4: Accepted Information depending on the modes - Acknowledgement of train data Test Case 1: Radio message with new Train Data and acknowledge of the RBC |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 131 |  | 1 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) is transmitted |  | O |  | RTM |  | Validated train data |  |  | L2 |  |  | SR |  | - |  |
|  | 132 |  | 2 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=129) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 133 |  | 3 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 134 |  | 4 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9; NID_MESSAGE=8) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | Feature 4080425: 4.8.4: Accepted Information depending on the modes - Movement Authority Test Case 3: New Level 2/3 MA is ACCEPTED by the on-board equipment because the SSP (Static Speed Profile) and GP (Gradient Profile) sent together with the MA on-board covers the whole length of the MA. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 350.00 |  | L2 |  | SR |  |  | SA-Data.Indication with Euroradio message (NID_MESSAGE=3) containing packet 15 (MA), packet 21 (Gradient), packet 27 (SSP) is received |  | I |  | RTM |  | Track description sending is optional New track description (or stored one if not sent) covers the movement authority distance |  |  | L2 |  |  | SR |  | - |  |
|  | 136 |  | 2 |  |  | 350.00 |  | L2 |  | SR |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SR |  | - |  |
|  | 137 |  | 3 |  |  | 350.00 |  | L2 |  | SR |  |  | The speed and distance monitoring information is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 138 |  | 4 |  |  | 350.00 |  | L2 |  | FS |  |  | SPEED AND DISTANCE MONITORING INFORMATION (NID_MESSAGE_JRU=20) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 6: Driver selects 'Shunting' and train is at standstill, 'Shunting granted by RBC' is received and the system switches to SH mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 139 |  | 1 |  |  | 800.00 |  | L2 |  | FS |  |  | The driver presses 'Shunting' button |  | I |  | DMI |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 140 |  | 2 |  |  | 800.00 |  | L2 |  | FS |  |  | Driver action is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 141 |  | 3 |  |  | 800.00 |  | L2 |  | FS |  |  | SA-DATA.Request with Euroradio Message "Request for Shunting" (NID_MESSAGE=130) |  | O |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 142 |  | 4 |  |  | 800.00 |  | L2 |  | FS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10; NID_MESSAGE=130) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 143 |  | 5 |  |  | 800.00 |  | L2 |  | FS |  |  | In the main window, the hourglass symbol is DISPLAYED |  | O |  | DMI |  | This hourglass symbol is ST05 (according to the document ERA_ERTMS_015560 v330), which shall be displayed to show that the onboard awaits an answer from the RBC |  |  | L2 |  |  | FS |  | - |  |
|  | 144 |  | 6 |  |  | 800.00 |  | L2 |  | FS |  |  | Message 28 ('SH Authorised') is received by the on- board system |  | I |  | RTM |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 145 |  | 7 |  |  | 800.00 |  | L2 |  | FS |  |  | Message 28 is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | FS |  | - |  |
|  | 146 |  | 8 |  |  | 800.00 |  | L2 |  | FS |  |  | The SH symbol is DISPLAYED. |  | O |  | DMI |  | SH symbol is MO01( according to the document ERA_ERTMS_015560 v330). The system switches to SH mode |  |  | L2 |  |  | SH |  | - |  |
|  | 147 |  | 9 |  |  | 800.00 |  | L2 |  | SH |  |  | The displayed symbol MO01 for SH mode is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | Feature 5050300: 5.5.3: End of Mission Procedure Test Case 3: Inform track-side about End of Mission procedure. A communication session with RBC exists. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 149 |  | 1 |  |  | 800.00 |  | L2 |  | SH |  |  | SA-DATA. Request with Euroradio Message "End of Mission" (NID_MESSAGE=150) containing packet 0 or packet 1 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | 150 |  | 2 |  |  | 800.00 |  | L2 |  | SH |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 152 |  | 1 |  |  | 800.00 |  | L2 |  | SH |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | SH |  | - |  |
|  | 153 |  | 2 |  |  | 800.00 |  | L2 |  | SH |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | 154 |  | 3 |  |  | 800.00 |  | L2 |  | SH |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | SH |  | - |  |
|  | 155 |  | 4 |  |  | 800.00 |  | L2 |  | SH |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | 156 |  | 5 |  |  | 800.00 |  | L2 |  | SH |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | SH |  | - |  |
|  | 157 |  | 6 |  |  | 800.00 |  | L2 |  | SH |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | SH |  | - |  |
|  | 158 |  | 7 |  |  | 800.00 |  | L2 |  | SH |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | SH |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 6: Driver selects 'Shunting' and train is at standstill, 'Shunting granted by RBC' is received and the system switches to SH mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 159 |  | 11 |  |  | 800.00 |  | L2 |  | SH |  |  | Train is moving |  | I |  | INT |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | 160 |  | 12 |  |  | 900.00 |  | L2 |  | SH |  |  | One Balise Group message is received |  | I |  | BTM |  | BG b was not included in the old list of balises for SH area |  |  | L2 |  |  | SH |  | - |  |
|  | 161 |  | 13 |  |  | 900.00 |  | L2 |  | SH |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SH |  | - |  |
|  | 162 |  | 14 |  |  | 900.00 |  | L2 |  | SH |  |  | The Mode symbol "Shunting" is displayed |  | O |  | DMI |  | No Trip after a BG b not in the old list of balises for SH area, because the list was deleted |  |  | L2 |  |  | SH |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 163 |  | 1 |  |  | 1050.00 |  | L2 |  | SH |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train reaches standstill. |  |  | L2 |  |  | NP |  | - |  |
|  | 164 |  | 2 |  |  | 1050.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 34 : Balise Group BG 3 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 132 d | Danger for shunting information |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 24 d |  |  |
| Q_ASPECT | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 34 : Balise Group BG 3 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 114 : Balise Group BG 04 d 1/2 - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 3 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 114 : Balise Group BG 04 d 2/2 - Distance 355.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 124 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 126 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 350.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |

| - Step 128 : Message 159 - Session Established (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 131 : Message 129 - Validated Train Data (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 2 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 126 d |  |  |
| NC_CDTRAIN | 4 | 7 d |  |  |
| NC_TRAIN | 15 | 4 d |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 0000 0001 b |  |  |
| M_AXLELOADCAT | 7 | 7 d |  |  |
| M_AIRTIGHT | 2 | 01 b |  |  |
| N_AXLE | 10 | 4 d |  |  |
| N_ITER | 5 | 1d |  |  |
| M_VOLTAGE | 4 | 3 |  |  |
| NID_CTRACTION | 10 | 15 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_NTC | 8 | 2 d |  |  |
| NID_NTC | 8 | 3 d |  |  |

| - Step 133 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 350.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 135 : Message 3 - Movement Authority (RBC n? 1) - Distance 350.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 50 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 49 d | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 111 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 112 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 550 d |  |  |
| M_MAMODE | 2 | 01 b |  |  |
| V_MAMODE | 7 | 4 d |  |  |
| L_MAMODE | 15 | 300 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 1 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 141 : Message 130 - Request for Shunting (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 130 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 450 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 144 : Message 28 - SH Authorised (RBC n? 1) - Distance 800.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 28 d |  |  |
| L_MESSAGE | 10 | 19 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_PACKET | 8 | 49 d | List of balises for SH area |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 43 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 33 d |  |  |

| - Step 149 : Message 150 - End of Mission (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 150 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| D_LRBG | 15 | 450 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 3 d |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 152 : Message 24 - General Message (RBC n? 1) - Distance 800.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |

| - Step 154 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 156 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 800.00 m Delay : 10.00 s Conditional Message Id : 156 Back Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048580 d |  |  |

| - Step 160 : Balise Group BG 33 d 1/2 - Distance 900.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 33 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 160 : Balise Group BG 33 d 2/2 - Distance 905.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 33 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |