| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_5100400_04_LNTC_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_5100400_04_LNTC_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 13:08 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | INECO |

# Sequence description

Driver Ack inside the expectation window. Several level transition from and to LNTC are tested here.

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | SB |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L0 [74.-1.1] |  |  | L0 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L0 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L0 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 27: The driver requires 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects 'Start' . |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU. |  | O |  | JRU |  | Triggered by: The driver acts on the on-board MMI. |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 58: After having selected the start, an acknowledgement request for UN mode is displayed to the driver. The driver acknowledges UN mode. Mode is changed to Unfitted. Transition Condition Id [60]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 33 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The Mode symbol 'Acknowledge Unfitted' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 34 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 35 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver acknowledges 'Unfitted' |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 36 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0100 ) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 37 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 38 |  | 6 |  |  | 0.00 |  | L0 |  | UN |  |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object : Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 39 |  | 7 |  |  | 0.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 40 |  | 8 |  |  | 0.00 |  | L0 |  | UN |  |  | Any Message ( NID_MESSAGE_JRU=all; NID_LRBG= 16777215) is recorded. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 41 |  | 1 |  |  | 50.00 |  | L0 |  | UN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC Announced |  |  | L0 |  |  | UN |  | - |  |
|  | 42 |  | 2 |  |  | 50.00 |  | L0 |  | UN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE08 |  |  | L0 |  |  | UN |  | - |  |
|  | 43 |  | 3 |  |  | 50.00 |  | L0 |  | UN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L0 |  |  | UN |  | - |  |
|  | 44 |  | 4 |  |  | 50.00 |  | L0 |  | UN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 1: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L0 to LNTC. The driver acknowledges before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 45 |  | 1 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 |  |  | L0 |  |  | UN |  | - |  |
|  | 46 |  | 2 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 47 |  | 3 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L0 |  |  | UN |  | - |  |
|  | 48 |  | 4 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 49 |  | 5 |  |  | 450.00 |  | L0 |  | UN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver ACKNOWLEDGES the level transition |  |  | L0 |  |  | UN |  | - |  |
|  | 50 |  | 6 |  |  | 450.00 |  | L0 |  | UN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 51 |  | 7 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L0 |  |  | UN |  | - |  |
|  | 52 |  | 8 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 53 |  | 9 |  |  | 450.00 |  | L0 |  | UN |  |  | The Level symbol "Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE08 |  |  | L0 |  |  | UN |  | - |  |
|  | 54 |  | 10 |  |  | 450.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 16: The train receives an order to switch immediately from L0 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 1 |  |  | 500.00 |  | L0 |  | UN |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_STMNTC |  | I |  | BTM |  | 'The onboard equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L0 |  |  | UN |  | - |  |
|  | 57 |  | 2 |  |  | 500.00 |  | L0 |  | UN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 58 |  | 3 |  |  | 500.00 |  | L0 |  | UN |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L0 |  |  | UN |  | - |  |
|  | 59 |  | 4 |  |  | 500.00 |  | L0 |  | UN |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  | 2 |  |  | 500.00 |  | L0 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 61 |  | 3 |  |  | 500.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 62 |  | 4 |  |  | 500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 63 |  | 1 |  |  | 600.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L1 is announced. |  |  | LNTC |  |  | SN |  | - |  |
|  | 64 |  | 2 |  |  | 600.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE10 |  |  | LNTC |  |  | SN |  | - |  |
|  | 65 |  | 3 |  |  | 600.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 66 |  | 4 |  |  | 600.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4080424: 4.8.4: Accepted Information depending on the modes - Mode Profile Test Case 12: The train receives a message including packet 80 (Mode Profile) that is transmitted via balise group. The message is accepted because a level transition is stored onborad |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 67 |  | 1 |  |  | 650.00 |  | LNTC |  | SN |  |  | One Balise Group message (N_TOTAL=0) containing packet 12, packet 21, packet 27, packet 80 is received |  | I |  | BTM |  | M_MAMODE inside packet 80 has to garantee a change of mode at the end of this TC. |  |  | LNTC |  |  | SN |  | - |  |
|  | 68 |  | 2 |  |  | 650.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | Complete message including Mode Profile is recorded |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 27: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L1. The driver acknowledges before entering the L1 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 72 |  | 1 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 1 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE10 is REMOVED. |  |  | LNTC |  |  | SN |  | - |  |
|  | 73 |  | 2 |  |  | 950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 74 |  | 3 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 1 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | LNTC |  |  | SN |  | - |  |
|  | 75 |  | 4 |  |  | 950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 76 |  | 5 |  |  | 950.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 1 announcement" |  | I |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 77 |  | 6 |  |  | 950.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 78 |  | 7 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 1 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | LNTC |  |  | SN |  | - |  |
|  | 79 |  | 8 |  |  | 950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 80 |  | 9 |  |  | 950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 1 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE10 |  |  | LNTC |  |  | SN |  | - |  |
|  | 81 |  | 10 |  |  | 950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit10=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 19: Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 82 |  | 1 |  |  | 1000.00 |  | LNTC |  | SN |  |  | One Balise Group message (N_TOTAL=1; NID_C=<comment>) containing packet 41 is received |  | I |  | BTM |  | At the level border the order to switch to ETCS level 1,2 or 3 |  |  | LNTC |  |  | SN |  | - |  |
|  | 83 |  | 2 |  |  | 1000.00 |  | LNTC |  | SN |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 84 |  | 3 |  |  | 1000.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded |  | O |  | JRU |  | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 87 |  | 6 |  |  | 1000.00 |  | L1 |  | FS |  |  | The Level symbol 'Level <L1/ L2/ L3>' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 88 |  | 7 |  |  | 1000.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit03/ 04/ 05=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 89 |  | 8 |  |  | 1000.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 90 |  | 9 |  |  | 1000.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 93 |  | 1 |  |  | 1100.00 |  | L1 |  | FS |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ LNTC is Announced. |  |  | L1 |  |  | FS |  | - |  |
|  | 94 |  | 2 |  |  | 1100.00 |  | L1 |  | FS |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE08 |  |  | L1 |  |  | FS |  | - |  |
|  | 95 |  | 3 |  |  | 1100.00 |  | L1 |  | FS |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | L1 |  |  | FS |  | - |  |
|  | 96 |  | 4 |  |  | 1100.00 |  | L1 |  | FS |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 8: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from L1 to LNTC. The driver acknowledges before entering the LNTC area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 1450.00 |  | L1 |  | FS |  |  | The Level symbol "Level NTC announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE08 is REMOVED |  |  | L1 |  |  | FS |  | - |  |
|  | 98 |  | 2 |  |  | 1450.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 99 |  | 3 |  |  | 1450.00 |  | L1 |  | FS |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | L1 |  |  | FS |  | - |  |
|  | 100 |  | 4 |  |  | 1450.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | The displayed symbol LE09 is RECORDED. |  |  | L1 |  |  | FS |  | - |  |
|  | 101 |  | 5 |  |  | 1450.00 |  | L1 |  | FS |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 102 |  | 6 |  |  | 1450.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 103 |  | 7 |  |  | 1450.00 |  | L1 |  | FS |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 104 |  | 8 |  |  | 1450.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 105 |  | 9 |  |  | 1450.00 |  | L1 |  | FS |  |  | The Level symbol "Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE08 |  |  | L1 |  |  | FS |  | - |  |
|  | 106 |  | 10 |  |  | 1450.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit08=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 34: The train receives an order to switch immediately from L1 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 108 |  | 1 |  |  | 1500.00 |  | L1 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC |  | I |  | BTM |  | 'The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L1 |  |  | FS |  | - |  |
|  | 109 |  | 2 |  |  | 1500.00 |  | L1 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 110 |  | 3 |  |  | 1500.00 |  | L1 |  | FS |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L1 |  |  | FS |  | - |  |
|  | 111 |  | 4 |  |  | 1500.00 |  | L1 |  | FS |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 112 |  | 2 |  |  | 1500.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 113 |  | 3 |  |  | 1500.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 114 |  | 4 |  |  | 1500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 14: The train receives from the balise group BGa the Level Transition announcement LX/LY. The driver is informed. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 115 |  | 1 |  |  | 1600.00 |  | LNTC |  | SN |  |  | An announcement of level transition to Level Y (Packet 41) is RECEIVED from Balise Group BGa: D_LEVELTR <> 0 and <> 32767 M_LEVELTR= M_LEVEL_Y (Level Y) |  | I |  | BTM |  | Order to switch to level Y at further location and this announcement will result in a change of the on-board level. _____ L0 is announced |  |  | LNTC |  |  | SN |  | - |  |
|  | 116 |  | 2 |  |  | 1600.00 |  | LNTC |  | SN |  |  | An indication of Announcement of Level Y is DISPLAYED. |  | O |  | DMI |  | ('Except in SL and SB modes') _____ DMI Symbol LE06 |  |  | LNTC |  |  | SN |  | - |  |
|  | 117 |  | 3 |  |  | 1600.00 |  | LNTC |  | SN |  |  | Displayed indication is recorded |  | O |  | JRU |  | ('Except in SL and SB modes') |  |  | LNTC |  |  | SN |  | - |  |
|  | 118 |  | 4 |  |  | 1600.00 |  | LNTC |  | SN |  |  | Balise Group BGa message, including Packet 41'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 24: The train enters in the acknowledgement window, and the driver is requested to acknowledge the transition from LNTC to L0. The driver acknowledges before entering the L0 area. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 119 |  | 1 |  |  | 1950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 0 announcement" is removed |  | O |  | DMI |  | The max safe front end of the train enters the acknowledgement window (defined in the level transition announcement with L_ACKLEVELTR). Symbol LE06 is REMOVED |  |  | LNTC |  |  | SN |  | - |  |
|  | 120 |  | 2 |  |  | 1950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 121 |  | 3 |  |  | 1950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 122 |  | 4 |  |  | 1950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 123 |  | 5 |  |  | 1950.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 124 |  | 6 |  |  | 1950.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 125 |  | 7 |  |  | 1950.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 126 |  | 8 |  |  | 1950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 127 |  | 9 |  |  | 1950.00 |  | LNTC |  | SN |  |  | The Level symbol "Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE06 |  |  | LNTC |  |  | SN |  | - |  |
|  | 128 |  | 10 |  |  | 1950.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit06=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 24: The train receives an order to switch immediately from LNTC to L0 at the current position. on-board equipment immediately switches to L0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 130 |  | 1 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 |  | I |  | BTM |  | Reception of level transition order (Packet 41) |  |  | LNTC |  |  | SN |  | - |  |
|  | 131 |  | 2 |  |  | 2000.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 132 |  | 3 |  |  | 2000.00 |  | LNTC |  | SN |  |  | Level 0 is DISPLAYED |  | O |  | DMI |  | (Except in SL mode ' desk is closed) The On-board equipment switches to Level 0. |  |  | LNTC |  |  | SN |  | - |  |
|  | 133 |  | 4 |  |  | 2000.00 |  | LNTC |  | SN |  |  | New Level 0 is RECORDED |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 6: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 135 |  | 1 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 136 |  | 2 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 137 |  | 3 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 138 |  | 4 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 139 |  | 5 |  |  | 2000.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 140 |  | 6 |  |  | 2000.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 3: ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 141 |  | 3 |  |  | 2000.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 142 |  | 4 |  |  | 2000.00 |  | L0 |  | UN |  |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object: MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 143 |  | 5 |  |  | 2000.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 16: The train receives an order to switch immediately from L0 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 145 |  | 1 |  |  | 2500.00 |  | L0 |  | UN |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_STMNTC |  | I |  | BTM |  | 'The onboard equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L0 |  |  | UN |  | - |  |
|  | 146 |  | 2 |  |  | 2500.00 |  | L0 |  | UN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 147 |  | 3 |  |  | 2500.00 |  | L0 |  | UN |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L0 |  |  | UN |  | - |  |
|  | 148 |  | 4 |  |  | 2500.00 |  | L0 |  | UN |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 149 |  | 2 |  |  | 2500.00 |  | L0 |  | UN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 150 |  | 3 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 151 |  | 4 |  |  | 2500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 3: The train has passed over the Level Transition border and the On-board equipment has switched from L0 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 152 |  | 1 |  |  | 2500.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 153 |  | 2 |  |  | 2500.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 154 |  | 3 |  |  | 2500.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within driver acknowledgement time[5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 155 |  | 4 |  |  | 2550.00 |  | LNTC |  | SN |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 156 |  | 5 |  |  | 2550.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 157 |  | 6 |  |  | 2550.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 158 |  | 7 |  |  | 2550.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 159 |  | 8 |  |  | 2550.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 160 |  | 9 |  |  | 2550.00 |  | LNTC |  | SN |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 161 |  | 10 |  |  | 2550.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 162 |  | 11 |  |  | 2550.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 163 |  | 12 |  |  | 2550.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 164 |  | 13 |  |  | 2550.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 23: The train receives an order to switch immediately from LNTC to L1 at the current position. on-board equipment immediately switches to L1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 1 |  |  | 2800.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 1 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=2 |  | I |  | BTM |  | - _____ MA for L1 added togheter with level transition order. |  |  | LNTC |  |  | SN |  | - |  |
|  | 166 |  | 2 |  |  | 2800.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 167 |  | 3 |  |  | 2800.00 |  | LNTC |  | SN |  |  | Level 1 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode ' desk is closed) The On-board equipment switches to Level 1. |  |  | LNTC |  |  | SN |  | - |  |
|  | 168 |  | 4 |  |  | 2800.00 |  | LNTC |  | SN |  |  | New Level 1 is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 19: Before the train reaches the level border, a full supervision MA + track description (SSP + gradient) is transferred to the on-board. The track description is available starting from the level border that means not for the whole length of the train but from the Front End. At the level border the order to switch to ERTMS / ETCS level 1, 2 or 3 is received. Then the on-board switches immediately to Full Supervision mode. Transition Condition Id [25]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 171 |  | 3 |  |  | 2800.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL= 2/ 3/ 4; M_MODE=0) is recorded |  | O |  | JRU |  | The level changes to ETCS level 1, 2 or 3. The on-board equipment switches immediately to FS mode |  |  | L1 |  |  | FS |  | - |  |
|  | 172 |  | 4 |  |  | 2800.00 |  | L1 |  | FS |  |  | The system status message 'Entering FS' is displayed. |  | O |  | DMI |  | Displayed to the driver until SSP and gradients are known for the whole length of the train. |  |  | L1 |  |  | FS |  | - |  |
|  | 173 |  | 5 |  |  | 2800.00 |  | L1 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 176 |  | 8 |  |  | 2800.00 |  | L1 |  | FS |  |  | The Mode symbol 'Full Supervision' is displayed |  | O |  | DMI |  | DMI Object: Symbol MO11 |  |  | L1 |  |  | FS |  | - |  |
|  | 177 |  | 9 |  |  | 2800.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit26=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 178 |  | 10 |  |  | 2800.00 |  | L1 |  | FS |  |  | The system status message 'Entering FS' is removed. |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 179 |  | 11 |  |  | 2800.00 |  | L1 |  | FS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23; SYSTEM_STATUS_MESSAGE=<Bit04=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 29: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 1, upon receipt of an order. The driver does not acknowledge within the acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 180 |  | 1 |  |  | 2800.00 |  | L1 |  | FS |  |  | The Level symbol "Acknowledge Level 1 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | L1 |  |  | FS |  | - |  |
|  | 181 |  | 2 |  |  | 2800.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 182 |  | 3 |  |  | 2800.00 |  | L1 |  | FS |  |  | NOT The driver acknowledges "Level 1 announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within the acknowledgement time [five seconds] after the Level Transition. |  |  | L1 |  |  | FS |  | - |  |
|  | 183 |  | 4 |  |  | 2850.00 |  | L1 |  | FS |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 184 |  | 5 |  |  | 2850.00 |  | L1 |  | FS |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 185 |  | 6 |  |  | 2850.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 186 |  | 7 |  |  | 2850.00 |  | L1 |  | FS |  |  | The driver acknowledges "Level 1 announcement" |  | I |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 187 |  | 8 |  |  | 2850.00 |  | L1 |  | FS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0111) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 188 |  | 9 |  |  | 2850.00 |  | L1 |  | FS |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 189 |  | 10 |  |  | 2850.00 |  | L1 |  | FS |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 190 |  | 11 |  |  | 2850.00 |  | L1 |  | FS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 191 |  | 12 |  |  | 2850.00 |  | L1 |  | FS |  |  | The Level symbol "Acknowledge Level 1 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE11 |  |  | L1 |  |  | FS |  | - |  |
|  | 192 |  | 13 |  |  | 2850.00 |  | L1 |  | FS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit11=0>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 34: The train receives an order to switch immediately from L1 to LNTC at the current position. on-board equipment immediately switches to LNTC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 194 |  | 1 |  |  | 3300.00 |  | L1 |  | FS |  |  | The train reaches the Level transition border and an order to switch immediately to Level NTC is received from balise group BG1 (packet 41): D_LEVELTR=0 M_LEVELTR=1 NID_NTC |  | I |  | BTM |  | 'The on-board equipment sends a signal to the appropriate NTC, in order to be prepared for immediate activation' |  |  | L1 |  |  | FS |  | - |  |
|  | 195 |  | 2 |  |  | 3300.00 |  | L1 |  | FS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | 196 |  | 3 |  |  | 3300.00 |  | L1 |  | FS |  |  | Level NTC is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level NTC. |  |  | L1 |  |  | FS |  | - |  |
|  | 197 |  | 4 |  |  | 3300.00 |  | L1 |  | FS |  |  | New Level NTC is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | FS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 6: ETCS level switches to Level NTC .Transition Condition Id [56]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 198 |  | 2 |  |  | 3300.00 |  | L1 |  | FS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=1; M_MODE=13) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 199 |  | 3 |  |  | 3300.00 |  | LNTC |  | SN |  |  | The Mode symbol 'National System' is displayed |  | O |  | DMI |  | DMI Object: MO19 |  |  | LNTC |  |  | SN |  | - |  |
|  | 200 |  | 4 |  |  | 3300.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit34=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 10: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level NTC, upon receipt of an order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 201 |  | 1 |  |  | 3300.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 202 |  | 2 |  |  | 3300.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 203 |  | 3 |  |  | 3300.00 |  | LNTC |  | SN |  |  | NOT The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within the driver acknowledgement time[5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 204 |  | 4 |  |  | 3350.00 |  | LNTC |  | SN |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 205 |  | 5 |  |  | 3350.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 206 |  | 6 |  |  | 3350.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 207 |  | 7 |  |  | 3350.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level NTC announcement" |  | I |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 208 |  | 8 |  |  | 3350.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1010) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 209 |  | 9 |  |  | 3350.00 |  | LNTC |  | SN |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 210 |  | 10 |  |  | 3350.00 |  | LNTC |  | SN |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 211 |  | 11 |  |  | 3350.00 |  | LNTC |  | SN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 212 |  | 12 |  |  | 3350.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level NTC announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE09 |  |  | LNTC |  |  | SN |  | - |  |
|  | 213 |  | 13 |  |  | 3350.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit09=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 24: The train receives an order to switch immediately from LNTC to L0 at the current position. on-board equipment immediately switches to L0. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 215 |  | 1 |  |  | 3800.00 |  | LNTC |  | SN |  |  | The train reaches the Level transition border and an order to switch immediately to Level 0 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=0 |  | I |  | BTM |  | Reception of level transition order (Packet 41) |  |  | LNTC |  |  | SN |  | - |  |
|  | 216 |  | 2 |  |  | 3800.00 |  | LNTC |  | SN |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 217 |  | 3 |  |  | 3800.00 |  | LNTC |  | SN |  |  | Level 0 is DISPLAYED |  | O |  | DMI |  | (Except in SL mode ' desk is closed) The On-board equipment switches to Level 0. |  |  | LNTC |  |  | SN |  | - |  |
|  | 218 |  | 4 |  |  | 3800.00 |  | LNTC |  | SN |  |  | New Level 0 is RECORDED |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 6: The train has passed over the Level Transition border and the On-board equipment has switched from L1 to Level 0, upon receipt of an order . The driver acknowledges within the driver acknowledgement time[5 seconds] after the Level transition. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 220 |  | 1 |  |  | 3800.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 221 |  | 2 |  |  | 3800.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 222 |  | 3 |  |  | 3800.00 |  | LNTC |  | SN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver acknowledges the Level Transition within the driver acknowledgement time [5 seconds] after the Level Transition. |  |  | LNTC |  |  | SN |  | - |  |
|  | 223 |  | 4 |  |  | 3800.00 |  | LNTC |  | SN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | 224 |  | 5 |  |  | 3800.00 |  | LNTC |  | SN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | LNTC |  |  | SN |  | - |  |
|  | 225 |  | 6 |  |  | 3800.00 |  | LNTC |  | SN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | LNTC |  |  | SN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 3: ERTMS/ ETCS Level switches to level 0. Transition Condition Id [21]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 226 |  | 3 |  |  | 3800.00 |  | LNTC |  | SN |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 227 |  | 4 |  |  | 3800.00 |  | L0 |  | UN |  |  | The Mode symbol 'Unfitted' is displayed |  | O |  | DMI |  | DMI Object: MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 228 |  | 5 |  |  | 3800.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 5100400: 5.10.4: Acknowledgement of the level transition Test Case 26: The train has passed over the Level Transition border and the On-board equipment has switched from LNTC to Level 0, upon receipt of the order. The driver does not acknowledge within the driver acknowledgement time[5 seconds] after the Level transition. The service brake is applied. The driver acknowledges and the brakes are released. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 229 |  | 1 |  |  | 3800.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is displayed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 230 |  | 2 |  |  | 3800.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 231 |  | 3 |  |  | 3800.00 |  | L0 |  | UN |  |  | NOT The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | The driver does not acknowledge the Level Transition within the driver acknowledgement time [five seconds] after the Level Transition |  |  | L0 |  |  | UN |  | - |  |
|  | 232 |  | 4 |  |  | 3850.00 |  | L0 |  | UN |  |  | Service brake commanded |  | O |  | TIU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 233 |  | 5 |  |  | 3850.00 |  | L0 |  | UN |  |  | The Status symbol "Service Brake intervention" is displayed |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 234 |  | 6 |  |  | 3850.00 |  | L0 |  | UN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 235 |  | 7 |  |  | 3850.00 |  | L0 |  | UN |  |  | The driver acknowledges "Level 0 announcement" |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 236 |  | 8 |  |  | 3850.00 |  | L0 |  | UN |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 0110) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 237 |  | 9 |  |  | 3850.00 |  | L0 |  | UN |  |  | Service brake not commanded |  | O |  | TIU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 238 |  | 10 |  |  | 3850.00 |  | L0 |  | UN |  |  | The Status symbol "Service Brake intervention" is removed |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 239 |  | 11 |  |  | 3850.00 |  | L0 |  | UN |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 240 |  | 12 |  |  | 3850.00 |  | L0 |  | UN |  |  | The Level symbol "Acknowledge Level 0 announcement" is removed |  | O |  | DMI |  | DMI Object: Symbol LE07 |  |  | L0 |  |  | UN |  | - |  |
|  | 241 |  | 13 |  |  | 3850.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit07=0>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 242 |  | 1 |  |  | 4000.00 |  | L0 |  | UN |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L0 |  |  | NP |  | - |  |
|  | 243 |  | 2 |  |  | 4000.00 |  | L0 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | N/A |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=0 |  |  |  |  |  |  |  |  | 0 s |  |  |  |  |  |  |  |  |

| - Step 41 : Balise Group BG 01 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 450 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 41 : Balise Group BG 01 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 01 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 02 d 1/2 - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 56 : Balise Group BG 02 d 2/2 - Distance 505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 02 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 1/2 - Distance 600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 63 : Balise Group BG 03 d 2/2 - Distance 605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 03 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 67 : Balise Group BG 04 d 1/2 - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 50 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 30 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 67 : Balise Group BG 04 d 2/2 - Distance 655.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 04 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 82 : Balise Group BG 05 d 1/2 - Distance 1000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 05 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 82 : Balise Group BG 05 d 2/2 - Distance 1005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 05 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 93 : Balise Group BG 06 d 1/2 - Distance 1100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 93 : Balise Group BG 06 d 2/2 - Distance 1105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 06 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 108 : Balise Group BG 07 d 1/2 - Distance 1500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 07 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 108 : Balise Group BG 07 d 2/2 - Distance 1505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 07 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 115 : Balise Group BG 08 d 1/2 - Distance 1600.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 08 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 400 d |  |  |
| M_LEVELTR | 3 | 0 |  |  |
| L_ACKLEVELTR | 15 | 50 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 115 : Balise Group BG 08 d 2/2 - Distance 1605.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 08 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 130 : Balise Group BG 09 d 1/2 - Distance 2000.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 09 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 130 : Balise Group BG 09 d 2/2 - Distance 2005.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 09 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 145 : Balise Group BG 10 d 1/2 - Distance 2500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 145 : Balise Group BG 10 d 2/2 - Distance 2505.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 10 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 165 : Balise Group BG 11 d 1/2 - Distance 2800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 2 |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 20 d |  |  |
| V_EMA | 7 | 0 d |  |  |
| T_EMA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 1200 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 20 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 165 : Balise Group BG 11 d 2/2 - Distance 2805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 11 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 194 : Balise Group BG 12 d 1/2 - Distance 3300.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 71 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 1 |  |  |
| NID_NTC | 8 | 2 d |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 194 : Balise Group BG 12 d 2/2 - Distance 3305.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 12 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 215 : Balise Group BG 13 d 1/2 - Distance 3800.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 000 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 215 : Balise Group BG 13 d 2/2 - Distance 3805.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 010 0001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 13 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |