| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_6060303_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_6060303_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 14:22 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | MLT |

# Sequence description

6.6.3.3 Messages received from RBC/RIU

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L1 |  |  | L1 |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L1 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L1 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - _____ The driver sets: - Train category to TILT2: * NC_CDTRAIN=5 (Cant Deficiency 180mm ) * NC_TRAIN=4 (Passenger train) - Loading gauge to G1 (M_LOADINGAUGE=1) - Axle load category to D2 (M_AXLELOADCAT=7) |  |  | L1 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L1 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L1 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L1 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 _____ The driver sets the Train running number to 12345678 |  |  | L1 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 1: The on-board equipment requests the driver to re-validate/enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when level is 0/1/NTC (Subset-026-5.4.3.2 S12). The driver is requested to enter/re-validate the Train running number if the status of the Train running number is 'unknown' or 'invalid'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 28 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | The DMI allows the driver to select the option ‘START’. |  | O |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 16: The driver selects 'Start' within the scope of the Start of Mission procedure in SB mode and in ERTMS Level 1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver selects 'Start'. |  | I |  | DMI |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L1 |  | SB |  |  | Driver’s selection of 'Start' is recorded on the JRU |  | O |  | JRU |  | Triggered by: The driver acts on the on-board DMI. |  |  | L1 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L1 |  | SB |  |  | An acknowledgement request for running in Staff Responsible mode is displayed on the DMI |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Staff Responsible mode |  |  | L1 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L1 |  | SB |  |  | The acknowledgement request for running in Staff Responsible mode is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L1 |  | SB |  |  | The driver acknowledges 'Staff Responsible' |  | I |  | DMI |  | Symbol MO10 |  |  | L1 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L1 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0011) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L1 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=2) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L1 |  | SR |  |  | The Mode symbol 'Staff Responsible' is displayed |  | O |  | DMI |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L1 |  | SR |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit24=1>) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L1 |  | SR |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | Feature 5190200: 5.19.2: Limited Supervision is requested for current location (from modes different from Stand By and Post Trip) Test Case 1: Limited Supervision is requested for the current location and the driver acknowledges within the driver acknowledgement time (refer to Appendix A3.1) after the change to LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 39 |  | 1 |  |  | 50.00 |  | L1 |  | SR |  |  | One Balise Group message containing packet 12, packet 80 (M_MAMODE=2) is received |  | I |  | BTM |  | One of the steps 1 or 2 will be chosen for testing Reception of the mode transition announcement (mode transition order at current location) If not yet stored, packet 21 gradient profile shall be sent in this message and gradient profile shall be at least available from the front end of the train If not yet stored, packet 27 ISSP shall be sent in this message and ISSP shall be at least available from the front end of the train _____ Packet 5 Linking added in BG |  |  | L1 |  |  | SR |  | - |  |
|  | 41 |  | 3 |  |  | 50.00 |  | L1 |  | SR |  |  | Complete message including Packet 80: Mode profile is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | SR |  | - |  |
|  | 42 |  | 4 |  |  | 50.00 |  | L1 |  | SR |  |  | The ERTMS/ETCS on-board equipment switches immediately to LS mode and a request for acknowledgement is displayed to the driver |  | O |  | DMI |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 43 |  | 5 |  |  | 50.00 |  | L1 |  | LS |  |  | Mode transition to LS is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 46 |  | 8 |  |  | 50.00 |  | L1 |  | LS |  |  | Driver acknowledges within 5 sec after the change to LS |  | I |  | DMI |  | The driver acknowledgement time (refer to Appendix A3.1) |  |  | L1 |  |  | LS |  | - |  |
|  | 47 |  | 9 |  |  | 50.00 |  | L1 |  | LS |  |  | Driver Acknowledgement of transition to LS mode is RECORDED |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 52 |  | 1 |  |  | 100.00 |  | L1 |  | LS |  |  | The train reaches the Level transition border and an order to switch immediately to Level 2 is received from balise group BGa. Packet 41: 'Level Transition Order' is RECEIVED. D_LEVELTR= 0 or = 32767 M_LEVELTR=3 |  | I |  | BTM |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | 53 |  | 2 |  |  | 100.00 |  | L1 |  | LS |  |  | Balise Group message, including Packet 41 'Level transition order', is RECORDED. |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 54 |  | 1 |  |  | 100.00 |  | L1 |  | LS |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L1 |  |  | LS |  | - |  |
|  | 55 |  | 2 |  |  | 100.00 |  | L1 |  | LS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L1 |  |  | LS |  | - |  |
|  | Feature 5100100: 5.10.1: Level Transitions General requirements Test Case 37: The train receives an order to switch immediately from L1 to L2 at the current position. On-board equipment immediately switches to L2. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 56 |  | 3 |  |  | 100.00 |  | L1 |  | LS |  |  | Level 2 is DISPLAYED. |  | O |  | DMI |  | (Except in SL mode) The On-board equipment switches to Level 2. |  |  | L2 |  |  | LS |  | - |  |
|  | 57 |  | 4 |  |  | 100.00 |  | L2 |  | LS |  |  | New Level 2 is RECORDED. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 58 |  | 3 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful. The onboard tries to set up a connection with the phone number included in the session management packet. |  |  | L2 |  |  | LS |  | - |  |
|  | 59 |  | 4 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 60 |  | 5 |  |  | 100.00 |  | L2 |  | LS |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 61 |  | 6 |  |  | 100.00 |  | L2 |  | LS |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 62 |  | 7 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 63 |  | 8 |  |  | 100.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 64 |  | 9 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board _____ M_VERSION = 001 0000 b (Version 1.0) |  |  | L2 |  |  | LS |  | - |  |
|  | 65 |  | 10 |  |  | 100.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 66 |  | 11 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 67 |  | 12 |  |  | 100.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 6: Transmission and translation of packet 11: 'Validated Train data' with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 69 |  | 1 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. |  | O |  | RTM |  | For Message 129 no translation is necessary. For Packet 11 the translation is: NID_OPERATIONAL=NID_OPERTAIONAL1 NC_CDTRAIN translated to NC_TRAIN and set to xxx xxxx x1xx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0101101 N_ITER set to 0 _____ Details about the values in packet 11: - NID_OPERATIONAL= 1234 5678 bcd - NC_TRAIN=000 1000 0100 0000 b - N_ITER=0 for the first occurence of N_ITER |  |  | L2 |  |  | LS |  | - |  |
|  | 70 |  | 2 |  |  | 100.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 71 |  | 3 |  |  | 100.00 |  | L2 |  | LS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | The RBC understands and acknowledges this set of Train Data. |  |  | L2 |  |  | LS |  | - |  |
|  | 72 |  | 4 |  |  | 100.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 6060303: 6.6.3.3 Messages received from RBC/RIU Test Case 2: The on-board equipment receives from an RBC certified to the system version number X=1 an Conditional Emergency Stop message that is rejected. The on-board equipment translated the received data correctly. Tested action [T = TRANSLATED]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 73 |  | 1 |  |  | 350.00 |  | L2 |  | LS |  |  | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received from an RBC certified with system version X=1. |  | I |  | RTM |  | The on-board translates the message for system version X=2 adding D_REF = 0 |  |  | L2 |  |  | LS |  | - |  |
|  | 74 |  | 2 |  |  | 350.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 75 |  | 3 |  |  | 350.00 |  | L2 |  | LS |  |  | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted to the RBC (X=1). |  | O |  | RTM |  | The on-board transmittes the value Q_EMERGENCYSTOP set to1 for X=1 RBC .Translation type from 3 to 1 ( CES is rejected). |  |  | L2 |  |  | LS |  | - |  |
|  | 76 |  | 4 |  |  | 350.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 77 |  | 5 |  |  | 350.00 |  | L2 |  | LS |  |  | Service brake not commanded |  | O |  | TIU |  | Stop location already passed by the train. Emergency stop rejected. |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 6060303: 6.6.3.3 Messages received from RBC/RIU Test Case 3: The on-board equipment receives from an RBC certified to the system version number X=1 a request for a Track Ahead Free, message 34. The on-board equipment translated the received data to system version X=2 correctly. Tested action [T= TRANSLATED] & [U= UNCHANGED]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 79 |  | 1 |  |  | 400.00 |  | L2 |  | LS |  |  | SA-DATA. Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received from an RBC certified with system version X=1. |  | I |  | RTM |  | The on-board translates the message for system version X=2 adding D_REF = 0 |  |  | L2 |  |  | LS |  | - |  |
|  | 80 |  | 2 |  |  | 400.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 81 |  | 3 |  |  | 450.00 |  | L2 |  | LS |  |  | The Driver Request symbol "Track Ahead Free" is displayed. |  | O |  | DMI |  | When the train reaches the D_TAFDISPLAY location Symbol DR02 |  |  | L2 |  |  | LS |  | - |  |
|  | 82 |  | 4 |  |  | 450.00 |  | L2 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21, bit_and [DMI_SYMB_STATUS,MASK]=MASK)) is recorded. |  | O |  | JRU |  | MASK=0200000000000000000000 |  |  | L2 |  |  | LS |  | - |  |
|  | 83 |  | 5 |  |  | 500.00 |  | L2 |  | LS |  |  | The driver confirms "Track ahead is free". |  | I |  | DMI |  | Before the train reaches the D_TAFDISPLAY + L_TAFDISPLAY location. |  |  | L2 |  |  | LS |  | - |  |
|  | 84 |  | 6 |  |  | 500.00 |  | L2 |  | LS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 85 |  | 7 |  |  | 500.00 |  | L2 |  | LS |  |  | The Driver Request symbol "Track Ahead Free" is removed |  | O |  | DMI |  | After driver ACK |  |  | L2 |  |  | LS |  | - |  |
|  | 86 |  | 8 |  |  | 500.00 |  | L2 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21, bit_and [DMI_SYMB_STATUS,MASK]=0)) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 87 |  | 9 |  |  | 500.00 |  | L2 |  | LS |  |  | SA-DATA. Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) containing packet 0 or 1 is transmitted to the RBC (X=1). |  | O |  | RTM |  | The transmitted data remains unchanged (action U). |  |  | L2 |  |  | LS |  | - |  |
|  | 88 |  | 10 |  |  | 500.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 11: Transmission of the message 132: 'MA Request' by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ' T[4]' & data is unchanged 'U[1]' when EVC in LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 89 |  | 1 |  |  | 600.00 |  | L2 |  | LS |  |  | No linked balise group message is received |  | I |  | BTM |  | The balise group missed was linked with service brake reaction |  |  | L2 |  |  | LS |  | - |  |
|  | 90 |  | 2 |  |  | 600.00 |  | L2 |  | LS |  |  | Service brake commanded. |  | O |  | TIU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 91 |  | 3 |  |  | 600.00 |  | L2 |  | LS |  |  | The Status symbol "Service Brake " is displayed. |  | O |  | DMI |  | Symbol ST01 |  |  | L2 |  |  | LS |  | - |  |
|  | 92 |  | 4 |  |  | 600.00 |  | L2 |  | LS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 93 |  | 5 |  |  | 600.00 |  | L2 |  | LS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 94 |  | 6 |  |  | 650.00 |  | L2 |  | LS |  |  | Train is at standstill (V_TRAIN=0; TIME= 60) |  | I |  | INT |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 95 |  | 7 |  |  | 650.00 |  | L2 |  | LS |  |  | SA-DATA. Request with Euroradio Message "MA Request" (NID_MESSAGE=132) containing packet 0 is transmitted to a system version X=1 RBC. |  | O |  | RTM |  | Q_TRACKDEL = 1 For Message132, data is translated as: Q_MARQSTREASON is replaced with Q_TRACKDEL And value x1xxx is set to 1 (track description deleted) When the applicable mode is LS, M_MAMODE=12 then, no translation is effected. |  |  | L2 |  |  | LS |  | - |  |
|  | 96 |  | 8 |  |  | 650.00 |  | L2 |  | LS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | 97 |  | 9 |  |  | 650.00 |  | L2 |  | LS |  |  | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 and packet 80 is received |  | I |  | RTM |  | Translation of Packets 21, 27 and 80 out of scope of test case target. |  |  | L2 |  |  | LS |  | - |  |
|  | 98 |  | 10 |  |  | 650.00 |  | L2 |  | LS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | LS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 64: In ERTMS Level 1, 2 or 3 in LS mode, a Mode Profile defining an On Sight area is on-board. The maximum safe front end of the train is inside the On Sight area and the estimated front end of the train is not inside an LS acknowledgement area. Transition Condition Id [73]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 100 |  | 2 |  |  | 650.00 |  | L2 |  | LS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 101 |  | 3 |  |  | 650.00 |  | L2 |  | OS |  |  | The Mode symbol 'Acknowledge On Sight' is displayed |  | O |  | DMI |  | Symbol MO08 |  |  | L2 |  |  | OS |  | - |  |
|  | 102 |  | 4 |  |  | 650.00 |  | L2 |  | OS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit23=1>) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 103 |  | 5 |  |  | 650.00 |  | L2 |  | OS |  |  | The driver acknowledges 'On Sight' |  | I |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 104 |  | 6 |  |  | 650.00 |  | L2 |  | OS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVER_ACTIONS=0000 0000) is recorded |  | O |  | JRU |  | Within the 5 sec ack. time |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 8: The transmission of Data Train (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 107 |  | 1 |  |  | 650.00 |  | L2 |  | OS |  |  | The driver enters/modifies the "Train Running Number" [MAIN]. (NID_OPERATIONAL2) |  | I |  | DMI |  | NID_OPERATIONAL1 ? NID_OPERATIONAL2 Only for SB mode: Train at standstill with a cab active. _____ The driver sets the train running number to 8765 4321 bcd (NID_OPERATIONAL2) |  |  | L2 |  |  | OS |  | - |  |
|  | 108 |  | 2 |  |  | 650.00 |  | L2 |  | OS |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49) is recorded |  | O |  | JRU |  | NID_OPERATIONAL2 _____ TC bug 1477 recorded in ADDITIONAL DATA, NID_MESSAGE_JRU=24 |  |  | L2 |  |  | OS |  | - |  |
|  | 109 |  | 3 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. |  | O |  | RTM |  | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL2 variable is included. Translation of packet 11 tested in FT6060304.6, 7. |  |  | L2 |  |  | OS |  | - |  |
|  | 110 |  | 4 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Check that message 136 with packet 5 is not recorded. |  |  | L2 |  |  | OS |  | - |  |
|  | 111 |  | 5 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | The RBC receives the information correctly and acknowledges this set of Train Data. |  |  | L2 |  |  | OS |  | - |  |
|  | 112 |  | 6 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 10: Exception: If the train running number is received from the RBC (X=1), the Train Data is not sent back to the RBC by the on-board equipment. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 113 |  | 1 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 140 (NID_OPERATIONAL2=FFFF1234) is received. |  | I |  | RTM |  | NID_OPERATIONAL1 ? NID_OPERATIONAL2 |  |  | L2 |  |  | OS |  | - |  |
|  | 114 |  | 2 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | NID_OPERATIONAL2 is recorded following the translation T[10]: 1234FFFF |  |  | L2 |  |  | OS |  | - |  |
|  | 115 |  | 3 |  |  | 650.00 |  | L2 |  | OS |  |  | NOT SA-DATA.Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted. |  | O |  | RTM |  | The on-board does not send back to the RBC the Train Data. |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 5: The on-board equipment terminates a communication session ordered by RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 116 |  | 1 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA.Indication with Euroradio Message "General message" (NID_MESSAGE=24) containing packet 42 (Q_RBC=0) is received |  | I |  | RTM |  | Q_RBC = 0 (Terminate communication session) |  |  | L2 |  |  | OS |  | - |  |
|  | 117 |  | 2 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 118 |  | 3 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | As soon as this message is received by trackside (RBC), the RBC shall consider the communication session terminated and it shall send the acknowledgement to the on-board. It shall be checked that no more messages are sent by the onboard |  |  | L2 |  |  | OS |  | - |  |
|  | 119 |  | 4 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 120 |  | 5 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is RECEIVED from the RBC and the on-board equipment considers the communication session terminated. |  |  | L2 |  |  | OS |  | - |  |
|  | 121 |  | 6 |  |  | 650.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | When the acknowledgement is received, the onboard shall consider the communication session terminated and it shall begin the release of the safe connection with trackside. |  |  | L2 |  |  | OS |  | - |  |
|  | 122 |  | 7 |  |  | 650.00 |  | L2 |  | OS |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The train tries to release the safe connection. |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 22: System version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 123 |  | 1 |  |  | 700.00 |  | L2 |  | OS |  |  | One Balise Group message (NID_RBC?16383) containing packet 42 (NID_RADIO ? FFFF FFFF FFFF FFFF) is received |  | I |  | BTM |  | Not use last known RBC and don’t use short number. |  |  | L2 |  |  | OS |  | - |  |
|  | 124 |  | 2 |  |  | 700.00 |  | L2 |  | OS |  |  | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 9: The transmission of Train Data (message 129 containing packet 11) by the on-board equipment to the RBC certified with X=1 instead of sending the Train running number (message 136 containing Packet 5) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 136 |  | 1 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-CONNECT. Request is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 137 |  | 2 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-CONNECT. Confirm is received |  | I |  | RTM |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 138 |  | 3 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | Type of action according to table 6.6.3.4.5 :[U] |  |  | L2 |  |  | OS |  | - |  |
|  | 139 |  | 4 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 140 |  | 5 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Configuration Determination" (NID_MESSAGE=32) is received |  | I |  | RTM |  | Message 32: 'RBC/RIU System Version' Type of action according to table 6.6.3.3.2:[U] |  |  | L2 |  |  | OS |  | - |  |
|  | 141 |  | 6 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | Versions are compatible Communication session established for on-board. |  |  | L2 |  |  | OS |  | - |  |
|  | 142 |  | 7 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Session Established" (NID_MESSAGE=159) containing packet 3 is transmitted. |  | O |  | RTM |  | Type of action according to table 6.6.3.4.5:[U][6] |  |  | L2 |  |  | OS |  | - |  |
|  | 143 |  | 8 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Communication session established for trackside. |  |  | L2 |  |  | OS |  | - |  |
|  | 144 |  | 9 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. |  | O |  | RTM |  | The RBC (X=1) is informed about the new train running number with packet 11 where NID_OPERATIONAL = NID_OPERATIONAL1 variable is included. Translation of packet 11 tested in FT6060304.6 or 7. |  |  | L2 |  |  | OS |  | - |  |
|  | 145 |  | 10 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | Check that message 136 with packet 5 is not recorded. |  |  | L2 |  |  | OS |  | - |  |
|  | 146 |  | 11 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | The RBC receives the information correctly and acknowledges this set of Train Data. |  |  | L2 |  |  | OS |  | - |  |
|  | 147 |  | 12 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060303: 6.6.3.3 Messages received from RBC/RIU Test Case 1: The on-board equipment receives from an RBC certified to the system version number X=1 a Conditional Emergency Stop message that is accepted. The on-board equipment translated the received data correctly. Tested action: [T = TRANSLATED]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received from an RBC certified with system version X=1. |  | I |  | RTM |  | The on-board translates the message for system version X=2 adding D_REF = 0 |  |  | L2 |  |  | OS |  | - |  |
|  | 149 |  | 2 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 150 |  | 3 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted to the RBC (X=1). |  | O |  | RTM |  | The on-board transmits the value Q_EMERGENCYSTOP set to 0 for X=1 RBC .Translation type from 1 to 0. |  |  | L2 |  |  | OS |  | - |  |
|  | 151 |  | 4 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 152 |  | 5 |  |  | 700.00 |  | L2 |  | OS |  |  | The system status message "Emergency stop" is displayed. |  | O |  | DMI |  | The driver is informed that the accepted CES message is received. |  |  | L2 |  |  | OS |  | - |  |
|  | 153 |  | 6 |  |  | 700.00 |  | L2 |  | OS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3100300: 3.10.3: Revocation of an Emergency Message Test Case 1: The train do not acknowledge the revocation of a Conditional Emergency Stop message when it is not requested. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 154 |  | 1 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA .Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) |  | I |  | RTM |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 155 |  | 2 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 11: Transmission of the message 132: 'MA Request' by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated ' T[4]' & data is unchanged 'U[1]' when EVC in LS mode. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 165 |  | 9 |  |  | 700.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Movement Authority" (NID_MESSAGE=3) containing packet 15, packet 21, packet 27 and packet 80 is received |  | I |  | RTM |  | Translation of Packets 21, 27 and 80 out of scope of test case target. |  |  | L2 |  |  | OS |  | - |  |
|  | 166 |  | 10 |  |  | 700.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 4: No transmission of packet 4: Error Reporting by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: data is unchanged 'U[2]'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 167 |  | 1 |  |  | 1100.00 |  | L2 |  | OS |  |  | NOT One Balise Group message is received |  | I |  | BTM |  | Train passes the expectation window of the first announced BG A. The balise group is not detected. |  |  | L2 |  |  | OS |  | - |  |
|  | 168 |  | 2 |  |  | 1150.00 |  | L2 |  | OS |  |  | NOT One Balise Group message is received |  | I |  | BTM |  | Train passes the expectation window of the second announced BG B. The balise group is not detected. |  |  | L2 |  |  | OS |  | - |  |
|  | 169 |  | 3 |  |  | 1150.00 |  | L2 |  | OS |  |  | The system status message "Balise read error" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 170 |  | 4 |  |  | 1150.00 |  | L2 |  | OS |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 171 |  | 5 |  |  | 1150.00 |  | L2 |  | OS |  |  | Service brake commanded. |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 172 |  | 6 |  |  | 1150.00 |  | L2 |  | OS |  |  | The Status symbol "Service Brake " is displayed. |  | O |  | DMI |  | Brake command indication due to RAMS related supervision (service brake) Symbol ST01 |  |  | L2 |  |  | OS |  | - |  |
|  | 173 |  | 7 |  |  | 1150.00 |  | L2 |  | OS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 174 |  | 8 |  |  | 1150.00 |  | L2 |  | OS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 175 |  | 9 |  |  | 1200.00 |  | L2 |  | OS |  |  | Train reaches standstill. Train is at standstill. |  | I |  | INT |  | V_MAIN = 0 |  |  | L2 |  |  | OS |  | - |  |
|  | 176 |  | 10 |  |  | 1200.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) to the RBC X=1. |  | O |  | RTM |  | Packet 4 is deleted because M_ERROR = 7 (X=2). The RBC not informed about the double linking error. |  |  | L2 |  |  | OS |  | - |  |
|  | 177 |  | 11 |  |  | 1200.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 178 |  | 12 |  |  | 1200.00 |  | L2 |  | OS |  |  | Service brake not commanded. |  | O |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 179 |  | 13 |  |  | 1200.00 |  | L2 |  | OS |  |  | The Status symbol "Service Brake intervention" is removed. |  | O |  | DMI |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 180 |  | 14 |  |  | 1200.00 |  | L2 |  | OS |  |  | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=0) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 181 |  | 15 |  |  | 1200.00 |  | L2 |  | OS |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; bit38=0) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 3180300: 3.18.3: Train Data Test Case 4: The on-board equipment sends the Train Data to the RBC following any entry/modification of Train Data when a communication session is already established |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 182 |  | 1 |  |  | 1200.00 |  | L2 |  | OS |  |  | Train is at standstill (V_TRAIN=0) |  | I |  | INT |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 183 |  | 2 |  |  | 1200.00 |  | L2 |  | OS |  |  | The driver enters/modifies and validates the "Flexible Train Data" |  | I |  | DMI |  | - _____ M_VERSION = 001 0000 b (Version 1.0) |  |  | L2 |  |  | OS |  | - |  |
|  | 184 |  | 3 |  |  | 1200.00 |  | L2 |  | OS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONs=21) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 185 |  | 4 |  |  | 1200.00 |  | L2 |  | OS |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 7: Transmission and translation of packet 11: 'Validated Train data' with message 129 by the on-board equipment to an RBC certified to the system version number X = 1. Tested action: Data is translated T[3] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 188 |  | 1 |  |  | 1200.00 |  | L2 |  | OS |  |  | SA-DATA. Request with Euroradio Message "Validated Train Data" (NID_MESSAGE=129) containing packet 11 is transmitted to the X=1 RBC. |  | O |  | RTM |  | For Message 129 no translation is needed. For Packet 11 the translation is: NID_OPERATIONAL = NID_OPERATIONAL1 NC_TRAIN set to xxx x1xx xxxx xxxx M_LOADINGGAUGE set to 0 M_AXLELOAD= 0100010 N_ITER set to 0 _____ Details about the values in packet 11: - NID_OPERATIONAL= 7676 7676 bcd - NC_TRAIN=000 0100 0001 0000 b - N_ITER=0 for the first occurence of N_ITER |  |  | L2 |  |  | OS |  | - |  |
|  | 189 |  | 2 |  |  | 1200.00 |  | L2 |  | OS |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 190 |  | 3 |  |  | 1200.00 |  | L2 |  | OS |  |  | SA-DATA. Indication with Euroradio Message "Acknowledgement of Train Data" (NID_MESSAGE=8) is received |  | I |  | RTM |  | The RBC understands and acknowledges this set of Train Data. |  |  | L2 |  |  | OS |  | - |  |
|  | 191 |  | 4 |  |  | 1200.00 |  | L2 |  | OS |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 17: The driver selects NON LEADIND mode, the train is at standstill and the 'non leading' input signal is received. The system switches to Non Leading Mode. Transition Condition Id [46]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 192 |  | 1 |  |  | 1200.00 |  | L2 |  | OS |  |  | Non-leading permitted |  | I |  | TIU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 193 |  | 2 |  |  | 1200.00 |  | L2 |  | OS |  |  | NON LEADING INPUT (NID_MESSAGE_JRU=32; M_NON_LEADING=1) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | OS |  | - |  |
|  | 194 |  | 3 |  |  | 1200.00 |  | L2 |  | OS |  |  | The driver selects 'NON LEADING' |  | I |  | DMI |  | The train is at standstill |  |  | L2 |  |  | OS |  | - |  |
|  | 195 |  | 4 |  |  | 1200.00 |  | L2 |  | OS |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0000 1100) is recorded |  | O |  | JRU |  | The on-board equipment switches to NL mode. |  |  | L2 |  |  | OS |  | - |  |
|  | 196 |  | 5 |  |  | 1200.00 |  | L2 |  | OS |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=11) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | NL |  | - |  |
|  | 197 |  | 6 |  |  | 1200.00 |  | L2 |  | NL |  |  | The Mode symbol 'Non-leading' is displayed |  | O |  | DMI |  | Symbol MO12 |  |  | L2 |  |  | NL |  | - |  |
|  | 198 |  | 7 |  |  | 1200.00 |  | L2 |  | NL |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit27=1>) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | NL |  | - |  |
|  | Feature 6060304: 6.6.3.4 Messages transmitted to RBC/RIU Test Case 3: Transmission of the message 136: 'Train Position report ' with packet 4: 'Error Reporting ' by the on-board equipment to an RBC/RIU certified to the system version number X = 1. Tested action: data is Unchanged 'U[2]'. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 200 |  | 1 |  |  | 1200.00 |  | L2 |  | NL |  |  | A safety critical fault occur |  | I |  | INT |  | - |  |  | L2 |  |  | NL |  | - |  |
|  | 201 |  | 2 |  |  | 1200.00 |  | L2 |  | NL |  |  | SAFETY CRITICAL FAULT IN MODE SL, NL OR PS (NID_MESSAGE_JRU=27) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | NL |  | - |  |
|  | 202 |  | 3 |  |  | 1200.00 |  | L2 |  | NL |  |  | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 is transmitted to the X=1 RBC. |  | O |  | RTM |  | Safety Critical fault For packet 4, data is translated as: M_ERROR value set from 6 to 7 The RBC understands the error as a Fatal Error (Equipment in SL or NL mode) |  |  | L2 |  |  | NL |  | - |  |
|  | 203 |  | 4 |  |  | 1200.00 |  | L2 |  | NL |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. |  | O |  | JRU |  | - |  |  | L2 |  |  | NL |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 204 |  | 1 |  |  | 1200.00 |  | L2 |  | NL |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 205 |  | 2 |  |  | 1200.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 39 : Balise Group BG 0 d 1/2 - Distance 50.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 5 d | Linking |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 147 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LINK | 15 | 50 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| D_LINK | 15 | 500 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 2 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 01 b |  |  |
| Q_LOCACC | 6 | 0 d |  |  |
| D_LINK | 15 | 100 d |  |  |
| Q_NEWCOUNTRY | 1 | 0 b |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINKORIENTATION | 1 | 1 b |  |  |
| Q_LINKREACTION | 2 | 10 b |  |  |
| Q_LOCACC | 6 | 0 d |  |  |
| NID_PACKET | 8 | 12 d | Level 1 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 73 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_MAIN | 7 | 40 d |  |  |
| V_EMA | 7 | 0000000 b |  |  |
| T_EMA | 10 | 1111111111 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 85 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 10 b |  |  |
| V_MAMODE | 7 | 20 d |  |  |
| L_MAMODE | 15 | 1500 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| Q_MAMODE | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 39 : Balise Group BG 0 d 2/2 - Distance 55.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 0 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 1 d 1/2 - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 41 d | Level transition order |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 63 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_LEVELTR | 15 | 0 d |  |  |
| M_LEVELTR | 3 | 011 b |  |  |
| L_ACKLEVELTR | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 0 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 52 : Balise Group BG 1 d 2/2 - Distance 105.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 1 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 62 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 64 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0010000 b |  |  |

| - Step 66 : Message 159 - Session Established (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 21 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 3 d |  |  |
| L_PACKET | 13 | 90 d |  |  |
| N_ITER | 5 | 1 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |

| - Step 69 : Message 129 - Validated Train Data (RBC n? 1) - Distance 100.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 150 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NID_OPERATIONAL | 32 | 12345678 h |  |  |
| NC_TRAIN | 15 | 000100010000000 b |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000000 b |  |  |
| M_AXLELOAD | 7 | 0101101 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_STM | 8 | 2 d |  |  |
| NID_STM | 8 | 3 d |  |  |

| - Step 71 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 100.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 73 : Message 15 - Conditional Emergency Stop (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 15 d |  |  |
| L_MESSAGE | 10 | 13 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_EM | 4 | 0 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_EMERGENCYSTOP | 15 | 0 d |  |  |

| - Step 75 : Message 147 - Acknowledgement of Emergency Stop (RBC n? 1) - Distance 350.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 147 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 00000000000000000000000000000000 b |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_EM | 4 | 0000 b |  |  |
| Q_EMERGENCYSTOP | 2 | 01 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 000100000000000000000001 b |  |  |
| D_LRBG | 15 | 000000011111010 b |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 000000000001100 b |  |  |
| L_DOUBTUNDER | 15 | 000000000001100 b |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0000111 b |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 79 : Message 34 - Track Ahead Free Request (RBC n? 1) - Distance 400.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 34 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_TAFDISPLAY | 15 | 350 d |  |  |
| L_TAFDISPLAY | 15 | 100 d |  |  |

| - Step 87 : Message 149 - Track Ahead Free Granted (RBC n? 1) - Distance 500.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 149 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 400 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 95 : Message 132 - MA Request (RBC n? 1) - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 132 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_TRACKDEL | 1 | 1 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 550 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1100 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 97 : Message 3 - Movement Authority (RBC n? 1) - Distance 650.00 m Delay : 10.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 43 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 84 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1500 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 109 : Message 129 - Validated Train Data (RBC n? 1) - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| D_LRBG | 15 | 550 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NID_OPERATIONAL | 32 | 87654321 h |  |  |
| NC_TRAIN | 15 | 000100010000000 b |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000000 b |  |  |
| M_AXLELOAD | 7 | 0101101 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_STM | 8 | 2 d |  |  |
| NID_STM | 8 | 3 d |  |  |

| - Step 111 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 650.00 m Delay : 20.00 s Conditional Message Id : 129 Back Delay : 20.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 113 : Message 24 - General Message (RBC n? 1) - Distance 650.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 17 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 140 d | Train running number from RBC |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 55 d |  |  |
| NID_OPERATIONAL | 32 | FFFF1234 h |  |  |

| - Step 116 : Message 24 - General Message (RBC n? 1) - Distance 650.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 24 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 0 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |

| - Step 118 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 650.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 120 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 650.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |

| - Step 123 : Balise Group BG 3 d 1/2 - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 0 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 42 d | Session management |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 113 d |  |  |
| Q_RBC | 1 | 1 b |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_RBC | 14 | 1515 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |
| Q_SLEEPSESSION | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 123 : Balise Group BG 3 d 2/2 - Distance 705.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| Q_UPDOWN | 1 | 1 b |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| Q_MEDIA | 1 | 0 b |  |  |
| N_PIG | 3 | 1 d |  |  |
| N_TOTAL | 3 | 1 d |  |  |
| M_DUP | 2 | 00 b |  |  |
| M_MCOUNT | 8 | 0 d |  |  |
| NID_C | 10 | 64 d |  |  |
| NID_BG | 14 | 3 d |  |  |
| Q_LINK | 1 | 1 b |  |  |
| NID_PACKET | 8 | 255 d | End of information |  |

| - Step 138 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 140 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 700.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0010000 b |  |  |

| - Step 142 : Message 159 - Session Established (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 21 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 3 d |  |  |
| L_PACKET | 13 | 0000001011010 b |  |  |
| N_ITER | 5 | 1 d |  |  |
| NID_RADIO | 64 | 003265342101FFFF h |  |  |

| - Step 144 : Message 129 - Validated Train Data (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 150 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NID_OPERATIONAL | 32 | 1234FFFF h |  |  |
| NC_TRAIN | 15 | 000100010000000 b |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 80 d |  |  |
| M_LOADINGGAUGE | 8 | 00000000 b |  |  |
| M_AXLELOAD | 7 | 0101101 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_STM | 8 | 2 d |  |  |
| NID_STM | 8 | 3 d |  |  |

| - Step 146 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 700.00 m Delay : 5.00 s Conditional Message Id : 129 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 148 : Message 15 - Conditional Emergency Stop (RBC n? 1) - Distance 700.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 15 d |  |  |
| L_MESSAGE | 10 | 13 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_EM | 4 | 1 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| Q_DIR | 2 | 01 b |  |  |
| D_EMERGENCYSTOP | 15 | 5000 d |  |  |

| - Step 150 : Message 147 - Acknowledgement of Emergency Stop (RBC n? 1) - Distance 700.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 147 d |  |  |
| L_MESSAGE | 10 | 25 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_EM | 4 | 1 d |  |  |
| Q_EMERGENCYSTOP | 2 | 00 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 250 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 7 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 154 : Message 18 - Revocation of Emergency Stop (RBC n? 1) - Distance 700.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 18 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_EM | 4 | 1 d |  |  |

| - Step 165 : Message 3 - Movement Authority (RBC n? 1) - Distance 700.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 3 d |  |  |
| L_MESSAGE | 10 | 43 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| NID_PACKET | 8 | 15 d | Level 2/3 movement authority |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 66 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| V_LOA | 7 | 0 d |  |  |
| T_LOA | 10 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| L_ENDSECTION | 15 | 2000 d |  |  |
| Q_SECTIONTIMER | 1 | 0 b |  |  |
| Q_ENDTIMER | 1 | 0 b |  |  |
| Q_DANGERPOINT | 1 | 0 b |  |  |
| Q_OVERLAP | 1 | 0 b |  |  |
| NID_PACKET | 8 | 21 d | Transmission of the gradient |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 54 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_GRADIENT | 15 | 0 d |  |  |
| Q_GDIR | 1 | 0 b |  |  |
| G_A | 8 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 27 d | International static speed profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 58 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_STATIC | 15 | 0 d |  |  |
| V_STATIC | 7 | 40 d |  |  |
| Q_FRONT | 1 | 0 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |
| NID_PACKET | 8 | 80 d | Mode profile |  |
| Q_DIR | 2 | 01 b |  |  |
| L_PACKET | 13 | 84 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| D_MAMODE | 15 | 0 d |  |  |
| M_MAMODE | 2 | 00 b |  |  |
| V_MAMODE | 7 | 10 d |  |  |
| L_MAMODE | 15 | 1500 d |  |  |
| L_ACKMAMODE | 15 | 0 d |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 176 : Message 136 - Train Position Report (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 500 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |

| - Step 188 : Message 129 - Validated Train Data (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 129 d |  |  |
| L_MESSAGE | 10 | 40 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 500 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0001 b |  |  |
| M_LEVEL | 3 | 011 b |  |  |
| NID_PACKET | 8 | 11 d | Validated train data |  |
| L_PACKET | 13 | 130 d |  |  |
| NID_OPERATIONAL | 32 | 1234FFFF h |  |  |
| NC_TRAIN | 15 | 000100010000000 b |  |  |
| L_TRAIN | 12 | 40 d |  |  |
| V_MAXTRAIN | 7 | 22 d |  |  |
| M_LOADINGGAUGE | 8 | 00000000 b |  |  |
| M_AXLELOAD | 7 | 0101101 b |  |  |
| M_AIRTIGHT | 2 | 00 b |  |  |
| N_ITER | 5 | 0 d |  |  |
| N_ITER | 5 | 2 d |  |  |
| NID_STM | 8 | 2 d |  |  |
| NID_STM | 8 | 3 d |  |  |

| - Step 190 : Message 8 - Acknowledgement of Train Data (RBC n? 1) - Distance 1200.00 m Delay : 30.00 s Conditional Message Id : 129 Back Delay : 30.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 8 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 1048577 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |

| - Step 202 : Message 136 - Train Position Report (RBC n? 1) - Distance 1200.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 136 d |  |  |
| L_MESSAGE | 10 | 28 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 1048579 d |  |  |
| D_LRBG | 15 | 450 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 1011 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |
| NID_PACKET | 8 | 4 d | Error reporting |  |
| L_PACKET | 13 | 29 d |  |  |
| M_ERROR | 8 | 00000111 b |  |  |