| - ERTMS/ETCS |  |
| --- | --- |
| Subset-076-6-3_9990200_01_v320.mdb State : Ordered list of Test Case Steps with communication information |  |
| REF : | Subset-076-6-3_9990200_01_v320.mdb |
| ISSUE : | 3.2.0 |
| DATE : | 10/04/17 14:36 |

| Company | Technical Approval | Management approval |
| --- | --- | --- |
| European Union Agency for Railways | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 3.2.0 10.04.2017 | All |  | Multitel |

# Sequence description

A3.2: List of National / Default Data

# Test cases

Important:	All columns called ‘Test Results’ are only placeholders for results of the test execution. They shall not be used for the description of test cases. 
A test is passed if all cells of ‘Test Result’ are passed.

|  | SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Step | Row |  |  | Dist. (m) |  | Previous |  |  |  |  | Description of Events |  | I/O |  | Interface |  | Comments |  |  | Next |  |  |  |  | Test Result |  |  |
|  |  |  |  |  |  |  | Levels |  | Mode |  |  |  |  |  |  |  |  |  |  |  | Levels |  |  | Mode |  |  |  |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 1 |  | 1 |  |  | 0.00 |  | N/A |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | N/A |  |  | NP |  | - |  |
|  | 2 |  | 2 |  |  | 0.00 |  | N/A |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 24: Start of Mission procedure is engaged |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 3 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | Driver opens desk |  | I |  | TIU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 4 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The 'CAB STATUS' message is RECORDED with the variable M_CAB_A_STATUS=1 (Active). |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 5 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The actual mode SB is DISPLAYED |  | O |  | DMI |  | The SB indication after opening the desk may not be recorded. Because the change of mode was already recorded while desk was closed |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 6 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | N/A |  |  | SB |  | - |  |
|  | 7 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 8 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 9 |  | 4 |  |  | 0.00 |  | N/A |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | N/A |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 10: Within the scope of the Start of Mission procedure in SB mode, the status of the ERTMS/ETCS Level data is INVALID. The driver selects a level (the existing ERTMS/ETCS Level or a new one). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 10 |  | 1 |  |  | 0.00 |  | N/A |  | SB |  |  | The driver selects "Level <L0/LNTC/L1/L2/L3>" |  | I |  | DMI |  | Depending on the implementation, a list of available levels may be presented to the driver on the DMI. The driver is then allowed to select one level among the proposed levels. For NTC levels, the abbreviations of the concerned National Systems shall be presented to the driver. _____ The Driver selects L0 |  |  | N/A |  |  | SB |  | - |  |
|  | 11 |  | 2 |  |  | 0.00 |  | N/A |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=<34/35/36/37/38>) is recorded |  | O |  | JRU |  | - |  |  | N/A |  |  | SB |  | - |  |
|  | 12 |  | 3 |  |  | 0.00 |  | N/A |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_LEVEL=<0/1/2/3/4>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 6: Within the scope of the Start of Mission procedure in SB mode, the On-Board equipment offers the driver to select either SH or NL or ‘Train Data Entry’ before the on-board equipment requests the driver to enter a complete set of Train Data. The driver selects ‘Train Data Entry’. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 13 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver can select either SH or NL or ‘Train Data Entry’ |  | O |  | DMI |  | Starting conditions The 'Non-Leading' button is enabled if the 'non leading' input signal is received. |  |  | L0 |  |  | SB |  | - |  |
|  | 14 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects ‘Train Data Entry’ |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 15 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | A Train data window is is displayed on the DMI. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 16 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver’s selection is recorded on the JRU. |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 47: The on-board equipment requests the driver to enter a complete set of Train Data within the scope of the Start of Mission procedure in SB mode when the status of the Train Data is unknown (S12) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 18 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter all Train Data unknown by the on-board equipment and prevents the driver from performing any other actions. |  | O |  | DMI |  | The driver might be requested to enter the 'Train category(ies)', 'Train length', 'Traction / brake parameters', 'Maximum train speed', 'Loading gauge', 'Axle load category', 'Train fitted with airtight system', but the driver shall never be requested to enter the 'Traction system(s) accepted by the engine', 'List of National Systems available on-board' or 'Axle number'. |  |  | L0 |  |  | SB |  | - |  |
|  | 19 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters each Train data unknown on-board |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 20 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is allowed to validate the train data |  | O |  | DMI |  | A complete set of valid Train Data is available |  |  | L0 |  |  | SB |  | - |  |
|  | 21 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the train data. |  | I |  | DMI |  | The on-board equipment allows the driver to validate as soon as a valid and complete set of Train Data is entered. |  |  | L0 |  |  | SB |  | - |  |
|  | 22 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded |  | O |  | JRU |  | Triggered by : The driver validation. |  |  | L0 |  |  | SB |  | - |  |
|  | 23 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN DATA (NID_MESSAGE_JRU=2) is recorded |  | O |  | JRU |  | Triggered by : Data Entry completed |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 46: After the set of Train Data has been entered/re-validated by the driver (in S12), if the status of the Train running number is 'unknown', the ERTMS/ETCS on-board equipment requests the driver to enter the Train running number (S13) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 25 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver is requested to enter the Train running number. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 26 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters the "Train Running Number" [MAIN] |  | I |  | DMI |  | Entered Train Running Number: NID_OPERATIONAL_1 |  |  | L0 |  |  | SB |  | - |  |
|  | 27 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | TRAIN RUNNING NUMBER ENTERED BY THE DRIVER (NID_MESSAGE_JRU=49; NID_OPERATIONAL=NID_OPERATIONAL_1) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040500: 5.4.5: Degraded Situations Test Case 12: Within the scope of the Start of Mission procedure in SB mode at S10, if valid Train Data is available the driver can select 'Start. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 29 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Start> button in the <Main> window is enabled. |  | O |  | DMI |  | In addition to possibilities offered at S10 |  |  | L0 |  |  | SB |  | - |  |
|  | 30 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver selects "Start" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 31 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS=0001 0011) is recorded |  | O |  | JRU |  | Start selected |  |  | L0 |  |  | SB |  | - |  |
|  | 32 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The <Default> window is displayed. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 33 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | The Mode symbol "Acknowledge Unfitted" is displayed |  | O |  | DMI |  | The on-board equipment will then waits for the driver to acknowledge the acknowledgement request for running in Unfitted mode DMI Object: Symbol MO17 |  |  | L0 |  |  | SB |  | - |  |
|  | 34 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit32=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 35 |  | 7 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver acknowledges "Unfitted" |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 36 |  | 8 |  |  | 0.00 |  | L0 |  | SB |  |  | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11; M_DRIVERACTIONS= 0000 0100) is recorded |  | O |  | JRU |  | Ack of Unfitted mode As soon as the driver acknowledges the on-board equipment switches to UN mode |  |  | L0 |  |  | SB |  | - |  |
|  | 37 |  | 9 |  |  | 0.00 |  | L0 |  | SB |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=4) is recorded |  | O |  | JRU |  | Triggering Event: When the mode changes |  |  | L0 |  |  | UN |  | - |  |
|  | 38 |  | 10 |  |  | 0.00 |  | L0 |  | UN |  |  | The Mode symbol "Unfitted" is displayed |  | O |  | DMI |  | DMI Object: Symbol MO16 |  |  | L0 |  |  | UN |  | - |  |
|  | 39 |  | 11 |  |  | 0.00 |  | L0 |  | UN |  |  | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit31=1>) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 9990200: A3.2: List of National / Default Data Test Case 14: The Roll Away Protection (RAP) commands the brakes because the train moves in a direction that conflicts with the current position of the direction controller in the active desk. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 40 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | Direction controller neutral |  | I |  | TIU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 41 |  | 2 |  |  | 0.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<greater than 0>) |  | I |  | INT |  | D=d1 The train moves in a direction: forwards, that conflicts with the current position of the direction controller in the active desk: neutral. The movement exceeds the national/default value for the allowed roll away distance. |  |  | L0 |  |  | UN |  | - |  |
|  | 42 |  | 3 |  |  | 2.00 |  | L0 |  | UN |  |  | D=d1 + D_NVROLL The brake is APPLIED. |  | O |  | TIU |  | The brake command is triggered at Roll Away Protection (RAP) after a distance specified by the national/default value: D_NVROLL (default value = 2m) Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 43 |  | 4 |  |  | 2.00 |  | L0 |  | UN |  |  | The brake indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 44 |  | 5 |  |  | 2.00 |  | L0 |  | UN |  |  | The brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 45 |  | 6 |  |  | 2.00 |  | L0 |  | UN |  |  | An indication is given to the driver showing that RAP is commanding the brakes |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 46 |  | 7 |  |  | 3.00 |  | L0 |  | UN |  |  | Train is at standstill (V_TRAIN=0; TIME= 10) |  | I |  | INT |  | D=d2 |  |  | L0 |  |  | UN |  | - |  |
|  | 47 |  | 8 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 48 |  | 9 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledges. |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 49 |  | 10 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 50 |  | 11 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 51 |  | 12 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake is RELEASED. |  | O |  | TIU |  | Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 52 |  | 13 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake indication is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 53 |  | 14 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake release is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 54 |  | 15 |  |  | 3.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<greater than 0>) |  | I |  | INT |  | D=d2 The train moves in a direction: backwards, that conflicts with the current position of the direction controller in the active desk: neutral. The movement exceeds the national/default value for the allowed roll away distance. |  |  | L0 |  |  | UN |  | - |  |
|  | 55 |  | 16 |  |  | 1.00 |  | L0 |  | UN |  |  | D=d2 - D_NVROLL The brake is APPLIED. |  | O |  | TIU |  | The brake command is triggered at Roll Away Protection (RAP) after a distance specified by the national/default value: D_NVROLL (default value = 2m) Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 56 |  | 17 |  |  | 1.00 |  | L0 |  | UN |  |  | The brake indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 57 |  | 18 |  |  | 1.00 |  | L0 |  | UN |  |  | The brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 58 |  | 19 |  |  | 1.00 |  | L0 |  | UN |  |  | An indication is given to the driver showing that RAP is commanding the brakes |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 59 |  | 20 |  |  | 0.00 |  | L0 |  | UN |  |  | Train is at standstill (V_TRAIN=0; TIME= 10) |  | I |  | INT |  | D=d3 |  |  | L0 |  |  | UN |  | - |  |
|  | 60 |  | 21 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 61 |  | 22 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledges. |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 62 |  | 23 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 63 |  | 24 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 64 |  | 25 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake is RELEASED. |  | O |  | TIU |  | Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 65 |  | 26 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake indication is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 66 |  | 27 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake release is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 67 |  | 28 |  |  | 0.00 |  | L0 |  | UN |  |  | Direction controller backward |  | I |  | TIU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 68 |  | 29 |  |  | 0.00 |  | L0 |  | UN |  |  | DIRECTION CONTROLLER POSITION (NID_MESSAGE_JRU=39) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 69 |  | 30 |  |  | 0.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<greater than 0>) |  | I |  | INT |  | D=d3 The train moves in a direction: forwards, that conflicts with the current position of the direction controller in the active desk: backwards. The movement exceeds the national/default value for the allowed roll away distance. |  |  | L0 |  |  | UN |  | - |  |
|  | 70 |  | 31 |  |  | 2.00 |  | L0 |  | UN |  |  | D=d3 + D_NVROLL The brake is APPLIED. |  | O |  | TIU |  | The brake command is triggered at Roll Away Protection (RAP) after a distance specified by the national/default value: D_NVROLL (default value = 2m) Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 71 |  | 32 |  |  | 2.00 |  | L0 |  | UN |  |  | The brake indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 72 |  | 33 |  |  | 2.00 |  | L0 |  | UN |  |  | The brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 73 |  | 34 |  |  | 2.00 |  | L0 |  | UN |  |  | An indication is given to the driver showing that RAP is commanding the brakes |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 74 |  | 35 |  |  | 3.00 |  | L0 |  | UN |  |  | Train is at standstill (V_TRAIN=0; TIME= 10) |  | I |  | INT |  | D=d4 |  |  | L0 |  |  | UN |  | - |  |
|  | 75 |  | 36 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 76 |  | 37 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledges. |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 77 |  | 38 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 78 |  | 39 |  |  | 3.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 79 |  | 40 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake is RELEASED. |  | O |  | TIU |  | Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 80 |  | 41 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake indication is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 81 |  | 42 |  |  | 3.00 |  | L0 |  | UN |  |  | The brake release is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 82 |  | 43 |  |  | 3.00 |  | L0 |  | UN |  |  | Direction controller forward |  | I |  | TIU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 83 |  | 44 |  |  | 3.00 |  | L0 |  | UN |  |  | DIRECTION CONTROLLER POSITION (NID_MESSAGE_JRU=39) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 84 |  | 45 |  |  | 3.00 |  | L0 |  | UN |  |  | Train is moving (V_TRAIN=<greater than 0>) |  | I |  | INT |  | D=d4 The train moves in a direction: backwards, that conflicts with the current position of the direction controller in the active desk: forward. The movement exceeds the national/default value for the allowed roll away distance. |  |  | L0 |  |  | UN |  | - |  |
|  | 85 |  | 46 |  |  | 1.00 |  | L0 |  | UN |  |  | D=d4 - D_NVROLL The brake is APPLIED. |  | O |  | TIU |  | The brake command is triggered at Roll Away Protection (RAP) after a distance specified by the national/default value: D_NVROLL (default value = 2m) Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 86 |  | 47 |  |  | 1.00 |  | L0 |  | UN |  |  | The brake indication is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 87 |  | 48 |  |  | 1.00 |  | L0 |  | UN |  |  | The brake intervention is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 88 |  | 49 |  |  | 1.00 |  | L0 |  | UN |  |  | An indication is given to the driver showing that RAP is commanding the brakes |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 89 |  | 50 |  |  | 0.00 |  | L0 |  | UN |  |  | Train is at standstill (V_TRAIN=0; TIME= 10) |  | I |  | INT |  | D=d5 |  |  | L0 |  |  | UN |  | - |  |
|  | 90 |  | 51 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is DISPLAYED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 91 |  | 52 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledges. |  | I |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 92 |  | 53 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 93 |  | 54 |  |  | 0.00 |  | L0 |  | UN |  |  | The driver acknowledgement request is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 94 |  | 55 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake is RELEASED. |  | O |  | TIU |  | Current mode is UNCHANGED |  |  | L0 |  |  | UN |  | - |  |
|  | 95 |  | 56 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake indication is REMOVED. |  | O |  | DMI |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | 96 |  | 57 |  |  | 0.00 |  | L0 |  | UN |  |  | The brake release is RECORDED. |  | O |  | JRU |  | - |  |  | L0 |  |  | UN |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 97 |  | 1 |  |  | 0.00 |  | L0 |  | UN |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - _____ The train is at standstill. |  |  | L0 |  |  | NP |  | - |  |
|  | 98 |  | 2 |  |  | 0.00 |  | L0 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L0 |  |  | NP |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 10: The ETCS on-board equipment is powered .Transition Condition Id [4]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 99 |  | 1 |  |  | 0.00 |  | L0 |  | NP |  |  | ERTMS/ETCS on-board unit is powered up |  | I |  | INT |  | The on-board equipment changes to SB mode. |  |  | L0 |  |  | NP |  | - |  |
|  | 100 |  | 2 |  |  | 0.00 |  | L0 |  | NP |  |  | GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=6) is recorded |  | O |  | JRU |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 7: The on-board equipment requests the driver to enter a driver ID within the scope of the Start of Mission procedure in SB mode when the status of the driver ID is unknown. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 101 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment requires the driver to enter a Driver ID There is NO initial Driver ID value displayed |  | O |  | DMI |  | Starting conditions : There is no value initially displayed since the status of the driver ID is unknown. |  |  | L0 |  |  | SB |  | - |  |
|  | 102 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The on-board equipment prevents the driver from continuing the procedure before the validation of the Driver ID is done. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 103 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver enters and validates a correct Driver ID |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 104 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The validated Driver ID is recorded on the JRU with the DRIVER_ID variable in a 'GENERAL MESSAGE'. |  | O |  | JRU |  | A 'GENERAL MESSAGE' is recorded every 5s by the on-board equipment. |  |  | L0 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 12: The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data before validating it within the scope of the Start of Mission procedure in SB mode when the status of the ERTMS/ETCS Level data is INVALID. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 105 |  | 1 |  |  | 0.00 |  | L0 |  | SB |  |  | A ETCS level is displayed to the driver |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 106 |  | 2 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver modifies the ERTMS/ETCS Level data (e.g. a new valid data is input). |  | I |  | DMI |  | The on-board equipment allows the driver to modify an existing ERTMS/ETCS Level data. _____ The driver selects L2. |  |  | L0 |  |  | SB |  | - |  |
|  | 107 |  | 3 |  |  | 0.00 |  | L0 |  | SB |  |  | The new ERTMS/ETCS Level data is displayed on the DMI The On-Board equipment still allows the driver to validate it. |  | O |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 108 |  | 4 |  |  | 0.00 |  | L0 |  | SB |  |  | The driver validates the ERTMS/ETCS level. |  | I |  | DMI |  | - |  |  | L0 |  |  | SB |  | - |  |
|  | 109 |  | 5 |  |  | 0.00 |  | L0 |  | SB |  |  | Driver’s validation action is recorded on the JRU. |  | O |  | JRU |  | Triggered by : The driver acts on the on-board DMI. |  |  | L0 |  |  | SB |  | - |  |
|  | 110 |  | 6 |  |  | 0.00 |  | L0 |  | SB |  |  | The ERTMS/ETCS Level data re-entered by the driver is recorded on the JRU |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 35: On-board equipment offers the driver the possibility to re-enter the Radio Network ID. The driver select a new Radio Network ID from the proposed list. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 111 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select: - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 112 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses the ‘Radio network ID’ button |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 113 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The Radio Network ID window is displayed with a list of available and allowed networks. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 114 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver selects a new Radio Network ID and validates it. |  | I |  | DMI |  | - _____ The driver selects "Network2" (NID_MN=654322h) |  |  | L2 |  |  | SB |  | - |  |
|  | 115 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | The Radio data window is displayed again. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 116 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | RADIO NETWORK ID ENTERED BY THE DRIVER (NID_MESSAGE_JRU=48) is recorded |  | O |  | JRU |  | NID_MN is identical to the data selected by the driver |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 14: If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to use the EIRENE short number (trackside call routing function) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 118 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The on-board equipment displays the Radio data window and offers the possibility to the driver to select: - ‘Contact last RBC’, - ‘Use short number’, - ‘Enter RBC data’ and - ‘Radio network ID’. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 119 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses the 'Use short number' button. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 120 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | RBC CONTACT INFORMATION ENTERED BY THE DRIVER (NID_MESSAGE_JRU=24; Q_RBCENTRY=1) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 122 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  |  | L2 |  |  | SB |  | - |  |
|  | 123 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  |  | L2 |  |  | SB |  | - |  |
|  | 124 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 125 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 126 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 127 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 128 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SB |  | - |  |
|  | 129 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 130 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 131 |  | 10 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 33: Train reports invalid position and RBC rejects the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 134 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | Onboard reports invalid position to RBC |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 135 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | Message 157 RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 136 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | RBC reports TRAIN REJECTED (A38) |  | I |  | RTM |  | Stored message 40, with NID_LRBG set to unknown |  |  | L2 |  |  | SB |  | - |  |
|  | 137 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | Message 40 RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 138 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | On-board Equipment deletes stored position data in case they were invalid (new status: 'unknown'). |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 7: The on-board equipment terminates a communication session without any explicit trackside order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 140 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | The onboard equipment detects an error condition that requires the termination of the communication session. It shall be checked that no more messages are sent If L1 message is sent to the RIU. If L2 or L3 message is sent to the RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 141 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 143 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is received from the RBC and the onboard equipment considers the communication session terminated. If L1 message is sent from the RIU. If L2 or L3 message is sent from the RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 144 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 146 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The onboard tries to release the safe connection. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 33: Train reports invalid position and RBC rejects the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 147 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | On-board equipment informs driver with the system status message 'Train is rejected'. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 43: During the SoM procedure, following S10 or S20, if the driver chooses to re-enter the level, the status of the stored ERTMS/ETCS level and RBC ID/Phone Number changes from valid to invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 148 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The Main window is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 149 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The Level symbol "L2 / L3" is displayed |  | O |  | DMI |  | The ERTMS/ETCS level status is valid |  |  | L2 |  |  | SB |  | - |  |
|  | 150 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Data view" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 151 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | The data from "Data View" window are displayed: RBC ID/Phone Number |  | O |  | DMI |  | The RBC ID/Phone Number status is valid |  |  | L2 |  |  | SB |  | - |  |
|  | 152 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Level" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 153 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | No Level symbol is displayed in C8 |  | O |  | DMI |  | The ERTMS/ETCS level status is invalid |  |  | L2 |  |  | SB |  | - |  |
|  | 154 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | The invalid ERTMS/ETCS level stored on-board is presented to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 155 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver confirms "Level" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 157 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Contact last RBC" |  | I |  | DMI |  | This button is available only if RBC ID/Phone Number is invalid |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 159 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  |  | L2 |  |  | SB |  | - |  |
|  | 160 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  |  | L2 |  |  | SB |  | - |  |
|  | 161 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 162 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 163 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 164 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 165 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SB |  | - |  |
|  | 166 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 167 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 168 |  | 10 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 20: Train reports unknown position and RBC rejects the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 171 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | Onboard reports unknown position to RBC |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 172 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | Message 157 RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 173 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | RBC reports TRAIN REJECTED (A38) |  | I |  | RTM |  | Stored message 40, with NID_LRBG set to unknown |  |  | L2 |  |  | SB |  | - |  |
|  | 174 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | Message 40 RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050500: 3.5.5: Terminating a communication session Test Case 7: The on-board equipment terminates a communication session without any explicit trackside order. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 176 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Termination of a communication session" (NID_MESSAGE=156) is transmitted |  | O |  | RTM |  | The onboard equipment detects an error condition that requires the termination of the communication session. It shall be checked that no more messages are sent If L1 message is sent to the RIU. If L2 or L3 message is sent to the RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 177 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 179 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "Acknowledgement of termination of a communication session" (NID_MESSAGE=39) is received |  | I |  | RTM |  | Message 39: 'Acknowledgement of termination of a communication session' is received from the RBC and the onboard equipment considers the communication session terminated. If L1 message is sent from the RIU. If L2 or L3 message is sent from the RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 180 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | If safe connection is set up with an RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 182 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DISCONNECT.Request is transmitted |  | O |  | RTM |  | The onboard tries to release the safe connection. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 20: Train reports unknown position and RBC rejects the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 183 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | On-board equipment informs driver with the system status message 'Train is rejected'. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 43: During the SoM procedure, following S10 or S20, if the driver chooses to re-enter the level, the status of the stored ERTMS/ETCS level and RBC ID/Phone Number changes from valid to invalid. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 184 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The Main window is displayed. |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 185 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The Level symbol "L2 / L3" is displayed |  | O |  | DMI |  | The ERTMS/ETCS level status is valid |  |  | L2 |  |  | SB |  | - |  |
|  | 186 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Data view" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 187 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | The data from "Data View" window are displayed: RBC ID/Phone Number |  | O |  | DMI |  | The RBC ID/Phone Number status is valid |  |  | L2 |  |  | SB |  | - |  |
|  | 188 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Level" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 189 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | No Level symbol is displayed in C8 |  | O |  | DMI |  | The ERTMS/ETCS level status is invalid |  |  | L2 |  |  | SB |  | - |  |
|  | 190 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | The invalid ERTMS/ETCS level stored on-board is presented to the driver |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 191 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver confirms "Level" |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 193 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses "Contact last RBC" |  | I |  | DMI |  | This button is available only if RBC ID/Phone Number is invalid |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 3050300: 3.5.3: Establishing a communication session Test Case 23: Within the Start of Mission procedure, system version is compatible and session is established. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 195 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Request is transmitted |  | O |  | RTM |  | According to EURORADIO specifications. This attempt shall be repeated until successful or up to 3 times. The onboard tries to set up a connection with the phone entered in step 1. |  |  | L2 |  |  | SB |  | - |  |
|  | 196 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-CONNECT.Confirm is received |  | I |  | RTM |  | Setting up of a safe connection is successful. |  |  | L2 |  |  | SB |  | - |  |
|  | 197 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The Status symbol "Safe radio connection - Connection Up" is displayed |  | O |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 198 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | JRU step 'DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded'. |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 199 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Initiation of a communication session" (NID_MESSAGE=155) is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 200 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 201 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Indication with Euroradio Message "RBC/RIU System Version" (NID_MESSAGE=32) is received |  | I |  | RTM |  | System versions M_VERSION (Version of the ERTMS/ETCS language) are compatible, so that the communication session established for on-board |  |  | L2 |  |  | SB |  | - |  |
|  | 202 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 203 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "Session Established" (NID_MESSAGE=159) including packet 2 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 204 |  | 10 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 205 |  | 11 |  |  | 0.00 |  | L2 |  | SB |  |  | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 206 |  | 12 |  |  | 0.00 |  | L2 |  | SB |  |  | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 32: On-board reports invalid position and RBC accepts the train |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 209 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | RBC is Not able to confirm position (D33), it accepts the train (D22), it reports train accepted (A23) |  | I |  | RTM |  | message 41, with NID_LRBG set to unknown |  |  | L2 |  |  | SB |  | - |  |
|  | 210 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | message 41, with NID_LRBG set to unknown RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 211 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | Any message (NID_MESSAGE_JRU=ALL; NID_LRBG=16777215) is recorded |  | O |  | JRU |  | On-board Equipment deletes stored position data in case they were invalid (new status: 'unknown'). |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5060200: 5.6.2: Table of requirements for 'Shunting Initiated by Driver' procedure Test Case 4: The train is at standstill and the driver selects shunting. A radio message 130 (request for shunting) is sent to the RBC together with a position report. The on-board equipment awaits the replay to the SH request. When the radio message 27 (shunting refused) is received from the RBC, the driver is informed that the shunting request has been refused by the RBC and the process ends. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 213 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver presses 'shunting' button. |  | I |  | DMI |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 214 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The driver’s action 'Shunting selected' is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 215 |  | 3 |  |  | 0.00 |  | L2 |  | SB |  |  | The radio message 130 (request for shunting) is sent to the RBC |  | O |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 216 |  | 4 |  |  | 0.00 |  | L2 |  | SB |  |  | The radio message 130 sent by the on-board is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 217 |  | 5 |  |  | 0.00 |  | L2 |  | SB |  |  | In the main window, the hourglass symbol is DISPLAYED |  | O |  | DMI |  | This hourglass symbol is ST05 (according to the document ERA_ERTMS_015560 v330), which shall be displayed to show that the onboard awaits an answer from the RBC |  |  | L2 |  |  | SB |  | - |  |
|  | 218 |  | 6 |  |  | 0.00 |  | L2 |  | SB |  |  | The radio message 27 (shunting refused) is received from RBC |  | I |  | RTM |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 219 |  | 7 |  |  | 0.00 |  | L2 |  | SB |  |  | The radio message 27 received from RBC is RECORDED |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | 220 |  | 8 |  |  | 0.00 |  | L2 |  | SB |  |  | The text message 'Shunting refused' is DISPLAYED |  | O |  | DMI |  | Inform driver |  |  | L2 |  |  | SB |  | - |  |
|  | 221 |  | 9 |  |  | 0.00 |  | L2 |  | SB |  |  | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded |  | O |  | JRU |  | - |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 5040300: 5.4.3: Table of requirements for 'Start of Mission' procedure Test Case 52: Driver selects 'Shunting' (train is at standstill in L2/L3) and the shunting request is refused by the RBC. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 222 |  | 2 |  |  | 0.00 |  | L2 |  | SB |  |  | The Main window is displayed. |  | O |  | DMI |  | As the RBC rejects the request for Shunting, the process goes back to S10 state. |  |  | L2 |  |  | SB |  | - |  |
|  | Feature 4060300: 4.6.3: Transitions Conditions Table Test Case 1: The ETCS on-board system is NOT powered. Transition Condition Id [29]. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 223 |  | 1 |  |  | 0.00 |  | L2 |  | SB |  |  | The ERTMS/ETCS on-board unit is powered down |  | I |  | INT |  | - |  |  | L2 |  |  | NP |  | - |  |
|  | 224 |  | 2 |  |  | 0.00 |  | L2 |  | NP |  |  | NOT GENERAL MESSAGE (NID_MESSAGE_JRU=1; M_MODE=-) is recorded |  | O |  | JRU |  | There are no messages recorded in the JRU during a period of time |  |  | L2 |  |  | NP |  | - |  |
| Step 10: DMI Events Distance 0.00 m |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Event Id. |  |  |  | Event Name |  |  |  |  |  |  | Data |  |  |  |  |  |  |  |  | Delay |  |  |  |  |  |  |  |  |
| 29 |  |  |  | Request Change Level |  |  |  |  |  |  | LEVEL=1 |  |  |  |  |  |  |  |  | 10 s |  |  |  |  |  |  |  |  |

| - Step 126 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 128 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 130 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 134 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 00 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 136 : Message 40 - Train Rejected (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 40 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |

| - Step 140 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 143 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |

| - Step 163 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 165 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 167 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 171 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 10 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 173 : Message 40 - Train Rejected (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 40 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |

| - Step 176 : Message 156 - Termination of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 156 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 179 : Message 39 - Acknowledgement of Termination of Communication Session (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 156 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 39 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |

| - Step 199 : Message 155 - Initiation of a Communication Session (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 155 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |

| - Step 201 : Message 32 - RBC/RIU System Version (RBC n? 1) - Distance 0.00 m Delay : 5.00 s Conditional Message Id : 155 Back Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 32 d |  |  |
| L_MESSAGE | 10 | 11 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |

| - Step 203 : Message 159 - Session Established (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 159 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 2 d | System versions that the on-board equipment is able to operate |  |
| L_PACKET | 13 | 33 d |  |  |
| M_VERSION | 7 | 0100001 b |  |  |
| N_ITER | 5 | 0 d |  |  |

| - Step 205 : Message 157 - SoM Position Report (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 157 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| Q_STATUS | 2 | 01 b |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 6 d |  |  |
| M_LEVEL | 3 | 000 b |  |  |

| - Step 209 : Message 41 - Train Accepted (RBC n? 1) - Distance 0.00 m Delay : 5.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 41 d |  |  |
| L_MESSAGE | 10 | 10 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 111111111111111111111111 b |  |  |

| - Step 215 : Message 130 - Request for Shunting (RBC n? 1) - Distance 0.00 m |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 130 d |  |  |
| L_MESSAGE | 10 | 24 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| NID_ENGINE | 24 | 76000 d |  |  |
| NID_PACKET | 8 | 0 d | Position report |  |
| L_PACKET | 13 | 114 d |  |  |
| Q_SCALE | 2 | 01 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| D_LRBG | 15 | 0 d |  |  |
| Q_DIRLRBG | 2 | 01 b |  |  |
| Q_DLRBG | 2 | 01 b |  |  |
| L_DOUBTOVER | 15 | 0 d |  |  |
| L_DOUBTUNDER | 15 | 0 d |  |  |
| Q_LENGTH | 2 | 00 b |  |  |
| V_TRAIN | 7 | 0 d |  |  |
| Q_DIRTRAIN | 2 | 01 b |  |  |
| M_MODE | 4 | 0110 b |  |  |
| M_LEVEL | 3 | 3 d |  |  |

| - Step 218 : Message 27 - SH Refused (RBC n? 1) - Distance 0.00 m Delay : 20.00 s |  |  |  |  |
| --- | --- | --- | --- | --- |
| Variable | Length | Value | Comment | User comment |
| NID_MESSAGE | 8 | 27 d |  |  |
| L_MESSAGE | 10 | 14 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |
| M_ACK | 1 | 0 b |  |  |
| NID_LRBG | 24 | 16777215 d |  |  |
| T_TRAIN | 32 | 0 d |  |  |