| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3100300 (3.10.3: Revocation of an Emergency Message) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-3100300 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | Creation | DLR |
| Version 2.9.0 05.10.2012 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 18.07.2013 | All | Second version/UNISIG review comments from 16.07.2013 included | INECO |
| Version 2.9.2 09.12.2013 | All | Third version/ Internal modification;TC_Editorv27; Correction of Requirements blank space; Additional corrections from the author; Review sheet version 005. | INECO |
| Version 2.9.3 26.12.2013 | All | Fourth version/ Internal version: Cross-reference errors corrected. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.10.3: Revocation of an Emergency Message |  | 3100300 |
| Test Case of Feature | The train do not acknowledge the revocation of a Conditional Emergency Stop message when it is not requested. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS / LS / OS |  |  |
| Target of Test | The target of the test is to check that: Reception of message 18 ‘Revocation of Emergency Stop’ from the supervising RBC, including a request for no acknowledgement needed. (M_ACK=0) The on-board equipment does not send any message 146 ‘Acknowledgement’ to the RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.3.2 |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU interface that: No message 146 is recorded after the reception of message 18. |
| Constraints | Information about ‘Conditional Emergency Stop’ from the supervising RBC stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Conditional Emergency Stop | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA .Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Acknowledgement” (NID_MESSAGE=146) is transmitted. | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | Revocation of Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 12 | FS / OS / LS |
| Radio communication session | ESTABLISHED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Conditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.10.3: Revocation of an Emergency Message |  | 3100300 |
| Test Case of Feature | The train do not acknowledge the revocation of an “Unconditional Emergency Stop” message when it is not requested. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: PT |  |  |
| Target of Test | The target of the test is to check that: Reception of message 18 ‘Revocation of Emergency Stop’ from the supervising RBC, including a request for no acknowledgement needed. (M_ACK=0) The on-board equipment does not send any message 146 ‘Acknowledgement’ to the RBC. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.10.3.2 |  |
|  |  | Subset-026-5.11.2.2 D130 |  |
|  |  | Subset-026-5.11.2.2 S130 |  |
|  |  | Subset-026-8.7.8 Message 18: Revocation of Emergency Stop |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM interface that: No message 146 is recorded after the reception of message 18. |
| Constraints | Information about an ‘Unconditional Emergency Stop’ received from the supervising RBC stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Unconditional Emergency Stop | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Revocation of Emergency Stop" (NID_MESSAGE=18) | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Acknowledgement” (NID_MESSAGE=146) is transmitted. | O | RTM | - | N/A | N/A | - |

| Step 1: Radio Message 18 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 18 | Revocation of Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | No acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 8 | PT |
| Radio communication session | ESTABLISHED | - |
| Unconditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |