| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 6060303 (6.6.3.3 Messages received from RBC/RIU) Total: 4 Test case(s) |  |
| REF: | Subset-076-5-2-6060303 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3 | INECO |
| Version 2.9.1 17.05.2013 | All | Second version/TCs Template updated and UNISIG remarks from 03/05/2013 included | INECO |
| Version 2.9.2 13.06.4862 | All | Third version/Internal modification;TC_Editorv27; Correction of Requirements blank space | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.3 Messages received from RBC/RIU |  | 6060303 |
| Test Case of Feature | The on-board equipment receives from an RBC certified to the system version number X=1 a Conditional Emergency Stop message that is accepted. The on-board equipment translated the received data correctly. Tested action: [T = TRANSLATED]. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: On-board equipment ( system version X=2) receives an emergency message 15: “Conditional Emergency Stop” from an RBC certified with a system version number X=1 The on-board equipment is able to translate the message by adding the Variable D_REF and setting it to 0 The on-board equipment sends the acknowledgement message 147: “Acknowledgement of Emergency Stop” to the RBC translated to system version number X=1 by setting the variable Q_EMERCENCYSTOP from 1 to 0. The driver is informed about the acceptance of an Conditional Emergency Stop. The text message “Emergency Stop” is displayed on DMI. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 6 |  |
|  |  | Subset-026-6.6.3.3.2 [1] |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 15 |  |
|  |  | Subset-026-6.6.3.4.5 [5] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: The driver is informed about the acceptance of the CES with the message “Emergency stop “ Check on the JRU that: Reception of message 15 is recorded and translated correctly. (D_REF set to 0) Transmission of message 147 is recorded and translated correctly. (Q_EMERGENCYSTOP set from 1 to 0) |
| Constraints | This section applies for the parts of radio messages, excluding the packets themselves, which are received from an RBC/RIU certified to the system version number X = 1. No " Conditional Emergency Stop" has been received before. Comment: Tested action: [T = TRANSLATED]. Exception [1] from table 6.6.3.3.2. D_REF set to 0 Exception [5] from table 6.6.3.4.5: Q_EMERGENCYSTOP set from 1 to 0 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only applicable for LS and OS modes |
| Conditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received from an RBC certified with system version X=1. | I | RTM | 001 0000 | The on-board translates the message for system version X=2 adding D_REF = 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted to the RBC (X=1). | O | RTM | 001 0000 | The on-board transmits the value Q_EMERGENCYSTOP set to 0 for X=1 RBC .Translation type from 1 to 0. | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | The system status message "Emergency stop" is displayed. | O | DMI | - | The driver is informed that the accepted CES message is received. | N/A | N/A | - |
| 6 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded. | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Conditional Emergency stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 3: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | Acknowledgement of Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | 0 | Translated from 1(X=2) to 0 (for X=1) |
| NID_PACKET | 8 | 0 | Packet 0 or 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Linking | STORED | UNCHANGED |
| Movement Authority | STORED | UNCHANGED |
| Gradient Profile | STORED | UNCHANGED |
| International SSP | STORED | UNCHANGED |
| Mode Profile | STORED | UNCHANGED Only applicable for LS and OS modes |
| Conditional Emergency Stop | STORED | CHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.3 Messages received from RBC/RIU |  | 6060303 |
| Test Case of Feature | The on-board equipment receives from an RBC certified to the system version number X=1 an Conditional Emergency Stop message that is rejected. The on-board equipment translated the received data correctly. Tested action [T = TRANSLATED]. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: On-board equipment ( system version X=2) receives an emergency message 15: “Conditional Emergency Stop” from an RBC certified with a system version number X=1 The on-board equipment is able to translate the message by adding the Variable D_REF and setting it to 0. The on-board sends the acknowledgement message 147: “Acknowledgement of Emergency Stop” to the RBC translated to system version number X=1 by setting the variable Q_EMERCENCYSTOP from 3 to 1. The train ignores this Conditional Emergency Stop. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 6 |  |
|  |  | Subset-026-6.6.3.3.2 [1] |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 15 |  |
|  |  | Subset-026-6.6.3.4.5 [5] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the DMI that: Target speed and target distance are Not Updated. Check on the JRU that: Reception of message 15 is recorded and translated correctly. (D_REF set to 0) Transmission of message 147 is recorded and translated correctly. (Q_EMERGENCYSTOP set from 3 to 1) |
| Constraints | This section applies for the parts of radio messages, excluding the packets themselves, which are received from an RBC/RIU certified to the system version number X = 1. Stop location D_EMERGENCYSTOP shall be not beyond EOA and SvL in this case the Conditional Emergency Stop shall be ignored . No " Emergency Stop" has been received before. Comment: Tested action: [T = TRANSLATED]. Exception [1] from table 6.6.3.3.2. D_REF set to 0 Exception [5] from table 6.6.3.4.5: Q_EMERGENCYSTOP set from 3 to 1 |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| Movement Authority | STORED | - |
| Gradient Profile | STORED | - |
| International SSP | STORED | - |
| Mode Profile | STORED | Only applicable for LS and OS modes |
| Conditional Emergency Stop | NOT STORED | - |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The speed and distance monitoring information is displayed. | O | DMI | Target Distance |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Conditional Emergency Stop" (NID_MESSAGE=15) is received from an RBC certified with system version X=1. | I | RTM | 0010000 | The on-board translates the message for system version X=2 adding D_REF = 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Acknowledgement of Emergency Stop" (NID_MESSAGE=147) is transmitted to the RBC (X=1). | O | RTM | 0010000 | The on-board transmittes the value Q_EMERGENCYSTOP set to1 for X=1 RBC .Translation type from 3 to 1 ( CES is rejected). | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |
| 5 | N/A | N/A | Service brake not commanded | O | TIU | - | Stop location already passed by the train. Emergency stop rejected. | N/A | N/A | - |
| 6 | N/A | N/A | Curent EOA and SvL are not Updated. | O | DMI | - | Optionnal for LS mode Stop location already passed by the train. Emergency stop rejected. | N/A | N/A | - |

| Step 1: Radio Message 15 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 15 | Conditional Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the emergency stop message |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_EMERGENCYSTOP | 15 | FINITE VALUE | Distance between LRBG and the position reference to the emergency stop |

| Step 3: Radio Message 147 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 147 | Acknowledgement of Emergency Stop |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_EM | 4 | FINITE VALUE | Identification number of the acknowledged emergency message |
| Q_EMERGENCYSTOP | 2 | 1 | Translated from 3( X=2) to 1 (for X=1) |
| NID_PACKET | 8 | 0 | Packet 0 or 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Linking | STORED | UNCHANGED |
| Movement Authority | STORED | UNCHANGED |
| Gradient Profile | STORED | UNCHANGED |
| International SSP | STORED | UNCHANGED |
| Mode Profile | STORED | Only applicable for LS and OS modes UNCHANGED |
| Conditional Emergency Stop | NOT STORED | UNCHANGED |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.3 Messages received from RBC/RIU |  | 6060303 |
| Test Case of Feature | The on-board equipment receives from an RBC certified to the system version number X=1 a request for a Track Ahead Free, message 34. The on-board equipment translated the received data to system version X=2 correctly. Tested action [T= TRANSLATED] & [U= UNCHANGED]. |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3: LS/ SR/ OS/ SB/ PT |  |  |
| Target of Test | The target of this test is to check that: On-board equipment receives a request for Track Ahead Free, message 34: “Track Ahead Free request” from an RBC certified with a system version number X=1 The on-board equipment translates correctly the received message by adding the variable D_REF and setting it to 0. The on-board equipment accepts and uses the received information . A request for driver acknowledgment, message “Track Ahead Free Request” is DISPLAYED at the right distance. After driver acknowledgment, the on-board equipment sends message 149: “Track Ahead Free Granted” to the RBC ( system version X=2) without the necessity of being translated. The transmitted data, message 149, to RBC is Unchanged “U”. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 13 |  |
|  |  | Subset-026-6.6.3.3.2 [1] |  |
|  |  | Subset-026-6.6.3.4.4 |  |
|  |  | Subset-026-6.6.3.4.5 Table - row 16 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on DMI that: Message “Track Ahead Free Request” is DISPLAYED at the right distance. After driver acknowledgment the on-board stops displaying the message “Track Ahead Free Request” Check on the JRU that: Reception of message 34 is recorded and translated correctly.(D_REF set to 0) Transmission of message 149 is recorded and remains unchanged. |
| Constraints | This section applies for the parts of radio messages, excluding the packets themselves, which are received from an RBC/RIU certified to the system version number X = 1. For SB mode : A cab needs to be active. For PT mode: The information “Recognition of Exit from TR mode” with a more recent time stamp was received. No " Track Ahead Free Request " has been received before. Comment: Tested action: [T = TRANSLATED] Exception [1] from table 6.6.3.3.2.: D_REF set to 0 & [U=UNCHANGED] from table 6.6.3.4.5. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 2/ 6/ 8/ 12 | OS/ SR/ SB / PT/ LS |
| Radio communication session | ESTABLISHED | - |
| Mode Profile | STORED | Only applicable for LS and OS modes |
| ERTMS/ETCS level | 3 / 4 | L2/ L3 |
| Track Ahead Free Request | NOT STORED | - |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "Track Ahead Free Request" (NID_MESSAGE=34) is received from an RBC certified with system version X=1. | I | RTM | 0010000 | The on-board translates the message for system version X=2 adding D_REF = 0 | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 3 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is displayed. | O | DMI | - | When the train reaches the D_TAFDISPLAY location Symbol DR02 | N/A | N/A | - |
| 4 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21, bit_and [DMI_SYMB_STATUS,MASK]=MASK)) is recorded. | O | JRU | - | MASK=0200000000000000000000 | N/A | N/A | - |
| 5 | N/A | N/A | The driver confirms "Track ahead is free". | I | DMI | - | Before the train reaches the D_TAFDISPLAY + L_TAFDISPLAY location. | N/A | N/A | - |
| 6 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 7 | N/A | N/A | The Driver Request symbol "Track Ahead Free" is removed | O | DMI | - | After driver ACK | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21, bit_and [DMI_SYMB_STATUS,MASK]=0)) is recorded. | O | JRU | - | - | N/A | N/A | - |
| 9 | N/A | N/A | SA-DATA. Request with Euroradio Message "Track Ahead Free Granted" (NID_MESSAGE=149) containing packet 0 or 1 is transmitted to the RBC (X=1). | O | RTM | 0010000 | The transmitted data remains unchanged (action U). | N/A | N/A | - |
| 10 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | - | N/A | N/A | - |

| Step 1: Radio Message 34 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 34 | Track Ahead Free Request |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | FINITE VALUE | 0/1. 0: No acknowledgement required 1: Acknowledgement required |
| NID_LRBG | 24 | FINITE VALUE | - |
| Q_SCALE | 4 | FINITE VALUE | - |
| Q_DIR | 2 | FINITE VALUE | - |
| D_TAFDISPLAY | 15 | FINITE VALUE | Distance from where on a track ahead free request shall be displayed |
| L_TAFDISPLAY | 15 | FINITE VALUE | Length on which a track ahead free request shall be displayed |

| Step 9: Radio Message 149 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 149 | Track Ahead Free Granted |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Packet 0 or 1 |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1/ 2/ 6/ 8 / 12 | OS/ SR/ SB / PT / LS |
| Radio communication session | ESTABLISHED | UNCHANGED |
| Mode Profile | STORED | Only applicable for LS and OS modes |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |
| Track Ahead Free Request | STORED | Status Changed |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 6.6.3.3 Messages received from RBC/RIU |  | 6060303 |
| Test Case of Feature | The on-board equipment receives from an RBC certified to the system version number X=1 a message 40 or 41 that is accepted. The on-board equipment translated the received data correctly. Tested action: [T = TRANSLATED] |  | 4 |
| Applicable Mode/Level Combinations | L2: SB L3: SB |  |  |
| Target of Test | The target of this test is to check that if, in the received message, the variable NID_LRBG ≠ 16777215, it shall be set to 16777215, i.e. the message shall be considered as having been received with NID_LRBG = 16777215. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | MULTITEL |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-6.6.3.3.2 Table - row 18 |  |
|  |  | Subset-026-6.6.3.3.2 [2] |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Within the procedure of start of mission in level 2 or 3 with session opened: On-board reports invalid position to RBC, (A34) RBC is not able to confirm position (D33) RBC accepts the train (D22, A23, A24) RBC reports "train accepted" with NID_LRBG set to a [0-16777214]. Check on JRU that: the received message 41 is translated correctly: NID_LRBG = 16777215 . |
| Constraints | The train has to be in SB MODE with session open. A cab needs to be active (SB Mode). |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | NOT ESTABLISHED | - |
| Train Position | INVALID | NID_LRBG and D_LRBG has a FINITE VALUE. |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION NOT SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Current mode and level are DISPLAYED. | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | M_VERSION | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver revalidates the RBC-ID/phone if invalid | - | - | - | Use_FT5040300.39 “If at least one Mobile Terminal is registered to a Radio Network, on-board equipment offers the driver the possibility to revalidate the RBC-ID/phone number if its status is “invalid”.” | N/A | N/A | - |
| 2 | N/A | N/A | Radio Communication session is established | - | - | - | Use_FT3050300.23 “Establishing a radio communication initiated by on-board to RBC via RBC ID and RBC phone number” | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA.Request with Euroradio Message "SoM Position Report" (NID_MESSAGE=157) containing packet 0 is transmitted | O | RTM | 001 0000 | Onboard reports invalid position to RBC | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | SoM position report RECORDED | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA.Indication with Euroradio Message "Train Accepted" (NID_MESSAGE=41) containing NID_LRBG !=16777215 is received | I | RTM | 001 0000 | RBC is Not able to confirm position (D33), it accepts the train (D22), it reports train accepted (A23) message 41, with NID_LRBG set to everything but unknown. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | message 41, with NID_LRBG = 16777215 is recorded | N/A | N/A | - |

| Step 3: Radio Message 157 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 157 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| Q_STATUS | 2 | 00 | INVALID |
| NID_PACKET | 8 | 0 | Packet 1 must be used if position report is based on 2 BG |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |

| Step 5: Radio Message 41 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 41 | - |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| M_ACK | 1 | 0 | - |
| NID_LRBG | 24 | FINITE VALUE | NID_LRBG != 16777215 |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 6 | SB |
| Radio communication session | ESTABLISHED | - |
| Train Position | UNKNOWN | NID_LRBG = 16777215 (unknown) |
| ERTMS/ETCS level | 3/ 4 | L2/ L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| Train data entry or NL or SH displayed | O | DMI | Driver is offered to select (Procedure S10) |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |