| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 4080409 (4.8.4: Accepted Information depending on the modes - Default Balise Information) Total: 3 Test case(s) |  |
| REF: | Subset-076-5-2-4080409 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 0.0.1 23.03.2012 | All | First version/review | DLR |
| Version 2.9.0 31.03.2012 | All | First version/review for Baseline 3 | CEDEX |
| Version 2.9.1 04.10.2013 | All | Requirements corrections | CEDEX |
| Version 2.9.2 20.11.2013 | ALL | REVIEW | CEDEX |
| Version 2.9.3 13.12.2013 | ALL | 2ND REVIEW | CEDEX |
| Version 2.9.4 17.12.2013 | ALL | 3D REVIEW | CEDEX |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Balise Information |  | 4080409 |
| Test Case of Feature | Verify that default information is received and stored in case of defect LEU. |  | 1 |
| Applicable Mode/Level Combinations | L0: SH, UN, SL, SB, NL, TR, PS L1: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L2: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV L3: FS, LS, OS, SR, SH, PS, SL, SB, TR, PT, NL, RV NTC : SN |  |  |
| Target of Test | The train passes over a balise group and detects a failure in the LEU when: Packet 254 is read in the telegram (Only default balise information is detected in telegram). The Driver is informed |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-4.8.3.1.1 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.7.2 table - Output information Trackside malfunction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The balise group message is RECORDED Check on the DMI that: Error indication is DISPLAYED. |
| Constraints | A cab needs to be active (if mode is SB). The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 254 is received | I | BTM | - | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Trackside malfunction" is DISPLAYED. | O | DMI | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/1) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | FINITE VALUE | Total number of balises in the balise group |
| M_DUP | 2 | 0 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 254 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 3 / 4 / 5 / 6 / 7 / 8 / 11 / 12 / 13 / 14 / 15 | FS / OS / SR / SH / UN / SL / SB / TR / PT / NL / LS / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Balise Information |  | 4080409 |
| Test Case of Feature | Verify that the onboard equipment ignores a telegram with “default balise information” out of a pair of duplicated balise telegram. |  | 2 |
| Applicable Mode/Level Combinations | L0: UN, SL, TR, SB, NL L1: FS, OS, SR, SB, TR, PT, NL, RV L2: FS, OS, SR, SB, TR, PT, NL, RV L3: FS, OS, SR, SB, TR, PT, NL, RV NTC :SN |  |  |
| Target of Test | The train passes over a balise group and detects a failure in the LEU when: First balise telegram Packet 254 is received. Second balise telegram is received. First balise telegram should be ignored. The Driver is informed about default error. the message from the second balise is managed and displayed to the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.2.4.8.1 |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.8.3.1.1 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |
|  |  | Subset-026-4.7.2 table - Output information Trackside malfunction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The first balise group message with Default Balise Information is RECORDED The second balise group message is RECORDED. Check on the DMI that: Error indication is DISPLAYED. The second balise group message is used from onboard device instead of first balise message. |
| Constraints | A cab needs to be active (if mode is SB). The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 254, packet 72 is received | I | BTM | Onboard device accepts and stores information of second balise telegram. THE TEXT MESAGE FROM FIRST BALISE SHOULD BE IGNORED, THE TEXT MESAGE FROM SECOND BALISE SHOULD BE DISPLAYED, the condition to display the message should be the same in the two balises | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Trackside malfunction" is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The "plain text message" is displayed | O | DMI | the message from the second balise | N/A | N/A | - |
| 5 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 1 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 254 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 10 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 6 / 7 / 8 / 11 / 12 / 13 / 14 | FS / OS / SR / UN / SB / TR / PT / NL / LS / SN / RV |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 4.8.4: Accepted Information depending on the modes - Default Balise Information |  | 4080409 |
| Test Case of Feature | Verify that the onboard equipment ignores first telegrams with “default balise information” out of a pair of duplicated balise telegram. |  | 3 |
| Applicable Mode/Level Combinations | L0: UN, SL, SB, NL, TR L1: FS, OS, SR, SB, TR, PT, NL, RV L2: FS, OS, SR, SB, TR, PT, NL, RV L3: FS, OS, SR, SB, TR, PT, NL, RV NTC :SN |  |  |
| Target of Test | The train passes over a balise group with two balises and detects a failure in the LEU when: First balise telegram Packet 254 is received. Second balise telegram Packet 254 is received. First balise telegrams should be ignored. The Driver is informed about default error. the message from the second balise is managed and displayed to the driver. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | RINA |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.2.4.8.2 |  |
|  |  | Subset-026-3.16.2.4.9 |  |
|  |  | Subset-026-4.8.3.1.1 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 Default Balise/Loop/RIU Information |  |
|  |  | Subset-026-4.8.4.2 [2] |  |
|  |  | Subset-026-4.4.17.1.2 |  |
|  |  | Subset-026-7.4.2.38 Packet Number 254: Default balise, loop or RIU information |  |
|  |  | Subset-026-4.7.2 table - Output information Trackside malfunction |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the JRU that : The first balise group message with Default Balise Information is RECORDED. The second balise group message with Default Balise Information is RECORDED. Check on the DMI that: Error indication is DISPLAYED. The second balise group message is used from onboard device instead of first balise message. |
| Constraints | A cab needs to be active (if mode is SB).The reception of balise information in SB requires the train moving without an immediate movement protection. The national value D_NVROLL has to be set to a high value, enabling the train to read a BG |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 13 / 14 / 15 | FS / OS / SR / UN / SL / SB / TR / PT / NL / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | One Balise Group message containing packet 254 , packet 72 is received | I | BTM | Onboard device accepts and stores information from the last received balise telegram out of the pair. THE TEXT MESAGE FROM FIRST BALISE SHOULD BE IGNORED, THE TEXT MESAGE FROM SECOND BALISE SHOULD BE DISPLAYED, the condition to display the message should be the same in the two balises | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | The system status message "Trackside malfunction" is DISPLAYED. | O | DMI | - | N/A | N/A | - |
| 4 | N/A | N/A | The "plain text message" is displayed | O | DMI | the message from the second balise | N/A | N/A | - |
| 5 | N/A | N/A | START DISPLAYING PLAIN TEXT MESSAGE (NID_MESSAGE_JRU=18) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 01 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 254 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | FINITE VALUE | Position in the group. Defines the position of the balise in the balise group. |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group |
| M_DUP | 2 | 10 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | Identity of the balise group |
| Q_LINK | 1 | FINITE VALUE | Marks the balise group as linked (Q_LINK = 1) or unlinked (Q_LINK = 0) |
| NID_PACKET | 8 | 72 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| Q_TEXTCLASS | 2 | FINITE VALUE | - |
| Q_TEXTDISPLAY | 1 | FINITE VALUE | Start/end events relation. |
| D_TEXTDISPLAY | 15 | FINITE VALUE | Start event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | Start event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | Start event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| L_TEXTDISPLAY | 15 | FINITE VALUE | End event. |
| T_TEXTDISPLAY | 10 | FINITE VALUE | End event. |
| M_MODETEXTDISPLAY | 4 | FINITE VALUE | End event. |
| M_LEVELTEXTDISPLAY | 3 | FINITE VALUE | End event. |
| NID_NTC | 8 | FINITE VALUE | If M_LEVELTEXTDISPLAY = 1 (NTC). |
| Q_TEXTCONFIRM | 2 | FINITE VALUE | - |
| Q_CONFTEXTDISPLAY | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| Q_TEXTREPORT | 1 | FINITE VALUE | If Q_TEXTCONFIRM ≠ 0. |
| NID_TEXTMESSAGE | 8 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_C | 10 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| NID_RBC | 14 | FINITE VALUE | Only If Q_TEXTREPORT = 1. |
| L_TEXT | 8 | FINITE VALUE | - |
| X_TEXT(L_TEXT) | 8 | FINITE VALUE | - |
| NID_PACKET | 8 | 254 | - |
| Q_DIR | 2 | FINITE VALUE | - |
| L_PACKET | 13 | FINITE VALUE | - |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 / 1 / 2 / 4 / 5 / 6 / 7 / 8 / 11 / 13 / 14 / 15 | FS / OS / SR / UN / SL / SB / TR / PT / NL / SN / RV / PS |
| ERTMS/ETCS level | 0 / 1 / 2 / 3 / 4 | L0 / LNTC / L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |