| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3151000 (3.15.10: Advance display of route related information) Total: 2 Test case(s) |  |
| REF: | Subset-076-5-2-3151000 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review | DLR |
| Version 2.9.1 16.09.2013 | All | review | DLR |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.0.0 24.11.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.10: Advance display of route related information |  | 3151000 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment shall display an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), of the indication location at MRSP speed (see 3.13.10.4.11) and of the first target at zero speed, if any (e.g. the EOA or the start location of a mode profile), with the remaining distances referred to the train front end position. With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances shall be computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. With regards to the gradient profile, the remaining distances shall be computed taking into account the estimated train front end position. With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. |  | 1 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: FS |  |  |
| Target of Test | The target of this test case is to check that : - on driver request, an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), of the indication location at MRSP speed and of the first target at zero speed, is displayed, with the remaining distances referred to the train front end position. - With regards to the MRSP, the track conditions, the indication location at MRSP speed and the first target at zero speed, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. - With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.10.1 |  |
|  |  | Subset-026-3.15.10.2 |  |
|  |  | Subset-026-3.15.10.3 |  |
|  |  | Subset-026-3.15.10.3.1 |  |
|  |  | Subset-026-3.15.10.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Gradient |  |
|  |  | Subset-026-4.7.2 table - Output information MRSP |  |
|  |  | Subset-026-4.7.2 table - Output information First Indication location |  |
|  |  | Subset-026-4.5.2.1 table - Advance display of route related information |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Pantograph control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Air tightness control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Radio hole, supervision of safe radio connection stopped |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The target of this test case is to check that: - independent from toggling the planning information in FS the following information is shown: - an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), EOA/LOA, is displayed, with the remaining distances referred to the train front end position. - With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. - With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position - With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. |
| Constraints | " A gradient profile is supervised A track condition is supervised An MRSP is supervised The first Indication location is supervised An EOA is supervised |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | V_EMA=0; so that the EOA is the first target at zero speed Gradient profile stored longer than MA in starting conditions |
| Gradient Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | FS | The speed and distance monitoring information is displayed | O | DMI | The overview of route related information is DISPLAYED. The overview includes: • the gradient profile (as received from trackside), • the MRSP, • the track conditions (except the tunnel stopping areas), • the EOA/LOA only up to the first target at zero speed With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position. With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. The overview of route related information is restricted to the elements contained within the movement authority and up to the first target at zero speed. | N/A | FS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0 | FS |
| Movement Authority | STORED | V_EMA = 0 |
| Gradient Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The overview of route related information is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.15.10: Advance display of route related information |  | 3151000 |
| Test Case of Feature | The ERTMS/ETCS on-board equipment shall display an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), of the indication location at MRSP speed (see 3.13.10.4.11) and of the first target at zero speed, if any (e.g. the EOA or the start location of a mode profile), with the remaining distances referred to the train front end position. With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances shall be computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. With regards to the gradient profile, the remaining distances shall be computed taking into account the estimated train front end position. With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. The overview of route related information shall be restricted to the elements contained within the movement authority and up to the first target at zero speed, if any. |  | 2 |
| Applicable Mode/Level Combinations | L1 / L2 / L3: OS |  |  |
| Target of Test | The target of this test case is to check that : - on driver request, an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), of the indication location at MRSP speed and of the first target at zero speed, is displayed, with the remaining distances referred to the train front end position. - With regards to the MRSP, the track conditions, the indication location at MRSP speed and the first target at zero speed, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. - With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | DLR |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.15.10.1 |  |
|  |  | Subset-026-3.15.10.2 |  |
|  |  | Subset-026-3.15.10.3 |  |
|  |  | Subset-026-3.15.10.3.1 |  |
|  |  | Subset-026-3.15.10.4 |  |
|  |  | Subset-026-4.7.2 table - Output information Gradient |  |
|  |  | Subset-026-4.7.2 table - Output information MRSP |  |
|  |  | Subset-026-4.7.2 table - Output information First Indication location |  |
|  |  | Subset-026-4.5.2.1 table - Advance display of route related information |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Power control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Pantograph control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Air tightness control |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Radio hole, supervision of safe radio connection stopped |  |
|  |  | Subset-026-4.7.2 table - Output information Track condition excluding sound horn, non stopping areas, tunnel stopping areas and big metal masses - Brakes control |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | The target of this test case is to check that: - an overview of the gradient profile (as received from trackside), of the MRSP, of the track conditions (except the tunnel stopping areas), EOA/LOA, is displayed, with the remaining distances referred to the train front end position. - With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. - With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position - With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. |
| Constraints | The DMI Planning speed and distance monitoring information is toggled on. (Only for OS) " A gradient profile is supervised A track condition is supervised An MRSP is supervised The first Indication location is supervised An EOA is supervised |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | V_EMA=0; so that the EOA is the first target at zero speed Gradient profile stored longer than MA in starting conditions |
| Gradient Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | The driver requests the speed and distance monitoring information to be removed | I | DMI | Only if information is currently displayed. The driver requests to hide the planning area. | N/A | N/A | - |
| 2 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | Only if information is currently displayed. | N/A | N/A | - |
| 3 | N/A | N/A | The speed and distance monitoring information is removed | O | DMI | Only if information is currently displayed. | N/A | N/A | - |
| 4 | N/A | N/A | The driver requests the speed and distance monitoring information to be displayed | I | DMI | The driver requests the display of an overview of route related information | N/A | N/A | - |
| 5 | N/A | N/A | DRIVER’S ACTIONS (NID_MESSAGE_JRU=11) is recorded | O | JRU | - | N/A | N/A | - |
| 6 | N/A | OS | The speed and distance monitoring information is displayed | O | DMI | The overview of route related information is DISPLAYED. The overview includes: • the gradient profile (as received from trackside), • the MRSP, • the track conditions (except the tunnel stopping areas), • the EOA/LOA only up to the first target at zero speed With regards to the MRSP, the track conditions, EOA/LOA, the remaining distances are computed taking into account the min safe, the estimated or the max safe train front end position depending on their respective supervision. With regards to the gradient profile, the remaining distances are computed taking into account the estimated train front end position. With regards to the first Indication location, the remaining distance shall be computed as specified in clauses 3.13.10.3.8 and 3.13.10.3.8.1. The overview of route related information is restricted to the elements contained within the movement authority and up to the first target at zero speed. | N/A | OS | - |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 1 | OS |
| Movement Authority | STORED | V_EMA = 0 |
| Gradient Profile | STORED | - |
| ERTMS/ETCS level | 2 / 3 / 4 | L1 / L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| NOT RELEVANT | - | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The overview of route related information is DISPLAYED | O | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |