| ERTMS/ETCS |  |
| --- | --- |
| Test cases of Feature 3160400 (3.16.4 Error reporting to RBC) Total: 5 Test case(s) |  |
| REF: | Subset-076-5-2-3160400 |
| ISSUE: | Version 3.3.0 |
| DATE: | 31.03.2017 |

| Edited by | Quality review | Approved by |
| --- | --- | --- |
| Subset-076 WG | European Union Agency for Railways | European Union Agency for Railways |

# Modification History

| Issue Number Date | Section Number | Modification / Description | Author |
| --- | --- | --- | --- |
| Version 2.9.0 31.03.2013 | All | First version/review for Baseline 3/TC Editor tool used and Dictionary Steps added | INECO |
| Version 2.9.1 17.07.2013 | All | Second version/ UNISIG review comments from 10.07.2013 included | INECO |
| Version 2.9.2 17.12.2013 | All | Third version/TC_Editorv31; TC2 - Requirement 8.4.1.2 added; Requirements related with Chapter 7 and 8 added. | INECO |
| Version 2.9.3 13.01.2014 | All | Fourth version/ Internal version: Cross-reference checked; Old reference removed; Remarks from UNISIG review sheet 006 inserted. | INECO |
| Version 3.0.0 14.02.2014 | All | Test cases renumbering | CEDEX |
| Version 3.1.0 12.06.2015 | All | Baseline 3 1st maintenance release version | CEDEX / INECO / DLR / MULTITEL |
| Version 3.3.0 31.03.2017 | All | Baseline 3 2nd release version | CEDEX / INECO / DLR / MULTITEL |

# Test Cases

## Test case 1

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.4 Error reporting to RBC |  | 3160400 |
| Test Case of Feature | In level 2/3, if a radio communication session is established, balise group error shall be reported as soon as the availability of a safe radio connection permits. |  | 1 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: A loss of safe radio connection is detected by the on-board equipment The on-boards equipment detects an error on reading a balise group message As soon as the safe connection is available again the on-board informs the trackside about the detected error by sending message 136 with packet 4 (M_ERROR =1). The on-board function “Report train position when a data consistency error is detected“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.4.1 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 l) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when a data consistency error is detected (only level 2/3) |  |
|  |  | Subset-026-8.4.4.4.2 |  |
|  |  | Subset-026-8.4.4.4.2 a) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM: Message 136 with packet 4 is send when the safe radio connection is available again. Check on the DMI that: The change of Safe Radio Connection Indication from “Connection Up“ to “Connection lost /Set-up failed“ status and to „Connection Up“ again is DISPLAYED Check on the JRU that: The messages 136 with packet 4 reporting the error (M_ERROR=1) and DMI messages are RECORDED |
| Constraints | A radio communication session shall be established. Linking information stored on-board with BG1 contained in the linking information but not set to any linking reaction (Q_LINKREACTION = 10) |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 11/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | BG1 contained Q_LINKREACTION set to 10 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Loss of safe radio connection. The on-board equipment execute the transition between “Connection Up” status and “Connection Lost / Set-up failed” status | - | - | Use_FT3050700.3 | N/A | N/A | - |
| 2 | N/A | N/A | Balise Group message (N_TOTAL=1) containing an error (one of the balises with N_PIG=2) is received | I | BTM | Balise group contained in the linking information | N/A | N/A | - |
| 3 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is REMOVED. | O | DMI | DMI Object ST04 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 and packet 4 is transmitted | O | RTM | Packet 4: M_ERROR = 1 | N/A | N/A | - |
| 12 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 2: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group: 2 |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 2: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 002 | Third |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group: 2 |
| M_DUP | 2 | 00 | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 11: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 1 | Linked Balise Group: message consistency error |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 11/ 12/ 14/ 15 | FS/ OS/ SR/ SB/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 2

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.4 Error reporting to RBC |  | 3160400 |
| Test Case of Feature | In level 2/3, if a radio communication session is established safe radio connection error shall be reported as soon as the availability of a safe radio connection permits. |  | 2 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS/ SR/ SB/ TR/ PT/ NL/ RV |  |  |
| Target of Test | The target of this test is to check that: Train receives message 24 from RBC The on-boards equipment detects a loss of a safe radio connection error (T_NVCONTACT expires) As soon as the safe radio connection is available again the on-board informs the trackside about the detected error by sending message 136 with packet 4 (M_ERROR =5). The on-board function “Report train position when a data consistency error is detected“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.4.1 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 l) |  |
|  |  | Subset-026-A.3.1 - Additional delay time to disconnection on supervision of safe radio connection |  |
|  |  | Subset-026-A.3.1 - "Connection status" timer for safe radio connection indication |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when a data consistency error is detected (only level 2/3) |  |
|  |  | Subset-026-8.4.4.4.2 |  |
|  |  | Subset-026-8.4.4.4.2 a) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |
|  |  | Subset-026-8.7.9 Message 24: General message |  |
|  |  | Subset-026-8.4.1.2 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM: Message 136 with packet 4 is send when the safe radio connection is available again. Check on the JRU that: The messages 136 with packet 4reporting the error (M_ERROR = 5) and DMI messages are RECORDED |
| Constraints | A radio communication session shall be established. No reaction due to loss of safe radio connection shall be considered. National Values stored on-board. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 11/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | SA-DATA. Indication with Euroradio Message "General message" (NID_MESSAGE=24; T_TRAIN1) is received | I | RTM | - | N/A | N/A | - |
| 2 | N/A | N/A | MESSAGE FROM RBC (NID_MESSAGE_JRU=9) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | RADIO ERROR (NID_MESSAGE_JRU=13) is recorded | O | JRU | No new consistent radio message is received within the T_NVCONTACT time + additional time delay (60s) | N/A | N/A | - |
| 4 | N/A | N/A | SA-DISCONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DISCONNECT. Indication is received | I | RTM | The on-board releases the safe connection | N/A | N/A | - |
| 6 | N/A | N/A | SA-CONNECT. Request is transmitted | O | RTM | - | N/A | N/A | - |
| 7 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is removed. | O | DMI | DMI Object: ST03 | N/A | N/A | - |
| 8 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is displayed. | O | DMI | after expiration of “connection status” timer”(45 sec) DMI Object ST04 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=1>) is recorded. | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | SA-CONNECT. Confirm is received | I | RTM | - | N/A | N/A | - |
| 12 | N/A | N/A | The Status symbol "Safe radio connection - Connection Lost/Set-Up failed" is REMOVED. | O | DMI | DMI Object ST04 | N/A | N/A | - |
| 13 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit41=0>) is recorded. | O | JRU | - | N/A | N/A | - |
| 14 | N/A | N/A | The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI object: ST03 | N/A | N/A | - |
| 15 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit40=1>) is recorded. | O | JRU | - | N/A | N/A | - |
| 16 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 0 and packet 4 is transmitted | O | RTM | Packet 4: M_ERROR = 5 | N/A | N/A | - |
| 17 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded | O | JRU | - | N/A | N/A | - |

| Step 1: Radio Message 24 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 24 | General Message |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | = T_TRAIN 1 |
| M_ACK | 1 | 0 | Acknowledge of message required = 1, Not required =0 |
| NID_LRBG | 24 | FINITE VALUE | - |

| Step 16: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 5 | Radio: safe radio connection error |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 2/ 6/ 7/ 8/ 11/ 12/ 14 | FS/ OS/ SR/ SB/ TR/ PT/ NL/ LS/ RV |
| Radio communication session | ESTABLISHED | - |
| National Values | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| The Status symbol "Safe radio connection - Connection Up" is displayed. | O | DMI | DMI Object: ST03 |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 3

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.4 Error reporting to RBC |  | 3160400 |
| Test Case of Feature | In level 2/3, if linking information is used on-board, no error reporting shall be done for balise groups marked as linked but not included in the linking information |  | 3 |
| Applicable Mode/Level Combinations | L2 / L3: FS/ LS/ OS |  |  |
| Target of Test | The target of this test is to check that: An inconsistent balise group marked as linked but not included in the linking information is received. The on-board does not inform the trackside about the detected error by sending message 136 with packet 4 |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.4.3 |  |
|  |  | Subset-026-3.16.2.4.3 |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on RTM: Message 136 with packet 4 is not send. Check on the JRU that: The messages 136 with packet 4 is not RECORDED |
| Constraints | A radio communication session shall be established. Linking information is used by on-board equipment but BG1 not contained. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | BG1 not contained Q_LINKREACTION set to 10 |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Balise Group message (N_TOTAL=1) is received. | I | BTM | Balise group marked as linked but not contained in the linking information | N/A | N/A | - |
| 2 | N/A | N/A | TELEGRAM FROM BALISE (NID_MESSAGE_JRU=6) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | BALISE GROUP ERROR (NID_MESSAGE_JRU=12) is recorded | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | NOT SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) is transmitted | O | RTM | - | N/A | N/A | - |

| Step 1: Eurobalise Telegram (balise 1/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 000 | First |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group: 2 |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| Step 1: Eurobalise Telegram (balise 2/2) |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| Q_UPDOWN | 1 | 1 | Defines the direction of the information: Down-link telegram (train to track) (0), Up-link telegram (track to train) (1) |
| M_VERSION | 7 | FINITE VALUE | Version of the ERTMS/ETCS system |
| Q_MEDIA | 1 | 0 | Defines the type of media: Balise (0) |
| N_PIG | 3 | 001 | Second |
| N_TOTAL | 3 | 1 | Total number of balises in the balise group: 2 |
| M_DUP | 2 | FINITE VALUE | Used to indicate whether the information of the balise is a duplicate of the balise before or after this one. |
| M_MCOUNT | 8 | FINITE VALUE | Message counter (M _MCOUNT) - 8 bits. To enable detection of a change of balise group message during passage of a balise group |
| NID_C | 10 | FINITE VALUE | Country or region |
| NID_BG | 14 | FINITE VALUE | BG1 |
| Q_LINK | 1 | 1 | Linked |
| NID_PACKET | 8 | 255 | = 255 (1111 1111) |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS/ LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 4

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.4 Error reporting to RBC |  | 3160400 |
| Test Case of Feature | A safety critical fault occurs. (M_ERROR= 6) The on-board equipment sends a radio message 136 “Train Position Report” containing the packet 4 “Error reporting” to the RBC. |  | 4 |
| Applicable Mode/Level Combinations | L2 / L3: SL / NL |  |  |
| Target of Test | The target of this test is to check that : The on-board informs the RBC when a safety critical fault occurs.(M_ERROR= 6). The on-board function “Report train position when a data consistency error is detected“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.4.1 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 l) |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when a data consistency error is detected (only level 2/3) |  |
|  |  | Subset-026-8.4.4.4.2 |  |
|  |  | Subset-026-8.4.4.4.2 a) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: A radio message 136 (Train Position Report) with the packet 4 (M_ERROR=6) is sent to the RBC. Check on the JRU that: The radio message 136 (Train Position Report) with the packet 4 (M_ERROR=6) sent by the on-board is recorded. |
| Constraints | Sending of error report cannot be guaranteed due to different possible effects of a safety critical fault. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | A safety critical fault occurs | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | SAFETY CRITICAL FAULT IN MODE SL, NL (NID_MESSAGE_JRU=27) is recorded | O | JRU | - | N/A | N/A | - |
| 3 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) containing packet 4 is transmitted. | O | RTM | - | N/A | N/A | - |
| 4 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |

| Step 3: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 6 | Safety critical Fault |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 5 / 11 | SL / NL |
| Radio communication session | ESTABLISHED | - |
| ERTMS/ETCS level | 3 / 4 | L2 / L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

## Test case 5

| IDENTIFICATION |  |  |  |
| --- | --- | --- | --- |
|  | Title |  | Unique Number |
| Tested Equipment | On-board equipment |  |  |
| Tested Feature | 3.16.4 Error reporting to RBC |  | 3160400 |
| Test Case of Feature | A double linking error occurs. (M_ERROR= 7) The on-board equipment sends a radio message 136 “Train Position Report” containing the packet 4 “Error reporting” to the RBC. |  | 5 |
| Applicable Mode/Level Combinations | L2: FS / LS / OS L3: FS / LS / OS |  |  |
| Target of Test | The target of this test is to check that : The on-board informs RBC about the error of not founding two balises group included in the linking information. (M_ERROR= 7). The on-board function “Report train position when a data consistency error is detected“ is active. |  |  |
| Version | 3.3.0 | 31.03.2017 |  |
| Author | INECO |  |  |
| Based on Requirements | ERTMS/ETCS - SRS 3.6.0 | Subset-026-3.16.4.1 |  |
|  |  | Subset-026-3.6.5.1.4 |  |
|  |  | Subset-026-3.6.5.1.4 l) |  |
|  |  | Subset-026-3.16.2.7.1.1 |  |
|  |  | Subset-026-4.5.2.1 table - Report train position when a data consistency error is detected (only level 2/3) |  |
|  |  | Subset-026-8.4.4.4.2 |  |
|  |  | Subset-026-8.4.4.4.2 a) |  |
|  |  | Subset-026-7.4.3.1 Packet Number 0: Position Report |  |
|  |  | Subset-026-7.4.3.4 Packet Number 4: Error reporting |  |
|  |  | Subset-026-8.6.4 Message 136: Train Position Report |  |

| METHOD OF TEST |  |
| --- | --- |
| Method | Check on the RTM that: A radio message 136 (Train Position Report) with the packet 4 (M_ERROR=7) is transmitted Check on the JRU that: The radio message 136 (Train Position Report) with packet 4 is RECORDED. |
| Constraints | Linking information stored on-board. The reference locations of the two consecutive BG (marked as linked and included in the linking information) are not found in the expectation windows. |

| STARTING CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | - |
| Linking | STORED | BGA and BGB contained |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| REQUIRED STARTING CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| NOT RELEVANT | - | TIU | - |
| NOT RELEVANT | - | DMI | - |
| NOT RELEVANT | - | BTM | - |
| NOT RELEVANT | - | INT | - |

| SEQUENCE OF TEST |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Step | Previous |  | Description of Events | I/O | Interface | Comments | Next |  | Test Result |
|  | Levels | Modes |  |  |  |  | Levels | Modes |  |
| 1 | N/A | N/A | Train passes the expectation window of the first announced BG A. The balise group is not detected. | I | INT | - | N/A | N/A | - |
| 2 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) with packet 4 (Error Reporting) is sent. | O | RTM | Packet 4 contains the value M_ERROR = 0 The RBC is informed about the linking consistency error | N/A | N/A | - |
| 3 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 4 | N/A | N/A | Train passes the expectation window of the first announced BG B. The balise group is not detected. | I | INT | - | N/A | N/A | - |
| 5 | N/A | N/A | SA-DATA. Request with Euroradio Message "Train Position Report" (NID_MESSAGE=136) with packet 4 (Error Reporting) is sent. | O | RTM | Packet 4 contains the value M_ERROR = 7 The RBC is informed about the double linking error. | N/A | N/A | - |
| 6 | N/A | N/A | MESSAGE TO RBC (NID_MESSAGE_JRU=10) is recorded. | O | JRU | - | N/A | N/A | - |
| 7 | N/A | N/A | Service brake commanded | O | TIU | - | N/A | N/A | - |
| 8 | N/A | N/A | SERVICE BRAKE COMMAND STATE (NID_MESSAGE_JRU=4; M_BRAKE_COMMAND_STATE=1) is recorded | O | JRU | - | N/A | N/A | - |
| 9 | N/A | N/A | The Status symbol "Service Brake intervention" is displayed | O | DMI | DMI Object: ST01 | N/A | N/A | - |
| 10 | N/A | N/A | DMI SYMBOL STATUS (NID_MESSAGE_JRU=21; DMI_SYMB_STATUS=<Bit38=1>) is recorded | O | JRU | - | N/A | N/A | - |
| 11 | N/A | N/A | The system status message "Balise read error" is displayed | O | DMI | - | N/A | N/A | - |
| 12 | N/A | N/A | DMI SYSTEM STATUS MESSAGE (NID_MESSAGE_JRU=23) is recorded | O | JRU | - | N/A | N/A | - |
| 13 | N/A | N/A | Train is at standstill (V_TRAIN=0; TIME= 60) | I | INT | V_ TRAIN= 0 | N/A | N/A | - |

| Step 2: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 0 | Linking consistency error |

| Step 5: Radio Message 136 |  |  |  |
| --- | --- | --- | --- |
| Variable | Length | Value | Comment |
| NID_MESSAGE | 8 | 136 | Train Position Report |
| L_MESSAGE | 10 | FINITE VALUE | - |
| T_TRAIN | 32 | FINITE VALUE | - |
| NID_ENGINE | 24 | FINITE VALUE | - |
| NID_PACKET | 8 | 0 | Position Report |
| L_PACKET | 13 | FINITE VALUE | - |
| Q_SCALE | 2 | FINITE VALUE | - |
| NID_LRBG | 24 | FINITE VALUE | - |
| D_LRBG | 15 | FINITE VALUE | - |
| Q_DIRLRBG | 2 | FINITE VALUE | - |
| Q_DLRBG | 2 | FINITE VALUE | - |
| L_DOUBTOVER | 15 | FINITE VALUE | - |
| L_DOUBTUNDER | 15 | FINITE VALUE | - |
| Q_LENGTH | 2 | FINITE VALUE | - |
| L_TRAININT | 15 | FINITE VALUE | If Q_LENGTH = "Train integrity confirmed by integrity monitoring device" or "Train integrity confirmed by driver". |
| V_TRAIN | 7 | FINITE VALUE | - |
| Q_DIRTRAIN | 2 | FINITE VALUE | - |
| M_MODE | 4 | FINITE VALUE | - |
| M_LEVEL | 3 | FINITE VALUE | - |
| NID_NTC | 8 | FINITE VALUE | If M_LEVEL = NTC. |
| NID_PACKET | 8 | 4 | Error Reporting |
| L_PACKET | 13 | FINITE VALUE | - |
| M_ERROR | 8 | 7 | Double linking error |

| END CONDITIONS (INTERNAL STATES) |  |  |
| --- | --- | --- |
| States of ERTMS/ETCS information | Status/Value | Description |
| ERTMS/ETCS Mode | 0/ 1/ 12 | FS/ OS /LS |
| Radio communication session | ESTABLISHED | - |
| Linking | NOT STORED | - |
| ERTMS/ETCS level | 3 / 4 | L2 /L3 |

| END CONDITIONS ON INTERFACES |  |  |  |
| --- | --- | --- | --- |
| State of interfaces | I/O | Interface | Comments |
| SAFE CONNECTION SET-UP | I/O | RTM | - |
| Service brake commanded | O | TIU | - |
| The Status symbol "Service Brake intervention" is displayed | O | DMI | ST01 |
| NOT RELEVANT | - | BTM | - |
| Train at standstill | I | INT | V_TRAIN=0 |